Key Project Information & project Design Document (PDD)

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.2 
RELATED SUPPORT

###### - TEMPLATE GUIDE Key Project Information & Project Design Document v.1.2

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or    demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Outcome of Stakeholder Consultations

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of Project participants (mandatory)

Appendix 3 - LUF Additional Information (project specific)

Appendix 3 - Summary of Approved Design Changes (project specific)

### KEY PROJECT INFORMATION

| GS ID of Project | GS5920 |
| --- | --- |
| Title of Project | ALTACA MERAM BIOGAS PLANT |
| Time of First Submission Date | 14/09/2019 |
| Date of Design Certification |  |
| Version number of the PDD | 09 |
| Completion date of version | 29/08/2023 |
| Project Developer | GTE KARBON SUSTAINABLE ENERJI EGITIM DANISMANLIK VE TICARET A.S. |
| Project Representative | TOROS MERAM YENİLENEBİLİR ENERJİ ÜRETIM A.Ş. |
| Project Participants and any communities involved | - |
| Host Country (ies) | Turkey |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | - |
| Methodology (ies) applied and version number | The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reduction from Manure Management Systems and Municipal Solid Waste |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 Climate Action (mandatory) | ## The project contributes to CO2e reduction, which represent direct and quantifiable impact on climate security. Moreover, the project is also expected to reduce SO2 and NOx emissions. | ## The project contributes to 110,277 tonnes of CO2e reduction annually, which represent direct and quantifiable impact on climate security. Also, the project is expected to reduce 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC) . |  |
| ## 7 Affordable and Clean Energy | ## Annual average estimated renewable electricity generation | ## 42.028 GWh |  |
| ## 8 Decent Work and Economic Growth | ## Project provides job opportunities. | ## Employee will gather H&S and other related trainings. Around 55 people will be employed. |  |
| ## 12 Ensure Sustainable Consumption and Production Patterns | ## The project provides better management option for wastes ( cattle and poultry manure) via anaerobic treatment | ## The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste) |  |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1 Purpose and general description of project

Meram Biogas Plant was constructed Toros Meram Yenilenebilir Enerji Üretim A.Ş.  in Konya, Turkey. The main feedstock for the BPP (Biogas Power Plant) is manure in Meram and nearest regions. The digesters and equipment are also designed to utilize other wastes in the region causing by agricultural or industrial activities which are also renewable biomass resources such as potatoes or sugar beet wastes. The plant uses 6.176 MWm / 6.004 MWe (with four gas engines, 4 x 1.544 MWm / 4 x 1.501 MWe) with an expected output of 42.028 GWh of electricity (as per generation license)

Main goals of the project are:

-	Waste collected from the animal shelters will be decomposed and turned in to more stable state fertilizer, which is environmentally favorable and odorless,

-	Heat demand of the facility will be met

-	Biogas from cattle excrement will be used to produce clean electrical energy,

-	The obtained fertilizer will have less odor compared to present fertilizers and will contain more free nitrogen (N)

The proposed project activity is a biogas-to-energy and generates renewable energy by capturing biogas from cattle manure, chicken manure and agricultural wastes -via anaerobic digestion- and utilising it to produce electricity. Prior to the project activity, baseline situation, cattle, chicken manure generated at farms managed by lagoons and agricultural wastes left in agricultural lands.  Hence:

- The situation of cattle and chichken manure is somply with the applied methodology which states “This methodology is applicable to manure management on one or multiple livestock farms where the existing anaerobic manure treatment system, within the project boundary, is replaced by one or a combination of more than one animal waste management systems (AWMSs) that result in less GHG emissions compared to the existing system”

- The situation of agricultural waste is complied with the applied methodology which states “In case of co-digestion, for one or more sources of substrates, it cannot be demonstrated that the organic matter would otherwise have been left to decay anaerobically, baseline emissions related to such organic matter shall be accounted for as zero,”

In addition, there was not any biogas plant in the region before this project and there was no anaerobic biogas generation activity too. Hence the project became a Greenfield Project activity in terms of biogas generation within the region. In terms of quantum of waste handled within project activity, Meram Biogas Plant project operates with a yearly organic flow of 245,645 tonne and 47.5 % of this amount is belonged to cattle manure (116,800 tonne/year), 50.9% of this amount is belonged to chicken manure (125,195 tonne/year) and rest of it belonged to agricultural wastes (3,650 tonne/year) as per the information given in EIA report of the project (page 5).

In addition, in the baseline situation, there is no renewable electricity generation and export to national grid via a biogas plant in the region. In relation to this, project increase the renewable capacity of Turkey as host party via electricity generation using biogas produced by using wastes indicated above. This renewable electricity generation strongly related with Sustainable Development Goal 7 which is mentioned on renewable energy generation and increasing capacity on that manner. Electricity generated by the project increased the renewable energy generation of Turkey. Moreover, project owner employed 22 employees for the present situation which contributes to the SDG 8 “Decent Work and Economic Growth“since project creates job opportunities. Also, as detailed situation given in paragraph above, project managed 245,645 tonnes/year of waste managed with this project which contributes to the sustainable development pathway of Turkey related with SDG 12.

Technology applied will involve use of cattle manure, chicken manure and agricultural wastes. “Mesophilic” technology will be applied according to the operating temperature in the planned biogas plant.  The optimal operating temperatures of these bacteria are 37ºC. Manure is mixed and hydrolyzed here to break the bonds and generate biogas. Biogas obtained will be fed to the cogeneration unit to generate heat and electricity whereas final product will be stored and used as fertilizer. Electricity generated will be fed to the national grid.

The system consists of following units:

Waste collection unit: Weigh bridge and data control/storage system

Waste preparation / feed unit: Equalization tank, vegetative waste stock area, central pumping system

Anaerobic Digestion Unit: Digestors, gas storage unit, desulfurization unit, early warning system, temperature control system

Cogeneration Unit: Cogeneration, gas treatment system, flare

Fertilizer Dewatering / Hygenization Unit: Post-digestion tanks, hygenization unit, separator

Automation system

Product Storage Area: Liquid fermented storage area, solid fermented storage area

Thanks to the project, an annual emission reduction of 110,277 tonnes of CO2e is achieved. The total emission reduction for the crediting period is 551,385 tonnes of CO2e. Also, the project is expected to reduce 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC which is the latest published national inventory data consisting SO2 and NOx emissins) .

In the absence of the project activity, the waste materials used as energy sources would have been disposed but not been utilised and the energy demand would be met using fossil fuels.

The milestones of the project are presented in Table 1 below.

Table 1 Milestones of Meram Biogas Power Plant Project

| Milestone | Date |
| --- | --- |
| Approval of The Feasibility Report | 05/08/2016 |
| EIA Approval | 13/12/2016 |
| LSC Meeting | 02/08/2017 |
| Generation License | 04/10/2018 |
| EPC Agreement (Construction Agreement) | 01/12/2019 |
| Site Delivery Protocol | 02/12/2019 |
| Gas Engine Purchase Agreement (Investment Decision Date) | 02/01/2020 |
| The Distribution System Agreement | 13/05/2020 |
| Gas Engine Commissioning | 27/11/2020 |
| Start Date of First Crediting Period | 01/01/2021 |
| Certificate of permission for use of building | 13/08/2021 |

Eligibility of the project under Gold Standard

Section 3.1.1 (titled as ‘Eligible Project Types’) of the document of Gold Standard for the Global Goals (GS4GG) Principles & Requirements (Version 1.2.)  states that

“A Project type is automatically eligible for Gold Standard Certification if there are approved Gold Standard Activity Requirements and/or Gold Standard Impact Quantification Methodologies associated with it or as referenced in Gold Standard Product Requirements” (ibid.:6)

In line with this statement, the proposed Project’s type, as renewable energy production out of animal manure, shall be considered as a Renewable Energy Activity. Hence, the document of “GS4GG Renewable Energy Activity Requirements (Version 1)” will be used to define whether the proposed Project activity is automatically eligible for Gold Standard Certification.

The Section 1. (titled as ‘Eligible Project Types & Scopes’) of the GS4GG Renewable Energy Activity Requirements states that

“In order to be eligible for certification, Gold Standard Renewable Energy Projects must meet the following Eligibility and Criteria:

- Project shall generate and deliver energy services (e.g. mechanical work/electricity/heat) from non-fossil and renewable energy sources

- Project shall comprise of renewable energy generation units, such as photovoltaic, tidal/wave, wind, hydro, geothermal, waste to energy and renewable biomass:

- Supplying energy to a national or a regional grid; or

- Supplying energy to an identified consumer facility via national/regional grid through a contractual agreement such as wheeling.” (ibid.:2)

In the case of the proposed Project, it generates and delivers energy services from non-fossil and renewable energy sources, comprises of renewable energy units and supplies electric energy to the national grid. Hence, it meets the Eligibility and Criteria.

As pointed out in the Section A.1. of this PDD, the proposed Project is designed to generate renewable energy, mainly electricity by using animal manure and agricultural waste. The proposed project, as a methane avoidance-based renewable energy activity, also meets the additional eligibility criteria prescribed for its project type, namely, project activity using biogas. Therefore, it is argued that the proposed Project is automatically eligible for Gold Standard Certification.

After reviewing all of the eligibility criteria as per section 3.1.1 of GS4GG Principles & Requirements document, it is argued that the proposed Project is automatically eligible for Gold Standard Certification. It is also important to note that the proposed Project will not benefit from other certification schemes or renewable energy labelling standards. The ways in which the proposed project activity meets eligibility criteria can be summarized as follows:

- The Project is seeking to issue solely GS4GG VER Certification;

- The Project is neither registered nor applied to any other Voluntary Emission Reductions Certification Scheme;

- The Project is a renewable energy installation activity;

- The Project activity includes physical action/implementation on the ground;

- The Project is located in Turkey, which is eligible for VER projects;

- The Project contributes to sustainable development goals;

- The Project is in compliance with GS safeguarding principles;

- Stakeholders are involved in the project implementation and planning during the local consultation meetings and feedback round;

- The Project outcomes are real and will be validated/verified by approved bodies;

- Additionality is demonstrated as per the applicable tools and methodologies.

The project activity meets the eligibility criteria as per section 3.1.1 of GS4GG Principles & Requirements document as described below:

- The project type is power generation using Biogas Energy which is an eligible project type as it is in accordance with 1.1.1 a) and 1.1.1 b) of the Eligible Project Types & Scope under Renewable Energy Activity Requirements.

- The project activity results in displacement of electricity from thermal power plants while contributing to sustainable development of Turkey. Hence, the project contributes to the Gold Standard Vision and Mission.

- Biogas power is an approved project type and does not require approval from Gold Standard.

- This project activity is not associated with geo-engineering or energy generated from fossil fuel or nuclear, fossil fuel switch, nor does it enhances or prolongs such energy generation.

- The project is not registered with any other schemes.

As stated in Section 2.1.3 of the document of Gold Standard for the Global Goals (GS4GG) Principles & Requirements, New Gold Standard Verified Emission Reductions (GS VER) or Gold Standard labels for Certified Emission Reductions (GS CER), Renewable Energy projects connected to national or a regional electricity grid must be located in either a;

a. Least Developed Country (LDC), Small Island Developing State (SIDS) or a Land Locked Developing Country (LLDC) or

b. Low Income and Low Middle-income country where the penetration level of the proposed Renewable Energy Technology type is less than 5% of the total grid installed capacity, at the time of the first submission to preliminary review.

Although the project is located in Turkey which is an upper middle income country, since the proposed biogas project is considered primarily as a waste to energy project, it is eligible and may receive Issuance of Certified Impact Statements or Products for maximum three Certification Renewal Cycles (15 years in total) unless mentioned otherwise in the Product Requirements as informed by Sustain-Cert.

Also, Turkey does not have an emission cap or a regulation on emission trading. If new regulations are issued in the future, adjustments will be made accordingly. Section 3.1.1 (titled as ‘Eligible Project Types’) of the document of Gold Standard for the Global Goals (GS4GG) Principles & Requirements (Version 1.2.) clearly states that

“A Project type is automatically eligible for Gold Standard Certification if there are approved Gold Standard Activity Requirements and/or Gold Standard Impact Quantification Methodologies associated with it or as referenced in Gold Standard Product Requirements” (ibid.:6)

Legal ownership of products generated by the project and legal rights to alter use of resources required to service the project

All legal rights of the Meram Biogas Power Plant were given to Toros Meram Yenilenebilir Enerji Üretim A.Ş. as per the generation license given by EMRA (Energy Market Regulatory Authority) on 04/10/2018.

##### A.2 Location of project

The project is located in 38900 block, plot 4 in Çomaklı Village of Meram District of Konya Province in Turkey. The coordinates are given in table below as per license of the project;

| Point | E | N |
| --- | --- | --- |
| 1 | 454402.24 | 4172864.04 |
| 2 | 454450.22 | 4172864.04 |
| 3 | 454402.24 | 4172852.05 |
| 4 | 454450.22 | 4172852.05 |

Figure 1. Location of the project

##### A.3 Technologies and/or measures

In terms of quantum of waste handled within project activity, Meram Biogas Plant project operates with a yearly organic flow of 245,645 tonne and 47.5 % of this amount is belonged to cattle manure (116,800 tonne/year), 50.9% of this amount is belonged to chicken manure (125,195 tonne/year) and rest of it belonged to agricultural wastes (3,650 tonne/year) as per the information given in EIA report of the project (page 5).

Organic waste mentioned above are fed to anaerobic digestor. The biogas released during the biodegradation of organic wastes will be used for electricity and heat production in cogeneration unit. Installed capacity of the project is 6.004 MWe and 42.028 GWh annual expected electricity generation. Electricity generated is directly fed to the national grid.

Technology applied will involve use of cattle manure, chicken manure and agricultural wastes. “Mesophilic” technology will be applied according to the operating temperature in the planned biogas plant.  The optimal operating temperatures of these bacteria are 37ºC. Manure is mixed and hydrolysed here to break the bonds and generate biogas. Biogas obtained will be fed to the cogeneration unit to generate electricity whereas final product will be stored and used as fertilizer. Electricity generated will be fed to the national grid.

The transport of the collected manure from the farms will be achieved by totally confined trucks. The transported manure will be directly transferred to the raw material pond without any storage time. Collected manure then fed to the fermenters via leakage proof pipes and biogas capture in the fermenters. Captured biogas is sent to the cogeneration units where it is utilized to generate electricity. Generated electricity is fed to the national grid.. Remaining sludge after the biogas extraction is sent to the separator and drier units to produce biologically enriched fertilizer.

The system consists of following units:

- Waste collection unit: Weigh bridge and data control/storage system

- Waste preparation / feed unit: Equalization tank, vegetative waste stock area, central pumping system

- Anaerobic Digestion Unit: Digestors, gas storage unit, desulfurization unit, early warning system, temperature control system

- Cogeneration Unit: Cogeneration, gas treatment system, flare

- Fertilizer Dewatering / Hygenization Unit: Post-digestion tanks, hygenization unit, separator

- Automation system

- Product Storage Area: Liquid fermented storage area, solid fermented storage area

- Hydrogen sulfide (H2S) in biogas will be reduced to 150 ppm and lower by biological internal desulfurization. In addition, the condensate path in the biogas line and condensate collection systems will remove saturated gas from the coarse water through a cooling unit and remove it from the moisture up to the saturation point. In this way, the moisture in the biogas will be reduced to the desired level.

Figure 2. Schematic View of the Process

Technical details of the equipment are presented below,

1. Gas engines

• Brand: Jenbacher

• Model: 4.20

• Electrical Power: 1.501 MWe

• Quantity: 4

2. Digesters

There are 2 main fermenters and 1 post digester in the plant. All fermenters are built from reinforced concrete. The main fermenters are cylindrical and each of them have a section at the top with the volume of ¼ of the total containing the lighter portion of the raw material and thus, preventing gas escape from the fermenter. In the post digestor, besides the section preventing the gas escape, there are 2 more chambers for the liquid and solid phase after the separator sieve. Fermenter volumes are presented below.

R-201: 12.000 m3

R-202: 11.000 m3

R-203: 11.000 m3

R-204: 11.800 m3

R-205: 12.200 m3

Reinforced concrete fermenters are made of C30 class concrete. The inner surfaces of the fermenter walls are waterproof. For the safety belt of the system process of the fermenters, there are serpentines, which are the jacket water of the gas engines, on the outer walls of the fermenters. In addition, the fermenter facades are filled with healthy rockwool panels to minimize their heat. In each fermenter, there are 4 pedal tips in small cycle. In the overflow eyes, there are 1 propeller ends.

3. Feed pump

• Brand: Netzsch

• Model: NM125BY02S12V

• Capacity: 60 m3/hour

• Quantity: 2 (with spare)

4. Electricity meters

main meter

• Brand: Landis

• Model: GYR E550

backup meter

• Brand: Makel

• Model: C 520

5. Weighbridge

• Brand: Tunaylar

• Model: Pi-Bridge

• Capacity: 60 tons

##### A.4 Scale of the project

The CDM Methodology Booklet (tenth edition) states that the Type (iii) project activities “that result in emission reductions of less than or equal to 60 kt CO2 equivalent per year”(pg. 41) shall be considered as “Small-scale” project activities. Since the total estimated emission reduction of the proposed project activity is 110,277 tCO2 equivalent per year, the Project’s scale shall be defined as “Large-scale”.

##### A.5 Funding sources of project

No public funding is used for the project activity.

#### SECTION B. APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

##### B.1. Reference of approved methodology (ies)

The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste (v1.0).

Referred tools

“Tool to calculate project or leakage CO2 emissions from fossil fuel combustion”, Version 03.0

“Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”, Version 03.0

“Project emissions from flaring”, Version 03.0

“Project and leakage emissions from anaerobic digesters”, Version 02.0

“Project and leakage emissions from biomass”, Version 04.0

“Tool to calculate the emission factor for an electricity system”, Version 07.0

“Tool to determine the remaining lifetime of equipment”, Version 01 ‘Project and leakage emissions from road transportation of freight’, Version 01.1.0 Tool to determine the mass flow of a greenhouse gas in a gaseous stream Version 03.0 “Combined tool to identify the baseline scenario and demonstrate additionality”, Version 07.0,

“Emissions from solid waste disposal sites”, Version 08.0

##### B.2. Applicability of methodology (ies)

The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste is chosen for this project.

This consolidated baseline methodology is based on elements from the following methodologies:

- AM0006: “GHG emission reductions from manure management systems”;

- AM0016: “Greenhouse gas mitigation from approved Animal Waste Management Systems in confined animal feeding operations”;

- ACM0022: “Alternative waste treatment processes”;

- AM0073: “GHG emission reductions through multi-site manure collection and treatment in a central plant”.

The justification of the choice of this methodology is presented in the Table below. In addition, the methodology is also applicable to the activities that involve co-digestion and/or co-composting of multiple organic matters that would have otherwise been left to decay anaerobically-.

| The methodology criteria | Applicability justification of the proposed project activity |
| --- | --- |
| This methodology is applicable to manure management on one or multiple livestock farms where the existing anaerobic manure treatment system, within the project boundary, is replaced by one or a combination of more than one animal waste management systems (AWMSs) that result in less GHG emissions compared to the existing system. The project activities where manure is collected by tank trucks, canalized and/or pumped from multiple livestock farms and the collected material is subsequently treated in a single central treatment plant may also claim emission reductions … The methodology is applicable to both Greenfield and existing facilities. | The proposed project activity provided a centralized anaerobic treatment plant which replaced the lagoons of nearby farms to manage animal manure (cattle and poultry manure) by collecting these waste with confined truck to achieve biogas and so methane which is used to gainfully generate electricity, as noted in the Section A.1. of the PDD also. Therefore, this consolidated baseline methodology shall be applied to the proposed project activity. |
| Farms where livestock populations, comprising of cattle, buffalo, swine, sheep, goats, and/or poultry, is managed under confined conditions | The Project’s primary feedstock, cattle-manure, is supplied mainly by farms of members of the Cooperative. The Cooperative, along with its members’ farms, functions under the control of the Ministry of Agriculture and Forestry. The Cooperative’s members manage the livestock population in their farms in accordance with the relevant Turkish laws and regulations confining conditions, especially regarding animal health, such as Law No. 5996, Law No. 3285 and Regulation on Livestock Farms. |
| Manure or the streams obtained after treatment are not discharged into natural water resources (e.g. river or estuaries) & Farms where manure is not discharged into natural water resources (e.g. rivers or estuaries) | As shown in the Section A.3. of the PDD (flow scheme), the waste streams after the treatment will not be discharged into natural water resources. The refined and processed fermented product will be sent to the separator and decanter (dehydration unit) to be separated (please see the Section A.5. of the PDD). Both the liquid and solid parts of the residual waste from the digestion will be further used as organic fertilizers. In the baseline scenario, as noted above, the farms where manure is collected for the Project are managed in accordance with the relevant laws and regulations which prohibit manure to directly discharge into natural water resources. Hence, the proposed project activity complies with this criteria/condition. |
| In case of anaerobic lagoons treatments systems, the depth of the lagoons used for manure management under the baseline scenario should be at least 1 m | In the baseline scenario, cattle manure generated at farms is stored generally in tanks in anaerobic conditions. Tanks’ depths, when applicable, are more than 1 m. Therefore, this criteria/condition is complied by the proposed project activity. |
| The annual average ambient temperature at the site where the anaerobic manure treatment facility in the baseline existed is higher than 5°C | The annual average temperature of baseline site where manure is collected from multiple farms, as members of the Cooperative, is 11.7°C. Hence, the proposed project activity complies with this criteria/condition. |
| In the baseline case, the minimum retention time of manure waste in the anaerobic treatment system is greater than 1 month | The retention time of manure waste is greater than one month in the baseline scenario. Therefore, this criteria/condition is complied by the proposed project activity. |
| The AWMS(s) in the project case results in no leakage of manure waste into ground water, e.g. the lagoon should have a non-permeable layer at the lagoon bottom | No leakage of manure waste into ground water occurs in the project case, since the lagoons have a non-permeable layer at the lagoon bottom. Hence, the proposed project activity complies with this criteria/condition. |
| If residues are stored in between collection activities, storage tanks shall comprise outdoor open equipments | Not applicable. Since residues collected from the farms are immediately fed into the digesters, this criteria/condition is not applicable to the proposed project activity. |
| If the manure/treated residue is used as fertilizer in the baseline, project proponents must ensure that this end use remains the same throughout the project activity | Liquid and solid fertilizers gathered after the anaerobic digestion are provided to all the farmers around who is willing to use. This issue is also covered during the remote site visit under validation progress and local stakeholders clearly stated that the solid and liquid fertilizers provided by this facility (the Project) contributed significantly to the production in their agricultural activities. At this point, the project owner only demands that the fertilizer receiving farms bear the cost of carrying the fertilizer, but does not earn any other financial income in this process. |
| In case residual waste from the digestion is handled aerobically and/or submitted to soil application, the proper conditions and procedures (not resulting in methane emissions) for storage and transportation and soil application must be ensured | Not applicable. Both the liquid and solid parts of the residual waste from the digestion will be further used as organic fertilizers. Residual wastes from biological treatment are separated into liquid and solid phases with separator unit. Then both solid phase and liquid phase are shared with farmers near to project site to be used as fertilizer. As discussed in remote site visit with local stakeholder, these fertilizers provided efficiency in agricultural activities in the region and farmers are satisfied with this situation. |
| In the case of co-digestion, for one or more sources of substrates, it cannot be demonstrated that the organic matter would otherwise have been left to decay anaerobically, baseline emissions related to such organic matter shall be accounted for as zero, whereas project emissions shall be calculated according to the procedures presented in this methodology for all co-digested substrates | Proposed project activity involves co-digestion of multiple source of biomass substrates, mainly cattle manure and plantal waste. Since it cannot be demonstrated that these feedstocks would otherwise been left to decay anaerobically, no emission reduction will be claimed for digestion of these secondary substrates. Under the baseline emission calculations agricultural wastes is not taken into account as per methodological criteria; however, project emissions due to these wastes are taken into consideration for the project such as emissions caused due transportation of these wastes to the project site. Additionally, MSW is not used as co-substrate in this project and as understood from the methodological statements, The project has an option to not use MSW in the operation of the project and so there is no need to apply ACM022 for applicability condition of MSW perspective. |
| CERs shall be claimed by the Central Treatment Plant managing person/entity, only. Other parties involved must sign a legally binding declaration that they will not claim CERs from the improved animal waste treatment practices. The DOE shall check such declaration during the validation (during verification if new parties added after project registration) and these documents shall be valid throughout the whole crediting period | Declaration is shared with DOE declaration which mentions the VERs shall be claimed by ENFAŞ (project owner) only. |

##### B.3. Project boundary

The project boundary includes the physical, geographical sites of:

(a)	The livestock (farms where the cattle and poultry manure gathered);

(b)	Animal manure management system (This project site)

(c)	Facilities which recover and combust or use methane.

As it could be seen in the figure below, project has three main components of emission reduction;

- manure management

- electricity generation

(d)	All power plants connected physically to the electricity system (grid) that the project 	plant is connected to.

e) Farms and facilities where the solid and liquid fraction(slurry) of the organic fertilizers transferred

Figure 3. Project Boundary

Figure 4. Physical diagram of project activity

Manure Supplier Farms with Name, Location and Distance to the Meram Biogas Project Site Which Have Agreements Between Project With Annual Animal Number Detail

| Name of the Farm | Animal Type | Annual Average Number of Animals |
| --- | --- | --- |
| Sibel Mest | Dairy Cattle | 350 |
| İlyas Şeker Hayvancılık ve Nakliyat | Dairy Cattle | 1,300 |
| İsmail Ördek Yemcilik ve Nakliyat | Dairy Cattle | 1,250 |
| Hüsamettin Şekerci Hayvancılık | Dairy Cattle | 450 |
| Mustafa Eroğlu Yemcilik | Dairy Cattle | 250 |
| Total Cattle Number |  | 3,600 |
| İsmail Ördek Yemcilik ve Nakliyat | Poultry | 450,000 |
| Sibel Mest | Poultry | 250,000 |
| Hüsamettin Şekerci Hayvancılık | Poultry | 260,000 |
| Tekkeli Gıda | Poultry | 690,000 |
| Dervet Hayvancılık | Poultry | 40,000 |
| Seltav Tavukçuluk | Poultry | 80,000 |
| Ceren Hayvancılık | Poultry | 63,000 |
| Azim Tavukçuluk | Poultry | 221,000 |
| Total Poultry Number |  | 2,054,000 |

Address Information and Coordinates of Manure Supplier Farms Which Have Agreements Between Project With Annual Animal Number Details

| Name of the Farm | Latitude | Longitude | Distance to project site |
| --- | --- | --- | --- |
| Sibel Mest | 37.805449° | 32.447042° | 13.5 km |
| İlyas Şeker Hayvancılık ve Nakliyat | 37.807214° | 32.446743° | 13.6 km |
| İsmail Ördek Yemcilik ve Nakliyat | 37.811927° | 32.456047° | 14.2 km |
| Hüsamettin Şekerci Hayvancılık | 37.798585° | 32.643548° | 33.6 km |
| Mustafa Eroğlu Yemcilik | 37.798862° | 32.469019° | 15.2 km |
| Tekkeli Gıda | 39.047492° | 33.041289° | 169 km |
| Dervet Hayvancılık | 37.923704° | 32.549826° | 32.6 km |
| Seltav Tavukçuluk | 37.799118° | 32.429682° | 11 km |
| Ceren Hayvancılık | 37.893047° | 32.545137° | 28.8 km |
| Azim Tavukçuluk | 37.796974° | 32.657322° | 34.6 km |
| Yenibahçe Region in Meram District of Konya Province (Agricultural Wastes) | 37.73226260991254 | 32.47484331276563 | 4.6 km |
| Çomaklı Region in Meram District of Konya Province (Agricultural Wastes) | 37.72968323368631 | 32.52640355908915 | 5.6 km |

For the agricultural waste transportation, waste transported from the farmlands located in two region which are Yenibahçe and Çomaklı (Google Map screenshots are given below). These two regions located in Meram District of Konya Province and so close to the project site. Yenibahçe region is 4.6 km away from the project site and Çomaklı region is 5.6 km. While calculation the project emissions due to transportation of waste, all waste amount (245,645 tonne/year) multiplied with the longest distance (where manure taken from) as 169 km in order to make a conservative project emission calculation. Details are present in excel spreadsheet.

Figure. Yenibahçe Region in Meram District of Konya Province (Agricultural Wastes taken from Yenibahçe Region)

Figure. Çomaklı Region in Meram District of Konya Province (Agricultural Wastes taken from Çomaklı Region)

The furthest farm to the project site is as it could be checked from the coordinates given above. To be on the safe side the longest distance used for the project emissions due to transportation of all wastes. Details could be checked via excel document of the project. As it could be checked the furthest points that waste gathered to project site is Kulu District and the distance to the project site is 169 km (to be on the safe side taken as 169 km in excel calc. doc. Also please check the figure below). Moreover, the distances of each farm to project site are given in table above. To be conservative the farthest point of each district is chosen on map to the project site for each farm where located in.

| Source |  | GHGs | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | Source 1: Emissions from waste treatment process | CO2 | No | CO2 emissions from the decomposition of organic waste are not accounted. |
|  |  | CH4 | Yes | The major source of emissions in the baseline. |
|  |  | N2O | Yes | Direct and indirect N2O emissions are accounted. |
|  | Source 2: Emissions from electricity generation | CO2 | Yes | CO2 emissions from the electricity generation is accounted since project use renewable energy source instead fossil fuel source. |
|  |  | CH4 | No | Excluded for simplification. This is conservative. |
|  |  | N2O | No | Excluded for simplification. This is conservative. |
| Project scenario | Source 1: Emissions from waste treatment process | CO2 | No | CO2 emissions from the decomposition of organic waste are not accounted. |
|  |  | CH4 | Yes | The emission from anaerobic digesters and aerobic treatment |
|  |  | N2O | Yes | . Direct and indirect N2O emissions are accounted. |
|  | Source 2: Emissions from on-site electricity use | CO2 | Yes | These emissions are counted for, since the Project will consume electricity from the National Grid. |
|  |  | CH4 | No | Excluded for simplification. This emission source is assumed to be very small. |
|  |  | N2O | No | Excluded for simplification. This emission source is assumed to be very small. |
|  | Source 3: Emissions from flare unit | CO2 | Yes | Project emission from flaring of the residual gas stream taken into account as per related methodological calculation pathway |
|  |  | CH4 | Yes | Project emission from flaring of the residual gas stream taken into account as per related methodological calculation pathway |
|  |  | N2O | No | Excluded for simplification. This emission source is assumed to be very small. |
|  | Source 4: Emissions from manure/waste residue transportation | CO2 | Yes | These emissions are counted due to related Tool (transported waste amounts, distances and default average fuel consumption data used) |
|  |  | CH4 | No | Excluded for simplification. This emission source is assumed to be very small. |
|  |  | N2O | No | Excluded for simplification. This emission source is assumed to be very small. |

##### B.4. Establishment and description of baseline scenario

In the baseline scenario, manure (both chicken and cattle) from the farms were being left to decay in anaerobic lagoons. Also, the agricultural waste was burnt before the project activity.

In the absence of the project activity, the waste materials used as energy sources would have been disposed but not been utilised and the energy demand would be met using fossil fuels.

Baseline scenario for managing the manure

Identify the baseline scenario and demonstrate additionality using the “Combined tool to identify the baseline scenario and demonstrate additionality”, following the requirements below

(ii) For Greenfield facilities

Baseline scenario for manure management was detail explained in following part “(ii) for greenfield facility”, but still in addition to that, the same reference document is also mentioned here for the baseline scenario as “uncovered manure management”. In the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 4, Chapter 10, Table 10.17), uncovered anaerobic lagoon system and related data to use for baseline consideration and calculations are given in table below. By looking at this table, MFC value of the baseline situation was taken as 70% corresponded to 11.7 Celcius temperature value which is average yearly temperature of the project location.

By implementing anaerobic digesters manure will be digested whilst capturing and utilizing GHGs from them, the proposed project activity will reduce the release to air of methane sourced from uncovered anaerobic animal manure management systems in livestock farms within the region and minimize the adverse impacts of manure on human health and the environment.

The consolidated baseline methodology prescribes the baseline scenario for Greenfield facilities as the situation where, in the absence of the project activity, animal manure is left to decay in uncovered anaerobic lagoons within the project boundary and methane is emitted to the atmosphere.

The Methodology states that, where the baseline scenario is an uncovered anaerobic lagoon, the baseline scenario for managing the manure for Greenfield facilities can be defined by applying the following two steps:

- “Define several anaerobic lagoon design options for the particular manure stream that meet the relevant regulations and take into consideration local conditions (e.g. environmental legislation, ground water table, land requirement, temperature). Design specifications shall include average depth and surface area of the anaerobic lagoon, residence time of the organic matter, as well as any other key parameters. Document the different design options in a transparent manner and provide transparent and documented evidence of key assumptions and data used, and offer conservative interpretations of this evidence;

- Carry out an economic assessment of the identified lagoon design option, as per step 3 (investment analysis) of the latest approved version of the “Combined tool to identify the baseline scenario and demonstrate additionality” and additional guidance given below. Choose the least cost anaerobic lagoon design option from the options identified through step (a) above. If several options with comparably low cost exist, choose the one with the lowest lagoon depth as the baseline lagoon design” (pg. 4).

The common practice for the livestock farm owners is to have uncovered anaerobic lagoons/ponds at their farms in Turkey. In terms of the residence time of the manure at the lagoons, it is possible to say that since the uncovered lagoons are also fed by rain waters, the lagoons are reaching their full capacities faster than usual in rainy seasons. Nevertheless, the minimum retention time of manure waste in the uncovered anaerobic lagoons is greater than one and half and/or two months through the implementation of the proposed project activity, the manure collected at these lagoons/ponds will be collected daily via special manure trucks and fed into the digesters at the biogas plants.

For step a, proposed project activity meets relevant regulations and take into consideration local conditions since it already has gathered EIA approval/permissions. Moreover, pre-project situation is defined above which has negative impact on human health and environment.

For step b, investment analysis is revised due to capacity extension and given in following parts.

According to Turkish Electricity Transmission Corporation (TEİAŞ) statistics, the fossil-fuel based electricity generation share in total electricity generation in Turkey is over 56.5% by 2019.

| ANNUAL DEVELOPMENT OF RENEWABLE ELECTRICITY GENERATION SHARE IN TURKEY TOTAL ELECTRICITY GENERATION (2019) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| YEAR | HYDRO (MW) | GEOTERMAL (MW) | WIND (MW) | SOLAR (MW) | BİOMASS (MW) | RENEWABLE INSTALLED CAPACITY (MW) | TOTAL INSTALLED CAPACITIY (MW) |
| 2019 | 88,822.8 | 8,951.7 | 21,730.7 | 9,249.8 | 3.522,7 | 88.822,8 | 88.550,8 |
| % in Renewable Installed Capacity | 67% | 7% | 16% | 7% | 3% | 100% | - |
| % in Total Installed Capacity | 29% | 3% | 7% | 3% | 1% | 43.5% | 100% |

Moreover, Turkey’s energy demand is rapidly increasing in line mainly with the increases in its population and its economy. TEİAŞ’s estimations on the demand of gross electricity consumption in Turkey between 2019 and 2028 can be seen in the Chart below.

Moreover, TEİAŞ, in its report on “The Projections on Capacity Generation”, analyses the estimations on changes in electricity generation capacities in Turkey between 2018 and 2022, based on two main scenarios, ‘optimistic’ and ‘pesimistic’. The distribution of total installed capacity of the Turkish national grid by primary energy resources in 2022 both in optimistic and pessimistic scenarios can be seen on the two charts below, respectively.

As seen in these two charts, although it is estimated that the share of renewable energy sources in the total installed capacity of the Turkish national grid would reach almost 50% by 2022, the share of biogas and biomass energy sources would remain the same, only 1%, combined. That means that the share of biogas- and biomass-based renewable energy sources in the Turkish national grid is estimated to show no changes from 2017 to 2022, despite their potential pointed out by the ‘Bioenergy and Food Security (BEFS) Assessment for Turkey: Sustainable bioenergy options from crop and livestock residues’ published by Food and Agriculture Organization of the United Nations (FAO) and European Bank for Reconstruction and Development (EBRD) in 2016.

Hence, it is reasonable to claim that it is estimated that whilst the share of biogas- and biomass-based renewable energy sources in the total installed capacity of the Turkish national grid could remain the same as only 1%, combined, by 2022, Turkey’s electricity grid would continue to be dominated by fossil fuel burning power plants which are seen as the quickest solutions in short term to meet the demand and to enable energy security in terms of supply.

##### B.5. Demonstration of additionality

Demonstration of additionality is carried as per the methodological tool: “Combined tool to identify the baseline scenario and demonstrate additionality”, Version 07.0, as given below.

Step 1 - Identification of alternatives to the project activity consistent with current laws and regulations

Sub-step 1a - Define alternatives to the project activity:

The most realistic and reliable alternatives to the project activity are:

- Proposed project  is not undertaken as a VER project activity

- Continuation of the current situation-supply of equal amount of electricity by the newly built grid connected power plants

The first alternative, which is the implementation of the project without carbon revenue is not financially attractive as discussed in investment analysis section below. The Second alternative (Scenario 2) is the baseline scenario and implementation of the proposed project as a VER activity would be additional to this scenario. Continuation of the current situation is not considered as a realistic alternative due to increasing electricity demand therefore new power plants should be constructed which includes mainly thermal power plants. Implementation of the project is additional to the baseline scenario which is alternative 2 above and therefore reduces the emissions.

If it is necessary to follow the steps of the relevant methodology;

Baseline scenario for MSW disposal

To identify baseline alternatives for the treatment of the fresh waste, the following alternatives or combinations of these alternatives shall, inter alia, be considered:

M1: The project activity without being registered as a CDM project activity (i.e. any (combination) of the waste treatment options; Composting, Co-composting or anaerobic digestion);

M2: Disposal of the fresh waste in a SWDS with a partial capture of the LFG and flaring of the captured LFG;

M3: Disposal of the fresh waste in a SWDS without a LFG capture system;

M4: Part of the fresh fraction of the solid waste is recycled and not disposed in the SWDS;

M5: Part of the fresh fraction of the solid waste is treated aerobically and not disposed in the SWDS;

M6: Part of the organic fraction of the solid waste is incinerated and not disposed in the SWDS;

M7: Part of the organic fraction of the solid waste is gasified and not disposed in the SWDS;

M8: Part of the organic fraction of the solid waste is treated in an anaerobic digester and not disposed in the SWDS;

M9: Part of the organic fraction of the solid waste is mechanically or thermally treated to produce RDF/SB and not disposed in the SWDS.

In used methodology it is indicated that:

“This methodology is applicable to manure management on one or multiple livestock farms where the existing anaerobic manure treatment system, within the project boundary, is replaced by one or a combination of more than one animal waste management systems (AWMSs) that result in less GHG emissions compared to the existing system.…. In addition, the methodology is also applicable to the activities that involve co-digestion and/or co-composting of multiple organic matters that would have otherwise been left to decay anaerobically in an animal waste management system (AWMS) or a solid waste disposal site (SWDS).

- “In case of co-digestion, for one or more sources of substrates, it cannot be demonstrated that the organic matter would otherwise have been left to decay anaerobically, baseline emissions related to such organic matter shall be accounted for as zero, whereas project emissions shall be calculated according to the procedures presented in this methodology for all co-digested substrates”

Hence, to return our project, MSW is not used in this project as indicated before and as understood from the methodological statements, we have an option (in case of…) to not use MSW in the operation of the project and so there is no need to apply ACM022 for applicability condition of MSW perspective.

Since the project does not use municipal solid waste (MSW), the alternatives given above is not related with this project.

Baseline scenario for electricity and heat generation

In addition to the alternative baseline scenarios identified for managing the manure, alternative scenarios for the use of gas generated from an anaerobic digester (biogas) shall also be identified if this is an aspect of the project activity:

For electricity generation, alternative(s) shall include, inter alia:

E1: Electricity generation from biogas, undertaken without being registered as CDM project activity;

E2: Electricity generation in existing or new renewable based captive power plant(s);

E3: Electricity generation in existing and/or new grid-connected power plant;

E4: Electricity generation in an off-grid fossil fuel fired captive power plant;

E5: Electricity generation in existing and/or new grid-connected power plant and fossil fuel fired

captive power plant(s).

Methodology indicates that baseline emissions due to electricity generation can be accounted for only if the baseline scenario is E3, E4 and E5.

In terms of this project baseline scenario E3 is valid since project provides electricity generation via biogas production from manure management instead of electricity generated from fossil fuels which supplied to national grid.

For heat generation, alternative(s) shall include, inter alia:

H1: Heat generation from biogas undertaken without being registered as CDM project activity;

H2: Heat generation in existing or new fossil fuel fired cogeneration plant(s);

H3: Heat generation in existing or new renewable based cogeneration plant(s);

H4: Heat generation in existing or new on-site or off-site fossil fuel based boiler(s) or air heater(s);

H5: Heat generation in existing or new on-site or off-site renewable energy based boiler(s) or air

heater(s);

H6: Any other source, such as district heat; and

H7: Other heat generation technologies (e.g. heat pumps or solar energy).

Since the project does not have heat generation unit, the alternatives given above is not related with this project.

For manure managements, alternative(s) shall include, inter alia (taken from IPCC Volume 4, Chapter 10, Table 10.17):

-Pasture/Range/Paddock

-Daily Spread

-Solid Storage

-Dry Lot

-Liquid/Slurry with or without natural crust cover

-Uncovered anaerobic lagoon

-Pit storage below animal confinements

-Anaerobic digester

-Burned for fuel

-Cattle or Swine Deep Bedding

-Composting in vessel/static pile/intensive windrow/passive windrow

-Poultry manure with or without litter

-Aerobic treatment

Methodology states that, for Greenfield facilities, the methodology only applies where the baseline scenario selected from the complete set of the list of the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 4, Chapter 10, Table 10.17), is an uncovered anaerobic lagoon. For the baseline scenario of this project farmers of the region stored animal manure at uncovered anaerobic lagoons. Related situation observed on site visit and also site photographic evidence was shared on that issue. Hence this project could apply the referenced methodology.

Sub-Step 1a : Define alternative scenarios to the proposed CDM project activity

Step 1a: Define alternatives to the project activity

The alternatives to the project activity are defined as per of the applied additionality tool:

a)	The tool suggests taking into account the implementation of the Project without being registered as a CDM Project activity.

S1: The proposed project activity undertaken without being registered as a CDM project activity;

b)	As per paragraph 20-b of the applied tool, other realistic and credible scenarios which deliver comparable output services should be considered.

S2: Where applicable, no investment is undertaken by the project participants but third party(ies) undertake(s) investments or actions which provide the same output to users of the project activity, for example

- In the case of a Greenfield power project, an alternative scenario may be that the project participants would not invest in another power plant but that power would be generated in existing and/or new power plants in the electricity grid. Since there is no common practice for profitable commercial activity in waste management, only comparable parameter is the building and operation of a power plant for energy generation.

c)	Continuation of the current situation (no project activity or other alternatives undertaken)

S3: Where applicable, the continuation of the current situation, not requiring any investment or expenses to maintain the current situation, such as, inter alia:

- The continued venting of methane from lagoons

d) Other plausible scenarios

S4: Where applicable, the continuation of the current situation, requiring an investment or expenses to maintain the current situation.

S5: Other plausible and credible alternative scenarios to the project activity scenario, including the common practices in the relevant sector, which deliver the same output, taking into account, where relevant, examples of scenarios identified in the underlying methodology.

- The common practice in the sector is the collection of waste in anaerobic lagoons, which leads to higher methane emissions and fails to deliver the same outputs.

S6: Where applicable, the “proposed project activity undertaken without being registered as a CDM project activity” to be implemented at a later point in time (e.g. due to existing regulations, end-of-life of existing equipment, financing aspects)

Scenarios 4, 5 and 6 are not applicable as there is no requirement for investment for continuation of the current situation or no reason expected for change in regulations etc. S1, which is the implementation of the project without carbon revenue is not financially attractive as discussed in  investment  analysis section below. S2 is not also realistic as the energy demand is increasing and the need for new power plant investment is a must for Turkey. The second and third scenarios partially and jointly form the baseline scenario and implementation of the proposed project as a VER activity would be additional to this scenario in terms of reducing methane venting and meeting increasing energy demand.

Sub-step 1b.	 Consistency with mandatory laws and regulation

The following applicable mandatory laws and regulations have been identified:

- Electricity Market Law

- Law on Utilization of Renewable Energy Resources for the Purpose of Generating Electricity Energy

- Energy Efficiency Law

- Forest Law

- Environment Law

- Waste Management Regulation

The resultant alternatives to the project as outlined in Step (1a) are in compliance with the applicable laws and regulations.

Outcome of Step 1b

Mandatory legislation and regulations for each alternative are taken into account in sub-step 1b. Based on the above analysis, the proposed project activity is not the only alternative amongst the project participants that is in compliance with mandatory regulations. Therefore, the proposed VER project activity is considered as additional.

Investment analysis

The investment analysis has been done in order to make an economic and financial evaluation of the project. No public funding or ODA are available in Turkey for finance of this type of projects. For investment analysis, loan conditions have been determined considering the average market rates/term sheets signed with the banks. In this document, investment analysis follows the recommendations of “Am-tool 27,v.10.0: Investment analysis”

Sub-step 2a - Determine appropriate analysis method

There are three options for the determination of analysis method which are:

- Simple Cost Analysis

- Investment Comparison Analysis and

- Benchmark Analysis

Since Project generates economic benefits from sales of electricity, the simple cost analysis is not applicable. Also, since the baseline of the project is generation of electricity by the grid, no alternative investment is considered at issue. So, it has been decided to use benchmark analysis for evaluation of the project investment.

Sub-step 2b: Option III. Apply benchmark analysis

For benchmark analysis, figure defined by World Bank for similar project types have been used which has been given as 20% in the document of the world bank ” published in June 2017 (page 40 of document). For the proposed project, in order to reach this equity IRR (after taxes) values, average electricity tariff must be above 13.5 $c/kWh in the absence of carbon revenue and assuming that initial investment figures are realized so that the investment will become reasonable.

Sub-step 2c. 	Calculation and comparison of financial indicators

| Parameters | Unit | Data Value |  |
| --- | --- | --- | --- |
| Installed Capacity | MWe | 6.004 | Project License Document |
| Electricity Generation | GWh/year | 42.028 | Project License Document |
| Capital Investment | 1000 $ | ~23,800 | Project EPC Contract/Agreement |
| O&M Cost (including annual rental fee of land) | 1000 $ | 1,329 | Project Owner Operational Data |
| Corporate tax rate | % | 20 | Articles 52, 68 and 72 of the relevant law. Related link is given below. https://www.mevzuat.gov.tr/mevzuatmetin/1.4.193.pdf |
| Average Sales Price | $ Cents/kWh | 13.3 | Related Regulation for The Renewable Energy Power Plants. Related link is given below. https://www.mevzuat.gov.tr/mevzuatmetin/1.5.5346.pdf |

Table. Main financial parameters used for investment analysis

| OPERATION COSTS (Financial Feasibility dated 04/09/2019) | (USDk) |
| --- | --- |
| TOTAL | 1,329 |
| Personel & ADM Cost (USDk) |  | 302 |  |
| Raw Material Transportation (USDk) | 360 |
| Electricity (USDk) | 210 |
| Consumables (USDk) | 25 |
| Maintanence (USDk) | 225 |
| Insurance (USDk) | 85.000 |
| System Usage Expense (USDk) | 59.126 |
| Other (USDk) |  | 63.277 |  |
| INVESTMENT COSTS | (USDk) |
| EPC Contract Value (01/12/2019) | 23,800 |

Table. Input values used for investment analysis

Internal Rate of Return (IRR) of the proposed project has been calculated as 15.61% based on the parameters given above without considering the carbon revenue. Project does not use any ODA or government incentive. Electricity tariff has been used as $13.3. Annual generation has been taken as 42.028 GWh as indicated in generation licence. Although detailed analysis show that actual generation will be lower.

Sub-step 2d - Sensitivity Analysis

Sensitivity analysis had been carried out for three main parameters identified for the first phase of the project. However, since the investment cost for extension is based on signed agreements and operating cost is not changed after extension, which is a conservative approach, only impact of change in tariff has been included in sensitivity analysis.

- Investment Cost

- Operating Cost

- Electricity Sales revenue

| For a range of ± 15% fluctuations in parameters above, table below has been obtained. % Fluctuation. | -15 | -10 | -5 | 0 | +5 | +10 | +15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Investment Cost | 19.56 | 18.07 | 16.77 | 15.61% | 14.56 | 13.62 | 12.77 |
| Operating Cost | 16.29 | 16.06 | 15.83 | 15.61% | 15.38 | 15.14 | 14.91 |
| Electricity Income | 12.29 | 13.41 | 14.51 | 15.61% | 16.69 | 17.77 | 18.85 |

Table. Sensitivity analysis for Meram Biogas Power Plant Project (without carbon revenue)

Outcome of Step 2:

The investment and sensitivity analysis shows that the VER revenues will improve the financial indicators of the Project remarkably. Considering that figures above are based on a higher price rather than the government guaranteed floor price, optimistic estimations for yearly generation and that those figures do not reflect the risk for investment, role of carbon income is a most significant number to enable the project to proceed.

Another important parameter affecting equity IRR is investment cost. However, since the agreements have been made and costs are realized as given in financial model, there is no chance to expect a decrease in the investment cost. Operating costs can also affect the equity IRR however, its impact is not significant and does not cause any significant change in equity IRR and the fluctuation percentage to reach the benchmark is very high and not likely. Based on the above information, it is seen that project is not the most attractive option. Therefore, project is considered as additional to the baseline scenario.

Step 3. Barrier Analysis

The proposed project does not use barrier analysis as the additionality has been demonstrated in other steps.

Step 4. Common Practice Analysis

According to the “Tool for the demonstration and assessment of additionality”, Version 07.0.0”, the common practice (Tool 24 - Common Practice version 03.1 has been followed) shall provide an analysis of any other activities that are similar to the Project Activity. Projects are considered similar if they are in the same country/region and/or rely on a broadly similar technology, are of a similar scale, and take place in a comparable environment with respect to regulatory framework, investment climate, access to technology, access to financing etc.

This project started commercial operation in 2020 due to its provisional acceptance protocol sign date (27/11/2020). Applicable geographical area has been selected as the whole host country (Turkey) as per paragraph 1 of Guidelines on Common Practice version 03.1. Projects which apply the same measure as the proposed project have been determined and biogas projects are selected as the same energy source type of projects. All of the selected plants deliver the same service which is the electricity generation. Applicable output range has been determined and all of the power plants are taken from the latest available year 2022. General Directorate of Energy Affairs have been used as a main resource (2022 Nihai YEK Listesi). Therefore, all of the compared power plants have been operational before the implementation of the project activity.

| No | Name of plant |
| --- | --- |
| 1 | RESPECT BİYOGAZ |
| 2 | Sakarya Düzenli Depolama Depo Gazı Tesisi |
| 3 | TAŞKÖPRÜ BİYOKÜTLE ENERJİ SANTRALI |
| 4 | Düzce Biyokütle Enerji Santralı |
| 5 | Samsun Avdan Biyogaz Tesisi |
| 6 | Kumrular Biyogaz Santralı |
| 7 | Ahi Suluova Biyogaz Tesisleri |
| 8 | Tire Biyogaz Elektrik Santrali |
| 9 | Toros Gönen Biyogaz Tesisi |
| 10 | Uşak Biogaz Elektrik Üretim Tesisi |

Stepwise Approach For Common Practice

Step 1: Calculate applicable output range as +/-50% of the design output or capacity of the proposed project activity.

The design capacity of the proposed project is 6.004 MWe (as per generation license). Therefore, the applicable output range is from 3.002 MWe – 9,006 MWe.

Step 2: identify similar projects (both CDM and non-CDM) which fulfil all of the following

conditions:

(a) The projects are located in the applicable geographical area;

(b) The projects apply the same measure as the proposed project activity;

(c) The projects use the same energy source/fuel and feedstock as the proposed project activity, if a technology switch measure is implemented by the proposed project activity;

(d) The plants in which the projects are implemented produce goods or services with comparable quality, properties and applications areas (e.g. clinker) as the proposed project plant;

(e) The capacity or output of the projects is within the applicable capacity or output range calculated in Step 1;

(f) The projects started commercial operation before the project design document (CDM-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity

The plants listed below meets the conditions of “a”, “b”, “d” and “e”. All of them in Turkey (2020 investments, within the applicable range of output/capacity and producing goods within same measure.

##### In terms of conditions “c” 5 of the 10 companies listed above has the same technology with Meram biogas projest since they also use manure for biogas and electricity generation. Moreover, all these 5 projects listed below comply with condition “f”.

| No | Name of plant | Capacity(MWe) |
| --- | --- | --- |
| 1 | RESPECT BİYOGAZ | 4.6 |
| 2 | Kumrular Biyogaz Santralı | 4.268 |
| 3 | Ahi Suluova Biyogaz Tesisleri | 6.637 |
| 4 | Tire Biyogaz Elektrik Santrali | 4.80 |
| 5 | Toros Gönen Biyogaz Tesisi | 3.62 |

Step 3: within the projects identified in Step 2, identify those that are neither registered CDM project activities, project activities submitted for registration, nor project activities undergoing validation. Note their number Nall.

Among the projects listed in Step 2, only Respect Biyogas has no registration/certification or validation process under VER mechanisms (references are present in common practice excel document of this project).

Therefore, Nall=1

Step 4: within similar projects identified in Step 3, identify those that apply technologies that are different to the technology applied in the proposed project activity. Note their number Ndiff.

The listed biogas power plant applies the same technology (references of technologies are present in common practice excel document of this project).

Therefore, Ndiff = 0

Step 5: calculate factor F=1-Ndiff/Nall representing the share of similar projects (penetration rate of the measure/technology) using a measure/technology similar to the measure/technology used in the proposed project activity that deliver the same output or capacity as the proposed project activity.

F=1-Ndiff/Nall=1-(0) = 1         (< 0.2)

Nall-Ndiff= 1 – 0 = 1                   ( ≤ 3)

According to the guidelines on common practice version 03.1, if the factor F is greater than 0.2 and Nall-Ndiff is greater than 3, then the proposed project is a “common practice”.

For the proposed project, F=1 (greater than 0.2) but Nall-Ndiff=1 (smaller than 3), therefore, the proposed project is not common practice within the region. Hence, the proposed project is additional.

B.5.1 Prior Consideration

N/A. The project is not retroactive and design change reviews were already finalized.

B.5.2 Ongoing Financial Need

N/A

##### B.6. Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| Sustainable Development Goals Targeted | Most relevant SDG Target | SDG Impact |
| --- | --- | --- |
|  |  | Indicator (Proposed or SDG Indicator) |
| 13 Climate Action (mandatory) | 13.3 Improve education, awareness-raising and human and institutional capacity on climate change mitigation, adaptation, impact reduction and early warning | 13.3.2 Number of countries that have communicated the strengthening of institutional, systemic and individual capacity-building to implement adaptation, mitigation and technology transfer, and development actions |
| 8 Decent Work and Economic | 8.5 By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value 8.8 Protect labour rights and promote safe and secure working environments for all workers, including migrant workers, in particular women migrants, and those in precarious employment | 8.5.1 Average hourly earnings of female and male employees, by occupation, age and persons with disabilities 8.8.1 Frequency rates of fatal and non-fatal occupational injuries, by sex and migrant status 8.8.2 Increase in national compliance of labour rights (freedom of association and collective bargaining) based on International Labour Organization (ILO) textual sources and national legislation, by sex and migrant status |
| 7 Affordable and Clean Energy | By 2030, increase substantially the share of renewable energy in the global energy mix | 7.2.1 Renewable energy share in the total final energy consumption |
| 12 Affordable and Clean Energy | By 2020, achieve the environmentally sound management of chemicals and all wastes throughout their life cycle, in accordance with agreed international frameworks, and significantly reduce their release to air, water and soil in order to minimize their adverse impacts on human health and the environment | Waste treated via anaerobic digestion such as cattle and poultry manure |

B.6.1 Explanation of methodological choices/approaches for estimating the SDG Impact

The proposed Project is designed to generate renewable energy, mainly electricity by using animal manure and agricultural waste. The proposed project, as a methane avoidance-based renewable energy activity, also meets the additional eligibility criteria prescribed for its project type, namely, project activity using biogas. Therefore, “The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reduction from Manure Management Systems and Municipal Solid Waste” used since the methodology guides the baseline/project emission calculations for this kind of projects. The estimated outputs of the project related with SDGs with approaches are given as follows.

SDG 7: Affordable and Clean Energy

Relevant Target of SDG 7

7.2 By 2030, increase substantially the share of renewable energy in the global energy mix

Relevant Indicator of SDG 7

7.2.1 Renewable energy share in the total final energy consumption

The baseline for the project is no project, thus leading to generation in the relevant grid which is dominated by fossil fuel and this is one of the methodological reason for choosing simple OM method to calculate emission factor. The clean energy generated by the project is calculated based on the amount of electricity generated by the project per annum. The project is expected to generate 42.028 GWh of clean energy per annum and contributes to share of low-cost / must-run sources. Hence, contribution of the project could be followed via indicator “Renewable energy share in the total final energy consumption” and following target: ‘’By 2030, increase substantially the share of renewable energy in the global energy mix”. This project increases the renewable energy sharing of global energy mix and contribute to improved air quality by reducing air pollution.

Project’s contribution to SDG 7 is to be calculated with the annual electricity generation of the project multiplying with the referenced emission factor. Details are present in following sections. Related monitoring parameter is present in Section B.7.1

SDG 8: Decent Work and Economic Growth (Promote sustained, inclusive and sustainable economic growth, full and productive employment and decent work for all):

Relevant Target of SDG 8

8.5 By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value

8.8 Protect labour rights and promote safe and secure working environments for all workers, including migrant workers, in particular women migrants, and those in precarious employment Relevant Indicators of SDG 8.

8.5.1 Average hourly earnings of female and male employees, by occupation, age and persons with disabilities

The project promotes job opportunities. Social security documents of employees are the certain prove of that issue.

8.8.1 Frequency rates of fatal and non-fatal occupational injuries, by sex and migrant status

8.8.2 Increase in national compliance of labour rights (freedom of association and collective bargaining) based on International Labour Organization (ILO) textual sources and national legislation, by sex and migrant status

Training (including H&S) & Other Certification processes required by certain necessary professions will be  provided to employees to protect human health & rights and develop. Hence, project contributes awareness of labour rights and safety precautions. Trainings of employees are done periodically as per national regulations and certifications are kept for the prove of the situation.

“Increase in national compliance of labour rights (freedom of association and collective bargaining) based on International Labour Organization (ILO) textual sources and national legislation, by sex and migrant status”:

Project’s contribution to SDG 8 is to be monitored with the social security documents and training documents/certificates of the employees. Related monitoring parameter is present in Section B.7.1

SDG 12: Ensure sustainable consumption and production patterns.

Relevant Target of SDG 12

SDG Target 12.4: “By 2020, achieve the environmentally sound management of chemicals and all wastes throughout their life cycle, in accordance with agreed international frameworks, and significantly reduce their release to air, water and soil in order to minimize their adverse impacts on human health and the environment”.

Relevant Indicator of SDG 12

Proposed indicator: Waste treated via anaerobic digestion such as cattle and poultry manure

By implementing anaerobic digesters animal manure (cattle and chicken manure) and agricultural waste is digested whilst capturing and utilizing GHGs from them, the proposed project activity will reduce the release to air of methane sourced from uncovered solid waste management systems and minimize the adverse impacts on human health and the environment. Indicator 12.4.1 may imply increase of waste management in line with international multilateral environmental agreements in which Turkey has already been one of the parties to the international conventions.

The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste). Project’s contribution to SDG 12 is to be monitored with the amount of waste accepted to the facility and recording the amount information. Related monitoring parameter is present in Section B.7.1

SDG13: Climate Action (Taking urgent action to combat climate change and its impacts):

Relevant Target of SDG 13

13.3 Improve education, awareness-raising and human and institutional capacity on climate change mitigation, adaptation, impact reduction and early warning

Relevant Indicator of SDG 13

13.3.2 Number of countries that have communicated the strengthening of institutional, systemic and individual capacity-building to implement adaptation, mitigation and technology transfer, and development actions

The project leads to mitigation of   110,277  tCO2 per annum. It is also expected to reduce SO2 and NOx emissions by 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC) . The project contributes to the target of SDG 13 which could be expressed via following indicator:

“Improve education, awareness-raising and human and institutional capacity on climate change mitigation, adaptation, impact reduction and early warning”: The project’s contribution is done through training and awareness raising of local people and setting good example by investing to the climate friendly technology.

Project’s contribution to SDG 13 is to be monitored with the baseline and project emission calculations which are explained in following sections in detail. Related monitoring parameter is also present in Section B.7.1

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 Climate Action (mandatory) | ## The project contributes to CO2e reduction, which represent direct and quantifiable impact on climate security. Moreover, the project is also expected to reduce SO2 and NOx emissions. | ## The project contributes to 110,277 tonnes of CO2e reduction annually, which represent direct and quantifiable impact on climate security. Also, the project is expected to reduce 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC) . |  |
| ## 7 Affordable and Clean Energy | ## Annual average estimated renewable electricity generation | ## 42.028 GWh |  |
| ## 8 Decent Work and Economic Growth | ## Project provides job opportunities. | ## Employee will gather H&S and other related trainings. Around 55 people will be employed. |  |
| ## 12 Ensure Sustainable Consumption and Production Patterns | ## The project provides better management option for wastes ( cattle and poultry manure) via anaerobic treatment | ## The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste) |  |

B.6.2 Data and parameters fixed ex ante

Copy the table for each piece of data and parameter; use headings to group parameter tables by SDG

SDG13

| Data/parameter | EFgrid |
| --- | --- |
| Unit | ton CO2/MWh |
| Description | Grid emission factor |
| Source of data | Country specific data |
| Value(s) applied | 0.5552 |
| Choice of data or Measurement methods and procedures |  |
| Purpose of data | Used in emission reduction calculations |
| Additional comment | Latest official emission factor of Turkey used in the projects depending on the project type published by the Ministry of Energy and Natural Resources. As per this reference; OM is 0.7258 and BM is 0.4153 tCO2/MWh for Turkey National Grid. During CM calculation as per related tool, these values were taken into considedration. Details are present in “CM” tab of project excel ER calculation file. |

| Data/parameter | VSLT |
| --- | --- |
| Unit | kg dm/animal/year |
| Description | Density of CH4 |
| Source of data | IPCC 2006 vol.4, chapter 10, Table10A-4(Dairy Cows eastern Europe) (4.5 kg/hd/day) |
| Value(s) applied | 1,642.5 (for cattle manure) |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | Used in baseline calculations. |
| Additional comment | For the calculation of VSLT,y Option 4 is chosen and default and regional IPCC data is used. |

| Data/parameter | VSLT |
| --- | --- |
| Unit | kg dm/animal/year |
| Description | Density of CH4 |
| Source of data | IPCC 2006 vol.4, chapter 10, Table10A-9(Chicken-layer) (0.02 kg/hd/day) |
| Value(s) applied | 7.30 (for chicken manure) |
| Choice of data or Measurement methods and procedures |  |
| Purpose of data | Used in baseline calculations. |
| Additional comment | For the calculation of VSLT,y Option 4 is chosen and default and regional IPCC data is used. |

| Data/parameter | DCH4 |
| --- | --- |
| Unit | t/m3 |
| Description | Density of CH4 |
| Source of data | The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste, pg. 25. |
| Value(s) applied | 0.00067 |
| Choice of data or Measurement methods and procedures | 0.00067 t/m3 at room temperature (20 °C) and 1 atm pressure |
| Purpose of data | Used in project emission/baseline calculations. |
| Additional comment | - |

| Data/parameter | EFCH4,default |
| --- | --- |
| Unit | Fraction |
| Description | Default emission factor for the fraction of CH4 produced that leaks from the anaerobic digester. |
| Source of data | Project and leakage emissions from anaerobic digesters”, Version 02.0, pg. 11-12 |
| Value(s) applied | 0.028 |
| Choice of data or Measurement methods and procedures | Digesters with steel or lined concrete or fiberglass digesters and a gas holding system and monolithic construction. |
| Purpose of data | Used in project emission calculation. |
| Additional comment | - |

| Data/parameter | GWPCH4 |
| --- | --- |
| Unit | t CO2e/t CH4 |
| Description | Global Warming Potential for CH4 |
| Source of data | IPCC |
| Value(s) applied | 28 |
| Choice of data or Measurement methods and procedures | Default value from IPCC is used as per the applied methodology. |
| Purpose of data | Used in project emission/baseline calculations. |
| Additional comment | - |

| Data/parameter | - fCH4,default |
| --- | --- |
| Unit | - m3 CH4 / m3 |
| Description | - Default value for the fraction of methane in the biogas (m3 CH4 / m3 - biogas) |
| Source of data | Project and leakage emissions from anaerobic digesters https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf |
| Value(s) applied | - 0.6 |
| Measurement methods (if any) | - Default value |
| Purpose of data | - To calculate project and leakage emissions |
| Additional comment | - fCH4,default |

| Data/parameter | - ηflare,m |
| --- | --- |
| Unit | - % |
| Description | - Flare efficiency |
| Source of data | - Tool to determine project emissions from flaring gases containing methane |
| Value(s) applied | - %50 |
| Choice of data or Measurement methods and procedures | - Used in project emission calculations |
| Purpose of data | - Default value for open flare provided that it can be demonstrated that the flare is operational. |
| Additional comment |  |

| Data/parameter | EF_SO2 |
| --- | --- |
| Unit | Kg/MWh |
| Description | SO2 emission factor |
| Source of data | Turkey 2022 National Inventory Submission to UNFCCC (2020 data covered) and TEIAS National 2020 Statistics |
| Value(s) applied | 4.85 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | Contribution to air quality calculations |
| Additional comment | - |

| Data/parameter | EF_NOx |
| --- | --- |
| Unit | Kg/MWh |
| Description | NOx emission factor |
| Source of data | Turkey 2022 National Inventory Submission to UNFCCC (2020 data covered) and TEIAS National 2020 Statistics |
| Value(s) applied | 1.05 |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | Contribution to air quality calculations |
| Additional comment | - |

B.6.3 Ex ante estimation of SDG Impact

1 – Affordable and Clean Energy (SDG 7):

The project is expected to generate 42.028 GWh of clean energy per annum and contributes to share of low-cost / must-run sources.

2 – Decent Work and Economic Growth (SDG 8):

The project promotes job opportunities. Social security documents of employees are the certain prove of that issue.

Training (including H&S) & Other Certification processes required by certain necessary professions will be  provided to employees to protect human health rights and develop. Hence, project contributes awareness of labour rights and safety precautions. Trainings of employees are done periodically as per national regulations and certifications are kept for the prove of the situation. Around 55 people will be employed.

All employees will attend trainings on first aid and health & safety. For positions that require specific skills (such as high voltage equipment) staff will either be trained or certified staff will be recruited. Training details can be found in the monitoring plan.

3-  Ensure sustainable consumption and production patterns (SDG 12)

The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste)

4 – Climate Action (SDG 13):

As per the tool, the latest official emission factor of Turkey that is used in the projects depending on the project type published by the Ministry of Energy and Natural Resources The ex-ante emission reductions (ERy) are calculated as follows:

The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste (pg. 21) states that the emission reduction ERy by the project activity during a given year y is the difference between the baseline emissions (BEy) and the sum of project emissions (PEy) and leakage, as follows:

ERy=BEy-PEy-LEy

Where:

ERy		= Emission reductions in year y (tCO2/yr)

BEy		= Baseline emissions in year y (tCO2/yr)

PEy		= Project emissions in year y (tCO2/yr)

LEy		= Leakage emissions in year y (tCO2/yr)

Accordingly,

ERy		=118,762 tCO2/yr – 8,485 tCO2/yr

ERy		= 110,277tCO2/yr

In detail,

As per applied methodology;

The emission reductions achieved in any year are the lowest value of the following:

ERy,ex post = min [(BEy,ex post – PEy,ex post – LEy,ex post), (MDy – PE,y,ex post– LEy,ex post)] (Equation 34 of applied methodology)

Where:

ERy,ex post = Emission reductions achieved by the project activity based on monitored values in year y (tCO2/a)

BEy,ex post = Baseline emissions calculated using equations provided by ACM0010 and ACM0022 with ex post monitored values in year y (tCO2/a)

PEy,ex post = Project emissions calculated using equations provided with ex post monitored values in year y (tCO2/a)

LEy,ex post = Leakage emissions calculated with ex post monitored values in year y (tCO2/a)

MDy = Methane captured and destroyed or used gainfully by the project activity in year y (tCO2e/a)

This project does not used methane for any other purpose apart from electricity generation via gas motors. Therefore; Equation 34 could be simplified for this project as follows

ERy,ex post = min [(BEy,ex post – PEy,ex post – LEy,ex post)

In detail:

Baseline Emissions from Animal Waste Treatment

As stated and justified earlier in the PDD, the Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste is applied to the proposed project activity. The consolidated baseline methodology calculates baseline emissions as follows:

BEManure=BECH4,y+BEN2O,y+BEelec/heat,y

(Equation 2 from The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013 )

Where:

BEmanure,y  = Baseline emissions in year y (tCO2/yr)

BECH4,y  = Baseline CH4 emissions in year y (tCO2/yr)

BEN2O,y  = Baseline N2O emissions in year y (tCO2/yr)

BEelec/heat,y  = Baseline CO2 emissions from electricity and/or heat used in the baseline (tCO2/yr)

NOTE: EN2O,D,y and EN2O,ID,y emissions equations for the baseline emissions (BEN2O,y page 11 of The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013) and project emissions (PEN2O,y page 15 of The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013) calculation equations are given same in the Gold Standard’s consolidated baseline methodology. Same situation is present in CDM Methodology ACM0010 (ver.08) that consolidated GS Methodology is based on. This would led emission calculations as zero since baseline and project emissions become equal within this calculation pathway.

Throught note stated above BEManure,y     becomes:

BEManure, y = BECH4, y + BEelec / heat, y

Baseline CH4 emissions from manure treatment (BECH4,y):

The manure management system in the baseline could be based on different livestock, treatment systems and on one or more stages. Therefore:

(Equation 3, from The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013 )

Where:

|  | = | Baseline CH4 emissions (tCO2/yr) |
| --- | --- | --- |
|  | = | Global Warming Potential (GWP) of CH4 (tCO2e/tCH4) |
|  | = | Density of CH4 (t/m3) |
|  | = | Annual methane conversion factor (MCF) for the baseline AWMSj |
|  | = | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg -dm) |
|  | = | Annual average number of animals of type LT for the year y (number) |
|  | = | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
|  | = | Fraction of manure handled in system j in the baseline |
| LT | = | Type of livestock |
| j | = | Type of treatment system |

For the calculation of NLT,y Option 2 is chosen from applied methodology. Project has signed agreements with annual average live animal number with farms. Hence these data used for NLT,y calculation. Details of animal number are given in Section B.3 and emission reduction calculation excel document.

For the calculation of VSLT,y Option 4 is chosen and default and regional IPCC data is used.

As a result of explained calculation pathway (details given in ER calculation sheet):

BECH4,y =  95,428 tonnes CO2/year

Baseline emissions associated with electricity generation (BEEC,y)

- Baseline emissions associated with electricity generation (BEEC,y)

- The baseline emissions associated with electricity generation in year y (BEEC,y) shall be calculated using the “Tool to calculate baseline, project and/or leakage emissions from electricity consumption”. When applying the tool:

- Where:

|  | = Baseline emissions associated with electricity generation in year y (t CO2) |
| --- | --- |
|  | = Quantity of net electricity supplied to the grid as a result of the implementation of the CDM project activity in year y (MWh) |
|  | = CO2 emission factor of the grid in year y (t CO2/MWh) |

- BEEG =  42,028 MWh x 0.5552 tons CO2/MWh

- 23,334 tCO2 e  (details are present in ER calculation sheet of the project)

Biogas production from agricultural wastes in digesters and subtracting emissions from electricity generation related to the mentioned biogas production from baseline emissions

Baseline emissions have two contributors as known. The biggest contributor is baseline methane emissions, and this emission category is calculated by using animal numbers. Hence agricultural wastes do not cause a calculation error or more here as it is not one of the parameters.

The second contributor is electricity generated and supplied to national grid. To subtract the biogas used for electricity generation which sourced by agricultural waste the pathway could be followed described below step-by-step

- In section B.7.1 the parameter “Vf” describes the total biogas produced in anaerobic digesters and directed to gas engines (this biogas sourced mostly by animal manure and a limited part is belonged to agricultural wastes). This data will be provided by project owner according to records of biogas meter on site.

- On the other hand, animal manure-based biogas and corresponding methane is already present in BECH4 calculation. When BECH4 value divided to DCH4 parameter (which has a constant value), the “m3” amount of biogas sourced by only animal manure could be known.

- Hence, the biogas belonged to animal manure can be subtracted from the monitoring value of the parameter “Vf” to represent the biogas and corresponding electricity generation belonged to agricultural wastes during the monitoring periods of the project.

- Baseline emissions associated with heat generation (BEHG,y)

The baseline emissions associated with heat generation are not considered within the scope of this project. The project does not seek emission reduction credits from the heat component.

- Total of Baseline Emissions

- BEy=BECH4,y+BEelec/heat,y

- BEManure=95,428+23,334=118,762 tonnes CO2/year

Project Emissions

The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste states that project emissions in year y are calculated for alternative waste treatment option implemented in the project activity as follows:

PEy = PEAD,y + PEAer,y + PEComp,y + PEN2O,y + PEEC/FC,y + PETran,y + PEStorage,y

Where:

PEy 		= Project emission in year y (t CO2)

PEAD,y	= Project emissions associated with the anaerobic digester / co-digestion in year t (tCO2e/yr)

PEAer,y 	= Project CH4 emissions from aerobic AWMS treatment (tCO2e/yr)

PEComp,y 	= Project CH4 emissions from composting/ co-composting (tCO2e/yr)

PEN2O,y	= Project N2O emissions in year y (tCO2/yr)

PEEC/FC,y	= Project emissions from electricity consumption and fossil fuel combustion (tCO2/yr)

PETran,y	= Project emissions from manure transportation in the year y (tCO2/yr)

PEStorage, y	= Project emissions from manure storage (tCO2e/yr)

Since the proposed project activity does not consist of aerobic AWMS treatment, PEAer,y shall be accounted as zero. In addition, for the Project does not involve composting, PEComp,y shall be accounted as zero too. Moreover, the proposed project activity does not consume fossil fuel for heat purposes, since the heat power generated as a by-product is used within the Project site. Thus, PEFC,y shall be accounted as zero. Furthermore, since the manure is not stored in outdoor open storage tanks more than 24 hours, PEStorage,y shall be accounted as zero. Moreover, since EN2O,D,y and EN2O,ID,y emissions equations for the baseline emissions (BEN2O,y page 11 of The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013) and project emissions (PEN2O,y page 15 of The Gold Standard Revised Consolidated Baseline Methodology for  GHG Emission Reductions from  Manure Management Systems and Municipal Solid Waste, December 2013) calculation equations are given same in the Gold Standard’s consolidated baseline methodology, this would led emission calculations net emissions based on N2O as zero as also explained in baseline emissions calculation part of this document. Hence, the project emission equation could be simplified as;

PEy = PEAD,y + PEEC,y + PETran,y

Project emissions associated with the anaerobic digester in year y (PEAD,y)

The consolidated baseline methodology states that PEAD,y is determined using the methodological tool ‘Project and leakage emissions from anaerobic digesters’ (pg. 13). As per this, the methodological tool, Version 02.0, as its latest approved version, shall apply to the proposed project activity.

The paragraph 13 of the Tool determines PEAD,y as follows:

PEAD,y = PEEC,y + PEFC,y + PECH4,y + PEflare,y

Where:

PEAD,y 	= Project emissions associated with the anaerobic digester in year y (tCO2e)

PEEC,y	= Project emissions from electricity consumption associated with the anaerobic digester in year y (tCO2e)

PEFC,y	= Project emissions from fossil fuel consumption associated with the anaerobic digester in year y (tCO2e)

PECH4,y	= Project emissions of methane from the anaerobic digester in year y (tCO2e)

PEflare,y	= Project emissions from flaring of biogas in year y (tCO2e)

Although, energy requirement for the plant will be supplied from national grid, possible emissions caused by the generator could be calculated and added to the project emission in related year when it is needed. There are two ways to calculate PE,FC,y which are Option A and Option B stated in “Tool to calculate project or leakage CO2 emissions from fossil fuel combustion version 03. Option A should be the preferred approach, if the necessary data is available. In addition, the emissions are negligible when the emission reductions from the Diesel Generator are lower than 1% of the total emission reductions; otherwise, the emissions will be calculated. But for now, since the anaerobic digester facility does not use fossil fuels, PEFC,y shall be accounted as zero for estimated calculations.

Moreover, In this equation, PEEC,y shall be accounted as zero since project emissions from electricity consumption associated with the anaerobic digester is going to be accounted separately. For the proposed project activity will consume electricity from the national grid for internal consumption of electricity at the plant, including the digesters, PEEC,y in this equation shall be regarded as zero since the calculation of project emissions from electricity consumption and fossil fuel combustion (PEEC/FC,y) that will be made below is going to consist of PEEC,y. This action is taken by the project developer in order to avoid double-counting.

Moreover, since the anaerobic digester facility does not use fossil fuels, PEFC,y shall be accounted as zero.

In addition, since the flare deviceis equipped for emergency situations only, PEflare,y shall also be accounted as zero.

Accordingly, project emissions associated with the anaerobic digester in year y (PEAD,y) is equivalent to project emissions of methane from the anaerobic digester in year y (PECH4,y) as follows:

PEAD,y =  PECH4,y

As per the paragraph 23 of the Tool, PECH4,y is calculates as follows:

PECH4,y=QCH4,y×EFCH4,default×GWPCH4

Where:

PECH4,y	= Project emissions of methane from the anaerobic digester in year y (tCO2e)

QCH4,y		= Quantity of methane produced in the anaerobic digester in year y (tCH4)

EFCH4,default	= Default emission factor for the fraction of CH4 produced that leaks from the anaerobic digester (fraction)

GWPCH4	= Global warming potential of CH4 (28 tCO2/tCH4) (IPCC Fifth Assessment Report)

PECH4,y		= 570 tonnes CO2/year (details are present in ER calculation sheet of the Project)

Hence, PEAD,y = 570 tonnes CO2/year

Project emissions from use of electricity (PEelec,y)

Project emissions from use of electricity (PEelec,y)

In accordance with the Gold Standard’s consolidated baseline methodology,

the project emissions from electricity consumption will be calculated following the latest version of ‘ Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation ver.03.0’ (pg. 17). Accordingly, the methodological tool ‘Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity’, Version 03.0, is applied to the proposed project activity to calculate Peelec,y.

PEEC,y=jECPJ,j,y×EFEF,j,y×(1+TDLj,y)

Where:

PEEC,y		= Project emissions from electricity consumption in year y (tCO2/yr)

ECPJ,j,y  	= Quantity of electricity consumed by the project electricity consumption source j in year y (MWh/yr) (42,028 MWh/year as assumed %10 of yearly electricity generation)

EFEF,j,y		= Emission factor for electricity generation for source j in year y (tCO2/MWh) (Combined Margin = 0.5552 tCO2/MWh)

TDLj,y	= Average technical transmission and distribution losses for providing electricity to source j in year y

Accordingly,

PEEC,y = 2,560 tCO2/year (details are present in ER calculation sheet of the Project)

Accordingly, The Total of Project Emissions

PEy		= PEAD,y + PEEC/FC,y

PEy	= 570 tCO2/yr + 2,560 tCO2/yr = 3,130 tonnes CO2/year

Project emissions from waste transportation (PETran,y)

The Gold Standard consolidated baseline methodology states that the project emissions from manure transportation from collection points to the central treatment plant shall be calculated according to the Methodological tool “Project and leakage emissions from transportation of freight” Accordingly the methodological tool ‘Project and leakage emissions from road transportation of freight’, Version 01.1.0, is applied to the proposed project activity.

As per the paragraph 20 of the Tool, PETran,y,

For this project, average return distance between the origin and the destination is taken as 169 km which is the distance to farthest farm to project site. By taking this distance the most conservative approach is taken for the emission due to transportation of wastes. Annual waste amount taken by project is 245,645 tonne/year according to finalized EIA (Total waste amount consisting of cattle manure, chicken manure and agricultural wastes. Breakdown amounts of wastes given in Section A1 and A3 and Table 1 above).

Default CO2 emission factor for freight transportation is chosen as 129 g CO2/t.km

Hence;

PETran,y = 5,355 tCO2/year (details are given in ER calculation sheet)

Accordingly, The Total of Project Emissions

PEy		= PEAD,y + PEEC/FC,y + PETran,y

PEy	= 570 tCO2/yr + 2,560 tCO2/yr + 5,355 tCO2/yr  = 8,485  tonnes CO2/year

Leakage Emissions

The Gold Standard Revised Consolidated Baseline Methodology for GHG Emission Reductions from Manure Management Systems and Municipal Solid Waste calculates leakage emissions, as follows

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y+LEComp,y+LEAD,y+LETrans,y

Where:

LEPJ,N2O,y	= Leakage N2O emissions released during project activity from land application of the treated manure in year y (tCO2e/yr)

LEBL,N2O,y	= Leakage N2O emissions released during baseline scenario from land application of the treated manure in year y (tCO2e/yr)

LEPJ,CH4,y	= Leakage CH4 emissions released during project activity from land application of the treated manure in year y (tCO2e/yr)

LEBL,CH4,y	= Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (tCO2e/yr)

LEComp,y	= Leakage emissions associated with the storage and disposal of compost in year y (tCO2e)

LEAD,y	= Leakage emissions associated with the anaerobic digester in year y (tCO2e)

LETrans,y	= Emissions from incremental distance travelled for waste/final compost/residue transportation in tCO2e/yr

As noted earlier, since the proposed project activity does not involve composting, LEComp,y shall be accounted as zero. Moreover, the solid and liquid digestate used as fertilizer in nearby agricultural land as free and this materials are not used/stored in any other conditions on land. Hence, LEPJ,N2O,y, LEPJ,CH4,y shall be zero. Before the project activity farm owners bought the inorganic fertilizer from related market which is costly.

In addition, as per the statement made by the Gold Standard consolidated baseline methodology as “leakage covers the emissions from land application of treated manure as well as the emissions related to anaerobic digestion in a digester, occurring outside the project boundary” (pg. 18), leakage emissions associated with the anaerobic digester (LEAD,y) and emissions from incremental distance travelled for waste/final compost/residue transportation (LETrans,y) shall also be accounted as zero since these emissions are occurring within the project boundary and already accounted as project emissions.

Accordingly, net leakage shall be negative. As per the statement made by the Gold Standard consolidated baseline methodology as “net leakage is only considered if they are positive” (pg. 18), LEy shall be accounted as zero.

LEy		= 0 tCO2/yr

B.6.4 Summary of ex ante estimates of each SDG Impact

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| 01/01/2021- 31/12/2021 | 118,762 | 8,485 | 110,277 |
| 2022 | 118,762 | 8,485 | 110,277 |
| 2023 | 118,762 | 8,485 | 110,277 |
| 2024 | 118,762 | 8,485 | 110,277 |
| 01/01/2025 – 31/12/2025 | 118,762 | 8,485 | 110,277 |
| Total | 593,810 | 42,425 | 551,385 |

| Total number of crediting years | 5 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 118,762 | 8,485 | 110,277 |

1 – Affordable and Clean Energy (SDG 7):

The project is expected to generate 42.028 GWh of clean energy per annum and contributes to share of low-cost / must-run sources.

2 – Decent Work and Economic Growth (SDG 8):

The project promotes job opportunities for 55 employees. Social security documents of employees are the certain prove of that issue.

Training  (including H&S) & Other Certification processes required by certain necessary professions will be  provided to employees to protect human health and rights and develop. Hence, project contributes awareness of labour rights and safety precautions. Trainings of employees are done periodically as per national regulations and certifications are kept for the prove of the situation.

3-  Ensure sustainable consumption and production patterns (SDG 12)

The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste)

4-  Take urgent action to combat climate change and its impacts (SDG 13)

The project contributes to 110,277 tonnes of CO2e reduction, which represent direct and quantifiable impact on climate security. Moreover, the project is also expected to reduce SO2 and NOx emissions by 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC) .

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 Climate Action (mandatory) | ## The project contributes to CO2e reduction, which represent direct and quantifiable impact on climate security. Moreover, the project is also expected to reduce SO2 and NOx emissions. | ## The project contributes to 110,277 tonnes of CO2e reduction annually, which represent direct and quantifiable impact on climate security. Also, the project is expected to reduce 204.0 tons of SO2 and 43.9 tons of NOx emissions (based on 2020 national gross electricity generation statistics and 2020 national greenhouse gas emission inventory submission to UNFCCC) . |  |
| ## 7 Affordable and Clean Energy | ## Annual average estimated renewable electricity generation | ## 42.028 GWh |  |
| ## 8 Decent Work and Economic Growth | ## Project provides job opportunities. | ## Employee will gather H&S and other related trainings. Around 55 people will be employed. |  |
| ## 12 Ensure Sustainable Consumption and Production Patterns | ## The project provides better management option for wastes ( cattle and poultry manure) via anaerobic treatment | ## The project provides better management option for wastes as around 245,645 tonne/year (125,195 tonne/year poultry manure and 116,800 cattle manure and 3,650 tonne/year agricultural waste) |  |

##### B.7. Monitoring plan

B.7.1 Data and parameters to be monitored

SDG 13 and SGD 7

| Data / Parameter | EGd,y |
| --- | --- |
| Unit | MWh |
| Description | Electricity generated using biogas in year y |
| Source of data | Project proponent |
| Value(s) applied | 42,028 |
| Measurement procedures (if any): | Archive electronically during project plus five years |
| Monitoring frequency | Annual |
| QA/QC procedures | Two calibrated meters backup each other. Maintenance and calibration of the metering devices are made by TEIAS. If there is a significant difference between the readings of two devices, maintenance and tests of the metering devices and the associated equipment are done before waiting for the periodical maintenance. The meters should comply with EPDK regulations which define the accuracy class of the meters as 0.2 or 0.5 depending on the capacity of the circuit as given in document in link (http://www.epdk.gov.tr/web/elektrik-piyasasi-dairesi/44). Generation of the plant is cross checked from TEIAS – EPIAS web site which is accessible using a password provided to electricity generation companies. EPIAS records will be taken in consideration while calculating EGfacility, y. Calibration information of the meters will be shared. |
| Additional comment | Estimated annual generation forming the basis for emission reduction calculation is 42,028 MWh as indicated in electricity generation licence. |

SDG 13

| Data / Parameter | B0,LT for Poultry manure |
| --- | --- |
| Unit | m3CH4/kg_dm |
| Description | Maximum methane producing potential of the volatile solid generated by animal type LT |
| Source of data | IPCC 2019 Refinement, Chapter 10, Volume 4 Table 10.16, Eastern Europe Data is taken |
| Value(s) applied | 0.39 |
| Measurement procedures (if any): | - |
| Monitoring frequency | Annually. |
| QA/QC procedures | - |
| Any comment | The value is taken from published sources. The parameter value should be updated on latest available public data source |

| Data / Parameter | B0,LT for Dairy Cow Manure |
| --- | --- |
| Unit | m3CH4/kg_dm |
| Description | Maximum methane producing potential of the volatile solid generated by animal type LT |
| Source of data | IPCC 2019 Refinement, Chapter 10, Volume 4 Table 10.16, Eastern Europe Data is taken |
| Value(s) applied | 0.24 |
| Measurement procedures (if any): | - |
| Monitoring frequency | The value is taken from published sources. The parameter value should be updated on latest available public data source |
| QA/QC procedures | - |
| Any comment | The value is taken from published sources. |

| Data / Parameter | T |
| --- | --- |
| Unit | Celcius |
| Description | Annual Average ambient temperature at project site |
| Source of data | Turkish State Meteorological Service. The value “11.7 Celcius” is taken for this project estimated emission reduction calculations. |
| Value(s) applied | 11.7 |
| Measurement procedures (if any): | - |
| Monitoring frequency | Annually. |
| QA/QC procedures | - |
| Any comment | The value I taken from published official governmental sources |

| Data / Parameter | MCFj |
| --- | --- |
| Unit | N/A |
| Description | Methane conversion factor |
| Source of data | 2019 IPCC Refinement, Table 10.17, p. 10.74 refered 70% ( the uncovered anaerobic lagoon for 11.7 oC, Konya Province conditions |
| Value(s) applied | 0.70 |
| Measurement procedures (if any): | Uncovered anaerobic lagoon, Value for 11.7 oC. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Any comment | Baseline emission calculations |

| Data / Parameter | Air Quality |
| --- | --- |
| Unit | tons |
| Description | Reduction of SO2 and NOx emissions due to implementation of project activity that would otherwise be emitted by thermal power plants |
| Source of data | Project proponent. |
| Value(s) applied | Annual expected SO2 emission reduction is 204 tonnes Annual expected NOx emission reduction is 43.9 tonnes |
| Measurement procedures (if any): | - |
| Monitoring frequency | Annually. |
| QA/QC procedures | - |
| Any comment | Total SO2 emission related to electricity generation is about 1488.80 Gg for 2020 according to National Inventory of Turkey. Considering that electricity generation in 2020 is 306,703.1 GWh, SO2 emission per MWh is calculated as 4.85 kg/MWh. Considering the annual expected electricity generation of this project as stated in licence (42.028 GWh) annual expected SO2 emission reduction is 204 tonnes. Total NOx emission related to electricity generation is about 320.68 Gg for 2020 according to National Inventory of Turkey. Considering that electricity generation in 2020 is 306,703.1 GWh NOx emission per MWh is calculated as 1.05 kg/MWh. Considering the annual expected electricity generation of this project as stated in licence (42.028 GWh) annual expected NOx emission reduction is 43.9 tonnes. |

| Data / Parameter | Vf |
| --- | --- |
| Unit | m3 |
| Description | Biogas (total biogas goes to gas engines) |
| Source of data | Project proponent. |
| Value(s) Applied | Project owner will provide during monitoring |
| Measurement procedures (if any): |  |
| Monitoring frequency | Continuously by flow meter and reported cumulatively on weekly basis |
| QA/QC procedures | The system should be built and operated to ensure that there is no air ingress into the biogas pipeline. Data will be archived electronically during the crediting period plus 2 years. Computerized monitoring for the whole system is available in order to keep track of pressure and temperature of the gas, gas meters will be calibrated as per the relevant industry standard. Equipment will be subject to regular maintenance. After the first calibration of meters , calibration of meters are valid for ten years due to related legislation. Link of related regulation given below. “ÖLÇÜ VE ÖLÇÜ ALETLERİ MUAYENE YÖNETMELİĞİ” / Article 9 https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5 Accuracy class of meters is class C (0.5) |
| Any comment | Flow meters located at the exit of the digesters. Calibrated flow meters will be used during operation phase. For this stage, the estimated value could be given by regarding waste handled for anaerobic digesters. 8.657.937 m3/year (site records) biogas value is that estimated value stated in plant capacity report. |

| Data / Parameter | FRG,m |
| --- | --- |
| Unit | m3 |
| Description | the flow rate of the residual gas to the flare |
| Source of data | Project owner |
| Value(s) Applied | Project owner will provide during monitoring if the flare unit is used |
| Measurement procedures (if any): | Calibrated meter of flare equipment |
| Monitoring frequency | Continuously |
| QA/QC procedures | After the first calibration of meters , calibration of meters are valid for ten years due to related legislation. Link of related regulation given below. “ÖLÇÜ VE ÖLÇÜ ALETLERİ MUAYENE YÖNETMELİĞİ” / Article 9 https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5 Calibration document is to be provided in first monitoring period. |
| Any comment | Option A is selected via Methodological tool “Project emissions from flaring Version 03.0” |

| Data / Parameter | Status of biogas destruction device |
| --- | --- |
| Unit | - |
| Description | Operational status of biogas destruction devices |
| Source of data | Project owner |
| Value(s) Applied | - |
| Measurement procedures (if any): | Monitoring and documenting is to be undertaken by recording the energy production from methane captured by gas engines on project site |
| Monitoring frequency | Continuous |
| QA/QC procedures | - |
| Any comment | Project owner is to be report if there is any operational period the gas engines is not under operation for biogas/methane destruction for electricity generation. As per, Methodological tool Tool to determine the mass flow of a greenhouse gas in a gaseous stream Version 03.0 |

| Data / Parameter | Vi, t, db |
| --- | --- |
| Unit | m³ gas i/m³ dry gas |
| Description | Volumetric fraction of greenhouse gas i in a time interval t on a dry basis |
| Source of data | Project and leakage emissions from anaerobic digesters https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf |
| Value(s) Applied | 60% |
| Measurement procedures (if any): | - |
| Monitoring frequency | Continuously |
| QA/QC procedures | After the first calibration of meters , calibration of meters are valid for ten years due to related legislation. Link of related regulation given below. “ÖLÇÜ VE ÖLÇÜ ALETLERİ MUAYENE YÖNETMELİĞİ” / Article 9 https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5 calibration document is to be provided in first monitoring period. |
| Any comment | Details are present in “baseline emission” tab of ER excel calculation document |

| Data / Parameter | SPECflare |
| --- | --- |
| Unit | Temperature - °C Flow rate or heat flux - kg/h or m3 /h Maintenance schedule - number of days |
| Description | Manufacturer’s flare specifications for temperature, flow rate and maintenance schedule |
| Source of data | Flare manufacturer |
| Value(s) Applied | Project proponent will provide during monitoring if the flare unit is used |
| Values to be applied | Document in the CDM-PDD the flare specifications set by the manufacturer for the correct operation of the flare for the following parameters: (a) Minimum and maximum inlet flow rate, if necessary converted to flow rate at reference conditions or heat flux; (b) Minimum and maximum operating temperature; and (c) Maximum duration in days between maintenance events |
| Monitoring frequency | Continuously by flow meter and reported cumulatively on weekly basis |
| Any comment | Option A is selected via Methodological tool Project emissions from flaring Version 03.0 |

SDG 8

| Data / Parameter | Quality of Employment |
| --- | --- |
| Unit | Number of personnel certified/trained during operation phase |
| Description | Contribution to quality of employment by ensuring that the staff is trained and certified for the required positions |
| Source of data | Training Records (including H&S) & Other Certificates required by certain professions, if necessary |
| Value(s) applied | - |
| Measurement methods and procedures | All employees will attend trainings on first aid and health & safety. For positions that require specific skills (such as high voltage equipment) staff will either be trained or certified staff will be recruited. |
| Monitoring frequency | Annually |
| QA/QC procedures | The training programmes help increase the efficiency of the workforce and provides employees skilled at their job. This not only helps the company but to self-improvement of individual employees. |
| Purpose of data | To monitor the contribution to SDG 8 and Principle 3.6.1. |
| Additional comment | - |

| Data / Parameter | T |
| --- | --- |
| Unit | Celcius |
| Description | Annual Average ambient temperature at project site |
| Source of data | Turkish State Meteorological Service |
| Value(s) applied | 11.7 |
| Measurement procedures (if any): | - |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Any comment | - |

SDG 12 and Principle 4.3.5  Hazardous and Non-Hazardous Waste

| Data/parameter | - FRf,m |
| --- | --- |
| Unit | - tonnes |
| Description | - Total mass of freight transported in freight transportation activity f in monitoring period m |
| Source of data | - Project owner |
| Value(s) Applied | - 241,995 tonne/year as total. Cattle manure (116,800 tonne/year) and chicken manure (125,195 tonne/year) as |
| Measurement procedures (if any): | - Calibrated weightbridge |
| Monitoring frequency | - Continuously |
| QA/QC procedures | After the first calibration of meters , calibration of meters are valid for ten years due to related legislation. Link of related regulation given below. “ÖLÇÜ VE ÖLÇÜ ALETLERİ MUAYENE YÖNETMELİĞİ” / Article 9 https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5 calibration document is to be provided in first monitoring period. |
| Any comment |  |

| Data/parameter | - FRf_agricultural waste |
| --- | --- |
| Unit | - tonnes |
| Description | - Total mass of freight transported in freight transportation activity f in monitoring period m |
| Source of data | - Project owner |
| Value(s) Applied | - Agricultural wastes 3,650 tonne/year |
| Measurement procedures (if any): | - Calibrated weightbridge |
| Monitoring frequency | - Continuously |
| QA/QC procedures | After the first calibration of meters , calibration of meters are valid for ten years due to related legislation. Link of related regulation given below. “ÖLÇÜ VE ÖLÇÜ ALETLERİ MUAYENE YÖNETMELİĞİ” / Article 9 https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5 calibration document is to be provided in first monitoring period. |
| Any comment |  |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Project owner (signed agreements/declaration of animal farms) |
| Value(s) applied | 3,600 (cattle) |
| Measurement methods and procedures | Manual numbering of animals |
| Monitoring frequency | daily |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Project owner (signed agreements/declaration of animal farms) |
| Value(s) applied | 2,054,000 (poultry) |
| Measurement methods and procedures | Manual numbering of animals |
| Monitoring frequency | daily |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Project owner (signed agreements/declaration of animal farms) |
| Value(s) applied | 2,054,000 (poultry) |
| Measurement methods and procedures | Manual numbering of animals |
| Monitoring frequency | daily |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Project owner (signed agreements/declaration of animal farms) |
| Value(s) applied | 3,600 (cattle) |
| Measurement methods and procedures | Manual numbering of animals |
| Monitoring frequency | daily |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data / Parameter | ndy |
| --- | --- |
| Unit | number |
| Description | Number of days treatment plant was operational in year y |
| Source of data | Project owner |
| Value(s) applied | 365 |
| Measurement methods and procedures | - |
| Monitoring frequency | daily |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data / Parameter | MS%j |
| --- | --- |
| Unit | % |
| Description | Fraction of manure handled in system j in the project activity |
| Source of data | Project proponent. %100 is taken for estimated emission reduction calculations |
| Value(s) applied | 100% |
| Measurement methods and procedures | - |
| Monitoring frequency | Annual |
| QA/QC procedures | - |
| Purpose of data | To calculate baseline emissions |
| Additional comment | - |

| Data/parameter | - N |
| --- | --- |
| Unit | - - |
| Description | - Total number of farms |
| Source of data | - Project proponent |
| Value(s) Applied | - 13 (5 cattle farm and 8 chicken farm) |
| Measurement procedures (if any): | - - |
| Monitoring frequency | - Annually |
| QA/QC procedures | - - |
| Any comment | - - |

Principle 9.5  Hazardous and Non-Hazardous Waste

| Data/parameter | - Qdm |
| --- | --- |
| Unit | - kg |
| Description | Mass of manure (manure taken from animal farms to fed-up the anaerobic digesters) disposed outside project boundary |
| Source of data | - Project proponent. |
| Measurement procedures (if any): | - - |
| Monitoring frequency | - Annually |
| QA/QC procedures | - - |
| Any comment | - No manure disposal to outside the project boundary. |

| Data / Parameter | Other Pollutants |
| --- | --- |
| Unit | - |
| Description | Proper management of waste oil |
| Source of data | Assessing disposal methods during site visits and checking waste oil disposal records. |
| Value(s) applied | Amount of waste oil generated and disposed during operation |
| Measurement methods and procedures | Waste oil from equipment will be collected properly in line with the relevant regulation and disposed via accredited abatement companies. |
| Monitoring frequency | Annually |
| QA/QC procedures | Waste oil will be disposed in line with regulation # 26952 on control of waste oils . |
| Purpose of data |  |
| Additional comment | - |

B.7.2 Sampling plan

N/A

B.7.3 Other elements of monitoring plan

Net electricity generation is being measured and recorded by both TEIAS and project owners for billing purposes for monitoring emission reduction. Plant Manager, is responsible for the electricity generated, gathering all relevant data and keeping the records. Calibration of the electricity metering devices are made by TEIAS and sealed before the commissioning of the power plant. As per the local regulations, meters should not require calibration at least ten years. The meters will be calibrated by TEIAS when there is an inconsistency between two devices.

Manure transported to project site arrived at the point of waste equalization basin. However, just before the equalization basin there is a weightbridge to measure the weight of the manure. Dry matter content of the manure is analyzed periodically to monitor the efficiency of digesters by plant personnel.

Biogas produced via anaerobic reactors recorded (by volume) continuously with digital biogas meters. Records of biogas meters could be monitored from control room where operational personnel monitoring all the process. Biogas meters are calibrated meters and calibration is valid for ten years as per related regulation.Recorded and saved biogas volume daya saved more than two years. In addition, site personnel have portable gas analyzer to measure the gas content of biogas also. However, in this study the default methane ratio will be used as mentioned in fix-ante parameters section above.  Related data recorded more than two years.

Moreover, the solid and liquid digestate used as fertilizer in nearby agricultural land and this materials are not used/stored in any other conditions on land. Digestate not kept in anaerobic conditions after anaerobic digester operations. It is stored under aerobic conditions and impermeable floor. The amount of liquid and solid digestate which will be used as fertilizer also will be recorded in volume/mass by site personnel. Related data stored more than two years.

#### SECTION C. DURATION AND CREDITING PERIOD

##### C.1. Duration of project

C.1.1 Start date of project

Start date of the project is 01/12/2019, date of construction agreement.

C.1.2 Expected operational lifetime of project

The generation license by EMRA dated 04/10/2018 is valid for 10 years (04/10/2028). The operational lifetime of the project is 8 years 9 months and 23 days starting from the start date of the project and ends on 04/10/2028 when the generation license is expired. The generation license is the legal permit for operation of project activity. Therefore, the operational lifetime of the project can be determined based on the period during which the generation license is valid.

##### C.2. Crediting period of project

C.2.1 Start date of crediting period

Start date of the crediting period of 5 years is 01/01/2021 and the crediting period ends on 31/12/2025.

C.2.2 Total length of crediting period

5 years for the first crediting period, 15 years for total.

#### SECTION D. SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

##### D.1 Safeguarding Principles that will be monitored

A completed Safeguarding Principles Assessment is in Appendix 1, ongoing monitoring is summarised below.

| Principles | Mitigation Measures added to the Monitoring Plan |
| --- | --- |
| SG Principle 7.1 Climate | CO2 reduction -The project contributes to 110,277 tonnes of CO2e reduction annually, which represent direct and quantifiable impact on climate security. |
| Principle 9.4 Release of Pollutants | Air Quality - Reduction of SO2 and NOx emissions due to implementation of project activity that would otherwise be emitted by thermal power plants |
| Principle 6.1: Labour Rights | Quality of Employment - Contribution to quality of employment by ensuring that the staff is trained and certified for the required positions |
| Principle 9.5 Hazardous and Non-hazardous Waste | Other Pollutants - Proper management of waste oil |

##### D.2. Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | As stated in Gold Standard Gender Policy document, “foundational gender sensitive certificiation” which is mandatory for every project requires compliance with the gender ‘do no harm’ safeguard, gender-gap analysis and gender sensitive stakeholder consultations. Although the project is a renewable energy project and does not have negative impacts on men and women, it complies with the criteria mentioned. Moreover, Turkey has ratified ILO convention 100 and 111 and discrimination based on gender is illegal in Turkey. The project try to align with the national gender strategy. So, the project does not involve and is not complicit in any form of discrimination based on gender difference. |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | The project aims to create new employment and income opportunities within the scope of SDG 8 . While doing this, a gender-equal strategy is implemented by creating employment opportunities for both man and woman without discrimination. According to the Woman Empowerment Strategy Paper and Action Plan prepared by the Ministry of Family and Social Policies (2018), there are 21 targets classified under 5 main fields in order to enhance participation of women to the society. In this action plan regarding the years 2018 – 2023, economy and employment-oriented strategies are represented for women since the labor force activity of women is not at desired level compared to the EU countries. According to 2017 statistics tabulated in the report, labor force activity of women is declared as 33.6% and it is aimed that this percentage will be 41% in 2023. Accordingly, the project shows parallelism with the national strategies developed for women. |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | No |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | - |

#### SECTION E. SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

The below is a summary of the 2 step GS4GG Consultation for monitoring purposes. Please refer to the separate Stakeholder Consultation Report for a complete report on the initial consultation and stakeholder feedback round.

##### E.1 Summary of stakeholder mitigation measures

Local Stakeholder Meeting was organized at 14:00 on 02/08/2017 in Boruktolu village which is the closest settlement to the project site. Considering the characteristics and possible impacts of the project activities, the stakeholders to be invited to the meeting were cooperatively selected by the project owner and GTE, in light of the Gold Standard rules.

Especially the village heads were asked to inform the locals about the project and the meeting (the hard copies of the nontechnical description of the project activity, description of the sustainable indicators, and questionnaire on sustainable development matrix and monitoring were also delivered to the locals on beforehand).

The meeting was also announced in a local newspaper on 26/07/2017.

The meeting date, time and place were defined by consulting with the village head to achieve a high level of participation.

A representative from Ministry of Environment and Urbanization which acts as the UNFCCC focal point in Turkey was also invited to the meeting.

In general, participants stated that the meeting was useful for them to better understand the project, biogas plant, benefits and effects on local regions. Moreover, stakeholders’ comments were positive since the project is beneficial for the national community. During the initial stakeholder consultation meeting, no significant issues have been raised by the participants that will require revisiting the sustainable development assessment.  Hence, no change in the project was considered.

##### E.2 Final continuous input / grievance mechanism

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | There is a process book on site and local stakeholders could write their opinion on it. Also, stakeholders could easily reach to project owner if they had any problem with the project. |
| GS Contact (mandatory) | help@goldstandard.org |
| Other |  |

### Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/potentially/no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | Turkey has ratified European Convention on Human Right on 10/03/1954. Therefore, the project is not expected to violate the rules regarding human rights. | No mitigation measure is required for this indicator. |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of non-discrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) | No | Turkey has ratified ILO convention 100, 111, 122 and 142, which provides gender equality and promotes women’s employment. It also shows parallelism with national strategies prepared for women employment by creating opportunities for all. Moreover, the project outputs serve everyone without regarding gender. It provides electricity for all. | No mitigation measure is required for this indicator. |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community | No | Turkey has ratified ILO convention 155 and about work safety and precautions | No mitigation is required for this indicator. |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | - No sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture were observed in the project area. | No mitigation is required for this indicator. |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | There were no settlement on project site on baseline scenario. | No mitigation is required for this indicator. |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | No | There were no settlement on project site on baseline scenario or not any private ownership. | No mitigation is needed for this indicator. |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | No indigenous people was identified. And so no one will be affected by negative noise conditions. | No mitigation is needed for this indicator. |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects | No | Turkey has ratified several conventions on bribery and corruption including OECD and UN conventions | No mitigation is needed for this indicator. |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | No | - Turkey has ratified ILO 87 and 98 conventions. - Turkey has ratified ILO convention 155 and about work safety and precautions. Staff will be trained for HSE during operation phases. - Turkey is a party of IPEC, since 1992 and ratified ILO convention 138 and 182. - All employee are to be recruited according to the national legislations. | Staff will be trained for HSE during operation phases. As per related regulations. For other issues/indicators no mitigation measure is required for this indicator. |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? | No | The investment is capital intensive but operating as low cost plant. VER revenues help in shorter return periods for investment. Details are given in sensitivity analysis part of this report and IRR calculation sheet of the project for the justification. | No mitigation needed for this indicator. |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | Since it is a biogas power plant, the project is expected to have a positive impact on Climate Change by eliminating fossil fuels with renewable sources. Expected yearly amount of CO2e: 110,277 tonnes (as indicated in emission reduction calculation sheet detaily). | Project expected to provide 110,277 tonne CO2 emission reduction and actual values will be monitored during crediting period. |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | Project does not use any local fuel resource. It is connected to the national grid and supply 42.028 GWh additional energy to the grid (as indicated in genetration licence of the project). | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | The project does not have any expected effects on the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) since there is no underground or other kind of water source on project site | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | The project does not directly or indirectly cause harm on soil and water. During the operation phase of the project, wastewater management will be carried out in accordance with the provisions of the "Water Pollution Control Regulation" (and all changes made in the regulation), which entered into force by publishing in the Official Gazette dated 31.12.2004 and numbered 25687. Moreover project will not need process water during the operation phase based on its technical configuration. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? | No | There were no settlement or agricultural activities on project site on baseline scenario or not any private ownership. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? |  | Meram district and the project site are in the 4th Degree Seismic Zone according to the earthquake zones determined by the General Directorate of Disaster Affairs. Hence construction of the project have been carried out accordingly. | All the design and construction works was performed in accordance with relevant regulation |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? | No | The project is not relevant since it is a renewable energy project. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? | Yes | Hazardous wastes are expected such as waste oils in the operation period. | Handling, storage and disposal of these wastes will be done according to the Turkish regulations. |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | Yes | Hazardous wastes are expected such as waste oils in the operation period. | Handling, storage and disposal of these wastes will be done according to the Turkish regulations. |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? | No | The project is not relevant since it is a renewable energy project. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? | No | There is no forest area in and in the close vicinity of the project area. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | The project is not relevant since it is a renewable energy project. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? | No | The project is not relevant since it is a renewable energy project. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? | No | No endemic, endangered and or threatened flora and fauna species were identified in the project site and its vicinity. | No mitigation measure is required for this indicator. |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? | No | No endemic, endangered and or threatened flora and fauna species were identified in the project site and its vicinity. | No mitigation measure is required for this indicator. |
| >> |  |  |  |

### Appendix 2- Contact information of project participants

| Organization name | Toros Meram Yenilenebilir Enerji Üretim A.Ş. |
| --- | --- |
| Registration number with relevant authority |  |
| Street/P.O. Box | Esentepe Mah. Büyükdere Cad |
| Building | Tekfen Tower No:209 Kat: 19 ve 20 |
| City | İstanbul |
| State/Region | 4. Levent -Şişli |
| Postcode | 34394 |
| Country | Turkey |
| Telephone |  |
| E-mail |  |
| Website |  |
| Contact person |  |
| Title | Corporate Risk Chief |
| Salutation |  |
| Last name | Sucu |
| Middle name |  |
| First name | Bahadır |
| Department |  |
| Mobile |  |
| Direct tel. | +90 212 357 02 02 /198 |
| Personal e-mail | bahadir.sucu@toros.com.tr |

| Organization name | GTE KARBON SURDURULEBILIR ENERJI EGT. DAN. VE TIC. A.S. |
| --- | --- |
| Registration number with relevant authority |  |
| Street/P.O. Box | Mustafa Kemal Mah. 2118. Cad. |
| Building | MAIDAN No: 4 C Blok 42 |
| City | Ankara |
| State/Region | Çankaya |
| Postcode | 06510 |
| Country | Turkey |
| Telephone |  |
| E-mail |  |
| Website | https://gte.com.tr/ |
| Contact person |  |
| Title | Director |
| Salutation |  |
| Last name | Demirkol |
| Middle name |  |
| First name | M.Kemal |
| Department |  |
| Mobile |  |
| Direct tel. | +90 312 514 63 63 |
| Personal e-mail | kemal@gtecarbon.com |

### Appendix 3- LUF Additional Information

| Risk of change to the Project Area during Project Certification Period: |  |
| --- | --- |
| Risk of change to the Project activities during Project Certification Period: |  |
| Land-use history and current status of Project Area: |  |
| Socio-Economic history: |  |
| Forest management applied (past and future) |  |
| Forest characteristics (including main tree species planted) |  |
| Main social impacts (risks and benefits) |  |
| Main environmental impacts (risks and benefits) |  |
| Financial structure |  |
| Infrastructure (roads/houses etc): |  |
| Water bodies: |  |
| Sites with special significance for indigenous people and local communities ‐ resulting from the Stakeholder Consultation: |  |
| Where indigenous people and local communities are situated: |  |
| Where indigenous people and local communities have legal rights, customary rights or sites with special cultural, ecological, economic, religious or spiritual significance: |  |

### Appendix 4-Summary of Approved Design Changes

Please refer to Design Change Requirements for more information on procedures governing Design Changes

##### Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.2 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.1 | 24 August 2017 | Updated to include section A.8 on ‘gender sensitive’ requirements |
| 1.0 | 10 July 2017 | Initial adoption |

https://globalgoals.goldstandard.org/421-wm-ghg-emission-reductions-from-manure-management-systems-and-municipal-solid-waste/

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

EIA Report

EIA Report

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

https://globalgoals.goldstandard.org/101-par-principles-requirements/

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/200-GS4GG-Renewable-Energy-Activity-Requirements-v1.1.pdf

The e-mail dated on 04/03/2020

https://globalgoals.goldstandard.org/101-par-principles-requirements/

https://cdm.unfccc.int/methodologies/documentation/1903/CDM-Methodology-Booklet_fullversion

https://globalgoals.goldstandard.org/421-wm-ghg-emission-reductions-from-manure-management-systems-and-municipal-solid-waste/

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-03-v3.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-05-v3.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-06-v3.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-16-v4.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v7.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-12-v1.1.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-08-v3.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-v7.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-04-v8.0.pdf

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://www.tarimorman.gov.tr/Belgeler/ENG/Legislation/law_veterinary_services.pdf

http://www.zmo.org.tr/mevzuat/mevzuat_detay.php?kod=55

http://www.resmigazete.gov.tr/eskiler/2006/08/20060809-4.htm

https://www.mgm.gov.tr/veridegerlendirme/il-ve-ilceler-istatistik.aspx?k=undefined&m=KONYA

https://www.mgm.gov.tr/veridegerlendirme/il-ve-ilceler-istatistik.aspx?k=undefined&m=KONYA

Türkiye’de Biyogaz Yatırımları için Gerekli Koşulların ve Potansiyelin Değerlendirilmesi. Yazar: DBFZ - Deutsches Biomasse Forschungs Zentrum gemeinnützige GmbH, Torgauer StraBe 116, 04347 Leipzig. Çeviri: Funda Cansu Ertem. Aralık, 2011

https://www.yatirimadestek.gov.tr/pdf/assets/upload/fizibiliteler/elazig-ili-biyogaz-tesisi-on-fizibilite-raporu2020.pdf

https://webapi.teias.gov.tr/file/3b6826b3-a880-4b42-8ba3-e5915a2281ea?download

TEİAŞ Report on Demand Estimations in 10 Years, Table 71, pg. 56 https://www.teias.gov.tr/sites/default/files/2019-06/taleprapor_2019-2028.pdf

https://www.teias.gov.tr/sites/default/files/2018-09/Kapasite_Projeksiyonu_2018_2022.pdf

http://www.fao.org/3/a-i6480e.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-v7.0.pdf

Law number 6446, Published in official gazette No. 28603 on 30/03/2013 http://www.mevzuat.gov.tr/MevzuatMetin/1.5.6446.pdf

Law number 5346, Published in official gazette No. 25819 on 18/05/2005 http://www.mevzuat.gov.tr/MevzuatMetin/1.5.5346.pdf

Law number 5627, Published in official gazette No. 26510 on 02/05/2007 http://www.mevzuat.gov.tr/MevzuatMetin/1.5.5627.pdf

Law number 6831, Published in official gazette No. 9402 on 08/09/1956 http://www.mevzuat.gov.tr/MevzuatMetin/1.3.6831.pdf

Law number 2872. Published in official gazette No. 18132 on 11/08/1983 http://www.mevzuat.gov.tr/MevzuatMetin/1.5.2872.pdf

https://www.resmigazete.gov.tr/eskiler/2015/04/20150402-2.htm

- https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-27-v10.0.pdf

- https://documents1.worldbank.org/curated/en/799701498842988254/pdf/ICR00004069-06192017.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-24-v1.pdf

https://www.epdk.gov.tr/Detay/DownloadDocument?id=4ETVkLvlwO0=

http://www.vivis.de/phocadownload/Download/2015_wm/2015_WM_79-84_Oeztuerk.pdf & https://www.sayistay.gov.tr/En/Upload/files/4-TCA_Waste_Management_Report.pdf

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

EIA Report

EIA Report

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf

- http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-06-v1.pdf  (footnote 3)

https://unfccc.int/documents/461898

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-05-v3.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-05-v3.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-12-v1.1.0.pdf

https://globalgoals.goldstandard.org/wp-content/uploads/2017/06/401.13-ER-MMS.pdf

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

https://webim.teias.gov.tr/file/8e92ef4c-48c6-410e-8852-dd4865cc816c?download

https://unfccc.int/documents/461898

EIA Report

EIA Report

https://www.mgm.gov.tr/veridegerlendirme/il-ve-ilceler-istatistik.aspx?k=undefined&m=KONYA

https://mgm.gov.tr/veridegerlendirme/il-ve-ilceler-istatistik.aspx?k=undefined&m=KONYA

https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=6381&MevzuatTur=7&MevzuatTertip=5

https://www.ilo.org/wcmsp5/groups/public/---europe/---ro-geneva/---ilo-ankara/documents/genericdocument/wcms_645630.pdf

http://www.sp.gov.tr/upload/xSPTemelBelge/files/RySPo+KADININ_GUCLENMESI_STRATEJI_BELGESI_VE_EYLEM_PLANI_2018-2023_.pdf, Page 17

http://www.sp.gov.tr/upload/xSPTemelBelge/files/RySPo+KADININ_GUCLENMESI_STRATEJI_BELGESI_VE_EYLEM_PLANI_2018-2023_.pdf, Page 76

http://www.sp.gov.tr/upload/xSPTemelBelge/files/RySPo+KADININ_GUCLENMESI_STRATEJI_BELGESI_VE_EYLEM_PLANI_2018-2023_.pdf, Page 79

- https://www.echr.coe.int/Documents/CP_Turkey_ENG.pdf

http://www.ilo.org/ankara/areas-of-work/equality-discrimination/lang--tr/index.htm

https://www.ilo.org/global/standards/subjects-covered-by-international-labour-standards/occupational-safety-and-health/WCMS_356966/lang--en/index.htm

http://www.masak.gov.tr/en/LaunderingProceedsofCrime/Chronology.htm

http://www.ilo.org/ipec/programme/lang--en/index.htm

http://www.ilo.org/ipec/Regionsandcountries/lang--en/index.htm

http://www.ilo.org/public/turkish/region/eurpro/ankara/about/sozlesmeler.htm