| Validation report form for renewal of crediting period for GS project activities |  |
| --- | --- |
| VALIDATION REPORT FOR RENEWAL OF CREDITING PERIOD (RCP) |  |
| Title of the project activity | Balikesir Susurluk 45 MW Wind Farm Project |
| GS Reference number of the project activity | GS 854 |
| Number and duration of the next crediting period | 3rd crediting period 7 years |
| Version number of the validation report for RCP | 3.0 |
| Completion date of the validation report for RCP | 13/08/2025 |  |
| Version number of PDD to which this report applies | 0.6 of 12/08/2025 |
| Project participant(s) | Susurluk Enerji A.Ş. |
| Host Party | Türkiye |
| Sectoral scope(s), selected methodology(ies) | Sectoral scope 1: Energy Industries (Renewable-/non-renewable sources) ACM0002, Consolidated baseline methodology for grid-connected electricity from renewable sources - Version 22.0 |
| Estimated annual average GHG emission reductions or net anthropogenic GHG removals in the next crediting period | 77,665 tCO2 |
| Name of VVB | RINA Services S.p.A. (RINA) |
| Name, position and signature of the approver of the validation report for RCP | Laura Severino (Authorized officer signing for the VVB) Decarb & Chain of Custody Product Management |

  - Executive summary

Purpose and general description of the project

The project activity is a wind power plant consisting of 18 turbines with each having 2.5 MW capacities making the total installed capacity of 45 MW, consisting of fifteen Nordex N100 and three N90 wind turbines, thus qualifies under large scale projects. The generated electricity is fed to the national grid. The estimated net electricity production is 121,939 MWh/year and the annual emission reductions are estimated to be 77,665 tCO2 /1/. The generation license has been revised for capacity extension, eleven turbines were added to the project on 16/07/2013, 13/09/2013 and 30/05/2017. With this addition, the installed capacity of the project has been raised to 75 MWm/72.5 MWe. Since the registered capacity of the project is considered, the electricity generation and the emission reduction of the added units are ignored for the issuances.

The proposed project activity comprises of renewable energy generation from wind utilization to generate and deliver electricity to the national grid without thermal energy production. The project type is wind power plant which is an eligible project type as it is in accordance with Eligible Project Types & Scope under Renewable Energy Activity Requirements. The project is located in Türkiye. Türkiye is upper middle-income economy where the penetration level of the proposed Renewable Energy Technology type is less than %5 of the total grid installed capacity, at the time of the first submission to Gold Standard (eligibility clause will come into effect from 24 Jan 2020). The project activity aims to reduce the greenhouse gas emissions in Türkiye by replacing fossil fuel power generation and contributing to the development of the waste energy sector in Türkiye, as well as aims to support the local economy by creating local employment and providing equipment locally.

The GHG benefit of the project activity was only accounted under Gold Standard. There are not any other RECs such as VERRA, GCC, ICR, CerCarbono or Social Carbon were being issued for the project activity. Furthermore, as a host country in Türkiye such any programme like a government-regulated system or programme for the constraint and monetisation of GHG emissions (such as emissions trading scheme, cap and trade or carbon tax mechanisms) has not been implemented.

Location

The project is located in Balıkesir Province, Turkey.

Scope of Validation

The validation scope is to review the updated PDD against the GS principles and requirements. The validation of the renewal of crediting period is also to be seen in conjunction with the validation report at the time of requesting registration of the project (re-carbon – re-validation report N° 659, version 0.3 issued on 12/12/2022) /25/. Validation of the renewal of crediting period is a requirement, and it is seen as necessary to assure about:

- the impact of new relevant national and/or sectoral policies and circumstances on the baseline;

- the correctness of the application of an approved baseline methodology for the determination of the continued validity of the baseline or its update, and the estimation of emission reductions for the applicable crediting period.

Validation process

Validation is conducted using RINA procedures in line with the GS requirements and principles and applying standard auditing techniques. The validation assessment involved a document review of relevant documentation, the interview and/or online audit, and the reporting. Validation is not meant to provide any consultancy of the project participants. However, stated requests, clarifications and/or corrective actions may have provided input for improvement of the project design.

Conclusion

RINA commissioned by Susurluk Enerji A.Ş. has performed the validation of the renewal of crediting period of the project activity “Balikesir Susurluk 45 MW Wind Farm Project”, about the relevant GS requirements and principles for project activities.

In conclusion, it is RINA’s opinion that the project activity “Balikesir Susurluk 45 MW Wind Farm Project “as described in the PDD version 0.6 of 12/08/2025 /1/ and previous versions, meets all relevant GS requirements and principles, and correctly applied the baseline and monitoring methodology ACM0002, Grid-connected electricity generation from renewable sources - version 22.0 of 31/05/2024.

  - Validation team, technical reviewer and approver

    - Validation team member

| No. | Role | Type of resource | Last name | First name | Affiliation (e.g. name of central or other office of VVB or outsourced entity) | Involvement in |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Desk review | On-site inspection | Interview(s) | Validation findings |
| 1. | Team Leader, Validator, Technical Expert TA 1.2 | IR | ERDOĞAN | Mehmet | RINA Türkiye |  |  |  |  |
| 2. | Auditor in Training, Validator in Training Technical Expert TA 1.2 in Training | IR | ONARAN | Ilayda | RINA Türkiye |  |  |  |  |

    - Technical reviewer and approver of the validation report for RCP

| No. | Role | Type of resource | Last name | First name | Affiliation (e.g. name of central or other office of VVB or outsourced entity) |
| --- | --- | --- | --- | --- | --- |
| 1. | Technical reviewer | IR | Amalorpavanathan | Cyril A A | RINA India |
| 2 | Approver | IR | SEVERINO | Laura | RINA HO |

  - Means of validation

    - Desk review

The latest PDD version 0.6 of 12/08/2025 and/or previous versions /1/, in particular the applicability of the methodology, the baseline determination, the emission reductions calculation, the sustainability indicators, were assessed as part of the validation. All documents reviewed or referenced during the validation are listed in Appendix 3.

    - On-site inspection

| Duration of Online Audit: 07/02/2025 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Activity performed on-site | Site location | Date | Team member |
| 1. | - Implementation and operation of the proposed project activity. - Baseline and Additionality (Financial Ongoing) - Emission reductions calculations - Interviewed key personnel of the plant to confirm the operational and data collection procedures, QA QC procedures - GS4GG Sustainable Indicators - Local stakeholder consultation - Grievance & Input mechanism | Susurluk Balıkesir Province in Türkiye | 07/02/2025 | Mehmet ERDOĞAN Ilayda ONARAN |

    - Interviews

| No. | Interviewee |  |  | Date | Subject | Team member |
| --- | --- | --- | --- | --- | --- | --- |
|  | Last name | First name | Affiliation |  |  |  |
| 1 | T. | İsmail | Susurluk/Demirkapı Muhtkar | 07/02/2025 | Implementation status of the project Monitoring equipment and operation Generated Electricity Monitoring of Gold Standard for Global Goal Parameters Local Employment Receiving General Opinion of the local stakeholder about the project Benefit of the project to the village Bird carcasses Grievance Process | Mehmet ERDOĞAN Ilayda ONARAN |
| 2 | K. | İlker | Susurluk/Demirkapı Stakeholder |  |  |  |
| 3 | B. | Serdar | Susurluk/Demirkapı Stakeholder |  |  |  |
| 4 | T. | Furkan | Susurluk/Demirkapı Stakeholder |  |  |  |

Online site visit was conducted on 07/02/2025 with the PP and by the VVB accordance with GS Site Visit and Remote Audit Requirements and Procedures. Latest physical site visit performed on 21/03/2023 during 3rd  verification (first verification of second crediting period). The project employees were interviewed about the implementation status of the project, monitoring equipment and operation, generated electricity of the project activity.

During online audit it was confirmed that no negative feedbacks or comments were received related to project activity during grievance mechanism and continuous inputs for the monitoring period. There was also no grievance about the impact such as noise and flickering that reached the VVB during the stakeholders' interviews. In addition to this, the VVB assessed whether a comment box or logbook available at the most appropriate and publicly accessible location (Ömerköy and Demirkapı villages in Susurluk District, the nearest residential area) so that stakeholders can provide feedback on the project. The continuous input/grievance mechanism has been verified through interview with the headman of village and the logbook /11/ have been checked. There isn’t any positive or negative comment written on them.

In addition to this during interview, it is asked to the stakeholders and project employees if any legal contests or disputes have arisen during the monitoring period and they confirmed that there is no legal contest or disputes have arisen.

    - Clarification requests, corrective action requests and forward action requests raised

| Area of validation findings | No. of CR | No. of CAR | No. of FAR |
| --- | --- | --- | --- |
| Compliance with PDD form | 1 |  |  |
| Compliance with GS Passport form |  |  |  |
| Application of baseline and monitoring methodology and standardized baseline | 1 | 2 |  |
| Validity of original baseline or its update | 1 |  |  |
| Do No Harm assessment |  |  |  |
| Impact assessment – Sustainable Development Matrix |  |  |  |
| Estimated GHG emission reductions or net anthropogenic GHG removals |  | 1 |  |
| Validity of monitoring plan |  |  |  |
| Validity of sustainable monitoring plan | 2 |  | 1 |
| Project participants |  |  |  |
| Local stakeholder consultation process |  |  |  |
| Others (GS Issuance Review) |  |  | 1 |
| Total | 5 | 3 | 2 |

  - Validation findings

    - Compliance with PDD form

| Means of validation | The title of the project activity is defined as “Balikesir Susurluk 45 MW Wind Farm Project” in the PDD version 0.6 of 12/08/2025 /1/ which is in line with the GS4GG Registry. The version and the completion date of the report are available at the cover page of the PDD. “GS4GG Project Design Document Template /5/, version 1.5” which is the latest version is applied with the PDD /1/. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA confirms that the PDD is based on the currently valid GS4GG Project Design Document Template /5/, version 1.5. |

    - Compliance with GS Passport form

| Means of validation | No GS Passport is available under GS4GG. |
| --- | --- |
| Findings | NA |
| Conclusion | No GS Passport is available under GS4GG. |

    - Application of baseline and monitoring methodology and standardized baseline

| Means of validation | The project correctly applies the approved baseline and monitoring methodologies “ACM0002”, “Grid-connected electricity generation from renewable sources”, version 22.0 of 31/05/2024 /6/. The applied methodologies are approved by CDM and the applied versions are the latest version at the time of PDD submission. In addition, the following methodological tools are applied in this PDD: - TOOL07 “Tool to calculate the emission factor for an electricity system” Version 07.0 /7/. - TOOL01 “Tool for the demonstration and assessment of additionality”, Version 07.0 /8/. - TOOL03 “Tool to calculate project or leakage CO2 emissions from fossil fuel combustion”, Version 03.0 /9/. - TOOL11 “Tool to assess the validity of the original/current baseline and to update the baseline at the renewal of a crediting period “ 03.0.1 /10/. The project activity is a greenfield, grid connected renewable electricity generation project. - The project activity is the installation of wind power plant. - The project does not involve capacity addition, a retrofit of (an) existing plant(s) or a replacement of (an) existing plant(s). - The project does not involve Integrate BESS with a Greenfield power plant; BESS + capacity addition to existing plant(s); BESS with no other changes to the existing plant(s) and BESS + retrofit to existing plant(s). - Project activity does not involve switching from fossil fuels to renewable energy sources at the site of project activity. - Project activity does not involve biomass fired power plants/units. - The project does not involve combined heat and power generation activity. - The geographic and system boundaries for the relevant electricity grid can be clearly identified and information on the characteristics of the grid is available. According to applicability conditions given in the tools; - Tool 01 Tool for the demonstration and assessment of additionality: The project uses relevant tool together with ACM0002 methodology. No new methodology is used. - Tool 07 Tool to calculate the emission factor for an electricity system: This tool is applicable and used for the calculation of OM, CM and CM since the project activity includes grid power plants and supplies electricity to the grid. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA confirms that the selected baseline and monitoring methodologies have been previously approved by the CDM Executive Board and are applicable to the project, which complies with all the applicability conditions therein the selected versions are valid at the time of submission of crediting period. It is also confirmed that the methodologies are correctly applied by comparing them with the actual text of the applicable versions. |

    - Validity of original baseline or its update

| Means of validation | The project applies for a renewal of the crediting period under the requirements of The Gold Standard Foundation so the Methodological Tool “Assessment of the validity of the original/current baseline and update of the baseline at the renewal of the crediting period”, Version 03.0.1 /10/ is applied to demonstrate that the conditions used to determine the baseline emissions in the previous crediting period still valid. The validity of the current baseline is assessed using the following Sub-steps: Step 1.1: Assess compliance of the current baseline with relevant mandatory national and/or sectoral policies It is validated that the project activity baseline is the “Grid-connected electricity generation from renewable sources”. It is still in compliance with the current legal framework as stated in the generation license /12/. There are no additional laws that came into force that has an impact on the project activity and the project activity is still in line with the available law and regulations. In addition to this since 2019 Turkish Ministry of Energy and Natural Resources calculates and presents the Türkiye National Electricity Generation Grid Emission Factor /14/. Step 1.2: Assess the impact of circumstances It is assessed that the conditions used to determine the baseline emissions in the previous crediting period are still valid. There is no need any of new fuels or raw materials and the impact of electricity or fuel prices in the identification of the current practice for the baseline emissions. As stated earlier the new national circumstances have an impact on the EF /14/ of the grid and thus on the project’s current baseline emissions. Step 1.3: Assess whether the continuation of use of current baseline equipment(s) or an investment is the most likely scenario for the crediting period for which renewal is requested. It is assessed that the remaining technical lifetime of the equipment that would have continued to be used in the absence of the project activity. In general, the technical lifetime of the project activity (including wind turbines) is defined as 25 years but in real case scenario it seems to be higher than the 25 years thanks to regular maintenance and new technical upgrades. So, the equipment’s lifetime exceeds the crediting period for which renewal is requested. Equipment only requires regular maintenance. The baseline scenario identified at the validation of the project activity was the continuation of grid-connected electricity generation from renewable sources. Under this scenario, no investment from the project’s proponent or third party (or parties) has been envisioned later specifically for the project. Step 1.4: Assessment of the validity of the data and parameters The data and parameters that were only determined at the start of the crediting period and not monitored during the crediting period are still valid have been assessed. For baseline calculation there are two main parameters: the electricity generation and the grid emission factor. In line with the tool and methodology /6/ /28/, only the grid emission factor has been updated during the third crediting period renewal. According to the tool /28/, the application of Steps 1.1, 1.2, 1.3 and 1.4 confirmed that the current baseline as well as data and parameters are still valid for the 3rd and last crediting period, then this baseline, data and parameters can be used for the renewed crediting period. Step 2.1: Update the current baseline The current baseline emissions for the subsequent crediting period have been updated. Step 2.2: Update the data and parameters In line with the tool and methodology, only the grid emission factor has been updated during the second crediting period renewal. The project activity is in its 3rd crediting period. During the previous crediting periods, the project has not missed or skipped neither timeline of verification nor CP renewal process. The ongoing financial need for carbon revenue continues for the last crediting period. Since the increased maintenance and repair costs, exchange rate effect and also the system usage and operation fees that need to be paid by the project activity to Turkish Electricity Transmission Corporation (TEİAŞ) is another important factor contributing to the project's expenditure. It is validated that the Balikesir Susurluk 45 MW Wind Farm Project needs the supporting income derived from carbon credits since the electricity generation market has challenges due to the Regulator intends to keep electricity prices at minimum. Project owner declared that VER revenue from the project is mostly used for the operation costs. The ratio of carbon sales revenue to electricity sales revenue is 0.14%. Financial ongoing excel sheet /32/ has been verified with carbon sales invoices /33/ as well. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA verified that the baseline was updated assessed according to the tool “Assessment of the validity of the original/current baseline and update of the baseline at the renewal of the crediting period” /10/. The conclusion is that the baseline of the type I component of the project activity complies and will continue to comply with the laws and regulations in the sector for the next crediting period. It can be concluded that the conditions used to determine the baseline emissions in the previous crediting period are still valid. |

    - Do No Harm Assessment

| Means of validation | As per GS4GG Safeguarding principles and requirements version 1.2, all projects shall conform to the Gold Standard for the Global Goals Safeguarding Principles & Requirements. SDG Impact tool has been also implemented as required by GS4GG and verified through the assessment tool by VVB /3/. The assessment is done as follows: Principle 1- Human Rights: The Project is not in conflict with the economic livelihood or other issues of the local community. Thus, the Project VVBs do not cause any human rights abuse and respects internationally proclaimed human rights issue. Türkiye has ratified European Convention on Human Right /18/ on 10/03/1954. Therefore, the project is not expected to violate the rules regarding human rights. Principle 2 – Gender Equality and Women’s Rights: Türkiye has ratified ILO convention 100, 111, 122 and 142, which provides gender equality and promotes women’s employment /19/. The project outputs serve everyone without regarding gender. It provides electricity for all. Principle 3 – Community Health, Safety and Working Conditions: Türkiye has ratified ILO convention 155 /24/ and about work safety and precautions. Staff will be trained during operation phases. Principle 4 – Cultural Heritage, Indigenous Peoples, Displacement and Resettlement: According to the registered PDD of 2nd crediting period /13/ and Validation Report /25/ of Balikesir Susurluk 45 MW Wind Farm Project, no sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture were observed in the project area. Principle 5 – Corruption: Türkiye has ratified several conventions on bribery and corruption /20/ including OECD and UN conventions. Principle 6 – Economic Impacts: Türkiye has ratified ILO 87 and 98 conventions. All employees are recruited according to the national legislations. Türkiye is a party of IPEC /22/, /23/ since 1992 and ratified ILO convention 138 and 182. Susurluk Enerji A.Ş. and appointed subcontractors do not involve in any form forced or compulsory labour Türkiye has ratified ILO 29 Forced Labour Convention /21/. ENVIRONMENTAL & ECOLOGICAL SAFEGUARDING PRINCIPLES: Principle 1 – Climate and Energy: The project reduces greenhouse gas emissions and fossil fuel use compared to the baseline scenario. On the contrary the project generates renewable energy and supplies to the grid. Principle 2 – Water: The project is a wind power project thus there is no direct impact of water resources due to the project. Staffs produce the insignificant amount of waste waters, and this wastewater has been collected in an impermeable septic tank and collected via vacuum trucks by municipality and disposed according to Regulation on Control of Water Contamination as confirmed through the previous registered PDD /1/. Principle 3 – Environment, ecology and land use: The Land for the project has been approved by all relevant local Authorities. The project is susceptible to decreased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme conditions. Balikesir Susurluk 45 MW Wind Farm Project systems do not affect the herbal life negatively. Furthermore, Balikesir Susurluk 45 MW Wind Farm Project not affect negatively organism, flora, fauna or GMOs as confirmed through the previous registered PDD /13/ The project takes a precautionary approach in regard to environmental challenges and is not complicit in practices contrary to the precautionary principle. The environment is protected by several Laws and Regulations in Türkiye. The purpose of the “Law on Environmental Protection” is to protect the environment with principles of sustainable development and environment. The project owner also follows necessary procedures for environmental safety at the project site at international standard (such as Bern Convention). All wastes are disposed of according to related regulations. The methods are categorized for all materials. Waste oil produced has been collected appropriately and disposed via accredited abatement companies in line with the legal regulations as confirmed through the registered PDD /13/. The same processes apply for this crediting period. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA confirms that conservative approach has been applied by PP to demonstrate sustainable development of the project activity which is in line with GS4GG requirements and SDG Tool Impact Requirements /3//31/. |

    - Impact assessment – Sustainable Development Matrix

| Means of validation | As per GS4GG, Sustainable Developments Goals are discussed under Section D.9 of this report. |
| --- | --- |
| Findings | NA |
| Conclusion | As per GS4GG, Sustainable Developments Goals are discussed under Section D.9 of this report. |

    - Estimated GHG emission reductions or net anthropogenic GHG removals

| Means of validation | Baseline Emissions The calculation of baseline emission reductions is carried out in a conservative manner providing documentation and references to data sources. An approved CDM methodology, ACM0002 /6/ and methodological tool “Tool to calculate the emission factor for an electricity system /7/ is applied. The official published data, Turkish Ministry of Energy and Natural Resources /14/ are used to determine the emission factor of the grid. The emission factor is calculated as a combined margin (CM), consisting of the combination of operating margin (OM) and build margin (BM) according to the procedures prescribed in the tool. OM Calculation Option (a) simple OM is selected to calculate the operating margin (OM) emission factor of the grid as per the tool since the average share of electricity generation by low-cost/must-run plants for five most recent years is found to be less than 50%. Latest and updated OM which has been published by Ministry of Energy and Natural Sources has been used. BM Calculation The build margin (BM) emission factor calculation is in line with the latest version of methodological tool “Tool to calculate the emission factor for an electricity system /6/. The build margin emission factor for the 2nd crediting period was used for the third crediting period. CM Calculation The combined Margin is calculated for wind and solar power generation project activities: wOM = 0.75 and wBM = 0.25 for the first crediting period and for subsequent crediting periods. Project Emissions Project emissions are neglected since the project activity is a wind power plant in line with the ACM0002 /6/. Leakage The leakage emissions are neglected as per ACM0002 /6/. Emission Reduction The ex-ante emission reduction has been calculated as the following equation in line with the ACM0002 /6/: ERy = BEy – PEy – LEy The project emission and leakage are neglected for the project activity. Therefore, the emission reduction equals baseline emissions. All the assumptions and data used by the project participants are listed in the PDD including their references and sources. All the documentation used by the project participants as the basis for assumptions and source of data are quoted and interpreted in the PDD and the Emission Reduction Calculation Spread Sheet /2/. |
| --- | --- |
| Findings | NA |
| Conclusion | It is RINA’s opinion: - All assumptions and data used by the PP are listed in the PDD; - All documentation used by the PP as the basis for assumption and source of data is correctly quoted and interpreted in the PDD - All values used in the PDD and ERs spreadsheet including GWPs are considered reasonable in the context of the proposed project activity - The baseline methodology and methodological tools have been applied correctly to calculate project emissions, baseline emissions, leakage and emission reductions; All estimates of the baseline and project emissions can be replicated using the data and parameters values provided in the PDD and ERs spreadsheet. |

    - Validity of monitoring plan

| Means of validation | The monitoring plan (data and parameters fixed ex ante) includes all data and parameters fixed ex ante required by the applied methodology /6/. Parameters ex-ante: Data/ parameter; Unit; Value applied; Assessment. /1/; Operating Margin of Türkiye National Grid (OM); tCO2/ MWh; 0.7108; The data used to determine the OM emission factor of the grid is obtained from “Turkish Ministry of Energy and Natural Resources” /14/. The re-validation team has confirmed that this is the most updated data at the time for submission for re- validation, which is considered as appropriate.. /2/; Build Margin of Türkiye National Grid (BM); tCO2/ MWh; 0.4153; According to Tool07 “Tool to calculate the emission factor for an electricity system” the data used to determine the BM emission factor of the grid is obtained from registered PD for 2nd crediting period was used for the third crediting period.. /3/; Combined Margin of Türkiye National Grid (CM); tCO2/ MWh; 0.6369; The emission factor is calculated as a combined margin (CM), consisting of the combination of operating margin (OM) and build margin (BM) according to the procedures prescribed in the tool /7/.. Parameters ex-post: Parameter; Description/Assessment. /1/; EGPJ,grid,y (MWh); Quantity of electricity generation supplied by the project plant to the grid in year y The quantity of electricity supplied by the project plants/units to the grid and the quantity of electricity delivered to the project plant/unit from the grid will be measured. Net generation will be calculated via subtracting energy delivered by the project activity to the grid for internal consumption from electricity fed to the grid. The electricity generation of the plant will be calculated based on meter readings and crosschecked with EPIAS records that are the basis of sold electricity. The net electricity will be measured continuously and recorded at least monthly. For this crediting period the value of the parameter is defined as 121,939 MWh/y /2/. Net electricity generation will be measured by two meters which will be sealed by TEIAS. The meters will comply with EMRA (Energy Market Regulatory Authority) regulation /15/. The accuracy class of the meters defined and confirmed as 0.5s are in line with the “Communiqué for Measurement Devices used in the Electricity Market” /16/ as explained in the PDD /1/. The maintenance and calibration of meters are under TEIAS responsibility. If any major discrepancy occurs between the two meters, TEIAS performs necessary calibration. Current Electricity Meters Electricity Meter(Primary); Electricity Meter (Secondary). Manufacturer; EMH; EMH. Model; LZQJ-XC; LZQJ-XC. Serial number; 9674603; 9674604. The Latest Calibration Date of the Meter; 02/11/2020; 02/11/2020. The Latest Test Dates of the Meter; 07/03/2022; 07/03/2022. The accuracy of meters; 0.5s; 0.5s. The meters have the accuracy of 0.5 s as confirmed through first index protocol /17/. The accuracy class of the meters complies with the “Communiqué for Measurement Devices used in the Electricity Market” /17/ and given information in the PDD /1/. The electricity meters are sealed by TEIAS as confirmed during the site visit. The meters were calibrated on 02/11/2020 as confirmed through first index protocol /17/ performed by TEİAŞ. The recalibration of these meters will be done in line with the equipment requirements and through the period defined by national metrology institutes country by country and for Türkiye this period is defined as 10 years. The calibration of meters is deemed appropriate and in compliance with the national regulation /15-16/. In addition, protocol between company and TEİAŞ; It is stated that the periodic inspection and testing of the meters should be done every 2 years. The last periodic control of the electricity meters done on: 07/03/2022.. |
| --- | --- |
| Findings | NA |
| Conclusion | It is RINA’s opinion that the monitoring plan is in accordance with the monitoring methodology; the monitoring plan will give opportunity for real measurement of achieved emission reductions. RINA has checked all the parameters presented in the monitoring plan against the requirements of the methodology and methodological tools. RINA confirms that the monitoring arrangements described in the monitoring plan, including the data management and quality assurance and quality control procedures, are feasible within the project design, and the means of implementation of the monitoring plan are sufficient to ensure the emission reductions achieved by/resulting from the proposed GS4GG project activity can be reported ex post and verified. |

    - Validity of the sustainability monitoring plan

| Means of validation | The Sustainability Monitoring Plan includes the way of monitoring for each GS4GG indicator. Parameter; Description/Assessment. 1; EGPJ,grid,y; SDG 7 - Indicator 7.2.1 – “renewable energy share in the total final energy consumption”: The electricity generation supplied to the grid will be monitored by EPIAS records. It is expected that the project generates 121,939 MWh/y /2/.. 2; Quantitative employment and income generation; SDG 8: Decent Work and Economic Growth: Number of employment generation will be monitored once for each monitoring period by employment SGK records (Social insurance registries of employees). The project provides 16 permanent employments.. 3; Quality of employment; SDG 8: Decent Work and Economic Growth: HSE trainings will be provided to all employees. Attendance records or training certificates of Health and Safety trainings will be provided to all employees.. 4; ERy; SDG 13 - Indicator 13.3.1 – Baseline emissions correspond to emission reductions and are calculated as the net electricity generated by the project activity, multiplied with combined margin CO2 emission factor for grid connected power generation in year y. Both measured and calculated Emission reductions will be calculated as considering the EPIAS records for the net electricity generated and the emission factor for the grid, which is calculated and published by The Ministry of Energy and Natural Resources of Türkiye /14/. According to EF and net electricity generation emission reduction is applied as 77,665 tCO2.. 5; Biodiversity; Safeguarding Principle 9.10 High Conservation Value Areas and Critical Habitats Monitoring of bird/bat carcasses and nests will be carried out by appointed personnel Mr. Furkan KAVAS l.. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA confirms that sustainability monitoring plan and indicators included in the PDD confirm to the sustainable development requirements of GS4GG. |

    - Crediting period

| Means of validation | The third and last crediting starting date is 13/02/2025 /1/. The length of crediting period is defined as 7 years so end date of CP is 12/02/2032. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA confirmed that this is the third crediting period of the registered GS4GG project activity. |

    - Project participants

| Means of validation | Project Participant of the project activity is Susurluk Enerji A.Ş. who is authorized to benefit from carbon income. |
| --- | --- |
| Findings | NA |
| Conclusion | Project Participant of the project activity is Susurluk Enerji A.Ş. who is authorized to benefit from carbon income. |

    - Local stakeholder consultation process

| Means of validation | The project validated via online audit on 07/02/2025 for re-validation. Based on the interviews, there are no complaints from the local community regarding the project implementation &operation /11/. The project participants applied a local stakeholder consultation (LSC) process on 14-15/07/2009 line with the regular requirements of GS. Also a stakeholder feedback round has been realized in compliance with the GS procedures governing the issue 60 days by sending the project summary and environmental and socioeconomic questionnaire by courier for initial validation of project. Also grievance logbooks have been located at Mukhtar’s office and always available for stakeholders to provide their opinion. Grievance box was checked during online visit and no compliance was observed. Also soft copies of logbooks have been shared with VVB. For the Balikesir Susurluk 45 MW Wind Farm Project, stakeholder engagement procedure was conducted online and the local stakeholders from Ömerköy and Demirkapı villages in Susurluk participated in this online meeting at the project site on 07/02/2025. During the meeting, the stakeholders stated that they had no complaints. In addition, feedback box was checked to see whether there have been any complaints from the local stakeholders, and it is seen that there were no negative comments from the local stakeholders. |
| --- | --- |
| Findings | NA |
| Conclusion | RINA verified that no negative feedback is received during the CP renewal process on 07/02/2025. |

    - Post-registration changes

| Type of post-registration changes (PRCs) | Confirmation (Y/N) |
| --- | --- |
| Temporary deviations from the registered monitoring plan, monitoring methodology or standardized baseline | N |
| Corrections | N |
| Inclusion of a monitoring plan to a registered project activity | N |
| Permanent changes from registered monitoring plan, monitoring methodology or standardized baseline | N |
| Changes to the project design of a registered project activity | N |
| Types of changes specific to afforestation and reforestation project activities | N |

  - Internal quality control

The draft final validation report before being submitted to the client is subjected to an independent technical review to confirm that all validation activities has been completed according to the pertinent RINA’s procedures. The technical review is performed by a technical reviewer(s) qualified in accordance with the RINA’s qualification procedure.

  - Validation opinion

RINA has performed the validation of the updated PDD version 0.6 of 12/08/2025 for the project activity “Balikesir Susurluk 45 MW Wind Farm Project” in Türkiye, GS Registration Reference No. 854; the validation is performed for the 3rd renewal crediting (from 13/02/2025 to 12/02/2032) and is based on the information available to us.

The review of the PDD version 0.2 and the subsequent follow-up interview have provided RINA with sufficient evidence to determine the validity of the original baseline and the sustainable indicators; the project correctly applied the baseline and monitoring methodologies ACM0002, Grid-connected electricity generation from renewable sources - version 22.0 of 31/05/2024.

In conclusion it is RINA’s opinion that the project activity meets all the relevant GS requirements (GS VERs) for the renewal of the 3rd renewal crediting period.

- Abbreviations

| Abbreviations | Full texts |
| --- | --- |
| BE | Baseline Emissions |
| CAR | Corrective Action Request |
| CDM | Clean Development Mechanism |
| CDM M&P | Modalities and Procedures CDM |
| CER(s) | Certified Emission Reduction(s) |
| CH4 | Methane |
| CL | Clarification Request |
| CO2 | Carbon dioxide |
| CO2e | Carbon dioxide equivalent |
| CRT | Coordination and Technical Control Staff |
| DCI | Certification Division of RINA Services Spa |
| DNA | Designated National Authority |
| VVB | Validation Verification Body |
| EB | Executive Board |
| ER | Emission Reductions |
| FAR | Forward Action Request |
| GHG(s) | Greenhouse gas(es) |
| GS4GG | Gold Standard for Global Goal |
| GWP | Global Warming Potential |
| IPCC | Intergovernmental Panel on Climate Change |
| LoA | Letter of Approval |
| MoV | Means of Verification |
| MR | Monitoring Report |
| NGO | Non-governmental Organization |
| ODA | Official Development Assistance |
| PDD | Project Design Document |
| PE | Project Emission |
| PP(s) | Project Participant(s) |
| Ref. | Document Reference |
| RINA | RINA Services Spa |
| SS(s) | Sectoral Scope(s) |
| TA(s) | Technical Area(s) |
| UNFCCC | United Nations Framework Convention on Climate Change |
| VVS | Validation and Verification Standard |

- Competence of team members and technical reviewers

- Documents reviewed or referenced

| No | Author | Title | References to the document | Provider |
| --- | --- | --- | --- | --- |
| 1 | Sekans Danışmanlık | PDD for “Balikesir Susurluk 45 MW Wind Farm Project” | Version 0.6 of 12/08/2025 Version 0.5 of 11/06/2025 Version 0.4 of 25/03/2025 Version 0.3 of 07/03/2025 Version 0.2 of 10/02/2025 Version 0.1 of 03/02/2025 | Project Participant |
| 2 | Sekans Danışmanlık | ER calculations_Susurluk_WPP_v01_03_02_2025 | Version 0.1 of 03/02/2025 | Project Participant |
| 3 | Gold Standard Foundation | GS4GG Standard Requirements | version 2.0 of 12/11/2024 | Publicly available |
| 4 | CDM Executive Board | CDM Validation and Verification Standard for Project Activities | version 03 of 09/09/2021 | Publicly Available |
| 5 | Gold Standard Foundation | GS4GG PDD Report Form | version 1.5 of 29/06/2023 | Publicly available |
| 6 | CDM Executive Board | “ACM0002”, “Grid-connected electricity generation from renewable sources” | version 22.0 of 31/05/2024 | Publicly Available |
| 7 | CDM Executive Board | Methodological Tool: Tool to calculate the emission factor for an electricity system | version 07.0 of 31/08/2018 | Publicly Available |
| 8 | CDM Executive Board | Methodological Tool: Tool for the demonstration and assessment of additionality | version 07.0 of 23/11/2012 | Publicly Available |
| 9 | CDM Executive Board | Methodological Tool: Tool to calculate project or leakage CO2 emissions from fossil fuel combustion | version 03.0 of 22/09/2017 | Publicly Available |
| 10 | CDM Executive Board | Methodological Tool: “Tool to assess the validity of the original/current baseline and to update the baseline at the renewal of a crediting period “ | version 03.0.1 of 02/03/2012 | Publicly Available |
| 11 | Demirkapı Susurluk Village | Logbook | seen on 07/02/2025 | Project Participant |
| 12 | Energy Market Regulatory Authority | Generation License EU/1690-2/1224 | date of 24/07/2008 | Project participant |
| 13 | Sekans Danışmanlık | Registered PDD of 2nd crediting period | version 8 of 22/11/2021 | Project participant |
| 14 | Turkish Ministry of Energy and Natural Resources | Türkiye National Electricity Grid Emission Factor Information Form | version 00 of 26/11/2024 | Project Participant |
| 15 | The Ministry of Trade and Industry | Regulation of Metering and Testing of Metering Systems | of 24/07/1994 | Project Participant |
| 16 | Turkish Energy Market Regulatory Authority | Communiqué for Measurement Devices used in the Electricity Market | of 22/03/2003 | Project Participant |
| 17 | TEIAS | 1st Index Protocol of the Electricity Meters | date of 02/11/2020 | Project participant |
| 18 | Website: Argument: Language: | https://www.mevzuat.gov.tr/MevzuatMetin/1.5.6701. pdf Human Rights Turkish | Retrieved on 05/07/2024 | Publicly Available |
| 19 | Website: Argument: Language: | http://www.ilo.org/ankara/areas-of-work/equality- discrimination/lang--tr/index.htm Gender Equality and Women’s Rights Turkish | Retrieved on 05/07/2024 | Publicly Available |
| 20 | Website: Argument: Language: | https://www.mevzuat.gov.tr/MevzuatMetin/1.5.3628.pdf Bribery and Corruption Turkish | Retrieved on 05/07/2024 | Publicly Available |
| 21 | Website: Argument: Language: | http://www.ilo.org/ipec/programme/lang-- en/index.htm Elimination of Child Labour English | Retrieved on 05/07/2024 | Publicly Available |
| 22 | Website: Argument: Language: | http://www.ilo.org/ipec/Regionsandcountries/lang-- en/index.htm Regions and Countries English | Retrieved on 05/07/2024 | Publicly Available |
| 23 | Website: Argument: Language: | http://www.ilo.org/dyn/normlex/en/f?p=1000:11200:0::NO::P11200_COUNTRY_ID:102893 Regions and Countries English | Retrieved on 05/07/2024 | Publicly Available |
| 24 | Website: Argument: Language: | https://www.ilo.org/dyn/normlex/en/f?p=NORMLEXPUB:12100:0::NO::P12100_ILO_CODE:C155 Occupational Safety and Health Convention English | Retrieved on 05/07/2024 | Publicly Available |
| 25 | Re-carbon | Validation report N° 659 | Version 0.3 of 12/12/2022 | Project Participant |
| 26 | Verra | https://registry.verra.org/app/search/VCS/All%20Projects, Argument: VERRA Database | Retrieved on 05/07/2024 | Other |
| 27 | GCC | https://projects.globalcarboncouncil.com/pages/submitted_projects | Retrieved on 05/07/2024 | Other |
| 28 | CDM Executive Board | Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation | version 03.0 of 22/09/2017 | Publicly Available |
| 29 | CDM Executive Board | Tool to determine the remaining lifetime of equipment | version 01.0 of 16/10/2009 | Publicly Available |
| 30 | Gold Standard Foundation | GS Validation and Verification Standard for Project Activities | Version 2.0 of 12/11/2024 | Publicly Available |
| 31 | Gold Standard Foundation | SDG Impact Tool | Version 0.2 of 11/06/2025 Version 0.1 of 11/02/2025 | Publicly Available |
| 32 | Sekans Danışmanlık | Carbon Revenues | Version 0.1 of 12/08/2025 | Project Participant |
| 33 | Sekans Danışmanlık | Carbon Sales Revenues | 22/04/2019 | Project Participant |

- Clarification requests, corrective action requests and forward action requests

Table 1.	Remaining FAR from previous verification

| FAR ID | 1 | Section no. |  | Date: |
| --- | --- | --- | --- | --- |
| Description of FAR |  |  |  |  |
| Project participant response |  |  |  | Date: |
| Documentation provided by project participant |  |  |  |  |
| VVB assessment |  |  |  | Date: |

Table 2.	CR from this validation

| CR ID | 1 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CR |  |  |  |  |
| 1- Cover page date of design certification: Also define for CP2 design certification date. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| Cover page has been revised. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CR ID | 2 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CR |  |  |  |  |
| 1- Section B.6.2 SDG 13 and Section B.7 SDG 13 source of data please add previous PD for BM reference. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| Previous PD as reference for BM has been included in the PDD. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CR ID | 3 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CR |  |  |  |  |
| 1- Section B.7 SDG 7 details for electricity meters. - It may be better to delete old meters. - - Please only share last test. 02/11/2020 date is calibration date not test date. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| - Old meters info has been deleted. Only the test date of the meters has been left in Section B.7. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CR ID | 4 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CR |  |  |  |  |
| 1- Principle D.1 please share appointment person’s title or name. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| Section D.1 has been revised. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CR ID | 5 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CR |  |  |  |  |
| 1- In PD it is stated that carbon revenue is the %2 of electricity revenue. Please share carbon and electricity sales revenue with supporting evidences. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| The ratio of the carbon and electricity sales revenue has been provided to the VVB. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The ratio of the carbon and electricity sales revenue has been provided to the VVB. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

Table 3.	CAR from this validation

| CAR ID | 1 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CAR |  |  |  |  |
| 1- Table 1 estimated electricity generation value please share the reference for 121,939.00 MWh and supporting. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| The reference has been included. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CAR ID | 2 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CAR |  |  |  |  |
| 1- Section A.2 Please check and control project location : The Balikesir Susurluk 45 MW Wind Farm Project is located near Ömerköy and Demirkapı Villages, Susurluk District of Amasya Provience, in Marmara Region of Western Turkey. 2- Section A.2 project coordinates it is not clear why not claimed-ignored wind turbined coordinated have been shared. 3- Section E.1 first para please control the statement of Üvecik Village and Mahmudiye Town. 4- Section E.1 last para Please check and control Mahmudiye Village Mukhtar (Village Head |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| - Section A.2 has been revised. - Section A.2 has been revised. - Section E.1 has been revised. - Section E.1 has been revised. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| 1,2,3,4- The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

| CAR ID | 3 | Section no. |  | Date: 10/02/2025 |
| --- | --- | --- | --- | --- |
| Description of CAR |  |  |  |  |
| 1- Section B.2 Table 7 please clarify Tool 11: Since CP Renewal of the project activity is executed, this tool has been used. Please see section B.4. Please clarify the statement. |  |  |  |  |
| Project participant response |  |  |  | Date: 10/02/2025 |
| The Section name has been revised from B.4. to B.5. |  |  |  |  |
| Documentation provided by project participant |  |  |  |  |
| The PDD has been revised. |  |  |  |  |
| VVB assessment |  |  |  | Date: 11/02/2025 |
| Closed. |  |  |  |  |

Table 4.	FAR from this validation

| FAR ID | 1 | Section No. |  | Date: |
| --- | --- | --- | --- | --- |
| Description of FAR |  |  |  |  |
| It is stated that the periodic inspection and testing of the meters should be done every 2 years. However, latest periodic control of the electricity meter had done on 07/03/2022. PP should conduct periodic tests for electricity meters and during next verification test records will be examined. If test has not been conducted on time max deviation will be applied. |  |  |  |  |
| Project participant response |  |  |  | Date: |
| Documentation provided by project participant |  |  |  |  |
| VVB assessment |  |  |  | Date: |

| FAR ID | 2 | Section No. |  | Date: |
| --- | --- | --- | --- | --- |
| Description of FAR |  |  |  |  |
| Crediting Period 2 end date is 12-02-2025 therefore, Project Developer shall not request issuance from 12-02-2025 to 18-02-2025 due to delayed request submission and VVB shall validate the corrective action in line with para 5.1.46 PRINCIPLES AND REQUIREMENTS, Version 2.1. i.e., Delay in the completion of re-validation beyond the last date of current certification cycle shall result in a reduction of any issuance of Certified Products and/or Impact Statements available during following certification cycle (for example, a delay of 1 year beyond the first cycle shall mean that no Certified Impact Statements shall be issued for the period of delay). |  |  |  |  |
| Project participant response |  |  |  | Date: |
| Documentation provided by project participant |  |  |  |  |
| VVB assessment |  |  |  | Date: |

Document information

| Version | Date | Description |
| --- | --- | --- |
| - 01.0 | - 10/06/2016 | - Initial publication. |