# Key Project Information & VPA Design Document (PDD)

###### PUBLICATION DATE  7.10.2020
VERSION  v. 1.1 
RELATED SUPPORT   
- TEMPLATE GUIDE Key Project Information & VPA Design Document v.1.1

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Summary of Local stakeholder consultation

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of VPA Implementer (mandatory)

Appendix 3 - Summary of Approved Design Changes (project specific)

### KEY PROJECT INFORMATION

| - GS ID of Project | GS4236 |
| --- | --- |
| - Title of Project | PoA GS2747: African Biogas Carbon Programme (ABC) - Kenya – VPA03 |
| - Time of First Submission Date | 30/04/2015 |
| - Date of Design Certification | 19/04/2017 |
| - Version number of the VPA-DD | 2.6 |
| - Completion date of version | 05/08/2024 |
| - Coordinating/managing entity | ABPL |
| - VPA Implementer (s) | Biogas Solutions Uganda Ltd |
| - Project Participants and any communities involved | N/A |
| Host Country (ies) | Uganda |
| GS ID and Title of applicable Design Certified VPA | N/A |
| GS ID and Title of applicable Performance Certified VPA | N/A |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | N/A |
| Methodology (ies) applied and version number | Methodology for animal manure management and biogas use for thermal energy generation V1.1 |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

##### Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## 13 Climate Action (mandatory) | ## Amount of GHGs emissions avoided or sequestered | ## 10,376 | ## tCO2e |
| ## 1 No Poverty | ## Average household savings in expenditure on basic services such cooking | ## 3,636,776 | ## kUGX/year |
|  |  | ## 909,194 | ## USD/year |
| ## 2. Zero hunger | ## Number of farmers adopted practices promoted by the project | ## 6,687 | ## Number |
|  | ## Area under sustainable agriculture | ## 23,230 | ## Hectare |
| ## 3 Good Health and Wellbeing | ## Number of households that observed reduction in PM2.5 & carbon monoxide (CO) concentration reductions | ## 5,636 | ## Number |
| ## 4. Quality education | Number of employees provided skill development training | ## 93 | ## Number |
| ## 5. Gender equality | ## Average time saving associated with cooking time and fuel collection | ## 3,665,210 | ## Hours/year |
| ## 7. Affordable and clean energy | ## Number of beneficiaries: Individuals | ## 48,510 | ## Number |
|  | ## Number of beneficiaries: households | ## 6,812 | ## Number |
| ## 8. Decent work and economic growth | ## Total number of jobs | ## 526 | ## Number of jobs |
| ## 9. Industry, innovation and infrastructure | ## Total number of companies supported for their integration into value chains and markets | ## 95 | ## Number of companies |
| ## 12. Responsible consumption and production | ## Compost produced and used for sustainable and local agriculture | ## 194,351 | ## Ton/year |
| ## 15. Life on land | ## Total non-renewable wood fuel saved | ## 44,243 | ## Ton/year |

DESCRIPTION OF PROJECT

Purpose and general description of project

The overall objective of the VPA is to contribute to the achievement of the Sustainable Development Goals (SDGs) through the dissemination of domestic biogas systems as a local, sustainable energy source and the development of a commercially viable, market-oriented biogas sector. The goal of the programme is to improve the livelihoods and quality of life of rural and peri-urban farmers in Uganda through utilizing the market and non-market benefits of domestic biogas.

Around 8,262 domestic biogas systems are installed as per 18/04/2023 throughout Uganda. The usage rate is around 63.2% as per the latest survey, which was implemented in July 2023, and thus around 5,223 units are in operation. Unit sizes vary from 4 to 60 m3 volume, the average size is 7.57 m3, and most are either sized 6 m3 (60.6%) and 9 m3 (22.3%).

The baseline scenario is using non-renewable biomass (NRB) and small quantities of fossil fuel for cooking and methane emissions from manure management systems. The project scenario is renewable biogas for cooking and avoiding methane emissions by capturing and utilizing biogas for cooking in a biodigester instead of NRB or fossil fuels.

Biogas Solutions Uganda Ltd (BSUL), founded in 2014, is the entity with the responsibility of coordinating, facilitating, and monitoring sector functions and supporting the technical, financial and institutional architecture necessary for development of the domestic biogas sector in Uganda under this VPA.

Eligibility of the VPA under approved PoA

- >>

Table 2 Eligibility for VPA inclusion as per PoA requirements

| No. | Eligibility Criterion | Description/ Required condition | Means of Verification/Supporting evidence for inclusion |
| --- | --- | --- | --- |
| (i) | The geographical boundary of the VPA including any time-induced boundary consistent with the geographical boundary set in the PoA | All biogas systems included in the VPA will demonstrate they fall within the geographical boundary of the PoA through: - Recording the address/location of the system in Salesforce or equivalent software - Warranty or household sales agreement | Project database with address and location is provided. Contractual agreement between CME and VPA implementer is provided (confidential document) Warranty or household sales contract of the first digester with plant code BSU/46 is provided |
| (ii) | Conditions to avoid double counting of GHG emission reductions or net anthropogenic GHG removals, such as unique identifications of product and end-user locations | Double counting is avoided by unique biodigester identification and as per methodology procedure indicator BGTA 1 and 2 | Project database with unique codes is provided. - Other responses are captured in parameter BGTA 1 and 2 in Section B.6.2 |
| (iii) | Conditions to confirm that V/CPAs are neither registered as project activities with other offset Schemes, included in other registered PoAs, nor the project activities that have been deregistered; | The required condition is that the biodigester is only registered under this project | - captured in parameter BGTA 2 in Section B.6.2 |
| (iv) | Specification of the technology/measure such as the level and type of service, as well as performance specification based on, inter alia, testing/certification; | Demonstration that the plant is installed according to specifications by conducting a commission check | Only plants that are commissioned are included, the evidence is the database containing the date of commissioning |
| (v) | Conditions to check the start dates of VPA through documentary evidence | The start date is defined as the first digester commissioned in a VPA | Warranty certificate or sales contract of the first plant commissioned with code BSU/46 is provided |
| (vi) | Conditions to ensure compliance with the applicability of the applied methodologies, the applied standardized baselines and the other applied methodological regulatory document | The VPA shall provide a discussion on the applicability of the applied methodology. | Description as per VPA-DD section B.2 |
| (vii) | Conditions to ensure that VPA meet the requirements for demonstration of additionality | As per CDM Tool 21 Demonstration of additionality of small-scale project activities. Version 13.1 | Additionality is demonstrated in VPA-DD section B.5 |
| (viii) | Conditions to ensure no diversion of official development assistance; | VPAs will demonstrate that any Official Development Assistance received for the VPA has not occurred on the condition that the resulting credits are transferred to the donor country | A signed ODA no diversion letter is uploaded to the registry |
| (ix) | Target group (e.g. domestic/commercial/industrial, rural/urban, grid-connected/offgrid) | The VPA will demonstrate which target group(s) is/are to be targeted by the VPA and the distribution mechanism. | As per table 4 in this VPA-DD, the VPA disseminates a technology developed for small farmers with sufficient manure available for feeding. Farmers typically live in rural areas. |
| (x) | Conditions related to sampling requirements for the PoA | The conditions as stipulated in the applied methodologies | Section B.7.2 of this VPA-DD contains the conditions related to sampling aligned with the applied methodologies |
| (xi) | Conditions to ensure that CPAs that will be included meet the small-scale thresholds and remain within those thresholds throughout the crediting period | The VPA Implementer will ensure that each VPA remains below the small-scale limits. For activities falling under Type I, each VPA in aggregate will remain below 15 MW (45MWth) per year. For activities falling under Type III, each VPA will achieve below 60,000 tCO2e in emission reductions annually. | See section A.4 in this VPA-DD for this demonstration |
| (xii) | Conditions to confirm that technologies in V/CPAs are eligible | Demonstration that only biodigesters are included up to 100 m3 | The ER spreadsheet contains the project database with the size of each plant installed. None of the digesters included are over 100 m3 |
| (xiii) | Conditions to be met by each VPA regarding SDG outcomes assessment | N/A. From 13 March 2022, the SDG Impact Tools are a mandatory part of the project development cycle. This tool has been adopted | SDG impact tool is used to identify SDGs as per GS requirements |

Legal ownership of products generated by the VPA and legal rights to alter use of resources required to service the project

>>

| Relevant point | Justification |
| --- | --- |
| i. full and uncontested legal ownership of all Products that are generated under Gold Standard Certification | The household information form which serves as sales agreement and warranty certificate includes a clause on waiving on carbon rights to BSUL and is signed by the household and the construction company. The relationship between BSUL and the CME (ABPL) with regarding to credit sales and rights is governed by a VPA implementer contract. |
| ii. legal rights concerning changes in use of resources required to service the Project (e.g., water rights) | N/A there is no change in the use of resources |
| iii. full and uncontested legal land title/tenure required to implement the Project (e.g., A/R projects, see LUF Activity Requirements) | N/A there is no change in land tenure required to implement the project |

Location of VPA

>>

The host party of this VPA is Uganda. The co-ordinates of Uganda are represented approximately by: 4°12’53.79” to -1°28’19.22” N, 29°34’17.52” to 35°2’33.81” E. The VPA disseminates biogas systems throughout the entire territory of Uganda.

Figure 1: Geographical boundary of Uganda and the VPA

Physical address BSUL office:

The VPA implementer is the BSUL. The physical location of their office is: Plot 36, Luthuli rise, Bugolobi, Kampala, Uganda.

Technologies and/or measures

>>

Biogas Solutions Uganda Ltd selected the appropriate biogas technology to be implemented through engagement with a wide range of stakeholders. They agreed on a fixed dome digester - a drawing of this model is shown in Figure 2. However, the VPA recognizes that in certain geographical regions availability of construction material may be limited, allowing for the inclusion of other types of biodigesters of similar capacities.

Figure 2: Fixed dome biogas system technical drawing

i. List of equipment to be installed.

The biogas system is made up of several interconnected parts. The specific role of each component is summarized below:

- Inlet – The main purpose of the inlet is to mix organic material and water into a semi solid state. This mixture is fed into the digester via an inlet pipe.

- Digester – The digester holds the mixture of manure and water, creating a conducive environment for anaerobic digestion where microorganisms produce biogas. The digester is cylindrical in shape and is usually made of brick masonry with a concave concrete cover, or dome. Typically, the digester is built underground with only the plumbing, inlet and outlets visible.

- Dome - The purpose of the dome is to collect the gas produced in the digester. This is typically plastered in several layers and painted with a special paint in order to minimize gas leakage. Gas accumulates under the dome creating pressure and pushing down the level of the slurry and increasing the slurry level in the connected slurry tank. It is the difference in slurry levels between the slurry tank and the inside of the dome that maintains the pressure to push the gas into the outlet pipe.

- Outlet - The outlet valve releases the collected gas under the dome to biogas appliances such as stoves or lamps.

- Slurry Tank - The slurry tank holds the slurry that the gas pressure from under the dome displaces. This slurry overflows into a composting tank as more manure is fed into the digester. This slurry can then be used as a fertiliser.

ii. The types and levels of services

Biodigesters will provide gas for cooking and bio-slurry for agricultural application. The type of services it provides, for the family, human health, the planet, economy, etc, is summarized in the figure below.

iii. The arrangement of facilities systems and equipment

Biodigesters are typically installed at the backyard of households, but depending on the space availability, could also be installed next, in front or even under the house/stable. The arrangement of the various elements and their functions is shown in the figure below:

iv. The age and average lifetime of the equipment based on the manufacturer’s specifications and industry standards

The expected technical lifespan of a good quality fixed dome digester is over 21 years. This is as per specifications of the program, which developed the technology along with the SNV Dutch Development Organisation. The model is based on older Indian and Chinese digester models, namely, the Deenbandhu and the Chinese dome digester, which have a lifespan of 25 to 35 years.

V. The installed capacities, load factors and efficiencies

The technical specifications of the most commonly installed digesters are:

Table 3: Technical specification of the digesters

| Specifics | Unit | 4m3 | 6m3 | 9m3 | 12m3 |
| --- | --- | --- | --- | --- | --- |
| Plant Volume | litre | 3,900 | 5,850 | 8,775 | 13,163 |
| Gas Storage Volume | litre | 900 | 1,350 | 2,025 | 3,038 |
| Digester Volume | litre | 3,000 | 4,500 | 6,750 | 10,125 |
| Min. Feeding | Kg/day | 25 | 38 | 56 | 84 |
| Max. Feeding | Kg/day | 38 | 56 | 84 | 127 |
| Min. daily gas production | m3/day | 1.00 | 1.50 | 2.25 | 3.38 |
| Max. daily gas production | m3/day | 1.50 | 2.25 | 3.38 | 5.06 |
| Average daily feeding | Kg/day | 31 | 47 | 70 | 105 |
| Average gas production | m3/day | 1.25 | 1.88 | 2.81 | 4.22 |

The average thermal capacity of digesters installed is 3.39 kWhth, and around 13,285 units in operation can be installed before reaching the small-scale thermal threshold of 45 MWhth.

The type III threshold of 60,000 tCO2 emission reduction is reached after installed an estimated 37,758 units.

vi. provide ranges in cases there is variation in equipment features.

In this VPA fixed dome digesters are installed. In case during implementation different models are installed, the variation in equipment features will be reported in the monitoring report.

Scale of the VPA

>>

This project is a small-scale project and has the following thresholds:

- Type I: the cumulative maximum output capacity of this VPA is 45 MWth
Type III: The GHG emission reductions from methane avoidance from shall not exceed 60,000-ton CO2/year

As per GS optional requirement ‘GHG emissions reductions & sequestration product requirements’ v2.1’

Funding sources of VPA

- >>

DGIS (Directorate General Internationale Samenwerking) of the Dutch Government through RVO (Netherlands Enterprise agency) is currently funding the project-Africa Biodigester Component (ABC) I Kenya and Uganda. This project started in 2022 as a build-up of ABPP (through which public funding was provided until 2019).

Financing provided in connection with the PoA or its VPAs has come from or will come from ODA that has not been or will not be provided under the condition, whether express of implied, that any or all of the carbon credits issued as a result of the project’s operation will be transferred directly or indirectly to the country of origin of the ODA. An ODA declaration is signed and uploaded to the registry.

APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

Reference of approved methodology (ies)

>>

Methodology for animal manure management and biogas use for thermal energy generation V1.1. URL: https://globalgoals.goldstandard.org/433-ee-ics-methodology-for-animal-waste-managment-and-biogas-application/

The above methodology replaced the methodology applied in CPI, which is TPDDTEC (Reduced Emissions From Cooking And Heating – Technologies And Practices To Displace Decentralised Thermal Energy Consumption) (v1.0) was applied. That methodology in its current version excludes biogas projects and therefore it was necessary to switch the methodology.

The methodology refers to the following tools:

- CDM tool 21: Demonstration of additionality of small-scale project activities. Version 0.90

- CDM tool 30: Calculation of the fraction of non-renewable biomass. Version 4.0

Applicability of methodology (ies)

>>

| Ref | Condition | Applicability |
| --- | --- | --- |
| 2.2.1 | This methodology applies only to the fraction of the manure which would decay anaerobically in the absence of the project activity, which is established by a survey | The ER calculations are based on fraction of manure that would decay anaerobically in absence of the project activity and this is captured in parameter BGTA 12 and BGTA 8 |
| 2.2.2 | The methodology offers two methods for baseline emission quantification from AWMS. - AWMS method 1 - IPCC Tier 1 approach, - AWMS method 2 - IPCC Tier 2 approach, | This VPA will apply Tier 1 for household farms |
| 2.2.3 | The methodology is applicable under the following conditions when applying AWMS method 1: - The category is limited to measures at individual households, small farms (e.g., installation of a domestic biogas digester) or livestock farms or institutional settings. - The activity shall ensure that: - The digestate must be handled aerobically. In soil application of the final digestate, proper conditions and procedures (resulting in negligible methane emissions) must be ensured. - The biogas captured from the biodigesters is utilised (e.g., combusted or burnt for thermal applications). | a) biodigester are installed at individual households, small farms (e.g., installation of a domestic biogas digester) or livestock farms or institutional settings b) The activity ensures that: - Digestate is a potent organic fertilizer and used by the vast majority of the households as fertilizer. Households are encouraged to compost the digestate in case storage is necessary. Both systems are aerobic, see parameter BGTA 32 where this is monitored. - All biodigesters sold include a biogas stove to ensure effective and efficient use of gas for cooking |
| 2.2.4 | The methodology is applicable under the following conditions when applying AWMS method 2: - The livestock population in the farm is managed fully or partly under confined conditions; - Manure or the streams obtained after treatment are not discharged into natural water resources (e.g., river or estuaries); - The annual average temperature of baseline site where anaerobic manure treatment facility is located is higher than 5°C; - In the baseline scenario, the retention time of manure waste in the anaerobic treatment system is greater than one month, and if anaerobic lagoons are used in the baseline, their depths are at least 1 m; - The baseline scenario should not involve methane recovery and destruction by flaring or combustion for gainful use. - The storage time of the manure after removal from the animal barns, including transportation, should not exceed 45 days before being fed into the anaerobic digester. If the project developer can demonstrate that the dry matter content of the manure when removed from the animal barns is larger than 20%, this time constraint will not apply. - A technical measure to ensure that the gas holding capacity of the biodigester is sufficiently large to capture the biogas during periods of non-usage. A justification to demonstrate compliance with this requirement pertaining to the biogas digester size shall be included in the Project Design Documentation (PDD). | Not applicable as this VPA applies Tier 1 for household scale digesters. |
| 2..5 | The activity is implemented by a project developer and can include additional project participants listed in Appendix 2 of the PDD template. The individual households may be represented collectively by community organisations, etc., but do not individually act as project participants | The VPA is implemented by BSUL, the project developer. There are no additional participants. |
| 2.2.6 | The developer must design incentive mechanism(s), which should be effective as fast as possible, for the displacing the use of inefficient baseline stoves or cooking practices by the project cooking devices for daily usage and describe the incentive mechanism(s) in the PDD/VPA-DD at the time of validation | The VPA implementer has set up an elaborative system to ensure that biodigesters are used effectively. This includes: - Households trainings on biogas and bio-slurry use -Warranty of 1 year for technical issues (if identified, these are repaid at cost) - 100% of newly constructed plants are called by the client support center to ensure biodigesters are installed properly - Mandatory after sales physical visits by biodigester construction company to rectify potential issues and ensure proper use of biogas and bio-slurry - Grievance mechanism has been set-up for households to report issues. |
| 2.2.7 | To avoid double counting or double claiming, the project developer must: - clearly communicate its ownership rights and intention of claiming the emission reductions resulting from the project activity to the following parties by contract or clear written assertions in the transaction paperwork: all other project participants; project technology manufacturers; and retailers of the project technology; and - inform and notify the end users that they cannot claim emission reductions from the project, and - exclude from the project activity, any biodigester and cookstoves that are included in any other voluntary market or CDM or Article 6 based mechanisms project activity/PoA and strive not to displace the cooking devices of another CDM or voluntary project/PoA. See data and parameters not monitored (section 3.11), Avoidance of double counting or double claiming with other mitigation actions (BGTA 2), for details on this demonstration. | a. Captured in parameter BGTA1. b. End users of a biodigester are informed and notified that they cannot claim emission reductions. This is a standard clause in the warranty certificate. The certificates are retained at the household and digital copies are available in the data management system used called Salesforce. c. Captured in parameter BGTA 2 |

VPA boundary

>>

| Source |  | GHG | Included ? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | Animal Waste Management System | CO2 | No | CO2 emissions from the decomposition of organic waste are not included. |
|  |  | CH4 | Yes | Major source of emissions. |
|  |  | N2O | No | This baseline emission source could be excluded for simplification and conservative as per methodology statement |
|  | Delivery of thermal energy | CO2 | Yes | Important source of emissions. |
|  |  | CH4 | Yes | Important source of emissions, unless justified negligible. |
|  |  | N2O | Yes | Can be significant for some fuels, unless justified negligible. |
|  | Production of fuel, transport of fuel | CO2 | no | Excluded as because Tier 1 is applied |
|  |  | CH4 | no | Excluded as because Tier 1 is applied |
|  |  | N2O | No | Excluded for simplification; conservative. |
| Project scenario | Animal waste management system | CO2 | No | CO2 emissions from the decomposition of organic waste are not included. |
|  |  | CH4 | Yes | Emissions from physical leakage, as well as emissions from the animal waste not treated in the bio-digester. |
|  |  | N2O | No | Excluded as a biodigester does not produce N2O gasses. |
|  | Delivery of thermal energy | CO2 | Yes | Important source of emissions. |
|  |  | CH4 | Yes | Important source of emissions. |
|  |  | N2O | Yes | Can be significant for some fuels. |
|  | Production of fuel, transport of fuel | CO2 | no | Excluded as production of fuel and transport does not take place and the VPA applies Tier 1 approach |
|  |  | CH4 | no | Excluded as production of fuel and transport does not take place and the VPA applies Tier 1 approach |
|  |  | N2O | NO | Excluded for simplification. |

Establishment and description of baseline scenario

>>

Animal waste management: The baseline scenario is the situation where, in the absence of the project activity, animal manure is left to decay anaerobically within the project boundary and methane is emitted to the atmosphere as defined by the methodology.

Thermal application: The baseline is the fuel consumption of the thermal application used or that would have been used in the absence of the project activity times an emission factor for the fossil fuel and non-renewable biomass displaced.

The baseline scenario of both pathways has been defined with a baseline survey amongst a representative sample of the population which have the technical potential to install a biodigester, see B.7.2. for details of the survey design.

The results, the list of facilities, systems and equipment is captured in BGTA 6. A succinct summary of the key findings are (sources are provided in BGTA 6):

- The baseline technologies serve 7.91 people on average for cooking

- Most people prepare breakfast, dinner and lunch, but only 63% boil water with the baseline technology.

- Wood is the predominant cooking fuel

- Three stone stove is the most used stove.

- Most wood is collected.

- Seasonal variation in fuel use is negligible

- The amount of waste that would decay anaerobically was estimated using the IPCC 2006 guidelines. For the first verification this will be reassessed using the IPCC 2019 revision to the 2006 guidelines as per applied methodology.

Demonstration of additionality

>>

As per Principles and Requirements section 5.1.47 it is not required to reassess the demonstration of additionality for renewal. Additionality was demonstrated during the initial project design validation/registration for CPI and further demonstration of additionality is not required during project design renewal.

>>

Prior Consideration

>>

Prior consideration was demonstrated during the initial project design validation/registration for CPI, see VPA03-DD of CPI section D.5 item 15.

Ongoing Financial Need

>>

N/A as per GS template guide VPA design document, ongoing financial has to be demonstrated by projects that are required to demonstrate financial additionality. This project did not have to demonstrate financial additionality during the initial project design validation/registration for CPI.

Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| Sustainable Development Goals Targeted | Most relevant SDG Target | SDG Impact |
| --- | --- | --- |
|  |  | Indicator (Selected in SDG tool) |
| 13 Climate Action (mandatory) | 13.2 Integrate climate change measures into national policies, strategies and planning | Amount of GHGs emissions avoided or sequestered |
| 1 No Poverty | 1.1 By 2030, eradicate extreme poverty for all people everywhere, currently measured as people living on less than $1.25 a day | Average household savings in expenditure on basic services such cooking |
| 2. Zero hunger | 2.4 By 2030, ensure sustainable food production systems and implement resilient agricultural practices that increase productivity and production, that help maintain ecosystems, that strengthen capacity for adaptation to climate change, extreme weather, drought, flooding and other disasters and that progressively improve land and soil quality | Number of farmers adopted practices promoted by the project |
|  |  | Area under sustainable agriculture |
| 3 Good Health and Wellbeing | 3.9 By 2030, substantially reduce the number of deaths and illnesses from hazardous chemicals and air, water and soil pollution and contamination | Number of households that observed reduction in PM2.5 & carbon monoxide (CO) concentration reductions |
| 4. Quality education | 4.4 By 2030, substantially increase the number of youth and adults who have relevant skills, including technical and vocational skills, for employment, decent jobs and entrepreneurship | Number of employees provided skill development training |
| 5. Gender equality | 5.4 Recognize and value unpaid care and domestic work through the provision of public services, infrastructure and social protection policies and the promotion of shared responsibility within the household and the family as nationally appropriate | Average time saving associated with cooking time and fuel collection |
| 7. Affordable and clean energy | 7.1 By 2030, ensure universal access to affordable, reliable and modern energy services | Number of beneficiaries: Individuals and households |
| 8. Decent work and economic growth | 8.5 By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value | Total number of jobs |
| 9. Industry, innovation and infrastructure | 9.3 Increase the access of small-scale industrial and other enterprises, in particular in developing countries, to financial services, including affordable credit, and their integration into value chains and markets | Total number of companies supported for their integration into value chains and markets |
| 12. Responsible consumption and production | 12.5 By 2030, substantially reduce waste generation through prevention, reduction, recycling and reuse | Bio-slurry produced and used for sustainable agriculture |
| 15. Life on land | 15.1 By 2020, ensure the conservation, restoration and sustainable use of terrestrial and inland freshwater ecosystems and their services, in particular forests, wetlands, mountains and drylands, in line with obligations under international agreements | Total amount non-renewable wood fuel saved |

Explanation of methodological choices/approaches for estimating the SDG Impact

SDG 13

This is achieved through the roll-out of a national programme that promotes the generation of GHG emission reductions due to the displacement of fossil fuels and non-renewable biomass and improved manure management practices. The parameter monitored are verified emission reductions (VERs).

GHG emission reductions are calculated as the difference between the baseline emissions and the project emissions. This project includes two sources of emission reduction:

- Avoidance of methane emissions from AWMS.

- Displacement of non-renewable biomass and fossil fuels

a. Baseline emissions AWMS

The applied methodology offers two methods for baseline emission quantification from AWMS:

- AWMS method 1 - IPCC Tier 1 approach

- AWMS method 2 - IPCC Tier 2 approach,

In this VPA IPCC Tier 1 approach is applied as the target is household farms.

AWMS method 1 requires livestock population data by animal species/category, climate region or temperature and data on baseline animal manure management practice in the project boundary. Baseline emissions for each production system (low productivity systems) are determined as follows:

| BEAWMS,y=Nb,p,y÷365×GWPCH4×UFb×Up,y×(NLT,y×VSLT,y×MS%Bl,j×EFLT,y)÷1000 | Eq. 1 |
| --- | --- |

Where:

| BEAWMS,y | = | Baseline emissions in year y (t CO2e) |
| --- | --- | --- |
| Nb,p,y | = | Number of project technology-days included in the project database for each project scenario in year y |
| GWPCH4 | = | Global Warming Potential (GWP) of CH4 applicable to the crediting period |
| UFb |  | Model correction factor to account for model uncertainties (0.89) |
| Up,y | = | Usage rate for technologies in project scenario p in year y (fraction). |
| LT | = | Index for all types of livestock |
| j | = | Index for animal waste management system |
| NLT,y | = | Annual average number of animals of type LT in year y (numbers) |
| VSLT,y | = | Volatile solids production/excretion per animal of livestock LT in year y (on a dry matter weight basis, kg-dm/animal/year) |
| MS%Bl,j | = | Fraction of animal manure handled in baseline animal manure management system j. (%). |
| EFLT,y | = | Emission factor for direct CH4 emissions by livestock LT, in manure management system j. |

As per applied methodology, for Tier 1, it is allowed to use survey methods to determine MS%bl,j and the annual average animal population (NLT,y). The annual average number of animals (NLT,y) is determined as follows:

| NLT,y=Nda,y×Np,y365 | Eq. 2 |
| --- | --- |

Where:

| Nda,y | = | Number of days animal is alive in the farm in the year y (numbers) |
| --- | --- | --- |
| Np,y | = | Number of animals produced annually of type LT for the year y (numbers) |

Volatile solids (VS) are the organic material in livestock manure and consist of both biodegradable and non-biodegradable fractions. The parameter VSLT,y shall be calculated as per the equation below:

| VSLT,y=(VSrate, LT ×TAMLT1000)×ndy | Eq. 3 |
| --- | --- |

Where:

| VSLT,y | = | Annual volatile solid excretions for livestock LT entering all animal waste management systems on a dry matter weight basis (kg-dm/animal/yr) |
| --- | --- | --- |
| VSrate, LT | = | VS excretion rate (kg VS / (1000 kg animal mass) / day) |
| TAMLT | = | Typical animal mass for livestock LT (kg/animal). |
| ndy | = | Number of days that the animal manure management system was operational in year y |

As per applied methodology paragraph 3.4.1, only the fraction of manure that would decay anaerobically in the baseline in absence of the project is considered for the baseline calculation. Thus, total VSrate is multiplied with the fraction (parameter BGTA 31):

| MS%i,y | = | Fraction of manure handled in project animal manure management system i |
| --- | --- | --- |

to obtain the fraction of manure that would decay anaerobically in the baseline in absence of the project.

b.	Thermal application method 1: Based on avoided quantity of fuel consumption

Baseline emissions from thermal application in year y shall be calculated as per the methods outlined below. Note that the methods below are only applicable where each unit (e.g., cook stove, heater) have a rated capacity equal to or less than 150 kW thermal.

| BETA,y=b,p(Nb,p,y×Up,y×(fNRB,i,y ×SEb,y,CO2+SEb,y,non-CO2)) | Eq. 4 |
| --- | --- |

Where:

| BETA,y | = | Baseline emissions for total project activity in year y (tCO2e/yr) |
| --- | --- | --- |
| b,p | = | Sum over all relevant baseline b/project p pairs |
| Nb,p,y | = | Number of project technology-days included in the project database for each project scenario in year y. The start date is the day a plant start producing biogas. A default of 2 weeks will be used for the period from installation date and start date. |
| Up,y | = | Usage rate for technologies in project scenario p in year y (fraction) |
| SEb,y,CO2 | = | Specific CO2 emissions for a baseline b technology in year y (tCO2/technology*day) |
| SEb,y,non-CO2 | = | Specific non-CO2 emissions for a baseline b technology in year y (tCO2e/technology*day) |
| fNRB,i,y | = | Fraction of biomass used in year y for baseline scenario b that can be established as non-renewable biomass (fraction). The parameter fNRB,b,y is excluded from this equation when the observed baseline fuel is fossil fuel. |

Specific emission CO2 & non-CO2 are determined by using one of the following options below, for each baseline scenario b /project scenario p pair separately.

| SEb,y,Co2= iPb,i,y×NCVb,i,fuel×EFb,i,CO2 | Eq. 5 |
| --- | --- |

| SEb,y,non-Co2= iPb,i,y×NCVb,fuel×EFb,i,non-CO2 | Eq. 6 |
| --- | --- |

Where:

| i | = | Index for the type of baseline/fossil fuel consumed |
| --- | --- | --- |
| Pb,i,y | = | Average yearly consumption of baseline fuel i per household before the start of the project activity or at the renewal of each crediting period, whichever is later (tonnes/household/day) |
| NCVb,i | = | Net calorific value of the fuel(s) i that is substituted in baseline b (TJ/tonne) |
| EFb,i,CO2 | = | CO2 emission factor arising from use of fuels i in baseline scenario (tCO2/TJ) |
| EFb,i,non-CO2 | = | Non-CO2 emission factor arising from use of fuels in baseline scenario (tCO2e/TJ) |

Pb,i,y will be determined via KPT survey (parameter  BGTA 20)

The fNRB is calculated as per Tool 30 v4.0 guidance using the following calculations. Section B.6.2 presents the outcome and references the data used.

###### b. Thermal application method 2: Based on thermal energy generated using measured quantity of biogas

The baseline emissions are calculated by multiplying the useful energy delivered by the biogas technologies with the emission factor of the baseline devices with their established fuel mix:

| BETA,y= EGp,useful,y× EFb | Eq. 7 |
| --- | --- |

Where:

| BETA,y | = | Baseline emissions (tCO2e) in the year y |
| --- | --- | --- |
| EGp,useful,y | = | The amount of useful energy applied in the project in year y (TJ) |
| EFb | = | Baseline emissions factor (tCO2e per TJ of useful energy) |

The baseline emissions factor is determined applying the equation below:

| EFb= ki,jPb,i,j×Percentage of fuel_i×NCVb,i×(EFb,i,CO2×fNRBi,y+EFb,i, non-CO2k ÷ki,jPb,i,j×Percentage of fuel_i×NCVb,i×ηb,i,jk | Eq. 8 |
| --- | --- |

Where:

| EFb | = | Baseline emissions factor (tCO2e per TJ of useful energy) |
| --- | --- | --- |
| Pb,i,j | = | Amount of baseline fuel i used in device j in the baseline (tonnes) |
| EFb,i,CO2 | = | CO2 Emission factor of the baseline fuel i (tCO2e/TJ) |
| EFb,i,non-CO2 | = | Non-CO2 Emission factor of the baseline fuel i (tCO2e/TJ) |
| fNRBi,y | = | Non-renewability status of woody biomass fuel i during year y |
| NCVb,i |  | The net calorific value of the baseline fuel type i (TJ/tonne) |
| ηb,i,j | = | Efficiency of baseline device j with fuel i (fraction) |
| Percentage of fueli | = | Percentage of fuel type i in the baseline situation (%). |
| k | = | Household k from the target population, where applicable |
| j | = | Baseline devices j |
| i | = | Baseline fuel i |

Useful thermal energy generated in the project activity is calculated as below:

| EGp,useful,y=dNb,p,y÷365×Up,y×Qbiogas,d,y×NCVbiogas×ηp,d,y/1000 | Eq. 9 |
| --- | --- |

Where:

| EGp,useful,y | = | Quantity of useful thermal energy generated by biogas in the project in year y (TJ) |
| --- | --- | --- |
| Qbiogas,d,y | = | The volume of biogas used in the project by device d in year y (m3). |
| NCVbiogas | = | Net calorific value of the biogas (GJ/unit mass or volume, dry basis). Use default value: 0.02154 GJ/m3 biogas (calculating from NCV of the methane: 0.0359 GJ/m3 multiply with default methane content in biogas: 60%) |
| ηp,d,y | = | Thermal efficiency of the project device (fraction) |

In this VPA method 1 is applied. However, the VPA implementer could opt for method 2 during monitoring, when for example, biogas meters become more practical and affordable. This will be justified in the monitoring report.

Project emissions

##### Project emission from AWMS system

###### AWMS method 1 (IPCC 2019 TIER 1 approach)

The project emissions involve emissions from the biodigester, which include:

- Physical leakage biogas

- Emissions from the use of fossil fuels or electricity for the operation

| PEAWMS,y=PEPL,y+PEpower,y | Eq. 10 |
| --- | --- |

Where:

| PEAWMS,y | = | Project emissions in year y (t CO2e) |
| --- | --- | --- |
| PEPL,y | = | Emissions due to physical leakage of biogas in year y (t CO2e) |
| PEpower,y | = | Emissions from the use of fossil fuel or electricity for the operation of the installed facilities in the year y (t CO2e) |

The physical leakage from biodigesters is calculated as 10% of the maximum methane producing potential of the manure fed into the management systems implemented by the project activity, as per the following equation:

| PEPL,y=0.10×Nb,p,y÷365×Up,y×GWPCH4×DCH4×k i,LTB0,LT×NLT,y×VSLT,y×MS%i,y | Eq. 11 |
| --- | --- |

Where:

| PEPL,y | = | Project emissions from physical leakage in year y (t CO2e) |
| --- | --- | --- |
| GWPCH4 | = | Global Warming Potential (GWP) of CH4 applicable to the crediting period. |
| DCH4 | = | CH4 density (0.00067 t/m3 at room temperature (20 ºC) and 1 atm pressure) |
| LT | = | Index for all types of livestock |
| i | = | Index for animal manure management system |
| k | = | Climate region k |
| B0, LT | = | Maximum methane producing potential of the volatile solid generated for animal type LT. |
| MS%i,y | = | Fraction of manure handled by the MS |

Emissions from use of fossil fuels or electricity for operation of the AWMS shall be determined via UNFCCC Methodological tool “Tool 03: Tool to calculate project or leakage CO2 emissions from fossil fuel combustion” and “Tool 05: Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”

However, these tools are not applicable for this VPA as all digesters are manually fed and operate passively (no external source of energy required).

##### B. Project emission from thermal application

###### Thermal application method 1: Based on avoided quantity of fuel consumption

The Project emission from thermal application in year y shall be calculated as follows:

| PEy=b,p(Nb,p,y×Up,y×(fNRB,i,y ×SEp,y,CO2+SEp,y,non-CO2)) | Eq. 12 |
| --- | --- |

Where:

| PEy | = | Project emissions for total project activity in year y (tCO2e/yr) |
| --- | --- | --- |
| b,p | = | Sum over all relevant baseline b/project p pairs |
| Nb,p,y | = | Number of project technology-days included in the project database for each project scenario in year y |
| Up,y | = | Usage rate for technologies in project scenario p in year y (fraction) |
| SEp,y,CO2 | = | Specific CO2 emissions for a project p technology in year y (tCO2/technology*day) |
| SEp,y,non-CO2 | = | Specific non-CO2 emissions for a project p technology in year y (tCO2e/technology*day) |
| fNRB,i,y | = | Fraction of biomass used in year y for project scenario p that can be established as non-renewable biomass (fraction). The parameter fNRB,p,y is excluded from this Eq. 18 of the methodology when the observed project additional fuel is fossil fuel. |

Specific emission CO2 & non-CO2 are determined below:

| SEp,y,Co2= iPp,i,y×NCVp,i,fuel×EFp,i,fuel,CO2 | Eq. 13 |
| --- | --- |
| SEp,y,non-Co2= iPp,i,y×NCVp,fuel×EFp,i,fuel,non-CO2 | Eq. 14 |

Where:

| i | = | Index for the type of project/fossil fuel consumed |
| --- | --- | --- |
| Pp,i,y | = | Average daily consumption of project fuel i per household (tonnes/household/day) |
| NCVp,i,fuel | = | Net calorific value of the fuel(s) i that is used in project p (TJ/tonne) |
| EFp,i,fuel,CO2 | = | CO2 emission factor arising from use of fuels i in project scenario (tCO2e/TJ) |
| EFp,i,fuel,non-CO2 | = | Non-CO2 emission factor arising from use of fuels in baseline scenario (tCO2e/TJ) |

Pp,i,y shall be determined via KPT survey.

###### b. Thermal application method 2: Based on thermal energy generated using measured quantity of biogas

In case the project developer follows Option 2 for thermal application then no project emission is considered for thermal application under the project activity

C. Leakage emissions

a. Animal waste management system

AWMS method 1 The proper soil application (which not resulting in methane emissions) of the digestate shall be verified on a sampling basis following the Requirements in the “Standard for sampling and surveys for CDM project activities and programme of activities”.  This will be part of the monitoring survey and is captured in monitoring parameters BGTA 32. The verification will be part of the SDG monitoring survey.

In addition to this, households may use bio-slurry as animal feed, insecticide, sell it as fertilizer, etc. These options do not result in methane emissions as well.

b. Thermal application

For all of the preceding methods, leakage emissions, LEp,y , shall be determined following one of two options below.

Option 1 - Apply a default adjustment factor of 0.95 to the emission reductions to approximate leakage emissions for thermal application.

Option 2 - The project developer must evaluate the following potential sources of leakage and provide an evidence-based description and preliminary quantification of each potential source and its relevance for the project:

- The displaced baseline technologies are reused outside the project boundary in place of lower emitting technology or with a higher intensity than would have occurred in the absence of the project.

- Members of the population who do not participate in the project, and previously used lower emitting energy sources, instead use the non-renewable biomass or fossil fuels saved under the project activity.

- The project significantly reduces the NRB fraction within an area where other GHG mitigation project activities account for NRB fraction in their baseline scenario.

- The project population compensates for loss of the space heating effect of inefficient technology by adopting some other form of space heating or by retaining some use of inefficient technology.

- By virtue of promotion and marketing of a new technology, the project stimulates substitution with this technology by households who commonly used a technology with relatively lower emissions.

For each source for which the leakage assessment expects an increase in fuel consumption by non-project households/users attributable to the project activity, then calculations must be undertaken to account for the leakage from this source. Leakage is either calculated as a quantitative emissions volume (tCO2e) or as a percentage of total emission reductions. The project documentation shall include a projection of leakage emissions based on available data and information. The monitoring plan must include monitoring parameters to be registered during the leakage investigation every two years to populate the leakage calculation.

Option 1 and 2 is captured in BGTA 43.

D. Emission reductions

The emission reductions for each of the component (AWMS and thermal energy) are calculated separately as follows:

| ERy=BEy-PEy-LEy | Eq. 15 |
| --- | --- |

Where:

| ERy | = | Emission reductions in year y (t CO2e/yr) |
| --- | --- | --- |
| BEy | = | Baseline emissions in year y (t CO2e/yr) |
| PEy | = | Project emissions in year y (t CO2e/yr) |
| LEy | = | Leakage emissions in year y (t CO2e/yr) |

The total emission reduction from project activity will be the sum of emission reductions from each component i.e., AWMS and Thermal energy.

SDG 1: GSDM-I1.1.1: Financial savings

The average household savings will be determined by asking the household have much money is saved on cooking fuel expenditure since using biogas. The savings will be annualized and reported as USD/year and in the local currency per year.

The contribution will then be calculated as:

GSDM-I.1.1.1= SDG1×FOREX×Ubp,y×Nb,p,y                                                      Eq. SDG1

| GSDM-I.1.1.1 |  | Financial savings (UGX or USD saved in the MP) |
| --- | --- | --- |
| SDG1 OM |  | Gross savings (local currency/day) Maintenance cost (local currency/day) |
| Forex |  | Foreign exchange rate at time of MR writing (for expression in USD only) |
| Nb,p,y |  | Technology days included |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |

SDG2: Zero hunger

- GSDM-I2.4.2 Number of farmers adopted the practice by the project

Bio-slurry is promoted as potent organic fertilizer to improve agricultural production and reduce reliance on artificial inputs. The number of farmers using bio-slurry will be obtained by including a question on bio-slurry usage in the SMS. The total contribution is then calculated by multiplying this with the number of biodigesters in use with the following equation:

GSDM-I.2.4.2= Up,y×Nd× BIOu                                          Eq. SDG2

| GSDM-I.2.4.2 |  | Number of farmers using bio-slurry |
| --- | --- | --- |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |
| Nd |  | Number of biodigesters constructed |
| BIOu |  | Bio-slurry usage rate (% farmers using bio-slurry) |

- GSDM-I2.4.3: Area under sustainable agriculture

This will be calculated by multiplying the outcome of GSDM-I.2.4.2 with the land area (Fa) on which bio-slurry is applied. The land area will be obtained by asking the farmer interviewed for the SMS how much land is owned and the share on which bio-slurry is applied.

SDG 3: GSDM-I3.9.1: Reduced incidence of disease caused by air pollutants.

Biogas is a clean fuel and using the gas for cooking instead of biomass will reduce the exposure to harmful pollutants and improve household air quality. The number of households reporting a decrease in harmful pollutants attributed to cooking will be obtained with the following equation:

GSDM-I3.9.1= Up,y×Nd × HHs ×HAPr                            Eq. SDG3

Where

| GSDM-I3.9.1 |  | Number of households reporting a decrease in PM2.5 and CO |
| --- | --- | --- |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |
| Nd |  | Number of biodigesters constructed |
| HHs |  | Average household size |
| HAPr |  | % of families mentioning reduction in household air pollution |

SDG 4: GSDM-I.4.4.1 Number of employees provided skill development training

The number of trainings provided will be determined by summing the number of masons trained during the monitoring period on biogas construction, refresher training and other biogas related trainings. This could also include training to other participants in the biogas value chain (bio-slurry capacity building).

SDG 5: GSDM-I5.4.1: Women empowerment and gender equality

The contribution will be reported as: (1) hours/year saved on fuel wood collection and cooking time (including cleaning) attributed to the installation of a biodigester and (2) usage of saved time. Time savings will be determined as follows:

- The female member of the household in charge of cooking and/or cooking fuel collection, will be asked:

- How much time did you save on cooking and other cooking activities due to cooking on biogas

Time savings will be calculated with this equation:

GSDM-I5.4.1=  Ts ×Nb,p,y×Up,y                          Eq. SDG5

Where

| GSDM-I5.4.1 |  | Hours’ time saved in the MP |
| --- | --- | --- |
| Ts |  | Time savings (hours per day) |
| Nb,p,y |  | Technology days included in the MP |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |

2. The same female member will be asked

- What did you do with your saved time? or if no time savings were reported, the reason why will be asked

Answers will be categorized as much as possible. Categories likely to be considered are I use my time for: productive use/leisure/family/education/social etc.

SDG 7: GSDM-I7.1.1: Increased access to energy

Refers to access to affordable and clean energy (SDG 7.1)

Description: Refers to number of unique households that were provided access to biogas fro cooking

Increased access to clean energy will be calculated with this equation:

GSDM-I7..1.1= Up,y×Nd × HHs Eq. SDG7

Where

| GSDM-I7.1.1 |  | Contribution to SDG 7 in number beneficiaries with increased access to energy |
| --- | --- | --- |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |
| Nd |  | Number of biodigesters constructed |
| HHs |  | Average household size |

The contribution will be calculated both for total number of households and number of individuals. In the latter case, the term ’HHs’ is excluded from the equation.

SDG 8: GSDM-I.8.5.1 total number of jobs

The total number of jobs are defined as the total number of masons provided biodigester jobs. This data will be collected from the project database and constitutes a simple count of the individual masons provided employment.

SDG9: GSDM-I.9.3.1 Total number of companies supported for their integration into value chains and markets

The total number of companies supported are defined as the total number of biodigester construction enterprises (BCE) involved in biodigester construction. This data will be collected from the project database and constitutes a simple count of individual BCE. On top of that, other companies active in the biodigester value chain, can be included, i.e. stove importers or assemblers.

SDG12: GSDM-I.12.5.2 Improved waste management services

This is defined as the total amount of bio-slurry produced by multiplying feeding (kg manure/day) with the total number of digesters in operation with the following equation:

GSDM-I.12..5.2= Fb,p× Nb,p,y  ×Up,y÷1000               Eq. SDG12

| GSDM-I.12.5.2 |  | Contribution to GSDM-I.12.5.2 in ton bio-slurry |
| --- | --- | --- |
| Fp,b |  | Biodigester feeding rate (kg manure/day) |
| Nb,p,y |  | Number of project technology-days included in the project database for each project scenario in year y |
| 1000 |  | Conversion from kg to ton |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |

SDG 15: Parameter: GSDM-1 15.1.1

Total non-renewable wood saved is calculated with the following equation:

GSDM-1 15.1.1= Nb,p,y  ×Up,y×(Pb,wood,y+ Pb,charcoal,y ×CC)- (Pp,wood,y+ Pp,charcoal,y ×CC))

Eq. SDG15

Where

| GSDM-1 15.1.1 | = | Contribution to SDG 15 in tonne/year |
| --- | --- | --- |
| Nb,p,y | = | Number of technology days in included |
| Up,y |  | Usage rate for technologies in project scenario p in year y (fraction) |
| Pb,wood,y | = | Average daily consumption of wood in the baseline scenario per household obtained from the BFT (tonnes/household/day) |
| Pb,charcoal,y | = | Average daily consumption of charcoal in the baseline scenario per household obtained from the BFT (tonnes/household/day) |
| CC | = | Wood to charcoal conversion factor |
| Pp,wood,y | = | Average daily consumption of wood in the project scenario per household obtained from the PFT (tonnes/household/day) |
| Pp,charcoal,y | = | Average daily consumption of charcoal in the project scenario per household obtained from the PFT (tonnes/household/day) |

Thus, the wood and charcoal savings will be multiplied with the number of technology days in a monitoring period that are in operation. See BGTA 7.5 in section B6.2 for the CC value.

Since wood saved in year 1, is also saved in year 2, the total contribution will include the savings of the previous years, e.g., year 1 = savings of month 1 to 12, year 2 is savings of year 1, plus the savings of month 13 to 24, etc.

Data and parameters fixed ex ante

| Data/parameter | BGTA 1: Avoidance of double counting or double claiming among project participants. |
| --- | --- |
| Unit | NA |
| Description | Evidence of avoidance of double counting or double claiming with other parties directly involved with the project or programme |
| Source of data | Warranty certificates |
| Value(s) applied | Warranty certificates require a two-party signature, the technology providers and the households which includes a clause of the ownership rights and intention of selling the emission reductions resulting from the project activity and transfer of those rights to the VPA implementer. |
| Choice of data or Measurement methods and procedures | Warranty certificates or equivalent |
| Purpose of data | Calculation of project outcome |
| Additional comment | In Uganda household forms serve as the warranty certificate |

SDG13

| Data/parameter | BGTA 2: Avoidance of double counting or double claiming with other mitigation actions. |
| --- | --- |
| Unit | NA |
| Description | Review and analysis of mitigation actions in other voluntary market or UNFCCC/compliance mechanisms. |
| Source of data | Applied methodology |
| Value(s) applied | As per methodology, this is undertaken at the time of project design review and VPA inclusion review. Since this VPA has already been included with a validated design, additional review is not required |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | NA |
| Additional comment | Undertake at the time of project design review and VPA inclusion review. |

| Data/parameter | BGTA 3: Regulatory framework for provision of animal waste management and thermal energy services. |
| --- | --- |
| Unit | N/A |
| Description | Evidence that the project does not undermine or conflict with any national, sub-national or local regulations or guidance for thermal energy supply/devices or fuel supply or use. |
| Source of data | https://www.uace.or.ug/Policy/draft-national-energy-policy.pdf |
| Value(s) applied | Draft National Energy policy 2019 Section 4.1.1 #4: Promote biogas production and use for small and large scale thermal and electrical applications # 5. Promote household and institutional biogas and bio-latrine installations 4.2 Clean cooking policy statement: The Government shall promote the uptake and sustained use of clean, modern cooking technologies. This VPA contributes to the implementation of this policy by promoting and installing biodigesters |
| Choice of data or Measurement methods and procedures | N/A |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Undertake at the start of each crediting period. |

| Data/parameter | BGTA 4: Project technology description |
| --- | --- |
| Unit | NA |
| Description | The detailed description of the project technology (including both biodigester and biogas stove) manufacturer name (if applicable), - product name (if applicable), - technology type, - capacity characteristics (e.g., volume of digester), - continuous useful energy output demonstration, - rated thermal efficiency of biogas stove, any performance certifications from national standards body or certification body recognized by national standards body also shall be provided. |
| Source of data | Biodigester manual containing the requested data are shared with VVB (OPERATION AND MAINTENANCE OF BIOGAS PLANTS, BIO-SLURRY MANAGEMENT AND USE) A declaration by the program implementer is added that the biodigester are constructed as per standards defined by the Uganda National Bureau of standards (file shared with VVB: 10Feb 23 BSUL Declaration UNBS signed standard) |
| Value(s) applied | The various technical documents are shared with the VVB |
| Choice of data or Measurement methods and procedures | N/A |
| Purpose of data | Calculation of project outcome |
| Additional comment | N/A |

| Data/parameter | BGTA 5: Expected technical life of project technology. |
| --- | --- |
| Unit | years |
| Description | The expected technical life of an individual project technology |
| Source of data | FAO BEFS Rapid Appraisal – Biogas Community Component p8 https://www.fao.org/3/bp844e/bp844e.pdf |
| Value(s) applied | Fixed dome digesters last at least 20 years. Since the units has no moving parts, and based on the same materials used to construct buildings, that the actual lifespan much longer |
| Choice of data or Measurement methods and procedures | n/a |
| Purpose of data | Calculation of project outcome |
| Additional comment | The expected or remaining technical life of project technology is longer than the crediting period, thus description of measures to ensure that end users are provided replacement technology of comparable service and quality level at the end of the technical life is not required |

| Data/parameter | BGTA 6: Baseline scenario survey results |
| --- | --- |
| Unit | NA |
| Description | Report of the results of the baseline scenario survey implemented in November 2021. |
| Source of data | ER spreadsheet VPA03 CPII SDG database sheet BGTA6 cell B3:E38 |
| Value(s) applied |  |
| Choice of data or Measurement methods and procedures | Household survey amongst 126 randomly selected households with the technical potential to install a biodigester |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Undertake at the start of the first crediting period. The AWMS baseline will be reassessed for the first verification. This is necessary as the current baseline is based on the IPCC 2006 guidelines and the 2019 revision as mandated by the methodology |

| Data/parameter | BGTA 7: GWPCH4 |
| --- | --- |
| Unit | tCO2e per tCH4 |
| Description | Global Warming Potential (GWP) of methane applicable to the crediting period |
| Source of data | IPCC |
| Value(s) applied | 28 using IPCC AR5 report. |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of project scenario Calculation of baseline scenario |
| Additional comment | It shall be updated according to any future COP/MOP decisions. |

| Data/parameter | BGTA 7.5: CC |
| --- | --- |
| Unit | Wood to charcoal conversion factor |
| Description | Amount of wood used to produce charcoal |
| Source of data | CDM tool 33 version 02.0: https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-33-v2.0.pdf |
| Value(s) applied | 4 |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline and project scenario |
| Additional comment | - |

a.		Parameters related to AWMS

| Data/parameter | BGTA 8: MS%Bl,j |
| --- | --- |
| Unit | % |
| Description | Fraction of manure handled in baseline animal manure management system j |
| Source of data | Ex-ante: Baseline survey, see sheet AWMS in VPA03 CPII SDG database spreadsheet cell B175:I184 |
| Value(s) applied | Ex-ante, will be updated for the first verification, see additional comments. Summary; Dairy cow; Other cattle; Growing swine; Breeding swine; Sheep; Goat; Poultry. Anaerobic lagoon; 0%; 0%; 0%; 0%; 0%; 0%; 0%. liquid/slurry; 3%; 2%; 4%; 9%; 0%; 0%; 6%. solid storage; 42%; 18%; 9%; 28%; 0%; 0%; 1%. dry lot; 37%; 17%; 56%; 31%; 14%; 40%; 45%. daily spread; 8%; 7%; 22%; 9%; 26%; 2%; 13%. Pasture; 8%; 54%; 5%; 0%; 46%; 17%; 31%. burned as fuel; 0%; 0%; 0%; 0%; 0%; 0%; 0%. composting passive windrow; 0%; 0%; 3%; 15%; 14%; 40%; 0%. another way; 2%; 2%; 0%; 7%; 0%; 0%; 4%. |
| Choice of data or Measurement methods and procedures | Proper survey methods. |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | The AWMS baseline will be reassessed for the first verification. This is necessary as the current baseline is based on the IPCC 2006 guidelines and the 2019 revision as mandated by the methodology. The results from that survey will be fixed for the whole CP. |

| Data/parameter | BGTA 9: EFLT |
| --- | --- |
| Unit | kgCH4 per animal per year for livestock type LT (kg CH4 / kg VS). |
| Description | Animal manure methane emission factor by average temperature |
| Source of data | IPCC 2019 guidelines volume 4, chapter 10 table 10.14 for low productivity species and montane tropical climate and VPA03 CPII SDG sheet AWMS cell B186:I195 |
| Value(s) applied | Summary; Dairy cow; Other cattle; Growing swine; Breeding swine; Sheep; Goat; Poultry. Uncovered Anaerobic lagoon; 66.20; 66.20; 147.70; 147.70; 66.20; 66.20; 198.60. liquid/slurry pit > 1 month; 51.40; 51.40; 114.60; 114.60; 37.45; 37.45; 154.20. solid storage; 4.40; 4.40; 9.70; 9.70; 4.40; 4.40; 13.10. dry lot; 1.70; 1.70; 3.90; 3.90; 1.70; 1.70; 5.20. daily spread; 0.90; 0.90; 1.90; 1.90; 0.87; 0.87; 1.61. Pasture; 0.41; 0.41; 0.91; 0.91; 0.41; 0.41; 0.76. burned as fuel; 8.70; 8.70; 19.40; 19.40; 8.71; 8.71; 2.60. composting passive windrow; 2.18; 2.18; 4.86; 4.86; 2.18; 2.18; 4.02. another way; 0.90; 0.90; 1.90; 1.90; 0.41; 0.41; 2.40. |
| Choice of data or Measurement methods and procedures | IPCC 2019 guidelines |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | The IPCC does not provide EF defaults for the categories for some values. However, the IPCC mentions: all values are calculated based on MCFs and B0s reported in Tables 10.17 and 10.16a, respectively, using the equation MCF*B0*0.67. Based on this the bottom 4 categories are calculated. This is consistent with Tier 1 and consistent with how the IPCC calculated the values. In the excel sheet the calculated values are in red for transparency The liquid slurry system for sheep and goat is calculated based on an MCF of 43%, it is assumed that manure resides 3 months in those systems before usage. This is an ex-ante estimation and will be re-assessed for the first verification |

| Data/parameter | BGTA 10: VSrate, LT |
| --- | --- |
| Unit | kg VS / (1000 kg animal mass) / day |
| Description | Default Volatile solids production/excretion rate per animal of livestock LT in year y. |
| Source of data | Default values from table 10.13A of chapter ‘Emissions from Livestock and Manure Management’ under the volume ‘Agriculture, Forestry and other Land use’ of the 2019 IPCC Guidelines for National Greenhouse Gas Inventories (kg VS / (1000 kg animal mass) / day) |
| Value(s) applied | Values for the region Africa and low productivity system are applied. Dairy cow; 15.2. Other cattle; 12.7. Swine finishing; 9.40. Swine breeding; 6.00. Sheep; 8.30. Goat; 10.40. Poultry; 13.00. |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline and leakage scenario |
| Additional comment | In the baseline survey swine finished is referred to market swine. Low productivity system is the norm. However in case are high productivity systems, the VS of high productivity systems will be applied |

| Data/parameter | BGTA 10a: VS, LT,y |
| --- | --- |
| Unit | kg VS / animal mass / day |
| Description | Default Volatile solids production/excretion rate per animal of livestock LT in year y. |
| Source of data | VPA03 CPII SDG database sheet VS cell D10, D17, D24, D31, D38, D45 and D52. |
| Value(s) applied | Dairy cow; 4.104. Other cattle; 3.429. Swine finishing; 0.461. Breeding swine; 0.246. Sheep; 0.257. Goat; 0.250. Poultry; 0.012. |
| Choice of data or Measurement methods and procedures | Calculated using default values from table 10.13A of chapter ‘Emissions from Livestock and Manure Management’ under the volume ‘Agriculture, Forestry and other Land use’ of the 2019 IPCC Guidelines for National Greenhouse Gas) for the region Africa and low productivity system are applied. |
| Purpose of data | Calculation of baseline and leakage scenario |
| Additional comment | In the baseline survey swine finished is referred to market swine. |

| Data/parameter | BGTA 11: B0, LT |
| --- | --- |
| Unit | m3 CH4/kg-dm |
| Description | Maximum methane production potential of the volatile solid generated for animal type LT. |
| Source of data | IPCC default values from tables 10.16 of 2019 IPCC Guidelines for National Greenhouse Gas Inventories volume 4 Chapter 10 |
| Value(s) applied | Values for low productivity systems are applied. Dairy cow; 0.13. Other cattle; 0.13. Swine finishing; 0.29. Swine breeding; 0.29. Sheep; 0.13. Goat; 0.13. Poultry; 0.24. |
| Choice of data or Measurement methods and procedures | Literature data |
| Purpose of data | Calculation of baseline and leakage scenario |
| Additional comment | Used to calculate missing EFawms categories as per IPCC 2019 revised guidelines volume 4, chapter 10 table 10.14 |

b.	Parameters related to Thermal application

| Data/parameter | BGTA 13: EFb,i,CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data | Methodology defaults |
| Value(s) applied | Wood: Methodology default, 112 tCO2/TJ Charcoal: The default including charcoal production is applied of 165.22 tCO2/TJ (includes charcoal production emissions) LPG: 63.1 tCO2/TJ Other fuels: IPCC defaults |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline emissions |
| Additional comment | This parameter is used to calculate SEb,y,CO2 Applicable for Thermal application method 1 and Thermal application method 2 |

| Data/parameter | BGTA 14:EFb,i,non-CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non-CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data | IPCC 2019 default data and methodology defaults. LPG non-CO2 emission factor is calculated based on IPCC defaults, see sheet BE in VPA03 CPII SDG database BE cell E95 |
| Value(s) applied | Wood: Methodology default: 9.46 tCO2e/TJ (AR5 GWP) Charcoal: Methodology defaults: - 44.83 tCO2e/TJ (AR5 GWP) (includes charcoal production emissions), LPG - 0.17 tCO2/TJ (AR5 GWP, calculated based on IPCC 2006 guidelines (no 2019 update to the values) |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | This parameter is used to calculate SEb,y,non-CO2 Applicable for Thermal application method 1 and Thermal application method 2 |

| Data/parameter | BGTA 15: EFp,i,CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor arising from use of fuels in project scenario |
| Source of data | Methodology defaults and IPCC 2006, volume 2, chapter 2 table 2.5 for LPG (no 2019 update of the value) |
| Value(s) applied | Wood: Methodology default, 112 tCO2/TJ Charcoal: The default including charcoal production is applied of 165.22 tCO2/TJ (includes charcoal production emissions) LPG: 63.1 tCO2/TJ |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of project emissions |
| Additional comment | This parameter is used to calculate SEp,y,CO2 Applicable for Thermal application method 1 and Thermal application method 2 |

| Data/parameter | BGTA 16:EFp,i,non,CO2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non-CO2 emission factor arising from use of fuels in project scenario |
| Source of data | IPCC 2019 default data and methodology defaults. LPG non-CO2 emission factor is calculated based on IPCC defaults, see sheet AWMS in VPA03 CPII SDG database BE cell E77 |
| Value(s) applied | Wood: Methodology default: - 9.46 tCO2e/TJ (AR5 GWP) Charcoal: Methodology defaults: - 44.83 tCO2e/TJ (AR5 GWP) (includes charcoal production emissions), LPG - 0.17 tCO2/TJ (AR5 GWP, calculated based on IPCC 2006 guidelines (no 2019 update ot the valueas) |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of project scenario |
| Additional comment | This parameter is used to calculate SEp,y,non-CO2 . Applicable for Thermal application method 1 |

| Data/parameter | BGTA 17: NCVb,i |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the baseline |
| Source of data | Methodology default and IPCC 2006, volume 2, chapter 1 table 1.2 for LPG (no 2019 update of the value) |
| Value(s) applied | Wood: Methodology default, 0.0156 TJ/ton Charcoal: Methodology default, 0.0295 TJ/ton LPG 0.0473 TJ/ton |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | The methodology default emission factor must be applied when the methodology default NCV is applied. This parameter is used to calculate SEb,y,CO2 and SEb,y,non-CO2 Applicable for Thermal application method 1 and Thermal application method 2 |

| Data/parameter | BGTA 17: NCVp,i |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the project |
| Source of data | Methodology default and IPCC 2006, volume 2, chapter 1 table 1.2 for LPG (no 2019 update of the value) |
| Value(s) applied | Wood: Methodology default, 0.0156 TJ/ton Charcoal: Methodology default, 0.0295 TJ/ton LPG: 0.0473 TJ/ton |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | The methodology default emission factor must be applied when the methodology default NCV is applied. This parameter is used to calculate SEp,y,CO2 and SEp,y,non-CO2 Applicable for Thermal application method 1 |

| Data/parameter | BGTA 19: NCVbiogas |
| --- | --- |
| Unit | GJ/m3 |
| Description | Net calorific value of the biogas (GJ/unit mass or volume, dry basis) |
| Source of data | Methodology default |
| Value(s) applied | Use default value: 0.0215 GJ/m3 biogas (assuming NCV of the methane: 0.0359 GJ/m3, default methane content in biogas: 60%) |
| Choice of data or Measurement methods and procedures | NA |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Applicable for Thermal application method 2 |

| Data/parameter | BGTA 20: Pb,i,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Average yearly consumption of baseline fuel i per household at the renewal of each crediting period |
| Source of data | KPT, Sheet BE in VPA03 SDG database cell E67, E72 and E77 |
| Value(s) applied | Wood: 3.750 Charcoal 0.294 LPG: 0.00 |
| Choice of data or Measurement methods and procedures | Baseline fuel test |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Applicable for Thermal application method 1 and Thermal application method 2 The value is much lower than the cap value. The average hh size is 7.191 (parameter GSDM-I7.1.1: HHs), and thus the per capita wood use is 3.750/7.191= 0.52 ton/person/year (cap =0.75 ton/person/year ) and for charcoal 0.294/7.191= 0.086 ton/person/year (cap = 0.25 ton/person/year) Note that in the baseline survey the hh size is 7.91 (see BGTA6), but in the first monitoring 7.191 (see GSDM-I7.1.1: HHs). The lowest value is applied which is conservative |

| Data/parameter | BGTA 21: Percentage of fuel_i |
| --- | --- |
| Unit | % |
| Description | Percentage of fuel type i in the baseline situation. |
| Source of data | Sheet BE in VPA03 SDG database cell E55:E56 |
| Value(s) applied | Ex-ante B1: 75%, B2:25% |
| Monitoring frequency | Once per CP. This value will be fixed during the first verification for the whole CP |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | N/A |

| Data/parameter | BGTA 22: ηb,i,j |
| --- | --- |
| Unit | Fraction |
| Description | Efficiency of baseline device j with fuel i (fraction) |
| Source of data | Default values or WBT or literature data |
| Value(s) applied | - Default values: The following default values may be applied for this parameter and for cases when it is not possible to conduct efficiency tests. - Three-stone fire or a conventional system for woody biomass lacking improved combustion air supply mechanism and flue gas ventilation system, that is without either a grate or a chimney: default efficiency 10%. - Other conventional systems using woody biomass: default efficiency 20% - Improved cookstoves: manufacturer specification, or if not available, default efficiency 30% - Fossil fuel combusting system: manufacturer specification, if available. - Sample testing: The baseline stove efficiency may be established based on water boiling test (WBT) conducted on randomly selected sample stoves. The WBT shall be carried out in accordance with national standards (if available) or international standards or guidelines (e.g., the WBT Protocol, or ISO 19867-1 listed by Clean Cooking Alliance and available at: https://cleancookingalliance.org/research-evidence-learning/standards-testing/protocols/) At minimum, a sample test on three stoves with three tests conducted for each baseline stove type by a recognized certification body. - Published third party reference: The baseline stove(s) efficiency may be applied from; - Credible published literature, or - Studies by academia, NGOs or multilateral institutions, - Official government publications or statistics, Source applied must not be more than 3 years old and shall be relevant to the project region and baseline technology. |
| Choice of data or Measurement methods and procedures | Default values or sample testing of third-party references. To be determined during monitoring |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Applicable for Thermal application method 2. |

| Data/parameter | BGTA 23 ηp,d,y |
| --- | --- |
| Unit | % |
| Description | Thermal efficiency of the project device |
| Source of data | Literature data. CPI VPA-DD parameter ηbiogas stove in section D.6.2 |
| Value(s) applied | 55% |
| Choice of data or Measurement methods and procedures | - |
| Purpose of data | Calculation of baseline scenario |
| Additional comment | Applicable for Thermal application method 2. |

| Data / Parameter | BGTA 41: fNRB,i,y |
| --- | --- |
| Unit | percentage |
| Description | Fractional non-renewability status of woody biomass fuel during year y, in case the baseline fuel is biomass or charcoal. |
| Source of data | fNRB Uganda spreadsheet fNRB cell D62 |
| Value(s) applied | 83.78% |
| Choice of data or Measurement methods and procedures | Determined by following the CDM TOOL30, Calculation of the fraction of non-renewable biomass. |
| Purpose of data | Calculation of project outcome |
| Additional comment | fixed at project design certification stage. See other notes in the excel sheet justification of the value against Bailis et al and other literature and comparison with above ground extraction of fuelwood. |

Ex ante estimation of SDG Impact

>>

SDG 13

Accounting for emission reductions due to the avoidance of methane emissions from manure handling.

The baseline emissions from the handling of animal waste are determined using IPCC Tier 1 approach using household surveys to establish MS.

Baseline data was collected of the manure management systems by animal type of target households, taking into consideration hours stabled and seasonality. Animals that are in the field for 6 hours for example, it is assumed that 6/24 of the manure is excreted in the field. If 40% of that manure is collected and composted, then 40% * 6/24 of the manure is composted of the manure excreted in the field. The outcome by animal type is pasted here below:

Table 4: baseline MS by animal T

| Summary | Dairy cow | Other cattle | Growing swine | Breeding swine | Sheep | Goat | Poultry |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anaerobic lagoon | 0% | 0% | 0% | 0% | 0% | 0% | 0% |
| liquid/slurry | 3% | 2% | 4% | 9% | 0% | 0% | 6% |
| solid storage | 42% | 18% | 9% | 28% | 0% | 0% | 1% |
| dry lot | 37% | 17% | 56% | 31% | 14% | 40% | 45% |
| daily spread | 8% | 7% | 22% | 9% | 26% | 2% | 13% |
| Pasture | 8% | 54% | 5% | 0% | 46% | 17% | 31% |
| burned as fuel | 0% | 0% | 0% | 0% | 0% | 0% | 0% |
| composting passive windrow | 0% | 0% | 3% | 15% | 14% | 40% | 0% |
| another way | 2% | 2% | 0% | 7% | 0% | 0% | 4% |

Using equation 1 in this document the obtained MS are subsequently multiplied with IPCC default VS values, animal numbers obtained from the first monitoring survey and the respective emission factors (EFamws), for each animal LT.  and the fraction of manure which would decay anaerobically in absence of the project to obtain BEAWMS. The results are shown in the table below

Table 5: Baseline emissions from animal waste management

The summation of these emissions by animals type as shown in the table, and multiplying this with the GWP of methane, and correction for uncertainties yields the baseline emissions from manure management. This is 1.122 tCO2e/hh/year

Project emissions from AWMS are the emissions from physical leakage (LE) and continued baseline emissions and are calculated using equations 10 and 11 for LE. The results are shown here below: Table 6: Project PE AMWS calculation

The summation of these emissions by animals type as shown in the table, and multiplying this with the GWP of methane and correction for uncertainties yields the project emissions from manure management. This is 2.711 tCO2e/hh/year

b. Fuel switch

Baseline fuel use in the baseline is calculated with equation 4 of this report, and equation 5 and 6 for the specific CO2 and non-CO2 emissions. The baseline emissions results are depicted below.

The total amount of fuel in the population is calculated by calculating the weighted average wood and charcoal consumption, thus by taking into account the relative fraction of each fuel use scenario and the amount consumed.

Equation 1: Baseline fuel consumption

The project emissions, calculated with equations 12 and 13 and 14 for the CO2 and non CO2 emissions respectively of this report. The PFT results from the last monitoring period were used. The project emission calculation is shown here below:

Table 7: Project fuel emissions

Finally, the emission reductions are calculated with equation 15 in this report, which is the sum of the calculated BE minus the PE in this section multiply by the number of number of units in operation for the length of the monitoring period. Section B6.4 presents the ERs for the 7 years in this CP.

Other SDGs – example calculations are tabulated below – all values are sourced from parameters monitored. All data here is sourced from the sheet SDG impact tool in file VPA03 CPII SDG database. For readability, only the cells are referenced.

| SDG | Contribution | Ex-ante estimate |
| --- | --- | --- |
| 1 | GSDM-I1.1.1 Average household savings i.e., decrease in expenditure on basic service such cooking, lighting, drinking | This SDG is calculated with equation SDG1 The household savings are 1909 UGX per day and is calculated by subtracting maintenance cost from savings incurred from fuel substitution. This is then multiplied with the number of technology days included. Cell H81, for annual figures see B.6.4 |
| 2 | GSDM-I2.4.2 Number of farmers adopted the practice by the project | This SDG is calculated with equation SDG2 98% of the farmers use bio-slurry, that times the number of plants in operation yield the annual result. Cell L78, for annual figures see B.6.4 |
|  | GSDM-I2.4.3: Area under sustainable agriculture | The outcome of GSDM-I.2.4.2 multiplied with the percentage of land on which bio-slurry is applied (56%) and land ownership (5.88 hec) gives the average per farmer (3.3 hec) Cell P79,81,83, for annual figures see B.6.4 |
| 3 | GSDM-I.3.9.1: Reduced indoor air pollution | This SDG is calculated with equation SDG3 The percentage of households observing a reduction in PM2.5 and CO is 83%. That multiplied by the number of units in operation gives the total result. Cell T79, for annual figures see B.6.4 |
| 4 | GSDM-I.4.4.1 Number of employees provided skill development training | This is based on the number of masons trained, which is 178 at the time of developing this report. Cell X79, for annual figures see B.6.4 |
| 5 | GSDM-I.5.4.1: Average time saving associated with cooking time and fuel collection | This SDG is calculated with equation SDG5 The total time savings of biodigester in operation is 1.48 hours (1 hour and 28.6 minutes). That time the number of plants in operation gives the annual result. Cell AB79, for annual figures see B.6.4 |
| 7 | GSDM-I7.1.1: Number of beneficiaries: individuals and households | This SDG is calculated with equation SDG7 The total number household with access to energy are biodigesters installed * household size (cell AF77) For annual figures see B.6.4 |
| 8 | GSDM-I.8.5.1 total number of jobs | This is a cumulative number of masons provided biodigester- jobs created, and this is 412 for year 1 in CPII, see cell AK59 The count is based on the total number of digesters constructed, regardless of operationality. For annual figures see B.6.4 |
| 9 | GSDM-I.9.3.1 Total number of companies supported for their integration into value chains and markets | This is the cumulative number of companies that gained access to the biogas value chain, which is 74 in year 1 of CPII, see AO59. The count is based on the total number of digesters constructed, regardless of operationality. For annual figures see B.6.4 |
| 12 | GSDM-I.12.5.2: Improved waste management services | This SDG is calculated with equation SDG2 This is obtained my multiplying the number of biodigester in operation with the feeding rate, manure and water (78.32 kg/day, cell AR80) and then annualized. For annual figures see B.6.4 |
| 15 | GSDM-I15.1.1 Total non-renewable wood fuel saved | This SDG is calculated with equation SDG15 This is based on equation 4 and 12 in this report and multiplied with the number of technology days included, see AW59 and is for is for year 1: 9,290 ton NRB For annual figures see B.6.4 |

Summary of ex ante estimates of each SDG outcome

GSDM-I13.2.1: Reduction in GHGs emissions

Unit: VER

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 45,675 | 37,704 | 7,971 |
| Year 2 | 48,903 | 40,368 | 8,535 |
| Year 3 | 53,286 | 43,986 | 9,300 |
| Year 4 | 58,776 | 48,518 | 10,258 |
| Year 5 | 64,309 | 53,086 | 11,223 |
| Year 6 | 69,842 | 57,653 | 12,189 |
| Year 7 | 75,374 | 62,220 | 13,154 |
| Total | 416,165 | 343,535 | 72,630 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 10,376 |  |  |

GSDM-I1.1.1 Average household savings i.e., decrease in expenditure on basic service such cooking, lighting, drinking

The results in UGX are tabulated below. The value will also be presented in USD in the monitoring report based on the prevailing forex exchange rate.

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 2 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 3 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 4 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 5 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 6 | 0 | 3,636,775,691 | 3,636,775,691 |
| Year 7 | 0 | 3,636,775,691 | 3,636,775,691 |
| Total | - | 25,457,429,835 | 25,457,429,835 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 3,636,775,691 |  |  |

GSDM-I2.4.2 Number of farmers adopted the practice by the project

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 5,127 | 5127 |
| Year 2 | 0 | 5,499 | 5499 |
| Year 3 | 0 | 5,996 | 5996 |
| Year 4 | 0 | 6,616 | 6616 |
| Year 5 | 0 | 7,237 | 7237 |
| Year 6 | 0 | 7,857 | 7857 |
| Year 7 | 0 | 8,478 | 8478 |
| Total | 0 | 8,478 | 8,478 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 6,887 |  |  |

GSDM-I2.4.3: Area under sustainable agriculture

Unit: hectare

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 17,233 | 17,233 |
| Year 2 | 0 | 18,485 | 18,485 |
| Year 3 | 0 | 20,153 | 20,153 |
| Year 4 | 0 | 22,239 | 22,239 |
| Year 5 | 0 | 24,325 | 24,325 |
| Year 6 | 0 | 26,411 | 26,411 |
| Year 7 | 0 | 28,497 | 28,497 |
| Total number of crediting years | - | 28,497 | 28,497 |
| Annual average over the crediting period | 23,230 |  |  |
| Total number of crediting years | 7 |  |  |

GSDM-I3.9.1 Number of households that observed reduction in PM2.5 & carbon monoxide (CO) concentration reductions

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 4,321 | 4321 |
| Year 2 | 0 | 4,635 | 4635 |
| Year 3 | 0 | 5,053 | 5053 |
| Year 4 | 0 | 5,576 | 5576 |
| Year 5 | 0 | 6,099 | 6099 |
| Year 6 | 0 | 6,622 | 6622 |
| Year 7 | 0 | 7,145 | 7145 |
| Total | 0 | 7,145 | 7,145 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 5,636 |  |  |

GSDM-I.4.4.1 Number of employees provided skill development training

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 228 | 228 |
| Year 2 | 0 | 298 | 298 |
| Year 3 | 0 | 368 | 368 |
| Year 4 | 0 | 438 | 438 |
| Year 5 | 0 | 508 | 508 |
| Year 6 | 0 | 578 | 578 |
| Year 7 | 0 | 648 | 648 |
| Total | - | 648 | 648 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 438 |  |  |

GSDM-I.5.4.1: Average time saving associated with cooking time and fuel collection

Unit: hours

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 2,813,654 | 2,813,654 |
| Year 2 | 0 | 3,020,745 | 3,020,745 |
| Year 3 | 0 | 3,282,509 | 3,282,509 |
| Year 4 | 0 | 3,620,711 | 3,620,711 |
| Year 5 | 0 | 3,961,527 | 3,961,527 |
| Year 6 | 0 | 4,314,166 | 4,314,166 |
| Year 7 | 0 | 4,643,159 | 4,643,159 |
| Total | 0 | 25,656,471 | 25,656,471 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 3,665,210 |  |  |

GSDM-I7.1.1: Number of beneficiaries: individuals

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 37,192 | 37,192 |
| Year 2 | 0 | 39,893 | 39,893 |
| Year 3 | 0 | 43,494 | 43,494 |
| Year 4 | 0 | 47,996 | 47,996 |
| Year 5 | 0 | 52,497 | 52,497 |
| Year 6 | 0 | 56,999 | 56,999 |
| Year 7 | 0 | 61,500 | 61,500 |
| Total | 0 | 61,500 | 61,500 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 48,150 |  |  |

GSDM-I7.1.1: Number of beneficiaries: households

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 5,223 | 5,223 |
| Year 2 | 0 | 5,602 | 5,602 |
| Year 3 | 0 | 6,108 | 6,108 |
| Year 4 | 0 | 6,740 | 6,740 |
| Year 5 | 0 | 7,372 | 7,372 |
| Year 6 | 0 | 8,004 | 8,004 |
| Year 7 | 0 | 8,637 | 8,637 |
| Total | - | 8,637 | 8,637 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 6,812 |  |  |

GSDM-I.8.5.1 total number of jobs

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 412 | 412 |
| Year 2 | 0 | 439 | 439 |
| Year 3 | 0 | 475 | 475 |
| Year 4 | 0 | 521 | 521 |
| Year 5 | 0 | 566 | 566 |
| Year 6 | 0 | 611 | 611 |
| Year 7 | 0 | 657 | 657 |
| Total | - | 657 | 657 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 526 |  |  |

GSDM-I.9.3.1 Total number of companies supported for their integration into value chains and markets

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 74 | 74 |
| Year 2 | 0 | 79 | 79 |
| Year 3 | 0 | 86 | 86 |
| Year 4 | 0 | 94 | 94 |
| Year 5 | 0 | 102 | 102 |
| Year 6 | 0 | 110 | 110 |
| Year 7 | 0 | 119 | 119 |
| Total | - | 119 | 119 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 95 |  |  |

GSDM-I.12.5.2: Improved waste management services

Unit: ton/year

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 0 | 149,197 | 149,197 |
| Year 2 | 0 | 160,178 | 160,178 |
| Year 3 | 0 | 174,058 | 174,058 |
| Year 4 | 0 | 191,992 | 191,992 |
| Year 5 | 0 | 210,064 | 210,064 |
| Year 6 | 0 | 228,763 | 228,763 |
| Year 7 | 0 | 246,208 | 246,208 |
| Total | 0 | 1,360,460 | 1,360,460 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 194,351 |  |  |

GSDM-I15.1.1 Total non-renewable wood fuel saved

Unit: ton/year

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| Year 1 | 21,537 | 12,247 | 9,290 |
| Year 2 | 44,596 | 25,360 | 19,237 |
| Year 3 | 69,722 | 39,647 | 30,075 |
| Year 4 | 97,437 | 55,407 | 42,029 |
| Year 5 | 127,760 | 72,651 | 55,109 |
| Year 6 | 160,693 | 91,378 | 69,315 |
| Year 7 | 196,234 | 111,588 | 84,645 |
| Total | 717,978 | 408,279 | 309,699 |

| Total number of crediting years | 7 |  |  |
| --- | --- | --- | --- |
| Annual average over the crediting period | 44,243 |  |  |

Monitoring plan

Data and parameters to be monitored

SDG 13

| Data / Parameter | BGTA 24: Avoidance of double counting or double claiming among project technology end users |
| --- | --- |
| Unit | NA |
| Description | Evidence of avoidance of double counting or double claiming with project technology end users |
| Source of data | Warranty certificates / sales contracts alerting end-users to the waiving of their carbon rights signed by end users |
| Value(s) applied | No double counting occurs due to the 100% of the users have received a warranty card. |
| Measurement methods and procedures | Inspection of warranty card |
| Monitoring frequency | Monitored whenever project technology is sold or otherwise disseminated. |
| QA/QC procedures | Unique identification of users in the database |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

| Data / Parameter | BGTA 25: Up,y |
| --- | --- |
| Unit | Percentage |
| Description | Average usage rate in project scenario p during year y |
| Source of data | VPA03 CPII SDG database sheet SDG outcome, cell G7 |
| Value(s) applied | Ex-ante: 63.2% |
| Measurement methods and procedures | Monitoring of operationality of the biogas systems, including the operationality of both the biogas digester and biogas cookstove, through - Census of users or survey of the users at randomly selected sample sites; The census will determine: - the status of the digester (in operation or out of operation - the capacity factor: % of days that the plant was in operation (see BGTA 28). - for plants out of operation, the period in use during a particular MP will be determined - Based on on-going rental/lease payments or a recurring maintenance fee by users; - Measurement campaigns using biogas flow meters. The project can claim up to 90% in cases where the operationality is determined based on user reported questionnaire survey i.e., using option (a) above to account for uncertainties. Maximum up to 100% usage rate can be claimed using option (b) and (c). |
| Monitoring frequency | Method; Frequency. Census of users or survey of the users at randomly selected sample sites; Annual. Based on on-going rental/lease payments or a recurring maintenance fee by users;; Continuous measurement. Measurement campaigns using biogas flow meters; Annual: The continuous use monitoring campaign shall be conducted for a minimum of 100 households for at least 30 days.. |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) or 95/10 (for biennial monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | Also applicable to SDG1,2,3,5,7,12 and 15 |

| Data / Parameter | BGTA 26: NLT,y |
| --- | --- |
| Unit | Number |
| Description | Annual average number of animals of type LT in year y |
| Source of data | VPA03 CPII SDG database sheet BE Cell E9,15,21,27,33,39,45 |
| Value(s) applied | Number of dairy cows; 6.72. Number of other cattle; 3.07. Swine finishing; 1.32. Number of breeding swine; 1.70. Number of sheep; 0.28. Number of goat; 3.96. Number of poultry; 37.15. |
| Measurement methods and procedures | AWMS Method 1 is applied in this VPA The annual average is determined by: - for animals with generally a static population: Asking the farmer how many on average he owns during a particular MP - for animals raised in cycles are fattening pigs. Asked will be how many rounds per year, the average length of a round (see BGTA 29), and number of animal per round to calculate the annual average. |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project and baseline scenario |
| Additional comment | N/A |

| Data / Parameter | BGTA27: TAMLT |
| --- | --- |
| Unit | kg/animal |
| Description | Typical animal mass for livestock LT at the project site. |
| Source of data | IPCC default TAM values as provided in Table 10A.5 as well as in the Annexes of Chapter 10 of chapter ‘Emissions from Livestock and Manure Management’ under the volume ‘Agriculture, Forestry and other Land use’ of the 2019 IPCC Guidelines for National Greenhouse Gas Inventories shall be selected accordingly. |
| Value(s) applied | Dairy cow; 270. Cattle; 270. Fattening pig; 49. Breeding pig; 41. Goat; 31. Sheep; 24. Poultry; 13. |
| Measurement methods and procedures | When IPCC values of VS are adjusted for site specific animal weight as per para 3.5.4 ¦ or 3.5.7 ¦ and Eq. 3 or Eq. 6 sampling procedures can be used to estimate this variable as per the Section 4.4 ¦General requirements for sampling of the applied methodology |
| Monitoring frequency | Annual |
| QA/QC procedures | Proper implementation of the methodology |
| Purpose of data | Calculation of project scenario |
| Additional comment | N/A |

| Data / Parameter | BGTA 28: ndy |
| --- | --- |
| Unit | Days |
| Description | Number of days the treatment plant was operational in year y. |
| Source of data | Monitoring survey |
| Value(s) applied | N/A, varies on the length of the MP. In the first monitoring survey this was estimated at 98% of all days. Thus, if an MP is 100 days, the days operational are 98. See VPA03 CPII SDG database sheet SDG outcome cell G4 |
| Measurement methods and procedures | Survey methods |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | N/A |

| Data / Parameter | BGTA 29:Nda,y |
| --- | --- |
| Unit | days |
| Description | Number of days animal is alive in the farm in the year y |
| Source of data | Ex-ante: VPA03 CPII SDG database sheet CPII MPI SDG survey cell CP206 |
| Value(s) applied | See BGTA 25, this applies to animals raised in cycles. Dairy cow; N/A static population. Cattle; N/A static population. Fattening pig; 118. Breeding pig; N/A static population. Goat; N/A static population. Sheep; N/A static population. Poultry; N/A static population. |
| Measurement methods and procedures | Monitoring survey |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | N/A |

| Data / Parameter | BGTA 30: Np,y |
| --- | --- |
| Unit | [-] |
| Description | Annual average number of animals of type LT in year y. |
| Source of data | VPA03 CPII SDG database sheet BE Cell E9,16,23,30,37,44,51 |
| Value(s) applied | Number of dairy cows; 6.72. Number of other cattle; 3.07. Swine finishing; 1.32. Number of breeding swine; 1.70. Number of sheep; 0.28. Number of goat; 3.96. Number of poultry; 37.15. |
| Measurement methods and procedures | In person interview |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

| Data / Parameter | BGTA 31: MS%i,y |
| --- | --- |
| Unit | % |
| Description | Fraction of manure handled in project animal manure management system i |
| Source of data | VPA03 CPII SDG database sheet PE Cell E12,13,23,24,34,35,45,46,57,58,67,68,78,79 |
| Value(s) applied | Ex-ante, to be updated annually; Percentage point biodigester MS. Dairy cow; 77. Cattle; 79. Fattening pig; 4. Breeding pig; 30. Sheep; 0. Goat; 0. Poultry; 11. |
| Measurement methods and procedures | Monitoring survey |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | The methodology describes for method 1 the IPCC default values. However, there are no default values for digester feeding in the guidelines. For that reason, this is monitored. |

| Data / Parameter | BGTA 32: Proper soil application |
| --- | --- |
| Unit | - |
| Description | Proper application of digestate which not resulting in methane emissions) |
| Source of data | VPA03 CPII SDG database sheet SDG impact tool cell K78:L81 |
| Value(s) applied | Ex-ante: 98% use digestate as fertilizer (= proper soil application). 1% lets it run over the fields and 1% gives it away and let it runs over the field. By far the majority is using bio-slurry properly not resulting in methane emissions |
| Measurement methods and procedures | The proper soil application (which not resulting in methane emissions) of the digestate shall be verified on a sampling basis following the Requirements in the “Standard for sampling and surveys for CDM project activities and programme of activities”. |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of leakage emissions |
| Additional comment | - |

b.	Parameters related to Thermal application

| Data / Parameter | BGTA 38: Nb,p,y |
| --- | --- |
| Unit | days |
| Description | Number of project technology-days included in the project database for each project scenario in year y. |
| Source of data | VPA03 CPII SDG database sheet SDG 13 and NP cell F16 for year 1 estimate |
| Value(s) applied | The value depends on the monitoring period length. The year 1 estimate is 3,013,306 days |
| Measurement methods and procedures | Calculated from the Project database as the sum of the number of installed project technology units times the calendar days during the year y that they were used at the end user locations. The methodological default of 2 weeks is used for the period from commissioning and start date |
| Monitoring frequency | Continuously, collected annually |
| QA/QC procedures | Cross check the results of the usage survey with the contents of the project database to confirm whether the project technology units surveyed are present at end user locations as expected, or not. If there is discrepancy, this must be explained or corrected. |
| Purpose of data | Calculation of project outcome |
| Additional comment | Conversion of methodology default of 2 weeks is by the following: in cumulative number of units installed in month ‘y’ are units installed in month ‘y-1’ + units installed in month ‘y/2’. This is conservative as the average length of a month is more than 14 days Applicable to SDG 1,5,12 and 15 as well |

| Data / Parameter | BGTA 39: LEp,y |
| --- | --- |
| Unit | tCO2e per year |
| Description | Leakage in project scenario p during year y |
| Source of data | BGTA 43 |
| Value(s) applied | 0 (ex-ante) |
| Measurement methods and procedures | Monitoring survey |
| Monitoring frequency | Every two years, or default discount value of 0.95 applied to emission reductions. |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project outcome |
| Additional comment | Monitoring parameters required for calculating leakage emissions shall be included in the monitoring plan in the PDD as required to monitor and quantify the sources of leakage determined by following section 3.7 ¦Leakage emissions. |

| Data / Parameter | BGTA 42: Pp,i,y |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Quantity of fuel that is consumed in project scenario p during year y. |
| Source of data | VPA03 CPI MRV |
| Value(s) applied | Fuelwood; 2.801. Charcoal; 0.0. LPG; 0. |
| Measurement methods and procedures | KPT protocol |
| Monitoring frequency | Updated every two years, or more frequently starting with the first verification. The KPT values are valid for two years and may be applied for before or after period. |
| QA/QC procedures | Compliance with the general requirements for sampling (Section 4.4 ¦), general requirements for QA/QC (Section 4.5 ¦) and Annex 2: COMPLEMENTARY GUIDELINES FOR KITCHEN PERFORMANCE TESTING. Of the applied methodology |
| Purpose of data | Calculation of project outcome |
| Additional comment | N/A |

| Data / Parameter | BGTA 43: Leakage assessment |
| --- | --- |
| Unit | tonnes/household/year |
| Description | Quantity of fuel that is consumed in project scenario p during year y attributed to leakage |
| Source of data | Monitoring survey. Ex-ante from VPA03 CPII SDG database sheet PE_LE cell E97 |
| Value(s) applied | 0 (actual value is negative, indicating there is negative leakage. The value is kept at 0 which is conservative) |
| Measurement methods and procedures | #; Source to evaluate; Assessment; Ex-ante assessment based on CPII MP results. The displaced baseline technologies are reused outside the project boundary in place of lower emitting technology or with a higher intensity than would have occurred in the absence of the project.; The biogas household will be asked what happened with their baseline technology and if this was used outside the project boundary displacing a lower emitting technology; All households either use it, demolished it, or keep it as back up stove.. Members of the population who do not participate in the project, and previously used lower emitting energy sources, instead use the non-renewable biomass or fossil fuels saved under the project activity.; This is not a risk. The project will save wood and therefore reduce the pressure on the forest. Fossils fuels are a minor sour of energy in the project.; N/A no hh gave their stove away. The project significantly reduces the NRB fraction within an area where other GHG mitigation project activities account for NRB fraction in their baseline scenario.; Biogas households are scattered throughout the country and not concentred. This is therefore not an issue. The project population compensates for loss of the space heating effect of inefficient technology by adopting some other form of space heating or by retaining some use of inefficient technology.; Households will be asked, do you use more fuels for space heating since you have installed a biogas plant. If so, how much?; The amount of fuel has decreased as per MPI results (cell E125 of the referenced sheet). By virtue of promotion and marketing of a new technology, the project stimulates substitution with this technology by households who commonly used a technology with relatively lower emissions.; This is the project activity. No assessment is required.. |
| Monitoring frequency | Updated every two years, or more frequently. |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project outcome |
| Additional comment | - |

SDG 1

| Data / Parameter | GSDM-I1.1.1: SDG1 |
| --- | --- |
| Unit | Monthly savings (local currency/day) |
| Description | Average household savings in expenditure on basic service such cooking |
| Source of data | Ex-ante: VPA03 CPII SDG database sheet impact tool from row cell H81 |
| Value(s) applied | 1,909 |
| Measurement methods and procedures | Survey – households will be asked how much they saved on cooking fuels, boiling water and lighting compared to the situation before having a biodigester |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project outcome |
| Additional comment | - |

| Data / Parameter | GSDM-I1.1.1: Forex |
| --- | --- |
| Unit | UGX/USD |
| Description | Forex exchange rate |
| Source of data | www.xe.com |
| Value(s) applied | 0.00025 on 8/8/2023 |
| Measurement methods and procedures | the website Xe.com will be used for the exchange rate |
| Monitoring frequency | Annual |
| QA/QC procedures | Include the day of the exchange rate for traceability |
| Purpose of data | Calculation of project outcome |
| Additional comment | - |

SDG 2

| Data / Parameter | GSDM-I2.2.3: BIOu |
| --- | --- |
| Unit | Bio-slurry usage rate |
| Description | % farmers using bio-slurry as fertilizer |
| Source of data | Ex-ante: VPA03 CPII SDG database sheet impact tool cell L78 |
| Value(s) applied | 98% ex-ante |
| Measurement methods and procedures | Survey method – households will be asked how much land is in cultivation and the proportion on which bio-slurry is applied as main fertilizer |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

| Data / Parameter | GSDM-I2.2.3: Fa |
| --- | --- |
| Unit | hectare/hh |
| Description | Land area on which bio-slurry is applied |
| Source of data | Ex-ante: VPA03 CPII SDG database sheet impact tool cell P83 |
| Value(s) applied | 3.3 |
| Measurement methods and procedures | Survey method – households will be asked how much land is in cultivation and the proportion on which bio-slurry is applied as main fertilizer |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

SDG 3

| Data / Parameter | GSDM-I3.3.9: HAPr |
| --- | --- |
| Unit | % |
| Description | Fraction of families mentioned reduction in household air pollution |
| Source of data | SMS Ex-ante: VPA03 CPII SDG database sheet impact tool cell T79 |
| Value(s) applied | Ex ante: 83% |
| Measurement methods and procedures | Survey method, households will be asked if a reduction in household air pollution is observed |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

SDG 4

| Data / Parameter | GSDM-I4.4.1: Quality education |
| --- | --- |
| Unit | Number |
| Description | Refers to the number of masons, companies and opthers who received training services of any type via project during the reporting period. |
| Source of data | VPA implementer tracking sheets Ex-ante: VPA03 CPII SDG database sheet impact tool cell Y59 |
| Value(s) applied | Ex-ante: 228 |
| Measurement methods and procedures | Training reports |
| Monitoring frequency | Annual |
| QA/QC procedures | Recording of training and participant lists |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

SDG 5

| Data / Parameter | GSDM-I5.4.1: Ts |
| --- | --- |
| Unit | Hour/day |
| Description | Time savings (hours per day) |
| Source of data | SMS Ex-ante: VPA03 CPII SDG database sheet impact tool cell AB79 |
| Value(s) applied | Ex-ante: 1.48 |
| Measurement methods and procedures | Sample surveys in representative households. The main cook will be asked how much time she saved on collecting wood, starting and tendering the fire and cleaning of pots attributed to the biodigester. |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

| Data / Parameter | GSDM-I5.4.1: Usage of time saved |
| --- | --- |
| Unit | Usage of time saved |
| Description | In order to understand whether clean and/or efficient cookstove or fuel is contributing to more gender-equitable development, it is important to know how users are spending their saved time and whether the act of saving time on fuel collection actually contributes to their well-being and empowerment, or if that time is just replaced with another household chore." |
| Source of data | SMS Ex-ante: VPA03 CPII SDG database sheet SDG outcome sheet B60:D64 |
| Value(s) applied | Ex-ante: usage of time; % experienced. Productive time use; 32; 67%. Leisure; 12; 25%. Family activities; 12; 25%. education activities; 0; 0%. Social activities; 18; 38%. |
| Measurement methods and procedures | Sample surveys in representative households |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

SDG 7

| Data / Parameter | GSDM-I7.1.1: HHs |
| --- | --- |
| Unit | Household size |
| Description | Average household size |
| Source of data | SMS Ex-ante: VPA03 CPII SDG database sheet SDG impact tool cell AF77 |
| Value(s) applied | Ex-ante: 7.121 |
| Measurement methods and procedures | Sample surveys in representative households |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

SDG 12

| Data / Parameter | GSDM-I.12.5.2: F(p,b) |
| --- | --- |
| Unit | kg feeding/day |
| Description | Biodigester feeding rate with manure and water |
| Source of data | SMS Ex-ante: VPA03 CPII SDG database sheet SDG impact tool cell AR80 |
| Value(s) applied | Ex-ante: 78.3 |
| Measurement methods and procedures | Sample surveys in representative households |
| Monitoring frequency | Annual |
| QA/QC procedures | Compliance with the general requirements for sampling and for all cases where sampling is applied, the “Standard for sampling and surveys for CDM project activities and programme of activities” shall be used for determining the sample size to achieve 90/10 (for annual monitoring) confidence/precision levels. |
| Purpose of data | Calculation of project scenario |
| Additional comment | - |

Sampling plan

The sampling plan is the following for the SDG monitoring survey (SMS) and the baseline survey:

|  | Requirement during monitoring | Baseline survey conversion |
| --- | --- | --- |
| Survey name | SDG monitoring survey (SMS) | Baseline survey |
| Target group | The project survey requires in person interviews with a robust sample of end users without project technologies that are representative of end users targeted in the project activity. | A baseline survey was executed in November 2021 for the development of this VPA—DD. Project households were randomly selected and based on this selection equivalent households with the potential for biogas, defined of having at least 2 cattle or 5 pigs, were selected near the project households. |
| Sampling frame | Project database | Nearest households from a randomly selected biogas household |
| Sample Sizing | > 100 (as the population is larger than 1000) | > 100 (actual n=126) |
| Cross-VPA sampling | Cross-VPA sampling is not allowed across groups larger than 10 VPAs. The requirements described here apply both when sampling is applied to a single VPA and to permissible cross-VPA sampling | N/A the survey is representative for Uganda |
| Sampling method | A statistically valid sample can be used to determine parameter values, as per the relevant requirements for sampling in the latest version of the CDM Standard for sampling and surveys for CDM project activities and programme of activities. 90% confidence interval and a 10% margin of error requirement shall be achieved for the sampled parameters unless mentioned otherwise in the methodology. The parameter of interest on the basis of which the compliance with the 90/10 sampling compliance will be assessed is the usage rate | The sampling frame of the existing biodigester users was used. This ensured that the survey took place in the localities where the actual implementation takes place. |
| Data to be collected | See parameters to be monitored | See section B7.1 and indicator BGTA 6: for baseline scenario survey results. |

Table 8: Kitchen performance test

|  | Requirement during montoring |  | Baseline survey conversion |
| --- | --- | --- | --- |
| KPT | PFT |  | BFT |
| Sample size | > 30 |  | B1: >30 (actual 42) |
|  |  |  | B2: >32(actual 32) |
| Purpose | The baseline and project performance field tests (BFT and PFT) measure real, observed technology performance in the field. When Option 1 (based on avoided quantity of fuel consumption) is applied, a field test is carried out both for baseline and project scenarios, either by an independent sample (different subjects for baseline and project scenarios). |  |  |
| Target group | Households with a biodigester | Equivalent households without a baseline. Criteria used in order of importance and deviation permissible were: - The HH size of BFT HH is a maximum deviation of +/- 2 people compared to the project HH size. - The Number of meals should match-no the deviation on this one, if Project HH cooked 3 meals then BFT HH should be cooking 3 meals per day. - The minimum number of animals should be met, by the BFT HH (>20 kg manure per day) - The Stove in use in the BFT HH should be equivalent with the baseline stove of the project hh |  |
| Sampling strategy | Simple random sampling | Simple random sampling based of biodigester households and selecting the nearest equivalent baseline household for the BFT. This ensures that the BFT takes place in areas with biodigesters |  |
| Statistical analysis | 90/30 rule. When the sample sizes are large enough to satisfy the “90/30 rule,” i.e., the endpoints of the 90% confidence interval lie within +/- 30% of the estimated mean GHG emission or fuel consumption, overall emission reductions can be calculated on the basis of the estimated MEAN annual emission reduction per unit or MEAN fuel annual savings per unit. Outliers will be excluded using the Grubb’s Test. | As with the project scenario. B1 90%/12% B2: 90%/11% And therefore, the mean is used No outliers were detected |  |

Project database including a total sales or dissemination record

A complete sale record is kept in the project database Salesforce. Salesforce include, at minimum the following information

- Date of installation and commissioning

- Geographic area of sale

- Model/type of project technology sold

- Quantity of project technologies sold

- Name and telephone number (if available), and address and/or GPS coordinates:

Mode of use: domestic, institutional commercial, other.

Other elements of monitoring plan

Quality assurance/Quality control

The CME will provide the necessary training to the VPA implementers, and the parties involved in the monitoring to ensure that the data recorded is complete and accurate. Training will include going through all the data that needs to be collected and how to do this, and to adequately filling out the questionnaire. The VPA Implementer will prepare data collection protocols to be given to the research assistants to guide them during the data collection exercise. The qualifications and experience of personnel involved in monitoring should be as follows:

- Knowledge of local language(s)

- Prior experience in household surveys, with experience in fuel surveys is an advantage.

- At least one team member with a good understanding of statistics and statistical software

- Knowledge of technical aspects of operating a biogas digester

- Ability to identify different manure management techniques.

- Prior involvement in surveys (baseline or monitoring) for other carbon projects is advantageous,

Response rates will be maximized by contacting all randomly selected biogas digester users beforehand to arrange a practical site visit date and sampling over the minimum required number to compensate for any non-responses in case sufficient telephone numbers of users are available. In special cases where participants refuse to participate in the monitoring, the reason shall be documented. The surveyor will explain that monitoring is part of the requirements of the programme and try to arrange an alternative date for a site visit or carry out monitoring with another member of the household.

All data monitored and required for verification and issuance will be kept for at least five years after the end of the crediting period or the last issuance of VERs for the project activity, whichever is later.

Analysis: All the sales data and the survey data will be captured in a computerized database. The analysis will include a calculation of the proportion of biogas system in use and of the emission reductions according to the methodology applied.

#### Implementation plan

The VPA implementer is responsible for entering user data into the Project Database, and for ensuring that the information in the Sales Agreements is complete and correct. The total number of Sales Agreements will reveal the quantity of biogas systems sold at the end of a Monitoring Period. The Project Database will record the start and end dates of each selling year y for each biogas system and calculate the emission reductions attributable to each Monitoring Period. Appropriate record keeping procedures will be implemented to ensure that each Monitoring Period dataset can be transparently attributed to its corresponding VPA, preventing any occurrences of double counting.

#### Monitoring Responsibilities

The VPA Implementer is responsible for all the monitoring activities carried out within their respective VPA, including data collection, data analysis and writing the Monitoring Report unless otherwise agreed to with the CME. It is possible for instance to outsource data collection and monitoring reporting to other qualified parties.

DURATION AND CREDITING PERIOD

Duration of project

Start date of VPA

11/11/2009

Expected operational lifetime of VPA

21 years

Crediting period of project

Start date of crediting period

The first crediting period started on 19/04/2015.

The second crediting period started on 19/04/2022.

Total length of crediting period

7 years, 2 times renewal

SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

Safeguarding Principles that will be monitored

No risks were identified of which mitigation measures are added to the monitoring plan.

Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | The project doesn’t involve the activity related to slavery, imprisonment, physical and mental drudgery, punishment or coercion of women and girls. The project enhances the women’s access and entitlement of benefits. And since the women will be direct user of the Biogas stoves, it will benefit women by reducing their exposure to the indoor air pollution thereby improving their health. The project will not restrict the women rights or access to resources but promotes the women rights to have access to clean energy and environment. In addition, the replacement of firewood after the installation of Biogas will reduce workload of women for the collection of firewood. The workload of women to collect wood is reduced, cooking is faster, and less cleaning is required as pots are not blackened by soot. Reduced workload results in time saving that the women can use for other productive activities. The project also improved convenience as biogas ignites directly and there is no need to tender the fire continuously. The project doesn’t involve any activity risking sexual harassment, or any forms of violence against women. On the contrary, it reduces the risk of sexual harassment or violence when women collect firewood in the baseline scenario But the project promotes to have access to clean energy and environment to the women. Benefits from the project is expected to create entrepreneurial opportunities for both men and women without any discrimination. Training and capacity building is equitably provided to both men and women without discrimination |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | The PD is in line with Uganda’s National Gender Policy’s (NGP) revision in 2007. The policy confirms Uganda's commitment to take actions that will bring about more equal gender relations. The policy will ensure that all Government policies and programs, in all areas and at all levels, are consistent with the long-term goal of eliminating gender inequalities. And in accordance with the policy, the project responds to this by building capacity and self-sufficiency of women in rural Uganda by use of clean and reliable energy. In addition, the Draft Energy Policy 2019 has identified biogas projects to facilitate easier access of energy by vulnerable groups and the marginalized. The PD seeks to address the policy’s access to affordable and modern energy sources to women-led households. |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | No |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | No, no particular gender challenges are expected |

SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

Below is a summary of the 2 step GS4GG Consultation for monitoring purposes. Please refer to the separate Stakeholder Consultation Report for a complete report on the initial consultation and stakeholder feedback round.

Summary of stakeholder mitigation measures

- The VPA conducted the LSC at the CPI registration stage as per PoA-DD requirements. The GS4GG Principles & Requirements, Stakeholder Consultation Requirements, and the Community Services activity Requirements do not mandate an LSC at CP renewal.

Final continuous input / grievance mechanism

| Method | Details |  | Justification |
| --- | --- | --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | A Process Book was available at the following location Biogas Solutions Uganda Ltd Plot 36 Luthuli Rise, Bugolobi, P.O.Box 8339, Kampala, Uganda and used until March 2021. |  | Ever since COVID-19 grievances are collected digitally and integrated in SalesForce. However, stakeholders are welcome to visit the office and file their grievance in person |
| GS Contact (mandatory) | help@goldstandard.org |  | Mandatory |
| Telephone access | Stakeholders will be able to call to provide input on the project’s performance at any time. The number available is: +256 800399236 |  | Outcome of CPI stakeholder consultation. The provided number is toll-free. Mobile phone use is the primary means of communication nationwide, especially since landlines are expensive. |
| Internet/mail access | Stakeholders will be able to provide continuous input/feedback via the following email address: Website: http://www.biogassolutions.co.ug/ Email: info@biogassolutions.co.ug |  | Outcome of CPI stakeholder consultation. |
| Nominated independent mediator | Not included | CPI Stakeholders did not discuss this as being necessary. Given that all three other methods of providing (optional) feedback are provided, it was not deemed necessary to also include a Nominated Independent Mediator. |  |

### Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form below.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/potentially/no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | The project respects human rights, including dignity, cultural property and uniqueness of indigenous people. Participation is completely voluntary and the project respects personal freedom and liberty. The project is not complicit in Human Rights abuses. The project respects internationally proclaimed human rights. Host country commitment to UN conventions on Human Rights: International Covenant on Economic, Social and Cultural Rights New York, 16 December 1966 Uganda Accession (a), 21 January 1987. | N/A |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of non-discrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) |  | - No, the project does not affect control of resources, entitlements and benefits as, on the contrary, it brings benefits on time and resources savings which are mainly accrued to women - Project does not adversely affect men and women in marginalised or vulnerable communities - See section D.2 on the gender assessment - N/A | N/A |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community |  | Biodigesters are installed in backyards of the households and do not affect the community. On the contrary, due to the hygienic disposal of animal waste in biodigesters, nuisances, such as odour, water pollution etc, are avoided leading to less health risks to the community. Masons and other workers involved in the construction are through trainings encouraged to use Helmets, Gloves, Dust coats, Overalls, Gumboots, Safety shoes, Goggles | N/A |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | The project does not involve and is not complicit in the alteration, damage or removal of any critical cultural heritage. Cultural heritage will not be altered by the project since the biogas units are constructed within the household compounds on a voluntary basis and no damage to cultural or religious heritage is expected. | N/A |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | The project does not involve and is not complicit in involuntary resettlement. The domestic biogas units of are small in size and are constructed within people’s homesteads. The project will therefore not involve any resettlement. | N/A |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | no | No, as above, the project does not result in a change in land tenure rights | N/A |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | No, the project activities take place at individual households. There is no usage of people’s lands, resources, territories. Therefore, it does not involve any influence on indigenous people. | N/A |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects |  | The project does not involve and is not complicit in corruption. To reduce the risk of corruption occurring, the programme has the following mechanisms in place: - A Code of Conduct or Service contract for all biogas masons promoting fair competition practices is in place - all Masons/BCEs must sign and comply with the conditions stipulated in the Code of Conduct. This emphasizes integrity among personal and business conduct while working with the programme. - An annually renewed contract/letter of signed by BCEs/Masons - based on an individual’s manner of business conduct, Including integrity. - Client call centre, information sharing platforms and stakeholder sensitization meetings help to ensure transparency. In addition, the process of acquiring a digester is transparently documented and recorded in Salesforce | N/A |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | N/A | 1. Trained field officers, technicians and masons are involved in construction, installation and operation and maintenance of plants. Therefore, no forced labour is involved in the project in accordance with the ILO conventions. Moreover, Uganda has ratified the ILO conventions, with 26 conventions in force to ensure labour standards and protection of workers health and safety. 2.Workers under the project can freely establish and join labour organisations in compliance with the fundamental Freedom of Association and Protection of the Right to Organise Convention, 1948 (No. 87) and Right to organise and Collective Bargaining Convention (No. 98) - 3. Working agreements with all VPA staff shall be documented and implemented in accordance with Uganda’s National Employment Policy 2011. 4. The project does not employ and is not complicit in any form of child labour. There shall be no employment of children under the age of 18 years. Trained workers are involved in construction, installation and operation and maintenance of plants. The workers are required to enter their age in the contract documents and present IDs to ensure that no children under the age of 18 years are contracted for work. 5. Proper training is provided to the workers on regular basis to ensure appropriate work practices are maintained. Under Hivos Code of Conduct adopted by BSUL, the workers shall promptly address any and all problems and issues of concern as they arise. If these initial remedial actions fail to resolve the matter, the parties will pursue their respective remedies guided by applicable laws of Uganda. | N/A |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? |  | Transactions in the program are commercial, meaning that farmers pay the cost of services for installation and repair after the warranty period. Thus, even after the project these services will remain offered ensuring continued use of the technology. No negative consequences and therefore foreseen during and after the project | N/A |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | As demonstrated in the various monitoring reports in the first CP, there are significant savings by displacing NRB with biogas and by avoiding methane emissions from manure management by capturing and subsequently using methane as cooking fuel | N/A |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | No grid connected energy is involved | N/A |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | no | No, the construction of biodigesters does not affect natural or pre-existing pattern of watercourses, ground-water and/or the watershed | N/A |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | No, the construction occurs in backyards and does not cause erosion. | N/A |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? |  | No, the construction of biodigesters does not use soil or crops. | N/A |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? |  | No, the construction of biodigesters is not related to these risks. | N/A |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? |  | This is not applicable to the project as it does not produce crops | N/A |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? |  | Digester effluent can cause local eutrophication similar to the baseline situation. However, most farmers use bio-slurry which is a superior fertilizer compared to farmyard manure and the impact is therefore less on the environment compared to the baseline. | N/A |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? |  | No hazardous and non-hazardous chemical are involved in biodigester use and construction | N/A |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? |  | Farmers are trained to use bio-slurry as effective organic fertilizer which can improve yields and soil quality. However, the project does not encourage using chemical fertilizers | N/A |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? |  | On the contrary, the project will result in a lower demand for firewood | N/A |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? |  | On the contrary, bio-slurry is a very good fertilizer which improves crop quality | N/A |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? |  | Not applicable, the project only builds digesters in backyards of farmers | N/A |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? |  | No, this does not occur as mentioned earlier, biodigesters are built in the backyards of the homestead of the farmer. | N/A |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? |  | No, the project only focusses on rural areas with technical potential for biogas. These exclude area where endangered species may live. | N/A |
| >> |  |  |  |

### Appendix 2- Contact information of VPA Implementer

| Organization name | Biogas Solutions Uganda Ltd |
| --- | --- |
| Registration number with relevant authority | 190220 (certificate of incorporation number) |
| Street/P.O. Box | Plot 36 |
| Building | Luthuli Rise, Bugolobi |
| City | Kampala |
| State/Region | Kampala |
| Postcode | P.O. Box 8339 |
| Country | Uganda |
| Telephone | 0750 899 238/ 0750 899 236 |
| E-mail | info@biogassolutions.co.ug |
| Website | www.biogassolutions.co.ug |
| Contact person | Michel Muvule Pinto |
| Title | Program Coordinator |
| Salutation | Mr |
| Last name | Pinto |
| Middle name | Muvule |
| First name | Michel |
| Department | Not applicable |
| Mobile | +256752496128 |
| Direct tel. | +256752496128 |
| Personal e-mail | mmuvule@biogassolutions.co.ug |

### Appendix 3- LUF Additional Information

N/A

### Appendix 4-Summary of Approved Design Changes

Please refer to Design Changes Requirements for more information on procedures governing Design Changes

##### Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 2.0 | 4 May 2022 |  |
| 1.1 | 7 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

VPA03 CPII SDG database sheet SDG13 and NP cell C4

VPA03 CPII SDG database sheet SDG13 and NP cell E7, F7 and G7

VPA03 CPII SDG database sheet units cell B2:D12. The largest plant, the 60m3 plant, can be found in sheet database with the account name UG-CEN-2201-021810

https://www.mdpi.com/1996-1073/5/8/2911

As per BSUL technical information.  The 13 m3 digester has a comparable performance with the 12 m3. Larger digesters than 13 m3 are also installed, but these compromise less than 1% of all units installed and therefore not detailed. The performance of these digesters is proportional to the increase in digester volume.

See sheet VPA03 CPII SDG database sheet thermal capacity cell C5 and C11

See sheet VPA03 CPII SDG database sheet thermal capacity cell C33

See file 20.ODA decl BSU G4236

URL to the page with tools: https://cdm.unfccc.int/methodologies/SSCmethodologies/approved

The current version is 13.1. The version 9.0 of the initial VPA validation is applied, however.

Co-digestion of manure with other organic waste streams such as agricultural residue, kitchen food waste, fresh septic tank sludge etc. is permitted.

The 2006 guidelines were used as at that time the current methodology was not yet published. The former methodology, TPDDTEC v3.1, mandates the application of the 2006 guidelines.

As per methodology: The project developer may apply the default IPCC values or conduct surveys to assess the animal manure management practices in the baseline.

Projects that include thermal generation units with rated capacity greater than 150 kW or electricity generation component shall apply other applicable GS or CDM methodologies for energy generation and applications component.

In the baseline situation, there might be different types of fuel, stove technologies which must be identified as different baseline scenarios. Stacking fuel could be a baseline scenario, but it would be distinguished to the single fuel baseline situation.

This approach was approved by the GS. BE AWMS considers all manure while PE only the fraction fed. The fraction not fed therefore is the continuation of the baseline. The fraction fed is the fraction of BE AWMS affected by and that can be attributed by the project.

A technical measure to ensure that the gas holding capacity of the biodigester is sufficiently large to capture the biogas during periods of non-usage. A justification to demonstrate compliance with this requirement pertaining to the biogas digester size shall be included in the PDD.

The project developer may conduct only the first two phases of the stove tests: cold-start high-power phase and hot-start high-power phase (not including the simmer phase) for calculation of the high-power thermal efficiency.

The guidance provided in the WBT protocol may be followed for calibration of testing equipment.

See VPA03 CPII SDG database  sheet AWMS

See VPA03 CPII SDG database  sheet AWMS cell B175:I184

See VPA03 VPA03 CPII SDG database  sheet BE C9:G52

See VPA03 VPA03 CPII SDG database  sheet Be cell E52

See VPA03 CPII SDG baseline sheet PE cell C9:G58

See VPA03 CPII SDG database sheet PE cell E58

See sheet BE in file VPA03 CPII SDG database cell C55:G:85

See sheet PE in VPA03 CPII SDG database cell C61:G79

Implemented in July and august 2023, result taken from VPA03 CPII SDG database sheet BFT_wood and BFT_charcoal

The default value for baseline scenario could be applied following the guidance for parameter BGTA 20

For more information on the Grubbs’ test, please refer to: https://www.itl.nist.gov/div898/handbook/eda/section3/eda35h1.htm check	of	the	significance	of	the	results,	please	refer	to	an	online	tool	available	on: http://www.graphpad.com/quickcalcs/Grubbs1.cfm.

VPA03 CPII SDG database sheet BFT_wood cell W39 and BFT_charcoal cell W39

In fact, all collected data is digital, and available in the impact registry for all monitoring periods

https://landportal.org/library/resources/lex-faoc163564/uganda-gender-policy-2007#:~:text=3.3%20Objectives%3A%201)%20To%20reduce,violations%2C%20demand%2C%20access%2C%20seek

https://www.energyandminerals.go.ug/site/assets/files/1081/draft_revised_energy_policy_-_11_10_2019-1_1.pdf

https://www.ilo.org/dyn/normlex/en/f?p=NORMLEXPUB:11200:0::NO::P11200_COUNTRY_ID:103324