Gold Standard for the Global Goals

Transition Annex

(To be used by all GS CDM/VER stand alone projects and PoAs, Micro Scale stand alone projects and Micro PoAs)

Version 1 – September 2017

KEY PROJECT INFORMATION

| Title of Project/PoA/Activity: | Campion II Mini Hydro Power project |
| --- | --- |
| GS ID of the project/PoA/activity: | GS5923 |
| GS Version: | Monitoring Report Version 01 |
| Brief description of Project: | The Campion II Mini hydropower project provides clean renewable electricity to the Sri Lanka national grid (“the national grid”), which is owned and operated by Ceylon Electricity Board (CEB). The install capacity is 1MW and annual electricity generation (actual) of the project activity is 4.267 GWh. The project is located at upper Kehelgamu Oya running through Campion Estate in Bogawantalawa in Nuwara Eliya District, Sri Lanka. |
| Project type: Energy/Land Use | Energy |
| For Renewable Energy Projects – intention to apply RECs Labels (y/n) | No |
| GS Stream (CDM/VER): | CDM |
| Scale (large/scale/micro): | Micro |
| GS Registration Date: | 23/05/2019 |
| GS Crediting period start date: | 12/05/2017 |
| CDM Registration Date: |  |
| CDM Crediting period start date: |  |
| Project Developer: | Bogawanthalawa Tea Estate PLC Campion Hydro Pvt Ltd |
| Project Representative: | Bogawanthalawa Tea Estate PLC |
| Project Participants and any communities involved: | Bogawanthalawa Tea Estate PLC |
| Host Country/Location: | Upper Kehelgamu Oya running through Campion Estate in Bogawantalawa in Nuwara Eliya District, Sri Lanka. |
| Methodologies applied: | “AMS-I.D. Grid Connected renewable electricity generation” (Version 18) |
| SDG Impacts: | 7 – Affordable and clean energy 8 – Decent Work and Economic Growth 13- Climate Action |
| Estimated amount of SDG Impact (GSVERs and others) | 7- Access to affordable and clean energy services Net electricity generation by project activity – 7,680 MWh. |
|  | 8 – Quality of employment 10 employees were participated to training program about awareness of health and safety 8 – Quantitative employment and income generation 10 employees were involoved to this project and their pay sheets are annexed in monitoring report 8- Livelihood of the poor 10 number of No of job created by Campion Energy Service Pvt Ltd |
|  | 13- Climate Action GHG emission reduction – 5538 tCO2e |

NOTE: This Annex shall be used for all PoAs if the sustainable development assessment is conducted at PoA level. In case sustainable development assessment is conducted at activity level, then this Annex shall be filled for each of the activities.

SECTION A Sustainable Development Goals (SDG) outcomes

A.1	Relevant target for each of the three SDGs

>> (Specify the relevant SDG target for at least each of three SDGs addressed by the project. Refer most recent version of targets here. Contribution to SDG 13 is mandatory to be demonstrated for all projects and activities. Contribution to SDG 7 is recommended to be demonstrated for all community service projects and activities)

| - Item | - SDG targets | - Values estimated in ex ante calculation of approved PDD |
| --- | --- | --- |
| - GHG emission reduction – - SDG 13 | Integrate climate change measures into national policies, strategies and planning | 3,013 tCO2/year |
| - Quality of employment - SDG 8 | Protect labor rights and promote safe and secure working environments for all workers, including migrant workers, in particular women migrants, and those in precarious employment | People in rural areas are not very familiar with health and safety in relation to their places of work. Therefore, new staff will be trained by generating awareness of health and safety. |
| - Quantitative employment and income generation – SDG 8 | Sustain per capita economic growth in accordance with national circumstances and, in particular, at least 7 per cent gross domestic product growth per annum in the least developed countries | No of Job created- more than 0 |
| Access to affordable and clean energy services – SDG 7 | By 2030, increase substantially the share of renewable energy in the global energy mix | 4,179 MWh annual |
| Livelihood of the poor – SDG 8 | Sustain per capita economic growth in accordance with national circumstances and, in particular, at least 7 per cent gross domestic product growth per annum in the least developed countries | No of job created- more than 0 |
| Maintain minimum ecological flow – SDG 6 | By 2020, protect and restore water-related ecosystems, including mountains, forests, wetlands, rivers, aquifers and lakes | There is no significant change to the livelihood of plants or animals before or after the project activity. Aquatic life is not affected when compared to the baseline scenario. The indicator is thus scored neutrally. |

A.2	Explanation of methodological choices/approaches for estimating the SDG outcome

>> (Explain how the methodological steps in the selected methodology(ies) or proposed approach for calculating baseline and project outcomes are applied. Clearly state which equations will be used in calculating net benefit.)

| - Item | - Methodology/proposed approach |
| --- | --- |
| - Climate Change – SDG 13 | - AMS-I.D. Grid Connected renewable electricity generation” (Version 18) |
| - Quality of employment - SDG 8 | - Number of employees participated to training programme |
| - Quantitative employment and income generation – SDG 8 | - No of jobs created by this project |
| Access to affordable and clean energy services – SDG 7 | Net electricity generation according to the electricity measuring meter |
| Livelihood of the poor – SDG 8 | Monthly pay sheets of existing employees throughout the monitoring period |
| Maintain minimum ecological flow – SDG 14 | Monitoring by CEA(Central Environmental Authority) |

A.3	Data and parameters fixed ex ante for monitoring contribution to each of the three SDGs

(Include a compilation of information on the data and parameters that are not monitored during the crediting period but are determined before the design certification and remain fixed throughout the crediting period like IPCC defaults and other methodology defaults. Copy this table for each piece of data and parameter.)

| Relevant SDG Indicator | SDG 13 |
| --- | --- |
| Data/parameter | GHG emission reduction |
| Unit | tCO2e |
| Description | Generation of electricity using reneawable energy souces by replacing - fuel oil and diesel based generating sources |
| Source of data | Monthly electricity bills (import and export) |
| Value(s) applied | 5538 tCO2e |
| Choice of data or Measurement methods and procedures | Once per monitoring period |
| - Purpose of data | To claim carbon credits and monitor SDG |
| Additional comment | N/A |

| Relevant SDG Indicator | SDG 8 |
| --- | --- |
| Data/parameter | Quality of employment |
| Unit | N/A |
| Description | New staff will be trained by generating awareness of health and safety. |
| Source of data | Regular training certificates provided by Campion Energy Service Pvt Ltd |
| Value(s) applied | 10 employees were participated to training program. (Refer annex 01) |
| Choice of data or Measurement methods and procedures | Once per verification period |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

| Relevant SDG Indicator | SDG 8 |
| --- | --- |
| Data/parameter | Quantitative employment and income generation |
| Unit | N/A |
| Description | No of job created by Campion Energy Service Pvt Ltd |
| Source of data | One month Pay sheets for all employees and employment contracts (Refer Annex 03) |
| Value(s) applied | 10 |
| Choice of data or Measurement methods and procedures | Once a year |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

| Relevant SDG Indicator | SDG 7 |
| --- | --- |
| Data/parameter | Access to affordable and clean energy services |
| Unit | MWh |
| Description | Net electricity generation by project activity |
| Source of data | net electricity generation according to the electricity export and import meter |
| Value(s) applied | 7,680 MWh |
| Choice of data or Measurement methods and procedures | monthly |
| Purpose of data | Monitor SDG |
| Additional comments | Net electricity generation is measured in kWh and converted into MWh |

| Relevant SDG Indicator | SDG 8 |
| --- | --- |
| Data/parameter | Livelihood of the poor |
| Unit | N/A |
| Description | No of job created by Campion Energy Service Pvt Ltd |
| Source of data | One month Pay sheets for all employees and employment contracts |
| Value(s) applied | 10 |
| Choice of data or Measurement methods and procedures | Once a year |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

SECTION B Safeguarding Principles Assessment

B.1 Analysis of social, economic and environmental impacts

>> (Refer the GS4GG Safeguarding Principles and Requirements document for detailed guidance on carrying out this assessment. The assessment of following Safeguarding Principles Assessment is required to be carried out by GS Version 2.0, 2.1 and 2.2 projects. GS v1.0 projects will carry out assessment of all the safeguarding principles discussed in the GS4GG Safeguarding Principles and Requirements document.)

| Safeguarding principles | Assessment questions | Assessment of relevance to the project (Yes/potentially/no) | Justification | Mitigation measure (if required) |
| --- | --- | --- | --- | --- |
| 4.1 Sites of Cultural and Historical Heritage | Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture | No | A preliminary consultation has resulted in positive feedback from local villagers. No cultural property is endangered by the project. | N/A |
| 4.3 Land Tenure and Other Rights | Does the Project require any change to land tenure arrangements and/or other rights? | No | This is not relevant for this project as no resettlement. | N/A |
| 6.2 Negative Economic Consequences | Does the project cause negative economic consequences during and after project implementation, e.g., for vulnerable and marginalised social groups in targeted communities? | No | 100 unskilled employees from Bogawana, Campion, Loinerno, and Kotiyagala Estates worked during the construction. In addition, 10 of those identified employees have been given permanent jobs at the power plant. | N/A |
| 7.1 Emissions | Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | The project does not have any negative impact on GHG emissions. Also the project utilises hydropower resources for generating electricity, the operation of the plant shall mitigate GHG emissions. | N/A |
| 7.2 Energy Supply | Will the Project use energy from a local grid or power supply that provides for other local users? | No | The electricity generated in the plant is fed into the national grid through the local grid. This leads to a high probability of improving the grid stability and availability of electricity to consumers, including local consumers (villagers). However, as the project contributes 1 MW to the grid. | N/A |
| 8.1 Impact on natural water patterns and flow | Will the Project affect the natural or pre-existing pattern of watercourses, groundwater and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | minimum water flow requirements have been prescribed for the project activity according to the CEA requirement. Minimal ecological flow is maintained as recorded in IEE (Annex 09) | N/A |
| 8.2 Erosion and/or water body stability | Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? | No | Erosion and water body stability is not affected by the project. | N/A |
| 9.1 Landscapte modification and soil | Does the Project involve the use of land and soil for production of crops or other products? | No | Soil excavations during the construction of the power plant were used for land filling for nursery and for making access road to power state. There is no any significant change to the soil condition before or after the project activity. | N/A |
| 9.2 Vulnerability to Natural Disaster | Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | N/A | N/A |
| 9.4 Release of pollutants | Could the project potentially result in the release of pollutants to the environment? | No | There is no change to the noise level before or after the project activity. Dynamite blasting is not allowed. Protective gears were provided to workers during construction (Ear Plugs, Masks, Safety Shoes) and regular maintenance of machinery | N/A |
| 9.5 Hazardous and Non-hazardous Waste | Will the Project involve the manufacture, trade, release, and/or use of hazardous and non- hazardous chemicals and/or materials? | No | There is no any impact on hazardous and non-hazardous Waste before or after the project activity. | N/A |
| 9.6 Pesticides and fertilizers | Will the Project involve the application of pesticides and/or fertilisers? |  | There is no any involvement on pesticides and fertilizers before or after the project activity. |  |
| 9.9 Animal Husbandry | Will the Project involve animal husbandry? | No | There is no significant change to the livelihood of animals before or after the project activity. Aquatic life is not affected when compared to the baseline scenario. A minimal ecological flow is always maintained according to the CEA Requirement. | N/A |

SECTION C	Monitoring plan

C.1	Data and parameters to be monitored

(Include specific information on how the data and parameters that need to be monitored in the selected methodology(ies) or proposed approaches or as per mitigation measures from safeguarding principles assessment or as per feedback from stakeholder consultations would actually be collected during monitoring. Copy this table for each piece of data and parameter.)

| Relevant SDG Indicator | SDG 7 – Renewable energy share in the total final energy consumption |
| --- | --- |
| Data/parameter: | EGBL,y |
| Unit | MWh/year |
| Description | Quantity of net electricity supplied to the grid as a result of the implementation of the CDM project activity (Safeguarding Principle 7 - Climate and Energy) |
| Measured/calculated/default | Measured |
| Source of data | Separate monthly invoice issued by the CEB to the power developer certifying the receipt of power from the power plant based on the export meter reading and the import of power from the CEB grid based on the import meter reading. The net electricity supplied to the grid is calculated as the difference between the export and import. |
| Value(s) of monitored parameter | 2017(May to December); 2533. 2018; 4859. 2019(January to February); 287. |
| Monitoring equipment | The electricity exported to the grid is measured by the main meter and check meter installed at the power plant. The electricity imported to the grid is measured by the main meter and check meter installed at the power plant site. Both the readings are recorded monthly at an agreed date and time by both the PP and CEB in the presence of PP representatives and personnel from CEB. Based on the readings, invoices are issued for export and import of electricity separately. The main and check meters are installed and sealed by the CEB. Only main meter readings are taken into account and in case of main meter failure check meter readings is considered for that particular month. Continuous Monitoring Monthly recording |
| Measuring/reading/recording frequency: | monthly |
| Calculation method (if applicable): |  |
| QA/QC procedures: | Ceylon Electricity Board (CEB) installed a main meter and if there is a failure of that meter, a backup meter (check meter) reading is taken. Accuracy: Acceptable to CEB - Accuracy class 1 as specified by Ceylon Electricity Board. Calibration of the above meters are done once a year. In cases where the CEB does not initiate meter calibration in line with the 1-year calibration frequency of the main meter, monthly energy reading of the main and check meters is compared from the due date of the calibration of the main meter. Then, lowest out of two readings is taken as the monthly net electricity supplied to the grid during the delayed calibration period. For the period of delayed calibration, the amount of net electricity supplied to the grid will be adjusted according to the guidelines provided in appendix-calibration of CDM validation and verification standard for Project Activities (version 02.0). Calibration Details Accuracy class; class 1. CEB serial No; DD2-9205435. Meter serial No; 214586122. Date of Calibration; 2018-12-28. Due date of calibration; 2019-12-27. Delayed period; 245 days. |
| Purpose of data: | Baseline calculation |
| Additional comments: | Net electricity generation will be measured in kWh and converted into MWh |

| Relevant SDG Indicator | SDG 8- Annual growth rate of real GDP per employed person |
| --- | --- |
| Data/parameter | Quantitative employment and income generation/ Livelihood of the poor |
| Unit | N/A |
| Description | No of job created by Campion Energy Service Pvt Ltd and their salary(Safe guarding Principle 6 – Economic Impacts) |
| Measured/ Calculated/Default | monitored |
| Source of data | Monthly pay sheets of existing employees throughout the monitoring period (Refer Annex 03) |
| Value(s) of monitored parameter | 10 |
| Monitoring equipment | N/A |
| Measuring/reading/recording frequency: | monthly |
| Calculation method (if applicable): | N/A |
| QA/QC Procedure | Hardcopies of pay sheets can be monitored and interview with employees |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

| Relevant SDG Indicator | SDG 13- Number of countries that have communicated the establishment or operationalization of an integrated policy/strategy/plan which increases their ability to adapt to the adverse impacts of climate change, and foster climate resilience and low greenhouse gas emissions development in a manner that does not threaten food production (including a national adaptation plan, nationally determined contribution, national communication, biennial update report or other) |
| --- | --- |
| Data/parameter | GHG emission reduction |
| Unit | tCO2e |
| Description | GHG emission reduction of electricity generation using reneawable energy souces by replacing fuel oil and diesel based generating sources (Safeguarding Principle 7 – Climate and Energy) |
| Measured/calculated/default | Calculated |
| Source of data | Separate monthly invoice issued by the CEB to the power developer certifying the receipt of power from the power plant based on the export meter reading and the import of power from the CEB grid based on the import meter reading. The net electricity supplied to the grid is calculated as the difference between the export and import. Grid Emission factors published by Sustainable Energy Authority of Sri Lanka - (Sri Lanka Energy Balance 2015) |
| Value(s) of monitored parameter | 2017(May to December); 2533. 2018; 4859. 2019(January to February); 287. - Operating margin CO2 emission factor for grid connected power generation in year y = 0.6938 tCO2e/ MWh - Build margin CO2 emission factor for grid connected power generation = 0.7490 tCO2e/ MWh - Combined margin CO2 emission factor for grid connected power generation= 0.7210 tCO2e/ MWh |
| Monitoring equipment | The electricity exported to the grid is measured by the main meter and check meter installed at the power plant. The electricity imported to the grid is measured by the main meter and check meter installed at the power plant site. Both the readings are recorded monthly at an agreed date and time by both the PP and CEB in the presence of PP representatives and personnel from CEB. Based on the readings, invoices are issued for export and import of electricity separately. The main and check meters are installed and sealed by the CEB. Only main meter readings are taken into account and in case of main meter failure check meter readings is considered for that particular month. Continuous Monitoring Monthly recording - (Sri Lanka Energy Balance 2015) is refered |
| Measuring/reading/recording frequency: | monitoring period |
| Calculation method (if applicable): | calculated by using “AMS-I.D. Grid Connected renewable electricity generation” (Version 18), |
| - QA/QC procedures: | Calibration of meters are done by CEB |
| - Purpose of data | To claim carbon credits and monitor SDG |
| Additional comment | N/A |

| Relevant SDG Indicator | SDG 8 - Frequency rates of fatal and non-fatal occupational injuries, by sex and migrant status |
| --- | --- |
| Data/parameter | Quality of employment |
| Unit | N/A |
| Description | Staff was trained by generating awareness of health and safety. (Safeguarding Principle 3 – Community Health, Safety and Working Conditions) |
| Measured/calculated/default | Monitored |
| Source of data | Regular training certificates provided by Campion Energy Service Pvt Ltd |
| Value(s) monitored parameter | 10 employees were participated to training program. (Refer annex 01) |
| Monitoring equipment | N/A |
| Measuring/reading/recording frequency: | Once per verification period |
| Calculation method (if applicable): | N/A |
| QA/QC procedures: | Interview with employees |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

| Relevant SDG Indicator | SDG 6 – Change in the extent of water-related ecosystems over time |
| --- | --- |
| Data/parameter | Water quality and quantity |
| Unit | Water quantity (m3) |
| Description | Maintain the Minimum ecological flow and monitored it by a third party(CEA – Central Environmental Authority) (Safeguarding Principle 8 - Water) |
| Measured/calculated/default | Monitored |
| Source of data | Report of minimum ecological flow |
| Value(s) of monitored parameter | Water quantity |
| Monitoring equipment | N/A |
| Measuring/reading/recording frequency: | Once a year |
| Calculation method (if applicable): | N/A |
| QA/QC procedures: | Measurement and verification by CEA |
| Purpose of data | Monitor SDG |
| Additional comments | N/A |

C.1.1	Other elements of monitoring plan (if applicable)

>>

- SECTION D	Duration and crediting period

D.1	Duration of project

D.1.1	Start date of project

>> (Specify start date of the project, in the format of DD/MM/YYYY)

05/07/2016

D.1.2	Expected operational lifetime of project

>> (Specify in years)

20 years

D.1	GS Crediting period of the project/activity

D.2.1	Start date of the ongoing GS crediting period

>> (Specify in dd/mm/yyyy)

12/05/2017

D.2.3	End date of the ongoing GS crediting period

>> (Specify in dd/mm/yyyy)

12/05/2024

D.2.3	Total length of the GS crediting periods

>> (Specify the total length of crediting period in years in line with GS4GG Principles & Requirements or relevant activity requirements)

7 years

- SECTION E	Stacking of new assets

- >> ( If project is looking to stack new assets over GSVERs the required information to demonstrate compliance to the relevant methodology, product specification and additionality shall be presented in the new PDD template launched with GS4GG)

N/A

Contact information of project participants

| Organization name | Bogawanthalawa Tea Estate PLC |
| --- | --- |
| Registration number with relevant authority | PQ 124 |
| Street/P.O. Box | 153, Nawala Road |
| Building | 2nd Floor, “Sayuru Sevana” |
| City | Narahenpita |
| State/Region | Colombo 05 |
| Postcode | 00500 |
| Country | Sri Lanka |
| Telephone | (+94) 112 369 845 |
| Fax |  |
| E-mail | thusithayapa@bpl.lk |
| Website | www.bogawantalawa.com |
| Contact person | Thusitha Bandara |
| Title | Mr. |
| Salutation | Director - Sustainability and R&D |
| Last name | Yapa |
| Middle name | Bandara |
| First name | Thusitha |
| Department | Sustainability and R&D |
| Mobile | (+94) 718 510 116 |
| Direct fax |  |
| Direct tel. |  |
| Personal e-mail | thusithayapa@bpl.lk |