Gold Standard for the Global Goals

Transition Annex

(To be used by all GS CDM/VER stand alone projects and PoAs, Micro Scale stand alone projects and Micro PoAs)

Version 1 – September 2017

KEY PROJECT INFORMATION

| Title of Project/PoA/Activity: | Expanding Access to LPG in Haiti through microfinance services |
| --- | --- |
| GS ID of the project/PoA/activity: | GS2564 |
| GS Version: | GS4GG |
| Brief description of Project: | In Haiti more than 90% of the population uses traditional, inefficient cookstoves fired with charcoal, which has a significant impact on deforestation. Entrepreneurs du Monde is implementing a progam to distribute improved cookstoves (ICS) using LPG to reduce this impact, which helps to reduce deforestation and GHG emissions through a cleaner and more efficient cooking method. Further, the use of such ICS reduces the incidence of respiratory diseases resulting from indoor air pollution and helps to ensure universal access to affordable, reliable and modern energy services. |
| Project type: Energy/Land Use | Energy |
| For Renewable Energy Projects – intention to apply RECs Labels (y/n) | No |
| GS Stream (CDM/VER): | VER |
| Scale (large/scale/micro): | Micro |
| GS Registration Date: | Registration date: 09/08/2016 Design change approved: 18/07/2017 |
| GS Crediting period start date: | 01/01/2015 |
| CDM Registration Date: | N/A |
| CDM Crediting period start date: | N/A |
| Project Developer: | Entrepreneurs du Monde |
| Project Representative: | Thomas Thivillon |
| Project Participants and any communities involved: |  |
| Host Country/Location: | Haiti |
| Methodologies applied: | Technologies and Practice to Displace Decentralized Thermal Energy Consumption (TPDDTEC), version 2 |
| SDG Impacts: | 1 – SDG3: Good Health and Well-Being 2 – SDG7: Affordable and Clean Energy. 3 – SDG13: Climate Action. |
| Estimated amount of SDG Impact (GSVERs and others) | SDG13: 86,229 over 10 years |

NOTE: This Annex shall be used for all PoAs if the sustainable development assessment is conducted at PoA level. In case sustainable development assessment is conducted at activity level, then this Annex shall be filled for each of the activities.

SECTION A Sustainable Development Goals (SDG) outcomes

A.1	Relevant target for each of the three SDGs

SDG3: Good Health and Well-Being

By 2030, reduce by one third premature mortality from non-communicable diseases through prevention and treatment and promote mental health and well-being

SDG7: Affordable and Clean Energy

By 2030, ensure universal access to affordable, reliable and modern energy services

SDG13: Climate Action

Integrate climate change measures into national policies, strategies and planning

A.2	Explanation of methodological choices/approaches for estimating the SDG outcome

| SDG | Project outcome | Baseline outcome | Proposed approaches for estimating the SDG outcome |
| --- | --- | --- | --- |
| SDG #3: Good health and well-being | LPG stoves are among the cleaneast cooking technology available, with a particulate matter formation potential over 50 times lower than charcoal stoves, which directly results in improved health conditions in the households and small businesses. Chiang and Johnson have shown that sustained use of Tier 4 stove (like LPG stove) reduce risk factor of cardiovascular disease associated with household air pollution. | In the baseline, the households would continue using the traditional charcoal fuelled cookstoves with high indoor pollution and a particulate matter formation potential. Air pollution from cooking with solid fuel is a key risk factor for childhood pneumonia as well as many other respiratory, cardiovascular and ocular diseases for people exposed to the smoke in households. | Monitoring surveys with a representative sample are conducted to assess the perception of impact on symptoms of coughing and respiratory desease with the ISC compared to the baseline cookstoves. This allows to determine the % of households that observe a reduction of such symptoms as a result from improved respiratory conditions and air pollution. |
| SDG #7: Affordable and Clean Energy. | LPG stove is a clean cooking technology. The project activity helps to increase the share of the population with primary reliance on clean fuels and technology. | Without the project activity, the households would continue using the traditional charcoal cookstoves that cannot classified as clean cookstoves. | The total stove days per year will be monitored as a measure for the active use of clean cookstoves technologies. |
| SDG #13: Climate action | Haiti is classiefied as a least-developed country (LDC). Through the implementation of the project activity, it receives specialized support, including finance, technology and capacity-building. The project activity will generate average emission reductions of around 8,462 tCO2 per year over the 10-year crediting period. | In the absence of the project activity, GHG emission levels from using non-renewable biomass would be higher. Without the project, no similar training and capacity building activities would be conducted. | The emission reductions are monitored and determined as per the applied methodology and monitoring procedure outlined in the PDD. |

SDG3: Good Health and Well-Being: 3.4.1 Mortality rate attributed to cardiovascular disease, cancer, diabetes or chronic respiratory disease

Improved conditions for cooking, i.e. decreased smoke and impacts shall be assessed based on the monitoring surveys in order to estimate the net benefit from the usage of the stoves.

SDG7: Affordable and Clean Energy: 7.1.2 Proportion of population with primary reliance on clean fuels and technology

The total number of days that the stoves will be used will reflect the growing access to cleaner fuels. This will be done by the monitoring surveys.

SDG13: Climate Action: 13.2.2 Total greenhouse gas emissions per year.

Emission reductions are verified and credited by comparing the emissions for a given project scenario to the emissions for the applicable baseline scenario following the guidelines established in the latest version of the Gold Standard methodology "Technologies and Practices to Displace Decentralized Thermal Energy Consumption” (TPDDTEC) .

The initial emissions profile of the baseline and project scenario is determined by the results of the respective baseline and project studies. Over the project period the results are updated and adjusted depending on results of the on-going monitoring.

Since the baseline fuel and the project fuel are different, the overall GHG reductions achieved by the project activity in year y are calculated as follows:

b,pNp,y*Up,y*fNRB,b,y*ERb,p,y,CO2+ERb,p,y,non-CO2-LEp,y

Where:

| ∑b,p | Sum over all relevant (baseline b/project p) couples |
| --- | --- |
| Np,y | Cumulative number of project technology-days included in the project database for project scenario p against baseline scenario b in year y |
| Up,y | Cumulative usage rate for technologies in project scenario p in year y, based on cumulative adoption rate and drop off rate (fraction) |
| ERb,p,y, CO2 | Specific CO2 emission savings for an individual technology of project p against an individual technology of baseline b in year y, in tCO2/day, and as derived from the statistical analysis of the data collected from the field tests |
| ERb,p,y , non-CO2 | Specific non-CO2 emission savings for an individual technology of project j against an individual technology of baseline b in year y, converted in tCO2/year, and as derived from the statistical analysis of the data collected from the field tests |
| f NRB,b, y | Fraction of biomass used in year y for baseline scenario b that can be established as non-renewable biomass (drop this term from the equation when using a fossil fuel baseline scenario) |
| LEp,y | Leakage for project scenario p in year y (tCO2e/yr) |

A.3	Data and parameters fixed ex ante for monitoring contribution to each of the three SDGs

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | EFb,CO2 |
| Unit | tCO2/TJ |
| Description | CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data | IPCC 2006 Vol2 Chap1 Table 1.4 |
| Value(s) applied | EFb,firewood,CO2= 112 tCO2/TJ ; EFb,LPG,CO2=63.1 tCO2/TJ ; EFb,charcoal = 747.04 tCO2/TJ ; EFb,kerosene,CO2= 19.6 tCO2/TJ |
| Choice of data or Measurement methods and procedures | For Firewood, LPG and Kerosene, the IPCC 2006 (Vol 2, Chap 1, Table 1.4) data have been applied. For charcoal, in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. Term can include a combination of emission factors from fuel production, transport, and use. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | EFb, nonCO2 |
| Unit | tCO2/TJ or tCO2/t_fuel |
| Description | Non-CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data | IPCC 2006 Vol2 Chap 2 Table 2.9 and latest GWP of CH4 and N2O |
| Value(s) applied | EFfw,nonCO2= 34.0 tCO2/TJ ; EFLPG,nonCO2= 0.03 tCO2/TJ EFb,charcoal,nonCO2 = 226.8 tCO2/TJ ; EFb,kerosene,nonCO2= 0.78 tCO2/TJ |
| Choice of data or Measurement methods and procedures | For Firewood and LPG and Kerosene, the IPCC 2006 (Vol 2, Chap 2, Table 2.9) data have been applied and multiplied by latest GWP data (GWPCH4= 25 and GWP N2O= 298) For charcoal, in accordance with the methodology, the value was estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | Term includes a combination of emission factors from fuel production, and use. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | EFp,CO2 |
| Unit | tCO2/TJ or tCO2/t_fuel |
| Description | CO2 emission factor arising from use of fuels in project scenario |
| Source of data | IPCC 2006 Vol2 Chap1 Table 1.4 |
| Value(s) applied | EFp,firewood,CO2= 112 tCO2/TJ ; EFp,LPG,CO2= 747.04 tCO2/TJ ; EFp,charcoal = 747.04 tCO2/TJ ; EFp,kerosene,CO2= 19.6 tCO2/TJ |
| Choice of data or Measurement methods and procedures | For Firewood and Kerosene, the IPCC 2006 (Vol 2, Chap 1, Table 1.4) data have been applied. For charcoal, to account for the production process in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. Because of the Gold Standard Foundation overall applicability criteria limiting the scope of fossil fuel switching projects to their energy efficiency component, the LPG Emission Factor value used here is equaled to the emission factor of charcoal, the fuel that is going to be displaced by the introduction of the stove. This will lead to a very conservative estimate of the actual emission reduction. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. Term can include a combination of emission factors from fuel production, and use. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | EFp, nonCO2 |
| Unit | tCO2/TJ or tCO2/t_fuel |
| Description | Non-CO2 emission factor arising from use of fuels in project scenario |
| Source of data | IPCC 2006 Vol2 Chap 2 Table 2.9 and latest GWP of CH4 and N2O (WGI AR5 2013 Table 8A.1 p.8-88) |
| Value(s) applied | EFfw,nonCO2= 34.0 tCO2/TJ ; EFLPG,nonCO2= 226.8 tCO2/TJ EFb,charcoal,nonCO2 = 226.8 tCO2/TJ EFb,kerosene,nonCO2= 0.78 tCO2/TJ ; |
| Choice of data or Measurement methods and procedures | For firewood and coal, the IPCC 2006 (Vol 2, Chap 2, Table 2.9) data have been applied and multiplied by latest GWP data (GWPCH4= 28 and GWP N2O= 265) For charcoal, in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. Because of the Gold Standard Foundation overall applicability criteria9 limiting the scope of fossil fuel switching projects to their energy efficiency component, the LPG Emission Factor value used here is equaled to the emission factor of charcoal, the fuel that is going to be displaced by the introduction of the stove. This will lead to a very conservative estimate of the actual emission reduction. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | Terms can include a combination of emission factors from fuel production and use. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | NCVb |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the baseline |
| Source of data | IPCC 2006 Vol 2, Chap 1, Table 1.2 |
| Value(s) applied | NCVb,fw = 0.0156 TJ/ton NCVb,LPG = 0.0473 TJ/ton NCVb,charcoal = 0.0296 TJ/ton NCVb,kerosene = 0.0438 TJ/ton |
| Choice of data or Measurement methods and procedures | These values are taken from IPCC Guidelines 2006 Vol 2, Chap 1, Table 1.2 |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | NCVp |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the project |
| Source of data | IPCC 2006 Vol 2, Chap 1, Table 1.2 |
| Value(s) applied | NCVb,fw = 0.0156 TJ/ton NCVb,LPG = 0.0473 TJ/ton NCVb,charcoal = 0.0296 TJ/ton NCVb,kerosene = 0.0438 TJ/ton |
| Choice of data or Measurement methods and procedures | These values are taken from IPCC Guidelines 2006 Vol 2, Chap 1, Table 1.2 |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | These values are taken from IPCC Guidelines 2006 Vol 2, Chap 1, Table 1.2 If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | fNRB,i,y |
| Unit | Fractional non-renewability |
| Description | Non-renewability status of woody biomass fuel in scenario i during year y |
| Source of data | registered PoADD GS2096. |
| Value(s) applied | 96% |
| Choice of data or Measurement methods and procedures | The biomass collection area of the project is the whole Haitian national territory. This value is derived from an assessment made at the national level. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | As applicable, NRB assessment may be used for multiple scenarios |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | Pb,y |
| Unit | kg/household-day |
| Description | Quantity of fuel that is consumed in baseline scenario p during year y. |
| Source of data | LPG, Kerosene, Firewood and Charcoal fuel consumption will be monitored. |
| Value(s) applied | Pcom,firewood= 0 kg/day/restaurant Pcom,charcoal= 6.46 kg/day/restaurant Pcom,LPG= 0 kg/day/restaurant Pcom,kerosene= 0.013 kg/day/restaurant Pdom,firewood= 0 kg/day/household Pdom,charcoal= 2.32 kg/day/household Pdom,LPG= 0.05 kg/day/household Pdom,kerosene= 0.08 kg/day/household |
| Choice of data or Measurement methods and procedures | Scales with a 100g accuracy has be used to weight the fuel. The BFT have been done before Palmis Eneji started promoting the stove included in the present project activity. |
| - Purpose of data | Assesment of SDG#13 |
| Additional comment | - |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data/parameter | Pb,y |
| Unit | kg/household-day |
| Description | Quantity of fuel that is consumed in baseline scenario p during year y. |
| Source of data | LPG, Kerosene, Firewood and Charcoal fuel consumption will be monitored. |
| Value(s) applied | Pcom,firewood= 0 kg/day/restaurant Pcom,charcoal= 6.46 kg/day/restaurant Pcom,LPG= 0 kg/day/restaurant Pcom,kerosene= 0.013 kg/day/restaurant Pdom,firewood= 0 kg/day/household Pdom,charcoal= 2.32 kg/day/household Pdom,LPG= 0.05 kg/day/household Pdom,kerosene= 0.08 kg/day/household |
| Choice of data or Measurement methods and procedures | Scales with a 100g accuracy has be used to weight the fuel. The BFT have been done before Palmis Eneji started promoting the stove included in the present project activity. |
| - Purpose of data | Assessment of SDG#13 |
| Additional comment | - |

SECTION B Safeguarding Principles Assessment

B.1 Analysis of social, economic and environmental impacts

| Safeguarding principles | Assessment questions | Assessment of relevance to the project (Yes/potentially/no) | Justification | Mitigation measure (if required) |
| --- | --- | --- | --- | --- |
| 3.2 Gender Equality and Women’s Rights | 1- Is there a possibility that the Project might reduce or put at risk women’s access to or control of resources, entitlements and benefits? 2- Is there a possibility that the Project can adversely affect men and women in marginalised or vulnerable communities (e.g., potential increased burden on women or social isolation of men)? 3- Is there a possibility that the Project might not take into account gender roles and the abilities of women or men to participate in the decisions/designs of the project’s activities (such as lack of time, child care duties, low literacy or educational levels, or societal discrimination)? 4- Does the Project take into account gender roles and the abilities of women or men to benefit from the Project’s activities (e.g., Does the project criteria ensure that it includes minority groups or landless peoples)? 5- Does the Project design contribute to an increase in women’s workload that adds to their care responsibilities or that prevents them from engaging in other activities? 6- Would the Project potentially reproduce or further deepen discrimination against women based on gender, for instance, regarding their full participation in design and implementation or access to opportunities and benefits? 7- Would the Project potentially limit women’s ability to use, develop and protect natural resources, taking into account different roles and priorities of women and men in accessing and managing environmental goods and services? 8- Is there a likelihood that the proposed Project would expose women and girls to further risks or hazards? | 1- No 2- No 3- No 4-No 5-No 6-No 7-No 8-Potentialy | 1- The project does not or put at risk women’s access to or control of resources, entitlements and benefits 2- The project makes cooking easier for women (LPG stove can be started instantly unlike charcoal where it takes a while to start the fire). 3- both women and men were represented in the Stakeholder consultation meeting (see section D of PDD) 4- The project target all Port au Prince, including some of the poorest neighbourhood (like Cite Soleil). 5- Cooking with LPG is faster than cooking with charcoal or firewood. 6- Women are the primary target of the proget of the project (since they do most of the cooking). 7- The project does not impact access or mangment or environmental goods or services. 8- LPG can pose some security concerns. | 8- A LPG security briefing has been integrated into the usage survey. This will be repeated during every survey throughout the crediting period, to ensure that the users are maintaining good practices. |
| 3.4.3 Land Tenure and Other Rights | 1- Does the Project require any change to land tenure arrangements and/or other rights? 2- For Projects involving land use tenure, are there any uncertainties with regards land tenure, access rights, usage rights or land ownership? | 1- No 2- No | 1- The project does not involve land tenure changes. 2- The project does not involve land tenure changes. |  |
| 3.6.2 Negative Economic Consequences | 1- The Project Developer shall demonstrate the financial sustainability of the Projects implemented, also including those that will occur beyond the Project Certification period. 2- The Projects shall consider economic impacts and demonstrate a consideration of potential risks to the local economy and how these have been taken into account in Project design, implementation, operation and after the Project. Particular focus shall be given to vulnerable and marginalised social groups in targeted communities and that benefits are socially inclusive and sustainable. | 1- No | 1- Palmis Eneji is a social business. It strive to achieve financial sustainability. The stove promoted by the project are purchased by the households. 2- The project relies on a network of micro franchised entrepreneurs to do the distribution of the stove. It provides economic opportunities to these resellers. |  |
| 4.1.1 Emissions | Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | - As per the registered PDD, GHG emission is expected to be reduced by 86,229 tCO2e over the crediting period |  |
| 4.1.2 Energy Supply | Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | The project does not use local source of energy. |  |
| 4.2.1 Impact on natural water patterns and flow | Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | The project does affect water resources or availability. |  |
| 4.2.1 Erosion and/or water body stability | Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? | - No | The project does affect water resources or availability.By reducing pressure on biomass, it will reduce deforestation and associated erosion. |  |
| 4.2.3 Landscapte modification and soil | Does the Project involve the use of land and soil for production of crops or other products? | No | No crops are used in the project |  |
| 4.3.2 Vulnerability to Natural Disaster | Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | None. By reducing pressure on biomass and deforestation the project will likely reduce erosion and associated landslide. |  |
| 4.3.3 Genetic Resources | Could the Project be negatively impacted by the use of genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development)? | No | The project does not involve the use of crop. |  |
| 4.3.4 Release of pollutants | Could the Project potentially result in the release of pollutants to the environment? | No | LPG reduce the emission of Black Carbon, Particulate Matter and Carbon monoxide compared to the baseline fuel (charcoal and firewood) |  |
| 4.3.5 Hazardous and Non-hazardous Waste | Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | No | The project use LPG imported by international companies independly from the project. The LPG used in the project is already available in Haiti. The combustion of LPG release CO2, CO and PM, but the quantity realeased into the atmosphere are lower than those associated with the baseline fuels (firewood, charcoal). |  |
| 4.3.6 Pesticides and fertilizers | Will the Project involve the application of pesticides and/or fertilisers? | No | No pesticide or fertilizer are needed. |  |
| 4.3.7 Harvesting of forests | Will the Project involve the harvesting of forests? | No | No forest product are needed in the project |  |
| 4.3.8 Food | Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | The project has no impact on food. |  |
| 4.3.9 Animal Husbandry | Will the Project involve animal husbandry? | No | The project does not involve animal husbandry. |  |

SECTION C	Monitoring plan

C.1	Data and parameters to be monitored

| Relevant SDG Indicator | 3.4.1 Mortality rate attributed to cardiovascular disease, cancer, diabetes or chronic respiratory disease |
| --- | --- |
| Data / Parameter | Share of users that perceive a reduced smoke in the operating area |
| Unit | % |
| Description | Share of users that perceive a reduced amount of smoke that result in improved living conditions and contribute to a reduction or mortality rates attributed to chronic respiratory disease and air pollution. |
| Source of data | Survey conducted during ICS monitoring visits |
| Value(s) applied | -- |
| Measurement methods and procedures | Data collected through monitoring survey. People are asked about the amount of smoke inside the house when using the ISC compared to the baseline surveys results. |
| Monitoring frequency | Annually |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #3 |
| Additional comment | The SDG mortality rate indicators are indirectly derived from this information, as mortality rates are intimately related to the air quality. If the air quality improves, the mortalities rates drop |

| Relevant SDG Indicator | Security measures for users around LPG |
| --- | --- |
| Data / Parameter | Share of users that perceive a reduction on physical burnings due to the use of the new LPG models compared to the baseline stoves. |
| Unit | % |
| Description | Share of users that perceive a reduction on physical burnings due to the use of the new LPG models compared to the baseline stoves are results in improved living conditions and contribute to a reduction o risks in the operacion area. |
| Source of data | Survey conducted during ICS monitoring visits |
| Value(s) applied | - |
| Measurement methods and procedures | Data collected through monitoring survey. People are asked about the amount the frecuency of burnings when using the ISC compared to the baseline surveys results. |
| Monitoring frequency | Annually |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Evaluate which mitigation measures should be implemented. |
| Additional comment | - |

| Relevant SDG Indicator | 7.1.2 Proportion of population with primary reliance on clean fuels and technology |
| --- | --- |
| Data / Parameter | Total stove days per year |
| Unit | - |
| Description | Number of total ISC days that reflects the active usage of clean cookstoves and the access to cleaner fuels |
| Source of data | Survey conducted during ICS monitoring visits and project database |
| Value(s) applied | - |
| Measurement methods and procedures | Data collected through monitoring survey. People are asked about the amount of smoke inside the house when using the ISC compared to the baseline surveys results. |
| Monitoring frequency | Annually |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #7 |
| Additional comment | The use of the LPG stoves demonstrate easy access to clearner fuels when compare with the baseline fuel. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data / Parameter | ERb,p,y,CO2 |
| Unit | tCO2/day |
| Description | Specific CO2 emission savings for an individual technology of project p against an individual technology of baseline b in year y. |
| Source of data | Calculated |
| Value(s) applied | - |
| Measurement methods and procedures | Calculated based on statistical analysis of the data collected from the field tests |
| Monitoring frequency | Updated at least every two years |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #13 |
| Additional comment | A single project fuel consumption parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data / Parameter | Pp,y |
| Unit | kg/household-day, kg/person-meal, etc. |
| Description | Quantity of fuel that is consumed in project scenario p during year y. LPG, Kerosene, Firewood and Charcoal fuel consumption will be monitored. |
| Source of data | Total sales record, Project FT, project FT updates, and any applicable adjustment factors |
| Value(s) applied | See paragraph B.6.4 of the PDD |
| Measurement methods and procedures | Scales with a 100g accuracy will be used to weight the fuel. The PFT will be done in the second half of the crediting year among a sample selected to insure proper representation of stove age (implicit stratification basic on sale date). |
| Monitoring frequency | Updated at least every two years |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #13 |
| Additional comment | A single project fuel consumption parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data / Parameter | Up,y |
| Unit | Percentage |
| Description | Usage rate in project scenario p during year y |
| Source of data | Annual usage survey |
| Value(s) applied | See paragraph B.6.4 of the PDD |
| Measurement methods and procedures | In this section the project participants shall provide a description of equipment used for measurement, if applicable, and its accuracy class. |
| Monitoring frequency | Annually or more frequently, in all cases on time for any request for issuance |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #13 |
| Additional comment | A single usage parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario. |

| Relevant SDG Indicator | 13.2.2 Total greenhouse gas emissions per year |
| --- | --- |
| Data / Parameter | Np,y |
| Unit | Project technologies credited (units) |
| Description | Technologies in the project database for project scenario p through year y |
| Source of data | Total sales record |
| Value(s) applied | See paragraph B.6.4 |
| Measurement methods and procedures | In this section the project participants shall provide a description of equipment used for measurement, if applicable, and its accuracy class. |
| Monitoring frequency | Continuous |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Assesment of SDG #13 |
| Additional comment | The total sales record is divided based on the project scenario to create the project database |

C.1.1	Other elements of monitoring plan (if applicable)

N/A

- SECTION D	Duration and crediting period

D.1	Duration of project

D.1.1	Start date of project

27/11/2013

D.1.2	Expected operational lifetime of project

15 years

D.1	GS Crediting period of the project/activity

Fixed (10 years)

D.2.1	Start date of the ongoing GS crediting period

01/01/2015

D.2.3	End date of the ongoing GS crediting period

31/12/2024

D.2.3	Total length of the GS crediting periods

10 years

- SECTION E	Stacking of new assets

None

Contact information of project participants

| Organization name | Entrepreneurs du Monde |
| --- | --- |
| Registration number with relevant authority |  |
| Street/P.O. Box | 4 allée du Textile |
| Building |  |
| City | Vaulx-en-Velin |
| State/Region | Rhône-Alpes |
| Postcode | 69 120 |
| Country | France |
| Telephone | +33 (0)4 37 24 76 50 |
| Fax |  |
| E-mail | thomas.thivillon@entrepreneursdumonde.org |
| Website | http://www.entrepreneursdumonde.org/ |
| Contact person | Thomas Thivillon |
| Title | Head of energy programs |
| Salutation | Mr. |
| Last name | Thivillon |
| Middle name |  |
| First name | Thomas |
| Department | Energy |
| Mobile |  |
| Direct fax |  |
| Direct tel. | Tel. +33 (0)6 10 48 24 98 |
| Personal e-mail | thomas.thivillon@entrepreneursdumonde.org |

http://www.cleancookingalliance.org/assets-facit/Comparative-Analysis-for-Fuels-FullReport.pdf (page 1-10)

https://www.ncbi.nlm.nih.gov/pmc/articles/PMC4529010/

See Toolkit Annex C for more details