# GS 2564 Monitoring Report

# Third Monitoring Report

Prepared for Entrepreneurs du Monde by Olivier Lefebvre

VER claimed: 10,728. Vintage 2017: 7,917. Vintage 2018: 2811

Monitoring period: February 1st 2017 to March 31st 2018

# Introduction

The monitoring effort was comprised of two different surveys looking with increased precision at the energy usage of the LPG stove users:

- The usage survey was addressed to the largest sample and aimed at establishing the usage rate and the overall satisfaction of the Palmis Eneji customers. This survey included people who stopped using their stove.

- The monitoring survey focused on people who were still using their stove and investigated their energy use patterns and the other benefits arising from the usage of the LPG stove.

The present report synthesizes the results from those surveys (done in 2018) and from the baseline surveys (done before the project implementation in 2014) to give the reader an understanding of the actual climate and socio-economic impact of the project.

The present report present result of the monitoring effort done to evaluate the impact of stove geared toward domestic use.

Version History

v1 submitted to GS for review on 29 June 2018

v2 submitted to GS for review on 20 September 2018

v3 submitted to GS for review on October 22nd 2018

Table of Contents

Introduction	2

I.	Number of stoves included in the project for the first monitoring year	5

A.	Stove sold and Stove.day equivalent	5

B.	Unique Identification and carbon right waiver	5

II.	Summary of the data source used for this report	6

1.	History of the survey carried out up to date	6

2.	Sample size of the survey carried out for the second monitoring period	6

III.	Calculation of Emission Reductions	7

A.	Overall methodology used	7

B.	Information on other LPG based projects in the targeted area	7

C.	Equation used and value applied	7

1.	Equation used	7

2.	Value available at PDD validation	8

3.	Value of parameter that were monitored	12

D.	Main findings	14

1.	Usage rate and Usage purpose	14

2.	Type and quantity of fuel used	15

3.	Seasonal variation in fuel consumption	17

E.	Green House Gas Emissions calculations	17

1.	Difference of mean	17

2.	90/30 check and VER adjustment	18

3.	VER at the project scale	18

4.	Breakdown of VER per vintage	19

5.	Comparison with Ex-ante VER estimation	19

IV.	Overall socio-economic impacts from the project	19

Annex I: Contact information on participants in the project activity	21

# Number of stoves included in the project for the first monitoring year

## Stove sold and Stove.day equivalent

In accordance with the monitoring methodology requirements, a project database was maintained with the relevant information for each stove: sale date, place, name of customer, stove serial number, receipt number, size of the stove sold, intended usage (domestic/commercial).

The project database is available in the project supporting documentation. A total of 2,030 LPG stove (geared toward domestic use) are included into the project.

However, they were not all sold before the start of the monitoring year. To account for that we calculated the number of days they have been used. The 2,037 stoves included in to the project have been used a combined 556,408 stove days.

## Unique Identification and carbon right waiver

The unique identification of the stove included into the project is done through the sale receipts of the stoves. Each sale receipt mentions the sales date, stove type and quantity and his signed by the buyer. Only stove with their corresponding sales receipt are included into the project. All the sales receipts are available in the supporting documentation on the registry. Each sale receipt has a unique identifier and is referenced in the database.

The sale receipt to end users mention that “by buying the item, the client waives its carbon credit rights to Entrepreneurs du Monde”. When the sale went through a reseller, individual contract between the reseller and the wholesaler waive the carbon right of the reseller (see scan of one of those contracts on the registry).

# Summary of the data source used for this report

### History of the survey carried out up to date

The baseline survey and Baseline KPT were carry out before the start of implementation and the data from these surveys were included into the registered PDD.

| SOURCE OF DATA | Domestic Stove |
| --- | --- |
| Baseline Survey | registered PDD |
| Baseline KPT | registered PDD |
| Monitoring Survey | 3rd Monitoring Report |
| Usage Survey | 3rd Monitoring Report |
| Project KPT | 2nd Monitoring Report |
| Project Database | 3rd Monitoring Report |

### Sample size of the survey carried out for the second monitoring period

The table here under summarize the number of household surveyed as part of the third monitoring report.

|  | Domestic Stove | Date |
| --- | --- | --- |
| Monitoring Survey | 110 | March 2018 |
| Usage Survey | 116 | March 2018 |
| Project KPT (previous monitoring) | 83 | February 2017 |

Sampling methodology

We surveyed every end user that could be found. Finding end users is difficult in the project context for the following reasons:

- A lot of stove are sold through resellers who do not have a contact details of each of their client

- When we have the phone number of the end users, they sometimes have changed their number which make finding them impossible

Verification that the samples sizes meet the GS guideline

- Usage Survey: We did a total of 116 usage survey: 78 surveys for the stoves sold in 2016 and 37 for the stoves sold in 2017. This is above the required 30 surveys per sales years as well as the 100 usage surveys threshold required by the methodology. In average the stove survey from 2016 were 653 days old (which is greater than 1.5 years old) and the stove from 2017 were 321 days old (which is greater than 0.5 years old).

- Monitoring Survey: the database includes 2037 stoves used for domestic purpose. In the case the TPDDTEC methodology require that at least 100 monitoring survey are carried out. We met this requirement by carrying out 110 Monitoring Survey.

All the surveys were performed by a team of surveyors hired and managed by Palmis Eneji staff in Port-au-Prince, the study was designed and the data analyzed by Olivier Lefebvre from Climate Solutions Consulting.

# Calculation of Emission Reductions

## Overall methodology used

We followed guidance from the TPDDTEC methodology to calculate emission reductions:

- Compute baseline fuel consumption in kg/day

- Compute project fuel consumption in kg/day

- Calculate emission savings per day

- Calculate emission savings at the project scale by accounting for the number of stove-day and the usage rate.

- Calculate emissions mean and standard deviation so that we can check whether the one sided 90% confidence interval fits within +- 30% of the mean.

## Information on other LPG based projects in the targeted area

To the best of our knowledge, no other LPG based carbon offset project is active within the project boundaries. We have closely examined the Gold Standard, CDM and VCS registry and found that no similar project is taking place in Haiti.

## Equation used and value applied

### Equation used

We used the following equation for the VER calculation:

ERy = ∑b,p (Np,y* Up,y* Pp,b,y* NCVb, fuel * (f NRB,b, y * EFfuel, CO2  + EFfuel, nonCO2))– ∑ LEp,y        (1)

Where:

| ∑b,p | Sum over all relevant (baseline b/project p) couples |
| --- | --- |
| Np,y | Cumulative number of project technology-days included in the project database for project scenario p against baseline scenario b in year y |
| Up,y | Cumulative usage rate for technologies in project scenario p in year y, based on cumulative adoption rate and drop off rate revealed by usage surveys (fraction) |
| Pp,b,y | Specific fuel savings for an individual technology of project p against an individual technology of baseline b in year y, in tons/day, as derived from the statistical analysis of the data collected from the field tests |
| f NRB,b, y | Fraction of biomass used in year y for baseline scenario b that can be established as non-renewable biomass (drop this term from the equation when using a fossil fuel baseline scenario) |
| NCVb,fuel | Net calorific value of the fuel that is substituted or reduced (IPCC default for wood fuel, 0.015 TJ/ton) |
| EFb,fuel,CO2 | CO2 emission factor of the fuel that is substituted or reduced. 112 tCO2/TJ for Wood/Wood Waste, or the IPCC default value of other relevant fuel |
| EFb,fuel,nonCO2 Non-CO2 emission factor of the fuel that is reduced LEp,y Leakage for project scenario p in year y (tCO2e/yr) |  |

### Value available at PDD validation

| Data / Parameter: | EFb,CO2 |
| --- | --- |
| Data unit: | tCO2/TJ |
| Description: | CO2emission factor arising from use of fuels in baseline scenario |
| Source of data used: | IPCC 2006 Vol2 Chap1 Table 1.4 |
| Value applied: | EFb,firewood,CO2= 112 tCO2/TJ ; EFb,LPG,CO2=63.1 tCO2/TJ ; EFb,charcoal = 747.04 tCO2/TJ EFb,kerosene,CO2= 19.6 tCO2/TJ ; |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | For Firewood, LPG and Kerosene, the IPCC 2006 (Vol 2, Chap 1, Table 1.4) data have been applied. For charcoal, in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. |
| Any comment: | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. Term can include a combination of emission factors from fuel production, transport, and use. |

| Data / Parameter: | EFb, nonCO2 |
| --- | --- |
| Data unit: | tCO2/TJ or tCO2/t_fuel |
| Description: | Non-CO2 emission factor arising from use of fuels in baseline scenario |
| Source of data used: | IPCC 2006 Vol2 Chap 2 Table 2.9 and latest GWP of CH4 and N2O |
| Value applied: | EFfw,nonCO2= 34.0 tCO2/TJ ; EFLPG,nonCO2= 0.03 tCO2/TJ EFb,charcoal,nonCO2 = 226.8 tCO2/TJ EFb,kerosene,nonCO2= 0.78 tCO2/TJ ; |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | For Firewood and LPG and Kerosene, the IPCC 2006 (Vol 2, Chap 2, Table 2.9) data have been applied and multiplied by latest GWP data (GWPCH4= 25 and GWP N2O= 298) For charcoal, in accordance with the methodology, the value was estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. |
| Any comment: | Term includes a combination of emission factors from fuel production, and use. |

| Data / Parameter: | EFp,CO2 |
| --- | --- |
| Data unit: | tCO2/TJ or tCO2/t_fuel |
| Description: | CO2 emission factor arising from use of fuels in project scenario |
| Source of data used: | IPCC 2006 Vol2 Chap1 Table 1.4 |
| Value applied: | EFp,firewood,CO2= 112 tCO2/TJ ; EFp,LPG,CO2= 747.04 tCO2/TJ ; EFp,charcoal = 747.04 tCO2/TJ EFp,kerosene,CO2= 19.6 tCO2/TJ ; |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | For Firewood and Kerosene, the IPCC 2006 (Vol 2, Chap 1, Table 1.4) data have been applied. For charcoal, to account for the production process in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. Because of the Gold Standard Foundation overall applicability criteria limiting the scope of fossil fuel switching projects to their energy efficiency component, the LPG Emission Factor value used here is equaled to the emission factor of charcoal, the fuel that is going to be displaced by the introduction of the stove. This will lead to a very conservative estimate of the actual emission reduction. |
| Any comment: | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. Term can include a combination of emission factors from fuel production, and use. |

| Data / Parameter: | EFp, nonCO2 |
| --- | --- |
| Data unit: | tCO2/TJ or tCO2/t_fuel |
| Description: | Non-CO2 emission factor arising from use of fuels in project scenario |
| Source of data used: | IPCC 2006 Vol2 Chap 2 Table 2.9 and latest GWP of CH4 and N2O (WGI AR5 2013 Table 8A.1 p.8-88) |
| Value applied: | EFfw,nonCO2= 34.0 tCO2/TJ ; EFLPG,nonCO2= 226.8 tCO2/TJ EFb,charcoal,nonCO2 = 226.8 tCO2/TJ EFb,kerosene,nonCO2= 0.78 tCO2/TJ ; |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | For firewood and coal, the IPCC 2006 (Vol 2, Chap 2, Table 2.9) data have been applied and multiplied by latest GWP data (GWPCH4= 28 and GWP N2O= 265) For charcoal, in accordance with the methodology, the value has been estimated by multiplying the firewood Emission Factor by a charcoal to firewood ratio of 6.67 kg of firewood/kg of charcoal. This ratio is provided on page 21 of the ESMAP report “Haiti: Strategy to Alleviate the Pressure of Fuel Demand on National Woodfuel Resources” published in 2007. Because of the Gold Standard Foundation overall applicability criteria9 limiting the scope of fossil fuel switching projects to their energy efficiency component, the LPG Emission Factor value used here is equaled to the emission factor of charcoal, the fuel that is going to be displaced by the introduction of the stove. This will lead to a very conservative estimate of the actual emission reduction. |
| Any comment: | Terms can include a combination of emission factors from fuel production and use. |

| Data / Parameter: | NCVb |
| --- | --- |
| Data unit: | TJ/ton |
| Description: | Net calorific value of the fuels used in the baseline |
| Source of data used: | IPCC 2006 Vol 2, Chap 1, Table 1.2 |
| Value applied: | NCVb,fw = 0.0156 TJ/ton NCVb,LPG = 0.0473 TJ/ton NCVb,charcoal = 0.0296 TJ/ton NCVb,kerosene = 0.0438 TJ/ton |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | These values are taken from IPCC Guidelines 2006 Vol 2, Chap 1, Table 1.2 |
| Any comment: | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. |

| Data / Parameter: | NCVp |
| --- | --- |
| Data unit: | TJ/ton |
| Description: | Net calorific value of the fuels used in the project |
| Source of data used: | IPCC 2006 Vol 2, Chap 1, Table 1.2 |
| Value applied: | NCVb,fw = 0.0156 TJ/ton NCVb,LPG = 0.0473 TJ/ton NCVb,charcoal = 0.0296 TJ/ton NCVb,kerosene = 0.0438 TJ/ton |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | These values are taken from IPCC Guidelines 2006 Vol 2, Chap 1, Table 1.2 |
| Any comment: | If EF is in units of tCO2/t_fuel, remove NCV term from emission calculations. |

| Data / Parameter: | fNRB,i,y |
| --- | --- |
| Data unit: | Fractional non-renewability |
| Description: | Non-renewability status of woody biomass fuel in scenario i during year y |
| Source of data used: | registered PoADD GS2096. |
| Value applied: | 96% |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | The biomass collection area of the project is the whole Haitian national territory. This value is derived from an assessment made at the national level. |
| Any comment: | As applicable, NRB assessment may be used for multiple scenarios |

| Data / Parameter: | Pb,y |
| --- | --- |
| Data unit: | kg/household-day |
| Description: | Quantity of fuel that is consumed in baseline scenario p during year y. LPG, Kerosene, Firewood and Charcoal fuel consumption will be monitored. |
| Source of data to be used: | Baseline FT |
| Value of data applied for the purpose of calculating expected emission reductions | Pcom,firewood= 0 kg/day/restaurant Pcom,charcoal= 6.46 kg/day/restaurant Pcom,LPG= 0 kg/day/restaurant Pcom,kerosene= 0.013 kg/day/restaurant Pdom,firewood= 0 kg/day/household Pdom,charcoal= 2.32 kg/day/household Pdom,LPG= 0.05 kg/day/household Pdom,kerosene= 0.08 kg/day/household |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | Scales with a 100g accuracy has be used to weight the fuel. The BFT have been done before Palmis Eneji started promoting the stove included in the present project activity. |
| Any comment: |  |

### Value of parameter that were monitored

| Data / Parameter: | Pp,y |
| --- | --- |
| Data unit: | kg/household-day, kg/person-meal, etc. |
| Description: | Quantity of fuel that is consumed in project scenario p during year y. LPG, Kerosene, Firewood and Charcoal fuel consumption will be monitored. |
| Source of data to be used: | Total sales record, Project FT, project FT updates, and any applicable adjustment factors |
| Value of data applied for the purpose of calculating emission reductions | See table hereunder |
| Monitoring frequency: | Updated at least every two years |
| Description of measurement methods and procedures to be applied: | Scales with a 100g accuracy will be used to weight the fuel. The PFT will be done in the second half of the crediting year among a sample selected to insure proper representation of stove age (implicit stratification basic on sale date). |
| QA/QC procedures to be applied: | Transparent data analysis and reporting |
| Any comment: | A single project fuel consumption parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario |

| Data / Parameter: | Up,y |
| --- | --- |
| Data unit: | Percentage |
| Description: | Usage rate in project scenario p during year y |
| Source of data to be used: | Annual usage survey |
| Value of data applied for the purpose of calculating emission reductions | Udom,2016=89.7% Udom,2017=86.5% |
| Monitoring frequency: | Annually or more frequently, in all cases on time for any request for issuance |
| Description of measurement methods and procedures to be applied: | In this section the project participants shall provide a description of equipment used for measurement, if applicable, and its accuracy class. |
| QA/QC procedures to be applied: | Transparent data analysis and reporting |
| Any comment: | A single usage parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario. |

| Data / Parameter: | Np,y |
| --- | --- |
| Data unit: | Project technologies credited (stove.day) |
| Description: | Technologies in the project database for project scenario p through year y |
| Source of data to be used: | Total sales record |
| Value of data applied for the purpose of calculating emission reductions | Ndom,2017=427,386 Ndom,2018=182,722 |
| Monitoring frequency: | Continuous |
| Description of measurement methods and procedures to be applied: | In this section the project participants shall provide a description of equipment used for measurement, if applicable, and its accuracy class. |
| QA/QC procedures to be applied: | Transparent data analysis and reporting |
| Any comment: | The total sales record is divided based on the project scenario to create the project database |

Details of fuel consumption monitored

| FUEL CONSUMPTION SUMMARY | Value | Unit | Source |
| --- | --- | --- | --- |
| Average charcoal baseline consumption of domestic users | 2.32 | kg/day | Baseline KPT |
| Average firewood baseline consumption of domestic users | - | kg/day | Baseline KPT |
| Average kerosene baseline consumption of domestic users | 0.08 | kg/day | Baseline KPT |
| Average LPG baseline consumption of domestic users | 0.05 | kg/day | Baseline KPT |
| Average charcoal project consumption of domestic users | 0.52 | kg/day | Project KPT |
| Average firewood project consumption of domestic users | - | kg/day | Project KPT |
| Average kerosene project consumption of domestic users | - | kg/day | Project KPT |
| Average LPG project consumption of domestic users | 0.64 | kg/day | Project KPT |
| Number of day in year for domestic users | 365.00 | Day/year | definition |
| Mean Number of person meal/day/household in the domestic use project scenario | 9.10 | person meal/day/household | KPT Survey |

## Main findings

### Usage rate and Usage purpose

The project distributes different types of LPG stove, the stove has different number of burners.

All those stoves rely on the same established and standardized combustion technology and on mass produced and standardized LPG fuel.

Like explained in the registered PDD, project scenarios are based on the type of usage rather than the type of stove itself. Most light duty stove are used for domestic purpose, but in some case light duty stove are used for commercial cooking. This is due to the open approach of the project implementation strategy which allow anyone to purchase a stove and use it as they see fit.

During the usage and monitoring survey we ask whether the stove where used and for what type of usage. The data collected allow us to go from the sale record (list of stove type sold) to a project database where we can sort the stove by project scenario.

The table here under show the number of stove derived from the sale record for each type of stove and show how each type of stove is used.

|  | stove day (sale database) | % of stove used for commercial purpose | % of stove used for domestic purpose |
| --- | --- | --- | --- |
| Light Duty stove | 556,408 | 8% | 92% |

As a criteria for usage, we are looking at the declared usage frequency. The respondent have to use the  stove twice a day to be considered users. If they use it less than twice a day they are not considered as user for the purpose of the Emission Reduction calculation. This is conservative.

The table hereunder show the number of stove day the usage rate for each age category of the stoves.

| Sale Year | stove day (domestic use) | Usage Rate |
| --- | --- | --- |
| 2016 | 132,579 | 89.7% |
| 2017 | 379,317 | 86.5% |

### Type and quantity of fuel used

In the baseline situation, charcoal is the main fuel with a few households also using other secondary fuel. After the purchase of a LPG stove from the project, LPG become the fuel used most often while charcoal continue to be used by some households.

This is confirmed by the fuel consumption measured during the KPT

| Domestic fuel consumption (kg/day/hh) | Charcoal | Firewood | Kerosene | LPG |
| --- | --- | --- | --- | --- |
| Baseline | 2.32 | - | 0.08 | 0.05 |
| Project | 0.52 | - | - | 0.64 |

### Seasonal variation in fuel consumption

To estimate the seasonality of fuel consumption, we asked respondent of the monitoring survey to indicate whether their consumption is average, below average or above average for each month of the year. We then tallied the answers together to get the following charts.

The chart above details the percentage of respondent who indicated that their fuel consumption varies for each month. It shows that the period where the most fuel is used is from September to December.

The project KPT was conducted in February when the fuel consumption was representative to the rest of the year.

## Green House Gas Emissions calculations

### Difference of mean

The table here under summarize the data used for the calculation of GHG emission reductions.

| GHG Emissions | Mean baseline emissions (tCO2eq/stove/year) | Baseline KPT sample size | Standard deviation of baseline emission (tCO2eq/stove/year) | Mean project emissions (tCO2eq/stove/year) | Project KPT sample size | Standard deviation of project emission (tCO2eq/stove/year) | Emission Reduction (tCO2/stove/year) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| domestic usage | 24.51 | 149 | 7.35 | 15.75 | 83 | 14.19 | 8.76 |

### 90/30 check and VER adjustment

| GHG Emissions (tCO2eq/stove/year) | 90% CI spread | 30% mean difference | adjusted VER/stove/year | adjusted VER/stove/day |
| --- | --- | --- | --- | --- |
| domestic usage | 2.10 | 2.63 | 8.76 | 0.0240 |

The following equation was used to compute the bounds of the 90% one-sided confidence interval:

CI bounds=μBL-μPR±1.26σBL2nBL+σPR2nPR

With the following notation respectively for the baseline and the project

σ : Standard deviation

µ: average

n: sample size

The bound of the 90% Confidence interval are within +-30% of the mean. So we can calculate emission reduction based on the difference of the mean.

### VER at the project scale

| Sales year |  | stove day (Feb 2017 to Dec 2017) |  | stove day (Jan 2018 to March 2018) |  | Usage Rate |  | adjusted VER/stove/day |  | VER (2017 vintage) |  | VER (2018 vintage) |  | Overall VER (tCO2e) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2016 |  | 107,442 |  | 25,137 |  | 89.7% |  | 0.024 |  | 2,314 |  | 541 |  | 2,856 |  |
| 2017 |  | 269,952 |  | 109,364 |  | 86.5% |  | 0.024 |  | 5,603 |  | 2,270 |  | 7,873 |  |
|  |  |  |  |  |  |  |  | Total |  | 7,917 |  | 2,811 |  | 10,729 |  |

### Breakdown of VER per vintage

Since this project is a micro scale, the VER are capped at 10,000 per year.

The table here under display the VER claimed by the previous monitoring report and the VER claimed by the present monitoring report to show that the threshold of 10,000 VER per year is respected.

| VER per vintage | 2nd Monitoring Report | 3rd Monitoring Report |
| --- | --- | --- |
| 2017 | 497 | 7,917 |
| 2018 | 0 | 2,811 |
| Total |  | 10,728 |

A total of 7,917 VER of vintage 2017 and 2,811 VER of vintage 2018 are claimed for this monitoring period (February 1st 2017 to March 31st 2018).

### Comparison with Ex-ante VER estimation

In the PDD, the estimated amount of VER ex ante in part B.6.4 was expected to reach 9,990 VER in 2017 and the years after.

We found that the number of VER issued in 2017 (including the one from the second monitoring report) is 7,917 which is less than the 9990 VER expected. This is due to the fact that many of the stove included into the project were sold during 2017 and they were not used for the full 365 days. The ex-ante estimation was based on a simplified assumption were all the stove added are assumed to be used for the whole year.

In 2018, the number of VER claimed is 2,811 which is lower than the expected 9,990 VER for 2018. This is due to the fact that monitoring period does not cover the whole year of 2018.

# Overall socio-economic impacts from the project

These two charts were established independently for traditional stove user and LPG stove users respectively. In all categories, the usual issues associated with cooking with dirty fuel are greatly mitigated.

Domestic users

# Annex I: Contact information on participants in the project activity

| Organization: | Entrepreneurs du monde |
| --- | --- |
| Street/P.O.Box: | 4 allée du Textile |
| Building: |  |
| City: | Vaulx-en-Velin |
| State/Region: | Rhône-Alpes |
| Postfix/ZIP: | 69 120 |
| Country: | France |
| Telephone: | +33 (0)4 37 24 76 50 |
| FAX: |  |
| E-Mail: | thomas.thivillon@entrepreneursdumonde.org |
| URL: | http://www.entrepreneursdumonde.org/ |
| Represented by: | Thomas Thivillon |
| Title: | Head of energy programs |
| Salutation: | Mr. |
| Last Name: | Thivillon |
| Middle Name: |  |
| First Name: | Thomas |
| Department: | Energy |
| Mobile: |  |
| Direct FAX: |  |
| Direct tel: | Tel. +33 (0)6 10 48 24 98 |
| Personal E-Mail: | thomas.thivillon@entrepreneursdumonde.org |

See Toolkit Annex C for more details