ANNEX R – PASSPORT TEMPLATE

CONTENTS

- Project title

- Project description

- Proof of project eligibility

- Unique Project Identification

- Outcome stakeholder consultation process

- Outcome sustainability assessment

- Sustainability monitoring plan

- Additionality and conservativeness deviations

# Annex 1 ODA declarations

SECTION A. 	Project Title

Title: Gaziantep LFG to Energy Project, Turkey Date: 10.07.2017 Version no.: 7

SECTION B. 	Project description

Estimated project start date: 13.03.2009 Document History: On 21/06/2011, the project activity has been registered as a Gold Standard Voluntary Emission Reduction (GS VER) Project Activity under with the ID GS745 and has been issuing GS VER credits since 20.01.2010. In line with the requirements of Gold Standard, the Project Participant wishes to renew the crediting period. The following sections of this Passport is based on the Passport version 6, which was registered by Gold Standard on 21/06/2011. Project Description: The Gaziantep Landfill Gas to Energy Project, Turkey is developed by “CEV Enerji Üretim Sanayi ve Tic. Ltd. Sti.”, the regional office of the South Korea-based company CEV Clean Energy & Vehicle. The proposed activity involves the collection and utilization of landfill gas with an electricity component with a maximum installed capacity reaching 5.655 MWe. The location of the proposed activity is the Gaziantep landfill site, Turkey. The Gaziantep landfill site is the main landfill collection point of Gaziantep city, the 6th largest city in Turkey with 1.2 million inhabitants. It started operations in 1996 and currently holds in excess of 3.2 million tones of waste. In 2008, the disposed waste amount reached 354,620 tones, corresponding to an average 1000 tones per day on average. The site is a managed landfill with no active LFG venting. In 2008, the Gaziantep Metropolitan Municipality opened a tender for managing solid waste at the landfill. The Project owner has acquired the right to commercially operate the landfill for 29 years and the Project is expected to be operated until 2038. The Project is expected to operate until 2038 and will have two phases; in the initial phase the capacity will be 1.131 MW. In the second phase, the capacity will be expanded up to 5.655 MW: Phase; Generation Units Installed; Total Installed Capacity; Commissioning Date. Phase 1; 1 generation unit of 1.131 MWe; 1.131 MWe; 20/01/2010. Phase 2; 4 generation units of 1.131 MWe each; 5.655 MWe; 12/08/2011. The important milestones of the project activity could be summarized as follows. Date; Milestone. 21/10/2008; Gaziantep Metropolitan Municipality organizes a tender for managing the waste on its landfill.. 28/10/2008; The Project owner wins the tender and concludes a contract with the Municipality.. 22/12/2008; Technical experts from Korea make a site visit to carry out various gas tests and measurements.. 12/2008; Feasibility study contract. 25/02/2009; Two construction companies are subcontracted for the preparation of the site (soil cover, piping etc.). 26/02/2009; The turbine contract is signed with the equipment supplier.. 13/03/2009; Start day of site preparations (mobilisation). 17/03/2009; License application to EMRA. 17/06/2009; Completion of site preparations. 20/01/2010; Commissioning of 1st generation unit. 20/01/2010; Start of first crediting period. 12/08/2011; Commissioning of 2nd; 3rd; 4th and 5th generation unit. 21/06/2016; Registration of project activity under Gold Standard. As of now the project activity is operational with a total of 5 gas engines with a total installed capacity of 5.655 MW. The activity generates approximately 21 GWh/year of energy and delivered to the national grid. The Project activity have resulted in 76,194 tCO2eq actual emission reductions throughout the monitoring period between 20/03/2010 – 31/07/2016 and is estimated to reduce 586,007 tCO2eq emission reductions during the second crediting period.

SECTION C. 	Proof of project eligibility

# C.1.	Scale of the Project

[See Toolkit 1.2.a]

Please tick where applicable:

| Project Type | Large | Small |
| --- | --- | --- |
|  | X |  |

# C.2.	Host Country

The project activity is hosted in Turkey.

# C.3.	Project Type

[See Toolkit 1.2.c and Annex C]

Please tick where applicable:

| Project type | Yes | No |
| --- | --- | --- |
| Does your project activity classify as a Renewable Energy project? | X |  |
| Does your project activity classify as an End-use Energy Efficiency Improvement project? |  | X |
| Does your project activity classify as waste handling and disposal project? | X |  |

Please justify the eligibility of your project activity:

The Project has two components: landfill gas destruction and electricity generation. The electricity generation component (up to 5.655 MW) is under the UNFCCC small scale limit of 15 MW. However, the methane destruction component exceeds the 60,000 tCO2 limit for small scale project activities. Therefore, the Project is defined as large scale. Turkey has officially ratified the Kyoto Protocol as of 26.08.2009 and has no GHG emission reduction targets or commitments under the Kyoto Protocol. The Gold Standard Toolkit 1.2.3 includes landfill gas to energy projects in the Renewable Energy category. The Gold Standard Toolkit Annex C requires electricity to be generated from landfill gas at a minimum of 65% of the total LFG collected. The Project aims at reaching this threshold and is thus eligible under the Gold Standard

| Pre-Announcement | Yes | No |
| --- | --- | --- |
| Was your project previously announced? |  | X |
| The project was not announced to go ahead without the revenues from carbon credits. |  |  |

# C.4.	Greenhouse gas

[See Toolkit 1.2.d]

| Greenhouse Gas |  |
| --- | --- |
| Carbon dioxide | x |
| Methane | x |
| Nitrous oxide |  |

# C.5.	Project Registration Type

[See Toolkit 1.2.f]

| Project Registration Type |
| --- |
| Regular |  |

| Pre-feasibility assessment | Retroactive projects (T.2.5.1) | Preliminary evaluation (eg: Large Hydro or palm oil-related project) (T.2.5.2) | Rejected by UNFCCC (T2.5.3) |
| --- | --- | --- | --- |
|  | x |  |  |

If Retroactive, please indicate Start Date of project activity dd/mm/yyyy:_13.03.2009

SECTION D. 	Unique project identification

# D.1.	GPS-coordinates of project location

[See Toolkit 1.6]

|  | Coordinates |
| --- | --- |
| Latitude | 36° 59' 12" N (36.986667) |
| Longitude | 37° 24' 11" E (37.403056) |

# Explain given coordinates

The Project is located at the Gaziantep landfill site, in Gaziantep province, Turkey. It is approximately 9 km south of the Gaziantep city centre, close to Akbulut town, Baglarbasi neighborhood.

# D.2.	Map

SECTION E. 	Outcome stakeholder consultation process

E.1.	Assessment of stakeholder comments

Currently the time of renewal of the crediting period the project activity is operational with a total installed capacity of 5.655 MW. The activity generates approximately 21 GWh of energy and delivered to the national grid. The Project activity have resulted in 76,194 tCO2eq actual emission reductions throughout the monitoring period between 20/03/2010 – 31/07/2016 and is estimated to reduce 586,007 tCO2eq emission reductions during the second crediting period. The project has been assessed to have a total of 7 points in the SD matrix and has been registered during the first crediting period. Before the start of first crediting period, a stakeholder’s consultation process has been undertaken. Through the consultation round conducted with regards to the proposed project’s impact to environment and sustainable development before the first crediting period, it has been concluded that the project is appraised as beneficial for the region. The project is welcomed by all stakeholders because it is environmentally benign, it generates income and jobs, it supports the development of the rural among other benefits. Within 2016 the Project Activity has conducted verification by independent and accredited DOE on not only covering achieved emission reductions but also on results of achieved sustainable development targets as well the impact of the project activity to the surrounding, region and the city. In addition to monitoring parameters required to calculate the emission reductions, the following sustainable development parameters have been also consistently and periodically monitored throughout the crediting period; - Air Quality - Water Quality and Quantity - Soil Condition - Quality of Employment - Access to affordable and clean energy services - Quantitative employment and income generation - Balance of payments and investment Furthermore, several stakeholders are periodically interviewed during the verification on site visits to assess the impact of the project activity. During the validation, onsite visit for the renewal of the crediting period will also involve interviews with several stakeholders, and the PP is confident that the outcome of these interviews will confirm the environmental and social wellbeing of the project activity. The monitoring of these indicators and interviews, on-site visits conducted by the DOE has demonstrated that the anticipated contribution of the proposed project activity has been realized. Based on the fact that the sustainability aspects of the project activity is and will remain in monitoring and the stakeholders have been consulted periodically on these impacts, no specific stakeholders consultation (except the interviews during the second validation onsite visit by the DOE) has been conducted before the start of the crediting period. Based on the above discussions, and the outcomes of the monitoring results throughout the first crediting period, it can be concluded that the assessment on the SD Matrix conducted during the first crediting period and its monitoring plan is valid and still applicable to the project activity. The outcome of the Stakeholders Consultation Process is summarized in the following sections of the Passport: A meeting organized by the Project owner on 26.12.2008 hosted several stakeholders and the opinions declared during that meeting were very positive and encouraging. Although this meeting was not organized according to the Gold Standard rules and approach, it was useful in outlining the Project to a large audience and to allow them to express their ideas about it. The stakeholders who participated in the meeting included a parliament member representing Gaziantep, the city governor, Korean Ambassador to Turkey, the mayor of Gaziantep, veterans of the Korean war, students, villagers, the president of the city chamber for industry and other attendees. There have been no negative comments from any stakeholder about the Project.

# E.2.	Stakeholder Feedback Round

The invited stakeholders are idintifed based on the Table 2.8 of the Toolkit. In total, 29 stakeholders are identified from 6 GS categories. GS Category Code; Nr. Of identified Stakeholders. A; 5. B; 9. C; 1. D; 6. E; 1. F; 7. The identified stakeholders have been chosen on the basis of relevance of the Project in terms of location (Gaziantep province, villages and neighborhoods in the vicinity), subject (environment, waste management, forestry health, agriculture, industry etc.) and Gold Standard Requirements (NGOs, DNA etc.). The local NGOs are identified with the support of other local stakeholders and online search. Regarding the neighborhoods (category A), the headmen (mukhtars) are invited, as they are elected representatives of these local communities. The host country, Turkey, has ratified the Kyoto Protocol in February 2009, but has no emission reduction commitments. There is no DNA in Turkey; therefore the UNFCCC national focal point (the Ministry of Environment and Forest) is invited. None of the identified stakeholders have been filtered out for any reason. No discrimination (e.g. gender, age or ethnicity) is applied in the selection of stakeholders. Several invitation methods are used to enable all stakeholders to be informed about the meeting. One or more stakeholders are identified for every GS category code, covering a wide range of relevant parties. The non-technical PDD summary, as well as the summary of the GS Passport, have been integral parts of the invitation. In case of invitations sent via post, the following documents are sent in local language: - Formal invitation letter - Non-technical PDD summary - Summary of the GS Passport In case of invitations sent via fax, these documents are sent along with a basic fax cover page. In case of e-mail invitations, these documents are sent as attachment with an e-mail text. The PDD and the Passport are only sent to the stakeholders of the category F, because of its size (page number / file size) and language (only in English). Those stakeholders that do not have a valid e-mail address are called by phone either before (to inform in advance), or after the invitation (to obtain a confirmation). For some invitations that were sent by e-mail, no read-receipt confirmation is received. For such cases, these stakeholders are called by phone to ask whether they have received the e-mail, and in case of non-delivery the invitation is resent by fax or other appropriate means. Some of the invited stakeholders did not attend the meeting and some of them have given a reason for their non-attendance (e.g. limited human resources). These reasons for non-attendance do not indicate any issues with the consultation organization (date, location, hour etc.). During the meeting, carbon consultants offered help to any stakeholder who is not able to write his/her comments due to any reason (illiteracy, internet access, age etc.), no such request is received. The invitation letter invited stakeholders to send their comments, even if they cannot attend the meeting. In addition, during the physical meeting, the participants are told that they can send their comments in any format (fax, E-mail, post etc.) to the carbon consultants until 17.04.2011 (8 weeks following the meeting date). During this period, no comments were received. The meeting was carried out such that participants could freely suggest their own ideas about the project. The meeting is conducted with a satisfactory organization and the free- speech atmosphere of the meeting was not disturbed by any participant. Stakeholders are interested in the Project and its design. In general, participants believe that the Project will contribute in many ways such as the environment, energy generation, odour and health improvement, municipality budget and local development. After the consultation process, there is no outstanding issue left and the stakeholders are satisfied with the answers they received and the Project itself.

E. 3. Discussion on continuous input / grievance mechanism

|  | Method Chosen (include all known details e.g. location of book, phone, number, identity of mediator) | Justification |
| --- | --- | --- |
| Continuous Input / Grievance Expression Process Book | A log book will be kept at the muchtar’s office, which will be accessible all time. | A continuous input process book has been selected as the most appropriate method since it is simple to use and both the locals will have the opportunity to provide their comments on writing and the project participant to provide feedback on the comments. |
| Telephone access | xxx | The stakehodlers will have the opportunity to call the contact number of the project participant |
| Internet/email access | xxx | Taking into account that locals might be reluctant to communicate through internet access or via email this option was not preferred. |
| Nominated Independent Mediator (optional) | n.a. | This method is not selected. |

SECTION F. 	Outcome Sustainability assessment

F.1.	‘Do no harm’ Assessment

| Safeguarding principles | Description of relevance to my project | Assessment of my project risks breaching it (low/medium/high) | Mitigation measure |
| --- | --- | --- | --- |
| 1. Human rights abuses or negative impacts on dignity, cultural property or indigenous people | There are no indigenous people inhabiting the project area4. | low | N/A |
| 2. Involuntary settlement | No relocation of people is necessary5. | low | N/A |
| 3. Alteration, damage or removal of critical cultural heritage | There is no cultural heritage on the site6. | low | N/A |
| 4. Employee’s freedom of association | Employees have the freedom of association. The Turkish Government has ratified the ILO conventions 87 (freedom of association) and 98 (right to collective bargaining.) | low | N/A |
| 5. Forced labour | The Turkish Government has ratified the ILO conventions 29 and 105 on elimination of forced and compulsory labor. | low | N/A |
| 6. Child labour | The Turkish Government has ratified the ILO conventions 138 (minimum age) and 182 (worst form of child labour). | low | N/A |
| 7. Discrimination | The Turkish Government has ratified the ILO conventions 100 (equal remuneration) and 111 (discrimination in employment/occupatio n). | low | N/A |
| 8. Safe and healthy work environment for workers | Workers can get injured | low | All relevant environment, health and safety regulations will be applied as required and all necessary precautions will be taken7. |
| 9. Precautionary approach for environmental challenges | Earthquake is a risk. * Water contamination is a possibility. * Flare unit might not be operated. | low | Gaziantep is located on a 3rd level earthquake risk8 geography, which has low probability of earthquakes. * As opposed to the baseline practice, contamination of rainwater will be minimised by effectively covering the waste with impermeable materials. Even if some water leaches into the depths (which is necessary for methane generation), the impermeable landfill bottom will not allow leachate to escape and contaminate ground or underground waters. Leachate recovery is already being carried out9. * Emission reductions from flaring above 35% of the collected LFG are not accepted under the Gold Standard. In case of an unexpected generator failure or overproduction of LFG, the Project owner will flare all remaining LFG in order to minimize health and safety risks as a precautionary approach. Flaring of the residual gas exceeding 35%, which could not be combusted, is not eligible for VERs. |
| 10. Degradation of natural habitats | There is no natural habitat on the site10. The surrounding of the landfill is a desolate topography with little trees and a lot of exposed rocks. | low | The Project will be constructed in an environmentally less sensitive area at the landfill site. Therefore the Project has minimum involvement with its surroundings. |
| 11. Anti-corruption | The Project is not related to any kind of corruption. The tender for the Project site was subject to the State Procurement Law11. | low | The Project owner does not have any negative track record related to corruption or any such activity whatsoever. The Project was acquired by the Project owner through an open tender, which was announced publicly in 2008 October by the Gaziantep Municipality. The tender was organised in October 2008 in accordance with the State Procurement Law regulated by the Public Procurement Authority. |
| Additional relevant critical issues for my project type | Description of relevance to my project | Assessment of relevance to my project (low/medium/high) | Mitigation measure |
| n.a. | n.a. | n.a. | n.a. |

F.2.	Sustainable Development matrix

| Indicator | Mitigation measure |  | Relevance to achieving MDG | Chosen parameter and explanation | Preliminary score |
| --- | --- | --- | --- | --- | --- |
| Gold Standard indicators of sustainable development. | If relevant copy mitigation measure from "do no harm" –table, or include mitigation measure used to neutralise a score of ‘–‘ |  | Check www.undp.or/mdg and www.mdgmonitor.org Describe how your indicator is related to local MDG goals | Defined by project developer | Negative impact: score ‘-‘ in case negative impact is not fully mitigated score 0 in case impact is planned to be fully mitigated No change in impact: score 0 Positive impact: score ‘+’ |
| Air quality | N/A |  | MDG 7a: Integrate the principles of sustainable development into country policies and programmes; reverse loss of environmental resources MDG 7b-2: CO2 emissions, total, per capita and per $1 GDP (PPP) | Sulphur oxide and nitrogen oxide emissions: VOC, H2S, SO2 and NOx emissions will be reduced. Odor: reduction of bad odor. | + |
| Water quality and quantity | N/A |  | MDG 7.10: Proportion of the population using improved drinking water sources | Cooling water discharge: The power generated by the Project will result in reduction of cooling water discharged by thermal power plants. Leachate management: The Project will try to ensure that leachate generated on site will be collected, stored and transported to prevent leakage into underground water. | + |
| Soil condition | N/A |  | MDG 7b-6: Proportion of terrestrial and marine areas protected | Hazardous waste processing: The Project will not function as a hazardous waste disposal or processing facility. | + |
| Other pollutants | N/A |  | MDG 7b-6: Proportion of terrestrial and marine areas protected |  | 0 |
| Biodiversity | N/A |  | MDG 7b-4 Proportion of fish stocks within safe biological limits MDG 7b-7 Proportion of species threatened with extinction |  | 0 |
| Quality of employment | N/A |  | MDG 1b-4 Growth rate of GDP per person employed MDG 1b-6 Proportion of employed people living below $1 (PPP) per day | Employment conditions: Proper work environment, social security, health and safety precautions. | + |
| Livelihood of the poor |  |  | MDG 7b-6 Proportion of terrestrial and marine areas protected | Bad odour in the vicinity: Reduction of odorous H2S emissions. | 0 |
| Access to affordable and clean energy services | N/A |  | MDG 1a-1 Proportion of population below $1 (PPP) per day | Electricity generated: Amount of avoided fossil fuel consumption. | + |
| Human and institutional capacity | N/A |  | MDG 2a-3 Literacy rate of 15-24 year-olds, women and men | Capacity Building and awareness raising: Personnel training and familiarity of stakeholders with LFG technologies | 0 |
| Quantitative employment and income generation |  |  | MDG 1b-4 Growth rate of GDP per person employed MDG 1b-5 Employment-to- population ratio MDG 1b-6 Proportion of employed people living below $1 (PPP) per day | Income generation: Number of people employed by the Project owner and its subcontractors | + |
| Balance of payments and investment |  |  | MDG 8d Deal comprehensively with the debt problems of developing countries through national and international measures in order to make debt sustainable in the long term | Electricity generated: Economical value of avoided natural gas consumption | + |
| Technology transfer and technological self-reliance |  |  | MDG 8f In cooperation with the private sector, make available the benefits of new technologies, especially information and communications | Technology and know how transfer: Introduction of a new technology | 0 |
| Justification choices, data source and provision of references |  |  |  |  |  |
| Air quality |  | Air quality will be improved as compared to the baseline emission levels of VOCs, odour (H2S), SO2 and NOx by the combustion of landfill gas. VOC, H2S, SO2 and NOx related to the energy generation from fossil fuels will be reduced by replacing electricity from fossil fuel combustion and related fuel consumption. The reductions of SO2 and NOx emissions will be calculated by multiplying net electricity generation of the Project activity with the SO2 and NOx intensities. The reduction of H2S will be monitored by odour reduction, as hydrogen sulfide has a characteristic foul odour. “When NOx and volatile organic compounds (VOCs) react in the presence of sunlight, they form photochemical smog, a significant form of air pollution, especially in the summer. Children, people with lung diseases such as asthma, and people who work or exercise outside are particularly susceptible to the adverse effects of smog which include damage to lung tissue and reduction in lung function”. Thermal treatment of VOCs and methane through flaring or combustion greatly reduces the emission of these compounp. The Project will effectively collect and destroy LFG, thus reducing VOCs and related health risks. The reductions of other hazardous gases in the landfill gas during combustion are not monitored because of its complexity to quantify them accurately. This is conservative. Reduction of odor: As a result of effective reduction in H2S and VOCs, the foul odor that was being emitted from the landfill site is expected to be reduced. The site has approximately 0.8 km distance to the nearest settlement, Akbulut neighborhood. |  |  |  |
| Water quality and quantity |  | Water quantity: Through substitution of electricity generated by fossil fuels, the amount of cooling water consumption in the baseline scenario is expected to drop*. Water quality: A leachate pumping and storage system is already in operation on the site. Leachate management is officially not a responsibility of the Project**, however in practice it has assumed some of the leachate related tasks until a leachate treatment system becomes operational on site. The Project owner has no control over the leachate related actions the Municipality takes, but it will try to ensure that leachate is collected, stored and transported safely, as much as it can. |  |  |  |
| Soil condition |  | As compared to the baseline scenario, the Project does not generate any hazardous waste or chemical, which would infiltrate into soil and decrease the soil condition in quality and quantity. The landfill site is operated by the Gaziantep Municipality as a household waste landfill (industrial waste is not accepted). There also exists a recycling company active at the landfill site that collects and sells recyclable materials, thereby contributing to sustainability of the landfill site. |  |  |  |
| Other pollutants |  | There is no nearby village that could potentially be disturbed from noise emissions of the combustion engine by the Project activity. |  |  |  |
| Biodiversity |  | Landfill operations can cause, inter alia, harboring of disease vectors such as rats and flies and injuries to wildlife. Landfills can facilitate the multiplication and dissemination of the pathogenic agents and their accompanying breed (insects, rats, crows, stray dogs), which may harm some mammals, birds, insects deserting the area, or other beings that feed on refuse (e.g. rats, crows). |  |  |  |
| Quality of employment |  | The Project owner is also liable for landfill site management for municipal waste. It will therefore employ some staff for the landfill operations. These staffs were personnel working on the site before for another company. Their working and living conditions were extremely poor. The Project owner has renovated a building as the social facility for workers (living quarters, clothing room, kitchen etc.) and has started providing significantly better living conditions to its employees. Employees are happy about these developments. |  |  |  |
| Livelihood of the poor |  | Landfill operations can cause, inter alia, simple nuisance problems such as dust, odour, vermin, or noise pollution. The Project will reduce visual, odour and vermin problems by covering the waste with impermeable materials and collecting LFG, thereby minimizing the risks for vermin propagation. Moreover, landfills can create a safety risks. |  |  |  |
| Access to affordable and clean energy services |  | Local communities already have electricity supply and no improvement is expected in additional access to energy services. The Project will contribute to energy security and the generated clean electricity will be supplied to the grid, thereby improving the overall electricity supply. Turkey imports a great amount of fossil fuels and the Project will also reduce the dependency on foreign import of fossil fuels. |  |  |  |
| Human and institutional capacity |  | The Project activity is a new technology in the host country and is one of the first examples in Turkey. In this regard, the Project will contribute to human and institutional capacity. |  |  |  |
| Quantitative employment and income generation |  | The Project is expected to temporarily employ, through its subcontractors, several people during construction and set-up phases of the Project. Although the Project activity is the LFG collection and utilization, the Project Owner is also responsible for the site management of the landfill, which is a labour- intensive operation. During the operation, local people will be preferred as much as available. The Project will result in employment of security guards, drivers, switchboard technicians, engineers etc., the number of which will be clarified in later stages. Local people will be prioritized for employment. This will result in an inflow of income to local communities and vitalisation of the local economy. |  |  |  |
| Balance of payments and investment |  | Initially, importing the equipment for the Project will have a negative impact on the balance of payments. In coming years, this impact will be reversed. Electricity generation will substitute the electricity generated from fossil fuels and thus decreases fossil fuel consumption overall. PMUM (TEIAŞ Market Financial Clearing Centre) is responsible for actively balancing the electricity supply among the generation companies; therefore, the Project will indirectly result in reduced imports of fossil fuels. This will improve the balance of payments of the host country. |  |  |  |
| Technology transfer and technological self-reliance |  | The Project owner is a South Korea based company, which will facilitate know how transfer to the host country. Transfer of knowhow is most common for equipment and knowledge, for agriculture, biogas, N2O and landfill gas projects. The Project will directly and indirectly allow Project personnel, equipment and service suppliers to gain experience in landfill gas extraction and utilization. It will further stimulate technological self-reliance by technology transfer. The Project owner will ensure that the landfill site is properly managed for the LFG collection and utilization with the support of international experts, sharing their know how with the local team. Moreover, there are several neighbor cities where there is not any similar project. Therefore, the Project might also attract attention of other municipalities and contribute to technology transfer to other cities in the region. Project personnel will be trained and the Project is expected to set a good example in terms of sustainable energy practices and to the improvement of human and institutional capacity. The Project will contribute to effective reduction of GHG emissions and odour, revenue stream to the municipality, sustainable development, explosion and fire risk mitigation. |  |  |  |

SECTION G. 	Sustainability Monitoring Plan

[See Toolkit 2.4.3 and Annex I]

# Copy Table for each

| # No |  | # SD.1.a |
| --- | --- | --- |
| # Indicator: |  | # Air Quality |
| # Mitigation Measure |  | # n.a. |
| # Chosen Parameter |  | # SO2: The air quality is related to pollutants such as SOx which are being emitted in the baseline scenario. As a representative indicator of air pollution, SO2 is selected as further oxidation of sulphur dioxide may lead to acid rains and as it is a precursor to particulates in the atmosphere, both of which are environmental concerns. |
| # Estimation of baseline situation of parameter |  | # Total host county emissions are 1007.78 tons in 2008. Net system electricity generation in 2008 is 189,762 MWh, corresponding to 5.31 kg/MWh SO2 intensity. |
| # Future target for parameter |  | # As the Project capacity will be expanded in later years, the net electricity generation is estimated to increase further and the avoided SO2 emissions will increase in parallel. |
| # Way of Monitoring | # How | # Indirectly; by measuring net electricity generation and multiplication with the baseline SO2 intensity of the Turkish grid. |
|  | # When | # Continuously |
|  | # By who | # Project Owner |

| # No |  | # SD.1.b |
| --- | --- | --- |
| # Indicator: |  | # Air Quality |
| # Mitigation Measure |  | # n.a. |
| # Chosen Parameter |  | # NOx: The air quality is related to pollutants such as NOx which are being emitted in the baseline scenario. As a representative indicator of air pollution, nitrogen oxide is selected because mono-nitrogen oxides eventually form nitric acid when dissolved in atmospheric moisture, forming a component of acid rain. |
| # Estimation of baseline situation of parameter |  | # The Project is expected to generate 25,655 MWh in the first year of operation, therefore the Project will substitute a baseline NOx emissions of: # 25,655 MWh * 1.65 kg/MWh = 42.3 tons NOx |
| # Future target for parameter |  | # As the Project capacity is expanded in later years, the net electricity generation is estimated to increase further and the avoided NOx emissions will increase in parallel. |
| # Way of Monitoring | # How | # Indirectly; by measuring net electricity generation and multiplication with the baseline NOx intensity of the Turkish grid. |
|  | # When | # Continuously |
|  | # By who | # Project Owner |

| # No |  | # SD.1.c |
| --- | --- | --- |
| # Indicator: |  | # Air Quality |
| # Mitigation Measure |  | # n.a. |
| # Chosen Parameter |  | # Reduction of Volatile Organic Compounds. |
| # Estimation of baseline situation of parameter |  | # The non-methane VOCs are typically roughly 2% of methane emissions. The methane destroyed by the Project in the first full operation year is 4,788 tonnes = 6,679,184 m3. Baseline VOCs emissions: 6,679,184 m3 * 2% = 133,584 m3 |
| # Future target for parameter |  | # The VOC destruction by combustion or flaring can be expected to be greater than 98%. This leads to the following # baseline emissions of VOCs: # VOCs Reduced = 133,584 m3 * 98% = 130,912 m3 # The amount of methane destroyed is expected to rise in forthcoming years, increasing also the amount of VOCs reduced. |
| # Way of Monitoring | # How | # By measuring the amount of methane destroyed through combustion or flaring. |
|  | # When | # Continuously |
|  | # By who | # Project Owner |

| # No |  | # SD.1.d |
| --- | --- | --- |
| # Indicator: |  | # Air Quality |
| # Mitigation measures |  | # n.a. |
| # Chosen Parameter |  | # H2S: H2S is emitted from the landfill site freely to the atmosphere, creating bad odour and worsens the living conditions of the nearby communities. Although there are a number of odorous gases in LFG, as a representative indicator of air pollution, hydrogen sulphide is selected because it is a very poisonous, flammable gas with the characteristic foul odour of rotten eggs. |
| # Estimation of baseline situation of parameter |  | # The gas measurements on the Project site show that the H2S concentration in the LFG is 52.8 ppm on average. All of the H2S is freely emitted to the atmosphere. |
| # Future target for parameter |  | # The Project is expected to destroy 4,788 tons of methane in the first year of its operation. This corresponds to 529 kg of avoided H2S. As the Project capacity is expanded in later years, this value is expected to increase in parallel to the combusted LFG amount. |
| # Way of Monitoring | # How | # By measuring LFG destruction and multiplying with the ex- ante baseline H2S intensity of the LFG. |
|  | # When | # Continuously |
|  | # By who | # Project Owner |

| # No |  | # SD.1.e |
| --- | --- | --- |
| # Indicator: |  | # Air Quality |
| # Mitigation measures |  | # n.a. |
| # Chosen Parameter |  | # Reduction of odour |
| # Estimation of baseline situation of parameter |  | # The gases were being emitted freely from the landfill site and blown away in the wind direction, frequently towards nearby neighborhoods. |
| # Future target for parameter |  | # By effective collection and destruction of landfill gas, the foul odor is expected to be reduced. |
| # Way of Monitoring | # How | # Interview with locals |
|  | # When | # Periodically |
|  | # By who | # Project owner |

| # No |  | # SD.2.a |
| --- | --- | --- |
| # Indicator: |  | # Water Quality and Quantity |
| # Mitigation measures |  | # n.a. |
| # Chosen Parameter |  | # Reduction of discharged cooling water in baseline |
| # Estimation of baseline situation of parameter |  | # In 2008, 4.45 billion m3 cooling water was discharged by thermal power plants in Turkey. Net electricity generation in 2008 was 189,76232 MWh, corresponding to 23.5 m3/MWh discharged cooling water intensity. |
| # Future target for parameter |  | # Estimated discharged cooling water reduction of in the first operational year is: 23.5 m3/MWh * 25,655 MWh = 601,614 m3 This amount is expected to increase in forthcoming years. |
| # Way of Monitoring | # How | # The net electricity generation of the Project will be recorded and multiplied with the ex-ante cooling water discharge intensity. |
|  | # When | # Continuously |
|  | # By who | # Project owner |

| # No |  | # SD.2.b |
| --- | --- | --- |
| # Indicator: |  | # Water Quality and Quantity |
| # Mitigation measures |  | # n.a. |
| # Chosen Parameter |  | # Leachate management |
| # Estimation of baseline situation of parameter |  | # The leachate on site is collected in a pond on site and is then transported to the municipal waste water treatment plant with lorries by the Municipality. |
| # Future target for parameter |  | # Most of these tasks are taken over by the Project owner until a leachate treatment system becomes operational at landfill site. Leachate collection, storage and transportation to the treatment plant are properly managed by the Project owner. The Municipality plans to setup a small treatment facility next to the leachate pond. |
| # Way of Monitoring | # How | # Available leachate management records will be evaluated. Leachate transportation truck information will be kindly requested from the Municipality and evidence of such correspondence will be provided for verification purposes. |
|  | # When | # Annually |
|  | # By who | # Project owner |

| # No |  | # SD.3 |
| --- | --- | --- |
| # Indicator: |  | # Soil Condition |
| # Mitigation measure |  | # n.a. |
| # Chosen Parameter |  | # Hazardous waste processing |
| # Estimation of baseline situation of parameter |  | # The landfill is expected not to accept hazardous waste. |
| # Future target for parameter |  | # The project shall not function as a hazardous waste disposal or processing facility (i.e. as a hazardous waste gasification – or incineration facility). |
| # Way of Monitoring | # How | # Assessment of waste management practice. |
|  | # When | # Continuously |
|  | # By who | # Project owner |

| # No |  | # SD.4 |
| --- | --- | --- |
| # Indicator: |  | # Quality of employment |
| # Mitigation measures |  | # n.a. |
| # Chosen Parameter |  | # Safe and healthy working conditions |
| # Estimation of baseline situation of parameter |  | # n.a. |
| # Future target for parameter |  | # Necessary safety equipment’s will be available and properly used. The working environment will not present health risks or hazardous tasks. |
| # Way of Monitoring | # How | # Assessment of working conditions by checking safety material, tasks of the personnel etc. |
|  | # When | # Continuously |
|  | # By who | # Project owner |

| # No |  | # SD.5 |
| --- | --- | --- |
| # Indicator: |  | # Access to affordable and clean energy services |
| # Mitigation measure |  | # n.a. |
| # Chosen Parameter |  | # Avoided fossil fuel consumption. # The Project will generate and supply clean energy to the national grid, improving the carbon intensity of the power mix. |
| # Estimation of baseline situation of parameter |  | # The baseline consists of consumption of different fossil fuels. The latest available (2008) fuel consumption and electricity generation figures will be compared with the Project generation to show how much baseline fuel consumption is avoided on a fuel type basis. # The fossil fuel consumption intensity (t/GWh, m3/GWh) are ex- ante and are as following; # Fuel Type; # Fuel Consumption Intensity [t/GWh, m3/GWh]. # Coal; # 395. # Lignite; # 1,585. # Fuel Oil; # 301. # Diesel Oil; # 492. # LPG; # 0. # Naphtha; # 243. # Natural Gas; # 218. |
| # Future target for parameter |  | # The Project is expected to substitute 25,655 MWh in the first year of operation. This corresponds to the following expected fuel avoidance; # Fuel Type; # Avoided Fuel Consumption [t/GWh, m3/GWh]. # Coal; # 1,026. # Lignite; # 10,862. # Fuel Oil; # 356. # Diesel Oil; # 21. # LPG. # Naphtha; # 2. # Natural Gas; # 3,536. |
| # Way of Monitoring | # How | # It is assumed that the net electricity generation during the monitoring period will be distributed among fossil fuels with the same weight as the fossil fuels. These values will be multiplied with the fuel consumption intensities given above to find the respective reduction of fossil fuel consumption. |
|  | # When | # Continuously |
|  | # By who | # Project owner |

| # No |  | # SD.6 |
| --- | --- | --- |
| # Indicator: |  | # Quantitative employment and income generation |
| # Mitigation measure |  | # n.a. |
| # Chosen Parameter |  | # Number of employees of the project |
| # Estimation of baseline situation of parameter |  | # 0 |
| # Future target for parameter |  | # 11 |
| # Way of Monitoring | # How | # By checking the social security and residence records of personnel. |
|  | # When | # Continuously |
|  | # By who | # Project owner. |

| # No |  | # SD.7 |
| --- | --- | --- |
| # Indicator: |  | # Balance of payments and investment |
| # Chosen Parameter |  | # Value of the imported natural gas avoided (conservative) |
| # Estimation of baseline situation of parameter |  | # Some portion of liquid and solid fossil fuels are extracted in the host country. However, most of the natural gas consumed in the host country is imported. Average natural gas price at the current situation is 3.8 USD/MMBtu. |
| # Future target for parameter |  | # he Project is expected to substitute 25,655 MWh in the first year of operation. This corresponds to a natural gas value of 461,323 USD. Almost all of this natural gas would have to be imported in the absence of the Project activity. |
| # Way of Monitoring | # How | # The avoided natural gas amount will be taken from the indicator “Access to affordable and clean energy services”. The energy content of this amount will then be multiplied by the ex-ante natural gas price. |
|  | # When | # Continuously |
|  | # By who | # Project owner |
| # Monitoring Result |  | # Within the monitoring period the a total of 44,494.78 MWh has been generated through utilization of LFG. This corresponds to a natural gas value of 800,096 USD. |
| # Additional comment: |  | # The same approach of estimating the avoided natural gas value consumption explained under “future target of parameter” has been used to calculate ex-post results. |

# Additional remarks monitoring

There are no additional comments

SECTION H. 	Additionality and conservativeness

H.1.	Additionality

n.a.

# H.2.	Conservativeness

n.a.

ANNEX 1 	ODA declaration

Based on the latest verified monitoring report, the project activity has generated 21,171 MWh electricity within year 2015.