Monitoring Report

PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT - TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

SECTION A - Description of project

SECTION B - Implementation of project

SECTION C - Description of monitoring system applied by the project

SECTION D - Data and parameters

SECTION E - Calculation of SDG Impacts

SECTION F - Safeguards Reporting

SECTION G - Stakeholder inputs and legal disputes

KEY PROJECT INFORMATION

Programme of Activity Information

| GS ID of Programme | GS10959 |
| --- | --- |
| Title of Programme | Safe Water Programme in Africa and Asia |
| Version of POA-DD applicable to this monitoring report | 05 |
| Name and GS ID of fully Validated CPA/VPAs (i.e. non compliance check) | GS10959 VPA01 Safe Water Project in Rwanda I GS10959 VPA02 Safe Water Project in Rwanda II GS10959 VPA03 Safe Water Project in Rwanda III GS10959 VPA04 Safe Water Project in Rwanda IV GS10959 VPA05 Safe Water Project in Rwanda V GS10959 VPA06 Safe Water Project in Rwanda VI GS10959 VPA07 Safe Water Project in Bangladesh I GS10959 VPA08 Safe Water Project in Bangladesh II GS10959 VPA09 Safe Water Project in Bangladesh III GS10959 VPA10 Safe Water Project in Bangladesh IV |

Key Project Information

| GS ID (s) of Project (s) | GS 11098, GS 11133, GS 11134, GS 11135, GS 11136, GS 11137 |
| --- | --- |
| Title of the project (s) covered by monitoring report | GS10959 VPA01 Safe Water Project in Rwanda I GS10959 VPA02 Safe Water Project in Rwanda II GS10959 VPA03 Safe Water Project in Rwanda III GS10959 VPA04 Safe Water Project in Rwanda IV GS10959 VPA05 Safe Water Project in Rwanda V GS10959 VPA06 Safe Water Project in Rwanda VI |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | 08 |
| Version number of the monitoring report | 03 |
| Completion date of the monitoring report | 07/04/2024 |
| Date of project design certification | 13/08/2021 |
| Date of Last Annual Report | N/A |
| Monitoring period number | 2nd |
| Duration of this monitoring period | VPA No.; Start date; End date. VPA01; 25/07/2022; 24/07/2023. VPA02; 25/07/2022; 24/07/2023. VPA03; 25/07/2022; 24/07/2023. VPA04; 25/07/2022; 24/07/2023. VPA05; 25/07/2022; 24/07/2023. VPA06; 25/07/2022; 24/07/2023. |
| Project Representative | Ji BAO |
| Host Country | Rwanda |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | Technologies and Practices to Displace Decentralized Thermal Energy Consumption (Version 3.1) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | VPA No. | Amount Achieved | Units/ Products |
| --- | --- | --- | --- | --- |
| SDG 13 Climate Action | Reduce emission from water boiling by non-renewable biomass | VPA01 | 59,964 | VERs |
|  |  | VPA02 | 56,191 |  |
|  |  | VPA03 | 57,324 |  |
|  |  | VPA04 | 59,233 |  |
|  |  | VPA05 | 59,801 |  |
|  |  | VPA06 | 57,505 |  |
|  |  | Total | 350,018 |  |
| SDG 3 – Good Health and Well-Being | Reduce the incidence of waterborne illness within the project area |  | 38.45% | Percentage |
| SDG 5 – Gender Equality | Reduce the time spent to fetch and purify water by women and girls |  | 51.11% | Percentage |
| SDG 6 – Clean Water and Sanitation | Provide safe water to local residents | VPA01 | 33,750 | Number of persons |
|  |  | VPA02 | 31,855 |  |
|  |  | VPA03 | 32,320 |  |
|  |  | VPA04 | 33,221 |  |
|  |  | VPA05 | 33,514 |  |
|  |  | VPA06 | 32,314 |  |
|  |  | Total | 196,974 |  |

Table 2 – Product Vintages

|  |  |  | Amount Achieved |
| --- | --- | --- | --- |
| VPA No. | Start Dates | End Dates | VERs |
| VPA01 | 25/07/2022 | 31/12/2022 | 26,286 |
|  | 01/01/2023 | 24/07/2023 | 33,678 |
| VPA02 | 25/07/2022 | 31/12/2022 | 24,632 |
|  | 01/01/2023 | 24/07/2023 | 31,559 |
| VPA03 | 25/07/2022 | 31/12/2022 | 25,128 |
|  | 01/01/2023 | 24/07/2023 | 32,196 |
| VPA04 | 25/07/2022 | 31/12/2022 | 25,965 |
|  | 01/01/2023 | 24/07/2023 | 33,268 |
| VPA05 | 25/07/2022 | 31/12/2022 | 26,214 |
|  | 01/01/2023 | 24/07/2023 | 33,587 |
| VPA06 | 25/07/2022 | 31/12/2022 | 25,208 |
|  | 01/01/2023 | 24/07/2023 | 32,297 |
|  |  | Total | 350,018 |

SECTION A. DESCRIPTION OF PROJECT

A.1. General description of project

>>

Many people in rural area of Rwanda rely on boreholes to provide clean water. Unfortunately, a lot of boreholes have fallen into disrepair because maintenance has been poorly managed due to lack of capacity, organization or fund. The VPAs, which are the VPA01 to VPA06 for PoA GS 10959 “Safe Water Programme in Africa and Asia” (Hereinafter referred to as “the PoA”), consist of the maintenance of 251 boreholes in Southern Province of Rwanda. The CME cooperates with local NGO, Association Rwandaise pour le Développement Endogène (Hereinafter referred to as “ARDE”), to implement the VPAs for providing safe water to local communities and ensure the water quality to meet the related requirements of Rwanda and Gold Standard for the Global Goals. The project boundary is the boundary of communities that use the boreholes maintained by the project activity.

Before the implementation of the project, local communities in the project location used fossil fuel and/or non-renewable biomass (Hereinafter referred to as “NRB”) to boil water for purification. Therefore, the baseline scenario is that fossil fuel and/or NRB is used to boil water as means of water purification in the absence of the project activity. As a result, water purification through boiling with wood made local people vulnerable to the negative effects of poor indoor air quality. In Rwanda, 31% of its population lack access to clean water and about 3,000 children die each year from diarrhea.  Boiling water with wood also resulted in significant greenhouse gas emissions through the use of non-renewable biomass, causing deforestation and threatening biodiversity. In addition, usually in local communities it is women and girls that take the unpaid work of fetching and purifying water, which minimizes their time for rest and study, and even their opportunities to have paid jobs.

The project provides a solution to mitigate the above problems. The fund from sale of carbon credits generated by the project makes it sustainable and extendable.

A.2. Location of project

>>

The project is located in Southern Province of Rwanda. The 251 boreholes spread over Muhanga District, Kamonyi District, Nyanza District, Nyamagabe District and Ruhango District. The geographic coordinates of Geographic coordinates of Southern Province are shown as follows.

Table 1: Geographic coordinates of Southern Province

| Name of Province | Eastmost | Southmost | Westmost | Northmost |
| --- | --- | --- | --- | --- |
| Southern Province | 2°6’7”S 30°1’15”E | 2°50’24”S 29°22’16”E | 2°28’55”S 29°15’53”E | 1°43’53”S 29°40’7”E |

Figure 1: Southern Province of Rwanda

A.3. Reference of applied methodology

>>

Technologies and Practices to Displace Decentralized Thermal Energy Consumption (Version 3.1)

A.4. Crediting period of project

>>

The crediting period for each VPA is:

GS 11098: 27/11/2021-26/11/2026, twice renewable to a total of 15 years

GS 11133: 26/11/2021-25/11/2026, twice renewable to a total of 15 years

GS 11134: 26/11/2021-25/11/2026, twice renewable to a total of 15 years

GS 11135: 09/01/2022-08/01/2027, twice renewable to a total of 15 years

GS 11136: 12/12/2021-11/12/2026, twice renewable to a total of 15 years

GS 11137: 10/01/2022-09/01/2027, twice renewable to a total of 15 years

SECTION B. IMPLEMENTATION OF PROJECT

B.1. Description of implemented project

>>

The major borehole maintenance activity is to repair broken hand pumps. India Mark V Hand Pump is used in the VPA. The India Mark V hand pump incorporates the latest

R & D in hand pump technology. It is based on non-corrodible below-ground components, using modern lightweight engineering plastics such as սPVC Riser Pipes and Fibre Reinforced Glass (FRG) Connecting Rods.

Technology:

Table 2: Manufacturer’s specifications of India Mark V Hand Pump

| Specification | Capacity |
| --- | --- |
| Stoke Length | 175 mm |
| Open Top Cylinder | 65mm |
| Number of persons can be served | 1,000-1,500 |
| Lifetime | 20 years |

Figure 2: Photo of India Mark V Hand Pump

There are 251 boreholes repaired in the project. In the monitoring period, the boreholes were maintained by the CME and VPA implementer, functioning well to provide enough clean water to over 190,000 local people.

There are no deviations or delays compared to information in Design Certified PDD.

Table 3:   Milestone for implementation of the project

| Time | Main action |
| --- | --- |
| 23/03/2021 | Time of First Submission Date |
| 13/08/2021 | Date of project design certification |
| 03/09/2021 | Local stakeholder consultation meeting |
| 15/11/2021 | Starting sign Donation and Carbon Transfer Agreements with villages |
| 11/2021 | Starting repair boreholes |
| 26/11/2021 | The first borehole was repaired |
| 16/05/2022-24/05/2022 | The first time of water quality test |
| 20/06/2022 | The last borehole was repaired |
| 26/07/2022-18/08/2022 | Water consumption field test was conducted |
| 25/07/2022-25/08/2022 | Project survey was conducted |
| 24/08/2022 | 2nd Water Quality Test |
| 15/11/2022 | 3rd Water Quality Test |
| 15/11/2022 | Hygiene campaign held at Mbuye sector, Nyakarekare cell Bereshi village |
| 15/11/2022 | Hygiene campaign held at Mugina sector, Kabugondo cell, Mataba sud village |
| 21/11/2022 | Hygiene campaign held at Byimana sector, Kamusenyi cell Kabusheshe village |
| 21/11/2022 | Hygiene campaign held at Kibirizi sector, Rwotso cell Seruhembe I village |
| 22/11/2022 | Hygiene campaign held at Ruhango sector, Munini cell Kabaja village |
| 22/11/2022 | Hygiene campaign held at Kinazi sector, Kinazi cell Kacyiru II village |
| 24/02/2023 | 4th Water Quality Test |
| 24/05/2023 | 5th Water Quality Test |

The dates of each borehole repaired are recorded in Excel spreadsheet “1-SDG Outcomes Calculation Form-MP2nd”. The locations, quantity of users, days out of work of each borehole are recorded too. During this monitoring period, a few boreholes have been out of work for several days, which is excluded for calculating emission reduction. The information of each VPA is shown as follows:

Table 4:   Information of each VPA

| VPA No. | Rehabilitation date of the first borehole in the VPA | Rehabilitation date of the last borehole in the VPA | Start date of MP2nd | End date of MP2nd | Number of boreholes rehabilitated |
| --- | --- | --- | --- | --- | --- |
| 01 | 27/11/2021 | 01/06/2022 | 25/07/2022 | 24/07/2023 | 42 |
| 02 | 26/11/2021 | 23/02/2022 | 25/07/2022 | 24/07/2023 | 42 |
| 03 | 26/11/2021 | 23/02/2022 | 25/07/2022 | 24/07/2023 | 38 |
| 04 | 09/01/2022 | 20/06/2022 | 25/07/2022 | 24/07/2023 | 46 |
| 05 | 12/12/2021 | 25/05/2022 | 25/07/2022 | 24/07/2023 | 42 |
| 06 | 10/01/2022 | 09/06/2022 | 25/07/2022 | 24/07/2023 | 41 |
|  |  |  |  | Total | 251 |

Number of Days out of work after rehabilitation for project boreholes during this monitoring period according to the maintenance log:

| Borehole ID | Number of Days out of work during MP2nd |
| --- | --- |
| IR3 | 2 |
| IR4 | 7 |
| IR7 | 7 |
| IR10 | 3 |
| IR18 | 14 |
| IR19 | 7 |
| IR26 | 10 |
| IR27 | 9 |
| IR28 | 6 |
| IR29 | 7 |
| IR33 | 5 |
| IR37 | 9 |
| IR69 | 15 |
| IR84 | 5 |
| IR92 | 3 |
| IR94 | 4 |
| IR100 | 2 |
| IR102 | 4 |
| IR103 | 5 |
| IR105 | 6 |
| IR106 | 19 |
| IR107 | 7 |
| IR109 | 3 |
| IR111 | 4 |
| IR112 | 2 |
| IR113 | 4 |
| IR114 | 8 |
| IR116 | 6 |
| IR118 | 16 |
| IR119 | 5 |
| IR120 | 3 |
| IR122 | 2 |
| IR123 | 16 |
| IR124 | 9 |
| IR125 | 12 |
| IR128 | 12 |
| IR131 | 9 |
| IR132 | 7 |
| IR133 | 6 |
| IR134 | 7 |
| IR139 | 6 |
| IR141 | 5 |
| IR142 | 4 |
| IR143 | 8 |
| IR148 | 3 |
| IR71 | 5 |
| IR72 | 10 |
| IR76 | 8 |
| IR85 | 5 |
| IR87 | 4 |
| IR89 | 3 |
| IR91 | 4 |
| IR95 | 3 |
| IR98 | 3 |
| IR99 | 4 |
| IR137 | 3 |
| IR147 | 13 |
| IR270 | 17 |
| IR274 | 5 |
| IR275 | 4 |
| IR277 | 3 |
| IR278 | 9 |
| IR279 | 6 |
| IR149 | 6 |
| IR150 | 6 |
| IR152 | 5 |
| IR153 | 5 |
| IR155 | 5 |
| IR237 | 8 |
| IR252 | 7 |
| IR296 | 6 |
| IR306 | 9 |
| IR313 | 10 |
| IR315 | 9 |
| IR166 | 8 |
| IR170 | 9 |
| IR180 | 6 |
| IR181 | 5 |
| IR182 | 9 |
| IR204 | 8 |
| IR218 | 8 |
| IR224 | 7 |
| IR235 | 7 |

There was no change from the project design that was envisaged at design certified VPA-DD in the monitoring period.

B.1.1 Forward Action Requests

>>

FAR from verification of the MP1:

The PD shall ensure that the next usage survey covers the question of whether the household has obtained access to an alternative water purification technology, such as household water filters, or access to other boreholes, since the start of the project. If such exist, the PD shall clarify if access to such technology is granted under another carbon credit project. If this is the case, the PD shall clarify how double claiming is avoided. The VVB shall verify this claim.

Reply: The PD has put the question of “If the household has obtained access to an alternative water purification technology, such as household water filters, or access to other boreholes, since the start of the project?” in Usage Survey during this monitoring period. Please refer to Question 5 of submitted document ”6-Usage Survey Form”. According to the result of the Usage Survey, there is no access to an alternative water purification technology.

B.2. Post-Design Certification changes

>>

B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

>>

N/A.

B.2.2. Corrections

>>

N/A.

B.2.3. Changes to start date of crediting period

>>

N/A.

B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

>>

N/A.

B.2.5. Changes to project design of approved project

>>

N/A.

SECTION C. DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

>>

ARDE is in charge of the implementation of the monitoring plan and reporting to the CME. The executive director of ARDE is responsible for supervising the whole monitoring procedure. The water and environment department is responsible for conducting baseline and project surveys as well as reporting to the executive director. The CME is in charge of designing the monitoring plan and completing the monitoring report.

Training about monitoring plan has been provided to ARDE, including survey method, data record and analysis. The monitoring plan has been carried out by qualified personnel trained for quality assurance and quality control. The CME inspects ARDE to confirm that the personnel are qualified and the monitoring plan is properly implemented.

A figure about the operational and management structure for monitoring is shown as the following:

CME

Executive Director of ARDE

Water and Environment Depart of ARDE

Figure 3:  Operational and Management Structure for Monitoring

Borehole database

The records of the information collected in an electronic database for enabling the unique identification of the project boreholes has been kept. The database includes the following information:

- Unique identification number of each borehole

- Date of rehabilitation of each borehole

- Quantity of boreholes rehabilitated

- GPS location of each borehole

- Model of the pump for each borehole

- The total number of people obtaining their water from each borehole

- Maintenance log recorded by trained staff

All the monitoring related data will be kept by the CME and ARDE for at least two years after the crediting period or the last issuance of the project.

Every village which agrees to participate in the project has established a Water Management Committee (Hereinafter referred to as “WMC”). The members of the WMC are respectable persons elected by villagers.

Borehole maintenance

After the rehabilitation of the boreholes, the WMC is responsible for reporting all the problems of the boreholes to the CME and ARDE. And there are trained local staff of ARDE working with the WMC. If the borehole is out of work, the trained local staff shall get there as soon as possible. They can solve common faults. If they can’t solve the problem, the CME and ARDE will offer more technical support and the issues can quickly be resolved through this direct line of communication.

In the case of repair is necessary, the total number of days when the borehole is out of work is recorded. This information is then used to adjust person·days for emission reductions calculation. During the monitoring period, there were a few boreholes out of work for a few days. The person·days has been adjusted.

Ongoing monitoring studies

The following ongoing monitoring studies have been conducted. The results are given in the parameter boxes in Section D.2. As per “Standard:  Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”, the 95/10 rule should be applied for cross VPA sampling.

- Water consumption field test (Hereinafter referred to as “WCFT”)(Parameters Qp,y , Qp,rawboil,y and Qp,cleanboil,y)- Completed at least biennially. The survey was conducted by local staff who trained before the survey in line with methodological requirements, which has been verified during last monitoring period. The WCFT distinguished between quantity of raw or unsafe water that is still boiled and quantity of safe water boiled after the implementation of the borehole rehabilitation project. Surveyors assessed the volume of water with jerricans. This process was repeated each day until an average daily use figure has been calculated, based on household size. The WCFT was conducted in four consecutive working days. Only water for drinking, cooking and basic personal hygiene was counted. As per the methodology, results of each household were capped at 7L. Hence the result of Qp,y is 6.66L. The result of WCFT is recorded in Excel spreadsheet “3-Data Recording Form for WCFT”, and has been submitted too. We used multi-stage sampling according to “Standard:  Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” to conduct Project Survey. We calculated through Equation (77) on Page 62 of “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” and concluded that 3 was the minimum number of boreholes should be sampled to satisfy the 95/10 rule of applied methodology. Hence, we randomly chose 8 villages of 8 boreholes. We randomly selected 20 households from each village, 160 households in total in 8 villages.

- Quality of the treated water - The quality of the treated water is assessed to ensure that it fits human consumption. The standard used to assess the water quality meets requirements of the applied methodology. It is conducted quarterly, with the first test conducted within 6 months of the project start date. At least once every two years, the water quality test should be performed in accredited laboratories. The samples were selected by Simple Random Sampling. We used equation (1) of “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” to calculate the sample size. The result is 18.78. Furthermore, according to paragraph 14 of “Standard:  Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, a minimum sample size of 30 shall be chosen. We chose 35 samples out of 251 boreholes to ensure the precision. We also used CDM sample size calculator for checking the precision of the sampling. The following result shows that it meets the requirement:

The water quality tests were conducted on 24/08/2022, 15/11/2022, 24/02/2023 and 25/05/2023 in the IPRC Kigali Engineering Laboratory during this monitoring period, which is a national accredited laboratory. The detailed results of each test have been submitted as a separate document.

- Usage Survey - Taken annually. The Usage Survey for the first monitoring period was included in the Project Survey. There was no individual questionnaire for Usage Survey. The Project Survey was carried out between 25/07/2022 to 25/08/2022, which surveyed the usage rate between 26/11/2021 to 24/07/2022. The Usage Survey for the second monitoring period was carried out between 12/08/2023 to 06/09/2023, which surveyed the usage rate between 25/07/2022 to 24/07/2023. Hence, the usage rate surveyed annually, which complies with the monitoring frequency.  We used multi-stage sampling according to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” to conduct Usage Survey. We calculated through Equation (33) of “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” and concluded that 1 was the minimum number of boreholes should be sampled to satisfy the 95/10 rule of applied methodology. Since we randomly chose 8 boreholes from all the VPAs and 20 end users from each borehole, 95/10 has been met. The result from the Usage Survey is 100% of the surveyed households using the project borehole, then 100% is used to calculate emission reductions of this monitoring period.

- Project Survey – Completed at least biennially. We used multi-stage sampling according to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” to conduct Project Survey. We calculated through Equation (77) on Page 62 of “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)” and concluded that 7 was the minimum number of boreholes should be sampled to satisfy the 95/10 rule of applied methodology. Hence, we randomly chose 8 villages of 8 boreholes. We randomly selected 20 households from each village, 160 households in total in 8 villages. Data collected during the project surveys explores the following characteristics:

  - General information - Name, address, telephone number etc.

  - Household socio-demographic information.

  - Water use characteristics.

During last monitoring period the Project Survey has been conducted between 25/07/2022 to 25/08/2022. A more detailed description of the results is made in a separate document “2-Data Recording Form for Project Survey”, and has been submitted too.

- Leakage - The PDD has described what sources of leakage were relevant to the VPA and what have been discounted.

The potential sources of leakage listed in the methodology have been investigated, and addressed below:

a) The displaced baseline technologies are reused outside the project boundary in place of lower emitting technology or in a manner suggesting more usage than would have occurred in the absence of the project.

The displaced baseline technology is three stones. It is not reused outside the project boundary because it is still used for cooking after the implementation of the Project. This leakage source can therefore be discounted.

b) Non-project users who previously used lower emitting energy sources with NRB or fossil fuels saved under the project activity.

The costs of low emitting water purification technologies, such as filtration and chlorination, are much higher than boiling with wood fuel. Users of these technologies are not price sensitive. Therefore, the implementation of the Project does not lead these users to boil water with wood fuel, even if the price of wood fuel becomes cheaper because of the reduction of demand caused by the Project. This leakage source can therefore be discounted.

c) The project significantly impacts the NRB fraction within an area where other CDM or VER project activities account for NRB fraction in their baseline scenario.

Considering that the project only saves 190,310 tons (Bb,y) of biomass annually while the total amount of above-ground biomass of Rwanda is 75 million tons, the Project does not affect NRB fraction.

d) The VPA population compensates for loss of the space heating effect of inefficient technology by adopting some other form of heating or by retaining some use of inefficient technology.

The space heating effect of boiling water is negligible. Therefore, it is highly unlikely that some other form of heating will be adopted for compensating the space heating effect of boiling water.

e) By virtue of promotion and marketing of new technology with high efficiency, the project stimulates substitution within households who commonly used a technology with relatively lower emissions, in cases where such a trend is not eligible as an evolving baseline.

The project does not promote any new technology with high efficiency. It does not stimulate people to boil water.

Therefore, a value of 0 is applied for leakage during this monitoring period.

Individual participants were selected from the borehole user data base using the random sampling process outlined in the monitoring plan. Sample sizes are in line with the Gold Standard requirements.

Hygiene campaign - Taken annually. We have held six hygiene campaigns in six villages from six VPA respectively during this monitoring period. The information is shown below:

| Date | Location |
| --- | --- |
| 15/11/2022 | Hygiene campaign held at Mbuye sector, Nyakarekare cell Bereshi village |
| 15/11/2022 | Hygiene campaign held at Mugina sector, Kabugondo cell, Mataba sud village |
| 21/11/2022 | Hygiene campaign held at Byimana sector, Kamusenyi cell Kabusheshe village |
| 21/11/2022 | Hygiene campaign held at Kibirizi sector, Rwotso cell Seruhembe I village |
| 22/11/2022 | Hygiene campaign held at Ruhango sector, Munini cell Kabaja village |
| 22/11/2022 | Hygiene campaign held at Kinazi sector, Kinazi cell Kacyiru II village |

Each hygiene campaign was held with many habitants of villages. The agenda of each campaign was:

a) The campaign was started with greeting from village’s leader.

b) Self-introduction of village administration committee leaders and local boreholes rehabilitation project staff.

c) Presentation of prepared module related to Hygiene and Sanitation campaign done by an expert from the borehole rehabilitation project.

d) Questions, wishes and recommendations proposed by habitants.

e) Community work of cleaning on boreholes.

The detailed description of the hygiene campaigns during this monitoring period has been provided in a separate document.

The above hygiene campaigns are for the first year of the crediting period （from 26/11/2021 to 25/11/2022）. The hygiene campaigns for the second year of the crediting period (from 26/11/2022 to 25/11/2023) were held from 18/10/2023 to 31/10/2023. Since they do not fall in the current monitoring period, they will be reported in the MR of the next monitoring period.

SECTION D. DATA AND PARAMETERS

D.1. Data and parameters fixed ex ante or at renewal of crediting period

>>

SDG13

| Data/parameter | Cj |
| --- | --- |
| Unit | Percentage |
| Description | Percentage of users of project technology who were already in baseline using a non-boiling safe water supply |
| Source of data | Baseline survey in registered PDD |
| Value(s) applied | 0.0083 |
| Choice of data or Measurement methods and procedures | The data is obtained through baseline survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The Baseline Survey was conducted by representative and random sampling during from 08/06/2021 to 15/06/2021. The result is 0.0083. |
| Purpose of data | Calculation of baseline and project emissions (SDG 13) as well as number of persons consuming safe water supplied by the Project (SDG 6) |
| Additional comment | Also used for SDG 6 |

| Data/parameter | - Xboil |
| --- | --- |
| Unit | - Percentage |
| Description | - Percentage of premises that in the absence of the project activity would have used non-GHG emitting technologies like chlorine treatment techniques (if available) in the project boundary. |
| Source of data | - Baseline survey in registered PDD |
| Value(s) applied | - 0 |
| Choice of data or Measurement methods and procedures | - The data is obtained through baseline survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The Baseline Survey was conducted by representative and random sampling during from 08/06/2021 to 15/06/2021. The result is 0. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | Wb,y |
| --- | --- |
| Unit | t/L |
| Description | Quantity of wood fuel or fossil fuel required to boil 1 litre of water using technologies representatives of baseline scenario b during year y |
| Source of data | Default value |
| Value(s) applied | 0.0004 |
| Choice of data or Measurement methods and procedures | - The default value of Wb,y for firewood as per "Application of TPDDTEC Methodology to Safe Water Supply Projects" |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | Should be updated if ongoing monitoring surveys show that baseline water boiling technologies change over time. |

| Data/parameter | Wp,y |
| --- | --- |
| Unit | t/L |
| Description | Quantity of wood fuel or fossil fuel required to boil 1 litre of water using technologies representatives of project scenario p during year y |
| Source of data | Default value |
| Value(s) applied | 0.0004 |
| Choice of data or Measurement methods and procedures | - According to the baseline and project survey, the same water boiling technology is applied in the baseline and project scenarios. So Wb,y and Wp,y are equal |
| Purpose of data | - Calculation of project emissions |
| Additional comment | Should be updated if ongoing monitoring surveys show that baseline water boiling technologies change over time. |

| Data/parameter | fNRB,b,y |
| --- | --- |
| Unit | Percentage |
| Description | Fraction of biomass used that can be established as non – renewable biomass in the baseline scenario b during year y |
| Source of data | As per the “TOOL30: Calculation of the fraction of non-renewable biomass, version 3.0”. Please refer to registered PDD. |
| Value(s) applied | 96.82% |
| Choice of data or Measurement methods and procedures | - - |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | EFb,fuel,co2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor of fuels used in the baseline scenario |
| Source of data | IPCC default value for Wood: IPCC 2006 Guidelines for National Greenhouse gas Inventories Chapter 2: Stationary Combustion Page 2.23 Table 2.5 |
| Value(s) applied | 112 |
| Choice of data or Measurement methods and procedures | - According to the baseline survey, wood is the only fuel used in the baseline scenario. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | GWPCH4 |
| --- | --- |
| Unit | - |
| Description | Global warming potential for CH4 |
| Source of data | IPCC Fifth Assessment Report: Climate Change 2014 |
| Value(s) applied | 28 |
| Choice of data or Measurement methods and procedures | - As per “IPCC Fifth Assessment Report: Climate Change 2014”, the value is 28. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | GWPN2O |
| --- | --- |
| Unit | - |
| Description | Global warming potential for N2O |
| Source of data | IPCC Fifth Assessment Report: Climate Change 2014 |
| Value(s) applied | 265 |
| Choice of data or Measurement methods and procedures | - As per “IPCC Fifth Assessment Report: Climate Change 2014”, the value is 265. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | EFb,fuel,non co2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non CO2 emission factor of fuels used in the baseline scenario |
| Source of data | IPCC default value for Wood: IPCC 2006 Guidelines for National Greenhouse gas Inventories Chapter 2: Stationary Combustion Page 2.23 Table 2.5 IPCC Fifth Assessment Report: Climate Change 2014 |
| Value(s) applied | 9.46 |
| Choice of data or Measurement methods and procedures | - According to the baseline survey, wood is the only fuel used in the baseline scenario. - As per “IPCC 2006 Guidelines for National Greenhouse gas Inventories”, the default emission factor of CH4 and N2O for stationary combustion is 0.3t/TJ and 0.004t/TJ, respectively. As per “IPCC Fifth Assessment Report: Climate Change 2014”, the global warming potential for CH4 and N2O is 28 and 265, respectively. So EFb,non co2 = 0.328+0.004265 = 9.46. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | NCVb,fuel |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the baseline |
| Source of data | IPCC default value for wood IPCC (2006) "IPCC Guidelines for National Greenhouse Gas Inventories", Volume 2, Energy, Chapter 1, Introduction, Page 1.19, Table 1.2 |
| Value(s) applied | 0.0156 |
| Choice of data or Measurement methods and procedures | - According to the baseline survey, wood is the only fuel used in the baseline scenario. |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data/parameter | fNRB,p,y |
| --- | --- |
| Unit | Percentage |
| Description | Fraction of biomass used that can be established as non – renewable biomass in the project scenario p during year y |
| Source of data | As per the “TOOL30: Calculation of the fraction of non-renewable biomass, version 3.0”. Please refer to registered PDD. |
| Value(s) applied | 96.82% |
| Choice of data or Measurement methods and procedures | - - |
| Purpose of data | - Calculation of project emissions |
| Additional comment | - |

| Data/parameter | EFp,fuel,co2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | CO2 emission factor of fuels used in the project scenario |
| Source of data | IPCC default value for Wood: IPCC 2006 Guidelines for National Greenhouse gas Inventories Chapter 2: Stationary Combustion Page 2.23 Table 2.5 |
| Value(s) applied | 112 |
| Choice of data or Measurement methods and procedures | - According to the project survey, wood is the only fuel used in the project scenario. |
| Purpose of data | - Calculation of project emissions |
| Additional comment | - |

| Data/parameter | EFp,fuel,non co2 |
| --- | --- |
| Unit | tCO2/TJ |
| Description | Non CO2 emission factor of fuels used in the project scenario |
| Source of data | IPCC default value for Wood: IPCC 2006 Guidelines for National Greenhouse gas Inventories Chapter 2: Stationary Combustion Page 2.23 Table 2.5 IPCC Fifth Assessment Report: Climate Change 2014 |
| Value(s) applied | 9.46 |
| Choice of data or Measurement methods and procedures | - According to the project survey, wood is the only fuel used in the project scenario. - As per IPCC 2006 Guidelines for National Greenhouse gas Inventories, the default emission factor of CH4 and N2O for stationary combustion is 0.3t/TJ and 0.004t/TJ, respectively. As per IPCC Fifth Assessment Report: Climate Change 2014, the global warming potential for CH4 and N2O is 28 and 265, respectively. So EFb,non co2 = 0.328+0.004265 = 9.46. |
| Purpose of data | - Calculation of project emissions |
| Additional comment | - |

| Data/parameter | NCVp,fuel |
| --- | --- |
| Unit | TJ/ton |
| Description | Net calorific value of the fuels used in the project scenario |
| Source of data | IPCC default value for wood IPCC (2006) "IPCC Guidelines for National Greenhouse Gas Inventories", Volume 2, Energy, Chapter 1, Introduction, Page 1.19, Table 1.2 |
| Value(s) applied | 0.0156 |
| Choice of data or Measurement methods and procedures | - According to the project survey, wood is the only fuel used in the project scenario. |
| Purpose of data | - Calculation of project emissions |
| Additional comment | - |

SDG 3

| Data/parameter | Ib |
| --- | --- |
| Unit | Percentage |
| Description | Waterborne illness incidence in the baseline scenario |
| Source of data | Baseline survey in the registered PDD |
| Value(s) applied | 38.45% |
| Choice of data or Measurement methods and procedures | - The data is obtained through baseline survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The Baseline Survey was conducted by representative and random sampling from 08/06/2021 to 15/06/2021. The result is 38.45%. |
| Purpose of data | - Calculation of reduction of waterborne illness incidence |
| Additional comment | - |

SDG 5

| Data/parameter | Tb |
| --- | --- |
| Unit | Hour |
| Description | Time spent to fetch and purify water by women and girls per household per day in the baseline scenario |
| Source of data | Baseline survey in the registered PDD |
| Value(s) applied | 0.90 |
| Choice of data or Measurement methods and procedures | - The data is obtained through baseline survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The Baseline Survey was conducted by representative and random sampling from 08/06/2021 to 15/06/2021. The result is 0.90. |
| Purpose of data | - Calculation of percentage reduction of time spent to fetch and purify water by women and girls |
| Additional comment | - |

D.2 Data and parameters monitored

>>

SDG 13

| Data/parameter | Qp,y |
| --- | --- |
| Unit | Litres per person per day |
| Description | Quantity of safe water in litres consumed in the project scenario p and supplied by project technology per person per day |
| Source of data | Water consumption field test |
| Value(s) applied | 6.66 |
| Measurement methods and procedures | - The data obtained by WCFT through sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The WCFT was conducted by CME and ARDE from 26/07/2022 to 18/08/2022. The actual value is 8.96L. According to the applied methodology, the cap value for full-day premises is 7. So we capped this value as 7 for each household participating in the WCFC. As an average value for all households, the result is 6.66. |
| Monitoring frequency | - At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | - Transparent data analysis and reporting |
| Purpose of data | - Calculation of baseline emissions |
| Additional comment | - |

| Data / Parameter | Qp,rawboil,y |
| --- | --- |
| Unit | Litres per person per day |
| Description | Quantity of raw or unsafe water that is still boiled after installation of the water treatment technology. |
| Source of data | Water consumption field test |
| Value(s) applied | 0 |
| Measurement methods and procedures | The data is obtained by water consumption field test through sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The WCFT was conducted by the CME and ARDE from 26/07/2022 to 18/08/2022 and the result is 0. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of baseline and project emissions |
| Additional comment |  |

| Data / Parameter | Qp,cleanboil,y |
| --- | --- |
| Unit | Litres per person per day |
| Description | Quantity of safe (treated, or from safe supply) water boiled in the project scenario p, after installation of project technology |
| Source of data | Water consumption field test |
| Value(s) applied | 0 |
| Measurement methods and procedures | The data is obtained by water consumption field test through sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The WCFT was conducted by the CME and ARDE from 26/07/2022 to 18/08/2022 and the result is 0. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of project emissions |
| Additional comment |  |

| Data / Parameter | Quality of the treated water |
| --- | --- |
| Unit | - |
| Description | As a first option, projects shall meet host country standards (where available) for treated water quality. Where national standards are not available, projects shall meet WHO standard of less than 1 Colony Forming Unit (CFU) of E.Coli /100 ml. |
| Source of data | Water quality test |
| Value(s) applied | Date; No.; Result. 24/08/2022; 2nd Water Quality Test; Pass. 15/11/2022; 3rd Water Quality Test; Pass. 24/02/2023; 4th Water Quality Test; Pass. 24/05/2023; 5th Water Quality Test; Pass. |
| Measurement methods and procedures | As per the local laboratories’ methods and procedures. The 95/10 precision rule must be followed in calculating the sample size required for testing water quality. |
| Monitoring frequency | Quarterly. The first test has been conducted within 6 months of the project start date. At least once every two years, the water quality test should be performed in accredited laboratories. The water quality tests have been made in the Water Quality Testing Laboratory of the IPRC Kigali, which is a national accredited laboratory. The lab collected the samples from the water sources. |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of emission reductions |
| Additional comment | - |

| Data / Parameter | Up,y |
| --- | --- |
| Unit | Percentage |
| Description | Usage rate in project scenario p during year y |
| Source of data | Annual usage survey |
| Value(s) applied | 100% |
| Measurement methods and procedures | It is determined by usage survey in this monitoring period with sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. And the Project Survey and Usage Survey have been conducted together using the same sample, thus there is no separate documents. Please refer to document “2-Data Recording Form for Project Survey”. |
| Monitoring frequency | Annually; The Usage Survey was carried out between 12/08/2023 to 06/09/2023. The result from the Usage Survey is 100% of the surveyed households using the project borehole, then 100% is used to calculate emission reductions of this monitoring period. |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of emission reductions (SDG 13) and number of persons consuming safe water supplied by the Project (SDG 6) |
| Additional comment | A single usage parameter is weighted to be representative of the quantity of project technologies of each age being credited in a given project scenario as per Section 3.1 of the applied methodology. Also used for SDG 6. |

| Data / Parameter | Np,y |
| --- | --- |
| Unit | Persons.days |
| Description | Number of person.days consuming water supplied by project scenario p through year y |
| Source of data | D16-22 of sheet SDG13 of Excel spreadsheet “1-SDG Outcomes Calculation Form-MP2nd” |
| Value(s) applied | Each VPA is shown as follows: VPA No.; Persons.days. 01; 12,340,897. 02; 11,564,471. 03; 11,797,656. 04; 12,190,541. 05; 12,307,382. 06; 11,834,797. Total; 72,035,744. |
| Measurement methods and procedures | The number of persons who use the project borehole is sourced from Water Management Committee of each borehole as per Document “Sworn Statement”. Sum of the total number of people using each borehole in the VPA multiplied by the number of operating days in the monitoring period. The number of days when the boreholes are in operation is monitored through project survey. The number of days when the boreholes are out of use is not taken into account for calculating Np,y. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of baseline emissions and project emissions |
| Additional comment | - |

| Data / Parameter | LEp,y |
| --- | --- |
| Unit | tCO2e per year |
| Description | Leakage in project scenario p during year y |
| Source of data | Baseline and monitoring surveys |
| Value(s) applied | 0 |
| Measurement methods and procedures | The result is 0 and the details are shown in Section E.3. |
| Monitoring frequency | Biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of emission reductions |
| Additional comment | - |

SDG 3

| Data / Parameter | Ip,y |
| --- | --- |
| Unit | Percentage |
| Description | Waterborne illness incidence in the project scenario during year y |
| Source of data | B127 of Excel spreadsheet “2-Data Recording Form for Project Survey” |
| Value(s) applied | 0 |
| Measurement methods and procedures | The data is obtained by project survey through sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The project survey was conducted by the CME and ARDE from 25/07/2022 to 25/08/2022 and the result is 0. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of reduction of waterborne illness incidence |
| Additional comment |  |

SDG 5

| Data / Parameter | Tp,y |
| --- | --- |
| Unit | Hour |
| Description | Time spent to fetch and purify water by women and girls per household per day in the project scenario during year y |
| Source of data | B128 of Excel spreadsheet “2-Data Recording Form for Project Survey” |
| Value(s) applied | 0.44 |
| Measurement methods and procedures | The data is obtained by project survey through sampling survey as per the applied methodology as well as “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”. The project survey was conducted by the CME and ARDE from 25/07/2022 to 25/08/2022 and the result is 0.44. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of percentage reduction of time spent to fetch and purify water by women and girls |
| Additional comment |  |

SDG 6

| Data / Parameter | Pp,y |
| --- | --- |
| Unit | Number |
| Description | Number of persons consuming water within the project area during year y |
| Source of data | Project survey |
| Value(s) applied | Each VPA is shown as follows: VPA No.; Pp,y. 01; 33,750. 02; 31,855. 03; 32,320. 04; 33,221. 05; 33,514. 06; 32,314. Total; 196,974. |
| Measurement methods and procedures | The data is obtained from Water Management Committee of each borehole as per Document “Sworn Statement”. |
| Monitoring frequency | At least biennially; the above value is valid from 26/11/2021 to no later than 25/11/2023 as per the applied methodology |
| QA/QC procedures | Transparent data analysis and reporting |
| Purpose of data | Calculation of number of persons consuming safe water supplied by the project |
| Additional comment |  |

| Data / Parameter | Hygiene campaigns |
| --- | --- |
| Unit | - |
| Description | Hygiene campaigns carried out among project technology users. |
| Source of data | Annual hygiene campaigns results |
| Value(s) applied | We have held six hygiene campaigns in six villages from six VPA respectively during this monitoring period. The information is shown below: Date; Location. 15/11/2022; Hygiene campaign held at Mbuye sector, Nyakarekare cell Bereshi village. 15/11/2022; Hygiene campaign held at Mugina sector, Kabugondo cell, Mataba sud village. 21/11/2022; Hygiene campaign held at Byimana sector, Kamusenyi cell Kabusheshe village. 21/11/2022; Hygiene campaign held at Kibirizi sector, Rwotso cell Seruhembe I village. 22/11/2022; Hygiene campaign held at Ruhango sector, Munini cell Kabaja village. 22/11/2022; Hygiene campaign held at Kinazi sector, Kinazi cell Kacyiru II village. |
| Measurement methods and procedures | Questionnaires are used in hygiene campaigns to assess hygienic handling of clean water. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Assessment of achievement of SDG 6 |
| Additional comment | Any major changes in the health status of the water users as a result of contaminated water (e.g. an outbreak of water related disease) must be reported. The above hygiene campaigns are for the first year of the crediting period （from 26/11/2021 to 25/11/2022）. The hygiene campaigns for the second year of the crediting period (from 26/11/2022 to 25/11/2023) were held from 18/10/2023 to 31/10/2023. Since they do not fall in the current monitoring period, they will be reported in the MR of the next monitoring period. |

D.3. Comparison of monitored parameters with last monitoring period

| Data/Parameter | Value obtained in this monitoring period | Value obtained last monitoring period |
| --- | --- | --- |
| Qp,y | 6.66 | 6.66 |
| Qp,rawboil,y | 0 | 0 |
| Qp,cleanboil,y | 0 | 0 |
| Quality of the treated water | Pass | Pass |
| Up,y | 100% | 100% |
| Np,y | VPA01: 12,340,897 | 6,238,172 |
|  | VPA02: 11,564,471 | 7,106,802 |
|  | VPA03: 11,797,656 | 7,177,865 |
|  | VPA04: 12,190,541 | 4,695,103 |
|  | VPA05: 12,307,382 | 6,851,892 |
|  | VPA06: 11,834,797 | 4,669,012 |
|  | Total: 72,035,744 | 36,738,846 |
| LEp,y | 0 | 0 |
| Ip,y | 0 | 0 |
| Tp,y | 0.44 | 0.44 |
| Pp,y | VPA01: 33,750 | 33,750 |
|  | VPA02: 31,855 | 31,855 |
|  | VPA03: 32,320 | 32,320 |
|  | VPA04: 33,221 | 33,221 |
|  | VPA05: 33,514 | 33,514 |
|  | VPA06: 32,314 | 32,314 |
|  | Total: 196,974 | 196,974 |

The values of Np,y have increased significantly due to the number of operating days of boreholes has increased during this monitoring period. In other words, the rehabilitation of boreholes in each VPA took several months so that the average operation period of boreholes is much shorter than one year in last monitoring period. So the values of Np,y obtained in last monitoring period are smaller.

D.4. Implementation of sampling plan

>>

a) Description of implemented sampling design

(1) Objectives and reliability requirements

The objective is to obtain an unbiased and reliable estimate of the proportion or mean value of the following parameters over the course of the monitoring period. As per “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”, the 95/10 rule should be applied for cross VPA sampling.

(2) Target population

The target populations are boreholes maintained by the VPA01-06 and households consuming safe water provided by the VPA01-06. The target population of boreholes is 251, the target population of households is 49,057.

(3) Sampling method

Since VPA01-06 of PoA GS 10959 are in the same province, using same technologies and sharing same baseline scenario, representative and random cross VPA sampling is applied in these VPAs.

Multi-stage sampling combines the cluster and simple random sampling approaches in a two-stage sampling scheme which enables us to randomly select some boreholes and then randomly sample some households who using those boreholes. As with simple random sampling and cluster sampling, we are assuming homogeneity across boreholes in the usage of boreholes. We know that the 49,057 households are using 251 boreholes. Qp,y, Qp,rawboil,y, Qp,cleanboil,y, Up,y, Ip,y and Tp,y are sampled by multiple stage sampling, and water quality is sampled by simple random method.

(4) Sampling size

As per the applied methodology, the following guidelines are applied to calculate the sample size:

Group size < 300 (for water quality): Minimum sample size 30 or population size, whichever is smaller;

Group size > 1,000(for Qp,y, Qp,rawboil,y, Qp,cleanboil,y, Up,y, Ip,y and Tp,y): Minimum sample size 100.

As per “Standard:  Sampling and surveys for CDM project activities and programmes of activities (Version 09 .0)” and “Guidelines for sampling and surveys for CDM project activities and programmes of activities (Version 04.0)”, the 95/10 rule are applied for cross VPA sampling.

The results are summarized in Excel spreadsheets “Data Recording Form for Project Survey” and “Data Recording Form for WCFT”, which have been submitted.

(5) Sampling frame

The frame is a list of all households supplied by the boreholes in the project. The samples are selected from this list. The samples are randomly selected across the population. The entire target population has an equal chance of being selected, thus the samples selected are deemed to be representative of population.

b) Collected data

The following parameters are determined by sampling in this monitoring period:

| Parameter | Description | Frequency |
| --- | --- | --- |
| Qp,y | Quantity of safe water in litres consumed in the project scenario p and supplied by project technology per person per day | At least biennially |
| Qp,rawboil,y | Quantity of raw or unsafe water that is still boiled after installation of the water treatment technology | At least biennially |
| Qp,cleanboil,y | Quantity of safe (treated, or from safe supply) water boiled in the project scenario p, after installation of project technology | At least biennially |
| Up,y | Usage rate in project scenario p during year y | Annually |
| Ip,y | Waterborne illness incidence in the project scenario during year y | At least biennially |
| Tp,y | Time spent to fetch and purify water by women and girls per household per day in the project scenario during year y | At least biennially |
| Water Quality | Performance of the treatment technology – less than 1 Colony Forming Unit (CFU) of E.Coli /100 ml of safe water | Quarterly |

The main survey methods applied in the sampling plan include hardcopy questionnaires, face to face interview and telephone interview. The potential of refusals and other means of non-responses have been taken into account for calculation of sample size. Meanwhile, in order to minimize the rates of non-response and answer bias, the questionnaires have been designed by professional team and widely tested before use. The CME and ARDE are in charge of the implementation of the sampling plan. The Usage Survey was carried out between 12/08/2023 to 06/09/2023. The data collected from the surveys was compiled into the “Data Recording Form for Usage Survey” Excel spreadsheet. The Project Survey was carried out between 25/07/2022 to 25/08/2022. The data collected from the surveys was compiled into the “Data Recording Form for Project Survey” Excel spreadsheet. The WCFT was carried out between 26/07/2022 to 18/08/2022. The data collected from the surveys was compiled into the “Data Recording Form for WCFT” Excel spreadsheet. The three documents have been submitted.

c) Analysis

Data obtained from the samples were used to estimate values for the parameters described above. The Column L of “4- Data Recording Form for Usage Survey” Excel spreadsheet shows 100% of sample households used project boreholes; the Column AB of “2- Data Recording Form for Project Survey” Excel spreadsheet shows women and girls spend 0.44 hour to fetch water every day.

The values have been used to calculate the SDG outcomes:

| Parameter | Meaning | Value | Unit |
| --- | --- | --- | --- |
| Up,y | Cumulative usage rate for technologies in project scenario p during year y, based on cumulative installation rate and drop off rate | 100% | percentage |
| Ip,y | Waterborne illness incidence in the project scenario during year y | 0 | Percentage |
| Tp,y | Time spent to fetch and purify water by women and girls per household per day in the project scenario during year y | 0.44 | h |
| Qp,y | Quantity of safe water in litres consumed in the project scenario p and supplied by project technology per person per day | 6.66 | L |
| Qp,rawboil,y | Quantity of raw or unsafe water that is still boiled after installation of the water treatment technology | 0 | L |
| Qp,cleanboil,y | Quantity of safe (treated, or from safe supply) water boiled in the project scenario p, after installation of project technology | 0 | L |
| Water Quality | Performance of the treatment technology – less than 1 Colony Forming Unit (CFU) of E.Coli /100 ml of safe water | Meet the requirement |  |

Please refer to the “1-SDG Outcomes Calculation Form-MP2nd” Excel spreadsheet.

d) Demonstration that the required confidence/precision level has been met

As calculated in above (4) of (a), the minimum number of boreholes to be sampled for Project Survey and WCFT is five. Since we have sampled six boreholes, the required 95/10 confidence level has been met. The minimum number of boreholes to be sampled for Usage Survey is one. Since we have sampled eight boreholes, the required 95/10 confidence level has been met.

e) Demonstration that the samples were randomly selected and are representative of the population

The samples were randomly selected using random number generating function of Excel. The entire target population has an equal chance of being selected. Therefore, the samples selected are representative of the population.

SECTION E. CALCULATION OF SDG IMPACTS

E.1. Calculation of baseline value or estimation of baseline situation of each SDG Impact

>>

SDG 13: Climate Action

Baseline Scenario Fuel Consumption Calculation:

Bb,y = (1 – Xboil) * (1 - Cj) * Np,y * Wb,y * (Qp,y + Qp,rawboil,y)                             (1)

Where:

Bb,y	Quantity of fuel consumed in baseline scenario b during the year in tons

Xboil	Percentage of premises that in the absence of the project activity would have used non-GHG emitting technologies like chlorine treatment techniques (if available) in the project boundary; the applied value is 0 as per section D.1.

Cj	Percentage of users of project technology who were already in baseline scenario using a non-boiling safe water supply; the applied value is 0.0083 as per section D.1.

Np,y	Number of person.days consuming water supplied by project scenario p through year y; the applied value as per Section D.2.

Wb,y	Quantity of fuel  in tons required to treat 1 litre of water using technologies representative of baseline scenario b in year y as per baseline water boiling Test; the applied value is 0.0004 as per Section D.1.

Qp,y     Quantity of safe water in litres consumed in the project scenario p and supplied by project technology per person per day; the applied value is 6.66 as per Section D.1.

Qp, rawboil,y  Quantity of raw water boiled in the project scenario p per person per day; ; the applied value is 0 as per Section D.1.

As a result, Bb,y is shown as follows:

| VPA No. | Bb,y |
| --- | --- |
| 01 | 32,603.28 |
| 02 | 30,552.05 |
| 03 | 31,168.10 |
| 04 | 32,206.05 |
| 05 | 32,514.73 |
| 06 | 31,266.22 |
| Total | 190,310.43 |

Baseline Emission Reductions:

BEb,y = Bb,y *(( fNRB,b,y * EFb,fuel,CO2) + EFb,fuel,non-CO2) * NCVb,fuel                               (2)

Where:

BEb,y		Baseline emissions during year y

Bb,y		Quantity of fuel consumed in baseline scenario b during the year in tons; the applied value as per calculation in this section above;

fNRB,b,y	Fraction of biomass used that can be established as non-renewable biomass in baseline scenario b during year y; the applied value is 0.9682 as per section D.1.

EFb,fuel,CO2	CO2 emission factor of fuels used in the baseline scenario; the applied value is 112 as per section D.1.

EFb,fuel,non-CO2	Non-CO2 emission factor of fuels used in the baseline scenario; the applied value is 9.46 as per section D.1.

NCVb,fuel	Net calorific value of fuels used in the baseline scenario; the applied value is 0.0156 as per section D.1.

As a result, BEb,y is shown as follows:

| VPA No. | BEb,y |
| --- | --- |
| 01 | 59,964 |
| 02 | 56,191 |
| 03 | 57,324 |
| 04 | 59,233 |
| 05 | 59,801 |
| 06 | 57,505 |
| Total | 350,018 |

SDG 3: Good health and well-being

The outcome of SDG 3 is quantified as the reduction of waterborne illness incidence compared to baseline scenario. The waterborne illness incidence in the baseline scenario is 38.45% according to Baseline Survey.

SDG 5: Gender equality

The outcome of SDG 5 is quantified as percentage reduction of time spent to fetch and purify water by women and girls. The time spent to fetch and purify water by women and girls per household per day in the baseline scenario is 0.90 hour according to Baseline Survey.

SDG 6: Clean Water and Sanitation

The outcome of SDG 6 is quantified as number of persons consuming safe water supplied by the project, the direct outcome is calculated, see section E.2.

E.2. Calculation of project value or estimation of project situation of each SDG Impact

>>

SDG 13: Climate Action

Project Scenario Fuel Consumption Calculation

Bp,y = (1 - Cj) * Np,y * Wp,y * (Qp,rawboil,y + Qp,cleanboil,y)                                        (3)

Where:

Bp,y		Quantity of fuel f consumed in project scenario p during the year y in tons

Cj	Percentage of users of project technology who were already in baseline scenario using a non-boiling safe water supply; the applied value is 0 as per section D.1.

Np,y             Number of person.days consuming water supplied by project scenario p through year y; the applied value as per Section D.2.

Wp,y	Quantity of fuel  in tons required to treat 1 litre of water using technologies representative of baseline scenario b in year y as per baseline water boiling Test; the applied value is 0.0004 as per Section D.1.

Qp,rawboil,y	 Quantity of raw water boiled in the project scenario p per person per day; the applied value is 0 as per Section D.2.

Qp,cleanboil,y	Quantity of safe water boiled in the project scenario p per person per day in year y; the applied value is 0 as per Section D.2.

As a result, Bp,y = 0 t

Project Emission Reductions:

PEp,y = Bp,y * ((fNRB,p,y * EFp,fuel,CO2) + EFp,fuel,non-CO2) * NCVp,fuel                                                    (4)

Where:

BEb,y		Baseline emissions during year y

PEp,y		Project emissions during year y

Bp,y		Quantity of fuel consumed in project scenario p during the year y in tons; the applied value  is 0 as per calculation in this section above;

fNRB,p,y	Fraction of biomass used that can be established as non-renewable biomass in project scenario p during year y; the applied value is 0.9682 as per section D.1.

EFb,fuel,non-CO2	Non-CO2 emission factor of fuels used in the baseline scenario; the applied value is 9.46 as per section D.1.

EFp,fuel,CO2	CO2 emission factor of fuels used in the project scenario; the applied value is 112 as per section D.1.

EFp,fuel,non-CO2	Non-CO2 emission factor of fuels used in the project scenario; the applied value is 9.46 as per section D.1.

NCVp,fuel	Net calorific value of fuels used in the project scenario; the applied value is 0.0156 as per section D.1.

ERy	Overall emission reductions achieved by the project activity during year y

Up,y	Cumulative usage rate for technologies in project scenario p during year y, based on cumulative installation rate and drop off rate; the applied value is 100% as per section D.2.

LEp,y	Leakage from project scenario p during year y; the applied value 0 as per section D.2.

As a result, PEp,y= 0 tCO2e

SDG 3: Good health and well-being

The outcome of SDG 3 is quantified as the reduction of waterborne illness incidence compared to baseline scenario, which is calculated as follows:

Ir,y = Ib – Ip,y                                                                                                (5)

Where:

Ir,y          Reduction of waterborne illness incidence in year y

Ib           Waterborne illness incidence in the baseline scenario; the applied value is 38.45% as per section D.1.

Ip,y         Waterborne illness incidence in the project scenario during year y; the applied value is 0 as per section D.2.

As a result, Ir,y = 38.45%

SDG 5: Gender equality

The outcome of SDG 5 is quantified as percentage reduction of time spent to fetch and purify water by women and girls, which is calculated as follows:

Tr,y = (Tb - Tp,y)/Tb                                                                                        (6)

Where:

Tr,y      Percentage reduction of time spent to fetch and purify water by women and girls in year y

Tb        Time spent to fetch and purify water by women and girls per household per day in the baseline scenario; the applied value is 0.90 as per section D.1.

Tp,y     Time spent to fetch and purify water by women and girls per household per day in the project scenario during year y; the applied value is 0.44 as per section D.2.

As a result, Tr,y = 51.11%

SDG 6: Clean Water and Sanitation

The outcome of SDG 6 is quantified as number of persons consuming safe water supplied by the Project, which is calculated as follows:

Py = Pp,y * (1-Cj) *Up,y                                                                                        (7)

Where

Py          Number of additional persons consuming safe water supplied by the Project compared to the baseline scenario during year y

Pp,y        Number of persons consuming water within the project area during year y; the applied value as per section D.2.

Cj	Percentage of users of project technology who were already in baseline scenario using a non-boiling safe water supply; the applied value is 0.0083 as per section D.1.

Up,y        Cumulative usage rate for technologies in project scenario p during year y; the applied value is 100% as per section D.2.

As a result, Py is shown as follows:

| VPA No. | Py |
| --- | --- |
| 01 | 33,750 |
| 02 | 31,855 |
| 03 | 32,320 |
| 04 | 33,221 |
| 05 | 33,514 |
| 06 | 32,314 |
| Total | 196,974 |

E.3. Calculation of leakage

>>

As per the applied methodology and the registered VPA DD, the leakage is expected to be 0, which can be ignored.

However, to confirm that leakage can be deemed very low, the below described leakage assessment regarding the potential leakage sources (listed in the methodology TPDDTEC, pages 15) shall be completed every two years. During this monitoring period, LEp,y is estimated as follows:

| Potential Influence Factor | Interpretation |
| --- | --- |
| The displaced baseline technologies are reused outside the project boundary in place of lower emitting technology or in a manner suggesting more usage than would have occurred in the absence of the project. | The displaced baseline technology is three stones. It has not been reused outside the project boundary because it still be used for cooking after the implementation of the project. |
| Non-project users who previously used lower emitting energy sources use the non-renewable biomass or fossil fuels saved under the project activity. | The costs of low emitting water purification technologies, such as filtration and chlorination, are much higher than boiling with wood fuel. Users of these technologies are not price sensitive. Therefore, the implementation of the VPA does not lead these users to boil water with wood fuel, even if the price of wood fuel becomes cheaper because of the reduction of demand caused by the project. |
| The project significantly impacts the NRB fraction within an area where other CDM or VER project activities account for NRB fraction in their baseline scenario. | Considering that the VPA only saves 190,310 tons (Bb,y) of biomass annually while the total amount of above-ground biomass of Rwanda is 75 million tons, the VPA does not affect NRB fraction. |
| The project population compensates for loss of the space heating effect of inefficient technology by adopting some other form of heating or by retaining some use of inefficient technology. | The space heating effect of boiling water is negligible. Therefore, it is highly unlikely that some other form of heating will be adopted for compensating the space heating effect of boiling water. |
| By virtue of promotion and marketing of new technology with high efficiency, the project stimulates substitution within households who commonly used a technology with relatively lower emissions, in cases where such a trend is not eligible as an evolving baseline. | The project does not promote any new technology with high efficiency. It does not stimulate people to boil water. |

In conclusion, LEp,y = 0

ERy = (ΣBEb,y - ΣPEp,y) * Up,y - ΣLEp,y                                                                    (8)

As a result,

| VPA No. | Start date | End date | ERy |
| --- | --- | --- | --- |
| VPA01 | 25/07/2022 | 24/07/2023 | 59,964 |
| VPA02 | 25/07/2022 | 24/07/2023 | 56,191 |
| VPA03 | 25/07/2022 | 24/07/2023 | 57,324 |
| VPA04 | 25/07/2022 | 24/07/2023 | 59,233 |
| VPA05 | 25/07/2022 | 24/07/2023 | 59,801 |
| VPA06 | 25/07/2022 | 24/07/2023 | 57,505 |
|  | Total |  | 350,018 |

E.4. Calculation of net benefits or direct calculation for each SDG Impact

| SDG | SDG Impact | GS ID | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- | --- |
| 13 | Reduce emission from water boiling by non-renewable biomass | GS 11098 | 59,964 | 0 | 59,964 |
|  |  | GS 11133 | 56,191 | 0 | 56,191 |
|  |  | GS 11134 | 57,324 | 0 | 57,324 |
|  |  | GS 11135 | 59,233 | 0 | 59,233 |
|  |  | GS 11136 | 59,801 | 0 | 59,801 |
|  |  | GS 11137 | 57,505 | 0 | 57,505 |
|  |  | Total | 350,018 | 0 | 350,018 |
| 3 | Reduce the incidence of waterborne illness within the project area |  | 38.45% | 0 | 38.45% |
| 5 | Reduce the time spent to fetch and purify water by women and girls |  | 0.90hour | 0.44hour | 51.11% |
| 6 | Provide safe water to local residents | GS 11098 | 283 | 34,033 | 33,750 |
|  |  | GS 11133 | 267 | 32,122 | 31,855 |
|  |  | GS 11134 | 271 | 32,591 | 32,320 |
|  |  | GS 11135 | 279 | 33,500 | 33,221 |
|  |  | GS 11136 | 281 | 33,795 | 33,514 |
|  |  | GS 11137 | 271 | 32,585 | 32,314 |
|  |  | Total | 1,652 | 198,626 | 196,974 |

E.5. Comparison of actual SDG Impacts with estimates in approved PDD

| SDG | GS ID | Values estimated in ex ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- | --- |
| 13 | GS 11098 | 58,480 tCO2e | 59,964 tCO2e |
|  | GS 11133 | 58,480 tCO2e | 56,191 tCO2e |
|  | GS 11134 | 58,480 tCO2e | 57,324 tCO2e |
|  | GS 11135 | 58,480 tCO2e | 59,233 tCO2e |
|  | GS 11136 | 58,480 tCO2e | 59,801 tCO2e |
|  | GS 11137 | 58,480 tCO2e | 57,505 tCO2e |
| 3 | GS 11098 | 18.45% | 38.45% |
|  | GS 11133 | 18.45% | 38.45% |
|  | GS 11134 | 18.45% | 38.45% |
|  | GS 11135 | 18.45% | 38.45% |
|  | GS 11136 | 18.45% | 38.45% |
|  | GS 11137 | 18.45% | 38.45% |
| 5 | GS 11098 | 44.44% | 51.11% |
|  | GS 11133 | 44.44% | 51.11% |
|  | GS 11134 | 44.44% | 51.11% |
|  | GS 11135 | 44.44% | 51.11% |
|  | GS 11136 | 44.44% | 51.11% |
|  | GS 11137 | 44.44% | 51.11% |
| 6 | GS 11098 | 32,726 persons | 33,750 persons |
|  | GS 11133 | 32,726 persons | 31,855 persons |
|  | GS 11134 | 32,726 persons | 32,320 persons |
|  | GS 11135 | 32,726 persons | 33,221 persons |
|  | GS 11136 | 32,726 persons | 33,514 persons |
|  | GS 11137 | 32,726 persons | 32,314 persons |

E.5.1. Explanation of calculation of value estimated ex ante calculation of approved PDD for this monitoring period

>>

According to approved PDD, annual emission reduction achieved by the project activity is 58,480 tCO2e. This monitoring period is from 25/07/2022 to 24/07/2023.

So, ex-ante emission reduction of each VPA for this monitoring period:

| GS ID | Start date | End date | Values estimated in ex ante calculation of approved PDD |
| --- | --- | --- | --- |
| GS 11098 | 25/07/2022 | 24/07/2023 | 58,480 |
| GS 11133 | 25/07/2022 | 24/07/2023 | 58,480 |
| GS 11134 | 25/07/2022 | 24/07/2023 | 58,480 |
| GS 11135 | 25/07/2022 | 24/07/2023 | 58,480 |
| GS 11136 | 25/07/2022 | 24/07/2023 | 58,480 |
| GS 11137 | 25/07/2022 | 24/07/2023 | 58,480 |
| Total |  |  | 350,880 |

E.6. Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

>>

The actual value of SDG 3 is higher than estimated value due to several reasons:

a) clean water supplied by the project has a good quality, which has been proven by the water quality tests;

b) local people have developed a better healthy habit by the convenience of getting clean water and hygiene campaigns.

The actual value of SDG 5 is higher than estimated value because estimated value ex-ante was a conservative estimate, whereas monitored data is an accurate representation by sampling.

The actual values of SDG 6 are a little bit higher than the ex-ante estimated values in VPA 01, 04 and 05 because the ex-antes values are from a general conservative estimation, whereas monitored data is from the sum of the actual data from each project borehole.

The actual value of SDG 13 is (350,880-350,018)/350,880=0.25% lower than estimated value, that is a reasonable margin of error.

When CME estimated the emission reduction ex-ante, the number of consuming days of the boreholes (average) was assumed 347. Due to the well and continuous maintenance by CME, the actual average consuming days of the boreholes was 363 during the second monitoring period. Therefore, some of the VPAs has higher values than the estimated. But still not exceed the 60,000 tCO2e limit. If the consuming dates were 363 instead of 347, other conditions are the same as the ones used for the ex-ante estimation in the registered VPA-DD, the ex-ante estimation of each VPA would be 61,177 tCO2e. Hence, the actual values in this monitoring period are conservative.

| VPA | In approved PDD (with 347 working days) | Actual value in this monitoring period | In PDD (with 363 working days) | Limit of each VPA |
| --- | --- | --- | --- | --- |
| 01 | 58,480 | 59,964 | 61,177 | 60,000 |
| 02 | 58,480 | 56,191 | 61,177 | 60,000 |
| 03 | 58,480 | 57,324 | 61,177 | 60,000 |
| 04 | 58,480 | 59,233 | 61,177 | 60,000 |
| 05 | 58,480 | 59,801 | 61,177 | 60,000 |
| 06 | 58,480 | 57,505 | 61,177 | 60,000 |

SECTION F. SAFEGUARDS REPORTING

>>

According to Section B.7 of Design Certified PDD, safety of water disinfectants used in the VPA should be monitored. But during this monitoring period, the water quality meets the relevant requirements, so we have not used water disinfectants. Hence no parameter needs to be monitored in this section.

SECTION G. STAKEHOLDER INPUTS AND LEGAL DISPUTES

G.1. List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

>>

We offer different ways of continuous input and grievance mechanism in accordance with PDD.

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | The offices of all the villages where the VPA has been implemented |
| GS Contact (mandatory) | help@goldstandard.org |
| Email | Mr. Ji BAO: baoji@icebergchina.com Mr. David BAZIRANKENDE: bazirankendebdavid@gmail.com |
| Cell Phone | Mr. David BAZIRANKENDE: +250728684787 |

During this monitoring period, no Inputs or Grievances has been received during this monitoring period.

G.2. Report on any stakeholder mitigations that were agreed to be monitored.

>>

Not applicable. No stakeholder mitigations were agreed to be monitored.

G.3. Provide details of any legal contest that has arisen with the project during the monitoring period

>>

No legal contest or dispute has arisen.

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

https://view.officeapps.live.com/op/view.aspx?src=https%3A%2F%2Fcdm.unfccc.int%2Ffilestorage%2Fe%2Fx%2Ft%2Fextfile-20150813144045637-Meth_guid48Calculator.xlsx%2FMeth_guid48Calculator.xlsx%3Ft%3DVHR8cm5rMmoxfDBG9EVaDsQHTLcuUyHkmRLI&wdOrigin=BROWSELINK

Please refer to submitted document “14-Rural Drinking Water Quality Management Framework 2019”, Page 28, Appendix 2: List of Laboratories.

Table 18, Global Forest Resources Assessment 2015

Whenever emission reductions are capped, both the original and capped values used for calculations must be transparently reported.  Use brackets to denote original values.