Monitoring Report

PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT – TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

SECTION A - Description of project

SECTION B - Implementation of project

SECTION C - Description of monitoring system applied by the project

SECTION D - Data and parameters

SECTION E - Calculation of SDG Impacts

SECTION F - Safeguards Reporting

SECTION G - Stakeholder inputs and legal disputes

KEY PROJECT INFORMATION

Key Project Information

| GS ID (s) of Project (s) | GS11335 |
| --- | --- |
| Title of the project (s) covered by monitoring report | Shuangbaotai Animal Manure Management System GHG Mitigation Project in Guangdong Province |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | 05 |
| Version number of the monitoring report | 03 |
| Completion date of the monitoring report | 15/08/2023 |
| Date of project design certification | 05/10/2022 |
| Date of Last Annual Report | 01/07/2022 |
| Monitoring period number | 02 |
| Duration of this monitoring period | The 2nd monitoring period from 01/04/2022 to 31/03/2023 (first and last days included) |
| Project Representative | The official focal point and project developer: Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. The project owner: Shuangbaotai Animal Husbandry Group Co., Ltd. |
| Host Country | People’s Republic of China |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | ACM0010 GHG emission reductions from manure management systems (Version 08.0) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | Amount Achieved | Units/ Products |
| --- | --- | --- | --- |
| SDG 13: Climate Action | Amount of GHGs emission avoided or sequestered | 201,468 | tCO2e |
| SDG 7: Affordable and Clean Energy | Annual amount of biogas utilized | 19,825,974.68 | m3 |
| SDG 8: Decent Work and Economic Growth | Total number of jobs | 18 full-time jobs created including 9 females and 9 males | Number |

Table 2 – Product Vintages

| SDG 7 Affordable and Clean Energy |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Annual amount of biogas utilized (m3) |
| 01/04/2022 | 31/12/2022 | 14,841,063.44 |
| 01/01/2023 | 31/03/2023 | 4,984,911.24 |
| 01/04/2022 | 31/03/2023 | 19,825,974.68 |

| SDG 8 Decent Work and Economic Growth |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Total number of jobs |
| 01/04/2022 | 31/12/2022 | 18 full-time jobs created including 9 males and 9 females |
| 01/01/2023 | 31/03/2023 | 18 full-time jobs created including 9 males and 9 females |
| 01/04/2022 | 31/03/2023 | 18 full-time jobs created including 9 males and 9 females |

| SDG 13 Climate Action |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Amount of GHGs emission avoided or sequestered (tCO2e) |
| 01/04/2022 | 31/12/2022 | 152,045 |
| 01/01/2023 | 31/03/2023 | 49,423 |
| 01/04/2022 | 31/03/2023 | 201,468 |

- DESCRIPTION OF PROJECT

General description of project

>>

The project installs 9 sets of Animal Waste Management Systems (AWMSs) in 9 existing swine farms in Guangdong Province. The purpose of the project activity is to treat the manure and wastewater from the 9 swine farms to avoid methane emissions.

The scenario prior to the implementation of the proposed project activity is that the manure and wastewater from the 9 swine farms were treated in uncovered anaerobic lagoons.

The project activity replaces the current open anaerobic lagoons with new AWMSs, and Each subsidiary swine farm installs one AWMS. All the manure and wastewater are collected and then be separated first. The separated solid are treated in aerobic composting system and the organic fertilizers are produced. part of the fertilizer is supplied to the farmers living around for free and the others is be sold to the market. The separated liquid is treated through anaerobic digestion process and the biogas generated during the treatment process is captured for electricity generation and surplus biogas is destroyed through the flaring system (if any). The electricity generated are all used by the operation of AWMSs and the swine farms. It is estimated that total 22,863,337.2 m3 biogas are expected to be generated annually and 20,577,003.48 m3 biogas will be sent to biogas generator for power generation after desulfurization purification system. The other biogas will be flared through the flaring system. Each generator is installed in each subsidiary swine farm, total installed capacity of the proposed project is 5.42MW and the annual electricity production is estimated to be 35,593.87MWh. After anaerobic digestion. The sludge produced from anaerobic digestion are treated through aerobic composting together with the separated solid, the effluent is supplied to the farmers living around free for agriculture irrigation.

The scenario prior to the implementation of the proposed project activity is that the manure and wastewater from the 9 swine farms were treated in uncovered anaerobic lagoons.

The project activity reduces GHG in the atmosphere through avoiding methane emissions from anaerobic treatment of swine manure and wastewater. The annual average CO2 emission reductions are estimated to be 238,913 tCO2e as per PDD.

The project has been put into operation in 20/12/2020. This monitoring report is for the 2nd monitoring period, which is from 01/04/2022 to 31/03/2023. Total emission reductions achieved in this monitoring period are 201,468 tCO2e.

Location of project

>>

The proposed project activity is located in Guangdong Province, China. There are nine  subsidiary farms involved in the project. All of the nine swine farm including market swine and breeding swine. The specific location of the nine subsidiary farms is shown below:

Table1. the specific location of the nine swine farms

| Swine Farm | Location | East longitude | North latitude |
| --- | --- | --- | --- |
| Huicheng Hengli swine Farm（惠城横沥） | Zenghu Village, Hengli Town, Huicheng District, Huizhou City | 114.553329202° | 23.300885296° |
| Dongyuan Huangtian swine Farm（东源黄田） | Niulanwu Village, Huangtian Town, Dongyuan County, Heyuan City | 114.986740472° | 23.849068143° |
| Enping Shahu swine Farm（恩平沙湖） | Juhou Village, Shahu Town, Enping City, Jiangmen city | 112.476473166° | 22.407609654° |
| Langtian Yuehui swine Farm（廊田粤辉） | Laoping farm, Langtian Town, Lechang City, Shaoguan City | 113.426920557° | 25.150475864° |
| Qujiang Fengwan swine Farm（曲江枫湾） | Dazhushan Shibei Village, Fengwan Town, Qujiang District, Shaoguan City | 113.765488° | 24.769464° |
| Pingyuan Zhongshi swine Farm（平远仲石） | Zhongsi Village, Zhonghang town, Pingyuan County, Meizhou City | 115.817507394° | 24.638366511° |
| Yangchun Tanshui swine Farm（阳春潭水） | Jun'an Village, Yaodong Village Committee, Tanshui Town, Yangchun City, | 111.579543° | 22.070908° |
| Xinfeng Shatian swine Farm（新丰沙田） | Changyin Village, Shatian town, Xinfeng county, Shaoguan City | 113.958179° | 24.033000° |
| Kaiping Cangcheng swine Farm（开平苍城） | Lianxing Village, Cangcheng Town, Kaiping City, Jiangmen City | 112.572581971° | 22.552391456° |

Guangdong Province

Figure 1 The Geographic Location of the project

Reference of applied methodology

>>

The approved methodology applied by this project is referenced as ACM0010” GHG emission reductions from manure management systems (Version 08.0)”.

Tool applied:

Tool 02: “Combined tool to identify the baseline scenario and demonstrate additionality(version07.0)”

Tool 08: “Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version03.0)”

Tool 14: “Project and leakage emissions from anaerobic digesters (Version 02.0)”

Tool 06:” Project emissions from flaring” (Version 4.0)

Tool 05:” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” (Version 3.0)

Tool 24:” Common practice (Version 03.1)”

All above methodology and tool can be available from:

http://cdm.unfccc.int/methodologies/PAmethodologies/approved

https://cdm.unfccc.int/Reference/tools/index.html

Crediting period of project

>>

Start date of crediting period: 20/12/2020.

End date of crediting period:19/12/2025.

Total length of crediting period: 20/12/2020 to 19/12/2025 (first and last days included) for the first crediting period (two renewal cycles and in total 15 years). The 2nd monitoring period is from 01/04/2022 to 31/03/2023 (first and last days included), which belong to the first crediting period.

Expected lifetime of the project: 15 years.

- IMPLEMENTATION OF PROJECT

Description of implemented project

>>

9 sets of AWMSs in 9 swine farms have been implemented. All the manure and wastewater are collected and then be separated first. The separated solid is treated in aerobic composting system, which is used as fertilizer. And part of the fertilizer products is distributed to the surrounding farmers free of charge to improve the life condition of farmers, the others is sold to the market. The separated liquid is treated through anaerobic digestion, the biogas generated during the treatment process is captured for power generation. if there is surplus biogas, then the biogas can be flared through the flaring system. The sludge produced from anaerobic digestion is treated through aerobic composting together with the separated solid and the effluent is treated aerobically and then used for agriculture irrigation.

Anaerobic tanks adopt Upflow Anaerobic Sludge Bed Reactor (UASB) as its anaerobic digester technology, and the biogas generated from the anaerobic digesters enter the biogas generator set after biological desulfurization and dehydration purification. The generated electricity is used for daily operation of AWMSs and the swine farms, and the waste heat of the generator is recycled for increasing the temperature of the anaerobic reactor. At the same time, and the residual biogas is flared if there is any surplus biogas.

The technical flow of the project activity is shown below:

Figure 2 Process flow of biogas project in the swine farms

The project was implemented in 9 existing swine farms and the technology implemented at each swine farm was same. While the treatment capacities of each AWMS and the size of each swine farm and corresponding design of the AWMS was different. The treatment capacity and equipment of the system are designed according to the farm size. The construction date, commissioning date and start date of operation of AWMS in each farm are summarized in Table2. The other mainly milestone for implementation of the project are summarized in Table3. The volume for original uncovered anaerobic lagoons and the equipment parameters used of each swine farm are shown in the following Table 4:

Table 2 The construction, commissioning and start operation date of AWMS in each swine farm involved in this project

| Swine farm name | Construction date | Commissioning date | Start Operation date |
| --- | --- | --- | --- |
| Huicheng Hengli swine Farm | 10/09/2020 | 14/12/2020 | 20/12/2020 |
| Dongyuan Huangtian swine Farm | 12/09/2020 | 15/12/2020 | 20/12/2020 |
| Enping Shahu swine Farm | 10/09/2020 | 13/12/2020 | 20/12/2020 |
| Langtian Yuehui swine Farm | 11/09/2020 | 16/12/2020 | 20/12/2020 |
| Qujiang Fengwan swine Farm | 10/09/2020 | 13/12/2020 | 20/12/2020 |
| Pingyuan Zhongshi swine Farm | 12/09/2020 | 15/12/2020 | 20/12/2020 |
| Yangchun Tanshui swine Farm | 13/09/2020 | 14/12/2020 | 20/12/2020 |
| Xinfeng Shatian swine Farm | 12/09/2020 | 13/12/2020 | 20/12/2020 |
| Kaiping Cangcheng swine Farm | 13/09/2020 | 15/12/2020 | 20/12/2020 |

Table 3 Milestone for implementation of the project

| Time | Main actions |
| --- | --- |
| 01/05/2020 | EIA |
| 16/05/2020 | EIA Approval by local Ecology and Environment Department |
| 01/06/2020 | Completion of Project evaluation report |
| 05/09/2020 | Start date of the project, the date on signing biogas generator purchase contract |
| 16/11/2020 | Training for the GS monitoring plan and biogas safety operation |
| 20/12/2020 | The project was put into operation |
| 03/09/2021 | Submission for preliminary review |
| 14/10/2021 | Local stakeholder consulting physical meeting |
| 12/11/2021 | Training for the GS monitoring plan and biogas safety operation |
| 05/10/2022 | Date of Project design certification |
| 07/11/2022 | Training for the GS monitoring plan and biogas safety operation in 2022 |

There are no deviations or delays compared to information in approved PDD.

During this monitoring period, there was no special event occurred, such as overhaul and downtimes of anaerobic digesters. All the equipment operated well, and no equipment was under maintenance during this monitoring period.

Table 4 technical parameters of main equipment and uncovered anaerobic lagoons

| Farm name |  | Huicheng Hengli | Dongyuan Huangtian | Enping Shahu | Langtian Yuehui | Qujiang Fengwan | Pingyuan Zhongshi | Yangchun Tanshui | Xinfeng Shatian | Kaiping Cangcheng |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Treatment capacities of AMMS(t/d) |  | 50 | 50 | 200 | 200 | 150 | 250 | 200 | 50 | 100 |
| Baseline anaerobic lagoon parameters |  |  |  |  |  |  |  |  |  |  |
| Size of baseline anaerobic lagoon(length*width*depth) (m3) |  | 40*40*3 | 40*40*3 | 100*50*4 | 100*50*4 | 80*50*4 | 80*80*4 | 100*50*4 | 40*40*3 | 50*80*3 |
| technical life |  | No Less than 15 years |  |  |  |  |  |  |  |  |
| Generator equipment parameters |  |  |  |  |  |  |  |  |  |  |
| Type |  | C120D5E | C180D5E | C1000D5E | C1000D5E | C500D5E | C1000D5E | C1000D5E | C120D5E | C500D5E |
| Rated power (kw) |  | 120 | 180 | 1000 | 1000 | 500 | 1000 | 1000 | 120 | 500 |
| Rated current(A) |  | 450 | 450 | 450 | 450 | 450 | 450 | 450 | 450 | 450 |
| Rated speed (rpm) |  | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 |
| Equipment Quantity |  | C120D5E | C180D5E | C1000D5E | C1000D5E | C500D5E | C1000D5E | C1000D5E | C120D5E | C500D5E |
| Equipment technical life |  | 30 years |  |  |  |  |  |  |  |  |
| Anaerobic tank parameters |  |  |  |  |  |  |  |  |  |  |
| Design capacity (m3) |  | 1000 | 1000 | 3500 | 3500 | 2500 | 4500 | 3500 | 1000 | 2000 |
| Stay time(day) |  | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 |
| Fermentation temperature(℃) |  | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 |
| Equipment Quantity |  | 1 | 1 | 2 | 2 | 1 | 2 | 2 | 1 | 1 |
| Equipment technical life |  | 20 years |  |  |  |  |  |  |  |  |
| Turnover machine parameters |  |  |  |  |  |  |  |  |  |  |
| Type |  | FP-2800 | FP-2800 | FP-3800 | FP-3800 | FP-3800 | FP-3800 | FP-3800 | FP-2800 | FP-2800 |
| Power(kw/H) |  | 4 | 4 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 4 | 4 |
| Speed of work(m/h) |  | 6.85 | 6.85 | 6.85 | 6.85 | 6.85 | 6.85 | 6.85 | 6.85 | 6.85 |
| Walking width(mm) |  | 2800 | 2800 | 3800 | 3800 | 3800 | 3800 | 3800 | 2800 | 2800 |
| Tossing speed (mm) |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| Equipment Quantity |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Equipment technical life |  | 15 years |  |  |  |  |  |  |  |  |
| Flared system |  |  |  |  |  |  |  |  |  |  |
| Type | Open flare |  | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare |
| Material | 304 stainless steels |  | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels |
| Height (m) | 3.5 |  | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Main wall thickness (mm) | 3 |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Equipment technical life | 15 years |  |  |  |  |  |  |  |  |  |

##### B.1.1 Forward Action Requests

>>

As per the remaining FAR from the previous verification, VVB is requested to conduct the sampling method during on-site as per the “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” to verify the data of monitored parameter Wsite.

So,during the site visit, VVB conducted the sampling method as per the “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)” to verify the data of monitored parameter Wsite which is reflected in the verification report by VVB.

Post-Design Certification changes

>>

##### B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

>>

NA

##### B.2.2. Corrections

>>

NA

##### B.2.3. Changes to start date of crediting period

>>

NA

##### B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

>>

NA

##### B.2.5. Changes to project design of approved project

>>

NA

- DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

>>

Parameters to be monitored

According to section B.7.1 of the PDD, parameters need to be monitored is as following,

Table 5 monitoring parameter

| parameter | Measurement methods and procedures |
| --- | --- |
| For Mitigation Measure for Safeguarding Principles |  |
| Employee Training of biogas safety operation | Determined by the training Notice which was held once a year. During this monitoring period, one training was conducted on 07/11/2022, which can be proved by training Notice and meeting attendance record. |
| SDG 7 |  |
| Annual amount of biogas utilized | Annual amount of biogas utilized in each farm can be continuously measured by the flow meters 2 installed at the inlet of the biogas generator of each farm. The calibration of flow meters, including the frequency of calibration, should be done per two years in accordance with national standards or requirements. Detailed information on flow meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| SDG 8 |  |
| Total number of jobs | Parameters for SDG 8 were determined based on Record keeping book and cross-checked by labor contract. 18 full-time jobs for local people were created in the 2nd monitoring period (including 9 females and 9 males). |
| SDG 13 |  |
| Number of animals of type LT produced annually for the year y, Np,LT | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals, can track automatically. This electronic ear tag can be connected to the Data Collection System (DCS), which can store and read information. Therefore, the number of swine produced in the farm can be monitored through the auto tracking devices of electronic ear tag once pig slaughter monthly and obtained by the DCS. At the same time, the technicians in farms record manually the number of swine produced in the farms monthly in “Export record form of market swine”. Monthly sale records for each farm can be crosschecked during this monitoring period. |
| Number of days animal of type LT is alive in the farm in the year y, Nda,LT | The days of swine alive in the farm can be traced through the electronic ear tag by the technical staff in each farm and obtained by the DCS. At the same time, the days of swine alive in the farm is recorded in “Export record form of market swine” manually by the technicians in farms. Monthly sale records for each farm used to crosschecked during this monitoring period. |
| Daily stock of animals in the farm, discounting dead and discarded animals, NAA,LT | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals, can track automatically. This electronic ear tag can be connected to the Data Collection System (DCS), which can store and read information. Therefore, the number of swine produced in the farm can be monitored through the auto tracking devices of electronic ear tag. Traced through the electronic ear tag by the technical staff in each farm and obtained by the DCS. At the same time recorded daily by the technicians in farms from the DCS system. The technicians in farms begin to record the daily stock number of animals from the first day of monitoring period, and this record data has included the new imported animal and discounting dead and discharge animals. The annual average number of animals (NAA,LT) was calculated based on the daily stock of breeding swine in the farms without considering dead animals and discarded animals. |
| Average animal weight of a defined livestock population at the project site, Wsite | Measured by the weight measurers at the end of each month. One weight measure was equipped for each farm at least, total 9 weight measures were included for this project. Wsite was conducted by sampling in the three age groups (Nursery phase, Growing phase and Mature phase) of two type swine in each swine farm monthly. The weight data is recorded by the technicians in farms and then average animal weight of a defined livestock population at the project site was calculated based on these data. The weight measures were calibrated periodically by an officially accredited entity. Detailed information on weight measures including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Number of days treatment plant was operational in year y, ndy | Monitored and recorded by PP and the production record from the DCS system can be crosscheck that the treatment plant is operational. |
| Fraction of volatile solids directed to aerobic treatment FAer | According to the applied methodology, FAer should be monitored annually. However, there is no monitoring equip installed in the project activity to monitor the influent into anaerobic digestion and aerobic system during this monitoring period, therefore the value of FAer is applied as 100%, which is conservative and consistency with PDD. |
| Biogas flow, Vf | Measured by the biogas flow meters (biogas flow meter 1 installed at the outlet of the anaerobic digestion, biogas flow meter 2 installed at the inlet of the biogas generator and biogas flow meter 3 installed in the inlet of the flare system) continuously, record hourly and saved automatically in the Data Collection System (DCS). There were three flow meters in each farm and total 27 flow meters were included for this project. Daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. The biogas flow meters used were calibrated periodically by an officially accredited entity. In addition, the biogas flow meters automatically measured the instantaneous flow rate, temperature and pressure per hour, so it is not necessary to install other equipment separately to measure the instantaneous flow rate (Vt,db), the biogas temperature (Tt) and pressure (Pt). Collected data is archived in both electronic and printed copies. Detailed information on flowmeters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Electricity generated using biogas, EGd,y | Measured continuously by the electricity meters 2 installed at the outlet of generator in each farm. Total 9 meters were installed. The electric meter reading was recorded daily by the staff and the electricity generation is not included during the commissioning period. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity generation is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity generation. The electricity meters were calibrated periodically by an officially accredited entity. Collected data is archived in both electronic and printed copies. Detailed information on meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Quantity of electricity consumed by the proposed project in year y, ECPJ,j,y | Measured continuously by the electricity meters 1 installed in project site in each farm. Total 9 meters were installed in this project. The electric meter reading was recorded daily and summarized monthly by the staff and the electricity consumption is not included during commissioning period. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity consumption is equal to the difference between the previous day's and the next day's readings. The staff in each swine farm recorded the meter reading at 24:00 of last day in each month and then reported the monthly electricity consumption to the grid company. The grid company checked and supplied the reading records (Electricity Transaction Notes) of the meters to the PP monthly. The electricity meters were calibrated periodically by an officially accredited entity. Collected data is archived in both electronic and printed copies. Detailed information on meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Average technical transmission and distribution losses for providing electricity to source j in year y, TDLj,y | Sourced from Tool 05” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation (version 03.0)”, the value is 20% in this monitoring period. This value will be renewed once the tool is updated. |
| Volumetric flow of the gaseous stream in time interval t on a dry basis, Vt,db | Measured by flowmeters, which can record hourly and saved automatically in the DCS system. As per described in monitoring parameter Vf, the flowmeters to monitor Vt,db are the same as the flowmeters that monitor Vf since the flowmeter can monitor other parameter such as Vt,db, Tt and Pt simultaneously. So, the data can be obtained from the system and the monthly data was exported by the staff at the beginning of next month. Daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. Original collected data can be exported through the system and is archived in electronic. |
| Volumetric fraction of greenhouse gas i in a time interval t on a dry basis, m³ gas i/m³ dry gas, Vi,t,db | Continuously measurement and record hourly by online gas analyser installed at the outlet of the anaerobic digester and saved automatically in the DCS system. One gas analyser was equipped in each farm and there are 9 gas analysers totally. Therefore, the data can be obtained from system and the monthly data was exported by the staff at the beginning of next month. Daily and monthly CH4 concentration was calculated based on the 24-hour average and daily average CH4 concentration. Gas analyzers were calibrated periodically by an officially accredited entity. Original collected data can be exported through the system and is archived in electronic. Detailed information on gas analyzers including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Temperature of the gaseous stream in time interval t, Tt; Pressure of the gaseous stream in time interval t, Pt. | Continuously Measurement by the biogas flow meters, which can record hourly and saved automatically in the DCS system. As per described in monitoring parameter Vf, the flowmeters to monitor Tt and Pt are the same as the flowmeters that monitor Vf since the flowmeter can monitor other parameter such as Vt,db, Tt and Pt simultaneously. So, the data can be obtained from DCS system, and the monthly data was exported by the staff at the beginning of next month. Daily and monthly Temperature and Pressure of the biogas flow was calculated based on the 24-hour average and daily average values. Original collected data can be exported through the system and is archived in electronic. Detailed information on Flow meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Density of greenhouse gas i in the gaseous stream in time interval t, ρi,t | This parameter was calculated based on the temperature and pressure of the gaseous stream in time interval t. The monitor of the temperature and pressure of the gaseous stream see the description of the above two parameters in this table. |
| Fraction of manure handled in system j in project activity MS%j | As there is no equipment installed to monitor this parameter in the actual operation. so, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%. |
| Maximum methane producing potential of the volatile solid generated by animal type LT, B0, LT | Not monitored during this monitoring period and the value is taken from IPCC 2006, which is a published source. The parameter value will be updated on latest available public data source. |
| Type of barn and AWMS, Type | During this monitoring period, the type of barn and AWMS was not changed as the design. |
| Annual average ambient temperature at project site, T | PP entrusted the local meteorological bureau to continuously monitor the temperature of the project site during the monitoring period. The local meteorological bureau had conducted measurements continuously in project site and aggregated in monthly and annual average values as per requirements of the methodology, and finally a Temperature Monitoring Report was issued by local meteorological bureau. As per the Temperature Monitoring Report, during this monitoring period, the average temperature of any month at the project site is greater than 5 ℃ and annual average ambient temperature at project site during this monitoring period is 21.9℃. According to the actual temperature (21.9℃) during this monitoring and the temperature of ex-ante estimated(21.8℃), the mean annual temperature at the project site is higher than 18℃. And the mean annual precipitation is 1771mm, which is higher than 1000mm. Therefore, as per Annex 10A.2, 2019 Refinement to the 2006 IPCC Guidelines, the climate zone of the project site can be determined to "Tropical Moist ", and the MCF value of 80% is applied, then multiplying MCF values with a value of 0.94. So, MCFj value of 75.2% (75.2%=80%*0.94) is applied in this monitoring period. |
| Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2, NEXLT | Not monitored during this MR since the parameter is calculated as the equation of option 2 in Appendix 2 of methodology ACM0010 Version 08.0 and the value of other parameter applied in the equation is taken from IPCC 2006, which is a published source. The parameter value will be updated on latest available public data source. |

Monitoring framework

The project owner is responsible for the whole monitoring work. The GS Monitoring Team is established to collect and record monitoring data within the project boundary. The GS monitoring team is responsible for the normal operation of the AWMSs and the collection and record of all the monitoring data. All the data is reviewed by the project developer and VVB. Each member of the GS monitoring team is trained by the project owner at least once a year. The overall monitoring system structure of the project shows as below:

Figure 3 The Organization Structure of the Monitoring Team

Monitoring equipment and installation

Installation and configuration of monitoring equipment are shown as Figure 4.  In order to ensure measurements with a low degree of uncertainty, the data monitoring equipment have been calibrated and checked by appropriately qualified third party according to appropriate national standard.

For this monitoring period, all the monitoring equipment were calibrated according to relevant regulations.

Figure 4 The monitoring system of the project (Same monitoring system for each farm)

Corrective actions

The whole GS monitoring team fellow recognized standard data evaluation methods to guarantee that the data was reliable and accurate. There was no correction of nonconformities occurred in implementation of the project or the monitoring plan during the 2nd monitoring period. For QA/QC procedure, please refer to QA/QC procedures in section D.2 of the monitoring report.

Data management

Records are the most important exercise in relation to the monitoring process. Without accurate and efficient record keeping, project emission reductions cannot be verified.

Once missing data on a given day or a certain period of time happened, the most appropriate values such as daily average or monthly average will be used for calculation, which is to ensure that emission reductions are conservative.

All data collected as part of the monitoring plan shall be archived in electronic form and paper until at least two years after the entry period.

Emergency procedure

Employees need to be familiar with the process flow and emergency situations that may occur during project operation and operate in strict accordance with the technical specifications of each process treatment equipment and monitoring equipment. During the operation of the project, all equipment and related instruments shall be checked regularly according to the manufacturer’s recommendations. If necessary, calibration and/or proper repair have to be carried out immediately.

In case of malfunction and/or damage of any system or piece of the process, it may cause damage to the operation of the system, especially serious damage to the biogas generation system and combustion system it must be dealt with as soon as possible and reported to the responsible technician immediately.

There was no emergency happened during this monitoring period.

Avoiding of double counting

All swine farms involved in this project can be identified through its GPS coordinates, which will prevent the farms counted in the project activity to be part of any other voluntary market or emission reduction mechanism (CDM, CCER, VCS etc.) as well. Besides, the project owner has signed the Declaration of No Double Counting Statement and Declaration of not involved in other GHG scheme on 30/07/2021. The emission reductions were not double counted during the 2nd monitoring period.

Table 6 General description of monitoring equipment

| Monitoring equipment | Parameter to be monitored | Installed location | Type | Series number | Accuracy | Date of calibration | Calibration standard | Validity | Calibration agency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow meter 1 | Vf Tt Pt | at the outlet of the anaerobic digestion | HD-LUB | Please refer to below Table 7 | 1.5 | 08/12/2020, 05/12/2022 | JJG1029-2007(Verification Regulation of Vortex-shedding Flowmeter) | 07/12/2022, 04/12/2024 | GUANGDONG INSTITUTE OF METEOLOGY |
| Flow meter 2 | Vf Tt Pt | at the inlet of generator | HD-LUB |  | 1.5 | 08/12/2020, 05/12/2022 | JJG1029-2007(Verification Regulation of Vortex-shedding Flowmeter) | 07/12/2022, 04/12/2024 | GUANGDONG INSTITUTE OF METEOLOGY |
| Flow meter 3 | Vf Tt Pt | in inlet of flare | HD-LUB |  | 1.5 | 08/12/2020, 05/12/2022 | JJG1029-2007(Verification Regulation of Vortex-shedding Flowmeter) | 07/12/2022, 04/12/2024 | GUANGDONG INSTITUTE OF METEOLOGY |
| Electricity meter 1 | EGd,y | at the outlet of generator | DSZ719 |  | 0.5s | 10/12/2020 | JJG596-2012 (Electrical Meters for Measuring Alternating-current Electrical Energy) | 09/12/2025 | GUANGDONG INSTITUTE OF METEOLOGY |
| Electricity meter 2 | ECPJ,j,y | in the project site | DSZ719 |  | 0.5s | 10/12/2020 | JJG596-2012 (Electrical Meters for Measuring Alternating-current Electrical Energy) | 09/12/2025 | GUANGDONG INSTITUTE OF METEOLOGY |
| Weight measurer | wsite | in each pig farm | XK3190-A12+（E） |  | Ⅲ level | 03/11/2021, 01/11/2022 | JJG539-2016(Digital Indicator Scale Verification Regulations) | 02/11/2022, 31/10/2023 | GUANGDONG INSTITUTE OF METEOLOGY |
| Gas analyzer | Vi,t,db | at the outlet of the anaerobic digesters | Gasboard-9060 |  | 1% | 05/12/2021, 02/12/2022 | JJG693-2011(Verification Regulation of Alarmer Detectors of Combustible Gas) | 04/12/2022, 01/12/2023 | GUANGDONG INSTITUTE OF METEOLOGY |

Table 7 Series number of monitoring equipment

| Swine farm | Series number |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Flowmeter 1 | Flowmeter 2 | Flowmeter 3 | Electricity meter 1 | Electricity meter 2 | Weight measurer | Gas analyzer |
| Huicheng Hengli swine Farm | 202001005378 | 202001005379 | 202001005380 | 1306132167000001 | 1306132167000002 | 2018097681 | 2020123109 |
| Dongyuan Huangtian swine Farm | 202001004657 | 202001004658 | 202001004659 | 1306156238000001 | 1306156238000002 | 2018084667 | 2020123828 |
| Enping Shahu swine Farm | 202002004624 | 202002004625 | 202002004626 | 1306147623000001 | 1306147623000002 | 2018022963 | 2020124035 |
| Langtian Yuehui swine Farm | 202001007321 | 202001007322 | 202001007323 | 1306173585000001 | 1306173585000002 | 2018107964 | 2020122805 |
| Qujiang Fengwan swine Farm | 202002004372 | 202002004373 | 202002004374 | 1306162378000001 | 1306162378000002 | 2018087662 | 2020126176 |
| Pingyuan Zhongshi swine Farm | 202001006520 | 202001006521 | 202001006522 | 1306111516000001 | 1306111516000002 | 2018098117 | 2020127520 |
| Yangchun Tanshui swine Farm | 202002003564 | 202002003565 | 202002003566 | 1306142589600001 | 1306142589600002 | 2019034022 | 2020125829 |
| Xinfeng Shatian swine Farm | 202001004476 | 202001004477 | 202001004478 | 1306135828000001 | 1306135828000002 | 2018102807 | 2020125481 |
| Kaiping Cangcheng swine Farm | 202002006384 | 202002006385 | 202002006386 | 1306155623000001 | 1306155623000002 | 2018123930 | 2020128914 |

- DATA AND PARAMETERS

Data and parameters fixed ex ante or at renewal of crediting period

>>

SDG13

| Data/parameter | GWPCH4 |
| --- | --- |
| Unit | tCO2e/tCH4 |
| Description | Global Warming Potential of CH4 |
| Source of data | IPCC Fourth/Fifth Assessment Report |
| Value(s) applied | 25/28 |
| Choice of data or Measurement methods and procedures | 100-year values are adopted from IPCC Fourth/Fifth Assessment Report |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | All monitoring, verifications and requests for performance certification of Certified products and / or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values (28) for calculating emission reductions and removals according to applicability of global warming potential for gold standard for the global goals projects. This monitoring period is from 01/04/2022 to 31/03/2023, so the emission reductions calculation applies AR5 GWP value (28). |

| Data/parameter | GWPN2O |
| --- | --- |
| Unit | tCO2e/tN2O |
| Description | Global Warming Potential of N2O |
| Source of data | IPCC Fourth/Fifth Assessment Report |
| Value(s) applied | 298/265 |
| Choice of data or Measurement methods and procedures | 100-year values are adopted from IPCC Fourth/Fifth Assessment Report |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | All monitoring, verifications and requests for performance certification of Certified products and / or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values (28) for calculating emission reductions and removals according to applicability of global warming potential for gold standard for the global goals projects. This monitoring period is from 01/04/2022 to 31/03/2023, so the emission reductions calculation applies AR5 GWP value (265). |

| Data/parameter | DCH4 |
| --- | --- |
| Unit | t/m3 |
| Description | Density of CH4 |
| Source of data | ACM0010 Version 08.0 |
| Value(s) applied | 0.00067 |
| Choice of data or Measurement methods and procedures | 0.00067 t/m3 at room temperature 20℃ and 1 atm pressure. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | MCFj |
| --- | --- |
| Unit | - |
| Description | Methane conversion factor for the baseline AWMSj |
| Source of data | 2019 Refinement to the 2006 IPCC Guidelines table 10.17, chapter 10, volume 4 |
| Value(s) applied | 75.2% |
| Choice of data or Measurement methods and procedures | MCFj value for uncovered anaerobic lagoon (baseline AWMS) is chosen. The MCFj values given in table 10.17, chapter 10, volume 4, 2019 Refinement to the 2006 IPCC Guidelines should be used. MCFj values depend on the climate zone where the uncover anaerobic lagoon in the baseline. According to the registered PDD, the annual average ambient temperature at the project site is 21.8℃. And as per the Temperature Monitoring Report published by the local meteorological bureau, during this monitoring period, the average temperature of any month at the project site is greater than 5 ℃ and annual average ambient temperature at project site during this monitoring period is 21.9℃. So, the mean annual temperature at the project site is higher than 18℃. And the mean annual precipitation is 1771mm, which is higher than 1000mm. Therefore, as per Annex 10A.2, 2019 Refinement to the 2006 IPCC Guidelines, the climate zone of the project site can be determined to "Tropical Moist ", and the MCF value of 80% is applied. A conservativeness factor should be applied by multiplying MCFj values (i.e., 80%) with a value of 0.94, to account for the 20% uncertainty in the MCFj values as reported by IPCC. So, MCFj value of 75.2% (75.2%=80%*0.94) is applied in this monitoring period. |
| Purpose of data | project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | MS%Bl,j |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of manure handled in system j in the baseline. |
| Source of data | Project proponents |
| Value(s) applied | 100% |
| Choice of data or Measurement methods and procedures | In this project, the baseline manure management system is uncovered anaerobic lagoon only. The amount of manure handled by the anaerobic lagoon is 100%. |
| Purpose of data | Calculation of baseline emissions |
| Additional comment | N/A |

| Data/parameter | Wdefault |
| --- | --- |
| Unit | kg |
| Description | Default average animal weight of a defined population |
| Source of data | IPCC 2006 Table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | Wdefault (Market swine) =28kg Wdefault (Breeding swine) =28kg |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | N/A |

| Data/parameter | VSdefault |
| --- | --- |
| Unit | kg-dm/animal/day |
| Description | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population |
| Source of data | IPCC 2006 Table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | VSdefault(Market swine)=0.3 VSdefault(Breeding swine)=0.3 |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | Nrate(T) |
| --- | --- |
| Unit | kg N (1000 kg animal mass)-1 day-1 |
| Description | default N excretion rate |
| Source of data | IPCC 2006 Table 10.19, chapter 10, volume 4 |
| Value(s) applied | Nrate(T) (Market swine) =0.42 Nrate(T)(Breeding swine) =0.24 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter | TAM |
| --- | --- |
| Unit | kg animal-1 |
| Description | typical animal mass for livestock category |
| Source of data | IPCC 2006 Table 40A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | TAM (Market swine) =28 TAM (Breeding swine) =28 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter | NEXIPCC default |
| --- | --- |
| Unit | kg N/ animal/year |
| Description | Default value for the nitrogen excretion per head of a defined livestock population |
| Source of data | Calculated by the equation: NEXIPCC default =Nrate(T)* TAM/1000*365 |
| Value(s) applied | Period; Market Swine; Breeding Swine. 01/04/2022-30/04/2022; 0.35; 0.20. 01/05/2022-31/05/2022; 0.36; 0.21. 01/06/2022-30/06/2022; 0.35; 0.20. 01/07/2022-31/07/2022; 0.36; 0.21. 01/08/2022-31/08/2022; 0.36; 0.21. 01/09/2022-30/09/2022; 0.35; 0.20. 01/10/2022-31/10/2022; 0.36; 0.21. 01/11/2022-30/11/2022; 0.35; 0.20. 01/12/2022-31/12/2022; 0.36; 0.21. 01/01/2023-31/01/2023; 0.36; 0.21. 01/02/2023-28/02/2023; 0.33; 0.19. 01/03/2023-31/03/2023; 0.36; 0.21. |
| Choice of data or Measurement methods and procedures | NEXIPCC default is calculated as equation 10.30 in IPCC2006, Nrate(T) and TAM are default value from IPCC 2006. |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

| Data/parameter | Fgas MS,j,LT |
| --- | --- |
| Unit | Fraction |
| Description | Default values for nitrogen loss due to volatilization of NH3 and NOX from manure management |
| Source of data | IPCC 2006 Table 10.22, chapter 10, volume 4 |
| Value(s) applied | FgasMS,j,LT, (anaerobic lagoon) : 40% FgasMS,j,LT , (solid storage) : 45% |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFN2O,D,j |
| --- | --- |
| Unit | Kg N2O-N/kg N |
| Description | Direct N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 Table 10.21, chapter 10, volume 4 |
| Value(s) applied | EFN2O,D,j=0 for anaerobic lagoon and digester, EFN2O,D,j =0.01 for aerobic lagoon |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFN2O,ID |
| --- | --- |
| Unit | kgN2O-N/kg NH3-N and NOX-N |
| Description | Indirect N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 Table 11.3, chapter 11, volume 4 |
| Value(s) applied | 0.01 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFCH4,default |
| --- | --- |
| Unit | t CH4 leaked / t CH4 produced |
| Description | Default emission factor for the fraction of CH4 produced that leak from the anaerobic digester (fraction) |
| Source of data | Tool 14:” Project and leakage emissions from anaerobic digesters (version02.0) |
| Value(s) applied | 0.05 |
| Choice of data or Measurement methods and procedures | UASB (Upflow Anaerobic Sludge Blanket) type digesters, floating gas holders with no external water seal |
| Purpose of data | Calculation of project emissions |
| Additional comment | PECH4,y for ex ante estimation adopted equation(4) of Methodological tool “Project and leakage emissions from anaerobic digesters”, the amount of biogas collected at the digester will be monitored in section B.7 of PDD. |

| Data/parameter | RVS,n |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated |
| Source of data | Refer to Appendix 1 of methodology ACM0010 |
| Value(s) applied | RVS,n, aerobic treatment and anaerobic digester: 20%, 80% for leakage N2O emission released during project activity RVS,n, one cell lagoon :85% for leakage N2O emission released during baseline scenario |
| Choice of data or Measurement methods and procedures | Estimated from Table provided in Appendix 1 of ACM0010. The most conservative value for the given technology must be used. |
| Purpose of data | project emission/ leakage calculation |
| Additional comment | N/A |

| Data/parameter | RN,n |
| --- | --- |
| Unit | Fraction |
| Description | Nitrogen reduction factor |
| Source of data | Refer to Appendix 1 of methodology ACM0010 |
| Value(s) applied | RN,n, project activity(aerobic treatment and anaerobic digester): 5%,25% RN,n, baseline ( uncovered anaerobic lagoon) : 80% |
| Choice of data or Measurement methods and procedures | Anaerobic digester has been assumed as a covered first cell of two cell lagoon. Uncovered anaerobic lagoon has been assumed as one cell lagoon. The most conservative value for the given technology has been used. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | EF1, EF4, EF5 |
| --- | --- |
| Unit | kg N2O-N/kg N for EF1, EF5 and [kg N2O-N/(kg NH3-N and NOX-N) for EF4 |
| Description | Emission factor for N2O emissions from N inputs; from N leaching and runoff; from atmospheric deposition of N on soils and water surfaces |
| Source of data | IPCC 2006 Guidelines default values are be used, since country specific or region-specific data are not available. EF1 from Table 41.1, chapter 11, volume 4. EF4 and EF5 from Table 41.3, chapter 11, volume 4 |
| Value(s) applied | EF1 = 0.010 EF4 =0.010 EF5 = 0.0075 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | Fgasm |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of N lost due to volatilization |
| Source of data | Default values from Table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.2 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | Fleach |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff |
| Source of data | Default values from Table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.3 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | MCFd |
| --- | --- |
| Unit | - |
| Description | Methane conversion factor for leakage calculation |
| Source of data | Methodology ACM0010(version 08.0) |
| Value(s) applied | 1 |
| Choice of data or Measurement methods and procedures | According to paragraph 51 of the methodology ACM0010” GHG emission reductions from manure management systems (version 08.0)”, Methane conversion factor for leakage calculation assumed to be equal 1 |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | EFEF,j,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Emission factor for electricity generation |
| Source of data | Published by Ministry of Ecology and Environment of the People's Republic of China, which is the DNA of China |
| Value(s) applied | 0.50885 |
| Choice of data or Measurement methods and procedures | According to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | Ru |
| --- | --- |
| Unit | Pa.m3/kmol.K |
| Description | Universal ideal gases constant |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 8,314 |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | MMi |
| --- | --- |
| Unit | kg/kmol |
| Description | Molecular mass of greenhouse gas I |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 16.04 kg/mol for methane |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | ηflare,m |
| --- | --- |
| Unit | % |
| Description | Flare efficiency in minute m |
| Source of data | Tool 06” Project emissions from flaring (Version 04.0)” |
| Value(s) applied | 0% |
| Choice of data or Measurement methods and procedures | 0% is the conservative value. |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

Data and parameters monitored

>>

SDG 13

| Data / Parameter | Np,LT |
| --- | --- |
| Unit | Number |
| Description | Number of animals of type LT produced annually for the year y |
| Source of data | Production record of market swine |
| Value(s) of monitored parameter | Period; Np,LT. 01/04/2022-30/04/2022; 21,420. 01/05/2022-31/05/2022; 21,604. 01/06/2022-30/06/2022; 21,463. 01/07/2022-31/07/2022; 21,477. 01/08/2022-31/08/2022; 21,629. 01/09/2022-30/09/2022; 21,580. 01/10/2022-31/10/2022; 21,490. 01/11/2022-30/11/2022; 21,587. 01/12/2022-31/12/2022; 21,536. 01/01/2023-31/01/2023; 21,504. 01/02/2023-28/02/2023; 21,629. 01/03/2023-31/03/2023; 21,524. Total; 258,443. |
| Measurement methods and procedures | To be collected for each swine population in all of the pig barns. The number of swine produced in the swine farms had been recorded in the “Export record form of Market swine” manually by the responsible staff. Please refer to Section C of the MR for more details. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | According to the Equation 4 in this MR, the calculated NLT is 127,451 heads (258,443 heads *180days/365days). However, the each swine farm of NLT is calculated in Equation 4 in the MR through the Production Record in each swine farm. The calculation result of NLT is 127,447 head. And the detailed calculation process also is included in ER spreadsheet “Monitoring results”. According to the conservative principle, the lower value of 127,447 heads was applied in the calculations of emission reductions in MR. Np,LT has been cross-checked through the monthly sale records of 9 swine farms during this monitoring period. And the values between the Export record form of Market swine and the monthly sale records are consistent. Therefore, Np,LT is reasonable and credible. |

| Data / Parameter | Nda,LT |
| --- | --- |
| Unit | Number |
| Description | Number of days animal of type LT is alive in the farm in the year y |
| Source of data | Production record of market swine |
| Value(s) of monitored parameter | Period; Nda,LT. 01/04/2022-30/04/2022; 180. 01/05/2022-31/05/2022; 180. 01/06/2022-30/06/2022; 180. 01/07/2022-31/07/2022; 180. 01/08/2022-31/08/2022; 180. 01/09/2022-30/09/2022; 180. 01/10/2022-31/10/2022; 180. 01/11/2022-30/11/2022; 180. 01/12/2022-31/12/2022; 180. 01/01/2023-31/01/2023; 180. 01/02/2023-28/02/2023; 180. 01/03/2023-31/03/2023; 180. |
| Measurement methods and procedures | The days of swine alive in the farm was recorded in the Export record form of Market swine manually by the responsible staff. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | The number of days the animal alive in the farm sourced from the Export record form of Market swine and was crosschecked through the monthly sale records of 9 swine farms during this monitoring period and it can be concluded that the data is consistent. In addition, this value is in line with the number of days for pigs to be slaughtered by existing large-scale breeding groups in China. |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | Number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Daily stock record of breeding swine |
| Value(s) of monitored parameter | Period; NAA,LT. 01/04/2022-30/04/2022; 94,180. 01/05/2022-31/05/2022; 93,502. 01/06/2022-30/06/2022; 93,732. 01/07/2022-31/07/2022; 93,352. 01/08/2022-31/08/2022; 94,055. 01/09/2022-30/09/2022; 93,324. 01/10/2022-31/10/2022; 93,347. 01/11/2022-30/11/2022; 92,517. 01/12/2022-31/12/2022; 92,988. 01/01/2023-31/01/2023; 92,822. 01/02/2023-28/02/2023; 93,454. 01/03/2023-31/03/2023; 94,666. Average; 93,495. |
| Measurement methods and procedures | Monitored daily through the auto tracking devices of electronic ear tag and the data can be obtained from DCS. The technicians in farms record the daily stock number of animal, and this record data including the new imported animal and discounting dead and discharge animals. Archive electronically during project plus 5 years Please see the Section C of MR for details. |
| Monitoring frequency | daily |
| QA/QC procedures | The system on monitoring the number of livestock population was described in the PDD. |
| Purpose of data | Use for the calculation of baseline/project emission and leakage |
| Additional comment | The system on monitoring the daily stock of breeding swine in the farm was descried in Section C of MR. |

| Data / Parameter | Wsite |
| --- | --- |
| Unit | kg |
| Description | Average animal weight of a defined livestock population at the project site |
| Source of data | Weight record table |
| Value(s) of monitored parameter | Period; Market swine; Breeding swine. 01/04/2022-30/04/2022; 57.3; 74.5. 01/05/2022-31/05/2022; 56.9; 74.3. 01/06/2022-30/06/2022; 57.5; 74.7. 01/07/2022-31/07/2022; 57.2; 73.8. 01/08/2022-31/08/2022; 57.6; 74.8. 01/09/2022-30/09/2022; 58.4; 73.6. 01/10/2022-31/10/2022; 58.6; 73.7. 01/11/2022-30/11/2022; 57.6; 74.5. 01/12/2022-31/12/2022; 58.0; 75.2. 01/01/2023-31/01/2023; 57.8; 73.6. 01/02/2023-28/02/2023; 56.3; 74.0. 01/03/2023-31/03/2023; 57.9; 74.0. Average; 57.6; 74.2. |
| Measurement methods and procedures | Measured by the weight measurers and record by the technical staff in each farm monthly Please refer to Section D.4 of the MR for more details. |
| Monitoring frequency | monthly |
| QA/QC procedures | As this project involved 9 swine farms and two types of swine i.e., Market swine and Breeding swine, and as per applied methodology, each defined livestock population should be classified into a minimum of three age categories (The three age categories of marketing swine are classified according to the age in days, i.e. Nursery phase with 30-60days, Growing phase with 60-130days and Mature phase with 130-180days. The three age categories of breeding swine are classified according to the age in days, i.e., Nursery phase with 30-70days, Growing phase with 70-220days and Mature phase with 220-310days.), so the sampling method is Stratified random sampling. The specific sampling methods are as follows: The sampling method is followed as PDD and described as below during this monitoring period. - the overall sample size based on the design population of pigs in each farm firstly is calculated as described in PDD. - As the project involved 9 swine farms, then the sample size in each swine farm is determined based on the proportion of the number of each farm in the total number of 9 farms. - Similarly, the sample size of each age group of Market swine and Breeding swine in each farm is also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm. - After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample is conducted in every swine farm. Since swine in different age are kept in the different pig houses, samples can be randomly selected from pig houses of this age group. - After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table. - The sample will be marked after weighting and if the sample swine is still drawn next month, then this sample swine needs to be discarded and a new sample swine is drawn. This will be done every month until the end of the monitoring period. |
| Purpose of data | Used for estimating VSLT,y |
| Additional comment | Please see section D.4 for detail about sampling. |

| Data/parameter | ndy |
| --- | --- |
| Unit | number |
| Description | Number of days treatment plant was operational in year y. |
| Source of data | Project proponents |
| Value(s) of monitored parameter | The actual number of days 9 treatment plants that was operational are 365 days during this monitoring periods Period; ndy. 01/04/2022-30/04/2022; 30. 01/05/2022-31/05/2022; 31. 01/06/2022-30/06/2022; 30. 01/07/2022-31/07/2022; 31. 01/08/2022-31/08/2022; 31. 01/09/2022-30/09/2022; 30. 01/10/2022-31/10/2022; 31. 01/11/2022-30/11/2022; 30. 01/12/2022-31/12/2022; 31. 01/01/2023-31/01/2023; 31. 01/02/2023-28/02/2023; 28. 01/03/2023-31/03/2023; 31. Average; 365. |
| Measurement methods and procedures | The staff in each manure treatment plant record the number of operation days daily and then record the total monthly number of operation days in end of each month through monthly operation record. The 9 treatment plants involved in this project is normal operational every day during this monitoring period according to the actual daily reading of flowmeters installed in the outlet of anaerobic digesters. |
| Monitoring frequency | Daily |
| QA/QC procedures | The production record from DCS system can be cross-checked that the AWMSs are actually running on a daily basis. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | ndy is the number of days treatment plants was operational. As the anaerobic fermentation is the main process of the treatment process for the project, so the operating days of the anaerobic tank in each farm can be considered as the operating days of the treatment plant. as long as the anaerobic tank is running, biogas will be generated, so the period of biogas generation monitored by the flow meter is the operating days of the treatment plant. Therefore, each farm was operational for 365 days and there was no breakdown/ non –operational day at any of the farms according to the daily meter reading of flowmeter installed in the outlet of anaerobic digesters in each farm. |

| Data / Parameter | FAer |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of volatile solids directed to aerobic treatment |
| Source of data | PDD |
| Value(s) of monitored parameter | 100% |
| Measurement methods and procedures | As this parameter is not monitored in the actual operation. So, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%, which is consistent with PDD. |
| Monitoring frequency | 100% |
| QA/QC procedures | Value of FAer is applied as 100%, which is conservative. |
| Purpose of data | Annually |
| Additional comment | N/A |

| Data/parameter | vf |
| --- | --- |
| Unit | m3 |
| Description | Biogas flow |
| Source of data | Project proponents |
| Value(s) of monitored parameter | Huicheng Hengli swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 50,251.90; 48,744.56; 0.00. 01/05/2022-31/05/2022; 45,131.60; 43,776.92; 0.00. 01/06/2022-30/06/2022; 50,394.34; 48,882.37; 0.00. 01/07/2022-31/07/2022; 47,796.41; 46,363.16; 0.00. 01/08/2022-31/08/2022; 49,365.89; 47,885.58; 0.00. 01/09/2022-30/09/2022; 48,393.58; 46,941.54; 0.00. 01/10/2022-31/10/2022; 49,255.11; 47,777.98; 0.00. 01/11/2022-30/11/2022; 50,560.46; 49,043.17; 0.00. 01/12/2022-31/12/2022; 48,317.02; 46,868.23; 0.00. 01/01/2023-31/01/2023; 49,666.50; 48,176.57; 0.00. 01/02/2023-28/02/2023; 48,745.75; 47,284.10; 0.00. 01/03/2023-31/03/2023; 49,595.28; 48,107.59; 0.00. Dongyuan Huangtian swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 60,583.96; 58,462.67; 0.00. 01/05/2022-31/05/2022; 54,692.38; 52,778.21; 0.00. 01/06/2022-30/06/2022; 60,987.41; 58,852.99; 0.00. 01/07/2022-31/07/2022; 59,134.94; 57,064.10; 0.00. 01/08/2022-31/08/2022; 60,718.46; 58,593.60; 0.00. 01/09/2022-30/09/2022; 59,127.29; 57,056.10; 0.00. 01/10/2022-31/10/2022; 60,805.46; 58,677.09; 0.00. 01/11/2022-30/11/2022; 60,860.85; 58,730.36; 0.00. 01/12/2022-31/12/2022; 58,614.34; 56,563.11; 0.00. 01/01/2023-31/01/2023; 60,710.51; 58,585.21; 0.00. 01/02/2023-28/02/2023; 58,905.26; 56,844.00; 0.00. 01/03/2023-31/03/2023; 60,924.13; 58,791.31; 0.00. Enping Shahu swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 308,659.56; 297,854.35; 0.00. 01/05/2022-31/05/2022; 277,584.36; 267,872.53; 0.00. 01/06/2022-30/06/2022; 307,145.52; 296,399.94; 0.00. 01/07/2022-31/07/2022; 298,655.28; 288,204.73; 0.00. 01/08/2022-31/08/2022; 307,530.12; 296,764.52; 0.00. 01/09/2022-30/09/2022; 296,706.96; 286,321.12; 0.00. 01/10/2022-31/10/2022; 304,633.02; 293,970.68; 0.00. 01/11/2022-30/11/2022; 307,260.06; 296,510.50; 0.00. 01/12/2022-31/12/2022; 297,498.96; 287,086.88; 0.00. 01/01/2023-31/01/2023; 306,736.32; 295,998.73; 0.00. 01/02/2023-28/02/2023; 295,408.08; 285,062.68; 0.00. 01/03/2023-31/03/2023; 304,935.84; 294,261.74; 0.00. Langtian Yuehui swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 298,734.80; 289,690.62; 0.00. 01/05/2022-31/05/2022; 272,032.99; 263,634.02; 0.00. 01/06/2022-30/06/2022; 299,953.15; 290,928.42; 0.00. 01/07/2022-31/07/2022; 289,955.69; 281,287.79; 0.00. 01/08/2022-31/08/2022; 299,802.81; 290,966.33; 0.00. 01/09/2022-30/09/2022; 291,930.94; 283,202.81; 0.00. 01/10/2022-31/10/2022; 299,794.92; 290,696.26; 0.00. 01/11/2022-30/11/2022; 301,013.21; 291,951.25; 0.00. 01/12/2022-31/12/2022; 291,142.37; 282,285.98; 0.00. 01/01/2023-31/01/2023; 302,722.07; 293,686.59; 0.00. 01/02/2023-28/02/2023; 289,963.34; 281,240.85; 0.00. 01/03/2023-31/03/2023; 299,707.87; 290,805.30; 0.00. Qujiang Fengwan swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 161,056.61; 155,904.20; 0.00. 01/05/2022-31/05/2022; 146,364.55; 141,680.24; 0.00. 01/06/2022-30/06/2022; 164,508.99; 159,241.36; 0.00. 01/07/2022-31/07/2022; 155,596.45; 150,617.35; 0.00. 01/08/2022-31/08/2022; 163,953.74; 158,708.03; 0.00. 01/09/2022-30/09/2022; 157,886.12; 152,832.88; 0.00. 01/10/2022-31/10/2022; 161,257.80; 156,098.13; 0.00. 01/11/2022-30/11/2022; 162,875.34; 157,663.14; 0.00. 01/12/2022-31/12/2022; 157,574.60; 152,534.00; 0.00. 01/01/2023-31/01/2023; 161,644.07; 156,472.29; 0.00. 01/02/2023-28/02/2023; 155,650.97; 150,668.66; 0.00. 01/03/2023-31/03/2023; 161,611.86; 156,440.22; 0.00. Pingyuan Zhongshi swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 347,594.35; 333,687.01; 0.00. 01/05/2022-31/05/2022; 316,346.03; 303,695.48; 0.00. 01/06/2022-30/06/2022; 354,845.28; 340,649.01; 0.00. 01/07/2022-31/07/2022; 342,167.36; 328,482.85; 0.00. 01/08/2022-31/08/2022; 352,943.96; 338,824.59; 0.00. 01/09/2022-30/09/2022; 338,935.21; 325,367.43; 0.00. 01/10/2022-31/10/2022; 351,895.73; 337,818.06; 0.00. 01/11/2022-30/11/2022; 352,662.05; 338,551.46; 0.00. 01/12/2022-31/12/2022; 337,612.97; 324,106.42; 0.00. 01/01/2023-31/01/2023; 348,201.63; 334,275.30; 0.00. 01/02/2023-28/02/2023; 339,026.16; 325,462.70; 0.00. 01/03/2023-31/03/2023; 352,156.01; 338,064.36; 0.00. Yangchun Tanshui swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 308,520.42; 299,475.89; 0.00. 01/05/2022-31/05/2022; 279,358.44; 271,002.28; 0.00. 01/06/2022-30/06/2022; 310,443.66; 300,925.84; 0.00. 01/07/2022-31/07/2022; 300,342.24; 291,050.42; 0.00. 01/08/2022-31/08/2022; 307,652.88; 298,523.18; 0.00. 01/09/2022-30/09/2022; 298,473.12; 289,498.55; 0.00. 01/10/2022-31/10/2022; 310,640.04; 301,220.27; 0.00. 01/11/2022-30/11/2022; 307,751.10; 298,558.22; 0.00. 01/12/2022-31/12/2022; 299,407.68; 290,311.41; 0.00. 01/01/2023-31/01/2023; 308,905.08; 299,561.65; 0.00. 01/02/2023-28/02/2023; 298,386.00; 289,334.74; 0.00. 01/03/2023-31/03/2023; 307,849.32; 298,609.38; 0.00. Xinfeng Shatian swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 42,354.93; 40,787.50; 0.00. 01/05/2022-31/05/2022; 38,601.02; 37,173.24; 0.00. 01/06/2022-30/06/2022; 43,332.63; 41,729.90; 0.00. 01/07/2022-31/07/2022; 41,040.38; 39,522.08; 0.00. 01/08/2022-31/08/2022; 43,546.50; 41,934.97; 0.00. 01/09/2022-30/09/2022; 41,875.68; 40,326.07; 0.00. 01/10/2022-31/10/2022; 42,492.41; 40,920.81; 0.00. 01/11/2022-30/11/2022; 42,431.31; 40,860.19; 0.00. 01/12/2022-31/12/2022; 41,823.94; 40,276.57; 0.00. 01/01/2023-31/01/2023; 43,088.19; 41,493.64; 0.00. 01/02/2023-28/02/2023; 41,846.11; 40,298.03; 0.00. 01/03/2023-31/03/2023; 43,111.10; 41,516.71; 0.00. Kaiping Cangcheng swine farm period; Vf1- outlet of the anaerobic digestion (m3); Vf2- inlet of biogas generator (m3); Vf3- inlet of flare system (m3). 01/04/2022-30/04/2022; 159,137.88; 154,642.07; 0.00. 01/05/2022-31/05/2022; 142,946.46; 138,982.84; 0.00. 01/06/2022-30/06/2022; 159,227.88; 154,895.14; 0.00. 01/07/2022-31/07/2022; 156,150.72; 151,805.97; 0.00. 01/08/2022-31/08/2022; 156,093.42; 151,643.83; 0.00. 01/09/2022-30/09/2022; 154,202.40; 149,890.79; 0.00. 01/10/2022-31/10/2022; 159,866.22; 155,396.29; 0.00. 01/11/2022-30/11/2022; 160,766.46; 156,293.09; 0.00. 01/12/2022-31/12/2022; 152,539.20; 148,263.92; 0.00. 01/01/2023-31/01/2023; 159,252.42; 154,900.22; 0.00. 01/02/2023-28/02/2023; 150,068.16; 145,845.86; 0.00. 01/03/2023-31/03/2023; 157,672.92; 153,122.81; 0.00. period; Vf1- at outlet of the anaerobic digestion (m3); Vf2- at the inlet of biogas generator (m3); Vf3- in inlet of flare system (m3). 01/04/2022-30/04/2022; 1,736,894.41; 1,679,248.87; 0.00. 01/05/2022-31/05/2022; 1,573,057.83; 1,520,595.76; 0.00. 01/06/2022-30/06/2022; 1,750,838.86; 1,692,504.97; 0.00. 01/07/2022-31/07/2022; 1,690,839.47; 1,634,398.45; 0.00. 01/08/2022-31/08/2022; 1,741,607.78; 1,683,844.63; 0.00. 01/09/2022-30/09/2022; 1,687,531.30; 1,631,437.29; 0.00. 01/10/2022-31/10/2022; 1,740,640.71; 1,682,575.57; 0.00. 01/11/2022-30/11/2022; 1,746,180.84; 1,688,161.38; 0.00. 01/12/2022-31/12/2022; 1,684,531.08; 1,628,296.52; 0.00. 01/01/2023-31/01/2023; 1,740,926.79; 1,683,150.20; 0.00. 01/02/2023-28/02/2023; 1,677,999.83; 1,622,041.62; 0.00. 01/03/2023-31/03/2023; 1,737,564.33; 1,679,719.42; 0.00. Total; 20,508,613.23; 19,825,974.68; 0.00. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month. |
| Monitoring frequency | Continuously measured by flowmeter 1, flowmeter 2 and flowmeter 3 |
| QA/QC procedures | Total 27 flow meters were calibrated on 08/12/2020 and 05/12/2022 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | To calculated CH4 generated in the anaerobic digester in the project situation QCH4,y |
| Additional comment | For this monitoring period, all the biogas produced is used for power generation, no surplus biogas for the flaring system. |

| Data / Parameter | EGd,y |
| --- | --- |
| Unit | MWh |
| Description | Electricity generated using biogas in year y |
| Source of data | Electricity daily report |
| Value(s) of monitored parameter | Huicheng Hengli swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 62.58940. 01/05/2022-31/05/2022; 64.64462. 01/06/2022-30/06/2022; 63.37138. 01/07/2022-31/07/2022; 64.49957. 01/08/2022-31/08/2022; 66.20892. 01/09/2022-30/09/2022; 63.27113. 01/10/2022-31/10/2022; 65.03828. 01/11/2022-30/11/2022; 63.83256. 01/12/2022-31/12/2022; 64.94501. 01/01/2023-31/01/2023; 65.80485. 01/02/2023-28/02/2023; 59.09982. 01/03/2023-31/03/2023; 65.99138. Dongyuan Huangtian swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 91.30435. 01/05/2022-31/05/2022; 93.74929. 01/06/2022-30/06/2022; 91.29252. 01/07/2022-31/07/2022; 93.88363. 01/08/2022-31/08/2022; 93.96915. 01/09/2022-30/09/2022; 90.50054. 01/10/2022-31/10/2022; 93.73702. 01/11/2022-30/11/2022; 90.94972. 01/12/2022-31/12/2022; 94.06686. 01/01/2023-31/01/2023; 93.54163. 01/02/2023-28/02/2023; 84.44504. 01/03/2023-31/03/2023; 94.16456. Enping Shahu swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 504.35411. 01/05/2022-31/05/2022; 519.34149. 01/06/2022-30/06/2022; 501.06388. 01/07/2022-31/07/2022; 514.44900. 01/08/2022-31/08/2022; 518.88543. 01/09/2022-30/09/2022; 502.40137. 01/10/2022-31/10/2022; 518.00096. 01/11/2022-30/11/2022; 498.87040. 01/12/2022-31/12/2022; 514.96040. 01/01/2023-31/01/2023; 521.24884. 01/02/2023-28/02/2023; 468.77059. 01/03/2023-31/03/2023; 518.69200. Langtian Yuehui swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 450.01123. 01/05/2022-31/05/2022; 465.29396. 01/06/2022-30/06/2022; 453.07681. 01/07/2022-31/07/2022; 465.28171. 01/08/2022-31/08/2022; 467.17249. 01/09/2022-30/09/2022; 451.85296. 01/10/2022-31/10/2022; 469.82465. 01/11/2022-30/11/2022; 450.02310. 01/12/2022-31/12/2022; 465.14660. 01/01/2023-31/01/2023; 463.63641. 01/02/2023-28/02/2023; 422.19520. 01/03/2023-31/03/2023; 465.52729. Qujiang Fengwan swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 248.51864. 01/05/2022-31/05/2022; 261.86691. 01/06/2022-30/06/2022; 252.17570. 01/07/2022-31/07/2022; 257.56095. 01/08/2022-31/08/2022; 260.14449. 01/09/2022-30/09/2022; 251.67814. 01/10/2022-31/10/2022; 258.17790. 01/11/2022-30/11/2022; 248.60573. 01/12/2022-31/12/2022; 258.12646. 01/01/2023-31/01/2023; 257.23962. 01/02/2023-28/02/2023; 233.77345. 01/03/2023-31/03/2023; 262.75375. Pingyuan Zhongshi swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 525.56907. 01/05/2022-31/05/2022; 542.12192. 01/06/2022-30/06/2022; 520.60448. 01/07/2022-31/07/2022; 540.51184. 01/08/2022-31/08/2022; 541.68891. 01/09/2022-30/09/2022; 518.57352. 01/10/2022-31/10/2022; 534.83769. 01/11/2022-30/11/2022; 520.74417. 01/12/2022-31/12/2022; 540.91163. 01/01/2023-31/01/2023; 533.90492. 01/02/2023-28/02/2023; 485.90750. 01/03/2023-31/03/2023; 545.04235. Yangchun Tanshui swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 466.13115. 01/05/2022-31/05/2022; 477.47726. 01/06/2022-30/06/2022; 463.23028. 01/07/2022-31/07/2022; 482.11334. 01/08/2022-31/08/2022; 477.62971. 01/09/2022-30/09/2022; 464.68072. 01/10/2022-31/10/2022; 479.42068. 01/11/2022-30/11/2022; 463.09507. 01/12/2022-31/12/2022; 477.78214. 01/01/2023-31/01/2023; 478.82369. 01/02/2023-28/02/2023; 433.56430. 01/03/2023-31/03/2023; 481.80856. Xinfeng Shatian swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 63.23502. 01/05/2022-31/05/2022; 67.09644. 01/06/2022-30/06/2022; 64.52204. 01/07/2022-31/07/2022; 65.47230. 01/08/2022-31/08/2022; 65.37816. 01/09/2022-30/09/2022; 64.44232. 01/10/2022-31/10/2022; 66.39028. 01/11/2022-30/11/2022; 64.47648. 01/12/2022-31/12/2022; 66.42558. 01/01/2023-31/01/2023; 65.26048. 01/02/2023-28/02/2023; 59.47644. 01/03/2023-31/03/2023; 66.76691. Kaiping Cangcheng swine farm Period; EGd,y(MWh). 01/04/2022-30/04/2022; 242.84560. 01/05/2022-31/05/2022; 242.75648. 01/06/2022-30/06/2022; 239.81556. 01/07/2022-31/07/2022; 248.62395. 01/08/2022-31/08/2022; 250.02400. 01/09/2022-30/09/2022; 237.22896. 01/10/2022-31/10/2022; 247.66936. 01/11/2022-30/11/2022; 233.38600. 01/12/2022-31/12/2022; 245.21292. 01/01/2023-31/01/2023; 247.49123. 01/02/2023-28/02/2023; 222.31033. 01/03/2023-31/03/2023; 247.63120. Period; EGd,y(MWh). 01/04/2022-30/04/2022; 2,654.56. 01/05/2022-31/05/2022; 2,734.35. 01/06/2022-30/06/2022; 2,649.15. 01/07/2022-31/07/2022; 2,732.40. 01/08/2022-31/08/2022; 2,741.10. 01/09/2022-30/09/2022; 2,644.63. 01/10/2022-31/10/2022; 2,733.10. 01/11/2022-30/11/2022; 2,633.98. 01/12/2022-31/12/2022; 2,727.58. 01/01/2023-31/01/2023; 2,726.95. 01/02/2023-28/02/2023; 2,469.54. 01/03/2023-31/03/2023; 2,748.38. Total; 32,195.72. |
| Measurement methods and procedures | Use electricity meters 2 installed at the outlet of biogas generator. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity generation is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity generation. |
| Monitoring frequency | Continuous measurement and daily recording |
| QA/QC procedures | total 9 meters were calibrated on 10/12/2020 by an officially accredited entity in compliance with JJG596-2012” Electrical Meters for Measuring Alternating-current Electrical Energy” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | N/A |
| Additional comment | And total electricity of 32195.72 MWh is generated by the project during this monitoring period covering 365 days from 01/04/2022 to 31/03/2023, and the installed capacity of biogas generator is 5.42 MW as described in PDD, then generation efficiency of biogas generator is 80% as the technical specification of biogas generator, therefore, he calculated result of load factor for the project in this monitoring period is about 84.76% (32,195.27 MWh/ (365days*24h)/5.42MW/80%). |

| Data/parameter | ECPJ,j,y |
| --- | --- |
| Unit | MWh |
| Description | Quantity of electricity consumed by the project in year y |
| Source of data | Electricity daily report |
| Value(s) of monitored parameter | Langtian Yuehui swine farm Period; ECPJ,j,y (MWh). 01/04/2022-30/04/2022; 2.38321. 01/05/2022-31/05/2022; 2.46415. 01/06/2022-30/06/2022; 2.47943. 01/07/2022-31/07/2022; 2.46408. 01/08/2022-31/08/2022; 2.4741. 01/09/2022-30/09/2022; 2.47273. 01/10/2022-31/10/2022; 2.48814. 01/11/2022-30/11/2022; 2.46272. 01/12/2022-31/12/2022; 2.46337. 01/01/2023-31/01/2023; 2.45537. 01/02/2023-28/02/2023; 2.47546. 01/03/2023-31/03/2023; 2.46539. Yangchun Tanshui swine farm Period; ECPJ,j,y (MWh). 01/04/2022-30/04/2022; 2.14010. 01/05/2022-31/05/2022; 2.19219. 01/06/2022-30/06/2022; 2.19767. 01/07/2022-31/07/2022; 2.21348. 01/08/2022-31/08/2022; 2.19289. 01/09/2022-30/09/2022; 2.20455. 01/10/2022-31/10/2022; 2.20111. 01/11/2022-30/11/2022; 2.19703. 01/12/2022-31/12/2022; 2.19359. 01/01/2023-31/01/2023; 2.19837. 01/02/2023-28/02/2023; 2.20385. 01/03/2023-31/03/2023; 2.21208. Kaiping Cangcheng swine farm Period; ECPJ,j,y (MWh). 01/04/2022-30/04/2022; 1.25766. 01/05/2022-31/05/2022; 1.25720. 01/06/2022-30/06/2022; 1.28337. 01/07/2022-31/07/2022; 1.28759. 01/08/2022-31/08/2022; 1.29484. 01/09/2022-30/09/2022; 1.26953. 01/10/2022-31/10/2022; 1.28264. 01/11/2022-30/11/2022; 1.24896. 01/12/2022-31/12/2022; 1.26992. 01/01/2023-31/01/2023; 1.28172. 01/02/2023-28/02/2023; 1.27467. 01/03/2023-31/03/2023; 1.28245. Period; ECPJ,j,y (MWh). 01/04/2022-30/04/2022; 5.78097. 01/05/2022-31/05/2022; 5.91354. 01/06/2022-30/06/2022; 5.96047. 01/07/2022-31/07/2022; 5.96515. 01/08/2022-31/08/2022; 5.96183. 01/09/2022-30/09/2022; 5.94681. 01/10/2022-31/10/2022; 5.97189. 01/11/2022-30/11/2022; 5.90871. 01/12/2022-31/12/2022; 5.92688. 01/01/2023-31/01/2023; 5.93546. 01/02/2023-28/02/2023; 5.95398. 01/03/2023-31/03/2023; 5.95992. Total; 71.18561. |
| Measurement methods and procedures | Use electricity meters 1 installed in the 9 manure treatment plants. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity consumption is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity consumption. |
| Monitoring frequency | Continuous measurement and daily recording |
| QA/QC procedures | Total 9 meters were calibrated on 10/12/2020 by an officially accredited entity in compliance with JJG596-2012” Electrical Meters for Measuring Alternating-current Electrical Energy” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | The electricity consumption of AWMSs were sourced from the grid company when the biogas generators were not operation. When the biogas generators were not operation, only the anaerobic tanks worked. Therefore, the electricity sourced from the grid company is few as per Electricity daily report. And during this monitoring period, most of the electricity consumed by the project came from the electricity generated by the biogas generated from the anaerobic fermentation process of the project. Only a small amount of the electricity consumption of AWMSs were sourced from the grid company when the biogas generators were not operation. And according to the record of monitoring data, there are only three swine farms (Liangtian Yuehui Swine Farm, Yangchun Tanshui Swine Farm and Kaiping Cangcheng Swine Farm) consumed small amount of electricity sourced from the grid company. The other swine farms only used self-generated electricity during the 2nd monitoring period. The quantity of electricity consumed by the project are crosschecked with Electricity transaction Notes issued by the grid company, and the value is consistent. |

| Data/parameter | TDLj,y |
| --- | --- |
| Unit | % |
| Description | Average technical transmission and distribution losses for providing electricity to source j in year y |
| Source of data | Tool 05 (Version 03.0)” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Value(s) of monitored parameter | 20 |
| Measurement methods and procedures | Default value of 20% is applied, which is conservative according to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Monitoring frequency | The value can be changed once the tool is updated. |
| QA/QC procedures | Default value of 20% is applied, which is the maximum value as per Methodology tool 05 Version 03.0 and conservative |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | Vt,db |
| --- | --- |
| Unit | m³ dry gas/h |
| Description | Volumetric flow of the gaseous stream in time interval t on a dry basis |
| Source of data | Direct monitored by Flowmeters |
| Value(s) of monitored parameter | period; Vt,db (m³ dry gas/h) monitored by flow meter 1; Vt,db (m³ dry gas/h) monitored by flow meter 2; Vt,db (m³ dry gas/h) monitored by flow meter 3. 01/04/2022-30/04/2022; 2,412.35; 2,332.29; 0.00. 01/05/2022-31/05/2022; 2,114.33; 2,043.81; 0.00. 01/06/2022-30/06/2022; 2,431.72; 2,350.70; 0.00. 01/07/2022-31/07/2022; 2,272.63; 2,196.77; 0.00. 01/08/2022-31/08/2022; 2,340.87; 2,263.23; 0.00. 01/09/2022-30/09/2022; 2,343.79; 2,265.89; 0.00. 01/10/2022-31/10/2022; 2,339.57; 2,261.53; 0.00. 01/11/2022-30/11/2022; 2,425.25; 2,344.67; 0.00. 01/12/2022-31/12/2022; 2,264.15; 2,188.57; 0.00. 01/01/2023-31/01/2023; 2,339.96; 2,262.30; 0.00. 01/02/2023-28/02/2023; 2,497.02; 2,413.75; 0.00. 01/03/2023-31/03/2023; 2,335.44; 2,257.69; 0.00. Average; 2,343.09; 2,265.10; 0.00. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 27 flow meters were calibrated on 08/12/2020 and 05/12/2022 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Vi,t,db |
| --- | --- |
| Unit | m³ gas i/m³ dry gas |
| Description | Volumetric fraction of greenhouse gas i in a time interval t on a dry basis |
| Source of data | Monitored by gas analyzers |
| Value(s) of monitored parameter | period; Average CH4 Concentration. 01/04/2022-30/04/2022; 58.26. 01/05/2022-31/05/2022; 61.64. 01/06/2022-30/06/2022; 59.96. 01/07/2022-31/07/2022; 62.28. 01/08/2022-31/08/2022; 61.75. 01/09/2022-30/09/2022; 58.60. 01/10/2022-31/10/2022; 59.51. 01/11/2022-30/11/2022; 61.89. 01/12/2022-31/12/2022; 59.51. 01/01/2023-31/01/2023; 59.57. 01/02/2023-28/02/2023; 59.70. 01/03/2023-31/03/2023; 62.68. Average; 60.45. |
| Measurement methods and procedures | Measured continuously and record hourly by online gas analysers installed at the outlet of the anaerobic digesters and saved automatically in the DCS (Data Collection System). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next. Daily and Monthly CH4 concentration was calculated based on the 24-hour average and daily average CH4 concentration. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | All the gas analysers were calibrated on 05/12/2021 and 02/12/2022 by an officially accredited entity in compliance with JJG693-2011” Verification Regulation of Alarmer Detectors of Combustible Gas” in China. And the zero verification with an inert gas (N2) and one reading verification with a standard gas (single calibration gas) are implemented in the process of calibration, which is in accordance with JJG693-2011. All calibration gases are from Nanjing Changyuan Industrial Gas Co. Ltd, which is a qualified reference materials manufacturer issued by the General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China on 06/09/2017. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Tt |
| --- | --- |
| Unit | K |
| Description | Temperature of the gaseous stream in time interval t |
| Source of data | Measured by instrument and recorded in production record |
| Value(s) of monitored parameter | period; Average Tt (K). 01/04/2022-30/04/2022; 308.90. 01/05/2022-31/05/2022; 309.81. 01/06/2022-30/06/2022; 311.66. 01/07/2022-31/07/2022; 311.37. 01/08/2022-31/08/2022; 308.84. 01/09/2022-30/09/2022; 309.20. 01/10/2022-31/10/2022; 310.22. 01/11/2022-30/11/2022; 311.41. 01/12/2022-31/12/2022; 310.15. 01/01/2023-31/01/2023; 311.46. 01/02/2023-28/02/2023; 310.46. 01/03/2023-31/03/2023; 309.11. Average; 310.22. |
| Measurement methods and procedures | Measured continuously by the biogas flow meters, since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The readout of flowmeter was indicated degrees Celsius (°C). Therefore, the temperature Tt(K) is calculated as the equation T(K)=t(°C) +273.15 The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month. Daily and Monthly temperature was calculated based on the 24-hour average and daily average temperature. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 27 flow meters were calibrated on 08/12/2020 and 05/12/2022 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Pt |
| --- | --- |
| Unit | Pa |
| Description | Pressure of the gaseous stream in time interval t |
| Source of data | Measured by instrument and recorded in production record |
| Value(s) of monitored parameter | period; Average Pt (Pa). 01/04/2022-30/04/2022; 101,325. 01/05/2022-31/05/2022; 101,325. 01/06/2022-30/06/2022; 101,325. 01/07/2022-31/07/2022; 101,325. 01/08/2022-31/08/2022; 101,325. 01/09/2022-30/09/2022; 101,325. 01/10/2022-31/10/2022; 101,325. 01/11/2022-30/11/2022; 101,325. 01/12/2022-31/12/2022; 101,325. 01/01/2023-31/01/2023; 101,325. 01/02/2023-28/02/2023; 101,325. 01/03/2023-31/03/2023; 101,325. Average; 101,325. |
| Measurement methods and procedures | Measured continuously by the biogas flow meters since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month. Daily and Monthly pressure was calculated based on the 24-hour average and daily average pressure. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 27 flow meters were calibrated on 08/12/2020 and 05/12/2022 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | ρi,t |
| --- | --- |
| Unit | kg gas i/m³ gas i |
| Description | Density of greenhouse gas i in the gaseous stream in time interval t |
| Source of data | Calculated |
| Value(s) of monitored parameter | period; ρi,t (kg gas i/m³ gas i). 01/04/2022-30/04/2022; 0.00063. 01/05/2022-31/05/2022; 0.00063. 01/06/2022-30/06/2022; 0.00063. 01/07/2022-31/07/2022; 0.00063. 01/08/2022-31/08/2022; 0.00063. 01/09/2022-30/09/2022; 0.00063. 01/10/2022-31/10/2022; 0.00063. 01/11/2022-30/11/2022; 0.00063. 01/12/2022-31/12/2022; 0.00063. 01/01/2023-31/01/2023; 0.00063. 01/02/2023-28/02/2023; 0.00063. 01/03/2023-31/03/2023; 0.00063. Average; 0.00063. |
| Measurement methods and procedures | Calculated based on temperature of the gaseous stream in time interval t and pressure of the gaseous stream in time interval t. |
| Monitoring frequency | Pressure and temperature are measured continuously by the biogas flow meters |
| QA/QC procedures | Calculated value according to the pressure and temperature |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | MS%j |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of manure handled in system j in project activity |
| Source of data | PDD |
| Value(s) of monitored parameter | 100%. As this parameter is not monitored during this monitoring period. so, to be conservative, the value of this parameter in the emission reduction calculation is 100%, which is consistent with PDD. |
| Measurement methods and procedures | All the manure flew into AWMSs to be treated, therefore, value of 100% is applied. |
| Monitoring frequency | Annually |
| QA/QC procedures | All the manure flew into AWMSs to be treated; no any manure will be discharged outside. Therefore, the value of this parameter is 100%. |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | B0, LT |
| --- | --- |
| Unit | m3CH4/kg -dm |
| Description | Maximum methane producing potential of the volatile solid generated by animal type LT |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | B0, LT (Market swine) =0.29 B0, LT (Breeding swine) =0.29 |
| Measurement methods and procedures | B0, LT can be measured as per ISO 11734:1995. As this parameter is not monitored in the actual operation. so, in the monitoring period 0.29 m3CH4/kg -dm is still applied. While the actual methane producing potential of the volatile solid generated by swine manure is 479.4ml/g VS according to the public literature,which is higher. Therefore 0.29 m3CH4/kg -dm applied in monitoring period is conservative. |
| Monitoring frequency | Annually |
| QA/QC procedures | The value is taken from published sources. The parameter value should be updated on latest available public data source. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | Type |
| --- | --- |
| Unit | / |
| Description | Type of barn and AWMS |
| Source of data | Project proponent |
| Value(s) applied | NA |
| Measurement methods and procedures | The swine barn and AWMS layout and configuration are collected. Archive electronically during project plus 5 years. |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | After the first verification, only changes in the type of barn and AWMS will be reported. |
| Purpose of data | Confirm whether the implementation of project as design |
| Additional comment | Swine barn and AWMS layout and configuration. |

| Data/parameter | T |
| --- | --- |
| Unit | °C |
| Description | Annual average ambient temperature at project site |
| Source of data | Official public information |
| Value(s) applied | Year; T(°C). During the monitoring period (01/04/2022-31/03/2023); 21.9. |
| Measurement methods and procedures | PP entrusted the local meteorological bureau to continuously monitor the temperature of the project site during the monitoring period. The local meteorological bureau had conducted measurements continuously in project site and aggregated in monthly and annual average values as per requirements of the methodology, and finally a Temperature Monitoring Report was issued by local meteorological bureau. As per the Temperature Monitoring Report, during this monitoring period, the average temperature of any month at the project site is greater than 5 ℃ and annual average ambient temperature at project site during this monitoring period is 21.9℃. According to the actual temperature (21.9℃) during this monitoring and the temperature of ex-ante estimated(21.8℃), the mean annual temperature at the project site is higher than 18℃. And the mean annual precipitation is 1771mm, which is higher than 1000mm. Therefore, as per Annex 10A.2, 2019 Refinement to the 2006 IPCC Guidelines, the climate zone of the project site can be determined to "Tropical Moist ", and the MCF value of 80% is applied, then multiplying MCF values with a value of 0.94. So, MCFj value of 75.2% (75.2%=80%*0.94) is applied in this monitoring period. |
| Monitoring frequency | monthly |
| QA/QC procedures | After comparing the actual temperature 21.9℃ and the temperature of ex-ante estimated(21.8℃), it can be determined that the mean annual temperature at the project site is higher than 18℃. |
| Purpose of data | Used to determine the climate zones of the project site and then select the annual MCFj from 2019 Refinement to the 2006 IPCC Guidelines |
| Additional comment | NA |

| Data/parameter | NEXLT,y |
| --- | --- |
| Unit | kg N/animal/year |
| Description | Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2 |
| Source of data | Calculated as the equation of option 2 in Appendix 2: While NEXIPCC default calculated as equation 30 of the IPCC 2006, volume 4, chapter 10, i.e., NEXIPCC default =Nrate(T)* TAM/1000*365 |
| Value(s) applied | period; NEXLT,y. Market swine; breeding swine. 01/04/2022-30/04/2022; 0.72; 0.54. 01/05/2022-31/05/2022; 0.74; 0.55. 01/06/2022-30/06/2022; 0.72; 0.54. 01/07/2022-31/07/2022; 0.74; 0.55. 01/08/2022-31/08/2022; 0.75; 0.56. 01/09/2022-30/09/2022; 0.74; 0.53. 01/10/2022-31/10/2022; 0.76; 0.55. 01/11/2022-30/11/2022; 0.73; 0.54. 01/12/2022-31/12/2022; 0.76; 0.56. 01/01/2023-31/01/2023; 0.75; 0.55. 01/02/2023-28/02/2023; 0.66; 0.50. 01/03/2023-31/03/2023; 0.75; 0.55. |
| Measurement methods and procedures | Nrate(T) and TAM are default value from IPCC 2006 |
| Monitoring frequency | Annually |
| QA/QC procedures | The parameter is calculated as the equation of option 2 in Appendix 2 and the value of other parameter applied in the equation is taken from published sources (2006 IPCC). The value should be updated on latest available public data source. |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

SDG 8

| Data / Parameter | Total number of jobs |
| --- | --- |
| Unit | Number of full-time jobs created |
| Description | Full-time Jobs created for both men and women. |
| Source of data | Record keeping book |
| Value(s) of monitored parameter | 18 full-time jobs are created including 9 males and 9 females |
| Measurement methods and procedures | Source of data was record keeping book and was cross checked by the labor contracts. |
| Monitoring frequency | annually. |
| QA/QC procedures | After the first verification, only changes in employees will be reported. The results shall be cross checked with labor contract. According to the recording keeping book, both men and women have the same salary, which can be cross checked through labor contracts. |
| Purpose of data | To demonstrate contribution to SDG 8. |
| Additional comment | N/A |

SDG 7

| Data / Parameter | Annual amount of biogas utilized |
| --- | --- |
| Unit | m3 |
| Description | The amount of biogas utilization by biogas anaerobic digestion |
| Source of data | Direct measurement by the flowmeters 2 |
| Value(s) of monitored parameter | Period; Vf2- at the inlet of biogas generator (m3). 01/04/2022-30/04/2022; 1,679,248.87. 01/05/2022-31/05/2022; 1,520,595.76. 01/06/2022-30/06/2022; 1,692,504.97. 01/07/2022-31/07/2022; 1,634,398.45. 01/08/2022-31/08/2022; 1,683,844.63. 01/09/2022-30/09/2022; 1,631,437.29. 01/10/2022-31/10/2022; 1,682,575.57. 01/11/2022-30/11/2022; 1,688,161.38. 01/12/2022-31/12/2022; 1,628,296.52. 01/01/2023-31/01/2023; 1,683,150.20. 01/02/2023-28/02/2023; 1,622,041.62. 01/03/2023-31/03/2023; 1,679,719.42. Total; 19,825,974.68. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month. Daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 9 flow meters used to monitor this parameter and were calibrated on 08/12/2020 and 05/12/2022 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | To demonstrate contribution to SDG 7. |
| Additional comment | N/A |

Mitigation Measure for Safeguarding Principles

| Data / Parameter | Employee Training of biogas safety operation |
| --- | --- |
| Unit | / |
| Description | Employees was trained on the safety operation of the biogas |
| Source of data | Training notice |
| Value(s) of monitored parameter | Training for the biogas safety operation was held on 07/11/2022 |
| Measurement methods and procedures | Training once a year The training content includes basic biogas knowledge, Operating condition control, operating procedure, patrol inspection, Operation management and failure process. All 18 staffs from 9 swine farms attended this training. All staff strictly follow the operation procedures in the process of operation and there are no safety accidents occurred during this monitoring period. Therefore, the training is effective. |
| Monitoring frequency | once a year |
| QA/QC procedures | Meeting attendance record was used to cross-checked. |
| Purpose of data | To demonstrate Mitigation Measure for Safeguarding Principles |
| Additional comment | N/A |

Comparison of monitored parameters with last monitoring period

There are 22 parameters that need to be monitored, the monitoring results in this monitoring period and last monitoring period are as follows:

| Data/Parameter | Value obtained in1st monitoring period | Value obtained in 2nd monitoring period |
| --- | --- | --- |
| Np,LT | Period; Np,LT; Period; Np,LT. 20/12/2020-31/12/2020; 8,896; 01/08/2021-31/08/2021; 22,294. 01/01/2021-31/01/2021; 22,289; 01/09/2021-30/09/2021; 22,307. 01/02/2021-28/02/2021; 22,295; 01/10/2021-31/10/2021; 22,291. 01/03/2021-31/03/2021; 22,302; 01/11/2021-30/11/2021; 22,313. 01/04/2021-30/04/2021; 22,281; 01/12/2021-31/12/2021; 22,280. 01/05/2021-31/05/2021; 22,325; 01/01/2022-31/01/2022; 22,251. 01/06/2021-30/06/2021; 22,346; 01/02/2022-28/02/2022; 22,311. 01/07/2021-31/07/2021; 22,316; 01/03/2022-31/03/2022; 22,336. Total; 343,427. | Period; Np,LT. 01/04/2022-30/04/2022; 21,420. 01/05/2022-31/05/2022; 21,604. 01/06/2022-30/06/2022; 21,463. 01/07/2022-31/07/2022; 21,477. 01/08/2022-31/08/2022; 21,629. 01/09/2022-30/09/2022; 21,580. 01/10/2022-31/10/2022; 21,490. 01/11/2022-30/11/2022; 21,587. 01/12/2022-31/12/2022; 21,536. 01/01/2023-31/01/2023; 21,504. 01/02/2023-28/02/2023; 21,629. 01/03/2023-31/03/2023; 21,524. Total; 258,443. |
| Nda,LT | Period; Nda,LT; Period; Nda,LT. 20/12/2020-31/12/2020; 180; 01/08/2021-31/08/2021; 180. 01/01/2021-31/01/2021; 180; 01/09/2021-30/09/2021; 180. 01/02/2021-28/02/2021; 180; 01/10/2021-31/10/2021; 180. 01/03/2021-31/03/2021; 180; 01/11/2021-30/11/2021; 180. 01/04/2021-30/04/2021; 180; 01/12/2021-31/12/2021; 180. 01/05/2021-31/05/2021; 180; 01/01/2022-31/01/2022; 180. 01/06/2021-30/06/2021; 180; 01/02/2022-28/02/2022; 180. 01/07/2021-31/07/2021; 180; 01/03/2022-31/03/2022; 180. | Period; Nda,LT. 01/04/2022-30/04/2022; 180. 01/05/2022-31/05/2022; 180. 01/06/2022-30/06/2022; 180. 01/07/2022-31/07/2022; 180. 01/08/2022-31/08/2022; 180. 01/09/2022-30/09/2022; 180. 01/10/2022-31/10/2022; 180. 01/11/2022-30/11/2022; 180. 01/12/2022-31/12/2022; 180. 01/01/2023-31/01/2023; 180. 01/02/2023-28/02/2023; 180. 01/03/2023-31/03/2023; 180. |
| NAA,LT | Period; NAA,LT; Period; NAA,LT. 20/12/2020-31/12/2020; 95,993; 01/08/2021-31/08/2021; 95,959. 01/01/2021-31/01/2021; 95,900; 01/09/2021-30/09/2021; 95,861. 01/02/2021-28/02/2021; 95,947; 01/10/2021-31/10/2021; 95,835. 01/03/2021-31/03/2021; 95,778; 01/11/2021-30/11/2021; 95,945. 01/04/2021-30/04/2021; 95,748; 01/12/2021-31/12/2021; 95,942. 01/05/2021-31/05/2021; 95,839; 01/01/2022-31/01/2022; 95,905. 01/06/2021-30/06/2021; 95,920; 01/02/2022-28/02/2022; 95,944. 01/07/2021-31/07/2021; 95,933; 01/03/2022-31/03/2022; 95,875. | Period; NAA,LT. 01/04/2022-30/04/2022; 94,180. 01/05/2022-31/05/2022; 93,502. 01/06/2022-30/06/2022; 93,732. 01/07/2022-31/07/2022; 93,352. 01/08/2022-31/08/2022; 94,055. 01/09/2022-30/09/2022; 93,324. 01/10/2022-31/10/2022; 93,347. 01/11/2022-30/11/2022; 92,517. 01/12/2022-31/12/2022; 92,988. 01/01/2023-31/01/2023; 92,822. 01/02/2023-28/02/2023; 93,454. 01/03/2023-31/03/2023; 94,666. Average; 93,495. |
| Wsite | Period; Market swine; Breeding swine. 20/12/2020-31/12/2020; 61.4; 74.6. 01/01/2021-31/01/2021; 61.7; 74.6. 01/02/2021-28/02/2021; 61.2; 72.9. 01/03/2021-31/03/2021; 60.4; 73.1. 01/04/2021-30/04/2021; 60.6; 73.9. 01/05/2021-31/05/2021; 60.9; 74.3. 01/06/2021-30/06/2021; 59.7; 73.6. 01/07/2021-31/07/2021; 61.3; 73.8. 01/08/2021-31/08/2021; 61.6; 73.4. 01/09/2021-30/09/2021; 59.6; 73.6. 01/10/2021-31/10/2021; 61.8; 74.2. 01/11/2021-30/11/2021; 60.5; 74.4. 01/12/2021-31/12/2021; 60.5; 74.6. 01/01/2022-31/01/2022; 61.8; 74.3. 01/02/2022-28/02/2022; 60.7; 75.2. 01/03/2022-31/03/2022; 60.4; 74.5. Average; 60.9; 74.1. | Period; Market swine; Breeding swine. 01/04/2022-30/04/2022; 57.3; 74.5. 01/05/2022-31/05/2022; 56.9; 74.3. 01/06/2022-30/06/2022; 57.5; 74.7. 01/07/2022-31/07/2022; 57.2; 73.8. 01/08/2022-31/08/2022; 57.6; 74.8. 01/09/2022-30/09/2022; 58.4; 73.6. 01/10/2022-31/10/2022; 58.6; 73.7. 01/11/2022-30/11/2022; 57.6; 74.5. 01/12/2022-31/12/2022; 58.0; 75.2. 01/01/2023-31/01/2023; 57.8; 73.6. 01/02/2023-28/02/2023; 56.3; 74.0. 01/03/2023-31/03/2023; 57.9; 74.0. Average; 57.6; 74.2. |
| ndy | Period; ndy; Period; ndy. 20/12/2020-31/12/2020; 12; 01/08/2021-31/08/2021; 31. 01/01/2021-31/01/2021; 31; 01/09/2021-30/09/2021; 30. 01/02/2021-28/02/2021; 28; 01/10/2021-31/10/2021; 31. 01/03/2021-31/03/2021; 31; 01/11/2021-30/11/2021; 30. 01/04/2021-30/04/2021; 30; 01/12/2021-31/12/2021; 31. 01/05/2021-31/05/2021; 31; 01/01/2022-31/01/2022; 31. 01/06/2021-30/06/2021; 30; 01/02/2022-28/02/2022; 28. 01/07/2021-31/07/2021; 31; 01/03/2022-31/03/2022; 31. Total; 467. | Period; ndy. 01/04/2022-30/04/2022; 30. 01/05/2022-31/05/2022; 31. 01/06/2022-30/06/2022; 30. 01/07/2022-31/07/2022; 31. 01/08/2022-31/08/2022; 31. 01/09/2022-30/09/2022; 30. 01/10/2022-31/10/2022; 31. 01/11/2022-30/11/2022; 30. 01/12/2022-31/12/2022; 31. 01/01/2023-31/01/2023; 31. 01/02/2023-28/02/2023; 28. 01/03/2023-31/03/2023; 31. Average; 365. |
| ECPJ,j,y | period; ECPJ,j,y (MWh). 20/12/2020-31/12/2020; 2.1131. 01/01/2021-31/01/2021; 5.4611. 01/02/2021-28/02/2021; 5.4583. 01/03/2021-31/03/2021; 5.4543. 01/04/2021-30/04/2021; 5.4563. 01/05/2021-31/05/2021; 5.4543. 01/06/2021-30/06/2021; 5.4568. 01/07/2021-31/07/2021; 5.4557. 01/08/2021-31/08/2021; 5.4584. 01/09/2021-30/09/2021; 5.4534. 01/10/2021-31/10/2021; 5.4551. 01/11/2021-30/11/2021; 5.4601. 01/12/2021-31/12/2021; 5.4601. 01/01/2022-31/01/2022; 5.4565. 01/02/2022-28/02/2022; 5.4550. 01/03/2022-31/03/2022; 5.4592. Total; 83.9677. | period; ECPJ,j,y (MWh). 01/04/2022-30/04/2022; 5.78097. 01/05/2022-31/05/2022; 5.91354. 01/06/2022-30/06/2022; 5.96047. 01/07/2022-31/07/2022; 5.96515. 01/08/2022-31/08/2022; 5.96183. 01/09/2022-30/09/2022; 5.94681. 01/10/2022-31/10/2022; 5.97189. 01/11/2022-30/11/2022; 5.90871. 01/12/2022-31/12/2022; 5.92688. 01/01/2023-31/01/2023; 5.93546. 01/02/2023-28/02/2023; 5.95398. 01/03/2023-31/03/2023; 5.95992. Total; 71.18561. |
| TDLj,y | 20% | 20% |
| FAer | 100% | 100% |
| MS%j | 100% | 100% |
| vf | period; Vf1- at outlet of the anaerobic digestion (m3); Vf2- at the inlet of biogas generator (m3); Vf3- in inlet of flare system (m3). 20/12/2020-31/12/2020; 696,005.63; 674,296.55; 0.00. 01/01/2021-31/01/2021; 1,797,291.41; 1,741,035.15; 0.00. 01/02/2021-28/02/2021; 1,623,700.93; 1,572,845.26; 0.00. 01/03/2021-31/03/2021; 1,796,266.46; 1,740,195.40; 0.00. 01/04/2021-30/04/2021; 1,738,115.02; 1,684,075.99; 0.00. 01/05/2021-31/05/2021; 1,796,803.61; 1,740,315.41; 0.00. 01/06/2021-30/06/2021; 1,739,429.21; 1,685,218.14; 0.00. 01/07/2021-31/07/2021; 1,797,531.32; 1,741,282.49; 0.00. 01/08/2021-31/08/2021; 1,797,753.74; 1,741,515.49; 0.00. 01/09/2021-30/09/2021; 1,738,971.70; 1,684,592.37; 0.00. 01/10/2021-31/10/2021; 1,796,744.25; 1,740,170.89; 0.00. 01/11/2021-30/11/2021; 1,739,656.38; 1,685,442.06; 0.00. 01/12/2021-31/12/2021; 1,797,595.87; 1,741,539.35; 0.00. 01/01/2022-31/01/2022; 1,797,289.15; 1,741,517.97; 0.00. 01/02/2022-28/02/2022; 1,623,639.55; 1,572,761.93; 0.00. 01/03/2022-31/03/2022; 1,797,067.50; 1,740,642.92; 0.00. Total; 27,073,861.73; 26,227,447.37; 0.00. | period; Vt,db (m³ dry gas/h) monitored by flow meter 1; Vt,db (m³ dry gas/h) monitored by flow meter 2; Vt,db (m³ dry gas/h) monitored by flow meter 3. 01/04/2022-30/04/2022; 1,736,894.41; 1,679,248.87; 0.00. 01/05/2022-31/05/2022; 1,573,057.83; 1,520,595.76; 0.00. 01/06/2022-30/06/2022; 1,750,838.86; 1,692,504.97; 0.00. 01/07/2022-31/07/2022; 1,690,839.47; 1,634,398.45; 0.00. 01/08/2022-31/08/2022; 1,741,607.78; 1,683,844.63; 0.00. 01/09/2022-30/09/2022; 1,687,531.30; 1,631,437.29; 0.00. 01/10/2022-31/10/2022; 1,740,640.71; 1,682,575.57; 0.00. 01/11/2022-30/11/2022; 1,746,180.84; 1,688,161.38; 0.00. 01/12/2022-31/12/2022; 1,684,531.08; 1,628,296.52; 0.00. 01/01/2023-31/01/2023; 1,740,926.79; 1,683,150.20; 0.00. 01/02/2023-28/02/2023; 1,677,999.83; 1,622,041.62; 0.00. 01/03/2023-31/03/2023; 1,737,564.33; 1,679,719.42; 0.00. Average; 20,508,613.23; 19,825,974.68; 0.00. |
| Vt,db | period; Vt,db (m³ dry gas/h) monitored by flow meter 1; Vt,db (m³ dry gas/h) monitored by flow meter 2; Vt,db (m³ dry gas/h) monitored by flow meter 3. 20/12/2020-31/12/2020; 2,416.69; 2,341.31; 0.00. 01/01/2021-31/01/2021; 2,415.71; 2,340.10; 0.00. 01/02/2021-28/02/2021; 2,416.22; 2,340.54; 0.00. 01/03/2021-31/03/2021; 2,414.34; 2,338.97; 0.00. 01/04/2021-30/04/2021; 2,414.05; 2,338.99; 0.00. 01/05/2021-31/05/2021; 2,415.06; 2,339.13; 0.00. 01/06/2021-30/06/2021; 2,415.87; 2,340.58; 0.00. 01/07/2021-31/07/2021; 2,416.04; 2,340.43; 0.00. 01/08/2021-31/08/2021; 2,416.34; 2,340.75; 0.00. 01/09/2021-30/09/2021; 2,415.24; 2,339.71; 0.00. 01/10/2021-31/10/2021; 2,414.98; 2,338.94; 0.00. 01/11/2021-30/11/2021; 2,416.19; 2,340.89; 0.00. 01/12/2021-31/12/2021; 2,416.12; 2,340.78; 0.00. 01/01/2022-31/01/2022; 2,415.71; 2,340.75; 0.00. 01/02/2022-28/02/2022; 2,416.13; 2,340.42; 0.00. 01/03/2022-31/03/2022; 2,415.41; 2,339.57; 0.00. Average; 2,415.63; 2,340.12; 0.00. | period; Vt,db (m³ dry gas/h) monitored by flow meter 1; Vt,db (m³ dry gas/h) monitored by flow meter 2; Vt,db (m³ dry gas/h) monitored by flow meter 3. 01/04/2022-30/04/2022; 2,412.35; 2,332.29; 0.00. 01/05/2022-31/05/2022; 2,114.33; 2,043.81; 0.00. 01/06/2022-30/06/2022; 2,431.72; 2,350.70; 0.00. 01/07/2022-31/07/2022; 2,272.63; 2,196.77; 0.00. 01/08/2022-31/08/2022; 2,340.87; 2,263.23; 0.00. 01/09/2022-30/09/2022; 2,343.79; 2,265.89; 0.00. 01/10/2022-31/10/2022; 2,339.57; 2,261.53; 0.00. 01/11/2022-30/11/2022; 2,425.25; 2,344.67; 0.00. 01/12/2022-31/12/2022; 2,264.15; 2,188.57; 0.00. 01/01/2023-31/01/2023; 2,339.96; 2,262.30; 0.00. 01/02/2023-28/02/2023; 2,497.02; 2,413.75; 0.00. 01/03/2023-31/03/2023; 2,335.44; 2,257.69; 0.00. Average; 2,343.09; 2,265.10; 0.00. |
| Vi,t,db | period; Average CH4 Concentration. 20/12/2020-31/12/2020; 61.34. 01/01/2021-31/01/2021; 60.89. 01/02/1021-28/02/2021; 60.99. 01/03/2021-31/03/3021; 60.44. 01/04/2021-30/04/2021; 60.80. 01/05/2021-31/05/2021; 60.36. 01/06/2021-30/06/2021; 61.18. 01/07/2021-31/07/2021; 61.10. 01/08/2021-31/08/2021; 61.11. 01/09/2021-30/09/2021; 60.53. 01/10/2021-31/10/2021; 59.78. 01/11/2021-30/11/2021; 60.96. 01/12/2021-31/12/2021; 59.99. 01/01/2022-31/01/2022; 61.26. 01/02/2022-28/02/2022; 60.17. 01/03/2022-31/03/2022; 60.32. Average; 60.70. | period; Average CH4 Concentration. 01/04/2022-30/04/2022; 58.26. 01/05/2022-31/05/2022; 61.64. 01/06/2022-30/06/2022; 59.96. 01/07/2022-31/07/2022; 62.28. 01/08/2022-31/08/2022; 61.75. 01/09/2022-30/09/2022; 58.60. 01/10/2022-31/10/2022; 59.51. 01/11/2022-30/11/2022; 61.89. 01/12/2022-31/12/2022; 59.51. 01/01/2023-31/01/2023; 59.57. 01/02/2023-28/02/2023; 59.70. 01/03/2023-31/03/2023; 62.68. Average; 60.45. |
| Tt | period; Average Tt (K). 20/12/2020-31/12/2020; 309.88. 01/01/2021-31/01/2021; 309.61. 01/02/1021-28/02/2021; 309.46. 01/03/2021-31/03/3021; 309.48. 01/04/2021-30/04/2021; 310.03. 01/05/2021-31/05/2021; 309.56. 01/06/2021-30/06/2021; 310.18. 01/07/2021-31/07/2021; 309.29. 01/08/2021-31/08/2021; 309.23. 01/09/2021-30/09/2021; 309.74. 01/10/2021-31/10/2021; 309.32. 01/11/2021-30/11/2021; 309.61. 01/12/2021-31/12/2021; 309.50. 01/01/2022-31/01/2022; 309.97. 01/02/2022-28/02/2022; 309.81. 01/03/2022-31/03/2022; 309.55. Average; 309.64. | period; Average Tt (K). 01/04/2022-30/04/2022; 308.90. 01/05/2022-31/05/2022; 309.81. 01/06/2022-30/06/2022; 311.66. 01/07/2022-31/07/2022; 311.37. 01/08/2022-31/08/2022; 308.84. 01/09/2022-30/09/2022; 309.20. 01/10/2022-31/10/2022; 310.22. 01/11/2022-30/11/2022; 311.41. 01/12/2022-31/12/2022; 310.15. 01/01/2023-31/01/2023; 311.46. 01/02/2023-28/02/2023; 310.46. 01/03/2023-31/03/2023; 309.11. Average; 310.22. |
| Pt | 101.325K | 101.325K |
| ρi,t | 0.00063 t/m3 | 0.00063 t/m3 |
| B0, LT | B0, LT (Market swine) =0.29 B0, LT (Breeding swine) =0.29 | B0, LT (Market swine) =0.29 B0, LT (Breeding swine) =0.29 |
| Type | N/A. T The type of barn and AWMS was not changed as the design during 1st MP. | N/A. The type of barn and AWMS was not changed as the design during 2nd MP. |
| T | Year; T(°C). 2020; 22.8. 2021; 23.0. 2022; NA. | Year; T(°C). 2022; 22.2. 2023; NA. |
| Total numbers of jobs | 18 full-time jobs created including 9 females and 9 males | 18 full-time jobs created including 9 females and 9 males |
| EGd,y | period; EGd,y(MWh). 20/12/2020-31/12/2020; 1,095.11. 01/01/2021-31/01/2021; 2,827.91. 01/02/2021-28/02/2021; 2,554.84. 01/03/2021-31/03/2021; 2,826.23. 01/04/2021-30/04/2021; 2,734.77. 01/05/2021-31/05/2021; 2,827.21. 01/06/2021-30/06/2021; 2,736.90. 01/07/2021-31/07/2021; 2,828.33. 01/08/2021-31/08/2021; 2,828.71. 01/09/2021-30/09/2021; 2,736.21. 01/10/2021-31/10/2021; 2,827.09. 01/11/2021-30/11/2021; 2,737.30. 01/12/2021-31/12/2021; 2,828.35. 01/01/2022-31/01/2022; 2,827.87. 01/02/2022-28/02/2022; 2,554.73. 01/03/2022-31/03/2022; 2,827.49. Total; 42,599.08. | period; EGd,y(MWh). 01/04/2022-30/04/2022; 2,654.56. 01/05/2022-31/05/2022; 2,734.35. 01/06/2022-30/06/2022; 2,649.15. 01/07/2022-31/07/2022; 2,732.40. 01/08/2022-31/08/2022; 2,741.10. 01/09/2022-30/09/2022; 2,644.63. 01/10/2022-31/10/2022; 2,733.10. 01/11/2022-30/11/2022; 2,633.98. 01/12/2022-31/12/2022; 2,727.58. 01/01/2023-31/01/2023; 2,726.95. 01/02/2023-28/02/2023; 2,469.54. 01/03/2023-31/03/2023; 2,748.38. Total; 32,195.72. |
| Employee Training of biogas safety operation | Training for the biogas safety operation was held on 16/11/2020 and 12/11/2021 | Training for the biogas safety operation was held on 07/11/2022. |

Comparing to the previous monitoring period, the number of market swine and breeding swine is slightly lower, which causes the decreasing of biogas and electricity generation. The values of other parameters were the same or remained basically stable compared to the first monitoring period.

Implementation of sampling plan

>>

According to the PDD, the sample size is calculated according to the design scale and fixed during the monitoring period. During this monitoring period, the sampling is implemented according to the stratified sampling method for the monitoring of Wsite described in Section C of MR. The detailed steps are as following:

- the overall sample size based on the design population of swine firstly is calculated.

- As the project involved 9 swine farms, then the sample size in each swine farm is adjusted based on the proportion of the number of each farm in the total number of 9 swine farms.

- Similarly, the sample size of each age group of Market swine and Breeding swine in each farm is also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm.

- After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample is conducted in every swine farm. Since swine in different age are kept in the different swine houses, samples can be randomly selected from swine houses of this age group.

- After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table.

- The sample will be marked after weighting and if the sample swine is still drawn next month, then this sample swine needs to be discarded and a new sample swine is drawn, this will be done every month until the end of the monitoring period.

Step 1. overall sample size calculation

According to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, the desired confidence level is 95% and the desired precision is 10%. According to statistical principles and sampling survey method, 422 samples should be sufficient to satisfy the desired confidence and precision. For mean value sampling, the required sampling quantity is calculated by the following steps:

n2= n1N/(N+n1)

n3=Bn2

n4= n3/r

n= n4 (110%)

Where:

Z    the Z-statistic and the value taken is 1.96 (corresponding to 90% confidence level);

σ   the standard deviation;

d   the maximum error of estimate;

X   the mean value of samples;

V   the coefficient of variation, V= σ/X≤1 and its maximum value 1 is taken;

e   the allowed relative sampling error (“precision”) and the value taken is 10%

N   the total number of swine population 237,270;

B   the survey design effect and the B value taken is 1 because for stratification sampling the B value is less than or equal to 1;

r    the survey reply rate and the value taken is 100%;

110% 10% contingency was added to produce the final sample size;

n1, n2, n3, n4, n  the adjusted values after each step with n being the final number of samples to be taken. Actually, n4 is the required number of samples based on calculation, 10% contingency was added to n4 to produce n in this sampling.

| Z | V | e | N | B | r |
| --- | --- | --- | --- | --- | --- |
| 1.96 | 1 | 10% | 237,270 | 1 | 100% |
| n1 | Z2V2/e2 | 384 |  |  |  |
| n2 | n1N/(N+n1) | 384 |  |  |  |
| n3 | Bn2 | 384 |  |  |  |
| n4 | n3/r | 384 |  |  |  |
| n | n4 (110%) | 422 |  |  |  |

Step 2 adjust sample size based on the proportion of the number of each farm in the total number of 9 farms

Stratified sampling (with samples of 422) was used for the monitoring of Wsite. However, the samples number in the 9 subsidiary farms is decimals according to the proportion of the number of each farm in the total number of 9 farms. To be conservative, both the samples number of the 9 subsidiary farms is adjusted to be integer value. As a result, 430 samples of swine population were selected within the project boundary using Excel to ensure conservation.

The distribution of the samples in each swine farm shows as follow:

| Swine farm | Sample size |  |
| --- | --- | --- |
|  | Market swine | Breeding swine |
| Huicheng Hengli swine Farm（惠城横沥） | 5 | 8 |
| Dongyuan Huangtian swine Farm（东源黄田） | 13 | 3 |
| Enping Shahu swine Farm（恩平沙湖） | 44 | 29 |
| Langtian Yuehui swine Farm（廊田粤辉） | 56 | 19 |
| Qujiang Fengwan swine Farm（曲江枫湾） | 19 | 21 |
| Pingyuan Zhongshi swine Farm（平远仲石） | 50 | 43 |
| Yangchun Tanshui swine Farm（阳春潭水） | 51 | 21 |
| Xinfeng Shatian swine Farm（新丰沙田） | 4 | 7 |
| Kaiping Cangcheng swine Farm（开平苍城） | 11 | 26 |
| Total | 253 | 177 |

Step 3 Determine sample size based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in two kinds of farms.

According to the methodology ACM0010, each defined livestock population should be classified into a minimum of three age categories. Nursery phase, Growing phase and Mature phase in fattening farms and breeding farms was be selected through the proportion of swine population in three age categories during this monitoring period. At the same time, to be conservative, both the samples number of the three age categories is adjusted to be integer value.

In conclusion, 451 samples of swine population are fixed and the distribution of the samples for three age categories in each swine farm shows as follow:

| Swine farm | Market swine |  |  | Breeding swine |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Nursery phase | Growing phase | Mature phase | Nursery phase | Growing phase | Mature phase |
| Huicheng Hengli swine Farm | 2 | 2 | 2 | 3 | 3 | 3 |
| Dongyuan Huangtian swine Farm | 5 | 5 | 5 | 1 | 1 | 1 |
| Enping Shahu swine Farm | 15 | 15 | 15 | 10 | 10 | 10 |
| Langtian Yuehui swine Farm | 19 | 19 | 19 | 7 | 7 | 7 |
| Qujiang Fengwan swine Farm | 7 | 7 | 7 | 7 | 7 | 7 |
| Pingyuan Zhongshi swine Farm | 17 | 17 | 17 | 15 | 15 | 15 |
| Yangchun Tanshui swine Farm | 17 | 17 | 18 | 7 | 7 | 7 |
| Xinfeng Shatian swine Farm | 2 | 2 | 2 | 3 | 3 | 3 |
| Kaiping Cangcheng swine Farm | 4 | 4 | 4 | 9 | 9 | 9 |
| Total |  |  |  |  |  | 451 |

Step4. sampling and measure

After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample and measure are conducted in every swine farm. Since swine in different age are kept in the different pig houses, samples can be randomly selected from pig houses of this age group. After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table. In the processing of weighting, the sample will be marked after weighting to avoid the same pig be selected.  This was done every month until the end of the monitoring period.

For this monitoring period, then samples are selected randomly from pig houses of this age group by the technicians in the swine farms and then be measured by weight measures in the end of each month. At the same time, the measured value is recorded in the weight record table by the technicians in the swine farms.

At last, the monthly average Wsite of market swine and breeding swine was calculated by weighted the Wsite of all market swine sample and breeding swine sample.

The Monitoring Team implemented the sampling investigation strictly follow the guidance. The monitoring data will be collected throughout the entire crediting period of the project.

According to Standard: Sampling and surveys of CDM project activities and programmes of activities (Version 09.0), confidence/precision have been calculated and checked as follows:

1. The stratified estimated overall mean

(1)

The sample estimate of the overall mean operation hour is calculated with the equation below:

mStrat=i=akgiN×mi

Where:

mStrat  The stratified estimated overall mean

gi  Size of the ith district where i=a,…,k

N  Population total

mi  Mean of the ith district where i=a,…,k

2. The standard error of the stratified estimated overall mean

(2)

The standard error of the stratified estimated overall mean is calculated as follows:

s.e.mStrat=i=ak(giN)2×(1-nigi)×SDi2ni

Where:

s.e.(mStrat)  Standard error of the stratified estimated overall mean

gi  Size of the ith district where i=a,…,k

N  Population total

ni  Number of sampled units the ith district where i=a,…,k

SDi2  Variance of the ith district where i=a,…,k

3. t-value

t-value is depending on:

(i) The level of confidence, and

(ii) The size of the sample.

The t-value associated with 95% confidence and the sample size of 451 is 1.9652 as derived in Microsoft Excel using the TINV function.

4. Precision

The precision associated with an estimate is:

t-value × standard error of the mean

5. Calculation results

For Wsite, according to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, the desired confidence level is 95% and the desired precision is 10%. According to statistical principles and sampling survey method, 451 samples should be sufficient to satisfy the desired confidence and precision.

The calculation results are shown below. For more details of these calculations, please refer to the ER calculation spread sheet.

| time | t-value | mStrat | s.e. (mStrat) | Precision | Relative Precision |
| --- | --- | --- | --- | --- | --- |
| 01/04/2022-30/04/2022 | 1.9652 | 64.42 | 1.4055 | 2.7621 | 4.29% |
| 01/05/2022-31/05/2022 | 1.9652 | 64.11 | 1.4271 | 2.8045 | 4.37% |
| 01/06/2022-30/06/2022 | 1.9652 | 64.48 | 1.3985 | 2.7483 | 4.26% |
| 01/07/2022-31/07/2022 | 1.9652 | 64.14 | 1.4089 | 2.7687 | 4.32% |
| 01/08/2022-31/08/2022 | 1.9652 | 64.91 | 1.3855 | 2.7228 | 4.19% |
| 01/09/2022-30/09/2022 | 1.9652 | 64.56 | 1.3796 | 2.7113 | 4.20% |
| 01/10/2022-31/10/2022 | 1.9652 | 64.67 | 1.3931 | 2.7378 | 4.23% |
| 01/11/2022-30/11/2022 | 1.9652 | 64.73 | 1.3931 | 2.7378 | 4.23% |
| 01/12/2022-31/12/2022 | 1.9652 | 64.92 | 1.4017 | 2.7548 | 4.24% |
| 01/01/2023-31/01/2023 | 1.9652 | 64.26 | 1.4110 | 2.7729 | 4.31% |
| 01/02/2023-28/02/2023 | 1.9652 | 63.82 | 1.3897 | 2.7312 | 4.28% |
| 01/03/2023-31/03/2023 | 1.9652 | 64.50 | 1.3934 | 2.7384 | 4.25% |

In conclusion, the 451 samples are sufficient to satisfy the desired confidence and precision.

- CALCULATION OF SDG IMPACTS

Calculation of baseline value or estimation of baseline situation of each SDG Impact

>>

SDG 7 Impact of baseline situation

From 01/04/2022 to 31/12/2022, 0 m3 biogas can be utilized without the project activity.

From 01/01/2023 to 31/03/2023, 0 m3 biogas can be utilized without the project activity.

In baseline scenario, 0 m3 biogas can be utilized during the 2nd monitoring period from 01/04/2022 to 31/03/2023.

SDG 8 Impact of baseline situation

From 01/04/2022 to 31/12/2022, 0 full-time jobs created in the baseline situation.

From 01/01/2023 to 31/03/2023, 0 full-time jobs created in the baseline situation.

Without the project activity, 0 full-time jobs created during the 2nd monitoring period from 01/04/2022 to 31/03/2023.

SDG 13 Impact of baseline situation

From 01/04/2022 to 31/12/2022, the amount of GHGs emission avoided or sequestered in baseline is 0 tCO2e.

From 01/01/2023 to 31/03/2023, the amount of GHGs emission avoided or sequestered in baseline is 0 tCO2e.

Without the project activity, the amount of GHGs emission avoided or sequestered in this monitoring period is 0 tCO2e

Calculation of project value or estimation of project situation of each SDG Impact

>>

SDG 7 Impact of project situation

From 01/04/2022 to 31/12/2022, 14,841,063.44 m3 biogas were utilized.

From 01/01/2023 to 31/03/2023, 4,984,911.24 m3 biogas were utilized.

For the 2nd monitoring period from 01/04/2022 to 31/03/2023, total 19,825,974.68 m3 biogas were utilized.

SDG 8 Impact of project situation

From 01/04/2022 to 31/12/2022, 18 full-time jobs created (including 9 females and 9 males).

From 01/01/2023 to 31/03/2023, 18 full-time jobs created (including 9 females and 9 males).

For the 2nd monitoring period from 01/04/2022 to 31/03/2023, 18 full-time jobs created (including 9 females and 9 males).

SDG 13 Impact of baseline situation

As per section B.6.1 of the PDD, in project situation, the amount of GHGs emissions avoided or sequestered can be obtained by baseline emission – project emissions – leakage emissions, and the baseline emission, project emissions, leakage emissions can be determined by ACM0010” GHG emission reductions from manure management systems (Version 08.0)”, the specific calculation method and calculation result in this monitoring period are described as follows.

Baseline emission

As per section B.6 of the PDD, baseline emissions in year y (BEy) can be calculated as follows,

Baseline emissions are：

BEy=BECH4,y+BEN2O,y+BEelecheat,y        (Equation 1)

where:

| BEy | Baseline emissions in year y (t CO2/yr) |
| --- | --- |
| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| BEN2O, y | Baseline N2O emissions in year y (t CO2/yr) |
| BEelec/heat,y | Baseline CO2 emissions from electricity and/or heat used in the baseline (t CO2/yr) |

i) Baseline CH4 emissions (BECH4, y)

BECH4,y=GWPCH4*DCH4*j,LT(MCFj*B0,LT*NLT*VSLT,y*MS%Bl,j)    (Equation 2)

where:

| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| --- | --- |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/t CH4) |
| DCH4 | Density of CH4 (t/m3). 0.00067t/m3 at room temperature(20℃)and 1am pressure. |
| MCFj | Annual methane conversion factor (MCFj) for the baseline AWMSj. IPCC 2006, Table 10.17, chapter 10, volume 4. |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg-dm) |
| NLT | Annual average number of animals of type LT for the year y (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%Bl,j | Fraction of manure handled in system j in the baseline. In this project, the baseline manure management system is uncovered anaerobic lagoon only. The amount of manure handled by the anaerobic lagoon is 100%. MS%Bl,j =100% |
| LT | Type of livestock |
| j | Type of treatment system |

Estimation of various variables and parameters for above equations:

(A) As per PDD, VSLT,y shall be determined in following way.

Scaling default IPCC values VSdefault to adjust for a site-specific average animal weight as shown in equation below:

VSLT,y=WsiteWdefault*VSdefault*ndy                      (Equation 3)

where:

| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| VSdefault | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population (kg-dm/animal/day) |
| ndy | Number of days treatment plant was operational in year y |

(B)Annual average number of animals of type LT (NLT) shall be determined in following ways

Option 1:

NLT=Nda,LT*Np,LT365          (Equation 4)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| Nda,LT | Number of days animal of type LT is alive in the farm in the year y (number) |
| Np,LT | Number of animals of type LT produced annually for the year y (number) |

Option 2:

If the project developer can monitor in a reliable and traceable way the daily stock of animals in the farm, discounting dead animals and animals discarded from the productive process from the daily stock, then the annual average number of animals (NLT) may be calculated as follows:

NLT=1365NAA,LT365     (Equation 5)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| NAA,LT | Daily stock of animals of type LT in the farm, discounting dead and discarded animals (number) |

There are two types of swine in this project, i.e., Market swine and Breeding swine. For Market swine, Option 1 is adopted to calculate NLT. For Breeding swine, the PP can monitor the daily stock of breeding swine in a reliable way, discounting dead breeding swine and discarded them from the productive process from the daily stock. So, the Option 2 is adopted to calculate NLT for Breeding swine.

(C) Maximum methane producing potential(B0,LT)

According to applied methodology ACM0010(Version 08.0), this value varies by species and diet. Default values are used, and they are taken from tables 10A-4 through 10A-9 (IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10).

The proposed project is located in Guangdong Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

(D) Annual methane conversion factor (MCFj) for the baseline AWMSj

(a) The MCFj values given in table 10.17, chapter 10, volume 4, 2019 Refinement to the 2006 IPCC Guidelines should be used. MCFj values depend on the climate zone where the uncover anaerobic lagoon in the baseline. According to the registered PDD, the annual average ambient temperature at the project site is 21.8℃. And as per the Temperature Monitoring Report published by the local meteorological bureau, during this monitoring period, the average temperature of any month at the project site is greater than 5 ℃ and annual average ambient temperature at project site during this monitoring period is 21.9℃. So, the mean annual temperature at the project site is higher than 18℃. And the mean annual precipitation is 1771mm , which is higher than 1000mm. Therefore, as per Annex 10A.2, 2019 Refinement to the 2006 IPCC Guidelines, the climate zone of the project site can be determined to "Tropical Moist ", and the MCF value of 80% is applied.

(b)A conservativeness factor should be applied by multiplying MCFj values (estimated as per above bullet) with a value of 0.94, to account for the 20% uncertainty in the MCFj values as reported by IPCC. So, MCFj value of 75.2% (75.2%=80%*0.94) is applied in this monitoring period.

ii)Baseline N2O emissions (BEN2O,y)

BEN2O,y=GWPN2O*CFN2O-N,N*11000*（EN2O,D,y+EN2O,ID,y）        (Equation 6)

where：

| BEN2O,y | Annual baseline N2O emissions in (t CO2e/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%Bl,j              (Equation 7)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N).(Estimated with site-specific, regional or national data if such data is available, otherwise use default EF3 from Table 10.21, chapter 10, volume 4, in the IPCC 2006 Guidelines for National Greenhouse Gas Inventories). The site-specific, regional or national data are not available, so this project activity adopts default EF3. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in Appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%Bl,j                   (Equation 8)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N). (Estimated with site-specific, regional or national data if such data is available. Otherwise, default values for EF4 from Table 11.3, chapter 11, volume 4 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories can be used). The site-specific, regional or national data are not available, so this project activity adopts default EF4. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

Estimation of various variables and parameters for above equations:

(A) As per PDD, the procedure for estimating NEXLT,y  in following way:

In the absence of availability of project specific information on protein intake, which should be justified in the “CDM-PDD”, national or regional data should be used for the nitrogen excretion NEXLT,y, if available. In the absence of such data, default values from Table 10.19 of the IPCC 2006, volume 4, chapter 10) may be used and should be corrected for the animal weight at the project site in the following way:

NEXLT,y=WsiteWdefault*NEXIPCC default                     (Equation 9)

where:

| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| NEX IPCC default | Default value for the nitrogen excretion per head of a defined livestock population (kg N/animal/year) |

NEXIPCC default=NrateT*TAM1000*365                (Equation 10)

where:

| Nrate(T) | the default N excretion rate, kg N/ (1000 kg animal mass)/ day, Table 10.19, chapter 10, volume 4 of IPCC 2006 Guidelines |
| --- | --- |
| TAM | Typical animal mass for livestock in kg/animal |

Table 8 data of parameter

| Time | NLT | NAA,LT | Wsite (kg) |  | VSLT,y (kg-dm/ animal/year) |  | ndy(day) | NEXIPCC (kg N/animal/year) |  | NEXLT,y (kg N/animal/year) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Market swine | Breeding swine | Market swine | Breeding swine | Market swine | Breeding swine |  | Market swine | Breeding swine | Market swine | Breeding swine |
| 01/04/2022-30/04/2022 | 127,447 | 94,180 | 57.3 | 74.5 | 18.42 | 23.95 | 30 | 0.35 | 0.20 | 0.72 | 0.54 |
| 01/05/2022-31/05/2022 | 127,447 | 93,502 | 56.9 | 74.3 | 18.90 | 24.68 | 31 | 0.36 | 0.21 | 0.74 | 0.55 |
| 01/06/2022-30/06/2022 | 127,447 | 93,732 | 57.5 | 74.7 | 18.48 | 24.01 | 30 | 0.35 | 0.20 | 0.72 | 0.54 |
| 01/07/2022-31/07/2022 | 127,447 | 93,352 | 57.2 | 73.8 | 19.00 | 24.51 | 31 | 0.36 | 0.21 | 0.74 | 0.55 |
| 01/08/2022-31/08/2022 | 127,447 | 94,055 | 57.6 | 74.8 | 19.13 | 24.84 | 31 | 0.36 | 0.21 | 0.75 | 0.56 |
| 01/09/2022-30/09/2022 | 127,447 | 93,324 | 58.4 | 73.6 | 18.77 | 23.66 | 30 | 0.35 | 0.20 | 0.74 | 0.53 |
| 01/10/2022-31/10/2022 | 127,447 | 93,347 | 58.6 | 73.7 | 19.46 | 24.48 | 31 | 0.36 | 0.21 | 0.76 | 0.55 |
| 01/11/2022-30/11/2022 | 127,447 | 92,517 | 57.6 | 74.5 | 18.51 | 23.95 | 30 | 0.35 | 0.20 | 0.73 | 0.54 |
| 01/12/2022-31/12/2022 | 127,447 | 92,988 | 58.0 | 75.2 | 19.26 | 24.98 | 31 | 0.36 | 0.21 | 0.76 | 0.56 |
| 01/01/2023-31/01/2023 | 127,447 | 92,822 | 57.8 | 73.6 | 19.20 | 24.45 | 31 | 0.36 | 0.21 | 0.75 | 0.55 |
| 01/02/2023-28/02/2023 | 127,447 | 93,454 | 56.3 | 74.0 | 16.89 | 22.20 | 28 | 0.33 | 0.19 | 0.66 | 0.50 |
| 01/03/2023-31/03/2023 | 127,447 | 94,666 | 57.9 | 74.0 | 19.23 | 24.58 | 31 | 0.36 | 0.21 | 0.75 | 0.55 |

Table 9 The calculation result of BECH4,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | tCO2/tCH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFj | 80% | 80% | % |
| Conservative Factor | 0.94 | 0.94 | / |
| MCF with cons. Factor | 0.752 | 0.752 | / |
| Bo,LT | 0.29 | 0.29 | m3 CH4 /kg-dm |
| NLT | Refer to Table 8 |  |  |
| Wsite | Refer to Table8 |  |  |
| Wdefault | 28 | 28 | kg |
| VSdefault | 0.3 | 0.3 | Kg-dm/animal/day |
| VSLT,y | Refer to Table 8 |  |  |
| MS%Bl,j | 100% | 100% | % |
| ndy | Refer to Table8 |  |  |
| BECH4,y in 01/04/2022-31/12/2022 | 172,344 |  | tCO2e |
| BECH4,y in 01/01/2023-31/03/2023 | 56,131 |  | tCO2e |
| BECH4,y | 228,475 |  | tCO2e |

Table 10 The calculation result of BEN2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| EFN2O,ID,j | 0.01 | 0.01 | kg N2O-N/kg N |
| Nrate(T) | 0.42 | 0.24 | kg N/1000kg animal mass/day |
| NEXIPCCdefault | Refer to Table 8 |  |  |
| Wsite | Refer to Table 8 |  |  |
| TAM | 28 | 28 | kg |
| Wdefault | 28 | 28 | kg |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%Bl,j | 100% | 100% | / |
| Fgasm | 40% | 40% | / |
| GWPN2O | 265 | 265 | tCO2/t N2O |
| CFN20,N-N | 1.57 | 1.57 | Conversion Factor N2O-N to N20 |
| BEN2O,y in 01/04/2022-31/12/2022 | 2,174 |  | tCO2e |
| BEN2O,y in 01/01/2023-31/03/2023 | 707 |  | tCO2e |
| BEN2O,y | 2,881 |  | tCO2e |

Table 11 The calculation results of baseline emissions for SDG 13

| Period | BECH4(tCO2e) | BEN2O,y(tCO2e) | BEy(tCO2e) |
| --- | --- | --- | --- |
| 01/04/2022-31/12/2022 | 172,344 | 2,174 | 174,518 |
| 01/01/2023-31/03/2023 | 56,131 | 707 | 56,838 |
| 2nd monitoring period | 228,475 | 2,881 | 231,356 |

Project emission

As per section B.6 of the PDD, project emissions can be calculated as follows,

PEy=PEAD,y+PEAer,y+PEN2O,y+PEECFC,y            (Equation 11)

where：

| PEy | Project emissions in year y |
| --- | --- |
| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e/yr) |
| PEAer,y | Project CH4 emissions from aerobic AWMS treatment (t CO2e/yr) |
| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| PEEC/FC,y | Project emissions from electricity consumption and fossil fuel combustion (t CO2e/yr) |

i) Project emissions associated with the anaerobic digester in year y (PEAD,y)

Based on the methodology ACM0010” GHG emission reductions from manure management systems (Version 08.0)”. PEAD,y is determined using the methodological tool “Project and leakage emissions from anaerobic digesters”.

According to the Tool14” Project and leakage emissions from anaerobic digesters (Version 02.0)”, the project emissions associated with the anaerobic digester (PEAD,y) are determined as follows:

PEAD,y=PEEC,y+PEFC,y+PECH4,y+PEflare,y              (Equation 12)

where:

| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e) |
| --- | --- |
| PEEC,y | Project emissions from electricity consumption associated with the anaerobic digester in year y (t CO2e) |
| PEFC,y | Project emissions from fossil fuel consumption associated with the anaerobic digester in year y (t CO2e) |
| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| PEflare,y | Project emissions from flaring of biogas in year y (t CO2e) |

The biogas generated during the treatment process in this project was captured for power generation and all the electricity generated from this project was used by the AWMSs and 9 swine farms. The electricity generated was not connected to another user or the regional power grid. Unless no electricity generation for this project, the electricity consumption of the project was supplied by the grid company. Since the electricity consumption of the anaerobic digestion system cannot be measured separately from the entire AWMSs, so the Project emissions from electricity consumption associated with the anaerobic digester and that is not related to the anaerobic digester is calculated together.

The project emissions from electricity consumption calculated according to the “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”,

PEEC,y=j,LTECPJ,J,y*EFEF,j,y*(1+TDLj,y)              (Equation 13)

Where

| PEEC,y | Project emissions from electricity consumption in year y (t CO2e) |
| --- | --- |
| EGPJ,J,y | Quantity of electricity consumed by the project electricity consumption source j in year y (MWh/yr) |
| EFEF,j, y | Emission factor for electricity generation for source j in year y (t CO2/MWh) |
| TDLj,y | Average technical transmission and distribution losses for providing electricity to source j in year y |

The anaerobic digestion process of this project does not involve the use of fossil fuels, so the project emissions from fossil fuel consumption associated with the anaerobic digester is 0, i.e., PEFC,y=0

The residual gas is sent to flare system (if any) and project emissions from flaring of biogas (PEflare,y) shall be estimated using the tool  “ Project emissions from flaring” (version 04.0).

As described in PDD, project emissions from flaring of biogas (PEflare,y) is determined by the below equation

PEflare,y=GWPCH4,y*Vt,db*υi,t,db*ρi,t*1-ηflare,m*10-3       (Equation 14)

| Vt,db | Volumetric flow of the residual gas for flaring in time interval t on a dry basis (m³ dry gas/h) |
| --- | --- |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream for flaring in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |

Since the gaseous stream was distributed though the same pipeline, and the temperature and pressure of gaseous stream can be stable. Therefore the υi,t,db and ρi,t  of residual gas are same with the biogas produced in the anaerobic digester.

In this monitoring period, all of the biogas generated in the anaerobic process was utilized by the biogas generator to generate electricity, hence, PEflare,y=0 .

Determination of project emissions of methane from the anaerobic digester (PECH4,y)

PECH4,y was determined following the step 4 of the applied tool “Project and leakage emissions from anaerobic digesters(Version 02.0)”.Project emissions of methane from the anaerobic digester include emissions during maintenance of the digester, physical leaks through the roof and side walls, and release through safety valves due to excess pressure in the digester. These emissions are calculated using a default emission factor (𝐸𝐹𝐶𝐻4, 𝑑𝑒𝑓𝑎𝑢𝑙𝑡), as follows:

PECH4,y=QCH4,y*EFCH4,default*GWPCH4             (Equation 15)

where:

| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| --- | --- |
| QCH4,y | Quantity of methane produced in the anaerobic digester in year y (t CH4) |
| EFCH4,default | Default emission factor for the fraction of CH4 produced that leak from the anaerobic digester (fraction) |
| GWPCH4 | Global warming potential of CH4 (t CO2/tCH4) |

Estimation of various variables and parameters for above equations:

(A) Quantity of methane produced in the anaerobic digester QCH4,y

As per PDD, the biogas is produced and collected from anaerobic digestion process. The flowmeters are installed at the outlet of the biogas digesters and the measured on an hourly basis time interval. So the quantity of methane produced in the digester in year y (𝑄𝐶𝐻4,𝑦) is the accumulation of the mass flow of methane in the gaseous stream in an hourly basis time interval. i.e., QCH4,y =i=18760Fi,t

The mass flow of greenhouse gas i (Fi,t) is determined as follows:

Fi,t=Vt,db*υi,t,db*ρi,t               (Equation 16)

ρi,t=Pt*MMiRu*Tt                  (Equation 17)

where:

| Fi,t | Mass flow of greenhouse gas i in the gaseous stream in time interval t (kg gas/h) |
| --- | --- |
| Vt,db | Volumetric flow of the gaseous stream in time interval t on a dry basis (m³ dry gas/h) |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |
| Pt | Absolute pressure of the gaseous stream in time interval t (Pa) |
| MMi | Molecular mass of greenhouse gas i (kg/kmol) |
| Ru | Universal ideal gases constant (Pa.m3/kmol.K) |
| Tt | Temperature of the gaseous stream in time interval t (K) |

In summary, during this monitoring period, the Project emissions associated with the anaerobic digester in year y (t CO2e) is the sum of Project emissions from electricity consumption and Project emissions of methane from the anaerobic digester in year y (t CO2e). i.e., PEAD,y= PEEC,y + PECH4, y.

ii) Project CH4 emissions from aerobic AWMS treatment (PEAer, y)

IPCC guidelines specify emissions from aerobic lagoons as 0.1 per cent of total methane generating potential of the waste processed, which can be used as a default for all types of aerobic AWMS treatment.

PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j+PEsl,y               (Equation 18)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (4) or (5) |
| PEsl,y | Project CH4 emissions from sludge disposed of in storage pit prior to disposal during the year y (t CO2e/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

All sludge produced from the aerobic composting will be used for land application, which is calculated as leakage emission, so the PEsl,y=0.

So, PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j      (Equation 19)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (4) or (5) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

iii) Project N2O emissions in year y (PEN2O,y)

PEN2O,y=GWPN2O*CFN2O-N,N*11000*(EN2O,D,y+EN2O,ID,y)          (Equation 20)

where:

| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%j             (Equation 21)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%j     (Equation 22)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

iV) Project emissions from use of heat and/or electricity (PEelec/heat)

PEECFC,y=PEEC,y+jPEFC,j,y        (Equation 23)

where:

| PEEC,y | Project emissions from electricity consumption in year y. The project emissions from electricity consumption will be calculated following the latest version of the “Tool to calculate baseline, project and/or leakage emissions from electricity consumption”. In case, the electricity consumption is not measured then the electricity consumption shall be estimated as follows ECPJ,y=iCPi,y*8760, where CPi,y is the rated capacity (in MW) of electrical equipment i used for the project activity. |
| --- | --- |
| PEFC,y | Project emissions from fossil fuel combustion in process j during the year y. The project emissions from fossil fuel combustion will be calculated following the latest version of the “Tool to calculate project or leakage CO2 emissions from fossil fuel combustion”. For this purpose, the processes j in the tool corresponds to all fossil fuel combustion in the AWMS (not including fossil fuels consumed for transportation of feed material and sludge or any other on-site transportation). |

These emissions should only be considered for consumption of electricity or heat that is not related to the anaerobic digester. As described in above, since the electricity consumption that is not related to the anaerobic digester cannot be separated from the total electricity consumption, therefore the emission for consumption of electricity is calculated in PEEC,y.

No fossil fuel is used for this project. Therefore, PEelec/heat=0

Table 12 data for parameter ECPJ,j,y, QCH4,y and FCH4,RG,m

| period | ECPJ,j,y(MWh) | QCH4,y(t CH4) | FCH4,RG,m (t CH4) |
| --- | --- | --- | --- |
| 01/04/2022-30/04/2022 | 5.78097 | 637.51 | 0.00 |
| 01/05/2022-31/05/2022 | 5.91354 | 610.87 | 0.00 |
| 01/06/2022-30/06/2022 | 5.96047 | 661.38 | 0.00 |
| 01/07/2022-31/07/2022 | 5.96515 | 663.42 | 0.00 |
| 01/08/2022-31/08/2022 | 5.96183 | 677.53 | 0.00 |
| 01/09/2022-30/09/2022 | 5.94681 | 623.00 | 0.00 |
| 01/10/2022-31/10/2022 | 5.97189 | 652.59 | 0.00 |
| 01/11/2022-30/11/2022 | 5.90871 | 680.85 | 0.00 |
| 01/12/2022-31/12/2022 | 5.92688 | 631.55 | 0.00 |
| 01/01/2023-31/01/2023 | 5.93546 | 653.35 | 0.00 |
| 01/02/2023-28/02/2023 | 5.95398 | 631.11 | 0.00 |
| 01/03/2023-31/03/2023 | 5.95992 | 686.14 | 0.00 |

Table 13 the calculation result of PEEC,y

| Parameter | Value | Unit |
| --- | --- | --- |
| ECPJ,j,y | Refer to table 12 | MWh |
| EFEF,j,y | 0.50885 | tCO2/MWh |
| TDLj,y | 20% | / |
| PEEC,y in 01/04/2022-31/12/2022 | 33 | tCO2e |
| PEEC,y in 01/01/2023-31/03/2023 | 11 | tCO2e |
| PEEC,y in 2nd monitoring period | 44 | tCO2e |

Table 14 the calculation result of PECH4,y

| Parameter | Value | Unit |
| --- | --- | --- |
| GWPCH4 | 28 | tCO2/tCH4 |
| EFCH4,default | 0.05 | / |
| QCH4,y | Refer to table 12 | t CH4 |
| PECH4,y in 01/04/2022-31/12/2022 | 8,174 | tCO2e |
| PECH4,y in 01/01/2023-31/03/2023 | 2,758 | tCO2e |
| PECH4,y in 2nd monitoring period | 10,932 | tCO2e |

Table 15 the calculation result of PEflare,y

| Parameter | Value | Unit |
| --- | --- | --- |
| GWPCH4 | 25/28 | tCO2/tCH4 |
| ηflare,m | 0 | / |
| FCH4,RG,m | 0 | t CH4 |
| PEflare,y in 01/04/2022-31/12/2022 | 0 | tCO2e |
| PEflare,y in 01/01/2023-31/03/2023 | 0 | tCO2e |
| PEflare,y in 2nd monitoring period | 0 | tCO2e |

Table 16 calculation of PEAD,y

| Period | PEEC,y (tCO2e) | PECH4,y (tCO2e) | PEflare,y (tCO2e) | PEAD,y (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/04/2022-31/12/2022 | 33 | 8,174 | 0 | 8,207 |
| 01/01/2023-31/03/2023 | 11 | 2,758 | 0 | 2,769 |
| 2nd monitoring period | 44 | 10,932 | 0 | 10,976 |

Table 17 The calculation result of PEAer,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | tCO2/tCH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
|  | 0.001 | 0.001 | kg N/1000kg animal mass/day |
| FAer | 100% | 100% | / |
| 1-Rvs,1 | 20% | 20% | / |
| B0,LT | 0.29 | 0.29 | m3CH4/kg-dm |
| NLT | Refer to Table 8 |  |  |
| VSLT,y | Refer to Table 8 |  |  |
| MS%j | 100% | 100% |  |
| PEAer,y in 01/04/2022-31/12/2022 | 54 |  | tCO2e |
| PEAer,y in 01/01/2023-31/03/2023 | 18 |  | tCO2e |
| PEAer,y in 2nd monitoring period | 72 |  | tCO2e |

Table 18 The calculation result of PEN2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | / |
| EN2O,D,y in 01/04/2022-31/12/2022 | 0 |  | kg N2O-N/year |
| EN2O,D,y in 01/01/2023-31/03/2023 | 0 |  | kg N2O-N/year |
| EN2O,D,y in the 2nd monitoring period | 0.00 |  | kg N2O-N/year |
| EFN2O,D,j | 0.006 | 0.006 | kg N2O-N/kg N |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | / |
| EN2O,D,y in 01/04/2022-31/12/2022 | 7,846 |  | kg N2O-N/year |
| EN2O,D,y in 01/01/2023-31/03/2023 | 2,555 |  | kg N2O-N/year |
| EN2O,D,y in the 2nd monitoring period | 10,401 |  | kg N2O-N/year |
| EFN2O,ID | 0.01 | 0.01 | kg N2O-N/kg NH3 and NOX-N |
| FgasMS,j,LT | 0.4 | 0.4 | / |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | % |
| GWPN2O | 265 | 265 | tCO2/tN2O |
| EN2O,ID,y in 01/04/2022-31/12/2022 | 5,239 |  | kg N2O-N/year |
| EN2O,ID,y in 01/01/2023-31/03/2023 | 1,706 |  | kg N2O-N/year |
| EN2O,ID,y in the 2nd monitoring period | 6,945 |  | kg N2O-N/year |
| EFN2O,iD | 0.01 | 0.01 | kg N2O/kg N |
| Fgasm | 0.45 | 0.45 | / |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | % |
| GWPN2O | 265 | 265 | tCO2/tN2O |
| EN2O,ID,y in 01/04/2022-31/12/2022 | 5,895 |  | kg N2O-N/year |
| EN2O,ID,y in 01/01/2023-31/03/2023 | 1,919 |  | kg N2O-N/year |
| EN2O,ID,y in the 2nd monitoring period | 7,814 |  | kg N2O-N/year |
| PEN2O,y in 01/04/2022-31/12/2022 | 7,903 |  | tCO2e |
| PEN2O,y n 01/01/2023-31/03/2023 | 2,573 |  | tCO2e |
| PEN2O,y in the 2nd monitoring period | 10,476 |  | tCO2e |

Table 19 Calculation results of project emissions for SDG 13

| Period | PEAD,y (tCO2e) | PEAer,y (tCO2e) | PEN2O,y (tCO2e) | PEy (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/04/2022-31/12/2022 | 8,207 | 54 | 7,903 | 16,164 |
| 01/01/2023-31/03/2023 | 2,799 | 18 | 2,573 | 5,360 |
| 2nd monitoring period | 10,976 | 72 | 10,476 | 21,524 |

Leakage emission

As per section B.6.1 of the PDD, Leakage can be calculated as follows,

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y            (Equation 24)

Where:

| LEPJ, N2O, y | Leakage N2O emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEBL, N2O, y | Leakage N2O emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEPJ, CH4, y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| LEBL, CH4, y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEAD, y | Leakage emissions associated with the anaerobic digester in year y (t CO2e) |

i) Estimation of leakage N2O emissions released during baseline scenario from land application of the treated manure in year y, LEBL, N2O, y

LEBL,N2O,y=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)       (Equation 25)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT         (Equation 26)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT        (Equation 27)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT      (Equation 28)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

ii) Estimation of leakage N2O emissions released during project activity from land application of the treated manure in year y, LEPJ, N2O

LEPJ,N2O=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)  (Equation 29)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 30)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 31)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT    (Equation 32)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

It is not possible to measure the quantity of manure applied to land in kg manure/yr (QDM) and the nitrogen concentration in kg N/kg manure (NDM) in the manure to estimate the total quantity of nitrogen applied to land. In this case, n=1N1-RN,n*LTNEXLT,y*NLT does not need to be substituted byQDM*NDM. 

iii) Estimation of leakage CH4 emissions from land application of the treated manure

The calculation of methane emissions from land application of manure in the baseline and project cases are estimated as below:

LEBL,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LT(B0,LT*NLT*VSLT,y*MS%j)   (Equation 33)

LEPJ,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j    (Equation 34)

where:

| LEBL,CH4,y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEPJ,CH4,y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to sludge being treated |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| DCH4 | Density of CH4 (t/m3) |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5), expressed (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |
| MCFd | Methane conversion factor (MCF) assumed to be equal to 1 |

Table 20 Calculation results of LEBL,CH4,y、LEPJ,CH4,y、LEBL,N2O,y and LEPJ,N2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| NLT | Refer to Table 8 |  |  |
| NEXLT,y | Refer to Table 8 |  |  |
| RN,n | 80% | 80% | / |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | KgN-N2O-N/kg NH3-N+NOX-N |
| Fleach | 0.3 | 0.3 | / |
| Fgasm | 0.2 | 0.2 | / |
| GWPN2O | 265 | 265 | tCO2/t N2O |
| LEN2O,land,y in 01/04/2022-31/12/2022 | 2,615 |  | kg N2O-N/year |
| LEN2O,land,y in 01/01/2023-31/03/2023 | 851 |  | kg N2O-N/year |
| LEN2O,land,y in 2nd monitoring period | 3,466 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/04/2022-31/12/2022 | 581 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/01/2023-31/03/2023 | 188 |  | kg N2O-N/year |
| LEN2O,runoff,y in 2nd monitoring period | 769 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/04/2022-31/12/2022 | 511 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/01/2023-31/03/2023 | 167 |  | kg N2O-N/year |
| LEN2O,vol,y in 2nd monitoring period | 678 |  | kg N2O-N/year |
| LEBL,N2O,y in 01/04/2022-31/12/2022 | 1,543 |  | tCO2e |
| LEBL,N2O,y in 01/01/2023-31/03/2023 | 502 |  | tCO2e |
| LEBL,N2O,y in 2nd monitoring period | 2,045 |  | tCO2e |
| NLT | Refer to Table 8 |  |  |
| NEXLT | Refer to Table 8 |  |  |
| RN | 25% | 25% | - |
| RN | 5% | 5% | - |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | KgN-N2O-N/kg NH3-N+NOX-N |
| Fleach | 0.3 | 0.3 | / |
| Fgasm | 0.2 | 0.2 | / |
| LEN2O,land,y in 01/04/2022-31/12/2022 | 9,325 |  | kg N2O-N/year |
| LEN2O,land,y in 01/01/2023-31/03/2023 | 3,038 |  | kg N2O-N/year |
| LEN2O,land,y in 2nd monitoring period | 12,363 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/04/2022-31/12/2022 | 2,106 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/01/2023-31/03/2023 | 686 |  | kg N2O-N/year |
| LEN2O,runoff,y in 2nd monitoring period | 2,792 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/04/2022-31/12/2022 | 1,872 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/01/2023-31/03/2023 | 610 |  | kg N2O-N/year |
| LEN2O,vol,y in 2nd monitoring period | 2,482 |  | kg N2O-N/year |
| LEPJ,N2O,y in 01/04/2022-31/12/2022 | 5,540 |  | tCO2e |
| LEPJ,N2O,y in 01/01/2023-31/03/2023 | 1,805 |  | tCO2e |
| LEPJ,N2O,y in 2nd monitoring period | 7,345 |  | tCO2e |
| GWPCH4 | 28 | 28 | tCO2e/t CH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| B0,LT | 0.29 | 0.29 | m3 CH4/kg-dm |
| RVS | 85% | 85% | / |
| MS%j | 100% | 100% | / |
| LEBL,CH4,y in 01/04/2022-31/12/2022 | 34,369 |  | tCO2e |
| LEBL,CH4,y in 01/01/2023-31/03/2023 | 11,194 |  | tCO2e |
| LEBL,CH4,y in 2nd monitoring period | 45,563 |  | tCO2e |
| GWPCH4 | 28 | 28 | tCO2e/t CH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| B0,LT | 0.29 | 0.29 | m3 CH4/kg-dm |
| RVS | 80% | 80% | / |
| RVS | 20% | 20% | / |
| MS%j | 100% | 100% | / |
| LEPJ,CH4,y in 01/04/2022-31/12/2022 | 36,681 |  | tCO2e |
| LEPJ,CH4,y y in 01/01/2023-31/03/2023 | 11,946 |  | tCO2e |
| LEPJ,CH4,y in 2nd monitoring period | 48,627 |  | tCO2e |

Table 21 Calculation results of leakage for SDG 13

| Period | LEPJ,CH4,y (tCO2e) | LEBL,CH4,y (tCO2e) | LEBL,N2O,y (tCO2e) | LEPJ,N2O,y (tCO2e) | LEy (tCO2e) |
| --- | --- | --- | --- | --- | --- |
| 01/04/2022-31/12/2022 | 36,681 | 34,369 | 1,543 | 5,540 | 6,309 |
| 01/01/2023-31/03/2023 | 11,946 | 11,194 | 502 | 1,805 | 2,055 |
| 2nd monitoring period | 48,627 | 45,563 | 2,045 | 7,345 | 8,364 |

In summary, the baseline emission, project emission, leakage emission and emission reductions in this monitoring period are listed below:

| Period | Baseline emission (tCO2e) | Project emission (tCO2e) | Leakage emission (tCO2e) | Emission reductions (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/04/2022-31/12/2022 | 174,518 | 16,164 | 6,309 | 152,045 |
| 01/01/2023-31/03/2023 | 56,838 | 5,360 | 2,055 | 49,423 |
| 2nd monitoring period | 231,356 | 21,524 | 8,364 | 201,468 |

Therefore, the project value or estimation of project situation of SDG13 in this monitoring period is:

Amount of GHGs emission avoided or sequestered from 01/04/2022 to 31/12/2022 is 152,045 tCO2e.

Amount of GHGs emission avoided or sequestered from 01/01/2023 to 31/03/2023 is 49,423 tCO2e.

For the 2nd monitoring period from 01/04/2022 to 31/03/2023, Amount of GHGs emission avoided or sequestered is 201,468 tCO2e.

Calculation of leakage

>>

The calculation process of leakage emission in this monitoring period has been described in above Section E.2 of this MR. The calculation result is shown in table 21 of this MR.

Calculation of net benefits or direct calculation for each SDG Impact

| SDG | SDG Impact | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- |
| 13 | Amount of GHGs emission avoided or sequestered | 0 tCO2e | 201,468 tCO2e | 201,468 tCO2e |
| 7 | Annual amount of biogas utilized | 0 m3 biogas utilized | 19,825,974.68 m3 biogas utilized | 19,825,974.68 m3 biogas utilized |
| 8 | Total number of jobs | 0 full-time jobs created | 18 full-time jobs created (9 females and 9 males) | 18 full-time jobs created (9 females and 9 males) |

Furthermore, if the actual methane captured from anaerobic digesters in project activity is lower than (BECH4,y - PEAD,y), then (BECH4,y - PEAD,y) is replaced by actual methane captured QCH4,y.

Total biogas captured through the anaerobic treatment during monitoring period is 20,508,613.23 m3, which equals to 218,660 tCO2e emission. And then 19,825,974.68 m3 of biogas is utilized for power generation after desulfurization purification system. Baseline methane emission (BECH4,y) is 228,475 tCO2e. Project emissions associated with anaerobic digester is (PEAD,y) is 10,976 tCO2e. Actual methane captured from anaerobic digesters is higher than the difference of BECH4,y and  PEAD,y. Therefore, the equation (BECH4,y - PEAD,y) can be used to calculate emission reduction.

Comparison of actual SDG Impacts with estimates in approved PDD

| SDG | Values estimated in ex ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- |
| 13 | 238,913 tCO2e of GHGs emission avoided or sequestered | 201,468 tCO2e of GHGs emission avoided or sequestered |
| 7 | 20,577,003.48 m3 biogas utilized | 19,825,974.68 m3 biogas utilized |
| 8 | 18 full-time jobs created (9 females and 9 males) | 18 full-time jobs created (9 females and 9 males) |

    - Explanation of calculation of value estimated ex ante calculation of approved PDD for this monitoring period

>>

According to PDD, the estimated outcomes of SDG 7 for the 2nd monitoring period are 20,577,003.48 m3 biogas utilized which is calculated according to the amount of manure produced from the number of animals and this data sourced from project evaluation report. while the actual outcomes of SDG 7 are 19,825,974.68 m3 biogas utilized in the 2nd monitoring period, which is measured by the flowmeters.

For SDG8, total number of jobs created by the project during the 2nd monitoring period were determined by the record keeping book and cross-checked with the labor contract. 18 full-time jobs for local people were created (including 9 females and 9 males), which is consistent with the estimated outcomes in the PDD.

The estimated value of the outcomes of SDG 13 for the 2nd monitoring period is 238,913 tCO2e, while the actual outcomes of SDG13 is 201,468 tCO2e. The calculation of the estimated and actual value of SDG 13 is based on methodology ACM0010” GHG emission reductions from manure management systems” (version 08.0).

The mainly difference between the estimated value and actual value for SDG7 and SDG 13 is due to the changes of the number of animals in the swine farm (NLT), the average animal weight (Wsite) and the amount of biogas generation and utilized during the anaerobic treatment.

Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

>>

The actual outcomes of SDG 7 are lower than the estimated value in the PDD. The main reason for the difference is that the actual biogas generation through the anaerobic treatment is related to the actual amount of manure produced.

The estimated value of the outcomes of SDG 13 for the 2nd monitoring period is 238,913 tCO2e, and the actual value is 201,468 tCO2e, which is lower than the estimated value in PDD. The difference between the estimated value and actual values is mainly due to the number of animals in the swine farm and the average animal weight is lower than the estimated values in the PDD. In addition, for this monitoring period, no surplus biogas is flared, which can result less project emission.

For SDG 8, 18 full-time jobs for local people were created (including 9 females and 9 males), which is consistent with the estimated outcomes in the PDD.

- SAFEGUARDS REPORTING

>>

According to the Section D.1 and Appendix 1 in PDD, only one assessment question answered ‘Potentially’ i.e., in case of biogas is not handled properly during the operation period of the project, methane explosion may be caused. Therefore, mitigation measure for safeguarding principles is required and the parameter of Employee Training of biogas safety operation during the operation period shall be monitored, which is described in section D.2 of this MR.

During this monitoring period, no biogas explosion or leakage was happened. The monitoring of this parameters was fully implemented in accordance with the monitoring plan in PDD and the monitoring activities was successful, so there is no need to update the mitigation measures.

- STAKEHOLDER INPUTS AND LEGAL DISPUTES

List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

>>

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | A grievance book was put in each farm involved in this project. All stakeholders are allowed to record their grievances or comments in the book. The project owner will assign specific person to deal with related issues. When assessing the relevance and appropriateness of the stakeholders’ comments, the project owner will apply a gender lens. |
| GS Contact (mandatory) | help@goldstandard.org |
| Other | Feedback can be made directly through visiting Shuangbaotai Animal Husbandry Group Co., Ltd. (201, 2nd Floor, Office Building, No. 799, Torch Street, Nanchang High-paying Technology Industrial Development Zone, Nanchang City, Jiangxi Province) or through e-mail snowgirl1984@hotmail.com. |

No inputs/grievances received during the 2nd monitoring period through all the methods.

Report on any stakeholder mitigations that were agreed to be monitored.

>>

No inputs/grievances received during the 2nd monitoring period; therefore, stakeholder mitigations do not need to be reported.

Provide details of any legal contest that has arisen with the project during the monitoring period

>>

No legal contest or dispute has been arisen with the project during the 2nd monitoring period.

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

As per purchase agreement of anaerobic tanks, the technical lifetime of anaerobic tanks is 20 years which is more than 15 years.

365 in this formula represents 365 days per year, which corresponds to the unit of this parameter: kg N / animal / year and Nrate(T) means the default N excretion rate with the unit of kg N/ (1000 kg animal mass)/ day.

https://zhuanlan.zhihu.com/p/38676811

http://finance.people.com.cn/n1/2017/1121/c1004-29658996.html

The project is located in Guangdong Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

http://www.doc88.com/p-9902182440715.html

http://com.gd.gov.cn/ggfw/tzznx/zjgd/gdgk/content/post_3116050.html

http://slt.gd.gov.cn/ysgk_new/gdslgk/content/post_3292694.html

According to PDD, this value is the total of population of animals in the farms, including market swine and breeding swine. It is the design scale for 9 swine farms and also is the maximum population of swine in the swine farms. And the actual scale of the farm will not exceed the design value during the operation period of swine farms, therefore the sample size is calculated in Section D.4 of MR according to the design scale and fixed during the monitoring period, which is conservative.

This formula refers to the formula 10.30 in chapter 10 volume IPCC2006.

Biogas captured during this monitoring period is 20,508,613.23 m3. The quality of methane is 7,809.30 tCH4, which can be calculated by the amount of biogas, the density of biogas and the content of methane in this monitoring period. Then the quality of methane multiplied by the global warming potential of CH4 i.e. 28tCO2/tCH4.is the result of 218,660 tCO2e.Please refer to the ER calculation sheet for more details.