| Project design document form for CDM project activities (Version 05.0) |  |
| --- | --- |
| PROJECT DESIGN DOCUMENT (PDD) |  |
| Title of the project activity | Silivri Wind Power Plant, Turkey |
| Version number of the PDD | 04 |
| Completion date of the PDD | 11.10.2016 |
| Project participant(s) | Silivri Enerji A.Ş. |
| Host Party | Turkey |
| Sectoral scope and selected methodology(ies), and where applicable, selected standardized baseline(s) | 01 Energy Industries (Renewable/non-renewable sources) ACM0002, v. 16.0.0 |
| Estimated amount of annual average GHG emission reductions | 80.442 tCO2e |

Description of project activity

Purpose and general description of project activity

Silivri Elektrik Üretim A.Ş. (hereafter referred to as “Silivri Enerji”), plans to invest in to a new power project called Silivri Wind Power Plant, Turkey (hereafter referred to as the “Project” or “Silivri WPP”), and Installed Capacity of Silivri WPP is 57.5 MWm / 45 MWe with 18 turbines. The license of the project is issued by Energy Market Regulatory Authority (EMRA) in 29.04.2013. The generated electricity will be fed to grid at Silivri TM, 154 kV. While the project is located in the district of Silivri, city of Istanbul, Turkey, the connection point is Silivri Transformer center. Silivri WPP is in line with the Greenfield description of the methodology.

The estimated net electricity generation of the plant is 143,3272 GWh per year. By the efficient utilization of the available wind energy project activity will replace the grid electricity, which is constituted of different fuel sources, mainly fossil fuels. The electricity produced by project activity will result in a total emission reduction of 80.442 tonnes of CO2e per year.  Moreover, project activity will contribute further dissemination of wind energy and extension of national power generation. Construction work started in October 2013, and operation of the plant started on 20.08.2014.

The project will help Turkey to stimulate and commercialize the use of grid connected renewable energy technologies and markets. Furthermore, the project will demonstrate the viability of grid connected wind farms which can support improved energy security, improved air quality, alternative sustainable energy futures, improved local livelihoods and sustainable renewable energy industry development. The specific goals of the project are to:

- reduce greenhouse gas emissions in Turkey compared to the business-as-usual scenario;

- help to stimulate the growth of the wind power industry in Turkey;

- create local employment during the construction and the operation phase of the wind farm;

- reduce other pollutants resulting from power generation industry in Turkey, compared to a business-as-usual scenario;

- help to reduce Turkey’s increasing energy deficit;

- And differentiate the electricity generation mix and reduce import dependency.

As the project developer, Silivri believes that efficient utilization of all kinds of natural resources with a harmony coupled with responsible environmental considerations is vital for sustainable development of Turkey and the World. This has been a guiding factor for the shareholders towards the concept of designation and installation of a wind power project. Other than the objective of climate change mitigation through significant reduction in greenhouse gas (GHG) emissions, the project has been carried out to provide social and economic contribution to the region in a sustainable way. The benefits that will be gained by the realization of the project compared to the business-as-usual scenario can be summarized under four main indicators:

Environmental

The project activities will replace the grid electricity, which is constituted of different fuel sources causing greenhouse gas emissions. By replacing in the consumption of these fuels, it contributes to conservation of water, soil, flora and faunas and transfers these natural resources and also the additional supply of these primary energy sources to the future generations. In the absence of the project activity, an equivalent amount of electricity would have been generated from the power plants connected to the grid, majority of which are based on fossil fuels. Thus, the project is replacing the greenhouse gas emissions (CO2, CH4) and other pollutants (SOX, NOX, particulate matters) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid.

Economical

Firstly, the project will help to accelerate the growth of the wind power industry and stimulate the designation and production of renewable energy technologies in Turkey. Then, other entrepreneurs irrespective of sector will be encouraged to invest in wind power generations. It will also assist to reduce Turkey’s increasing energy deficit and diversify the electricity generation mix while reducing import dependency, especially natural gas. Importantly, rural development will be maintained in the areas around the project site by providing infrastructural investments to these remote villages.

Social

The employment of local people that have necessary technical qualifications for the required post will be the priority and enhanced by all project activities during construction and operation of wind farm. As a result, local poverty and unemployment will be partially eliminated by increased job opportunities and project business activities. Construction materials for the foundations, cables and other auxiliary equipment will preferentially be sourced locally. Moreover as contribution of the project to welfare of the region, the quality of the electricity consumed in the region will be increased by local electricity production, which also contributes decreasing of distribution losses.

Technological

Implementation of the proposed project will contribute to wider deployment of wind power technology in local and national level. It will demonstrate the viability of larger grid connected wind farms, which will support improved energy security, alternative sustainable energy, and also renewable energy industry development. This will also strengthen pillars of Turkish electricity supply based on ecologically sound technology.

Location of project activity

Host Party

The host country is Republic of Turkey.

Region/State/Province etc.

Project area is in Silivri District of Istanbul city of Marmara region.

City/Town/Community etc.

The project is located in Silivri Districts; Ağırsırtı-Maltepe-Karaosman Çifligi-Mezarlıktepe-Derebasitepe-Cataltepe-Balikdere

Physical/Geographical location

Location of the project is given below in the Map 1.

The project site is located about 20 km northeast of Silivri District and will be situated on several hills, between Fenerköy and Gazitepe Village. The closest settlement is Gazitepe village by 2 km. The turbine towers will be built in a 5 km long strip placed approximately 200-400 m apart.

Map 1: Location of Silivri Wind Power Plant Project

Table 1: Geographical coordinates of the wind turbines of the project activity

| # Wind Turbine No. | # Latitude (N) | # Longitude (E) |
| --- | --- | --- |
| # 1 | # 41⁰ 10’ 19.2288’’ | # 28⁰ 15’ 30.7723’’ |
| # 2 | # 41⁰ 10’ 16.8816’’ | # 28⁰ 15’ 42.7736’’ |
| # 3 | # 41⁰ 10’ 15.1212’’ | # 28⁰ 15’ 55.4856’’ |
| # 4 | # 41⁰ 10’ 14.3760’’ | # 28⁰ 16’ 36.5837’’ |
| # 5 | # 41⁰ 10’ 06.9600’’ | # 28⁰ 16’ 38.7999’’ |
| # 6 | # 41⁰ 10’ 13.6488’’ | # 28⁰ 17’ 18.7547’’ |
| # 7 | # 41⁰ 10’ 00.6852’’ | # 28⁰ 17’ 20.7751’’ |
| # 8 | # 41⁰ 09’ 53.9388’’ | # 28⁰ 17’ 26.6504’’ |
| # 9 | # 41⁰ 09’ 48.0168’’ | # 28⁰ 17’ 34.3867’’ |
| # 10 | # 41⁰ 09’ 18.1836’’ | # 28⁰ 17’ 31.5264’’ |
| # 11 | # 41⁰ 10’ 27.7932’’ | # 28⁰ 16’ 31.3508’’ |
| # 12 | # 41⁰ 09’ 17.8056’’ | # 28⁰ 18’ 10.6051’’ |
| # 13 | # 41⁰ 09’ 08.5356’’ | # 28⁰ 18’ 15.9560’’ |
| # 14 | # 41⁰ 09’ 03.8664’’ | # 28⁰ 18’ 24.5725’’ |
| # 15 | # 41⁰ 09’ 01.8648’’ | # 28⁰ 18’ 38.2183’’ |
| # 16 | # 41⁰ 09’ 01.4292’’ | # 28⁰ 19’ 16.3491’’ |
| # 17 | # 41⁰ 09’ 15.8508’’ | # 28⁰ 19’ 28.2658’’ |
| # 18 | # 41⁰ 09’ 12.2832’’ | 28⁰ 19’ 39.6063’’ |

Technologies and/or measures

The technology chosen is wind turbines utilizing wind power in to energy. The Installed Capacity of Silivri WPP is 57.5 MWm / 45 MWe with 18 turbines of Nordex N100 2.5MW 80 each. The key parameters about the technical design of the selected model Nordex N100 2.5MW 80 turbines are listed below in Table 2.

Table 2: Technical specifications of Nordex N100 2.5MW 80

| Specifications | Nordex N100 2.5 80 |
| --- | --- |
| Rated Power (kW) | 2500 |
| Rotor Diameter (m) | 99.8 |
| Blade Length (m) | 58 |
| Num. of Blades | 3 |
| Swept Area (m²) | 7823 |
| Cut-out wind speed (m/s) | 25 |

Technical life time of the Silivri WPP is determined by using the ‘Tool to determine the remaining lifetime of equipment ’’ (v. 1). In the tool it is stated that;

Project participants may use one of the following options to determine the remaining lifetime of the equipment:

(a) Use manufacturer’s information on the technical lifetime of equipment and compare to the date of first commissioning;

(b) Obtain an expert evaluation;

(c) Use default values.

For the project option (c) is used. So in the tool it is said that default lifetime for the on-shore wind turbines is 25 years. In addition to this, operational lifetime of the project is 49 years.

With figures taken from the Generation License, Plant Load Factor (PLF) is calculated as follows;

PLF= Annual Gen. / Installed Cap. * (working hours)

=143327/45*8760

=0.364

The project activity will achieve emission reductions by avoiding CO2 emissions from the business-as-usual scenario electricity generation produced by mainly fossil fuel-fired power plants within the Turkish national grid (Figure 2) Total emission reduction over the 7 year crediting period is expected to reach 563,094 tCO2e with the assumed total net electricity generation of 143,327 GWh/annually.

Figure 1: Share of Sources in Installed Capacity 2013

Figure 2: Share of Sources in Electricity Generation 2013

Although Turkey has a very good wind resource, substantial space, a reasonably good electrical infrastructure and an approaching shortage of electricity; it uses negligible capacity (less than 5%) of its onshore potential, which is estimated as 53,000 MW by Ministry of Energy and Natural Resources (MENR). Lack of attractive incentives and tax advantages, limited grid access and restricted turbine supply constitutes the major barriers in front of the wind energy.

Renewable energy law, enacted in 2005, which had amendments in end of 2010 regarding feed-in tariffs, stipulates a purchase obligation by the retail companies for 10 years with a purchase price 7.3 USDc/kWh (~5.5 €c/kWh) for the power plants put in operation by end of 2015. This tariff is much below the average remuneration in the leading wind markets and does not constitute a sufficient incentive for investments in little experienced wind energy sector of Turkey. The revenues calculated according to these regulations are considered in the investment planning of the projects and do not lead to returns that let the project be profitable or attractive for capital investors and lenders.

These numbers and figures show the contribution of a wind power project like Silivri WPP to the development of environmental friendly electricity generation instead of above described Turkish mix of hydroelectric and fossil fuelled power plants, which are better known and financially more attractive from an investor’s point of view. The emission reductions would not occur in the absence of the proposed project activity because of various real and perceived risks that impede the provision of financing.

Silivri WPP, as a large wind power plant project, will serve as a perfect project to demonstrate long-term potential of wind energy as a means to efficiently reducing GHG emissions as well as to diversifying and increasing security of the local energy supply and contributing to a sustainable development. Wind driven turbines will rotate in generators and electricity generated here will be transferred to the grid for consumer without any greenhouse gas emissions. The Gold Standard certification shall help to realize this seminal technology by providing an adequate compensation for the lacking financial incentives in the Turkish renewable energy market.

Generation of emission reduction and by the way crediting period will start with the first day of documented electricity supply to the national grid. The first 7-year crediting period is expected to be from 20.08.2014 to 19.082021. With 143,327 GWh/annual production of the plant and applying the approved methodology to the project (detailed in the Section B), annual average amount of emission reduction is estimated to be as listed in Table 3.

Table 3: Estimated annual emission reductions of the project over the crediting period.

| Year | Baseline emissions (t CO2e) | Project emissions (t CO2e) | Leakage (t CO2e) | Emission reductions (t CO2e) |
| --- | --- | --- | --- | --- |
| August 2014* | 26,887 | 0 | 0 | 26,887 |
| 2015 | 80,442 | 0 | 0 | 80,442 |
| 2016 | 80,442 | 0 | 0 | 80,442 |
| 2017 | 80,442 | 0 | 0 | 80,442 |
| 2018 | 80,442 | 0 | 0 | 80,442 |
| 2019 | 80,442 | 0 | 0 | 80,442 |
| 2020 | 80,442 | 0 | 0 | 80,442 |
| August 2021** | 53,555 | 0 | 0 | 53,555 |
| Total | 563,094 | 0 | 0 | 563,094 |
| Total number of crediting years | 7 | 7 | 7 | 7 |
| Annual average over the crediting period | 80,442 | 0 | 0 | 80,442 |

* 20/08/2014 (actual)

** 19/08/2021 (estimated)

Parties and project participants

The project participant is listed in the table below, and the contact information of the project participant is provided in Annex 1.

Table 4: Parties involved

| Party involved (host) indicates a host Party | Private and/or public entity(ies) project participants (as applicable) | Indicate if the Party involved wishes to be considered as project participant (Yes/No) |
| --- | --- | --- |
| Turkey (host) | Silivri Enerji A.Ş. (private entity) | No |

Silivri Elektrik Üretim A.Ş. is the developer and owner of the Project.

The Republic of Turkey is the host country. Turkey has recently ratified the Kyoto Protocol (on 5th February of 2009).

Public funding of project activity and standardized baseline

>>

The project activity does not have any public funding or Official Development Assistance (ODA) funding.

Application of selected approved baseline and monitoring methodology and standardized baseline

Reference of methodology and standardized baseline and standardized baseline

>>

For the determination of the baseline, the official methodology ACM0002 version 16.0.0, “Large-scale Consolidated baseline methodology for grid-connected electricity generation from renewable sources”, is applied, using conservative options and data as presented in the following section. This methodology refers to five Tools, which are:

- Tool to calculate the emission factor for an electricity system (Version 04.0.0);

- Tool for the demonstration and assessment of additionality (Version 07.0.0);

- Combined tool to identify the baseline scenario and demonstrate additionality (Version 05.0.0);

- Tool to calculate project or leakage CO2 emissions from fossil fuel combustion (Version 02.0.0).

- Tool to determine the remaining lifetime of the equipment

For baseline calculation the first tool, for additionality assessment the second tool is used. As third tool is the combination of the first and second tool, it is not used. Since no project emission or leakage calculation is required for wind power project fourth tool is not used, and finally to determine the remaining lifetime of the equipment fifth tool is used.

Applicability of methodology and standardized baseline and standardized baseline

The choice of methodology ACM0002 version 16 is justified as the proposed project activity meets its applicability criteria:

| Applicability Conditions in the ACM0002/Version16.0 | Applicability to this project activity |
| --- | --- |
| This methodology is applicable to grid-connected renewable energy power generation project activities that: (a) Install a Greenfield power plant; (b) Involve a capacity addition to (an) existing plant(s); (c) Involve a retrofit of (an) existing operating plants/units; (d) Involve a rehabilitation of (an) existing plant(s)/unit(s); or (e) Involve a replacement of (an) existing plant(s)/unit(s). | The project activity consists of installation of Greenfield power plant at a site where no renewable power plant was operated prior to the implementation of the project activity. Thus, it meets the said applicability condition. |
| The project activity may include renewable energy power plant/unit of one of the following types: hydro power plant/unit with or without reservoir, wind power plant/unit, geothermal power plant/unit, solar power plant/unit, wave power plant/unit or tidal power plant/unit. | The project activity is the installation of 16 numbers of wind turbine generators (WTGs). Hence, meets this criterion. |
| In the case of capacity additions, retrofits, rehabilitations or replacements (except for wind, solar, wave or tidal power capacity addition projects the existing plant/unit started commercial operation prior to the start of a minimum historical reference period of five years, used for the calculation of baseline emissions and defined in the baseline emission section, and no capacity expansion, retrofit, or rehabilitation of the plant/unit has been undertaken between the start of this minimum historical reference period and the implementation of the project activity. | The project activity does not involve capacity additions, retrofits, rehabilitations or replacements. Hence this criterion is not applicable to the project activity. |
| In case of hydro power plants, one of the following conditions shall apply: (a) The project activity is implemented in existing single or multiple reservoirs, with no change in the volume of any of the reservoirs; or (b) The project activity is implemented in existing single or multiple reservoirs, where the volume of the reservoir(s) is increased and the power density calculated using equation (3), is greater than 4 W/m2; or (c) The project activity results in new single or multiple reservoirs and the power density, calculated using equation (3), is greater than 4 W/m2; or (d) The project activity is an integrated hydro power project involving multiple reservoirs, where the power density for any of the reservoirs, calculated using equation (3), is lower than or equal to 4 W/m2, all of the following conditions shall apply: (i) The power density calculated using the total installed capacity of the integrated project, as per equation (4), is greater than 4 W/m2; (ii) Water flow between reservoirs is not used by any other hydropower unit which is not a part of the project activity; (iii) Installed capacity of the power plant(s) with power density lower than or equal to 4 W/m2 shall be: a. Lower than or equal to 15 MW; and b. Less than 10 per cent of the total installed capacity of integrated hydro power project. | The project activity is not a hydro power plant. Hence this applicability criterion is not relevant to the project activity. |
| In the case of integrated hydro power projects, project proponent shall: (a) Demonstrate that water flow from upstream power plants/units spill directly to the downstream reservoir and that collectively constitute to the generation capacity of the integrated hydro power project; or (b) Provide an analysis of the water balance covering the water fed to power units, with all possible combinations of reservoirs and without the construction of reservoirs. The purpose of water balance is to demonstrate the requirement of specific combination of reservoirs constructed under CDM project activity for the optimization of power output. This demonstration has to be carried out in the specific scenario of water availability in different seasons to optimize the water flow at the inlet of power units. Therefore this water balance will take into account seasonal flows from river, tributaries (if any), and rainfall for minimum five years prior to implementation of CDM project activity. | The project activity is not a hydro power plant. Hence this applicability criterion is not relevant to the project activity. |
| The methodology is not applicable to: (a) Project activities that involve switching from fossil fuels to renewable energy sources at the site of the project activity, since in this case the baseline may be the continued use of fossil fuels at the site; (b) Biomass fired power plants/units | Project activity does not involve: • Switching from fossil fuels to renewable energy sources at the site of the project activity. • Biomass fired plants. Hence this criterion is not applicable. |
| In the case of retrofits, rehabilitations, replacements, or capacity additions, this methodology is only applicable if the most plausible baseline scenario, as a result of the identification of baseline scenario, is “the continuation of the current situation, i.e. to use the power generation equipment that was already in use prior to the implementation of the project activity and undertaking business as usual maintenance”. | The project is not a retrofit, rehabilitations, replacements or capacity addition; hence this applicability criterion is not relevant. |
| In addition, the applicability conditions included in the tools referred to above apply. | Applicability conditions of the applied tool are justified |

From the above it is concluded that the project activity meets all the applicability conditions of the

methodology ACM0002 version 16.0 “Grid connected electricity generation from renewable

sources”.

The project activity also meets the following applicability conditions of “Tool to calculate the

emission factor for an electricity system”.

| SI No | Applicability condition | Applicability to this project activity |
| --- | --- | --- |
| 1 | This tool may be applied to estimate the OM, BM and/or CM when calculating baseline emissions for a project activity that substitutes grid electricity, i.e. where a project activity supplies electricity to a grid or a project activity that results in savings of electricity that would have been provided by the grid (e.g. demand-side energy efficiency projects). | The project activity substitutes grid electricity by supplying renewable power to grid. Hence this criterion is applicable. |
| 2 | In case of CDM projects the tool is not applicable if the project electricity system is located partially or totally in an Annex I country. | Project is not a CDM project. |

The project activity also meets the applicability conditions given in “Tool for the demonstration and assessment of additionality”.

Other tools mentioned in the methodology are not applicable for this project activity.

In addition to this, all approved Standardized baselines are not applicable for the project activity.

Project boundary

The project uses wind energy to produce electricity. Kinetic power of the wind is converted to electrical energy, which then will be transferred to the grid. Back-up power generators in the wind farm will only be used when the wind farm is out of service and power cannot be supplied from grid. Hence, emissions due to usage of back-up power generation are expected to be very low and are taken to be zero complying with the ”Tool to calculate project or leakage CO2 emissions from fossil fuel combustion “(v.2).

The project activity evacuates power to the National Grid. A diagrammatic presentation of the project boundary is given in Figure 3.

Figure 3:  Operation diagram of a WPP

Based on the above operation diagram, the baseline and project activity related greenhouse gases which are considered in baseline calculation is given below, in Table 5:

Table 5: Emissions sources included in or excluded from the project boundary

| Source |  | GHGs |  | Included? |  | Justification/Explanation |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline scenario | CO2 emissions from electricity generation in fossil fuel fired power plants that are displaced due to the project activity |  | CO2 |  | Yes |  | Main emission source |  |
|  |  |  | CH4 |  | No |  | Minor emission sources |  |
|  |  |  | N2O |  | No |  |  |  |
| Project scenario | For geothermal power plants, fugitive emissions of CH4 and CO2 from non-condensable gases contained in geothermal steam |  | CO2 |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |
|  |  |  | CH4 |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |
|  |  |  | N2O |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |
|  | For hydro power plants, emissions of CH4 from the reservoir |  | CO2 |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |
|  |  |  | CH4 |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |
|  |  |  | N2O |  | No |  | As per ACM 002 project emissions are not considered for wind power project. |  |

Establishment and description of baseline scenario

The baseline scenario is identified according to the “Baseline Methodology Procedure” of ACM0002 ver.16 (page 10). The project activity is installation of a new grid-connected wind farm with 18 turbines and is not modification/retrofit of an existing grid-connected power plant. So, first identification of this procedure is selected for proposed project activity, which is described as:

“Electricity delivered to the grid by the project activity would have otherwise been generated by the operation of grid-connected power plants and by the addition of new generation sources, as reflected in the combined margin (CM) calculations described in the “Tool to calculate the emission factor for an electricity system”(v.4).

To describe the baseline and its development for the project activity, long-term electricity demand and supply projections for Turkey are assessed.

Demand for electricity in Turkey is growing rapidly with average 6.27% for previous ten years. TEİAŞ, who is responsible from the grid reliability has prepared an electricity demand projection for next ten years period (2013-2022) for Turkey and announced on November 2013, given in Table 6 and Figure 4 reflecting the continuation of current demand growth.

Table 6: Low and High Demand Projection Scenarios for Ten Years Period (TWh)

| Scenarios | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Scenario | 258.14 | 278.96 | 301.3 | 320.47 | 340.71 | 362.1 | 384.67 | 408.5 | 430.51 | 453.56 |
| Low Scenario | 253.77 | 265.78 | 278.16 | 289.33 | 300.39 | 314.85 | 330.44 | 346.51 | 362.13 | 378 |

Figure 4: Electricity Demand Projections for Ten Years

In this projection, electricity supplies are also forecasted taking into account all power plants, which are operational, under construction and newly licensed. Generation projection based on project generation is given in:

Table 7: Projection of Total Generation Capacity by Fuel Types (TWh)

| YEARS | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | SHARE IN 2017 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LIGNITE | 52,712 | 52,715 | 52,939 | 56,143 | 60,470 | 61,870 | 14.55% |
| HARDCOAL | 3,967 | 3,967 | 3,967 | 4,969 | 7,020 | 8,070 | 1.90% |
| IMPORTED COAL | 26,827 | 26,827 | 26,786 | 29,697 | 33,356 | 42,567 | 10.01% |
| NATURAL GAS | 149,344 | 166,022 | 177,262 | 180,853 | 186,092 | 187,249 | 44.02% |
| GEOTHERMAL | 1,184 | 1,294 | 1,702 | 2,206 | 2,410 | 2,410 | 0.57% |
| FUEL OIL | 9,604 | 9,604 | 9,604 | 9,604 | 10,009 | 10,414 | 2.45% |
| DIESEL | 148 | 148 | 148 | 148 | 148 | 148 | 0.03% |
| NUCLEER | 0 | 0 | 0 | 0 | 0 | 0 | 0.00% |
| OTHER | 1,373 | 1,373 | 1,373 | 1,373 | 1,373 | 1,373 | 0.32% |
| THERMAL TOTAL | 245,157 | 261,948 | 273,780 | 284,991 | 300,879 | 314,102 | 73.85% |
| BIOGAS+WASTE | 1,136 | 1,260 | 1,404 | 1,481 | 1,538 | 1,538 | 0.36% |
| HYDRO | 62,413 | 66,805 | 80,483 | 87,269 | 96,097 | 98,335 | 23.12% |
| WIND | 7,950 | 8,153 | 8,677 | 9,724 | 10,902 | 11,356 | 2.67% |
| TOTAL | 316,657 | 338,166 | 364,344 | 383,465 | 409,416 | 425,331 | 100.0% |

According to the 5-year projection it is clear that fossil fuels will remain the main sources for electricity generation (73.85 % in 2017). Natural gas will continue to dominate the market. Hydro will account for 23.12% of the mix whereas all non-hydro renewable combined (geothermal/biogas/waste/wind) will only account for 3.03% of all electricity generation. This projection is consistent with continuing fossil fuel dependent characteristics of Turkish electricity sector, which is illustrated in Figure 5. The share of fossil fuels in the mix has been continuously increasing since the 1970s, reaching 71.6% in 2013.

Figure 5: Fossil Fuels and Renewable in Turkish Electricity Mix (1970-2013)

In the shed of above analysis for the baseline scenario (continuation of current situation) it can be concluded that:

- Conclusion-1: Energy demand in Turkey has been increasing with significant rates since ten years, and it is expected to continue at least for next five years.

- Conclusion-2: Even all operational plants, construction phase plants and licensed ones are taken into account lack of supply is projected after five operational years. So, there is significant need for electricity generation investments to satisfy demand, which means electricity to be generated by the project activity would otherwise be generated by new power plants to avoid power shortage in coming years.

- Conclusion-3: Fossil fuels will hold the dominance in generation mix till the end of 2021 with 73.85% share. Hydro included renewable will remain low with 23.12% share and non-hydro energy contribution will stay negligible with only 3.03% of total share by the end of that period. This also shows that most of new capacity additions will be fossil fuel fired power plants.

The combination of aforementioned trends indicates that if Silivri WPP would not be built, power from a new grid-connected thermal plant would be the most likely scenario

Demonstration of additionality

For the explanation of how and why the project activity leads to emission reductions that are additional to what would have occurred in the absence of the project activity, the Baseline Methodology refers to the consolidated “Tool for the demonstration and assessment of additionality” version 7.0.0 (Tool), which defines a step-wise approach to be applied to the proposed project.

Step 1. Identification of alternatives to the project activity consistent with current laws and regulations.

Sub-step 1a. Define Alternatives to the project activity

To identify the realistic and credible alternative scenario(s) for project participants, scenarios in the Tool are assessed:

a) The proposed project activity undertaken without being registered as a GS VER project activity

This alternative is realistic and credible as Silivri WPP may undertake project activity if it sees no risk for project and/or if the project turns out to be financially attractive without GS VER credit income. However, investments analyze shows that the project is not economically feasible without GS VER credit income. Detail information is given in Step-3.

b) Other realistic and credible alternative scenario(s) to the proposed GS VER project activity scenario that deliver electricity with comparable quality, properties and application areas, taking into account, where relevant, examples of scenarios identified in the underlying methodology;

The project activity is power generation activity without any greenhouse gas emission harnessing the energy of the wind. Being a private entity, Silivri doesn’t have to invest power investments even proposed project activity. Also, since Silivri has a license only for wind power investment and since in the proposed project area there is no hydro or other sources for electricity generation, other project activities delivering same electricity in the same project area is not realistic for project participant.

c) Continuation of the current situation, i.e. Silivri WPP is not built

The decision in favor or against a project investment depends on the expected revenues and risks, like for every other private investment. Investment decisions other than Silivri WPP are independent from the question whether Silivri WPP is built or not. This alternative is also realistic and credible.

According to baseline scenario, which is described in B.4, there is a need for energy investment to satisfy increasing demand and if the Silivri WPP is not built, the same amount of energy will be supplied by other private investors to the grid. Forecasts shows that electricity supplied in the absence of Silivri WPP will be mainly based on fossil fuels as the projections for the year of 2017 forecasts 73.85% share for fossil fuels in the energy mix.

In the absence of the project the power will be produced by new and existing power plants in accordance with the baseline in ACM0002 version 16.

Outcome of Step 1.a: Therefore, two realistic and credible alternative scenarios are identified for the project activity:

a) The proposed project activity undertaken without being registered as a GS VER project activity.

b) Continuation of the current situation, i.e. Silivri WPP is not built.

Sub-step 1b. Consistency with mandatory laws and regulations

Both alternatives are (building or not building the project activity) in compliance with the following identified applicable mandatory laws and regulations:

(1) Electricity Market Law

(2) Law on Utilization of Renewable Energy Resources for the Purpose of Generating Electricity Energy

(3) Environment Law

Table 8: Project Implementation Schedule

| Date (DD/MM/YYYY) | Activity |
| --- | --- |
| 11/06/2009 | “EIA is not required” certificate |
| 29/04/2013 | Issuance of the License |
| 06/01/2012 | Agreement with Carbon consultant |
| 15/03/2013 | Agreement with Equipment provider |
| 03/02/2012 | Silivri Wind Data Analysis and Energy Production Generation Assessment Report |
| 29/03/2013 | Loan Proposal Date |
| 21/10/2013 | Date for start of construction |
| 20/08/2014 | start date of operation |

According to Turkish regulations, to get necessary permits for further project implementation, license issued by EMRA is required. Hence, issuance of license cannot be considered as ‘Investment Decision Taken Date’ but a prerequisite to proceed for further project development activities. Notice to proceed date of electromechanical equipment contract is set to be Investment Decision Taken Date (15.03.2013). From above Implementation Schedule it can be seen that Silivri Res after having license started to analysis of VER and decided to get consultancy for VER development, much before planned construction starting date.

Aforementioned project implementation schedule shows us that Silivri Res started to consideration of VER from the beginning of the project implementation and VER Revenue has decisive impact on decision of proceeding to the project.

In the following, the investment analysis is applied to clearly demonstrate that the project activity is unlikely to be financially/economically attractive without the revenue from the sale of VERs.

Outcome of Step 1.b: Silivri consisted with mandatory laws and regulations. Project implementation schedule shows us that Silivri started to consideration of VER from the beginning of the project implementation and VER Revenue has decisive impact on decision of proceeding to the project.

Step 2. Investment analysis

“Guidelines on the assessment of investment analysis” version 5 is taken into account when applying this step.

Applied tool: “Tool for the demonstration and assessment of additionality version 7.0.0”

Sub-step 2a: Determine Appropriate Analysis Method

Three options can be applied for the investment analysis: the simple cost analysis, the investment comparison analysis and the benchmark analysis.

- Option I: Simple cost analysis

- Option II: Investment comparison analysis

- Option III: Benchmark analysis

The simple cost analysis is not applicable for the proposed project because the project activity will have revenue (from electricity sales) other than CDM related income. The investment comparison analysis is also not applicable for the proposed project because the baseline scenario, providing the same annual electricity output by the Turkish National Grid, is not an investment project.

To conclude, the benchmark analysis will be used to identify whether the financial indicators (Equity IRR in this case) of the proposed project is better than relevant benchmark value.

Sub-step 2b: Option III: Benchmark analysis

While applying the Benchmark Analysis, Option III, the Equity IRR is selected as the financial indicator for the demonstration of the additionality of the project as permitted in the additionality tool.

Benchmark rate is calculated in line with “Tool for the demonstration and assessment of additionality” (v.7) which suggests using the government bond rates, increased by a suitable risk premium. The government bonds are used for determining the Benchmark because there is no pre-determined value for IRR or any other financial indicator for wind power projects in Turkey at the investment decision date of the project.

According to the Tool, benchmark can be derived from ‘Estimates of the cost of financing and required return on capital (e.g. commercial lending rates and guarantees required for the country and the type of project activity concerned), based on bankers views and private equity investors/funds’.

Ministry of Development publishes “Main Economic Indicators” on a monthly basis. Since the project is a mid-term investment (exceeding one yerar), lending rate for medium term investment has been selected as the benchmark. The lending rate for the medium term investment as estimated by the Turkish Development Bank is 13% for the March 2013. This 13% is taken as the benchmark value.

Assessment of likelihood conditions for each parameter to reach benchmark IRR is provided below:

Electricity Price

In order to reach %13 equity IRR benchmark, electricity price 61.4 EUR/MWh which is 10.7% higher than assumed price (55.5 EUR/MWh). This feed-in-tariff price is already very high compared to general market price and not likely to increase. Thus, this 10.7% increase of the feed-in-tariff is not likely to happen.

Investment Cost

In order to reach benchmark IRR, investment costs shall be decreased more than 10,6% and be 44,030,725 EUR, comparing with investment costs used in financial analysis (48,923,028EUR). Since the equipment contract which has the higher share of the total costs is fixed, 10,6% decrease in the investment cost is unlikely. Thus it is not likely for project activity to have threshold investment cost and reach to benchmark IRR.

Energy Yield

To have benchmark IRR, annual energy yield amount shall increase to 154,827 MWh/yr, which is 10,7% more than base case electricity generation amount used in financial analysis. Although most of the wind power project uses p90 electricity generation amount from energy yield reports, to be conservative in financial investment analysis of the project activity p75 is used (139,862 MWh/yr). Even p50 figure of the project activity (154,919,882kWh/yr) is almost equal to the threshold energy yield amount. These figures are annual electricity generation to be sold. Using electricity generation amount in financial analysis, which have less than 10,7% probability of occurrence is not rational. Thus it is not likely for project activity to generate threshold energy yield to reach benchmark IRR.

Operation Cost

In order to reach benchmark IRR, annual operation decrease more than 38%. Such huge decrease in annual operation cost is not likely.

Sub-step 2c: Calculation and comparison of the IRR

In the paragraph 12 of the ‘Guidance on the Assessment of Investment Analysis’ version 5, it is stated that:

‘Required/expected returns on equity are appropriate benchmarks for equity IRR’. Since, benchmark identified in the Sub-step 2b is required/expected returns on equity, equity IRR (after tax) of the project activity shall be calculated for comparison.

Table 9: IRR inputs

| Item | Value | Units | Source | Date of The Source Document |
| --- | --- | --- | --- | --- |
| Installed Power | 57.5/45 | MWm/MWe | License of The project | 29/04/2013 |
| Technical lifetime of the project | 25 | years | Tool to determine the remaining lifetime of equipment (v.1) | 16/10/2009 |
| Gross Generation | 143,327 | MWh | Silivri Wind Data Analysis and Energy Production Generation Assessment Report | 03/02/2012 |
| Electricity tariff | 55.5 | EUR Per MWh | Feed-in-tariff | 18/05/2005 |

Technical life time of the Silivri WPP is determined by using the ‘Tool to determine the remaining lifetime of equipment’’ (v.1). In the tool it is said that default lifetime for the on-shore wind turbines is 25 years.

The equity IRR (after tax) of Silivri WPP is calculated on the basis of expected cash flows (investment, operating costs and revenues from electricity sale), as used in the financial analysis for the feasibility assessment of the project.

The parameters and values used for the IRR calculation are available to DOE during validation and also can be seen from the chart below.

| FINANCIALS |  |
| --- | --- |
| Equity Amount | 10.423.028 EUR |
| Loan Amount | 38.500.000 EUR |
| Interest Rate | 6,00% |
| Repayment Period | 5 |
| Grace Period (years) | 2 |
| Construction Start Date | 21.10.2013 |
| Power Plant Operation Start Date | 20.08.2014 |
| Investment Decision Date | 15.03.2013 |
| USD/TL | 1,8085 |
| EUR/USD | 1,3054 |
| EUR/TL | 2,3608 |
| Corporate Tax Rate | 20% |

The resulting IRR for 25 years is stated in below table.

Table 10: Equity IRR value for project activity (after tax)

| Period | IRR |
| --- | --- |
| 25 years | 10,2070% |

The Benchmark is 15%, it does clearly exceed the resulting equity IRRs, thus rendering the project activity economically unattractive.

PLF= Annual Gen. / Installed Cap. * (working hours)

=143,327/45*8760

=0.364

Sub-step 2d: Sensitivity analysis

While the main parameter determining the income of the project is the electricity sales revenue, investment cost and operation cost, a variation of the accordant values shall demonstrate the reliability of the IRR calculation. Key parameters are varied with +/-10%. The worst, base and best-case results for each parameter variation are given below, in Table 11.

The sensitivity analysis confirms that the proposed project activity is unlikely to be economically attractive without the revenues from VERs as even the maximum IRR result for the best case scenario (12,8284%) is below the benchmark, which is 13%

Table 11: Equity IRR results according to different parameters

| Parameter | Electricity Price |  |  | Investment Cost |  |  | Energy Yield |  |  | Operating Cost |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variance | -10% | 0% | 10% | -10% | 0% | 10% | -10% | 0% | 10% | -10% | 0% | 10% |
| IRR | 7,60% | 10,21% | 12,83% | 12,81% | 10,21% | 8,37% | 7,60% | 10,21% | 12,83% | 11,71% | 10,21% | 8,55% |

Step 3. Barrier analysis

The investment analysis has fully demonstrated and explained the additionality of the project, so step 3 is skipped.

Step 4: Common Practice Analysis

The section below provides the analysis as per step 4 of the “Tool for the demonstration and assessment of additionality”, version 7.0.0 and according to the Guidelines on Common Practice version 02.0

Step 1. Calculate applicable capacity or output range as +/-50% of the total design capacity or output of the proposed project activity: The proposed project has a capacity of 45 MWe. Per the guideline of +/-50%, the applicable output range for the project is 22.5 MW to 67.5 MW.

Step 2: Identify similar projects (both CDM and non-CDM) which fulfil all of the following conditions:

(a) The projects are located in the applicable geographical area; (Turkey)

(b) The projects apply the same measure as the proposed project activity; (Renewable Energy Projects)

(c) The projects use the same energy source/fuel and feedstock as the proposed project activity, if a technology switch measure is implemented by the proposed project activity; (Wind Power Plants)

(d) The plants in which the projects are implemented produce goods or services with comparable quality, properties and applications areas (e.g. clinker) as the proposed project plant; (N.A.)

(e) The capacity or output of the projects is within the applicable capacity or output range calculated in Step 1; (22.5MW-67.5 MW)

(f) The projects started commercial operation before the project design document (CDM-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity; (N.A.)

Table 12 WPP’s with the same output range

| Reference*** |  | TÜM HİDROLİK SANTRALLER - SANTRAL ADI | Kurulu Güç MW | Proje Üretim Kapasitesi GWh | Fuel Type |
| --- | --- | --- | --- | --- | --- |
| Page 97 | 7 | ALİZE ENERJİ (SARIKAYA ŞARKÖY) | 28,8 | 96 | Wind |
| Page 97 | 9 | AKDENİZ ELEK. MERSİN RES | 33 | 100 | Wind |
| Page 97 | 10 | AKRES (AKHİSAR RÜZGAR) | 43,8 | 165 | Wind |
| Page 97 | 11 | ANEMON ENERJİ (İNTEPE) | 30,4 | 92 | Wind |
| Page 97 | 12 | ASMAKİNSAN (BANDIRMA-3 RES) | 24 | 85 | Wind |
| Page 97 | 13 | AYEN ENERJİ (AKBÜK) | 31,5 | 123 | Wind |
| Page 97 | 16 | BARES (BANDIRMA) | 30 | 105 | Wind |
| Page 97 | 17 | BELEN HATAY | 36 | 114 | Wind |
| Page 97 | 19 | BORASKO BANDIRMA | 60 | 240 | Wind |
| Page 97 | 21 | ÇANAKKALE RES (ENERJİ-SA) | 29,9 | 92 | Wind |
| Page 97 | 26 | MARE MANASTIR | 39,2 | 129 | Wind |
| Page 97 | 27 | MAZI 3 | 30 | 105 | Wind |
| Page 97 | 28 | KİLLİK RES (PEM EN.) | 40 | 86 | Wind |
| Page 97 | 30 | KUYUCAK (ALİZE ENER.) | 25,6 | 110 | Wind |
| Page 97 | 33 | DATÇA RES | 29,6 | 84 | Wind |
| Page 97 | 34 | ERTÜRK ELEKT. (ÇATALCA) | 60 | 210 | Wind |
| Page 97 | 35 | İNNORES ELEK. YUNTDAĞ | 52,5 | 213 | Wind |
| Page 97 | 36 | LODOS RES (TAŞOLUK)KEMERBURGAZ | 24 | 85 | Wind |
| Page 97 | 37 | SARES (GARET ENER.) | 22,5 | 91 | Wind |
| Page 97 | 38 | SAYALAR RÜZGAR (DOĞAL ENERJİ) | 34,2 | 108 | Wind |
| Page 97 | 39 | SEBENOBA (DENİZ ELEK.)SAMANDAĞ | 30 | 110 | Wind |
| Page 97 | 40 | SEYİTALİ RES (DORUK EN.) | 30 | 110 | Wind |
| Page 97 | 43 | SUSURLUK (ALANTEK EN.) | 45 | 112 | Wind |
| Page 97 | 45 | TURGUTTEPE RES (SABAŞ ELEK.) | 24 | 70 | Wind |
| Page 97 | 46 | ÜTOPYA ELEKTRİK | 30 | 92 | Wind |
| Page 97 | 47 | ZİYARET RES | 57,5 | 210 | Wind |

Step 3: Within the projects identified in Step 2, identify those that are neither registered CDM project activities, project activities submitted for registration, nor project activities undergoing validation. Note their number Nall.

Nall = 0

Step 4: within similar projects identified in Step 3, identify those that apply technologies that are different to the technology applied in the proposed project activity. Note their number Ndiff.

Ndiff= 0

Step 5: Calculate factor F=1-Ndiff/Nall representing the share of similar projects (penetration rate of the measure/technology) using a measure/technology similar to the measure/technology used in the proposed project activity that deliver the same output or capacity as the proposed project activity.

F = 1-Ndiff/Nall

= 1 – (0/0)

Factor F is therefore unidentified.

The proposed project activity is a “common practice” within a sector in the applicable geographical area if the factor F is greater than 0.2 and Nall-Ndiff is greater than 3. Since factor F is unidentified and also Nall-Ndiff is 0, the proposed is not a common practice as per the guidelines. The proposed project activity is therefore additional under common practice analysis. An Excel sheet is provided for the calculation.

Emission reductions

Explanation of methodological choices

Baseline scenario is identified and described in B.4. Emission reductions due to project activity will be calculated according to “Tool to calculate the emission factor for an electricity system”(v4) (Tool) as indicated in ACM0002 ver. 16.0.0.

A brief explanation of this methodology is given in Tool as (page 4):

“This methodological tool determines the CO2 emission factor for the displacement of electricity generated by power plants in an electricity system, by calculating the “combined margin” emission factor (CM) of the electricity system.”

Data and parameters fixed ex ante

| Data / Parameter | Gross electricity generation |
| --- | --- |
| Unit | MWh |
| Description | Gross Electricity supplied to the grid by relevant sources (2011-2013) |
| Source of data | Turkish Electricity Transmission Company (TEİAŞ), Annual Development of Turkey’s Gross Electricity Generation of Primary Energy Resources (2006-2013) TEİAŞ www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/uretim%20tuketim(23-47)/37(06-13).xls |
| Value(s) applied | See Table 15 |
| Choice of data or Measurement methods and procedures | TEIAS is the national electricity transmission company, which makes available the official data of all power plants in Turkey. |
| - Purpose of data | Data used for emission reduction calculation |
| Additional comment |  |

| Data / Parameter | Net electricity generation |
| --- | --- |
| Unit | GWh |
| Description | Quantity of net electricity generation that is produced and fed into the grid as a result of the implementation of the CDM project activity |
| Source of data | Turkish Electricity Transmission Company (TEIAS), Annual Development of Electricity Generation- Consumption and Losses in Turkey (1984-2013) TEIAS, See http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/uretim%20tuketim(23-47)/34(84-13).xls |
| Value(s) applied | See Table 16, Table 17 |
| Choice of data or Measurement methods and procedures | This data is used to find relation between the gross and net electricity delivered to the grid by fossil fuel fired power plants. (Table 16). Import and Export data is used to identify total net electricity fed into the grid in the years of 20101, 2012 and 2013 (Table 17). TEIAS is the national electricity transmission company, which makes available the official data of all power plants in Turkey. |
| - Purpose of data | Data used for emission reduction calculation |
| Additional comment |  |

| Data / Parameter | HVi,y |
| --- | --- |
| Unit | Mass or volume unit |
| Description | Heating Values of fuels consumed for electricity generation in the years of 2011, 2012 and 2013 |
| Source of data | Heating Values Of Fuels Consumed In Thermal Power Plants In Turkey By The Electric Utilities, TEİAŞ. See: http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/yak%C4%B1t48-53/51.xls |
| Value(s) applied | See Table 22 |
| Choice of data or Measurement methods and procedures | There is no national NVC data in Turkey. However, TEİAŞ announces Heating values of fuels. This data is used to calculate annual NCVs for each fuel type TEİAŞ is the national electricity transmission company, which makes available the official data of all power plants in Turkey. |
| - Purpose of data | Data used for emission reduction calculation |
| Additional comment |  |

| Data / Parameter | FCi,y |
| --- | --- |
| Unit | Mass or volume unit |
| Description | Amount of fuel type i consumed in the project electricity system in year y |
| Source of data | Annual Development of Fuels Consumed In Thermal Power Plants In Turkey By The Electric Utilities, TEİAŞ. See: http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2012/yak%C4%B1t48-53/49.xls |
| Value(s) applied | See Table 23 |
| Choice of data or Measurement methods and procedures | TEİAŞ is the national electricity transmission company, which makes available the official data of all power plants in Turkey. |
| - Purpose of data | Data used for emission reduction calculation |
| Additional comment |  |

| Data / Parameter | NCVi,y |
| --- | --- |
| Unit | GJ/mass or volume |
| Description | Net Calorific Value of fuel types in the years of 2011, 2012 and 2013 |
| Source of data | Calculated by using HVi,y to FCi,y as Net Calorific Values of fuel types are not directly available in Turkey. |
| Value(s) applied | See Table 24, Table 22, Table 23 |
| Choice of data or Measurement methods and procedures | TEİAŞ is the national electricity transmission company, which makes available the official data of power plants in Turkey. Calculation of NCVs from national HVi,y and FCi,y data is preferred to default IPCC data as these are more reliable. |
| - Purpose of data |  |
| Additional comment |  |

| Data / Parameter | Sample Group for BM emission factor |
| --- | --- |
| Unit | Name of the plants, MW capacities, fuel types, annual electricity generations and dates of commissioning. |
| Description | Most recent power plants which compromise 20% of total generation |
| Source of data | Annual Development Of Fuels Consumed In Thermal Power Plants In Turkey By The Electric Utilities, TEIAS: http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2011.pdf http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2012.pdf http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf |
| Value(s) applied | Table 19 |
| Choice of data or Measurement methods and procedures | TEIAS is the national electricity transmission company, which makes available the official data of all power plants in Turkey. The latest data available during PDD preparation was for 2012. |
| - Purpose of data |  |
| Additional comment |  |

| Data / Parameter | EFCO2,m,i,y |
| --- | --- |
| Unit | tCO2/GJ |
| Description | CO2 emission factor of fuel type i in year y |
| Source of data | IPCC default values at the lower limit of the uncertainty at a 95% confidence interval as provided in table 1.4 of Chapter1 of Vol. 2 (Energy) of the IPCC Guidelines on National GHG Inventories. http://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/2_Volume2/V2_1_Ch1_Introduction.pdf |
| Value(s) applied | See Table 13 |
| Choice of data or Measurement methods and procedures | No plant specific and national emission factor data is available in Turkey. So, IPCC default data is used. For Fuel Oil Power Plants: 'Gas/Diesel Oil' data is used for conservativeness. For Coal Power Plants: In the 205th page of official document given in the link below, it is stated that Çolakoğlu and İçdaş utilizes 'Taşkömürü' (Hardcoal). And at the Table-2 in page 157 of the same document, Taşkömürü is dived in two groups: Bituminous and Antharcite. Since Sub-Bituminous Coal is under Brown Coal in the same table and since Other Bituminous Coal has lower EF than Anthracite in 1.4 of IPCC Guidelines, EF for 'Other Bituminous Coal' is used. See: http://www.dpt.gov.tr/DocObjects/Icerik/4225/Enerji_Hammaddeleri_(Linyit_Taskömuru-Jeotermal). |
| - Purpose of data |  |
| Additional comment |  |

| Data / Parameter | ηm,y |
| --- | --- |
| Unit | - |
| Description | Average energy conversion efficiency of power unit m in year y |
| Source of data | Annex I the “Tool to calculate the emission factor for an electricity system”(v.4) |
| Value(s) applied | See Table 20 |
| Choice of data or Measurement methods and procedures | For efficiency rates of Coal and Lignite Power Plants See Annex-1 of the Tool (highest rate is applied to be conservative) For Natural Gas and Oil plants efficiencies, default value given in the tool is applied: http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v2.pdf |
| - Purpose of data |  |
| Additional comment |  |

Ex ante calculation of emission reductions

Stepwise approach of „Tool to calculate the emission factor for an electricity system” version 04.0.0  is used to find this combined margin (emission coefficient) as described below:

Step 1. Identify the relevant electric systems

There are 21 regional distribution regions in Turkey but no regional transmission system is defined. In Article 20 of License Regulation it is stated that:

“TEIAS shall be in charge of all transmission activities to be performed over the existing transmission facilities and those to be constructed as well as the activities pertaining to the operation of national transmission system via the National Load Dispatch Center and the regional load dispatch centers connected to this center and the operation of Market Financial Reconciliation Center”.

As it can be understood from this phrase, only one transmission system, which is national transmission system is defined and only TEİAŞ is in the charge of all transmission system related activities. Moreover, a communication with representative of TEIAS, which indicates that: “There are not significant transmission constraints in the national grid system which is preventing dispatch of already connected power plants” is submitted to the DOE. Therefore, the national grid is used as electric power system for project activity. The national grid of Turkey is connected to the electricity systems of neighboring countries. Complying with the rules of the tool, the emission factor for imports from neighboring countries is considered 0 (zero) tCO2/MWh for determining the OM.

There is no information about interconnected transmission capacity investments, as TEİAŞ, who operates the grid, also didn’t take into account imports-exports for electricity capacity projections. Because of that, for BM calculation transmission capacity is not considered.

Step 2. Choose whether to include off-grid power plants in the project electricity system (optional)

According to Tool project participants may choose between the following two options to calculate the operating margin and build margin emission factor:

Option I: Only grid power plants are included in the calculation.

Option II: Both grid power plants and off-grid power plants are included

For this project Option I is chosen.

Step 3: Select a method to determine the operating margin (OM)

The calculation of the operating margin emission factor (EFgrid,OM,y) is based on one of the following methods:

(a) Simple OM; or

(b) Simple adjusted OM; or

(c) Dispatch data analysis OM; or

(d) Average OM.

The Simple Operating Margin (OM) emission factor (EFgrid, OM, y) is calculated as the generation weighted average CO2 emissions per unit net electricity generation (tCO2/MWh) of all the generating plants serving the system, excluding low-cost/must-run power plants. As electricity generation from solar and low cost biomass facilities is insignificant and there are no nuclear plants in Turkey, the only low cost /must run plants considered are hydroelectric, wind and geothermal facilities.

The Turkish electricity mix does not comprise nuclear energy. Also there is no obvious indication that coal is used as must run resources. Therefore, the only low cost resources in Turkey, which are considered as must-run, are Hydro, Renewable and Waste, Geothermal and Wind (according to statistics of TEİAŞ).

Table 13: Share of Low Cost Resource (LCR) Production 2009-2013 (Production in GWh)

|  | 2009 | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- | --- |
| Gross production | 194,812.9 | 211,207.7 | 229,395.1 | 239,496.8 | 240,153.95 |
| TOTAL LCR Production | 38,229.6 | 55,837.6 | 58,226.0 | 65,345.8 | 69,512.70 |
| Hydro | 35,958.4 | 51,795.5 | 52,338.6 | 57,865.0 | 59,420.47 |
| Renewable and Waste | 340.1 | 457.5 | 469.2 | 720.7 | 1,171.20 |
| Geothermal and Wind | 1,931.1 | 3,584.6 | 5,418.2 | 6,760.1 | 8,921.04 |
| Share of LCRs | 19.62% | 26.44% | 25.38% | 27.28% | 28.95% |
| Average of last five years | 25.53% |  |  |  |  |

As average share of low cost resources for the last five years is far below 50% (23.22%), the Simple OM method is applicable to calculate the operating margin emission factor (EFgrid,OM,y)

For the Simple OM method, the emissions factor can be calculated using either of the two following data vintages:

- Ex-ante option: A 3-year generation-weighted average, based on the most recent data available at the time of submission of the CDM-PDD to the DOE for validation, or

- Ex-post option: The year, in which the project activity displaces grid electricity, requiring the emissions factor to be updated annually during monitoring.

The ex-ante option is selected for Simple OM method, with the most recent data for the baseline calculation stemming from the years 2011 to 2013.

Step 4. Calculate the operating margin emission factor according to the selected method

The Simple OM emission factor is calculated as the generation-weighted average CO2 emissions per unit net electricity generation (tCO2/MWh) of all generating power plants serving the system, not including low-cost/must-run power plants. The calculation of the simple OM emission factor can be based on:

- net electricity generation and corresponding CO2 emission factor of each power unit (Option A), or

- total net electricity generation of all power plants serving the system and the fuel types and total fuel consumption of the project electricity system (Option B).

Option B is chosen to calculate the Simple OM, as there is no power plant specific data available. Renewable power generation is considered as low-cost power source and amount of electricity supplied to the grid by these sources is known.

Where Option B is used, the simple OM emission factor is calculated based on the net electricity supplied to the grid by all power plants serving the system, not including low-cost / must-run power plants, and based on the fuel type(s) and total fuel consumption of the project electricity system, as per formula in the tool:

(1)

Where:

| EFgrid,OMsimple,y | = | Simple operating margin CO2 emission factor in year y (tCO2/MWh) |
| --- | --- | --- |
| FCi,y | = | Amount of fossil fuel type i consumed in the project electricity system in year y (mass or volume unit) |
| NCVi,y | = | Net calorific value (of fossil fuel type i in year y (GJ / mass or volume unit) |
| EFCO2,i,y | = | CO2 emission factor of fossil fuel type i in year y (tCO2/GJ) |
| EGy | = | Net electricity generated and delivered to the grid by all power sources serving the system, not including low-cost / must-run power plants / units, in year y (MWh) |
| i | = | All fossil fuel types combusted in power sources in the project electricity system in year y |
| y | = | three most recent years for which data is available at the time of submission of the PDD to the DOE for validation |

For the calculation of the OM the consumption amount and heating values of the fuels for each sources used for the years 2011, 2012 and 2013, is taken from the TEİAŞ annual statistics, which holds data on annual fuel consumption by fuel types as well as electricity generation amounts by sources and electricity imports. All the data needed for the calculation, including the emission factors and net calorific values (NCVs), are provided in part B of this Annex. Total CO2 emission due to electricity generation in Turkey for the years of 2011, 2012 and 2013 are given in Table 14.

Table 14: CO2 emissions from electricity production 2011-2013 (ktCO2)

|  | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| CO2-Emmissions | 109,963 | 110,931 | 104,840 |

Table 15 presents the gross electricity production data by all the relevant energy sources. Low-cost/must run resources like hydro, wind, geothermic and biomass do not emit fossil CO2 and thus are not taken into account in calculations.

Table 15: Gross electricity production by fossil energy sources 2011-2013 (GWh)

| Energy Source | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Natural Gas | 104,047.6 | 104,499.2 | 105,116.3 |
| Lignite | 38,870.4 | 34,688.9 | 30,262 |
| Coal | 27,347.5 | 33,324.2 | 33,524 |
| Fuel Oil | 900.5 | 981.3 | 1,192.5 |
| Motor Oil | 3.1 | 657.4 | 546.4 |
| Naphtha | 0.0 | 0.0 | 0.0 |
| LPG | 0.0 | 0.0 | 0.0 |
| Total fossil fuels | 171,169.1 | 174,151.0 | 170,641.2 |

Above table shows gross data, but EGy in the above described formula means electricity delivered to the grid, i.e. net generation, the following table shall help to derive net data by calculating the net/gross proportion on the basis of overall gross and net production numbers.

Table 16: Net/gross electricity production 2011-2013 (GWh)

|  | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Gross Production | 229,395.10 | 239,496.80 | 240,153.95 |
| Net Production | 217,557.70 | 227,707.30 | 228,977.00 |
| Relation | 94.84% | 95.08% | 95.35% |

Multiplying these overall gross/net relation percentages with the fossil fuels generation amount does in fact mean an approximation. However this is a conservative approximation as the consumption of plant auxiliaries of fossil power plants is higher than for the plants that are not included in the baseline calculation. In the end this would lead to a lower net electricity generation and therefore to a higher OM emission factor and higher emission reductions.

Table 17 shows the resulting net data for fossil fuel generation and adds electricity imports.

Table 17: Electricity supplied to the grid, relevant for OM (GWh)

|  | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Net El. Prod. by fossil fuels | 162,336.3 | 165,578.2 | 162,699.4 |
| Electricity Import | 4,555.8 | 5,826.7 | 7,429.4 |
| Electricity supplied to grid by relevant sources | 166,892.1 | 171,404.9 | 170,128.8 |

Electricity import is added to the domestic supply in order to fulfill the Baseline Methodology requirements. Imports from connected electricity systems located in other countries are weighted with an emission factor of 0 (zero) tCO2/MWh.

The last step is to calculate EFgrid,OMsimple,y:

Table 18: Calculation of Weighted EFgrid,OMsimple,y (ktCO2/GWh)

|  | 2011 |  | 2012 | 2013 |
| --- | --- | --- | --- | --- |
| CO2-Emmissions (ktCO2) | 109,963 |  | 110,931 | 104,840 |
| Net Electricity Supplied to Grid by relevant sources (GWh) | 166,892.1 |  | 171,404.9 | 170,128.8 |
| EFgrid,OMsimple,y (ktCO2/GWh) | 0.6589 |  | 0.6472 | 0.6162 |
| 3-year Generation Weighted Average EFgrid,OMsimple,y (ktCO2/GWh) |  | 0.6407 |  |  |  |

EFgrid,OMsimple,y = 0.6407(ktCO2/GWh)

Step 5. Calculate the build margin (BM) emission factor

Option 1: For the first crediting period, calculate the build margin emission factor ex ante based on the most recent information available on units already built for sample group m at the time of CDM-PDD submission to the DOE for validation. For the second crediting period, the build margin emission factor should be updated based on the most recent information available on units already built at the time of submission of the request for renewal of the crediting period to the DOE. For the third crediting period, the build margin emission factor calculated for the second crediting period should be used. This option does not require monitoring the emission factor during the crediting period.

Option 2: For the first crediting period, the build margin emission factor shall be updated annually,

ex post, including those units built up to the year of registration of the project activity or, if information up to the year of registration is not yet available, including those units built up to the latest year for which information is available. For the second crediting period, the build margin emissions factor shall be calculated ex ante, as described in Option 1 above. For the third crediting period, the build margin emission factor calculated for the second crediting period should be used.

Again, the project proponents can chose between two options according to the calculation tool: calculate the BM ex-ante based on the latest available data or update the BM each year ex post. Option 1, the ex-ante approach, is again chosen.

The sample group of power units m used to calculate the build margin should be determined as per the following procedure, consistent with the data vintage selected above. The last plant of the sample group is built in 2010 and until the end of the 2012 which is the latest year for official statistics published for plants put in operation. VER plants are excluded from sample group. While identifying the sample group dismantled, revised, retrofits are not included. Only new capacity additions (power plants / units) are taken into account. All power plants in operation by 2012 are given in Annex.

Total electricity generation in 2012 is 239,496.8 GWh and 20% of this generation is 47,899.36 (AEGSET->20%) GWh. Total electricity generation of last five power plants in operation is 369 GWh (AEGSET-5-units) which is lower than 20% total generation in 2012.  Since AEGSET->2%0 is bigger than  AEGSET-5-units , SET->20% is chosen as SETsample. Also in the sample group there is no power plant started supply electricity to grid more than 10 years ago, steps d, e and f are ignored

Sample group for BM emission factor is given the Annex. The derivation of the values presented in

Table 19 is contained in a separate excel file which is available for validation.

Table 19: Sample group generation for BM emission factor calculation (GWh)

| Energy Source | 2010 | 2011 | 2012 | Sample Group Total Generation (GWh) |
| --- | --- | --- | --- | --- |
| Natural Gas | 1,056.3 | 11,815.1 | 10,540.0 | 23,411.4 |
| Lignite | 0 | 0.0 | 40.0 | 40.0 |
| Coal | 8,012.0 | 4,320.0 | 201.0 | 12,533.0 |
| Fuel Oil | 0 | 701.2 | 0.0 | 701.2 |
| Hydro | 3,336.8 | 3,730.4 | 5,354.0 | 12,421.2 |
| Renewable | 2.4 | 150.0 | 677.0 | 829.4 |
| TOTAL | 12,407.5 | 20,716.7 | 16,812.0 | 49,936.2 |

The build margin emissions factor is the generation-weighted average emission factor (tCO2/MWh) of all power units m during the most recent year y for which power generation data is available, calculated as follows:

(2)

Where:

| EFgrid,BM,y | = | Build margin CO2 emission factor in year y (tCO2/MWh) |
| --- | --- | --- |
| EGm,y | = | Net quantity of electricity generated and delivered to the grid by power unit m in year y (MWh) |
| EFEL,m,y | = | CO2 emission factor of power unit m in year y (tCO2/MWh) |
| m | = | Power units included in the build margin |
| y | = | Most recent historical year for which power generation data is available |

Because of only fuel types and electricity generation data are available for the sample group, Option B2 of Simple OM method is used to calculate emission factor. The formulation of emission factor is given below:

(3)

Where:

| EFEL,m,y | = | CO2 emission factor of power unit m in year y (tCO2/MWh) |
| --- | --- | --- |
| EFCO2,m,i,y | = | Average CO2 emission factor of fuel type i used in power unit m in year y (tCO2/GJ) |
| ηm,y | = | Average net energy conversion efficiency of power unit m in year y (%) |
| y | = | Three most recent years for which data is available at the time of submission of the PDD to the DOE for validation |

BM emission factor calculation and resulted BM factor is given in the Table 20. For BM factor calculation, since no official emission factors for different fuel types are available, lower confidence default values of IPCC Guidelines are applied.  Explanation of emission factor selection for each energy sources and references are given in B.6.2 part of the PDD.

Table 20: BM emission factor calculation using equation (2) and (3)

| Energy Source | Sample Group Total Generation (GWh) | Effective CO2 emission factor (tCO2/TJ) | Average Efficiency (ηm,y) | CO2 Emission (ktCO2) |
| --- | --- | --- | --- | --- |
| Natural Gas | 23,411.4 | 54.3 | 60.00% | 7,627.4 |
| Lignite | 40.0 | 90.9 | 50.00% | 26.2 |
| Coal | 12,533.0 | 89.5 | 50.00% | 8,076.3 |
| Fuel Oil | 701.2 | 72.6 | 46.00% | 398.4 |
| Hydro | 12,421.2 | 0.0 | 0.00% | 0.0 |
| Renewables | 829.4 | 0.0 | 0.00% | 0.0 |
| Total | 49,936.2 |  |  | 16,128.3 |
| EFgrid,BM,y (tCO2/MWh) | 0.3230 |  |  |  |

EFgrid,BM,y = 0.3230 tCO2/MWh

Step 6. Calculate the combined margin emission factor

The calculation of the combined margin (CM) emission factor (EFgrid,CM,y) is based on one of the following methods:

(a) Weighted average CM; or

(b) Simplified CM.

The combined margin emission factor is calculated by using weighted average CM as per tool formula below:

(5)

Where:

| EFgrid,BM,y | = | Build margin CO2 emission factor in year y (tCO2/MWh) |
| --- | --- | --- |
| EFgrid,OM,y | = | Operating margin CO2 emission factor in year y (tCO2/MWh) |
| wOM | = | Weighting of operating margin emissions factor (%) |
| wBM | = | Weighting of build margin emissions factor (%) |

According to the Tool for wind power generation project activities: wOM = 0.75 and wBM = 0.25. Then:

EFgrid,CM,y = 0.66407 tCO2/MWh * 0.75 + 0.3230 tCO2/MWh * 0.25 = 0.5612tCO2/MWh

EFgrid,CM,y=0.5612tCO2/MWh

Emission reductions are calculated as follows:

ERy = BEy − PEy − LEy 		(5)

Where:

ERy  = Emission reductions in year y (t CO2/yr).

BEy    = Baseline emissions in year y (t CO2/yr).

PEy   = Project emissions in year y (t CO2/yr).

LEy = Leakage emissions in year y (t CO2/yr).

Project emissions

The proposed project activity involves the generation of electricity by development of a wind farm. The generation of electricity does not result in greenhouse gas emissions and therefore is taken as 0 tCO2/year.

Leakage

LEy is 0, as it is not considered according to ACM0002. PEy is 0 because project is a wind power generation activity (Only for geothermal and Hydro project activities, it should be considered according to ACM0002).

Then: ERy = BEy

Baseline emissions

Baseline emissions include only CO2 emissions from electricity generation in fossil fuel fired power plants that are displaced due to the project activity, calculated as follows:

BEy = (EGy - EGbaseline) x EFgrid,CM,y		(6)

Where:

BEy	 = Baseline emissions in year y (tCO2/yr).

EGy 	 = Electricity supplied by the project activity to the grid (MWh).

EGbaseline 	 = Baseline electricity supplied to the grid in the case of modified or retrofit facilities (MWh). For new power plants this value is taken as zero.

EFgrid,CM,y 	 = Combined margin CO2 emission factor for grid connected power generation in year y calculated using the latest version of the “Tool to calculate the emission factor for an electricity system”(v.4).

The project activity is the installation of a new grid-connected renewable power plant so, EGbaseline = 0

Then:

ERy = BEy = EGy * EFgrid,CM, = 143,327 MWh/year * 0.5612tCO2/MWh = 80,442 tCO2/year

Summary of ex ante estimates of emission reductions

Table 22 Emission Reductions of the plant

| Year | Baseline emissions (t CO2e) | Project emissions (t CO2e) | Leakage (t CO2e) | Emission reductions (t CO2e) |
| --- | --- | --- | --- | --- |
| August 2014* | 26,887 | 0 | 0 | 26,887 |
| 2015 | 80,442 | 0 | 0 | 80,442 |
| 2016 | 80,442 | 0 | 0 | 80,442 |
| 2017 | 80,442 | 0 | 0 | 80,442 |
| 2018 | 80,442 | 0 | 0 | 80,442 |
| 2019 | 80,442 | 0 | 0 | 80,442 |
| 2020 | 80,442 | 0 | 0 | 80,442 |
| August 2021** | 53,555 | 0 | 0 | 53,555 |
| Total | 563,094 | 0 | 0 | 563,094 |
| Total number of crediting years | 7 | 7 | 7 | 7 |
| Annual average over the crediting period | 80,442 | 0 | 0 | 80,442 |

* 20/08/2014 (actual)

** 19/08/2021 (estimated)

Monitoring plan

Data and parameters to be monitored

| Data / Parameter | EGfacility,y |
| --- | --- |
| Unit | MWh/yr |
| Description | Quantity of net electricity generation supplied by the project plant to the grid in year y |
| Source of data | Electricty meter(s) |
| Value(s) applied | 143,327 MWh |
| Measurement methods and procedures | Two electricity meters will be placed (one main and one reserve) at the substation. These meters are sealed by TEIAS and intervention by project proponent is not possible. The fact that two meters are installed in a redundant manner keeps the uncertainty level of the only parameter for baseline calculation low. High data quality of this parameter is not only in the interest of the emission reduction monitoring, but paramount for the business relation between the plant operator and the electricity buyers. Monthly meters will be used as base and PMUM (Piyasa Mali Uzlaştırma Merkezi) readings will be used as to cross check the monthly meter readings. The above described measurement method follows Article 81 of the official regulation “Electricity Market Balancing And Settlement Regulation” |
| Monitoring frequency | Continuous measurement and at least monthly readings |
| QA/QC procedures | According to the Article 2 of the Communiqué of Meters in Electricity Sector: ‘The meters to be used in the electricity market shall be compliant with the standards of Turkish Standards Institute or IEC and have obtained “Type and System Approval” certificate from the Ministry of Trade and Industry.’ Therefore, Ministry of Trade and Industry (Ministry) is responsible from control and calibration of the meters. Also according to Article 11 of this Communiqué, meters shall be in class of 0.5s, which means error interval for measuring is in +-0.5% range which is well acceptable according to rules. Paragraph b) of the Article 9 of the 'Regulation of Metering and Testing of Metering Systems' (Regulation) of Ministry states that: ‘ b) Periodic tests of meters of electricity, water, coal gas, natural gas and current and voltage transformers are done every 10 years.’ Therefore periodic calibration of the meters will be done every 10 years. Also according to Article 67 (page 20) of this regulation, the calibration shall be done in calibration stations which have been tested and approved by Ministry of Trade and Industry. Article 10 d) of Communiqué requires the meters shall be three phase four wire and Article 64 of Regulation clearly states how calibration shall be performed for this kind of meters. As above mentioned, the data acquisition and management and quality assurance procedures that are anyway in place, no additional procedures have to be established for the monitoring plan. |
| Purpose of data | Calculation of Baseline and Project Emission |
| Additional comment | Plant Manager will be responsible for monitoring data. |

Sampling plan

>>

Not Applicable.

Other elements of monitoring plan

>>

As the necessary baseline emission factors are all defined ex ante (Operating and Built Margin, see baseline description), the most important information to be monitored is the amount of electricity fed into the grid by Silivri WPP. This value will be monitored continuously by redundant metering devices, one of them being the main one in the substation, which provides the data for the monthly invoicing to TEİAŞ.

Under data handling process, the meter is read remotely at the end of every month and published through website of TEİAŞ which can be accessed by project owner to check the correctness of the data. Besides, project owner carries monthly protocol to cross-check the data. By this way a cross check is done. Once the meter data is correct then the amount is also published in PMUM website which is also accessible by project owner. When all data is correct, an invoice (receipt of sale) is prepared by Silivri Enerji and delivered to TEIAŞ. The electricity generation amount, the collected data, are kept in both hardcopy and softcopy in daily and monthly basis indefinitely. Moreover, these data are submitted to several authorized people. Thus, several additional copies of these data are stored at different computers. The collected data during the monitoring period will be kept by the project owner at least two years after the last issuance of VERs”

Given a data vintage based on ex ante monitoring and selection of a renewable 7 year crediting period, the Combined Margin will be recalculated at any renewal of the crediting period using the valid baseline methodology.

A backup power generator will be installed in power plant. In case, emissions from back-up power generator exceed 1% of the total emission reductions, they will be accounted as project emissions in each verification period. Operating hours of back-up power generator will be monitored with that purpose.

Potential leakage emissions in the context of power sector projects are emissions arising due to activities such as power plant construction, fuel handling and land inundation. However, according to the methodology, those emission sources do not need to be taken into account.

Operational and Management Structure

As described before, there are two main factors important for the calculation of emission reductions. The only relevant data that have to be monitored is only net electricity generation (EGfacility,y) per year. Since project emission is zero no additional monitoring is required. The generation data are subject to the strict internal quality control systems of both parties.  The monthly meter reading documents are stored by Silivri and TEİAŞ. The settlement notification, which is issued by TEİAŞ and includes the meter reading data, is stored on a TEİAŞ file server and accessible for Silivri via a secured website. The meters themselves can always be read as plausibility check for verification. The other important parameter is the emission factor. It is approved according to strict quality control parameters from an independent external party.  With this, no additional structures or processes have to be implemented to insure the availability and high quality of the necessary data for monitoring.

At the end of each monitoring period, which is planned to generally last one year, from the monthly meter reading records the net electricity generation amounts as calculated by electricity supplied to the grid minus withdrawn from the system, will be added up to the yearly net electricity generation and total project emissions will be subtracted from this amount and result data will be multiplied with the combined margin emission factor with the help of an excel spread sheet that also contains the combined margin calculation. Thus, the complete baseline approach is always transparent and traceable. For the elaboration and quality assurance of the monitoring report, Lifenerji, an expert in the project mechanisms who already supported in the project design, is assigned. However, in order to continue improving the monitoring procedures and therefore also the future monitoring reports, internal quality check shall be fulfilled by Lifenerji. The monitoring reports are checked and in cases of mistakes and inconsistencies in the monitoring report, revisions with improvements shall be done. Furthermore, external year verification assures that the emission reductions calculations are transparent and traceable.

For the operation of Silivri WPP, below hierarchy is planned:

Operation Manager

Electrical Technicians

Administrative Officers

Mechanical Technicians

Figure 6: Operation and Management diagram

Silivri will keep all the data needed for the calculation of emission reductions during the crediting period and until two years after the last issuance of GS VERs for SİLİVRİ WPP.

Because of the data acquisition and management and quality assurance procedures that are anyway in place, no additional procedures have to be established for the monitoring plan. Dedicated emergency procedures are not provided, as there is no possibility of overstating emission reductions due to emergency cases.

      - Date of completion of application of methodology and standardized baseline and contact information of responsible persons/ entities

Date: 18th August 2015

Name of entity determining the baseline: Lifenerji Ltd. Şti.

(Project consultant)

Tel	: +90 312 481 21 42

Fax	: +90 312 480 88 10

e-mail: info@lifenerji.com

Contributor: Silivri Enerji A.Ş.

Lifenerji is not a project participant.

      - Date of completion of application of methodology and standardized baseline and contact information of responsible persons/ entities

>> Not applicable

Duration and crediting period

Duration of project activity

Start date of project activity

>>

The project activity begins in 15.03.2013 the date of Notice to Proceed of the electromechanical equipment agreement between Silivri Enerji A.Ş. and Nordex

Expected operational lifetime of project activity

>>

The expected lifetime of the Nordex WPP is 25 years. In addition to this, operational lifetime of the project is 49 years.

Crediting period of project activity

Type of crediting period

>>

Renewable Crediting Period

Start date of crediting period

>>

20/08/2014 (actual)

Length of crediting period

The length of the first crediting period is 7 years, 0 months.

Environmental impacts

Analysis of environmental impacts

>>

The project activities will replace the grid electricity, which is constituted of different fuel sources causing greenhouse gas emissions. By replacing in the consumption of these fuels, it contributes to conservation of water, soil, flora and faunas and transfers these natural resources and also the additional supply of these primary energy sources to the future generations. In the absence of the project activity, an equivalent amount of electricity would have been generated from the power plants connected to the grid, majority of which are based on fossil fuels. Thus, the project is replacing the greenhouse gas emissions (CO2, CH4) and other pollutants (SOX, NOX, particulate matters) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid.

Environmental impact assessment

>>

Generation of electricity using wind power is a clean process and does not involve any type of emissions during its operations. There are no significant negative environmental impacts of the project activity.Also, project received the “EIA Not Required” certificate on 11/06/2009 and the Project Introductory File has been prepared on October 2008, both taken before the start date of the construction.

Local stakeholder consultation

Solicitation of comments from local stakeholders

>>

n.a.

Summary of comments received

>>

n.a.

Report on consideration of comments received

>>

n.a.

Approval and authorization

>>

Not applicable.

- - - - -

Contact information of project participants

| Project participant and/or responsible person/ entity | Project participant Responsible person/ entity for application of the selected methodology (ies) and, where applicable, the selected standardized baselines to the project activity |
| --- | --- |
| Organization name | Silivri Enerji A.Ş. |
| Street/P.O. Box | Fahrettin Kerim Gökay Cad No:36 Altunizade/Üsküdar |
| Building | - |
| City | İstanbul |
| State/Region |  |
| Postcode |  |
| Country | Turkey |
| Telephone | +90 216 544 24 00 |
| Fax |  |
| E-mail |  |
| Website |  |
| Contact person |  |
| Title | Electrical Engineer |
| Salutation | Mr. |
| Last name | Çiftçi |
| Middle name |  |
| First name | Abdülhadi |
| Department | Project Development |
| Mobile |  |
| Direct fax |  |
| Direct tel. | 0 216 544 24 00 |
| Personal e-mail | abdulhadi.ciftci@iltekiletisim.com |

| Project participant and/or responsible person/ entity | Project participant Responsible person/ entity for application of the selected methodology (ies) and, where applicable, the selected standardized baselines to the project activity |
| --- | --- |
| Organization name | Lifenrji Ltd.Şti. |
| Street/P.O. Box | Çetin Emeç Bulvarı, No:19/18 |
| Building | - |
| City | Çankaya |
| State/Region | Ankara |
| Postcode |  |
| Country | Turkey |
| Telephone | +90 312 481 21 42 |
| Fax | +90 312 480 88 10 |
| E-mail | info@lifenerji.com |
| Website | http://www.lifenerji.com.tr |
| Contact person |  |
| Title |  |
| Salutation | Miss |
| Last name | Dirlik |
| Middle name |  |
| First name | İklim |
| Department |  |
| Mobile |  |
| Direct fax |  |
| Direct tel. |  |
| Personal e-mail | iklim.dirlik@hotmail.com |

Affirmation regarding public funding

- There is no public funding.

Applicability of selected methodology and standardized baseline

Calculation of Total CO2 from OM Power Plants:

Table 22: HVi,y (Heating Values for Fossil Fuels for Electricity Generation (TCal)

| Energy Sources | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Hard Coal+Imported Coal | 57,567 | 71,270 | 68,785 |
| Lignite | 107,210 | 93,587 | 81,676 |
| Fuel Oil | 5.280 | 5.625 | 5,837 |
| Diesel Oil | 155 | 1.884 | 1,363 |
| LPG | 0 | 0 | 0 |
| Naphta | 0 | 0 | 0 |
| Natural Gas | 202,064 | 203,766 | 203,244 |

Table 23: FCi,y (Fuel Consumptions for Fossil Fuels for Electricity Generation (million m3 for Natural Gas and ton for others)

| Energy Sources | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Hard Coal+Imported Coal | 10,574,434 | 12,258,462 | 12,105,930 |
| Lignite | 61,507,310 | 55,742,463 | 47,120,306 |
| Fuel Oil | 531,608 | 564.796 | 573,534 |
| Diesel Oil | 15,047 | 176.379 | 129,359 |
| LPG | 0 | 0 | 0 |
| Naphta | 0 | 0 | 0 |
| Natural Gas | 22,804,587 | 23,090,121 | 22,909,746 |

| 1 | Tcal = | 4.1868 | TJ |
| --- | --- | --- | --- |

Table 4: NCVi,y (Average Net Calorific Values for Fossil Fuels for Electricity Generation (TJ/million m3 for Natural Gas and TJ/kton for others) and EFi (Emission Factor of Fossil Fuels)

| Energy Sources | NCVi 2011 (TJ/Gg) | NCVi 2012 (TJ/Gg) | NCVi 2013 (TJ/Gg) | EFCO2, I (kg/TJ) |
| --- | --- | --- | --- | --- |
| Hard Coal+Imported Coal | 22.79 | 22.34 | 23.79 | 89.50 |
| Lignite | 7.30 | 7.03 | 7.26 | 90.90 |
| Fuel Oil | 41.58 | 41.70 | 42.61 | 72.60 |
| Diesel Oil | 43.15 | 44.71 | 0.00 | 72.60 |
| LPG | 0.00 | 0.00 | 0.00 | 61.60 |
| Naphta | 0.00 | 0.00 | 0.00 | 69.30 |
| Natural Gas | 37.10 | 36.95 | 37.14 | 54.30 |

Table 25: CO2 Emission by each Fossil Fuels Types (ktCO2e)

| Energy Sources | 2011 | 2012 | 2013 |
| --- | --- | --- | --- |
| Hard Coal+Imported Coal | 21,572 | 26,706 | 25,775 |
| Lignite | 40,802 | 35,617 | 31,084 |
| Fuel Oil | 1,605 | 1,710 | 1,774 |
| Diesel Oil | 47 | 573 | 0 |
| Lpg | 0 | 0 | 0 |
| Naphta | 0 | 0 | 0 |
| Natural Gas | 45,938 | 46,325 | 46,206 |
| TOTAL | 109,963 | 110,931 | 104,840 |

- Identification of Sample Group

Table 27: Sample Group PPs for BM Emission Factor Calculation

| No | Information to clearly identify the Plant (Name of the Plant) | Date of Commissioning | Capacity in MW | Fuel Type | Annual Generation (GWh) |
| --- | --- | --- | --- | --- | --- |
| 1 | Eren Enerji (Addition) | 2010 | 600.0 | Imported coal | 4006.00 |
| 2 | Eren Enerji (Addition) | 2010 | 600.0 | Imported coal | 4006.00 |
| 3 | MARMARA PAMUKLU MENS. SN.TİC.A.Ş. (Addition) | 2010 | 26.2 | Natural Gas | 203.76 |
| 4 | Aliağa Çakmaktepe Enerji A.Ş.(Aliağa/İZMİR) (Addition) | 2010 | 69.8 | Natural Gas | 556.00 |
| 5 | FRİTOLAY GIDA SAN.VE TİC. AŞ. (Addition) | 2010 | 0.3 | Biogas | 2.40 |
| 6 | Sönmez Enerji Üretim (Uşak) (Addition) | 2010 | 2.6 | Natural Gas | 19.77 |
| 7 | Ak-Enerji (Uşak OSB) | 2010 | - 15.2 | Liqued Fuel + N.Gas | 0.00 |
| 8 | Ak-Enerji (DG+N) (Deba-Denizli) | 2010 | - 15.6 | Liqued Fuel + N.Gas | 0.00 |
| 9 | Polyplex Europa Polyester Film | 2010 | 7.8 | Natural Gas | 61.00 |
| 10 | ALTEK ALARKO Elektrik Santralleri | 2010 | 21.9 | Natural Gas | 151.36 |
| 11 | Aksa Enerji (Demirtaş/Bursa) | 2010 | - 1.1 | Natural Gas | 0.00 |
| 12 | RASA ENERJİ (VAN) (Addition) | 2010 | 10.1 | Natural Gas | 64.41 |
| 13 | SİLOPİ ELEKTRİK ÜRETİM A.Ş.(ESENBOĞA) | 2010 | - 44.8 | Fuel Oil | 0.00 |
| 14 | International Hospital Istanbul | 2010 | 0.8 | Natural Gas | 6.00 |
| 15 | Tuzla Jeotermal | 2010 | 7.5 | Geothermal | 0.00 |
| 16 | Menderes Jeotermal Dora-2 | 2010 | 9.5 | Geothermal | 0.00 |
| 17 | Selimoğlu Reg. Ve Hes | 2010 | 8.0 | Hydro (run of river) | 0.00 |
| 18 | Kulp IV HES | 2010 | 12.3 | Hydro (run of river) | 46.00 |
| 19 | Cindere HES (Denizli) (Addition) | 2010 | 9.1 | Hydro (With Dam) | 28.29 |
| 20 | Bayburt Hes | 2010 | 14.6 | Hydro (run of river) | 51.00 |
| 21 | UZUNÇAYIR HES (Tunceli) (Addition) | 2010 | 27.3 | Hydro (With Dam) | 105.00 |
| 22 | Alakır Hes. | 2010 | 2.1 | Hydro (run of river) | 6.00 |
| 23 | Peta Müh. En. (Mursal II Hes.) | 2010 | 4.5 | Hydro (run of river) | 19.00 |
| 24 | Asa Enerji (Kale Reg. Ve Hes.) | 2010 | 9.6 | Hydro (run of river) | 0.00 |
| 25 | Hetaş Hacısalihoğlu (Yıldızlı Hes) | 2010 | 1.2 | Hydro (run of river) | 5.00 |
| 26 | Doğubay Elektrik (Sarımehmet Hes) | 2010 | 3.1 | Hydro (run of river) | 10.00 |
| 27 | Nuryol Enerji (Defne Reg. Ve hes.) | 2010 | 7.2 | Hydro (run of river) | 22.00 |
| 28 | ÖZGÜR ELEKTRİK (AZMAK I REG.VE HES) | 2010 | 5.9 | Hydro (run of river) | 0.00 |
| 29 | Birim Hidr. Üretim A.Ş. (Erfelek Hes) | 2010 | 3.2 | Hydro (run of river) | 19.00 |
| 30 | Beytek El. Ür. A.Ş. (Çataloluk Hes.) | 2010 | 9.5 | Hydro (run of river) | 0.00 |
| 31 | Nisan E. Mekanik En. (Başak Reg. Hes.) | 2010 | 6.9 | Hydro (run of river) | 22.00 |
| 32 | UZUNÇAYIR HES (Tunceli) (Addition) | 2010 | 27.3 | Hydro (With Dam) | 105.00 |
| 33 | Fırtına Elektrik Üretim A.Ş. (Sümer Hes) | 2010 | 21.6 | Hydro (run of river) | 70.00 |
| 34 | KAR-EN Karadeniz El. A.Ş. Aralık Hes | 2010 | 12.4 | Hydro (run of river) | 0.00 |
| 35 | Birim Hidr. Üretim A.Ş. (Erfelek Hes) | 2010 | 3.2 | Hydro (run of river) | 19.00 |
| 36 | Karadeniz El. Üret. (Uzundere-1 Hes) | 2010 | 62.2 | Hydro (run of river) | 165.00 |
| 37 | Akım Enerji (Cevizli Reg. Ve Hes.) | 2010 | 91.4 | Hydro (run of river) | 330.00 |
| 38 | Çakıt Hes. (Çakıt Enerji) | 2010 | 20.2 | Hydro (run of river) | 0.00 |
| 39 | Ceyhan Hes. (Oşkan Hes.) (Enova En.) | 2010 | 23.9 | Hydro (run of river) | 98.00 |
| 40 | Erenler Reg. Ve Hes. (BME Bir. Müt. En.) | 2010 | 45.0 | Hydro (run of river) | 85.00 |
| 41 | Paşa Reg. Ve Hes (Özgür Elektrik) | 2010 | 8.7 | Hydro (run of river) | 0.00 |
| 42 | Güzelçay-I-II Hes (İlk Elektrik Enerji) | 2010 | 8.1 | Hydro (run of river) | 0.00 |
| 43 | Kale Reg. Ve Hes (Kale Enerji Ür.) | 2010 | 34.1 | Hydro (run of river) | 116.00 |
| 44 | Erikli-Akocak Reg. Ve Hes | 2010 | 82.5 | Hydro (run of river) | 0.00 |
| 45 | Çamlıkaya Reg. Ve Hes | 2010 | 5.6 | Hydro (run of river) | 19.00 |
| 46 | Dinar Hes. (Elda Elekrik Üretim) | 2010 | 4.4 | Hydro (run of river) | 15.00 |
| 47 | Damlapınar Hes. (Cenay Elektrik Üretim) | 2010 | 16.4 | Hydro (run of river) | 0.00 |
| 48 | Dim Hes (Diler Elektrik Üretim) | 2010 | 38.3 | Hydro (run of river) | 123.00 |
| 49 | ÖZGÜR ELEKTRİK (AZMAK I REG.VE HES) | 2010 | 5.9 | Hydro (run of river) | 0.00 |
| 50 | Kirpilik Reg. Ve Hes (Özgür Elektrik) | 2010 | 6.2 | Hydro (run of river) | 22.00 |
| 51 | Yavuz Reg. Ve Hes (Masat Enerji) | 2010 | 22.5 | Hydro (run of river) | 83.00 |
| 52 | Kayabükü Reg. Ve Hes (Elite Elektrik) | 2010 | 14.6 | Hydro (run of river) | 0.00 |
| 53 | Gök Reg. Ve Hes (Gök Enerji El. San.) | 2010 | 10.0 | Hydro (run of river) | 43.00 |
| 54 | Bulam Reg. Ve Hes (MEM Enerji ELK.) | 2010 | 7.0 | Hydro (run of river) | 0.00 |
| 55 | Karşıyaka HES (Akua Enerji Üret.) | 2010 | 1.6 | Hydro (run of river) | 8.00 |
| 56 | Ceyhan Hes. (Berkman Hes) (Enova En.) | 2010 | 25.2 | Hydro (run of river) | 103.00 |
| 57 | Güdül I Reg. Ve HES (Yaşam Enerji) | 2010 | 2.4 | Hydro (run of river) | 14.00 |
| 58 | Tektuğ Elektrik (Andırın Hes) | 2010 | 40.5 | Hydro (run of river) | 106.00 |
| 59 | Selen Elektrik (Kepezkaya Hes) | 2010 | 28.0 | Hydro (run of river) | 0.00 |
| 60 | REŞADİYE 2 HES (TURKON MNG ELEKT.) | 2010 | 26.1 | Hydro (run of river) | 0.00 |
| 61 | Kozan Hes (Ser-Er Enerji) | 2010 | 4.0 | Hydro (run of river) | 9.00 |
| 62 | Kahraman Reg. Ve Hes (Katırcıoğlu) | 2010 | 1.4 | Hydro (run of river) | 6.00 |
| 63 | Narinkale Reg. Ve Hes (EBD Enerji) | 2010 | 3.1 | Hydro (run of river) | 10.00 |
| 64 | Erenköy Reg. Ve Hes (Türkerler) | 2010 | 21.5 | Hydro (run of river) | 87.00 |
| 65 | Kahta I HES (Erdemyıldız Elektrik Üretim) | 2010 | 7.1 | Hydro (run of river) | 35.00 |
| 66 | Azmak II Reg. Ve Hes | 2010 | - 18.1 | Hydro (run of river) | 0.00 |
| 67 | Ulubat Kuvvet Tüneli ve Hes | 2010 | 97.0 | Hydro (With Dam) | 372.00 |
| 68 | REŞADİYE 1 HES (TURKON MNG ELEKT.) | 2010 | 15.7 | Hydro (run of river) | 0.00 |
| 69 | Egemen 1 HES (Enersis Elektrik) | 2010 | 19.9 | Hydro (run of river) | 0.00 |
| 70 | Sabunsuyu II HES (Ang Enerji Elk.) | 2010 | 7.4 | Hydro (run of river) | 21.00 |
| 71 | Burç Bendi ve Hes (Akkur Enerji) | 2010 | 27.3 | Hydro (run of river) | 113.00 |
| 72 | Murgul Bakır (Ç.kaya) (Addition) | 2010 | 19.6 | Hydro (run of river) | 40.50 |
| 73 | Güzelçay II Hes (İlk Elektrik Enerji) (Addition) | 2010 | 5.0 | Hydro (run of river) | 0.00 |
| 74 | REŞADİYE 1 HES (TURKON MNG ELEKT.) | 2010 | 15.7 | Hydro (run of river) | 0.00 |
| 75 | Egemen 1 HES (Enersis Elektrik) | 2010 | 8.8 | Hydro (run of river) | 0.00 |
| 76 | Yedigöze HES (Yedigöze Elektrik) | 2010 | 155.3 | Hydro (With Dam) | 474.00 |
| 77 | Umut III Reg. Ve HES (Nisan Elek.) | 2010 | 12.0 | Hydro (run of river) | 26.00 |
| 78 | FEKE 2 Barajı ve HES (Nisan Elek.) | 2010 | 69.3 | Hydro (With Dam) | 223.00 |
| 79 | Egemen 1B HES (Enersis Elektrik) | 2010 | 11.1 | Hydro (run of river) | 0.00 |
| 80 | Kalkandere Reg. Ve Yokuşlu HES. | 2010 | 14.5 | Hydro (run of river) | 63.00 |
| 81 | ROTOR ELEKTRİK (OSMANİYE RES) | 2010 | 55.0 | Wind | 0.00 |
| 82 | Asmakinsan (Bandırma 3 RES) | 2010 | 24.0 | Wind | 0.00 |
| 83 | Soma Enerji Üretim (Soma Res) | 2010 | 34.2 | Wind | 0.00 |
| 84 | Deniz Elektrik (Sebenoba Res) | 2010 | 10.0 | Wind | 0.00 |
| 85 | Akdeniz Elektrik (Mersin Res) | 2010 | 33.0 | Wind | 0.00 |
| 86 | Boreas Enerji (Boreas I Enez Res) | 2010 | 15.0 | Wind | 0.00 |
| 87 | Bergama Res En. Ür. A.Ş. Aliağa Res | 2010 | 90.0 | Wind | 0.00 |
| 88 | Bakras En. Elek. Ür. A.Ş. Şenbük Res | 2010 | 15.0 | Wind | 0.00 |
| 89 | ALİZE ENERJİ (KELTEPE RES) | 2010 | 1.8 | Wind | 0.00 |
| 90 | ROTOR ELEKTRİK (Gökçedağ Res) | 2010 | 22.5 | Wind | 0.00 |
| 91 | MAZI-3 RES ELEKT.ÜR. A.Ş. (MAZI-3 RES) | 2010 | 7.5 | Wind | 0.00 |
| 92 | BORASKO ENERJİ (BANDIRMA RES) | 2010 | 12.0 | Wind | 0.00 |
| 93 | Ziyaret Res (Ziyaret Res Elektirk) | 2010 | 35.0 | Wind | 0.00 |
| 94 | Soma Res (Bilgin Rüzgar San. En. Ür.) | 2010 | 90.0 | Wind | 0.00 |
| 95 | Belen ELEKTRİK BELEN Res (Addition) | 2010 | 6.0 | Wind | 0.00 |
| 96 | ÜtOPYA ELEKTRİK (DÜZOVA RES) (Addition) | 2010 | 15.0 | Wind | 0.00 |
| 97 | Kuyucak Res (Alize Enerji Ür.) | 2010 | 25.6 | Wind | 0.00 |
| 98 | Sares Res (Garet Enerji Üretim) | 2010 | 15.0 | Wind | 0.00 |
| 99 | Turguttepe Res (Sabaş Elektrik Ür.) | 2010 | 22.0 | Wind | 0.00 |
| 100 | AKIM ENERJİ BAŞPINAR (SÜPER FİLM) | 2011 | 25.3 | Natural Gas | 177.00 |
| 101 | AKSA AKRİLİK (İTHAL KÖM.+D.G) | 2011 | 25.0 | Natural Gas | 189.08 |
| 102 | AKSA ENERJİ (Antalya) | 2011 | 600.0 | Natural Gas | 3600.00 |
| 103 | ALİAĞA ÇAKMAKTEPE ENERJİ (İlave) | 2011 | 139.7 | Natural Gas | 1051.60 |
| 104 | BEKİRLİ TES (İÇDAŞ ELEKTRİK EN.) | 2011 | 600.0 | Imported coal | 4320.00 |
| 105 | BOLU BELEDİYESİ ÇÖP TOP. TES. BİYOGAZ | 2011 | 1.1 | Landfill Gas | 0.00 |
| 106 | BOSEN ENERJİ ELEKTRİK ÜRETİM AŞ. | 2011 | 93.0 | Natural Gas | 698.49 |
| 107 | CENGİZ ÇİFT YAKITLI K.Ç.E.S. | 2011 | 131.3 | Natural Gas | 985.00 |
| 108 | CENGİZ ENERJİ SAN.VE TİC.A.Ş. | 2011 | 35.0 | Natural Gas | 281.29 |
| 109 | CEV ENERJİ ÜRETİM(GAZİANTEP ÇÖP BİOGAZ) | 2011 | 5.7 | Landfill Gas | 0.00 |
| 110 | FRAPORT IC İÇTAŞ ANTALYA HAVALİMANI | 2011 | 8.0 | Natural Gas | 64.00 |
| 111 | GLOBAL ENERJİ (PELİTLİK) | 2011 | 4.0 | Natural Gas | 29.91 |
| 112 | GORDİON AVM (REDEVCO ÜÇ EMLAK) | 2011 | 2.0 | Natural Gas | 15.00 |
| 113 | GOREN-1 (GAZİANTEP ORGANİZE SAN.) | 2011 | 48.7 | Natural Gas | 277.00 |
| 114 | GÜLLE ENERJİ(Çorlu) (İlave) | 2011 | 3.9 | Natural Gas | 17.97 |
| 115 | HASIRCI TEKSTİL TİC. VE SAN. LTD. ŞTİ. | 2011 | 2.0 | Natural Gas | 15.00 |
| 116 | HG ENERJİ ELEKTRİK ÜRET. SAN.TİC. A.Ş. | 2011 | 52.4 | Natural Gas | 366.00 |
| 117 | ISPARTA MENSUCAT (Isparta) | 2011 | 4.3 | Natural Gas | 33.00 |
| 118 | ITC ADANA ENERJİ ÜRETİM (İlave) | 2011 | 1.4 | Landfill Gas | 0.00 |
| 119 | ITC-KA EN. (ASLIM BİYOKÜTLE) KONYA | 2011 | 5.7 | Landfill Gas | 0.00 |
| 120 | ITC-KA ENERJİ (SİNCAN) (İlave) | 2011 | 1.4 | Landfill Gas | 0.00 |
| 121 | ITC-KA ENERJİ MAMAK KATI ATIK TOP. | 2011 | 2.8 | Landfill Gas | 0.00 |
| 122 | İSTANBUL SABİHA GÖKÇEN UL.AR. HAV. | 2011 | 4.0 | Natural Gas | 32.00 |
| 123 | KARKEY (SİLOPİ 1) | 2011 | 100.4 | Fuel Oil | 701.15 |
| 124 | KAYSERİ KATI ATIK DEPONİ SAHASI | 2011 | 1.6 | Landfill Gas | 0.00 |
| 125 | KNAUF İNŞ. VE YAPI ELEMANLARI SN. | 2011 | 1.6 | Natural Gas | 12.00 |
| 126 | LOKMAN HEKİM ENGÜRÜ SAĞ.(SİNCAN) | 2011 | 0.5 | Natural Gas | 4.00 |
| 127 | MARDİN-KIZILTEPE (AKSA ENERJİ) | 2011 | 32.1 | Natural Gas | 225.00 |
| 128 | NUH ENERJİ EL. ÜRT.A.Ş. (ENERJİ SANT.-2) | 2011 | 120.0 | Natural Gas | 900.00 |
| 129 | ODAŞ DOĞALGAZ KÇS (ODAŞ ELEKTRİK) | 2011 | 55.0 | Natural Gas | 415.00 |
| 130 | POLYPLEX EUROPA POLYESTER FİLM | 2011 | 3.9 | Natural Gas | 30.70 |
| 131 | SAMSUN TEKKEKÖY EN. SAN. (AKSA EN.) | 2011 | 131.3 | Natural Gas | 980.00 |
| 132 | SAMUR HALI A.Ş. | 2011 | 4.3 | Natural Gas | 33.00 |
| 133 | SARAY HALI A.Ş. | 2011 | 4.3 | Natural Gas | 33.00 |
| 134 | TEKİRDAĞ-ÇORLU TEKS.TES.(NİL ÖRME) | 2011 | 2.7 | Natural Gas | 21.00 |
| 135 | TİRENDA TİRE ENERJİ ÜRETİM A.Ş. | 2011 | 58.4 | Natural Gas | 410.00 |
| 136 | YENİ UŞAK ENERJİ ELEKTRİK SANTRALI | 2011 | 8.7 | Natural Gas | 65.00 |
| 137 | ZORLU ENERJİ (B.Karıştıran) | 2011 | 7.2 | Natural Gas | 54.07 |
| 138 | ŞANLIURFA OSB (RASA ENERJİ ÜR. A.Ş.) | 2011 | 116.8 | Natural Gas | 800.00 |
| 139 | AYDIN/GERMENCİK JEOTERMAL | 2011 | 20.0 | Geothermal | 150.00 |
| 140 | ÇEŞMEBAŞI REG. VE HES (GİMAK EN.) | 2011 | 8.2 | Hydro (run of river) | 39.00 |
| 141 | ÇUKURÇAYI HES (AYDEMİR ELEKTRİK ÜR.) | 2011 | 1.8 | Hydro (run of river) | 8.00 |
| 142 | DARCA HES (BÜKOR ELEKTRİK ÜRETİM) | 2011 | 8.9 | Hydro (run of river) | 0.00 |
| 143 | DERME (KAYSERİ VE CİVARI ENERJİ) | 2011 | 4.5 | Hydro (run of river) | 14.00 |
| 144 | DURU 2 REG. VE HES (DURUCASU ELEK.) | 2011 | 4.5 | Hydro (run of river) | 22.00 |
| 145 | ERENKÖY REG. VE HES (NEHİR ENERJİ) | 2011 | 21.5 | Hydro (run of river) | 87.00 |
| 146 | ERKENEK (KAYSERİ VE CİVARI ENERJİ) | 2011 | 0.3 | Hydro (run of river) | 0.00 |
| 147 | EŞEN-1 HES (GÖLTAŞ ENERJİ ELEKTRİK) | 2011 | 60.0 | Hydro (run of river) | 240.00 |
| 148 | GİRLEVİK (BOYDAK ENERJİ) | 2011 | 3.0 | Hydro (run of river) | 21.00 |
| 149 | GÖKMEN REG. VE HES (SU-GÜCÜ ELEKT.) | 2011 | 2.9 | Hydro (run of river) | 13.00 |
| 150 | HACININOĞLU HES (ENERJİ-SA ENERJİ) | 2011 | 142.3 | Hydro (run of river) | 360.00 |
| 151 | HAKKARİ (Otluca) (NAS ENERJİ A.Ş.) | 2011 | 1.3 | Hydro (run of river) | 6.00 |
| 152 | HASANLAR | 2011 | 9.4 | Hydro (run of river) | 39.00 |
| 153 | HASANLAR HES (DÜZCE ENERJİ BİRLİĞİ) | 2011 | 4.7 | Hydro (run of river) | 0.00 |
| 154 | İNCİRLİ REG. VE HES (LASKAR ENERJİ) | 2011 | 25.2 | Hydro (run of river) | 126.00 |
| 155 | KALKANDERE REG. VE YOKUŞLU HES | 2011 | 23.4 | Hydro (run of river) | 0.00 |
| 156 | KARASU 4-2 HES (İDEAL ENERJİ ÜRETİMİ) | 2011 | 10.4 | Hydro (run of river) | 0.00 |
| 157 | KARASU 4-3 HES (İDEAL ENERJİ ÜRETİMİ) | 2011 | 4.6 | Hydro (run of river) | 0.00 |
| 158 | KARASU 5 HES (İDEAL ENERJİ ÜRETİMİ) | 2011 | 4.1 | Hydro (run of river) | 0.00 |
| 159 | KARASU I HES (İDEAL ENERJİ ÜRETİMİ) | 2011 | 3.8 | Hydro (run of river) | 0.00 |
| 160 | KARASU II HES (İDEAL ENERJİ ÜRETİMİ) | 2011 | 3.1 | Hydro (run of river) | 13.00 |
| 161 | KAZANKAYA REG. VE İNCESU HES (AKSA) | 2011 | 15.0 | Hydro (run of river) | 48.00 |
| 162 | KESME REG. VE HES (KIVANÇ ENERJİ) | 2011 | 4.6 | Hydro (run of river) | 16.00 |
| 163 | KIRAN HES (ARSAN ENERJİ A.Ş.) | 2011 | 9.7 | Hydro (run of river) | 0.00 |
| 164 | KORUKÖY HES (AKAR ENERJİ SAN. TİC.) | 2011 | 3.0 | Hydro (run of river) | 22.00 |
| 165 | KOVADA-I (BATIÇİM ENERJİ ELEKTRİK) | 2011 | 51.2 | Hydro (run of river) | 36.20 |
| 166 | KOVADA-II (BATIÇİM ENERJİ ELEKTRİK) | 2011 | 8.3 | Hydro (run of river) | 4.10 |
| 167 | KOZDERE HES (ADO MADENCİLİK ELKT. ) | 2011 | 3.1 | Hydro (run of river) | 0.00 |
| 168 | KÖYOBASI HES (ŞİRİKOĞLU ELEKTRİK) | 2011 | 1.1 | Hydro (run of river) | 5.00 |
| 169 | KULP I HES (YILDIZLAR ENERJİ ELK.ÜR.) | 2011 | 22.9 | Hydro (run of river) | 78.00 |
| 170 | KUMKÖY HES (AES-IC İÇTAŞ ENERJİ) | 2011 | 17.5 | Hydro (run of river) | 98.00 |
| 171 | AKSU REG. VE HES (KALEN ENERJİ) | 2011 | 5.2 | Hydro (run of river) | 16.00 |
| 172 | ALKUMRU BARAJI VE HES (LİMAK HİD.) | 2011 | 261.3 | Hydro (run of river) | 828.00 |
| 173 | AYRANCILAR HES (MURADİYE ELEKTRİK) | 2011 | 32.1 | Hydro (run of river) | 0.00 |
| 174 | BALKONDU I HES (BTA ELEKTRİK ENERJİ) | 2011 | 9.2 | Hydro (run of river) | 33.00 |
| 175 | BAYRAMHACILI BARAJI VE HES | 2011 | 47.0 | Hydro (run of river) | 175.00 |
| 176 | BERDAN | 2011 | 10.2 | Hydro (run of river) | 47.20 |
| 177 | BOĞUNTU HES (BEYOBASI ENERJİ) | 2011 | 3.8 | Hydro (run of river) | 17.00 |
| 178 | CEVHER I-II REG. VE HES (ÖZCEVHER EN.) | 2011 | 16.4 | Hydro (run of river) | 0.00 |
| 179 | ÇAKIRMAN REG. VE HES (YUSAKA EN.) | 2011 | 7.0 | Hydro (run of river) | 22.00 |
| 180 | ÇAMLIKAYA REG.VE HES (ÇAMLIKAYA EN) | 2011 | 2.8 | Hydro (run of river) | 0.80 |
| 181 | ÇANAKÇI HES (CAN ENERJİ ENTEGRE) | 2011 | 9.3 | Hydro (run of river) | 39.00 |
| 182 | MENGE BARAJI VE HES (ENERJİSA ENERJİ) | 2011 | 44.7 | Hydro (run of river) | 0.00 |
| 183 | MOLU ENERJİ (Zamantı-Bahçelik HES) | 2011 | 4.2 | Hydro (run of river) | 30.00 |
| 184 | MURATLI REG. VE HES (ARMAHES EL.) | 2011 | 26.7 | Hydro (run of river) | 94.00 |
| 185 | NARİNKALE REG. VE HES (EBD ENERJİ) | 2011 | 30.4 | Hydro (run of river) | 108.00 |
| 186 | OTLUCA I HES (BEYOBASI ENERJİ ÜR.) | 2011 | 37.5 | Hydro (run of river) | 0.00 |
| 187 | OTLUCA II HES (BEYOBASI ENERJİ ÜR.) | 2011 | 6.4 | Hydro (run of river) | 0.00 |
| 188 | ÖREN REG. VE HES (ÇELİKLER ELEKTRİK) | 2011 | 6.6 | Hydro (run of river) | 16.00 |
| 189 | POYRAZ HES (YEŞİL ENERJİ ELEKTRİK) | 2011 | 2.7 | Hydro (run of river) | 10.00 |
| 190 | SARAÇBENDİ HES (ÇAMLICA ELEKTRİK) | 2011 | 25.5 | Hydro (run of river) | 0.00 |
| 191 | SARIKAVAK HES (ESER ENERJİ YAT. AŞ.) | 2011 | 8.1 | Hydro (run of river) | 0.00 |
| 192 | SAYAN HES (KAREL ELEKTRİK ÜRETİM) | 2011 | 14.9 | Hydro (run of river) | 0.00 |
| 193 | SEFAKÖY HES (PURE ENERJİ ÜRETİM AŞ.) | 2011 | 33.1 | Hydro (run of river) | 0.00 |
| 194 | DAREN HES ELEKTRİK (SEYRANTEPE) | 2011 | 49.7 | Hydro (run of river) | 181.13 |
| 195 | SIZIR (KAYSERİ VE CİVARI EL. T.A.Ş) | 2011 | 5.8 | Hydro (run of river) | 46.00 |
| 196 | SÖĞÜTLÜKAYA (POSOF III) HES | 2011 | 6.1 | Hydro (run of river) | 31.00 |
| 197 | TEFEN HES (AKSU MADENCİLİK SAN.) | 2011 | 33.0 | Hydro (run of river) | 141.00 |
| 198 | TUZTAŞI HES (GÜRÜZ ELEKTRİK ÜR.) | 2011 | 1.6 | Hydro (run of river) | 10.00 |
| 199 | ÜZÜMLÜ HES (AKGÜN ENERJİ ÜRETİM) | 2011 | 11.4 | Hydro (run of river) | 41.00 |
| 200 | YAMAÇ HES (YAMAÇ ENERJİ ÜRETİM A.Ş.) | 2011 | 5.5 | Hydro (run of river) | 0.00 |
| 201 | YAPISAN (KARICA REG. ve DARICA I HES) | 2011 | 13.3 | Hydro (run of river) | 0.00 |
| 202 | YAPRAK II HES (NİSAN ELEKTROMEK.) | 2011 | 10.8 | Hydro (run of river) | 32.00 |
| 203 | YAŞIL HES (YAŞIL ENERJİ ELEKTRİK) | 2011 | 3.8 | Hydro (run of river) | 15.00 |
| 204 | YEDİGÖL REG. VE HES (YEDİGÖL HİDR.) | 2011 | 21.9 | Hydro (run of river) | 77.00 |
| 205 | YEDİGÖZE HES (YEDİGÖZE ELEK.) (İlave) | 2011 | 155.3 | Hydro (run of river) | 425.00 |
| 206 | SARES RES (GARET ENERJİ ÜRETİM) | 2011 | 7.5 | Wind | 0.00 |
| 207 | SEYİTALİ RES (DORUK ENERJİ ELEKTRİK) | 2011 | 30.0 | Wind | 0.00 |
| 208 | SOMA RES (SOMA ENERJİ) (İlave) | 2011 | 36.9 | Wind | 0.00 |
| 209 | SUSURLUK RES (ALANTEK ENERJİ ÜRET.) | 2011 | 45.0 | Wind | 0.00 |
| 210 | ŞAH RES (GALATA WİND ENERJİ LTD. ŞTİ) | 2011 | 93.0 | Wind | 0.00 |
| 211 | TURGUTTEPE RES (SABAŞ ELEKTRİK) | 2011 | 2.0 | Wind | 0.00 |
| 212 | ZİYARET RES (ZİYARET RES ELEKTRİK) | 2011 | 22.5 | Wind | 0.00 |
| 213 | AKRES (AKHİSAR RÜZGAR EN. ELEKT.) | 2011 | 43.8 | Wind | 0.00 |
| 214 | AYVACIK RES (AYRES AYVACIK RÜZG.) | 2011 | 5.0 | Wind | 0.00 |
| 215 | BAKİ ELEKTRİK ŞAMLI RÜZGAR (İlave) | 2011 | 24.0 | Wind | 0.00 |
| 216 | ÇANAKKALE RES (ENERJİ-SA ENERJİ) | 2011 | 29.2 | Wind | 0.00 |
| 217 | ÇATALTEPE RES (ALİZE ENERJİ ELEKTRİK) | 2011 | 16.0 | Wind | 0.00 |
| 218 | İNNORES ELEKTRİK YUNTDAĞ RÜZGAR | 2011 | 10.0 | Wind | 0.00 |
| 219 | KİLLİK RES (PEM ENERJİ A.Ş.) | 2011 | 40.0 | Wind | 0.00 |
| 220 | ACARSOY TERMİK KOM.ÇEV.SANT. (ACARSOY EN.) | 2012 | 50.0 | Natural Gas | 375.00 |
| 221 | AFYON DGKÇ (DEDELİ DOĞALGAZ ELEKTRİK ÜR.) | 2012 | 126.1 | Natural Gas | 945.00 |
| 222 | AGE DOĞALGAZ KOM. ÇEV. SANT. (AGE DENİZLİ) | 2012 | 141.0 | Natural Gas | 1057.00 |
| 223 | AKDENİZ KİMYA SAN. VE TİC. A.Ş. | 2012 | 4.0 | Natural Gas | 30.00 |
| 224 | AKKÖPRÜ (DALAMAN) | 2012 | 115.0 | Hydro (run of river) | 176.00 |
| 225 | AKKÖY II HES (AKKÖY ENERJİ A.Ş.) | 2012 | 229.7 | Hydro (run of river) | 508.00 |
| 226 | AKKÖY-ESPİYE HES (KONİ İNŞAAT SAN. A.Ş.) | 2012 | 8.9 | Hydro (run of river) | 40.00 |
| 227 | AKSA AKRİLİK KİMYA SAN. A.Ş. (İTHAL KÖM.+D.G) | 2012 | 42.5 | Natural Gas | 298.00 |
| 228 | AKSU RES (AKSU TEMİZ ENERJİ ELEKTRİK ÜRETİM) | 2012 | 72.0 | Wind | 0.00 |
| 229 | ALABALIK REG. VE HES SANTRALI I-II (DARBOĞAZ ELK. ÜR. SAN.) | 2012 | 13.8 | Hydro (run of river) | 0.00 |
| 230 | ALES DOĞALGAZ KOM. ÇEV. SANT. (ALES ELEKT.) | 2012 | 49.0 | Natural Gas | 370.00 |
| 231 | ALPASLAN I (ELEKTRİK ÜRETİM A.Ş.) | 2012 | 80.0 | Hydro (run of river) | 0.00 |
| 232 | ALTINYILDIZ MENSUCAT VE KONF. FAB. (Tekirdağ) | 2012 | 5.5 | Natural Gas | 38.00 |
| 233 | ANAK HES (KOR-EN KORKUTELİ ELEK. ÜRET. SAN.) | 2012 | 3.8 | Hydro (run of river) | 9.00 |
| 234 | ARAKLI-1 REG. VE HES(YÜCEYURT ENERJİ ÜRETİM) | 2012 | 13.1 | Hydro (run of river) | 0.00 |
| 235 | ARCA HES (GÜRSU TEMİZ ENERJİ ÜRETİM A.Ş.) | 2012 | 5.5 | Hydro (run of river) | 0.00 |
| 236 | AREL ENERJİ BİYOKÜTLE TESİSİ (AREL ÇEVRE) | 2012 | 2.4 | Biomass | 0.00 |
| 237 | ARPA REG. VE HES (MCK ELEKTRİK ÜRETİM A.Ş.) | 2012 | 32.4 | Hydro (run of river) | 44.00 |
| 238 | ASAŞ ALÜMİNYUM SANAYİ VE TİCARET A.Ş. | 2012 | 8.6 | Natural Gas | 65.00 |
| 239 | ATAKÖY (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 5.5 | Hydro (run of river) | 11.00 |
| 240 | AVCILAR HES (AVCILAR ENERJİ ELEKTRİK ÜRET.) | 2012 | 16.7 | Hydro (run of river) | 28.00 |
| 241 | AYANCIK HES (İLK ELEKTRİK ENERJİ ÜRETİMİ SN.) | 2012 | 15.6 | Hydro (run of river) | 37.00 |
| 242 | AYRANCILAR HES (MURADİYE ELEKTRİK ÜRETİM) | 2012 | 9.3 | Hydro (run of river) | 0.00 |
| 243 | BAĞIŞTAŞ II HES (AKDENİZLİ ELEKTRİK ÜRETİM) | 2012 | 32.4 | Hydro (run of river) | 69.00 |
| 244 | BALIKESİR RES (BARES ELEKTRİK ÜRETİM A.Ş.) | 2012 | 30.3 | Wind | 0.00 |
| 245 | BALIKESİR RES (ENERJİSA ENERJİ ÜRETİM A.Ş.) | 2012 | 82.5 | Wind | 0.00 |
| 246 | BALKUSAN BARAJI VE HES 1 NOLU SANT. (KAREN) | 2012 | 13.0 | Hydro (run of river) | 0.00 |
| 247 | BALKUSAN BARAJI VE HES 2 NOLU SANT. (KAREN) | 2012 | 25.0 | Hydro (run of river) | 0.00 |
| 248 | BALSUYU MENSUCAT SAN. VE TİC. A.Ş. | 2012 | 9.7 | Natural Gas | 68.00 |
| 249 | BAMEN KOJENERASYON (BAŞYAZICIOĞLU TEKSTİL) | 2012 | 2.1 | Natural Gas | 14.00 |
| 250 | BANDIRMA RES (YAPISAN ELEKTRİK ÜRETİM A.Ş.) | 2012 | 5.0 | Wind | 0.00 |
| 251 | BANGAL REG. VE KUŞLUK HES (KUDRET ENERJİ) | 2012 | 17.0 | Hydro (run of river) | 32.00 |
| 252 | BEKTEMUR HES (DİZ-EP ELEKTRİK ÜRETİM LTD.) | 2012 | 3.5 | Hydro (run of river) | 11.00 |
| 253 | BEREKET ENERJİ ÜRETİM A.Ş. (BİOGAZ) | 2012 | 0.6 | Biogas | 5.00 |
| 254 | BEYKÖY (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 16.8 | Hydro (run of river) | 87.00 |
| 255 | BEYPİ BEYPAZARI TARIMSAL ÜRETİM PZ. SN. A.Ş. | 2012 | 8.6 | Natural Gas | 63.00 |
| 256 | BİLECİK DOĞALGAZ ÇS. (TEKNO DOĞALGAZ ÇEV.) | 2012 | 25.8 | Natural Gas | 190.00 |
| 257 | BİLECİK DOĞALGAZ KÇS. (DEDELİ DOĞALGAZ EL.) | 2012 | 126.1 | Natural Gas | 945.00 |
| 258 | BİLKUR TEKSTİL BOYA TİC. A.Ş. | 2012 | 2.0 | Natural Gas | 14.00 |
| 259 | BİNATOM ELEKTRİK ÜRETİM A.Ş. (Emet/KÜTAHYA) | 2012 | 10.4 | Natural Gas | 78.00 |
| 260 | BİS ENERJİ(Sanayi/ Bursa) | 2012 | 48.0 | Natural Gas | 361.00 |
| 261 | BOSEN ENERJİ ELEKTRİK ÜRETİM AŞ.(Bursa) | 2012 | 27.9 | Natural Gas | 210.00 |
| 262 | BOYABAT BARAJI VE HES (BOYABAT ELEKTRİK) | 2012 | 513.0 | Hydro (run of river) | 830.00 |
| 263 | BOZYAKA RES (KARDEMİR HADDECİLİK VE ELEKT.) | 2012 | 12.0 | Wind | 32.00 |
| 264 | BÜYÜKDÜZ HES (AYEN ENERJİ A.Ş.) | 2012 | 68.9 | Hydro (run of river) | 192.00 |
| 265 | CAN 1 HES (HED ELEKTRİK ÜRETİM A.Ş.) | 2012 | 1.8 | Hydro (run of river) | 6.00 |
| 266 | CEYHAN HES (BERKMAN HES) (ENOVA EN ÜRET.) | 2012 | 12.6 | Hydro (run of river) | 31.00 |
| 267 | CUNİŞ REG. VE HES (RİNERJİ RİZE ELEKTRİK ÜR.) | 2012 | 8.4 | Hydro (run of river) | 21.00 |
| 268 | ÇAĞLAYAN HES (ÇAĞLAYAN HES ENERJİ ÜRETİM) | 2012 | 6.0 | Hydro (run of river) | 12.00 |
| 269 | ÇARŞAMBA HES (ÇARŞAMBA ENERJİ ELEKTRİK) | 2012 | 11.3 | Hydro (run of river) | 36.00 |
| 270 | ÇILDIR (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 15.4 | Natural Gas | 20.00 |
| 271 | ÇINAR-1 HES (AYCAN ENERJİ ÜRETİM TİC. VE SN.) | 2012 | 9.3 | Hydro (run of river) | 19.00 |
| 272 | ÇUKURÇAYI HES (AYDEMİR ELEKTRİK ÜRETİM A.Ş.) | 2012 | 1.8 | Hydro (run of river) | 2.00 |
| 273 | DAĞPAZARI RES (ENERJİSA ENERJİ ÜRETİM A.Ş.) | 2012 | 39.0 | Wind | 0.00 |
| 274 | DEMİRCİLER HES (PAK ENERJİ ÜRETİMİ SAN.) | 2012 | 8.4 | Hydro (run of river) | 0.00 |
| 275 | DENİZ JEOTERMAL (MAREN MARAŞ ELEKTRİK) | 2012 | 24.0 | Geothermal | 0.00 |
| 276 | DENİZLİ JEOTERMAL (ZORLU DOĞAL ELEK. ÜR.A.Ş.) | 2012 | 15.0 | Geothermal | 105.00 |
| 277 | DİNAR RES (OLGU ENERJİ YATIRIM ÜRETİM) | 2012 | 16.1 | Wind | 51.00 |
| 278 | DOĞANKAYA HES (MAR-EN ENERJİ ÜRET. TİC.) | 2012 | 20.6 | Hydro (run of river) | 56.00 |
| 279 | DUMLU HES (DUMLU ENERJİ ELEKTRİK ÜRETİM) | 2012 | 4.0 | Hydro (run of river) | 5.00 |
| 280 | DURMAZLAR MAKİNA SANAYİ VE TİCARET A.Ş. | 2012 | 1.3 | Natural Gas | 10.00 |
| 281 | DURUM GIDA TERMİK KOJEN. SANT. (DURUM GIDA) | 2012 | 3.6 | Natural Gas | 29.00 |
| 282 | EGE SERAMİK ENERJİ SANTRALI | 2012 | 13.1 | Natural Gas | 90.00 |
| 283 | EGER HES (EGER ELEKTRİK ÜRETİM LTD. ŞTİ.) | 2012 | 1.9 | Hydro (run of river) | 6.00 |
| 284 | EKİM BİYOGAZ (EKİM GRUP ELEKTRİK ÜRETİM) | 2012 | 1.2 | Biogas | 10.00 |
| 285 | ENERJİ-SA (ÇANAKKALE) | 2012 | 0.9 | Wind | 0.00 |
| 286 | ENERJİ-SA (KÖSEKÖY) | 2012 | 120.0 | Natural Gas | 930.00 |
| 287 | ENERJİ-SA (MERSİN) | 2012 | 1.4 | Natural Gas | 11.00 |
| 288 | ERDEMİR(F.O+K.G+Y.F.G+DG)(Ereğli-Zonguldak) | 2012 | 53.9 | Natural Gas | 355.00 |
| 289 | EREN ENERJİ ELEKTRİK ÜRETİM A.Ş. | 2012 | 30.0 | Imported coal | 195.00 |
| 290 | ERİK HES (ELEKTRİK ÜRETİM A.Ş.) | 2012 | 6.5 | Hydro (run of river) | 21.00 |
| 291 | ERMENEK (ELEKTRİK ÜRETİM A.Ş.) | 2012 | 302.4 | Hydro (run of river) | 1187.00 |
| 292 | ERZURUM MEYDAN AVM (REDEVKO BİR EMLAK) | 2012 | 2.4 | Natural Gas | 16.00 |
| 293 | ES ES ESKİŞEHİR ENERJİ SAN. VE TİC. A.Ş. | 2012 | 2.0 | Biogas | 15.00 |
| 294 | ESENDURAK HES (MERAL ELEKTRİK ÜRETİM) | 2012 | 9.3 | Hydro (run of river) | 0.00 |
| 295 | FEKE 1 HES (AKKUR ENERJİ ÜRETİM TİC. VE SAN.) | 2012 | 29.4 | Hydro (run of river) | 0.00 |
| 296 | FEKE 2 BARAJI VE HES (AKKUR ENERJİ ÜRETİM) | 2012 | 69.3 | Hydro (run of river) | 0.00 |
| 297 | FINDIK I HES (ADV ELEKTRİK ÜRETİM LTD. ŞTİ.) | 2012 | 11.3 | Hydro (run of river) | 27.00 |
| 298 | GOODYEAR (İzmit/Köseköy) | 2012 | 5.2 | LPG | 35.00 |
| 299 | GÖKGEDİK HES (UHUD ENERJİ ÜRETİM TİC.) | 2012 | 24.3 | Hydro (run of river) | 75.00 |
| 300 | GÖKNUR GIDA MAD. EN. İM. İT. İH. TİC. VE SAN. AŞ. | 2012 | 1.6 | Imported coal | 6.00 |
| 301 | GÜDÜL 2 HES (YAŞAM ENERJİ ELEKTRİK ÜRETİM) | 2012 | 4.9 | Hydro (run of river) | 15.00 |
| 302 | GÜLLÜBAĞ BARAJI VE HES (SENENERJİ ENERJİ) | 2012 | 96.0 | Hydro (run of river) | 280.00 |
| 303 | GÜNAYDIN RES (MANRES ELEKTRİK ÜRETİM A.Ş.) | 2012 | 10.0 | Wind | 0.00 |
| 304 | GÜNDER REG. VE HES (ARIK ENERJİ ÜRETİM A.Ş.) | 2012 | 28.2 | Hydro (run of river) | 0.00 |
| 305 | GÜRTEKS İPLİK SANAYİ VE TİCARET A.Ş. | 2012 | 6.7 | Natural Gas | 53.00 |
| 306 | HATİPOĞLU PLASTİK YAPI ELEMANLARI SAN. | 2012 | 2.0 | Natural Gas | 14.00 |
| 307 | HORU REG. VE HES (MARAŞ ENERJİ YATIRIM SN.) | 2012 | 8.5 | Hydro (run of river) | 25.00 |
| 308 | HORYAN HES (HORYAN ENERJİ A.Ş.) | 2012 | 5.7 | Hydro (run of river) | 15.00 |
| 309 | ITC ADANA ENERJİ ÜRETİM (ADANA BİOKÜTLE SNT) | 2012 | 4.2 | Waste | 35.00 |
| 310 | ITC BURSA ENERJİ ÜRETİM SAN. VE TİC. A.Ş. | 2012 | 9.8 | Waste | 37.00 |
| 311 | İKİZDERE (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 18.6 | Hydro (run of river) | 100.00 |
| 312 | İNNORES ELEKTRİK YUNTDAĞ RÜZGAR (Aliağa-İZMİR) | 2012 | 5.0 | Wind | 0.00 |
| 313 | İŞBİRLİĞİ ENERJİ ÜRETİM SAN. VE TİC. A.Ş. | 2012 | 19.5 | Natural Gas | 146.00 |
| 314 | İZAYDAŞ (İZMİT ÇÖP)(Köseköy) | 2012 | 0.3 | Waste | 2.00 |
| 315 | İZMİR BÜYÜK EFES OTELİ KOJENERASYON TES. | 2012 | 1.2 | Natural Gas | 9.00 |
| 316 | JTI TORBALI KOJENERASYON SANTR. (JTI TÜTÜN) | 2012 | 4.0 | Natural Gas | 30.00 |
| 317 | KARADAĞ RES (GARET ENERJİ ÜRETİM) | 2012 | 10.0 | Wind | 0.00 |
| 318 | KARTALKAYA HES (SIR ENERJİ ÜRETİM SAN.) | 2012 | 8.0 | Hydro (run of river) | 15.00 |
| 319 | KAYADÜZÜ RES (BAKTEPE ENERJİ A.Ş.) | 2012 | 39.0 | Wind | 0.00 |
| 320 | KAYAKÖPRÜ 2 HES (ARSAN ENERJİ A.Ş.) | 2012 | 10.2 | Hydro (run of river) | 36.00 |
| 321 | KAYSERİ KATI ATIK DEPONİ SAHASI (HER ENERJİ) | 2012 | 1.4 | Waste | 10.00 |
| 322 | KESKİNOĞLU TAVUKÇULUK VE DAMIZLIK İŞLET. | 2012 | 6.0 | Natural Gas | 45.00 |
| 323 | KILAVUZLU HES (ELEKTRİK ÜRETİM A.Ş.) | 2012 | 40.5 | Hydro (run of river) | 150.00 |
| 324 | KIRIKDAĞ HES (ÖZENİR ENERJİ ELEKTRİK ÜRET.) | 2012 | 16.9 | Hydro (run of river) | 40.00 |
| 325 | KIVANÇ TEKSTİL SAN.ve TİC.A.Ş. | 2012 | 2.1 | Natural Gas | 11.00 |
| 326 | KOCAELİ ÇÖP BİYOGAZ (LFG) (KÖRFEZ ENERJİ) | 2012 | 2.3 | Waste | 18.00 |
| 327 | KOZBEYLİ RES (DOĞAL ENERJİ ELEKTRİK ÜRETİM) | 2012 | 20.0 | Wind | 60.00 |
| 328 | KOZDERE HES (ADO MADENCİLİK ELEKTRİK ÜR.) | 2012 | 6.1 | Hydro (run of river) | 5.00 |
| 329 | KÖKNAR HES (AYCAN ENERJİ ÜRETİM TİC.) | 2012 | 8.0 | Hydro (run of river) | 15.00 |
| 330 | KUZGUN (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 20.9 | Hydro (run of river) | 0.00 |
| 331 | KÜÇÜKER TEKSTİL SAN. VE TİC. A.Ş. | 2012 | 5.0 | Lignite | 40.00 |
| 332 | KÜRCE REG. VE HES (DEDEGÖL ENERJİ) | 2012 | 12.0 | Hydro (run of river) | 36.00 |
| 333 | MENGE BARAJI VE HES (ENERJİSA ENERJİ) | 2012 | 44.7 | Hydro (run of river) | 58.00 |
| 334 | MERCAN (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 20.4 | Hydro (run of river) | 78.00 |
| 335 | METRİSTEPE RES (CAN ENERJİ ENTEGRE ELEKT.) | 2012 | 39.0 | Wind | 0.00 |
| 336 | MİDİLLİ REG. VE HES (MASAT ENERJİ ELEKTRİK) | 2012 | 20.9 | Hydro (run of river) | 45.00 |
| 337 | MURAT I-II REG. VE HES (MURAT HES ENERJİ EL.) | 2012 | 35.6 | Hydro (run of river) | 107.00 |
| 338 | MURATLI REG. VE HES (ARMAHES ELEKTRİK ÜR.) | 2012 | 11.0 | Hydro (run of river) | 17.00 |
| 339 | MURSAL I HES (PETA MÜHENDİSLİK ENERJİ) | 2012 | 4.2 | Hydro (run of river) | 13.00 |
| 340 | MUTLU MAKARNACILIK SANAYİ VE TİCARET A.Ş.) | 2012 | 2.0 | Natural Gas | 18.00 |
| 341 | NAKSAN ENERJİ ELEKTRİK ÜRETİM A.Ş. | 2012 | 16.0 | Natural Gas | 120.00 |
| 342 | NİKSAR HES (NİKSAR ENERJİ ÜRETİM LTD. ŞTİ.) | 2012 | 40.2 | Hydro (run of river) | 140.00 |
| 343 | ODAŞ DOĞALGAZ KÇS (ODAŞ ELEKTRİK ÜRETİM) | 2012 | 128.2 | Natural Gas | 450.00 |
| 344 | OFİM ENERJİ SANTRALI (OSTİM FİNANS VE İŞ MER.) | 2012 | 2.1 | Natural Gas | 16.00 |
| 345 | ORTADOĞU ENERJİ (KÖMÜRCÜODA) (Şile/İSTANBUL) | 2012 | 2.8 | Waste | 17.00 |
| 346 | ORTADOĞU ENERJİ (ODA YERİ) (Eyüp/İSTANBUL) | 2012 | 4.1 | Waste | 22.00 |
| 347 | ÖREN REG. VE HES (ÇELİKLER ELEKTRİK ÜRETİM) | 2012 | 19.9 | Hydro (run of river) | 12.00 |
| 348 | ÖZMAYA SANAYİ A.Ş. | 2012 | 5.4 | Hydro (run of river) | 40.00 |
| 349 | PAMUKOVA YEN. EN. VE ELEK. ÜR. A.Ş. | 2012 | 1.4 | Waste | 0.00 |
| 350 | PANCAR ELEKTRİK ÜRETİM A.Ş. | 2012 | 34.9 | Natural Gas | 731.00 |
| 351 | PAPART HES (ELİTE ELEKTRİK ÜRETİM) | 2012 | 26.6 | Hydro (run of river) | 80.00 |
| 352 | PİSA TEKSTİL VE BOYA FABRİKALARI (İstanbul) | 2012 | 1.0 | Natural Gas | 7.00 |
| 353 | POLAT HES (ELESTAŞ ELEKTRİK ÜRETİM A.Ş.) | 2012 | 6.6 | Hydro (run of river) | 20.00 |
| 354 | POYRAZ RES (POYRAZ ENERJİ ELEKTRİK ÜRETİM) | 2012 | 50.0 | Wind | 0.00 |
| 355 | SAMSUN AVDAN KATI ATIK (SAMSUN AVDAN EN.) | 2012 | 2.4 | Waste | 18.00 |
| 356 | SAMURLU RES (DOĞAL ENERJİ ELEKTRİK ÜRET.) | 2012 | 22.0 | Hydro (run of river) | 60.00 |
| 357 | SARIHIDIR HES (MOLU ENERJİ ÜRETİM A.Ş.) | 2012 | 6.0 | Hydro (run of river) | 18.00 |
| 358 | SELÇUK İPLİK SAN. VE TİC. A.Ş. | 2012 | 8.6 | Natural Gas | 65.00 |
| 359 | SELVA GIDA SAN. A.Ş. | 2012 | 1.7 | Natural Gas | 14.00 |
| 360 | SEYRANTEPE HES (SEYRANTEPE ELEKT. ÜRET.) | 2012 | 56.8 | Hydro (run of river) | 161.00 |
| 361 | SEZER BİO ENERJİ (KALEMİRLER ENERJİ ELEKTR.) | 2012 | 0.5 | Waste | 4.00 |
| 362 | SIRAKONAKLAR HES (2M ENERJİ ÜRETİM A.Ş.) | 2012 | 18.0 | Hydro (run of river) | 39.00 |
| 363 | SİNEM JEOTERMAL (MAREN MARAŞ ELEKTRİK) | 2012 | 24.0 | Geothermal | 191.00 |
| 364 | SODA SANAYİ A.Ş. (Mersin) | 2012 | 252.2 | Natural Gas | 1765.00 |
| 365 | SOMA RES (SOMA ENERJİ ELEKTRİK ÜRETİM A.Ş.) | 2012 | 24.0 | Wind | 0.00 |
| 366 | SÖKE-ÇATALBÜK RES (ABK ENERJİ ELEKTRİK) | 2012 | 18.0 | Wind | 0.00 |
| 367 | SÖKE-ÇATALBÜK RES (ABK ENERJİ ELEKTRİK) | 2012 | 12.0 | Wind | 0.00 |
| 368 | SULUKÖY HES (DU ELEKTRİK ÜRETİM A.Ş.) | 2012 | 6.9 | Hydro (run of river) | 18.00 |
| 369 | ŞANLIURFA OSB (RASA ENERJİ ÜRETİM A.Ş.) | 2012 | 11.7 | Natural Gas | 82.00 |
| 370 | ŞENKÖY RES (EOLOS RÜZGAR ENERJİSİ ÜRETİM) | 2012 | 26.0 | Wind | 0.00 |
| 371 | ŞİFRİN REG. VE HES (BOMONTİ ELK. MÜH. MÜŞ.) | 2012 | 6.7 | Hydro (run of river) | 10.00 |
| 372 | TELEME REG. VE HES (TAYEN ELEKTRİK ÜRET.) | 2012 | 1.6 | Hydro (run of river) | 6.00 |
| 373 | TELLİ I-II HES (FALANJ ENERJİ ELEKTRİK ÜRET.) | 2012 | 8.7 | Hydro (run of river) | 18.00 |
| 374 | TERCAN (ZORLU DOĞAL ELEKTRİK ÜRETİMİ A.Ş.) | 2012 | 15.0 | Hydro (run of river) | 28.00 |
| 375 | TRAKYA YENİŞEHİR CAM SAN. A.Ş. | 2012 | 6.0 | Biogas | 45.00 |
| 376 | TUĞRA REG. VE HES (VİRA ELEKTRİK ÜRETİM A.Ş.) | 2012 | 4.9 | Hydro (run of river) | 10.00 |
| 377 | TUNA HES (NİSAN ELEKTROMEKANİK ENERJİ) | 2012 | 37.2 | Hydro (run of river) | 0.00 |
| 378 | TUZKÖY HES (BATEN ENERJİ ÜRETİMİ A.Ş.) | 2012 | 8.4 | Hydro (run of river) | 0.00 |
| 379 | TUZLAKÖY-SERGE REG. VE HES (TUYAT ELEKT.) | 2012 | 7.1 | Hydro (run of river) | 0.00 |
| 380 | UMUT I REG. VE HES (NİSAN ELEKTROMEKANİK) | 2012 | 5.8 | Hydro (run of river) | 0.00 |
| 381 | ÜÇKAYA HES (ŞİRİKÇİOĞLU ELEKTRİK ÜRETİM A.Ş.) | 2012 | 1.0 | Hydro (run of river) | 3.00 |
| 382 | VİZARA REG. VE HES (ÖZTÜRK ELEKT. ÜRET. LTD.) | 2012 | 8.6 | Hydro (run of river) | 0.00 |
| 383 | YAĞMUR REG. VE HES (BT BORDO ELK. ÜR.) | 2012 | 8.9 | Hydro (run of river) | 0.00 |
| 384 | YAMANLI III KAPS. GÖKKAYA HES (MEM ENERJİ) | 2012 | 28.5 | Hydro (run of river) | 0.00 |
| 385 | YAMANLI III KAPS. HİMMETLİ HES (MEM ENERJİ) | 2012 | 27.0 | Hydro (run of river) | 0.00 |
| 386 | YAVUZ HES (AREM ENERJİ ÜRETİM A.Ş.) | 2012 | 5.8 | Hydro (run of river) | 0.00 |
| 387 | YEDİSU HES (ÖZALTIN ENERJİ ÜRETİM VE İNŞAAT) | 2012 | 22.7 | Hydro (run of river) | 41.00 |
| 388 | YENİ UŞAK ENERJİ ELEKTRİK SANTRALI | 2012 | 9.7 | Natural Gas | 62.00 |
| 389 | YILDIRIM HES (BAYBURT ENERJİ ÜRETİM VE TİC.) | 2012 | 10.7 | Hydro (run of river) | 22.00 |
| 390 | YOKUŞLU KALKANDERE HES (SANKO ENERJİ) | 2012 | 5.2 | Hydro (run of river) | 0.00 |
| 391 | YONGAPAN (KASTAMONU ENTEGRE)(D.İskelesi) | 2012 | 15.0 | Natural Gas | 90.00 |
| 392 | ZORLU ENERJİ (B.Karıştıran) | 2012 | 25.7 | Natural Gas | 195.00 |
| 393 | YAĞMUR REG. VE HES (BT BORDO ELK. ÜR.) | 2012 | 8.9 | Hydro (run of river) | 0.00 |
| 394 | YAMANLI III KAPS. GÖKKAYA HES (MEM ENERJİ) | 2012 | 28.5 | Hydro (run of river) | 0.00 |
| 395 | YAMANLI III KAPS. HİMMETLİ HES (MEM ENERJİ) | 2012 | 27.0 | Hydro (run of river) | 0.00 |
| 396 | YAVUZ HES (AREM ENERJİ ÜRETİM A.Ş.) | 2012 | 5.8 | Hydro (run of river) | 0.00 |
| 397 | YEDİSU HES (ÖZALTIN ENERJİ ÜRETİM VE İNŞAAT) | 2012 | 22.7 | Hydro (run of river) | 41.00 |
| 398 | YENİ UŞAK ENERJİ ELEKTRİK SANTRALI | 2012 | 9.7 | Natural Gas | 62.00 |
| 399 | YILDIRIM HES (BAYBURT ENERJİ ÜRETİM VE TİC.) | 2012 | 10.7 | Hydro (run of river) | 22.00 |
| 400 | YOKUŞLU KALKANDERE HES (SANKO ENERJİ) | 2012 | 5.2 | Hydro (run of river) | 0.00 |
| 401 | YONGAPAN (KASTAMONU ENTEGRE)(D.İskelesi) | 2012 | 15.0 | Natural Gas | 90.00 |
| 402 | ZORLU ENERJİ (B.Karıştıran) | 2012 | 25.7 | Natural Gas | 195.00 |

Further background information on ex ante calculation of emission reductions

Further background information on monitoring plan

Summary of post registration changes

- - - - -

Document information

| Version | Date | Description |
| --- | --- | --- |
| 05.0 | 25 June 2014 | Revisions to: - Include the Attachment: Instructions for filling out the project design document form for CDM project activities (these instructions supersede the "Guidelines for completing the project design document form" (Version 01.0)); - Include provisions related to standardized baselines; - Add contact information on a responsible person(s)/ entity(ies) for the application of the methodology (ies) to the project activity in B.7.4 and Appendix 1; - Change the reference number from F-CDM-PDD to CDM-PDD-FORM; - Editorial improvement. |
| 04.1 | 11 April 2012 | Editorial revision to change version 02 line in history box from Annex 06 to Annex 06b |
| 04.0 | 13 March 2012 | Revision required to ensure consistency with the “Guidelines for completing the project design document form for CDM project activities” (EB 66, Annex 8). |
| 03.0 | 26 July 2006 | EB 25, Annex 15 |
| 02.0 | 14 June 2004 | EB 14, Annex 06b |
| 01.0 | 03 August 2002 | EB 05, Paragraph 12 Initial adoption. |
| Decision Class: Regulatory Document Type: Form Business Function: Registration Keywords: project activities, project design document |  |  |

Silivri Enerji A.Ş. License Page 1

Silivri Wind Data Analyses and Energy Production Generation Assessment Report Page 10

See; Silivi WPP Generation License

See; ACM0002 Version 16:

(https://cdm.unfccc.int/filestorage/0/X/6/0X6IERWMG92J7V3B8OTKFSL1QZH5PA/EB81_repan09_ACM0002_ver16.0_clean.pdf?t=Nkh8bmo0eXk4fDDbh2eGA5FSFCOJDpsfh5Aa)

See, Licence. Turbines are T1, T2, T3, T4, T5, 56, T7, T8, T9, T10, T21, T12, T13, T14, T15, T16, T17 and T18 in order.

See, (Convert UTM to Lat/Lon Coordinates), for unit conservation See; http://www.rcn.montana.edu/resources/tools/coordinates.aspx?nav=11&c=UTM&md=83&mdt=NAD83/WGS84&z=35&e=591938&n=4469295&h=N

Silivri Enerji A.Ş. License Page 1

Silivri Wind Data Analyses and Energy Production Generation Assessment Report Page 10

See; http://www.nordex-online.com/fileadmin/MEDIA/Gamma/Nordex_Gamma_en.pdf

See, http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

See, Silivri WPP Generation License

See, Annual Development of Turky’s Gross Electricity Generation of Primary Energy Resources (2006-2013): http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/uretim%20tuketim(23-47)/37(06-13).xls

See, The Distribution of Installed Capacity by Primary Energy Resources and The Electricity Utilities in Turkey (2013): www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/kguc(1-13)/8.xls

See, Presentation of Zeynep  Günaydın from MENR, http://www.senternovem.nl/mmfiles/MENR_tcm24-287950.pdf  (Page 9)

See : For feed-in-tariff (5.5 Eur/kWh) see, http://www.epdk.org.tr/TR/Dokuman/1859 (page 2-footnote 1)the Law stating the electricity selling price is revised and the new law is : http://www.epdk.org.tr/TR/Dokuman/6777 However, the price has not been changed in the Law, so the 5.5 €/kWh, which is stated in the former Law is still valid. This explanation is also given with the footnotes in the IRR Excel and PDD.

ACM0002 Version 16: (https://cdm.unfccc.int/filestorage/0/X/6/0X6IERWMG92J7V3B8OTKFSL1QZH5PA/EB81_repan09_ACM0002_ver16.0_clean.pdf?t=Ykx8bmo1MG5nfDBt9CgQqHHHPLb9pKkRE10Z)

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v4.0.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-01-v7.0.0.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-v5.0.0.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-03-v2.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

See, http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf   (Page 6, Table 1)

See, http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf  (Page 18-19, Table 7 for High and Table 8 for Low Scenarios)

See, http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf (Page 44, Table 26)

See, http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/uretim%20tuketim(23-47)/38.xls

See, http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf  (Page 72)

Version 7, http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-01-v7.0.0.pdf  (Page7)

See, http://www.emra.org.tr/EN/Document/6654

See, http://www.emra.org.tr/EN/Document/6659

See, http://www2.cevreorman.gov.tr/yasa/k/2872.doc (Enactment Date: 1983)

See, http://cdm.unfccc.int/Reference/Guidclarif/reg/reg_guid03.pdf

Republic of Turkey, Ministry of Development Website. Main Economic Indicators.Section 7, 13th Excel Sheet. Table 7.13- Lending and Deposit Interest Rates. Published for the second quarter of 2016, but the rate has been taken for the investment decision taken date, which is 15/03/2013

http://www.kalkinma.gov.tr/Pages/TemelEkonomikGostergeler.aspx

- See; Power Production Assesment

See, http://cdm.unfccc.int/Reference/Guidclarif/reg/reg_guid03.pdf (Page 3)

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

For feed-in-tariff (5.5 Eur/kWh) see, http://www.epdk.org.tr/TR/Dokuman/1859 (page 2-footnote 1)the Law stating the electricity selling price is revised and the new law is : http://www.epdk

.org.tr/TR/Dokuman/6777 However, the price has not been changed in the Law, so the 5.5 €/kWh, which is stated in the former Law is still valid. This explanation is also given with the footnotes in the IRR Excel and PDD.

See, http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v4.0.pdf (Version 04)

See, http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v4.0.pdf

See, http://www.ongurergan.av.tr/en-EN/mevzuat/Electric%20Market%20Licensing%20Regulation.doc  (Page 21)

See, http://www.teias.gov.tr/YayinRapor/APK/projeksiyon/KAPASITEPROJEKSIYONU2013.pdf

See: http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2012/uretim%20tuketim(23-47)/37(06-12).xls

See;  http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2012/uretim%20tuketim(23-47)/37(06-12).xls

For Net Production See, www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2013/uretim%20tuketim(23-47)/34(84-13).xls

See, http://www.emra.org.tr/EN/Document/6656

See, http://www.epdk.org.tr/TR/Dokuman/3156

See, http://www.mevzuat.gov.tr/Metin.Aspx?MevzuatKod=7.5.6381&MevzuatIliski=0&sourceXmlSearch=

See, http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

See, Silivri WPP Generation License

See; http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2012/yak%C4%B1t48-53/51.xls

See; http://www.teias.gov.tr/T%C3%BCrkiyeElektrik%C4%B0statistikleri/istatistik2012/yak%C4%B1t48-53/49.xls