Project Name:		Solar cookstoves project in Bolivia

Monitoring Report:		1st January 2013 – 31st December 2013.

Project Number:		GS813 (Gold Standard Registry).

Project start date:		January 2007 (Credits claimed from August 2009).

Registration date:		11th August 2011

Document Versions:		Version 2

Date: 2nd July 2015

Contact Details:     		Fondation GOODPLANET

ActionCarbone Programme,

1, Carrefour Longchamp,

Paris, FRANCE – 75116.

www.actioncarbone.org

Bolivia Inti Sud Soleil – Asociacion Inti Illimai

Calle Fernando Guachalla N. 790

Sopocachi,

La Paz – Bolivia.

http://www.boliviainti-sudsoleil.org/

http://asointiillimani.wordpress.com

Contents

- Introduction.

- Monitoring Surveys/ Field Tests–

a) Kitchen Survey.

b) Kitchen Test.

c) Follow-up meetings with the beneficiaries

- Sustainable development monitoring parameters.

- Emission reduction monitoring parameters.

- Emission Reductions Calculations.

- Annexe I - Kitchen survey scanned copy.

- Annexe II – Kitchen test scanned copy

- Annexe III - Scanned copy of test at the end of the workshop.

- Annexe IV - Scanned copies the follow-up meetings document.

- Glossary.

Introduction:

The project activity aims to provide solar cookstoves to families in Bolivia. The availability of high incidence of solar light and high altitude in the country makes the solar energy as one of the alternative source of fuel for daily energy needs. Since its implementation, the project activity is being regularly monitored, which has shown that the solar cook-stoves not only meets the daily cooking energy needs of the beneficiaries but has also benefited in other ways- such as income saving, health benefits due to safe drinking water (boiling water) etc. The project is implemented in partnership with the two French organizations namely- Fondation GoodPlanet, Bolivia Inti Sud Soliel (henceforth BISS) and its Bolivian partner, the Inti-Illimani team (henceforth “II”).

The purpose of the project is also to slow down the deforestation rate and the use of fossil fuels in Bolivia, and improving the living conditions of the women by promoting the use of a clean energy source like the solar energy. The use solar cookstoves has partly replaced fuels like – firewood, LPG and dung for the daily cooking needs of the project population. The project clearly demonstrates the sustainable development in the project areas through parameters like – time saving from the collection of firewood, income-savings occurring from less consumption of LPG (some beneficiaries had to travel for more than 2 hours to purchase the LPG tank), health improvement especially among the women, due to the avoidance of the smoke emissions from firewood, which is the potential cause for lung infections. And finally, the slow and gradual cooking process of the solar stoves preserves most the food nutrients while cooking.

Implementation of the project:

The II team disseminates the solar cookstoves through construction workshops for group of beneficiaries, and each group consists of a maximum of 25 beneficiaries. In each workshop, the beneficiaries build their own solar cookstoves under the technical guidance of the II teams. The II team arranges these construction workshops in the same villages/areas of the beneficiaries and preferably held in a village community hall or in an open public space. These workshops usually last for 4-5 days, where the beneficiaries are provided with all the materials that are required for the construction of the solar stoves.

Similar kinds of workshops are held throughout the year as per the demand for the solar cookers, which has been observed to be increasing every year and therefore, these workshops are carried out with an advanced planning. The II team follows-up each of these groups on a regular basis after these construction workshops, to assure that the solar cookers are being regularly used for daily cooking needs and have not faced any problems during.

One the workshops carried out for a group by the II technical experts and the project coordinators.

Monitoring surveys/ Field tests/Follow-up meetings:

a) Kitchen Survey:

The Kitchen surveys were conducted as per the guidelines given in the GS cookstove methodology. Since the solar cookstoves were disseminated throughout the year, the kitchen surveys were carried out during each workshop. At least 5 beneficiaries, out of the total (maximum 20 participants) present in the workshop, were asked the survey questions before the use of the solar cookers and the same 5 beneficiaries were followed up after the construction courses. The kitchen survey data was collected and updated regularly by the II team members. As per the kitchen survey data, the clusters are accordingly defined for the project activity.

The pattern of clusters has changed since the year 2013 with the evolving baseline surveys. The previous clusters defined were the LPG cluster and LPG Firewood (LPG Fw) cluster. During the present monitoring period, a new cluster has been defined as per the survey data collected for the evolving baseline – LPG Fw Dung cluster. All the survey data have been submitted to the Gold Standard for further verification. The following table provides the required data for the 3 clusters.

Table 1: Number of Solar Cookers distributed

| Year | LPG | *LPG + Fw | *LPG + Fw + Dung | Others | Total |
| --- | --- | --- | --- | --- | --- |
| 2007 | 235 | 146 | 0 | 20 | 401 |
| 2008 | 233 | 125 | 0 | 62 | 420 |
| 2009 | 235 | 85 | 0 | 80 | 400 |
| 2010 | 386 | 119 | 0 | 77 | 582 |
| 2011 | 425 | 233 | 0 | 2 | 660 |
| 2012 | 197 | 456 | 0 | 7 | 660 |
| 2013 | 170 | 132 | 196 | 2 | 500 |
| Total | 1881 | 1296 | 196 | 250 | 3623 |

*LPG Fw – LPG and firewood cluster.

Since 2007, a total of 3623 solar stoves have been disseminated through the workshops. The II team has carried out a total of 181 workshops in the different regions of Bolivia.

Considering the usage rate per year, a total of 2742 out of the 3373 solar stoves from the 3 clusters – LPG, LPG Fw and LPG Fw dung have been taken into account for the ER’s calculations for the year 2013.

b) Kitchen Tests:

The Kitchen tests were carried out for each of the above-mentioned clusters as per the guidelines given in the GS methodology. The results for the year 2013 are as follows:

| Cluster | LPG consumption without SC kg/day/hh | Firewood consumption without SC kg/day/hh | Dung consumption without SC kg/day/hh | LPG consumption with SC kg/day/hh | Firewood consumption with SC kg/day/hh | Dung consumption with SC kg/day/hh |
| --- | --- | --- | --- | --- | --- | --- |
| Average LPG | 0.50 | - | - | 0.31 | - | - |
| Low IC LPG | 0.35 | - | - | 0.22 | - | - |
| Average LPG Fw | 0.37 | 6.21 | - | 0.33 | 1.44 | - |
| Low IC LPG Fw | 0.25 | 4.35 | - | 0.23 | 1.01 | - |
| Average LPG Fw Dung | 0.30 | - | 4.79 | 0.29 | - | 1.04 |
| Low IC LPG Fw Dung | 0.21 | - | 3.35 | 0.20 | - | 0.73 |

Selection of the sample for the Kitchen tests:

Following considerations have been taken into account for KT:

Paired samples have been taken to compare pre- and post-installation consumption in the same houses. The selection of beneficiaries groups have been randomly chosen however practical aspects for carrying out the tests have been taken into account:

- The user’s availability and motivation to participate in the required tests,

- Dispersion of the households in the communities and ease of accessibility the areas,

- Distance between households of the same clusters located in different villages.

- The seasonal variations (to assure conservative approach while calculating the fuel consumption).

The period over which the tests have been carried out is one week of pre- and one week of post-installation to avoid risks of heterogeneity cooking habits and to include weekend cooking in the correct annual ratio to the weekday cooking. For the emission reduction calculations, the most conservative value has been taken into account.

The sample sizes used in the different KTs are bigger than the simple size calculated using the KPT methodology by Rob Bailis to show statistically significant reductions in per capita fuel consumption. Also, as it can be verified in the ER excel sheet, the end-points of the 90% confidence interval lie within +/- 30% of the estimated mean, so the rule 90/30 is met, it proves that the sample size is big enough. Therefore, the estimated mean value have been used to calculate the total VER’s.

c) Follow-up meetings with the beneficiaries:

The Inti Illimani team followed up with the group of beneficiaries after each of the construction workshops. This first follow-up meeting was usually planned after 2 weeks from the end of the workshops. These follow-ups was carried out to ensure that the beneficiaries have started using the solar cookers with the instructions already provided by the II team experts to use and maintain the solar cookers during the training programs.

The Inti Illimani team member and a local co-coordinator from the village carried out these follow-up meetings. The beneficiaries were asked to demonstrate and discuss the problems that they might have come across while using the cookers during the first 2 weeks. This was a good opportunity for II team to monitor and understand various aspects of the project activity and also to learn if the beneficiaries have tried any new food recipes during this period. It has been observed by the II team, that the beneficiaries have used the solar cookstoves regularly. The meeting includes an individual basis consultation with each of the beneficiaries.

These follow-up meetings are carried out for 4 months with each group. After completion of the 4-month follow-up meetings, the II team gives a gift to the beneficiary who has regularly completed all the activities/training/improvements proposed during the previous follow-up meetings. The gifts are usually in the form of plates/cooking pots/cups etc. as recognition of their proactive participation in the project activity.

The follow-up meetings are carried out by the BISS team member along with the help of the local coordinator of the village.

Sustainable Development Parameters:

| Data / Parameter: | Access to affordable and clean energy services |
| --- | --- |
| Data unit: | 3623 households |
| Description: | It has been evaluated with the number of solar cook stoves distributed since 2007. From the year 2007 till December 2013, a total of 3623 solar stoves have been distributed. Of these 3623 solar stoves: - 3373 solar stoves were defined into clusters and out of which, 2742 stoves taken into account for ER calculations after considering the usage factor from the last 5 years (2007-2012). - Considering a conservative approach to calculate the total VER’s for the year 2013, the remaining 250 solar stoves in ‘other clusters’ were not taken into account for calculations but were considered while evaluating the SD parameters. |
| Source of data used: | Project database. |
| Monitoring frequency | Continuous throughout the year. |
| QA/QC procedures to be applied: | The total number of households in the project activity is determined from the project database. The II team collects the information of beneficiaries and records them in a database. This project database is maintained and regularly updated by the Inti Illimani team. |
| Any comment: | As a part of the internal verification process, the GoodPlanet team (GPF) crosschecked the database maintained by the II team members. |

| Data / Parameter: | Human and institutional capacity |
| --- | --- |
| Data unit: | 181 workshops carried out from 2007 to 2013. |
| Description: | It has been evaluated with the number of solar cook stoves distributed in the project activity. Total of 3623 solar stoves have been distributed from 2007-2013. In the year 2013, a total of 26 workshops were carried out. During the workshops, apart from technical trainings to construct the solar stoves, the II team also gives each group of beneficiaries a special training on nutrition, environment and health benefits of the use of the solar cook-stoves. After the training workshops, the Inti Illimani team also visits the households as a part of the continuous monitoring of the project. |
| Source of data to be used: | Trainings programs database. |
| Monitoring frequency | Throughout the year. |
| QA/QC procedures to be applied: | The details of each beneficiary participating in the project activity are recoded in a database, which is maintained by the Inti Illimani team. This database also includes the details of the training programs carried out during the year (e.g.- date, total beneficiaries, project area/village etc.). |
| Any comment: | As a part of the internal verification process, the GoodPlanet team (GPF) crosschecked the details of the total number of training workshops carried out during the year. |

| Data / Parameter: | Quantitative employment and income generation |
| --- | --- |
| Data unit: | 9.70 Bs/hh/month (Bs – Bolivian Boliviano) |
| Description: | During the field visits of the II team, they have observed that certainly some of the beneficiaries have shown additional usage of the cookstove for income generating activities like selling peanuts, api, corn, etc. This information will be updated as needed in the coming verification period. There has been no change in the total number of Inti Illimani staff. There are 6 members. |
| Source of data to be used: | Survey among the beneficiaries |
| Monitoring frequency | Continuous survey |
| QA/QC procedures to be applied: | The data is collected during the follow up meetings or during the regular field visits to the project areas. |
| Any comment: | Savings from LPG use. |

| Data / Parameter: | Technology transfer and technological self-reliance |
| --- | --- |
| Data unit: | 3623 households |
| Description: | It has been evaluated with the number of solar cook stoves distributed since 2007. 3623 households have participated in the construction and maintenance workshops, where the beneficiaries were trained to construct their own solar cookers by the II team. The II team members also provided all the materials that are required for the construction of the solar stoves. |
| Source of data to be used: | Beneficiaries database managed by II team. |
| Monitoring frequency | Continuous throughout the year |
| QA/QC procedures to be applied: | The details of each beneficiary participating in the project activity are recoded in a database, which is maintained and regularly updated by the Inti Illimani team. The beneficiaries construct their own solar stoves under the technical guidance of the Inti Illimani experts. In these workshops the technical experts also train the beneficiaries on the maintenance of the stoves. A total of 181 workshops have been carried out till 2013. The workshops show that there is a direct transfer of technology to the project beneficiaries. |
| Any comment: | As a part of the internal verification process, the GoodPlanet team (GPF) crosschecked the data on the total number of workshops held during the year. |

| Data / Parameter: | Biodiversity |
| --- | --- |
| Data unit: | 61% |
| Description: | There have been no large deforestation or reforestation initiatives observed in the project areas. Therefore the NRB rate is constant. |
| Source of data to be used: | NRB study. Kitchen tests (Cluster- LPG Fw). |
| Monitoring frequency | - |
| QA/QC procedures to be applied: | - |
| Any comment: | - |

| Data / Parameter: | Soil condition |
| --- | --- |
| Data unit: | 61% |
| Description: | The project allows the people to use less firewood, which leads to the reduction in the deforestation rate in the project areas and which also helps conserves the soil, as there are less possibilities of soil-erosion in the areas. There has been no large deforestation or reforestation initiative in the project areas. Therefore the NRB rate has been constant. |
| Source of data to be used: | NRB study. Kitchen tests (Cluster- LPG Fw). |
| Monitoring frequency | Biennially |
| QA/QC procedures to be applied: | KT has been carried out as per the guidelines given in the GS methodology. |
| Any comment: | The change in the consumption of firewood is about 4 kg per day. |

| Data / Parameter: | Air quality |
| --- | --- |
| Data unit: | 100 % |
| Description: | From the present KT, we can see that there has been a reduction on firewood and dung consumption of about 4.75 kg/day & 3.70 kg/day respectively. Therefore it can be concluded that there has been a gradual decrease in indoor air pollution due to the use of solar cookers for daily cooking needs. |
| Source of data to be used: | KT studies. |
| Monitoring frequency |  |
| QA/QC procedures to be applied: | KT has been carried out as per the guidelines given in the GS methodology. |
| Any comment: | A total of 3623 stoves have been disseminated since the year 2007 and out of which 2742 stoves have been considered for the ER calculations. The remaining 250 stoves from ‘other clusters’ have not been taken into account for the ER calculations but as they are in regular use by the beneficiaries, therefore the total % for the sustainable parameters has been considered taking into account the total number of solar stoves that are disseminated i.e. 3623 stoves. |

| Data / Parameter: | Livelihood of the poor |
| --- | --- |
| Data unit: | 9.70 Bs/hh/month (Bs – Bolivian Boliviano) |
| Description: | There are savings on fuel consumption for both- the LPG and firewood. The total amount (Bolivian Boliviano) that has been saved by the beneficiaries has been recorded/documented by the Inti Illimani team. |
| Source of data to be used: | Follow-up surveys. |
| Monitoring frequency | Continuously |
| QA/QC procedures to be applied: |  |
| Any comment: | Savings from LPG use. |

Emission reduction monitoring parameters:

| Data / Parameter: | P.1 Number of Solar cookstoves (SCS) |
| --- | --- |
| Data unit: | 2742 cookstoves considered for ER calculations. |
| Description: | Number of beneficiaries by cluster and community since 2007. A total of 3623 solar stoves have been distributed till December 2013. Out of these total solar stoves – - 3373 solar stoves were defined into clusters and after considering the usage factor, a total of 2742 solar stoves have been taken into account for the ER calculations. - The remaining total 250 solar stoves in ‘other clusters’ were not taken into account for ER calculations but were taken into consideration for the SD parameters. |
| Source of data to be used: | Project database |
| Monitoring frequency | Measured every month according to the data collected during the workshops. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | The project database has been established from the very beginning of the project activity, giving all the all the details of each beneficiary. The Inti Illimani team members update this database regularly. |

| Data / Parameter: | P.2 LPG consumption by cluster in the project scenario |
| --- | --- |
| Data unit: | LPG cluster: 8.58 Kg/month/household LPG + FW cluster: 10.08 Kg/month/household LPG + FW + Dung cluster: 8.87 Kg/month/household |
| Description: | Amount of LPG consumed by beneficiaries having solar cookstoves |
| Source of data to be used: | Kitchen Tests, precision level of 90% for confidence interval. |
| Monitoring frequency | Study led biennially on a representative sample of beneficiaries |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | KT has been done according with guidelines given in the GS methodology. The "90/30 rule" is met, so mean value have been used. |

| Data / Parameter: | P.3 firewood consumption by cluster in the project scenario |
| --- | --- |
| Data unit: | LPG + Fw cluster: 43.48 Kg/month/household |
| Description: | Amount of firewood consumed by beneficiaries having solar cookstoves |
| Source of data to be used: | Kitchen Tests, precision level of 90% for confidence interval. |
| Monitoring frequency | Study led biennially on a representative sample of beneficiaries |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | KT has been done according with guidelines given in the GS methodology. The "90/30 rule" is met, so mean value have been used. |

| Data / Parameter: | P.4 Dung consumption by cluster in the project scenario |
| --- | --- |
| Data unit: | LPG Fw Dung cluster: 32 Kg/month/household |
| Description: | Amount of dung consumed by beneficiaries having solar cookstoves |
| Source of data to be used: | Kitchen Tests, precision level of 90% for confidence interval. |
| Monitoring frequency | Study led biennially on a representative sample of beneficiaries |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | KT has been done according with guidelines given in the GS methodology. The "90/30 rule" is met, so mean value have been used. |

| Data / Parameter: | P.5 - XNRB |
| --- | --- |
| Data unit: | 61% |
| Description: | Non-renewable fraction of biomass |
| Source of data to be used: | NRB study |
| Monitoring frequency | The NRB fraction may be updated if there any afforestation/deforestation activities are observed in the project areas. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: |  |

| Data / Parameter: | P.6 - Usage in year 2013 |
| --- | --- |
| Data unit: | 2013: 70% people who got their solar cookstove in 2007 75 % people who got their solar cookstove in 2008 75 % people who got their solar cookstove in 2009 80 % people who got their solar cookstove in 2010 85 % people who got their solar cookstove in 2011 90 % people who got their solar cookstove in 2012 |
| Description: | Percentage of stoves of age x remaining in use in year y |
| Source of data to be used: | A study carried out by third party for the solar cookers implemented in Bolivia. |
| Monitoring frequency | Survey led once a year on a representative sample of beneficiaries |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and BISS |
| Any comment: | A survey will be taken up in the next verification period to get the use of stoves data more precisely. |

| Data / Parameter: | P.7 – Age |
| --- | --- |
| Data unit: | 2013: Efficiency of solar cookstoves built in 2007: 100 % Efficiency of solar cookstoves built in 2008: 100 % Efficiency of solar cookstoves built in 2009: 100 % Efficiency of solar cookstoves built in 2010: 100 % Efficiency of solar cookstoves built in 2011: 100 % Efficiency of solar cookstoves built in 2012: 100 % |
| Description: | Efficiency compared with a new Solar Cooker |
| Source of data to be used: | Monitoring Survey |
| Monitoring frequency | Survey led once every two years on a representative sample of Solar Cookers well maintained. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | All tests results show that older solar cookers are more efficient than new ones. But in order to simplify calculations and to keep a conservative approach, we consider that efficiency of old solar cookers is the same that new ones. |

| Data / Parameter: | P.8 Money savings |
| --- | --- |
| Data unit: | LPG cluster: 9.70 Bs/hh/month |
| Description: | Amount of money saved by the beneficiaries by using a mix of LPG/solar cookstoves/firewood. |
| Source of data to be used: | Monitoring survey (scanned copy of the KS with the parameter has been provided) |
| Monitoring frequency | Calculated from KS data and from price of LPG. |
| QA/QC procedures to be applied: | Crosschecked by Inti Illimani, BISS and GoodPlanet. |
| Any comment: |  |

| Data / Parameter: | P.9 Time savings |
| --- | --- |
| Data unit: | 100% |
| Description: | % of people who consider that time-saving is one of the main benefits of the solar cooker. |
| Source of data to be used: | Monitoring survey (scanned copy of the KS with the parameter has been provided) |
| Monitoring frequency | Continuously done by the Inti Illimani team members |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and BISS |
| Any comment: | Questions were asked to the beneficiaries to collect information about time saved due to the use of the solar cookers and from the responses provided it shows that they have more time for other activities. |

| Data / Parameter: | P.10 Share of the solar energy in the total energy mix |
| --- | --- |
| Data unit: | LPG cluster: 37% is solar energy LPG Fw cluster: 63% is solar energy LPG Fw Dung cluster: 63 % is solar energy |
| Description: | % of the solar energy used along with others fuel mix |
| Source of data to be used: | Monitoring survey |
| Monitoring frequency | Biennially during the KT studies. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and GoodPlanet |
| Any comment: | Calculated from fuel consumption reduction (KT) and its energy equivalence. |

| Data / Parameter: | P.11 Distance |
| --- | --- |
| Data unit: | 2013: around 4908 Km/year |
| Description: | Total annual distance from La Paz city to communities. |
| Source of data to be used: | Monitoring survey |
| Monitoring frequency | Annually. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and BISS |
| Any comment: | As the project is carried out throughout the year, this parameter is calculated from distance per course and it is included in the reports from each course. |

| Data / Parameter: | P.12 Fraction of the distance using common transport |
| --- | --- |
| Data unit: | 100 % |
| Description: | Fraction of the distance using common transport (bus) |
| Source of data to be used: | Monitoring survey |
| Monitoring frequency | Calculated from distance per course and kind of transport used included in the reports from each course. |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and BISS |
| Any comment: | As Inti Illimani doesn’t have their own vehicle, the team uses the public transports during the travel. |

| Data / Parameter: | P.13 |
| --- | --- |
| Data unit: | 2013: 100% |
| Description: | Fraction of the courses where measures have been taken to ensure that not damage to soil is being caused by the paint |
| Source of data to be used: | Monitoring survey |
| Monitoring frequency | Final evaluation course questionnaire made by beneficiaries |
| QA/QC procedures to be applied: | Parameter double-checked by Inti Illimani and BISS |
| Any comment: | During the courses the instructors make sure that during painting soil is protected by a plastic sheet or the activity is carried out in a large community hall. This parameter will also be separately documented during the courses in the next verification period. |

Emission Reductions Calculations:

The project activity was initiated in the year 2007 and was registered with the Gold Standard in the year 2011. As per the Gold Standard rules the emission reductions have been considered since the year 2009.

This monitoring report gives the emission reduction calculations for the year 2013.

The emission reductions data per month per cluster is given in following table: -

Emission reductions for the year 2013

|  | Baseline Emissions kg CO2/month |  |  | Project Emissions kg CO2/month |  |  | Emission Reductions kg CO2/month |  |  | Total VER TCO2/ month |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LPG | LPG Fw | LPG Fw Dung | LPG | LPG Fw | LPG Fw Dung | LPG | LPG Fw | LPG Fw Dung |  |
| Jan | 57760 | 242800 | 0 | 39831 | 81173 | 0 | 17929 | 161627 | 0 | 180 |
| Feb | 58608 | 243052 | 0 | 40416 | 81257 | 0 | 18192 | 161795 | 0 | 180 |
| Mar | 59455 | 243808 | 0 | 41000 | 81510 | 0 | 18455 | 162298 | 0 | 181 |
| Apr | 60473 | 245824 | 4693 | 41702 | 82184 | 1564 | 18771 | 163641 | 3129 | 186 |
| May | 60854 | 245824 | 8773 | 41965 | 82184 | 2924 | 18889 | 163641 | 5849 | 188 |
| Jun | 60854 | 245824 | 16730 | 41965 | 82184 | 5576 | 18889 | 163641 | 11154 | 194 |
| Jul | 60896 | 248597 | 25299 | 41994 | 83111 | 8432 | 18903 | 165486 | 16867 | 201 |
| Aug | 61151 | 248597 | 25299 | 42169 | 83111 | 8432 | 18982 | 165486 | 16867 | 201 |
| Sep | 61151 | 254142 | 29176 | 42169 | 84964 | 9724 | 18982 | 169177 | 19452 | 208 |
| Oct | 61151 | 263215 | 29176 | 42169 | 87998 | 9724 | 18982 | 175217 | 19452 | 214 |
| Nov | 61278 | 272540 | 29176 | 42257 | 91115 | 9724 | 19021 | 181425 | 19452 | 220 |
| Dec | 61490 | 276069 | 39989 | 42403 | 92295 | 13328 | 19087 | 183774 | 26661 | 230 |
| Total |  |  |  |  |  |  |  |  |  | 2381 |

Conclusion:

A total of 3623 solar stoves have been implemented from 2007 till December 2013.

The emission reduction calculations have been considered from the period of 1st January to 31st December 2013.

The project activity not only promotes sustainable development but also reduces the demand for firewood and LPG in the project areas. The following data summarizes the emission reduction for the year 2013.

Total emission reductions for the year 2013: 2381 tCO2/yr

Annexe 1

Kitchen survey scanned copies from the project database have been uploaded on the Gold Registry for further verification.

Annexe 2

Kitchen tests scanned copies from the project database have been uploaded on the Gold Registry for further verification.

Annexe 3

Scanned copies the exam carried out at the end of the workshop for the beneficiaries to assure that they have followed the training programs have been uploaded on the Gold Registry for further verification.

Annexe 4

Scanned copies the follow-up meetings sample sheets filled in by beneficiaries from the project database have been uploaded on the Gold Registry for further verification.

Glossary:

Solar SC/ SC: 	Solar Cookers

LPG: 		Liquefied petroleum gas

Fwd/FW: 	Firewood

NRB:		Non-renewable Biomass.

Bs: 		Bolivianos (Bolivian currency)

II: 		Inti Illimani

BISS:		Bolivia Inti Sud Soliel