Key Project Information & project Design Document (PDD)

PUBLICATION DATE  14.10.2020
VERSION  v. 1.2 
RELATED SUPPORT

- TEMPLATE GUIDE Key Project Information & Project Design Document v.1.2

This document contains the following Sections

Key Project Information

SECTION A – Description of project

SECTION B - Application of approved Gold Standard Methodology (ies) and/or    demonstration of SDG Contributions

SECTION C – Duration and crediting period

SECTION D – Summary of Safeguarding Principles and Gender Sensitive Assessment

SECTION E – Outcome of Stakeholder Consultations

Appendix 1 – Safeguarding Principles Assessment (mandatory)

Appendix 2 - Contact information of Project participants (mandatory)

Appendix 3 - LUF Additional Information (project specific)

Appendix 4 - Summary of Approved Design Changes (project specific)

KEY PROJECT INFORMATION

| GS ID of Project | GS11333 |
| --- | --- |
| Title of Project | Swine Farm Animal Manure Management System GHG Mitigation Project in Hubei Province |
| Time of First Submission Date | 06/09/2021 |
| Date of Design Certification | NA |
| Version number of the PDD | 06 |
| Completion date of version | 06/03/2023 |
| Project Developer | Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. |
| Project Representative | The official focal point: Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. |
| Project Participants and any communities involved | The project owner: Jiangxi Zhengbang Breeding Co. Ltd |
| Host Country (ies) | People’s Republic of China |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Scale of the project activity | Micro scale Small Scale Large Scale |
| Other Requirements applied | GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) |
| Methodology (ies) applied and version number | ACM0010 GHG emission reductions from manure management systems (Version 08.0) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |
| Project Cycle: | Regular Retroactive |

Table 1 – Estimated Sustainable Development Contributions

| ## Sustainable Development Goals Targeted | ## SDG Impact (defined in B.6.) | ## Estimated Annual Average | ## Units or Products |
| --- | --- | --- | --- |
| ## SDG 13: Climate Action | ## Amount of GHGs emission avoided or sequestered | ## 282,734 tCO2e |  |
| SDG 8: Decent Work and Economic Growth | Total number of jobs | ## 18 full-time jobs created (including 9 females and 9 males). |  |
| ## SDG 7: 7 Affordable and Clean Energy | Total electricity produced | ## 40,134 MWh |  |

SECTION A. DESCRIPTION OF PROJECT

A.1 Purpose and general description of project

>>

Swine Farm Animal Manure Management System GHG Mitigation Project in Hubei Province installs new animal waste management systems (AWMSs) to a group of 9 swine farms in Hubei Province, which are owned by Jiangxi Zhengbang Breeding Co. Ltd (hereinafter called” Zhengbang”). The project activity has been designed as single project implemented at 9 animal farms. The purpose of the project activity is to treat the manure and wastewater from the 9 existing swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons.

The project activity will replace the current open anaerobic lagoons with 9 new AWMSs, and each subsidiary swine farm will install one AWMS. The manure and waster will be collected into the anaerobic digesters and then biogas generated during the anaerobic treatment process will be captured and total 31,354,493 m3 biogas are expected to be generated annually. And then the biogas will be sent into the biogas generating system for power generation. Each generator will be installed in each subsidiary swine farm, total installed capacity of the proposed project is 6 MW. The electricity generated are all used by the AWMSs and the swine farms, and the Residual biogas  will be flared(if any). After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. Wastewater from the anaerobic digesters will be treated aerobically and then used for agriculture irrigation.

Benefited from the “waste-biogas-fertilizer” pattern, harmlessness and ecological utilization of the swine manure can be realized. The annual average electricity generation is 40,134 MWh, all of which will be used by the AWMSs and 9 swine farms. The electricity generated will not be connected to other users or to the regional power grid and the baseline emissions associated with electricity generation will not be claimed.

The project activity will reduce of GHG in the atmosphere through avoiding methane emissions from anaerobic treatment of swine manure and wastewater. 282,734 tCO2e emission reductions will be produced annually and total emission reductions are 1,413,670 tCO2e in the first crediting period. The project activity can contribute to local sustainable development in the following aspects:

- Capture of methane will significantly reduce the odor nuisance in the vicinity of the facility.

- The project activity can provide 18 full-time job opportunities for local residents.

- The effluent and fermented sludge are ecological organic fertilizers which are partly supplied to the farmers living around for free, which can be used to yield organic products and improve the living conditions of the farmers.

- The project activity can capture methane generated during the swine manure treatment process and the biogas can be used for power generation, which can contribute to the mitigation of global climate change.

Eligibility of the project under Gold Standard

>>

The proposed project falls under section 3.1.1 of GS4GG Principles & Requirements (Version 1.2), section 2,3 of GS4GG Community Services Activity Requirements (Version 1.2) and section 2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) with the following eligibility criteria:

| Eligibility Criteria | Justification |
| --- | --- |
| - Types of Project Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) Eligible projects shall include physical action/implementation on the ground. Pre-identified eligible project types are identified in the Eligibility Principles and Requirements section. Section 2.1.2 of GS4GG Community Services Activity Requirements (Version 1.2) All CSA Projects shall lead to climate change mitigation and/or adaptation by providing or improving access to services/resources at the household or community or institution level. Eligible services include electricity and energy, water and sanitation, waste management, housing, etc. Section 3.1.1 of GS4GG Community Services Activity Requirements (Version 1.2) Pre-identified CSA project types are: a) Renewable energy; b) End-use energy efficiency; c) Waste management and handling; d) Water, sanitation and hygiene (WASH). Section 5.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) The Following Project types are eligible for issuance of GSVERs or GSCERs: a) Renewable Energy Supply; b) End-Use Energy Efficiency Improvement; c) Waste Handling & Disposal; d) Land Use and Forests. | The project activity will replace the current open anaerobic lagoons with 9 new closed anaerobic digesters. The biogas generated during the treatment process will be captured for power generation. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer and partly will be supplied to the farmers living around for free, the rest of the organic fertilizer will be sold at a price lower than the market price. Wastewater from the new AWMSs will be treated aerobically and then used for agriculture irrigation. Therefore, this project will by providing 9 new closed anaerobic digesters to improve the waste management led to climate change mitigation, which meets the section 2.1.2 of Community Services Activity Requirements (Version 1.2) and the project falls under the section 3.1.1 (c) “Waste management and handling: All waste management activities that deliver energy or a usable product with sustainable development benefits such as composting, biogas etc.” of GS4GG Community Services Activity Requirements (Version 1.2). Besides, the project activity falls under section 5.1.1 (c) “Waste Handling & Disposal: The waste handling and disposal category refers to all waste handling Projects that deliver an energy service (e.g. LFG with some of the recovered methane used for electricity generation) or a usable product with sustainable development benefits (e.g. composting).” of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1). As per section 4.1.3 of GS4GG Principles & Requirements (Version 1.2), “A Project type is automatically eligible for Gold Standard Certification if there are Gold Standard approved Activity Requirements and/or Impact Quantification Methodologies associated with it or it’s referenced in the Gold Standard Product Requirements”. The project type is automatically eligible for Gold Standard Certification. |
| - Location of Project: Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) Projects may be located in any part of the world. Section 3.1.2 of GS4GG Community Services Activity Requirements (Version 1.2) Project Area and Boundary shall be defined in line with the applicable Impact Quantification Methodologies and Product Requirements. Section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) Gold Standard VER Projects may be located in any host country or state. However, where host countries or states have mandatory operational schemes to reduce GHG emissions in any form (e.g. cap & trade, carbon tax etc.), Projects shall only be eligible if the Project Developer has either: (a) provided Gold Standard with satisfactory justification that no double counting of emission reductions occur or (b) has committed to retiring eligible units equal to the quantity of Gold Standard VERs. Refer to Annex A of this document. | The project location is the Republic of China (refer to section A.2 of the PDD for detailed project area and boundary). There is a cap & trade scheme in China. However, the project activity is not included the mandatory emission control scheme since the scheme only cover the high-emission industries, such as power generation sector that emitted at least 26,000 tons of CO2e/year. There is no emission cap enforced for the project owner. In addition, the project owner has signed the Declaration of No Double Counting Statement and Declaration of not involved in other GHG scheme to ensure that the project will not apply for emission reduction credits or labels under any other schemes other than GS. In addition, if any such risk of double counting exists, project developer confirms that the eligible units equal to the quantity of Gold Standard VERs will be retired/ returned/abandoned. Therefore, the emission reductions will not be double counted. As per section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), it is an eligible host country. |
| - Project Area, Project Boundary and Scale: Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) The Project Area and Project Boundary shall be defined. Projects may be developed at any scale although certain rules, requirements and limitations may apply under specific Activity Requirements, Impact Quantification Methodologies and Products Requirements. In order to avoid double counting the Project shall not be included in any other voluntary or compliance standards programme unless approved by Gold Standard (for example through dual certification). Also, if the Project Area overlaps with that of another Gold Standard or other voluntary or compliance standard programme of a similar nature, the project shall demonstrate that there is no double counting of impacts at design and performance certification (for example use of similar technology or practices through which the potential arises for double counting or misestimation of impacts amongst projects). Section 3.1.2 of GS4GG Community Services Activity Requirements (Version 1.2) Project Area and Boundary shall be defined in line with the applicable Impact Quantification Methodologies and Product Requirements. The definition of scale is the same for all Projects, except Microscale. Section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) Gold Standard VER Projects may be located in any host country or state. However, where host countries or states have mandatory operational schemes to reduce GHG emissions in any form (e.g. cap & trade, carbon tax etc.), Projects shall only be eligible if the Project Developer has either: (a) provided Gold Standard with satisfactory justification that no double counting of emission reductions occur or (b) has committed to retiring eligible units equal to the quantity of Gold Standard VERs. which must be done prior to VER issuance. Refer to Section 15 below of this document.. Section 9.1.1 GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) GS VER Projects may be “large scale”, “small scale” (for the applicability of methodologies and tools only) or “microscale”. Scale is defined in the relevant Gold Standard Activity Requirements or where these do not exist then per following paragraphs. Section 9.1.2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) All Projects exceeding the small-scale thresholds are defined as large scale. Small scale projects are defined in accordance with CDM project standard for project activities. | The project location is the Republic of China (refer to section A.2 of the PDD for detailed project area and boundary). There is a cap & trade scheme in China. However, the project activity is not included the mandatory emission control scheme since the scheme only cover the high-emission industries, such as power generation sector that emitted at least 26,000 tons of CO2e/year. There is no emission cap enforced for the project owner. In addition, the project owner has signed the Declaration of No Double Counting Statement and Declaration of not involved in other GHG scheme to ensure that the project will not apply for emission reduction credits or labels under any other schemes other than GS. In addition, if any such risk of double counting exists, project developer confirms that the eligible units equal to the quantity of Gold Standard VERs will be retired/ returned/abandoned. Therefore, the emission reductions will not be double counted. As per section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), it is an eligible host country. The project boundary is defined based on ACM0010 GHG emission reductions from manure management systems (Version 08.0). The spatial extent of the AWMSs and the electricity generation equipment, the flaring system and the power/heat source.. Refer to section B.3 of the PDD for more details. Annual emission reductions of the project activity are 282,734 tCO2e, which is more than 60,000 tCO2e. As per section 9.1.1 and 9.1.2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), it’s a large-scale GS VER project. Before implementation of the project, no other similar project has been implemented in the project area. Each Animal Waste Management System (AWMS) can be identified through its GPS coordinates. Therefore, the Project Area do not overlap with that of another Gold Standard or other voluntary or compliance standard programme of a similar nature. In addition, if any such risk of double counting exists, project developer confirms that the eligible units equal to the quantity of Gold Standard VERs will be retired. |
| - Host Country Requirements Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) Projects shall be in compliance with applicable Host Country’s legal, environmental, ecological and social regulations. Section 3.1.2 of GS4GG Community Services Activity Requirements (Version 1.2) Project Area and Boundary shall be defined in line with the applicable Impact Quantification Methodologies and Product Requirements. Section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) VER Projects may be located in any host country or state. However, where host countries or states have mandatory operational schemes to reduce GHG emissions in any form (e.g. cap & trade, carbon tax etc.), Projects shall only be eligible if the Project Developer has either: (a) provided Gold Standard with satisfactory justification that no double counting of emission reductions occur or (b) has committed to retiring eligible units equal to the quantity of Gold Standard VERs. Refer to Annex A of this document. | The project location is the Republic of China (refer to section A.2 of the PDD for detailed project area and boundary). As per Action Plan for Resource Utilization of Livestock Manure (2017-2020), the sustainable development of animal husbandry should focus on reduction of livestock waste, and harmless treatment and resource utilization of the manure and wastewater. In addition, according to the document “Notice on Strengthening the Resource Utilization Plan and Ledger Management of Livestock and Poultry Manure” issued on 24/11/2021 by General Office of Ministry of Agriculture and Rural Affairs and Ministry of Ecology and Environment, it can be seen that the resource utilization of manure has been continuously promoted and standardized management has been achieved. The project activity will replace the current open anaerobic lagoons with 9 new AWMSs. The biogas generated during the treatment process will be captured for power generation. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. Wastewater from anaerobic digesters will be treated aerobically and then used for agriculture irrigation. The project follows China’s legal, environmental, ecological and social regulations. There is a cap & trade scheme in China. However, the project activity is not included the mandatory emission control scheme since the scheme only cover the high-emission industries, such as power generation sector that emitted at least 26,000 tons of CO2e/year. There is no emission cap enforced for the project owner. In addition, the project owner has signed the Declaration of No Double Counting Statement and Declaration of not involved in other GHG scheme to ensure that the project will not apply for emission reduction credits or labels under any other schemes other than GS. In addition, if any such risk of double counting exists, project developer confirms that the eligible units equal to the quantity of Gold Standard VERs will be retired/ returned/abandoned. Therefore, the emission reductions will not be double counted. As per section 3.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), it is an eligible host country. |
| - Contact Details Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) As part of the Project Documentation the Project Developer shall provide (i) name and (ii) contact details of all Project Participants; AND in case of an organization (iii) the legal registration details and (iv) documentation by the governing jurisdiction that proves that the entity is in good standing (defined as being a legal or other appropriate entity registered in or allowed to operate within the required jurisdiction and with no evidence of insolvency or legal/criminal notices placed against it or any of its Directors). Gold Standard retains the right (at its own discretion) to refuse use of the Standard where reputational concerns are highlighted. | The Project Participants’ contact details are provided in Appendix 2. Contact information of project participants of this PDD. As the project owner, Jiangxi Zhengbang Breeding Co. Ltd. is in the state of ongoing operation and has no administrative discrimination information, not listed as business abnormal catalo information and serious illegal untrustworthy enterprise list (blacklist) information by querying the national enterprise credit information publicity system. Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. is a private owned company with limited liability instead of an organization incorporated in the Republic of China. It is in good standing as per credit china. By querying the national enterprise credit information publicity system, it shows that Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. is in the state of ongoing operation and has no administrative discrimination information, not listed as business abnormal catalog information and serious illegal untrustworthy enterprise list (blacklist) information. |
| - Legal Ownership Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) Full and uncontested legal ownership of any Products that are generated under Gold Standard Certification, (for example carbon credits) shall be demonstrated. Where such ownership is transferred from project beneficiaries this must be demonstrated transparently and with full, prior and informed consent (FPIC). Note that for certain Project types there is a requirement for full and uncontested legal land title/tenure to be demonstrated. These are contained within specific Activity or Product Requirements. All projects shall immediately report to Gold Standard any land title/tenure disputes arising. Section 3.1.4 of GS4GG Community Services Activity Requirements (Version 1.2) Projects involving the distribution of a large number of devices for services such as heating, cooking, lighting, electricity generation, water treatment technology such as water filter, etc. shall provide a clear description of the ownership of the Products that are generated under Gold Standard Certification all along the investment chain. In line with the FPIC requirement, the proofs that end-users are aware of and willing to give up their rights on Products shall be provided. The transfer of Product ownership shall be discussed during local stakeholder consultations for projects. | All the Animal Waste Management Systems (AWMSs) were invested by the project owner, who has full and uncontested legal ownership of the credits that will be generated under Gold Standard Certification. In addition, confirmation of the GS VER ownership was also be discussed during the local stakeholder consultation held on 18/10/2021. The project owner has the legal ownership of the emission reductions generated by the project activity. Refer to section A.1.2 of the PDD for further details. |
| - Other Rights Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) As well as legal title and ownership, the Project Developer shall also demonstrate where required uncontested legal rights and/or permissions concerning changes in use of other resources required to service the Project (for example, access rights, water rights etc.). Any known disputes or contested rights must be declared immediately to Gold Standard by the Project Developer and resolved prior to further project implementation in affected areas. | The project activity will replace the current open anaerobic lagoons with 9 new AWMSs. The biogas generated during the treatment process will be captured for power generation. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. Wastewater from the anaerobic digesters will be treated aerobically and then used for agriculture irrigation. Comprehensive utilization and resourceful treatment of the manure waste are encouraged by the local government. And all the manure from the swine farms will be put into the AWMSs as it is prohibited to discharge into any natural water resources without treatment according to Regulations on Prevention and Control of Pollution from Livestock and Poultry Farming. Therefore, the project does not involve any activity that causes alteration of any resource, or contested legal rights and other disputes, therefore the need for acquiring any specific legal right is not applicable. |
| - Official Development Assistance (ODA) Declaration Section 3.1.1 of GS4GG Principles & Requirements (Version 1.2) All Project Developers applying for project activities located in a country named by the OECD Development Assistance Committee’s ODA recipient list and seeking Gold Standard Certification for carbon credits shall declare the Official Development Assistance (ODA) support. The Project Developer shall follow the GHG Emissions Reduction & Sequestration Product Requirements and submit the declaration at the time of Design Certification. Section 6.1.1 and 6.1.2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) Projects are ineligible for carbon crediting under GS4GG if the OFFICIAL DEVELOPMENT ASSISTANCE (ODA) is provided to the project under the condition that the credits generated by the Project will be transferred, either directly or indirectly, to the donor country providing ODA support. The OECD defines ODA as financial flows: a. To developing countries and multilateral institutions; b. Provided by government agencies (e.g., USAID); c. Whose main objective is the economic development and welfare of developing countries; and d. That are concessional in character, conveying a grant element of at least 25%. Project Developer submitting a Project located in a country named by the OECD Development Assistance Committee’s ODA recipient list shall sign and submit the ODA Declaration. | The project owner has signed the ODA declaration template and confirms that no ODA is provided under the condition that the credits generated by the project will be transferred, either directly or indirectly, to the donor country providing ODA support. |
| - Suppressed Demand Section 3.1.3 of GS4GG Community Services Activity Requirements (Version 1.2) Certain Impact Quantification methodologies allow projects to account Suppressed Demand scenario when establishing a baseline. In such cases, the application of Suppressed Demand baseline is limited to Small Scale and Microscale Projects. Where a Suppressed Demand baseline is applied, it is not possible to ‘stack’ Gold Standard Certified Impact Statements or Products as the definition of the baseline may be contradictory. | Suppressed demand baseline is not applicable to the project. As per Section 3.1.3 of GS4GG Community Services Activity Requirements (Version 1.2), it can be a large-scale GS VER project. |
| - Eligible Greenhouse Gases Section 4.1.1 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1) Only Carbon Dioxide (CO2), Methane (CH4) and/or Nitrous Oxide (N2O) are eligible for GSVERs or GSCERs, provided Projects comply with all GS4GG Requirements and eligibility criteria. | The project considers the emission reductions of Carbon Dioxide (CO2), methane (CH4) and/or Nitrous Oxide (N2O), which are eligible for Gold Standard crediting. Refer to section B.3 of the PDD for further details. |

Legal ownership of products generated by the project and legal rights to alter use of resources required to service the project

>>

All the Animal Waste Management Systems (AWMSs) of the 9 swine farms were invested by the project owner, who has full and uncontested legal ownership of the credits that will be generated under Gold Standard Certification. In addition, confirmation of the GS VER ownership will also be discussed during the local stakeholder consultation, which was held on 18/10/2021. The project owner has the legal ownership of the emission reductions generated by the project activity.

Investment decision of the entire project were made by Jiangxi Zhengbang Breeding Co. Ltd.(Project Owner). All the 9 swine farms were owned by the project owner, who will make detailed plan for the whole project. Specific implementation, operation and monitoring of the project were carried out by each swine farm under the guidance of the project owner. Related trainings on project requirements and monitoring plan will be organized by the project owner.

As project developer, Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. is mainly responsible for preparation of PDD, MR and other documents related to emission reductions. It doesn’t participate in the actual implementation and operation of the entire project.

A.2 Location of project

>>

The proposed project activity is located in Hubei Province, China. The project activity has been designed as single project implemented at nine animal farms, nine swine farms both include market swine and breeding swine: Yongjiahes wine farm, Zhoujiachong swine farm, Jiucaiyuan swine farm, Gaoshanmiao swine farm, Shayang 1st phase swine farm, Shayang 2nd phase swine farm, Ezhou swine farm, Tuchong swine farm and Sangzihu swine farm. The location of the nine subsidiary farms is shown below:

| Swine farm | Location | East longitude | North latitude |
| --- | --- | --- | --- |
| Yongjiahe swine farm (永佳河猪场) | Xumenzhai village, Yongjiahe Town, Hong'an County, Huanggang City | 114°39'41.8" | 31°08'39.5" |
| Zhoujiachong swine farm (周家冲猪场) | Zhoujiachong village, Chengguan Town, Hong'an County, Huanggang City | 114°32'08.5" | 31°17'02.0" |
| Jiucaiyuan swine farm (韭菜园猪场) | Gaogaomiao village, shangxinji Town, Hong'an County, Huanggang City | 114°25'22.3" | 31°21'37.1" |
| Gaoshanmiao swine farm (高山庙猪场) | Gaogaomiao village, shangxinji Town, Hong'an County, Huanggang City | 114°26'56.9" | 31°21'00.0" |
| Shayang 1st phase swine farm (沙洋一期猪场) | Gulin village, shihuiqiao Town, Shayang County, Hubei Province | 112°13'55.0" | 30°36'29.0" |
| Shayang 2nd phase swine farm (沙洋二期猪场) | Gulin village, shihuiqiao Town, Shayang County, Hubei Province | 112°13'39.0" | 30°37'26.0" |
| Ezhou swine farm (鄂州猪场) | Du Shan Zhen Xu Dong Cun, Echeng District, Ezhou City, Hubei Province | 114°46'46.4" | 30°23'41.2" |
| Tuchong swine farm (涂冲猪场) | Tuchong village, Xincheng Town, Dawu County, Hubei Province | 114°15'08.3" | 31°32'22.0" |
| Sangzihu swine farm (桑梓湖猪场) | Sangzi Lake farm, Shashi District, Hubei Province | 112°30'25.6" | 30°16'38.5" |

Figure 1 The Geographic Location of the project

A.3 Technologies and/or measures

>>

The project activity is designed to install new AWMSs to a group of 9 existing swine farms to treat the manure and wastewater from the 9 existing swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. The biogas generated during the treatment process will be captured for power generation, and surplus biogas is destroyed through the flaring system (if any), the electricity generated are all used by the operation of AWMSs and swine farms, which is supplied by the grid company in baseline scenario. The grid company is dominated by thermal power generation. Therefore, the project activity will reduce of GHG and provide clean energy, which is in line with SGD 13(Climate Action) and SGD 7(Affordable and Clean Energy).

Temporary and permanent full-time job opportunities are created for locals during the construction and operation period of the project, 18 full-time jobs created (including 9 females and 9 males) during the operation period of the project for project implementation and monitoring activities. Therefore, the project activity is in line with SDG8 (Decent Work and Economic Growth).

The proposed project implemented 9 sets of Animal Waste Management Systems (AWMSs) in 9 swine farms involving 130,976 heads of commercial pigs, 226,952 heads of breeding pigs. Live pigs are kept for 180 days in the farms and are estimated to produce 522,575 tons of manure and 31,354,493 m3 of biogas every year. All these 9 existing swine farms were put into operation before the implementation of the proposed project, detailed operation start date can be referred below Table 2. The construction date, commissioning date and start operation date of AWMSs in each farm are summarized in Table 3..

The technical flow of the project activity is shown below:

Table 2. The operation date of swine farms

| No. | Swine farm name | Operation start date |
| --- | --- | --- |
| 1 | Yongjiahe swine farm | 02/05/2016 |
| 2 | Zhoujiachong swine farm | 14/06/2017 |
| 3 | Jiucaiyuan swine farm | 09/08/2016 |
| 4 | Gaoshanmiao swine farm | 18/12/2018 |
| 5 | Shayang 1st phase swine farm | 28/09/2015 |
| 6 | Shayang 2nd phase swine farm | 22/07/2016 |
| 7 | Ezhou swine farm | 18/06/2017 |
| 8 | Tuchong swine farm | 22/07/2016 |
| 9 | Sangzihu swine farm | 25/04/2018 |

Table 3 The construction, commissioning and start operation date of AWMSs in each swine farm involved in this project

| Swine farm name | Construction date | Commissioning date | Start Operation date |
| --- | --- | --- | --- |
| Yongjiahe swine farm | 17/09/2020 | 15/12/2020 | 01/01/2021 |
| Zhoujiachong swine farm | 18/09/2020 | 17/12/2020 | 01/01/2021 |
| Jiucaiyuan swine farm | 23/09/2020 | 18/12/2020 | 01/01/2021 |
| Gaoshanmiao swine farm | 23/09/2020 | 19/12/2020 | 01/01/2021 |
| Shayang 1st phase swine farm | 25/09/2020 | 13/12/2020 | 01/01/2021 |
| Shayang 2nd phase swine farm | 22/09/2020 | 18/12/2020 | 01/01/2021 |
| Ezhou swine farm | 25/09/2020 | 20/12/2020 | 01/01/2021 |
| Tuchong swine farm | 22/09/2020 | 21/12/2020 | 01/01/2021 |
| Sangzihu swine farm | 24/09/2020 | 15/12/2020 | 01/01/2021 |

Baseline scenario

Prior to the implementation of the project, all manure waste produced from the 9 existing swine farms was left to decay in 9 uncovered anaerobic lagoons at the livestock farms and methane is emitted to the atmosphere directly without any methane recovery and destruction facility. As per “technical specification for sanitation treatment of livestock and poultry manure ”, In China, the uncovered anaerobic lagoon is a kind of manure treatment method recognized by the state. In addition, there is no legal regulation to mandate the livestock farm owners to implement anaerobic digestion, aerobic or other biological treatment techniques and to capture and/or utilize methane generated at these lagoons.

After the implementation of the project, original 9 uncovered anaerobic lagoons had been removed, the volume for original uncovered anaerobic lagoons is described in Table 4.

Project technology description

There are 9 existing swine farms for this project, and each AWMS is implemented in each swine farm. The specific process is as follows:

All the manure and wastewater are collected and then be separated first. The separated solid will be treated in aerobic composting system, which will be used as fertilizer. And part of the fertilizer products will be distributed to the surrounding farmers free of charge to improve the life condition of farmers, the others will be sold to the market. The separated liquid will be treated through anaerobic digestion, the biogas generated during the treatment process will be captured for power generation if there is surplus biogas, then the biogas will be flared through the flaring system. The sludge produced from anaerobic digestion will be treated through aerobic composting together with the separated solid and the effluent will be treated aerobically and then used for agriculture irrigation.

Pretreatment process

The mixed manure enters the homogenate regulating tank, where the homogenate is further hydrolyzed to remove impurities such as mud and sand. The feed solution is heated to 30 °C in the homogenate hydrolysis tank, and then pumped into the anaerobic tank.

Anaerobic digestion process

Anaerobic tanks adopt Upflow Anaerobic Sludge Bed Reactor (UASB) as its anaerobic digester technology, and the biogas generated from the anaerobic digesters is entered the biogas generator set after desulfurization and purification system. The generated electricity is used for daily operation of AWMSs and the swine farms, and the waste heat of the generator is recycled for increasing the temperature of the anaerobic reactor. At the same time, and the residual biogas is flared through open flaring if there is any surplus biogas. The temperature of the anaerobic treatment unit of the project is designed as medium temperature, and the optimal temperature range is 35 ~ 38 ℃.

Solid-liquid separation

All the manure and wastewater are collected and then be separated first by solid-liquid separator. The separated solid are treated in aerobic composting system and the organic fertilizers are produced. The separated liquid is treated through anaerobic digestion and the biogas generated during the treatment process is captured for electricity generation.

Aerobic composting process

The separated solid after solid-liquid separation and the sludge produced from anaerobic digestion are treated through aerobic composting. The aerobic composting process of this project adopts trough composting. The trough composting system combines controlled ventilation and regular turning. The specific process of aerobic composting is as follows:

a. Oxygen supplemental high temperature fermentation: The separated solid manure was transported to the composting troughs by belt and the sludge produced from anaerobic tank was transported to the composting troughs by pipeline. Then, the high-temperature fermentation process with supplemental oxygen is carried out in the composting troughs. With the turning process being conducted many times, the temperature of the composting troughs will rise gradually, and the pathogenic micro-organisms and harmful substances will be deactivated. The air blower is used to supply the oxygen required for fermentation during the turning process.

b. Secondary fermentation: After the oxygen supplemental high temperature fermentation, as the compost temperature drops, the compost enters the secondary fermentation. This process also can be called post-mature fermentation or aging stage. This process is conducive to the decomposition of refractory organic compounds into relatively stable organic matters such as humus and amino acids, which will greatly improve the efficiency of the fertilizer. The secondary aging fermentation will end when the fertilizer temperature drops to room temperature.

c. Sell of the organic fertilizer: The organic fertilizer produced after the secondary fermentation is a semi-finished fertilizer, which is harmless but not reaching the Chinese organic fertilizer standard. Most of the semi-finished fertilizer produced by this project was loaded into a transporter by the electric forklift and sold to organic fertilizer manufacturers. A small part of the semi-finished fertilizer produced by this project was distributed to nearby farmers for free.

The proposed project was implemented in 9 existing swine farms and the technology implemented at each swine farm was same. While the treatment capacities of each AWMS and the size of each swine farm and corresponding design of the AWMS was different. The treatment capacity and equipment of the system are designed according to the farming scale.  The volume for original uncovered anaerobic lagoons and the equipment parameters used of each swine farm are shown in the following Table 4.

Figure 2 Process flow of biogas project in the swine farms(the technical flow in each farm was same)

Table 4. The volume for original uncovered anaerobic lagoons and the equipment parameters

| Farm name | Yongjiahe |  | Zhoujiachong |  | Jiucaiyuan |  | Gaoshanmiao |  | Shayang 1st phase |  | Shayang 2nd phase | Ezhou |  | Tuchong |  | Sangzihu |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Treatment capacities of AWMS(t/d) | 450 |  | 180 |  | 150 |  | 100 |  | 100 |  | 100 | 100 |  | 400 |  | 400 |  |
| Baseline anaerobic lagoon parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Size of baseline anaerobic lagoon(length*width*depth) (m3) | 100*100*4.5 |  | 80*80*4 |  | 50*80*4 |  | 50*80*4 |  | 50*80*4 |  | 50*80*4 | 50*80*4 |  | 100*80*4.5 |  | 100*80*4.5 |  |
| technical life | No Less than 15 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Generator equipment parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | KDGH-1500G |  | KDGH-500G |  | KDGH-500G |  | KDGH-300G |  | KDGH-250G |  | KDGH-300G | KDGH-250G |  | KDGH-1200G |  | KDGH-1200G |  |
| Unit power(kw/kvA) | 1500 |  | 500 |  | 500 |  | 300 |  | 250 |  | 300 | 250 |  | 1200 |  | 1200 |  |
| Frequency (Hz) | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 | 50 |  | 50 |  | 50 |  |
| Power generation(kwh/h) | 1500 |  | 500 |  | 500 |  | 300 |  | 250 |  | 300 | 250 |  | 1200 |  | 1200 |  |
| Equipment Quantity | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 | 1 |  | 1 |  | 1 |  |
| Equipment technical life | No Less than 30 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Anaerobic tank parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Design capacity (m3) | 2500*3 |  | 3000 |  | 2500 |  | 2000 |  | 2000 |  | 2000 | 2000 |  | 2000*2+2500 |  | 2000*2+2500 |  |
| Stay time(day) | 15.8 |  | 15.8 |  | 15.8 |  | 15.8 |  | 15.8 |  | 15.8 | 15.8 |  | 15.8 |  | 15.8 |  |
| Anaerobic technology | UASB |  | UASB |  | UASB |  | UASB |  | UASB |  | UASB | UASB |  | UASB |  | UASB |  |
| Fermentation temperature(℃) | 38 |  | 38 |  | 38 |  | 38 |  | 38 |  | 38 | 38 |  | 38 |  | 38 |  |
| Equipment Quantity | 3 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 | 1 |  | 3 |  | 3 |  |
| Equipment technical life | No Less than 15 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Turnover machine parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | FJL6000*1350 |  | FJL 6000*1050 |  | FJL 6000*1050 |  | FJL 6000*1050 |  | FJL 6000*1050 |  | FJL 6000*1050 | FJL 6000*1050 |  | FJL6000*1350 |  | FJL6000*1350 |  |
| Supporting power(kw) | 41.5 |  | 25.74 |  | 25.74 |  | 25.74 |  | 25.74 |  | 25.74 | 25.74 |  | 41.5 |  | 41.5 |  |
| Speed of work(m/h) | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 | 50 |  | 50 |  | 50 |  |
| No-load speed(m/h) | 100 |  | 100 |  | 100 |  | 100 |  | 100 |  | 100 | 100 |  | 100 |  | 100 |  |
| Height of mixing rod(mm) | 4500 |  | 3400 |  | 3400 |  | 3400 |  | 3400 |  | 3400 | 3400 |  | 4500 |  | 4500 |  |
| Turning depth(mm) | 1500-1800 |  | 1500-1800 |  | 1500-1800 |  | 1500-1800 |  | 1500-1800 |  | 1500-1800 | 1500-1800 |  | 1500-1800 |  | 1500-1800 |  |
| Equipment Quantity | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 | 1 |  | 1 |  | 1 |  |
| Equipment technical life | No Less than 15 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flared system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type | Open flare | Open flare |  | Open flare |  | Open flare |  | Open flare |  | Open flare |  |  | Open flare |  | Open flare |  | Open flare |
| Material | 304 stainless steels | 304 stainless steels |  | 304 stainless steels |  | 304 stainless steels |  | 304 stainless steels |  | 304 stainless steels |  |  | 304 stainless steels |  | 304 stainless steels |  | 304 stainless steels |
| Height (m) | 3.5 | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  | 3.5 |  |  | 3.5 |  | 3.5 |  | 3.5 |
| Wall thickness (mm) | 3 | 3 |  | 3 |  | 3 |  | 3 |  | 3 |  |  | 3 |  | 3 |  | 3 |
| Technical life | No Less than 15 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.4 Scale of the project

>>

The proposed project introduces new animal waste management systems to treat the manure and wastewater. As per section 9.1.2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), it belons to the Type 3: other project activities. The annual emission reductions of the project activity are 282,734tCO2e, which is more than 60,000 tCO2e. Thus, the project does not meet the definition of small-scale projects in section 9.1.2 of GS4GG GHG Emissions Reduction & Sequestration Product Requirements (Version 2.1), Therefore, the project is a large-scale GS VER project.

A.5 Funding sources of project

>>

The project will be financed by the project owner only during the entire operational lifetime. No public funding will be involved. On the long run, the project will be financially sustainable through income from organic fertilizer and carbon credits.

Project financing for this project activity will not use Official Development Assistance (ODA) Funds. The signed ODA declaration template will be provided as evidence.

SECTION B. APPLICATION OF APPROVED GOLD STANDARD METHODOLOGY (IES) AND/OR DEMONSTRATION OF SDG CONTRIBUTIONS

B.1. Reference of approved methodology (ies)

>>

The approved methodology applied by this project is referenced as ACM0010” GHG emission reductions from manure management systems (Version 08.0)”.

The project also applies the following Tools,

Tool 02: “Combined tool to identify the baseline scenario and demonstrate additionality(version07.0)”

Tool 05:” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” (Version 3.0)

Tool 06:” Project emissions from flaring” (Version 4.0)

Tool 08: “Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version03.0)”

Tool 14: “Project and leakage emissions from anaerobic digesters (Version 02.0)”

Tool 24: “Common practice (Version 03.1)”

B.2. Applicability of methodology (ies)

>>

The following table summarize the justification of the applicability conditions:

| ACM0010 (Version 08.0)” GHG emission reductions from manure management systems” |  |
| --- | --- |
| Applicability Criteria | Justification |
| This methodology applies to project activities that include destruction of methane emissions and displacement of a more GHG-intensive service in manure management of livestock farms by introducing a new animal waste management system or a combination of animal waste management systems that result in less GHG emissions. | This project introduces new animal waste management systems to treat the manure and wastewater from the 9 existing swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. The biogas generated during the treatment process will be captured for power generation and all the electricity will be used by the operation of AWMSs and 9 swine farms and will not be connected to another user or to the regional power grid. |
| This methodology is applicable to manure management on livestock farms where the existing anaerobic manure treatment system, within the project boundary, is replaced by one or a combination of more than one animal waste management systems (AWMSs) that result in less GHG emissions compared to the existing system. The methodology is also applicable to Greenfield facilities. | For this project, 9 identical sets of new AWMS are installed in 9 existing swine farms in order to treat the manure and wastewater from these swine farms, which was treated in the baseline uncovered anaerobic lagoons prior to the implementation of the project. The project activity will reduce of GHG in the atmosphere through avoiding methane emissions from anaerobic treatment of swine manure and wastewater. |
| This methodology is applicable to manure management projects under the following conditions: - Farms where livestock populations, comprising of cattle, buffalo, swine, sheep, goats, and/or poultry, is managed under confined conditions; - Farms where manure is not discharged into natural water resources (e.g. rivers or estuaries); - In case of anaerobic lagoons treatments systems, the depth of the lagoons used for manure management under the baseline scenario should be at least 1 m; - The annual average ambient temperature at the site where the anaerobic manure treatment facility in the baseline existed is higher than 5°C; - In the baseline case, the minimum retention time of manure waste in the anaerobic treatment system is greater than one month; - The AWMS(s) in the project case results in no leakage of manure waste into ground water, for example the lagoon should have a non-permeable layer at the lagoon bottom. | (a) This project installs new AWMSs to a group of 9 swine farms in Hubei Province, which are owned by Jiangxi Zhengbang Breeding Co. Ltd., which is a large national agricultural and livestock enterprise specializing in swine breeding, feed sales and swine slaughtering. All the swine farms of the project had obtained the licenses for production and operation of the breeding livestock and poultry issued by local government and livestock and poultry housing layout in each farm should be reasonable as per the requirement of this license. Therefore, all swine is managed under confined conditions. (b)all the swine manure is taken into animal waste management systems and it is prohibited to discharge into any natural water resources without treatment according to Regulations on Prevention and Control of Pollution from Livestock and Poultry Farming. (c) The open anaerobic lagoons considered in the baseline scenario are designed for deep storage and has a depth of 3-5 meters. (d) The annual average ambient temperature at the site is 16°C，which is higher than 5°C. (e)As per the basic parameters of baseline anaerobic lagoon in each swine farm (the size of anaerobic lagoon in each swine farm are listed in section A.3), the minimum retention time of manure waster in the anaerobic treatment system can more than 60 days. (f) The anaerobic tanks of the AWMS in the project case is fully enclosed and have steel layer, which can ensure that no leakage of manure waste into ground water takes place. |

| Tool 02: “Combined tool to identify the baseline scenario and demonstrate additionality(version07.0)” |  |
| --- | --- |
| The tool is applicable to all types of proposed project activities. However, in some cases, methodologies referring to this tool may require adjustments or additional explanations as per the guidance in the respective methodologies. This could include, inter alia, a listing of relevant alternative scenarios that should be considered in Step 1, any relevant types of barriers other than those presented in this tool and guidance on how common practice should be established. | The project activity is designed to introduce new animal waste management systems to a group of 9 existing swine farms to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. Alternative scenarios, barrier analysis, investment analysis and common practice analysis will be carried out based on Tool 02. Refer to section B.4 and B.5 of the PDD for more details. |

| Tool 05: “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation (version 03.0)” |  |
| --- | --- |
| If emissions are calculated for electricity consumption, the tool is only applicable if one out of the following three scenarios applies to the sources of electricity consumption: (a) Scenario A: Electricity consumption from the grid. The electricity is purchased from the grid only, and either no captive power plant(s) is/are installed at the site of electricity consumption or, if any captive power plant exists on site, it is either not operating or it is not physically able to provide electricity to the electricity consumer; (b) Scenario B: Electricity consumption from (an) off-grid fossil fuel fired captive power plant(s). One or more fossil fuel fired captive power plants are installed at the site of the electricity consumer and supply the consumer with electricity. The captive power plant(s) is/are not connected to the electricity grid; or (c) Scenario C: Electricity consumption from the grid and (a) fossil fuel fired captive power plant(s). One or more fossil fuel fired captive power plants operate at the site of the electricity consumer. The captive power plant(s) can provide electricity to the electricity consumer. The captive power plant(s) is/are also connected to the electricity grid. Hence, the electricity consumer can be provided with electricity from the captive power plant(s) and the grid. | The electricity generated by the project was used firstly for the operation of AWMSs normally, then the excess electricity was supplied to the swine farms. Unless no electricity generation for this project, the electricity consumption of the project will be supplied by Central China Power Grid (CCPG) , which falls under scenario A of Tool 05 (Version 03.0). Therefore, emissions related to electricity consumption need to be calculated based on Tool 05. |
| This tool can be referred to in methodologies to provide procedures to monitor amount of electricity generated in the project scenario, only if one out of the following three project scenarios applies to the recipient of the electricity generated: (a) Scenario I: Electricity is supplied to the grid; (b) Scenario II: Electricity is supplied to consumers/electricity consuming facilities; or (c) Scenario III: Electricity is supplied to the grid and consumers/electricity consuming facilities. | This methodological tool is applied for calculating for emission by electricity consumption in project activity. So, this criterion is not applicable. |
| This tool is not applicable in cases where captive renewable power generation technologies are installed to provide electricity in the project activity, in the baseline scenario or to sources of leakage. The tool only accounts for CO2 emissions. | Tool 05 is only used to calculate project emissions of electricity consumption supplied by CCPG. For conservativeness, baseline emissions of captive biogas power generation system are ignored. Only CO2 emissions will be accounted. |

| Tool 06: “Project emissions from flaring (version 04.0)” |  |
| --- | --- |
| This tool provides procedures to calculate project emissions from flaring of a residual gas. The tool is applicable to enclosed or open flares and project participants should document in the GS-PDD the type of flare used in the project activity. | The biogas generated during the treatment process will be captured for power generation and the residual biogas will be flared. One opened flare system is constructed in each swine farm; Total 9 flares system are used by the project activity.. |
| This tool is applicable to the flaring of flammable greenhouse gases where: (a) Methane is the component with the highest concentration in the flammable residual gas; and (b) The source of the residual gas is coal mine methane or a gas from a biogenic source (e.g. biogas, landfill gas or wastewater treatment gas). | The source of the residual biogas of the project activity is from anaerobic treatment process of the swine manure (biogenic source). As per Feasibility Study Report of the project, methane accounts for 60% of the biogas, which is the highest concentration in the flammable residual gas. |
| The tool is not applicable to the use of auxiliary fuels and therefore the residual gas must have sufficient flammable gas present to sustain combustion. For the case of an enclosed flare, there shall be operating specifications provided by the manufacturer of the flare. | No auxiliary fuels will be used by the flaring system. As per Feasibility Study Report of the project, methane accounts for 60% of the biogas. And methane is a kind of flammable gas. Operating specifications were provided by the manufacturer of the flare. |

| Tool 08: “Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version03.0)” |  |
| --- | --- |
| Typical applications of this tool are methodologies where the flow and composition of residual or flared gases or exhaust gases are measured for the determination of baseline or project emissions. | The amount of biogas produced from the digester and the amount of biogas for electricity generation will be collected and monitored. Refer to section B.7 of the PDD for more details. |
| Methodologies where CO2 is the particular and only gas of interest should continue to adopt material balances as the means of flow determination and may not adopt this tool as material balances are the cost-effective way of monitoring flow of CO2 | The biogas generated during the treatment process including CH4, H2S、O2 and etc, therefore this tool is adopted used for determining the mass flow of a greenhouse gas. The biogas generated during the treatment process including CH4, H2S, O2 CO, CO2, N2 and H2, of which CH4 is the main component. Therefore, this tool is adopted used for determining the mass flow of a greenhouse gas. |
| The underlying methodology should specify: (a) The gaseous stream the tool should be applied to; (b) For which greenhouse gases the mass flow should be determined; (c) In which time intervals the flow of the gaseous stream should be measured; and (d) Situations where the simplification offered for calculating the molecular mass of the gaseous stream (equations (3) or (17) is not valid (such as the gaseous stream is predominantly composed of a gas other than N2). | a) Methodological tool” Tool to determine the mass flow of a greenhouse gas in a gaseous stream” is applied in the PDD. b) The mass flow is determined in the monitoring plan of the PDD. c) The flow of the gaseous stream will be measured continuously. d) The gaseous stream is dry, equation (5) and (6) are used to calculate the mass flow of greenhouse gas. |

| Tool 14: “Project and leakage emissions from anaerobic digesters (Version 02.0)” |  |
| --- | --- |
| The following sources of project emissions are accounted for in this tool: (a) CO2 emissions from consumption of electricity associated with the operation of the anaerobic digester; (b) CO2 emissions from consumption of fossil fuels associated with the operation of the anaerobic digester; (c) CH4 emissions from the digester (emissions during maintenance of the digester, physical leaks through the roof and side walls, and release through safety valves due to excess pressure in the digester); and (d) CH4 emissions from flaring of biogas. | Electricity will be used during the operation of the anaerobic digester and the anaerobic digestion process of this project does not involve the use of fossil fuels. Meanwhile the biogas generated during the treatment process will be captured for power generation and the residual biogas will be flared if there is any surplus biogas. So, the project meets the (a), (c) and (d). |
| The following sources of leakage emissions are accounted for in this tool: (a) CH4 and N2O emission from composting of digestate; (b) CH4 emissions from the anaerobic decay of digestate disposed in a SWDS or subjected to anaerobic storage, such as in a stabilization pond. | The project activity will replace the current open anaerobic lagoons with 9 new closed anaerobic digesters. The biogas generated during the treatment process will be captured for power generation. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. So, the proposed project meets this criterial. |
| Emission sources associated with N2O emissions from physical leakages from the digester, transportation of feed material and digestate or any other on-site transportation, piped distribution of the biogas, aerobic treatment of liquid digestate and land application of the digestate are neglected because these are minor emission sources or because they are accounted in the methodologies referring to this tool. | Emission sources associated with N2O emissions from physical leakages from the digester, transportation of feed material and digestate or any other on-site transportation, piped distribution of the biogas, aerobic treatment of liquid digestate and land application of the digestate are neglected because these are minor emission sources. |

| Tool 24: “Common practice (version 03.1)” |  |
| --- | --- |
| This methodological tool is applicable to project activities that apply the methodological tool “Tool for the demonstration and assessment of additionality”, the methodological tool “Combined tool to identify the baseline scenario and demonstrate additionality”, or baseline and monitoring methodologies that use the common practice test for the demonstration of additionality. | The project applies the methodological tool “Combined tool to identify the baseline scenario and demonstrate additionality” for the demonstration of additionality. |
| In case the applied approved baseline and monitoring methodology defines approaches for the conduction of the common practice test that are different from those described in this methodological tool, the requirements contained in the methodology shall prevail. | It is consistent of Tool 24 (Version 03.1) and ACM0010 (Version 8.0) on approaches for the conduction of the common practice test. |

B.3. Project boundary

>>

As per ACM0010” GHG emission reductions from manure management systems” (version 08.0), the spatial extent of the project boundary encompasses the site of the AWMS(s), including the flare or energy and/or heat generation equipment and the power/heat source.

The proposed project boundary considers the GHG emissions that come from AWMSs, including the GHG emissions from the anaerobic digesters, GHG emissions from the power generation equipment, GHG emissions from sludge treatment by aerobic composting and GHG emissions from opened flaring system in 9 swine farms. The current situation is shown as follows.

Figure 3 The current situation in the swine farms

The project activity boundary is defined as follows.

Figure 4 The project boundary of the project (It is the same for each farm since the same technology applied for the 9 swine farms)

The greenhouse gases included or excluded from the project boundary are summarized in following table.

Emission Sources Included in or Excluded from the Project Boundary

| Source |  | GHGs | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline scenario | Emissions from the waste treatment processes | CO2 | No | CO2 emissions from the decomposition of organic waste are not accounted |
|  |  | CH4 | Yes | The major source of emissions in the baseline |
|  |  | N2O | Yes | Direct and indirect N2O emissions are accounted |
|  | Emissions from electricity consumption/ generation | CO2 | No | Excluded for simplification. This is conservative. |
|  |  | CH4 | No | Excluded for simplification. This is conservative |
|  |  | N2O | No | Excluded for simplification. This is conservative |
|  | Emissions from thermal energy generation | CO2 | No | The anaerobic lagoon does not consume thermal energy. |
|  |  | CH4 | No | Excluded for simplification. This is conservative |
|  |  | N2O | No | Excluded for simplification. This is conservative |
| Project scenario | Emissions from thermal energy use | CO2 | No | Excluded for simplification. This project does not use thermal energy. |
|  |  | CH4 | No | Excluded for simplification. This project does not use thermal energy. |
|  |  | N2O | No | Excluded for simplification. This project does not use thermal energy. |
|  | Emissions from on-site electricity use | CO2 | Yes | May be an important emission source. If electricity is consumed from the grid company |
|  |  | CH4 | No | Excluded for simplification. |
|  |  | N2O | No | Excluded for simplification. |
|  | Emissions from the waste treatment processes | N2O | Yes | Direct and indirect N2O emissions are accounted |
|  |  | CO2 | No | CO2 emissions from the decomposition of organic waste are not accounted |
|  |  | CH4 | Yes | The emission from anaerobic digesters and aerobic treatment |

B.4. Establishment and description of baseline scenario

>>

Baseline scenario has been identified using the methodological tool 02 “Combined tool to identify the baseline scenario and demonstrate additionality (Version07.0)”.

Step1: Identification of alternative to the project activity consistent with current laws and regulations

Step 1a: Define alternative scenarios to the project activity

Prior to the implementation of the project, all manure waste produced from the existing 9 swine farms was left to decay in 9 uncovered anaerobic lagoons at the livestock farms and methane is emitted to the atmosphere directly without any methane recovery and destruction facility. The uncovered anaerobic lagoons were replaced by 9 sets of AWMSs in these 9 swine farms for this project. Therefore, it belongs to the existing facilities.

According to methodology, baseline alternatives for managing the manure for the existing facilities, shall take into consideration, inter alia, the complete set of existing/possible manure management systems listed in the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (Volume 4, Chapter 10, Table 10.17).

The baseline and available alternatives for the manure management are listed as below table as 2006 IPCC:

| No. | IPCC Alternatives | Applicability | Justification |
| --- | --- | --- | --- |
| 1 | The manure is collected from the pasture/Range/Paddock | Not Applicable | The swine in this project is bred in confined barns rather than pasture/range/paddock, so this alternative is excluded. |
| 2 | Daily spread: Manure removed from confinement and applied to pasture within 24 hours of excretion | Not Applicable | For a large-scale swine farm, it is highly labor intensive to remove the manure and apply on a daily basis. Therefore, this manure management system is not in an economically attractive prospect. So, the exclusion of this potential baseline scenario can be justified. |
| 3 | Solid Storage: The manure is disposed by solid storage. | Not Applicable | This involves the storage of manure, typically for a period of several months, in unconfined piles or stacks. It is only a storage method of manure, not a disposal method. In addition, it is suitable for small family farms. The proposed project involves large-scale swine farms and the use of a scraping and flushing approach to remove manure which has large volumes of water. So, this manure management system is not a potential alternative baseline scenario. |
| 4 | Dry lot | Not Applicable | In dry climates animals may be kept on unpaved feedlots where the manure is allowed to dry until it is periodically removed. Upon removal the manure may be spread on fields. This system is not applicable to the conditions of the 10 farms in the project. The proposed project is Non-free captive mode. This method is only a storage method of manure, not a disposal method. So, this system is excluded. |
| 5 | The manure is disposed as liquid/slurry. | Not Applicable | This method is only a storage method of manure, not a disposal method. Manure is stored as excreted or with some minimal addition of water in either tanks or earthen ponds outside the animal housing, usually for periods less than one year. Since the amount of discharged manure is very large even on a daily bases, storing the liquid manure in the tank to distribute them to the farmland requires a lot of labour work. Therefore, it is unrealistic to implement such a task for the farms under the competition of the market. Therefore, this option faces significant barrier and is excluded from the baseline scenario. |
| 6 | Uncovered anaerobic lagoon | Applicable | As per “Technical specification for sanitation treatment of livestock and poultry manure”, the uncovered anaerobic lagoon is a kind of harmless treatment of liquid manure. After being treated in the uncovered anaerobic pond, the liquid can be used for agricultural irrigation, the solid can be composted, and the fertilizer can be used by local farmers for farming. Animal waste that has been treated by uncovered anaerobic lagoon can satisfy the above regulations. However, these regulations have not yet imposed any restrictions on GHG emissions. So, the uncovered anaerobic lagoon is a alternatives baseline scenario |
| 7 | Pit storage below animal confinements, <1month | Not Applicable | This method is only a storage method of manure, not a disposal method. The farms involved in this project are large-scale livestock farm and the manure quantity produced is too large to implement pit storage structure under the barns. In addition, the manure stored in below animal confinements were removed within 1month requires a lot of labour work. so, this scenario is excluded. |
|  | Pit storage below animal confinements, >1month | Not Applicable | This method is only a storage method of manure, not a disposal method. The farms involved in this project are large-scale livestock farm and the manure quantity produced is too large to implement pit storage structure under the barns. In addition, the excreted volume accumulated under the barns produces enteric fermentation gas, if it is not discharged out of the barns in time, the pigs will be quickly killed by the accumulation of toxic fumes. |
| 8 | Anaerobic digester | Applicable | A single Anaerobic digester is suitable for treating manure, but to implement such technology need high invest compared to Uncovered anaerobic lagoon, which is the most common and economic method. However, a single anaerobic process is not yet able to meet the requirements for the use of the waste and must be followed up with disposal, which requires the use of a combination of aerobic and anaerobic processes together. At present a combine Anaerobic Digester-Aerobic Treatment system is considered to be one of the most advanced manure management systems, but to implement such technology need high invest and the proposed project will not be invested and constructed without being registered as a GS VER project. |
| 9 | Burned for fuel | Not Applicable | The farms involved in this project are large scale swine farm, the manure is flushed to the anaerobic digester, the dung and urine generated from the farm is too large even on a daily basis, so it is unlikely to dry the dung before using as fuel. |
| 10 | Cattle and Swine deep Bedding, <1month | Not Applicable | This method is only a storage method of manure, not a disposal method. The deep bedding is laborious and this is counter to achieving economies of scale associated with large animal counts. The concentration of nocuous gas in the bedding is high enough to poison swine if it is disposed inappropriately, and it is favorable for the survival and breeding of vermin and microorganisms due to its high temperature and humidity. So, the deep bedding practice is excluded from consideration. This system is more applicable to small scale farms. |
|  | Cattle and Swine deep Bedding, >1month | Not Applicable |  |
| 11 | Composting - In-vessel | Not Applicable | Manure in this project is collected by using scraping and flushing system. Manure in this project is in liquid with large volume of water. Therefore, it is excluded from the possible baseline scenario. |
| 12 | Composting - Static pile | Not Applicable | Composting in piles with forced aeration but no mixing will consume a great deal of electricity for forced aeration because of the large quantity of swine manure. It is suitable to solid treat manure, not applicable for manure with larger volume of water. Therefore, it is excluded from the list of possible baseline scenarios. |
| 13 | Composting - Intensive windrow | Not Applicable | Composting in windrows with regular turning for mixing and aeration emits a large quantity of odor and GHGs during turning and consumes a lot of electricity for the aeration operation. It is suitable to treat solid manure. So, it is excluded from the list of possible baseline scenarios. |
| 14 | Composting – Passive windrow | Not Applicable | Composting in windrows with infrequent turning for mixing and aeration takes a long time and occupies a large area of land. This kind of solid manure management system emits strong odors and GHGs during turning. For this reason, it is excluded from the list of possible baseline scenarios. |
| 15 | Poultry manure with litter | Not Applicable | The farms involved in this project are large-scale swine farm, not the Poultry farm, so no poultry manure produced. This system is excluded. |
| 16 | Poultry manure without litter | Not Applicable | The farms involved in this project are large-scale swine farm, not the Poultry farm, so no poultry manure produced. This system is excluded. |
| 17 | Aerobic treatment (Anaerobic Digester-Aerobic Treatment system) | Applicable | A single aerobic treatment technique is not suitable for treating low concentration organic wastewater in wastewater. This kind of wastewater contains a large amount of highly concentrated organic wastewater, which is difficult to be treated by the aerobic method and does not meet the Chinese environmental quality standards and pollutant discharge standards for discharge to water bodies. Anaerobic digester-Aerobic Treatment system is considered to be one of the most advanced manure management systems, but to implement such technology need high invest and the proposed project will not be invested and constructed without being registered as a GS VER project. |

Outcome of Step 1a: In summary, the alternatives to the baseline scenario are identified by the tool 02:” Combined tool to identify the baseline scenario and demonstrate additionality (Version 07.0)”as:

Scenario 6: “The manure is disposed in an uncovered anaerobic lagoon”

Scenario 8&17: Anaerobic Digester-Aerobic Treatment  i.e. the proposed project activity not being registered as a GS project activity

Step 1b: Consistency with mandatory laws and regulations

There is no legal law and regulation to mandate the livestock farm owners to implement anaerobic digestion, aerobic or other biological treatment techniques to treat the animal manure in China. Only the manure is prohibited to discharge directly into environment without any treatment as per Regulations on Prevention and Control of Pollution from Livestock and Poultry Farming issued by the State Council in Nov.2013.

According to “Technical specification for sanitation treatment of livestock and poultry manure”, in China, the uncovered anaerobic lagoon is a kind of manure treatment method recognized by the state, which need minimum financial cost and is compliance with laws and regulations. Therefore, it is the most economic, viable and reasonable for livestock farm owners.

As the “Specifications for the construction of manure resource utilization facilities for large-scale livestock and poultry farms (for trial implementation)”, anaerobic digester, aerobic treatment or other biological treatment techniques methods to dispose manure waste are encouraged by the state and not mandatory.

Therefore, the Scenario 6: “The manure is disposed in an uncovered anaerobic lagoon” and Scenario 8&17: Anaerobic Digester-Aerobic Treatment are all consistency with mandatory laws and regulations. So, the outcome of Step 1b is unchanged from Step 1a.

Step 2: Barrier analysis

There are no technology barriers, acceptability barriers and financial barriers that may prevent these two alternative scenarios to occur. As per Tool 02 Combined tool to identify the baseline scenario and demonstrate additionality (Version 07.0), go to Step 3 investment analysis.

Step 3: Investment analysis

Undertake investment analysis of all the alternatives that don’t face any barriers, as identified in Step II. For each alternative, all cost and economic benefits attributable to the waste management scenario should be illustrated in a transparent and complete manner. Table 5 shows the investment analysis results of the uncovered anaerobic lagoon.

Table 5 Investment analysis of the uncovered anaerobic lagoon (104 RMB)

| Costs and benefits | 1 | 2 | 3-4 | 5 | 6-15 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Cash inflow | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Revenues from organic fertilizers sales | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Subsidy income | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Fixed assets residue value | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Recovered current capital | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Cash outflow | 25.5 | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 |
| Construction investment | 15.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Current capital | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| O&M cost | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 | 10.5 |
| Sales tax and associate charge | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Net cash flow | -25.5 | -10.5 | -10.5 | -10.5 | -10.5 | -10.5 |
| NPV (discount rate=9%) | -101.04 |  |  |  |  |  |
| IRR | undefined |  |  |  |  |  |

There are no potential revenues involved in the baseline scenario. There are only negative flows in the baseline scenario, so the Internal Rate of Return (IRR) cannot be calculated, and the economic comparison should be based on the Net Present Value (NPV) indicator. Table 6 summarize the results of financial analysis of Anaerobic Digester-Aerobic Treatment without GS VER revenues.

Table 6 Investment analysis of the Anaerobic Digester-Aerobic Treatment without GS VER revenues (104 RMB)

| Costs and benefits | 1 | 2 | 3 | 4 | 5-15 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Cash inflow | 0.00 | 2,797.81 | 2,797.81 | 2,797.81 | 2,391.66 | 3,311.21 |
| Revenues from organic fertilizers sales | 0.00 | 2,391.29 | 2,391.29 | 2,391.29 | 2,391.29 | 2,391.29 |
| Subsidy income | 0.00 | 406.52 | 406.52 | 406.52 | 0.36 | 0.00 |
| Fixed assets residue value | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 589.81 |
| Recovered current capital | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 330.11 |
| Cash outflow | 11,599.47 | 1,605.97 | 1,275.86 | 1,275.86 | 1,275.86 | 1,275.86 |
| Construction investment | 11,599.47 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Current capital | 0.00 | 330.11 | 0.00 | 0.00 | 0.00 | 0.00 |
| O&M cost | 0.00 | 1,227.08 | 1,227.08 | 1,227.08 | 1,227.08 | 1,227.08 |
| Sales tax and associate charge | 0.00 | 48.78 | 48.78 | 48.78 | 48.78 | 48.78 |
| Net cash flow | -11,599.47 | 1,098.96 | 1,416.85 | 1,404.22 | 1,044.66 | 1,945.77 |
| NPV (discount rate=9%) | -2,073.72 |  |  |  |  |  |
| IRR (%) | 5.51 |  |  |  |  |  |

As shown in above table, the NPV of both the project activity and the uncovered anaerobic lagoon are negative and the NPV of the project activity without GS VER revenues is far more negative than that of the uncovered anaerobic lagoon which means the cost of the project activity without GS VER revenues is much higher than the uncovered anaerobic lagoon.

The cumulative method is the most common method used in NPV discount rate recognition and discount rate is equal to the sum of the risk-free rate of profit, the risk rate of profit and the inflation rate and the risk-free rate of profit is typically 3%, the risk rate of profit is generally taken as 3-5% and inflation rate for the last ten years is 2.229%. the discount rate is usually no more than 15%. Therefore 9% of NPV discount rate for this project is appropriate.

Anaerobic Digester-Aerobic Treatment is considered to be one of the most advanced manure management systems in the world. High costs involved in the investment compared to other available systems is required to implement such technology. Only the project proponents can prove that they can get revenues from Gold Standard certification, it can attract the banks to provide loans for this project. For these reasons, without the revenues, PP is willing to adopt other manure management systems of lower costs which will lead to more GHG emissions.

For the proposed project, the applied benchmark in the investment analysis is animal industry Project IRR (after tax) of 9.5%, which is sourced from the “Notice on adjusting financial benchmark rate of return of construction projects in some industries” issued by NDRC and Ministry of Housing and Urban-Rural Development of PRC. So, the applied benchmark in the investment analysis meets the Benchmarks supplied by relevant national authorities, which is also appropriate.

The proposed project will use benchmark analysis method based on the consideration that benchmark, Project IRR is available.

(a) Basic parameters

| item | value | source |
| --- | --- | --- |
| Total static investment | 11,599.47*104RMB | FSR, 16/03/2020 |
| O&M cost | 1,227.08*104RMB | FSR, 16/03/2020 |
| Operation period | 15 years | FSR, 16/03/2020 |
| Annual organic fertilizers sales | 108,695 t/year | FSR, 16/03/2020 |
| Price of organic fertilizers | 220 RMB/t | FSR, 16/03/2020 |
| Emission reduction | 282,734 tCO2e | Calculated |
| GS VER Price | about 2.32 US dollar/ton | Expected |

(b) Comparison of the project IRR for the proposed project and the benchmark following table.

Without income from selling GS VERs, the IRR of the proposed project is 5.51%, lower than the benchmark IRR 9.5% and the proposed project is financially unacceptable because of its low profitability. While considering such income, the IRR of the proposed project is 10.68%, higher than the benchmark, and the proposed project is financially acceptable.

| Item | Without income from GS VERs | Benchmark IRR | With income from GS VERs |
| --- | --- | --- | --- |
| IRR | 5.51% | 9.5% | 10.68% |

Sensitivity analysis:

The purpose of this step is to examine whether the conclusion regarding the financial attractiveness is robust to reasonable variations of the critical assumptions.

According to Para 27 of methodological tool 27” Investment Analysis” (Version 11.0), the “variables, including the initial investment cost, that constitute more than 20% of either total project costs or total project revenues should be subjected to reasonable variation”. Therefore, the total static investment, annual organic fertilizers generation and annual power generation were taken as uncertain factors for sensitive analysis. Furthermore, the O&M cost which was widely included in the sensitivity analysis for projects in China was also examined in the analysis. As a result, the following parameters are selected for the analysis:

	Total static investment

	Annual organic fertilizers sales

	O&M cost

The results of sensitivity analysis of the three parameters of the proposed project are shown in the following table and figure:

| Item | -10% | -5% | 0 | 5% | 10% |
| --- | --- | --- | --- | --- | --- |
| Total static investment | 7.15% | 6.30% | 5.51% | 4.78% | 4.10% |
| Annual organic fertilizers sales | 3.47% | 4.50% | 5.51% | 6.49% | 7.45% |
| O&M cost | 6.66% | 6.09% | 5.51% | 4.92% | 4.32% |

The sensitivity analysis was further conducted, and the project IRR (after tax) could reach the benchmark of 9.5% if one of the following conditions can be achieved:

	Total static investment: decrease 21.76%.

	Annual organic fertilizers sales: increase 21.05%.

	O&M cost: decrease 35.90%

Since all the data used for the investment analysis was sourced from the Feasibility Study Report, which was completed by an authorized entity. Therefore, the data used in the investment analysis are believed to be reliable and credible and none of above conditions can be achieved:

Total static investment decreasing 21.76%: But the change of this extent is unlikely to happen. According to the publicly latest available sources, on the whole, the price indices for steel, fuel, power and construction materials and price indices for fixed asset investment in China have been increasing in the past years. Besides, as per general contract of engineering construction and purchase agreement of the main equipment (power generator, anaerobic tanks, turnover machines and flaring system), the actual investment costs of the project(11,952.26*104 RMB) is higher than that of the estimated value(11,599.47*104 RMB) calculated based on the Feasibility Study Report. As a result, the IRR cannot increase through the change of investment costs.

Annual organic fertilizers sales increasing 21.05%: the organic fertilizers is produced through the aerobic composting system, which is determined by the manure of the swine farm. Since the number of the swine in each farm is design scale of swine farms and it is also the maximum population of swine in the swine farms. The number of swine will not exceed design size in the future, At the same time, sales of organic fertilizers are only a small part of the total organic fertilizers’ generation and the price of organic fertilizers is determined by the raw material, production technology, the quality of organic fertilizer and so on. Also, the organic fertilizers produced by the project belong to semi-finished Products (or called compost), which is harmless but not reaching the Chinese organic fertilizer standard. It can be directly applied to farmland or sold to commercial organic fertilizer plants for further processing. The price of the semi-finished products is lower than the commercial organic fertilizers. And according to the sale agreement of organic fertilizer, the organic fertilizer is sold to local organic fertilizer plant with the fixed price i.e.,220 RMB/ton, which is the same as the estimated value in Feasibility Study Report. Therefore, the increase of annual organic fertilizers sales to threshold is impossible to achieve.

O&M cost decreasing 35.90%: However, the decrease of it is not likely to occur. The O&M cost encompasses sub-items of maintenance cost, salary & welfare, insurance of fixed assets, and other cost. Since the cost of materials, power and fuel and labors keep increasing in recent years based on “China Statistical Yearbook, 2021” . Moreover, the equipment is getting more and more with the abrasion, which means the maintenance cost is increasing in the coming years. As a result, the drastic decreasing of 35.90% in O&M cost is not realistic.

In summary, the NPV of both the project activity without GS VER revenues and the uncovered anaerobic lagoon are negative and the NPV of the project activity without GS VER revenues is far more negative than that of the uncovered anaerobic lagoon which means the cost of the project activity without GS VER revenues is much higher than the uncovered anaerobic lagoon, so, Alternative 6, i.e.,  the uncovered anaerobic lagoon is the most attractive course of action and is considered to be the baseline scenario.

B.5. Demonstration of additionality

Additionality for the project activity is demonstrated using Tool 02” Combined tool to identify the baseline scenario and demonstrate additionality (Version 07.0)”. which were already done in section B.4 of this PDD.

As per Tool 24 “Common practice (Version 03.1)”, Step 4 common practice analysis will be conducted as follows:

Step 4: Common practice analysis

The proposed project will achieve obvious greenhouse gas (GHG) emission reductions through capturing methane generated during the swine manure treatment process and the biogas can be used for power generation or flared. Therefore, according to Tool 02 “Combined tool to identify the baseline scenario and demonstrate additionality” (version 07.0) and the methodological Tool 24 “Common Practice” (Version 03.1), the measure of the proposed project belongs to “Methane formation avoidance”. Thus, sub-steps are adopted to take the common practice analysis.

Sub-step 4a. The proposed CDM project activity(ies) applies measure(s) that are listed in the definitions section above.

The proposed project is belonged to type (D) measure listed in the latest version of the methodology Tool 24 “common practice (version 03.1)”, i.e., Methane formation avoidance (example: use of biomass that would have been left to decay in a solid waste disposal site resulting in the formation and emission of methane, for energy generation). As per methodology Tool 24, the “common practice (Version 03.1)” available on the UNFCCC website is applied as below.

Through searching UNFCC website, CDM website, China CER exchange info-platform, GS website, VCS website, and Department of Agriculture and rural affairs of Hubei Province , there are 5 projects that were registered at CDM, 9 projects that were registered at GS, 8 projects that were registered at VCS, and which has been listed in below.

| NO. | Project name | Reference number |
| --- | --- | --- |
| 1 | Methane Recovery and Utilization CDM Project at Muyuan Swine Farm in Henan Province | CDM 1301 |
| 2 | Animal Manure Management System (AMMS) GHG Mitigation Project, Shandong Minhe Livestock Co. Ltd., Penglai, Shandong Province, P.R. of China | CDM 1891 |
| 3 | Inner-Mongolian Mengniu Aoya Biogas Power Project (1.36MW) | CDM 3488 |
| 4 | Beijing Deqingyuan Chicken Farm 2.4MW Biogas Power Project | CDM 3716 |
| 5 | Rural Eco-energy Project of Henan Hengyou Husbandry Development Co., Ltd | CDM 7595 |
| 6 | Ji’an Swine Farm Animal Manure Management System GHG Mitigation Project | GS11238 |
| 7 | Ganzhou Swine Farm Animal Manure Management System GHG Mitigation Project | GS 11222 |
| 8 | Shangrao Swine Farm Animal Manure Management System GHG Mitigation Project | GS11239 |
| 9 | Muyuan Nanyang Swine Farm Animal Waste Management System GHG Mitigation Project | GS 11255 |
| 10 | Heilongjiang Swine Farm Animal Waste Management System GHG Mitigation Project | GS11334 |
| 11 | Shuangbaotai Animal Manure Management System GHG Mitigation Project in Guangdong Province | GS11335 |
| 12 | Shuangbaotai Animal Manure Management System GHG Mitigation Project in Guangxi Province | GS11336 |
| 13 | Shuangbaotai Animal Manure Management System GHG Mitigation Project in Jiangxi Province | GS11337 |
| 14 | Shuangbaotai Animal Manure Management System GHG Mitigation Project in Anhui Province | GS11338 |
| 15 | Aonong AWMS GHG Mitigation Project in Fujian Province | VCS 2692 |
| 16 | Aonong AWMS GHG Mitigation Project in Jiangxi Province | VCS 2691 |
| 17 | Zhengbang AWMS GHG Mitigation Project in Sichuan Province | VCS 2697 |
| 18 | Zhengbang AWMS GHG Mitigation Project in Guangdong Province | VCS 2696 |
| 19 | Shuangbaotai AWMS GHG Mitigation Project in Jiangsu Province | VCS 2706 |
| 20 | Dekang AWMS GHG Mitigation Project in Sichuan Province | VCS 2740 |
| 21 | Dekang AWMS GHG Mitigation Project in Guizhou Province | VCS 2741 |
| 22 | Tanghe AMMS GHG Mitigation Project | VCS 2844 |

Sub-step 4a.1: calculate applicable capacity or output range as +/-50% of the total design capacity or output of the proposed project activity.

For this project, 9 swine farms involving 130,976 heads of marketing swine, 226,952 heads of breeding swine in stock are included, and are estimated to produce 522,575 tons of manure every year, therefore, the project belonging to “Methane formation avoidance” that can handle manure from 261,287.5 tons to 783,862.5 tons are included. So as per the above summary table of the projects, through searching CDM website, GS website, VCS website, the number of projects meeting this conditions is 8, and the detailed information is as follows:

GS11238, GS11239, GS11335, GS11338, VCS 2692, VCS 2691, VCS 2706 and VCS 2741.

Sub-step 4a.2: identify similar projects (both CDM and non-CDM) which fulfill all of the following conditions:

(a) The projects are located in the applicable geographical area;

The common practice analysis is limited to the provincial level as the provincial government is at the highest level of local governments. Local regulatory frameworks are generally set by local governments (e.g., price regulation, investment policy). only the activities in the same province could be regarded as sharing the same “comparable environment”. Therefore, Hubei province is considered to be a geographical area with comparable investment climate and is selected for geographical boundaries of common practice analysis.

As per the result in the Sub-step 4a.1, 8 projects meet the conditions of Sub-step 4a.1, but now none of the projects in above table are located in Hubei Province, so the number of similar projects in this condition(a) are 0;

(b) The project applies the same measure as the proposed project activity;

As described above, the project installs new AWMSs to treat the manure from 9 existing swine farms, which can avoid methane emissions generated in the baseline uncovered anaerobic lagoons. And the measure of the proposed project belongs to “Methane formation avoidance” as per Tool 24“Common practice (version03.1).

As per the result in the Sub-step 4a.1, none of the projects in above table are located in Hubei Province, and the number of similar projects in this condition(a) are 0, Therefore, the number of projects meeting both condition(a) and condition(b) is 0 ;

(c) The projects use the same energy source/fuel and feedstock as the proposed project activity, if a technology switch measure is implemented by the proposed project activity;

The project is to treat the manure from the 9 swine farms avoid methane emissions generated in the baseline uncovered anaerobic lagoons through new 9 set of AWMSs. The biogas captured for power generation, which is used for the operation of AWMSs and the 9 swine farms.

Since the number of projects identified by condition(a) and condition(b) is 0, so the number of projects meeting conditions a, b and c at the same time is still 0.

(d) The plants in which the projects are implemented produce goods or services with comparable quality, properties and applications areas (e.g., clinker) as the proposed project plant;

522,575 tons of animal manure waste can be treated annually and the biogas captured for power generation. If the similar projects can treat the manure waste and produce electricity should be considered as the comparable service.

Since the number of projects identified by condition(a), condition(b) and condition(c) is 0, so the number of projects meeting conditions a, b, c and d at the same time is still 0.

(e) The capacity or output of the projects is within the applicable capacity or output range calculated in Step 1;

Based on the calculation in Sub-step 4a.1, the output/capacity ranges of similar project are that can handle manure from 261,287.5 tons to 783,862.5 tons annually.

Since the number of projects identified by condition(a), condition(b), condition(c) and condition(d) is 0, so the number of projects meeting conditions a, b, c, d and e at the same time is still 0.

(f) The projects started commercial operation before the project design document (CDM-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity.

The similar project stared commercial operation before the start date of proposed project at 09/09/2020, which is earlier date.

Since the number of projects identified by condition(a), condition(b), condition(c), condition(d) and condition(e) is 0, so the number of projects meeting conditions a, b, c, d ,e and f at the same time is still 0.

Based on above analysis, it can conclude the project belonging to “Methane formation avoidance” with treatment of manure operated before 09/09/2020, that can handle manure from 261,287.5 tons to 783,862.5 tons annually in Hubei province is the similar project as the proposed project activity.

In summary, similar projects need to fulfill the all the conditions(from a to f) at the same time, so the number of similar projects is 0.

Sub-step 4a.3: within the projects identified in Step 2, identify those that are neither registered CDM project activities, project activities submitted for registration, nor project activities undergoing validation. Note their number, Nall.

According to the results identified in step 2, the number of similar projects is 0, there are no similar projects in this step, therefore Nall=0

Sub-step 4a.4: within similar projects identified in Step 3, identify those that apply technologies different that the technology applied in the proposed project activity. Note their number Ndiff.

Ndiff=0

Sub-step 4a.5: calculate factor F=1-Ndiff/Nall representing the share of similar projects (penetration rate of the measure/technology) using a measure/technology similar to the measure/technology used in the proposed project activity that deliver the same output or capacity as the proposed project activity.

According to the analysis above, Nall= Ndiff =0, and the factor F=1-Ndiff/Nall=1-1 =0<0.2. Therefore, the result of common practice assessment is: Nall-Ndiff=0 < 3 Therefore, the proposed activity is not a common practice in the region.

B.5.1 Prior Consideration

>>

As per “principles & requirements (version 1.2)”, the Retroactive project is defined as” the Stakeholder Consultation (1st round) is conducted after the Project Start Date”. For this project, the Stakeholder Consultation (1st round) is conducted on 18/10/2021 and the project start date is 09/09/2020, so this project is a Retroactive project. According to clause 4.1.49 of GS4GG Principles & Requirements (Version 1.2), Retroactive projects shall submit the required documents for preliminary review (time of first submission) within one year of the project start date. Retroactive Project submitted at a date later than one year from the project start date will not be eligible for Gold Standard certification.

For this project, the project start date is 09/09/2020, and PP have submitted the required documents for preliminary review on 06/09/2021 which within one year of the project start date. Therefore, the project is eligible for Gold Standard certification.

The project proponent has completed an investment analysis in the Feasibility Study Report, which considered the expected carbon revenue. During the online board meeting conducted on 08/07/2020, the project proponent determined to apply for Gold Standard certification of this project. Key events of the project are shown in the following table.

| Time | Milestone |
| --- | --- |
| 16/03/2020 | Completion of Feasibility Study Report (Including prior consideration of carbon revenue). |
| 08/07/2020 | Online board meeting regarding to the project implementation with Gold Standard. |
| 09/09/2020 | Start date of the project (the date on signing the biogas power generator equipment purchase contract). |
| 17/09/2020 | Start construction date for AWMS in Yongjiahe swine farm |
| 18/09/2020 | Start construction date for AWMS in Zhoujiachong swine farm |
| 23/09/2020 | Start construction date for AWMS in Jiucaiyuan and Gaoshanmiao swine farm |
| 25/09/2020 | Start construction date for AWMS in Shayang 1st phase and Ezhou swine farm |
| 22/09/2020 | Start construction date for AWMS in Shayang 2nd phase and Tuchong swine farm |
| 24/09/2020 | Start construction date for AWMS in Sangzihu swine farm |
| 20/11/2020 | Training for the employees |
| 15/12/2020 | Finish construction and start commission date for AWMS in Yongjiahe and Sangzihu swine farm |
| 17/12/2020 | Finish construction and start commission date for AWMS in Zhoujiachong swine farm |
| 18/12/2020 | Finish construction and start commission date for AWMS in Jiucaiyuan and Shayang 2nd phase swine farm |
| 19/12/2020 | Finish construction and start commission date for AWMS in Gaoshanmiao swine farm |
| 13/12/2020 | Finish construction and start commission date for AWMS in Shayang 1st phase swine farm |
| 20/12/2020 | Finish construction and start commission date for AWMS in Ezhou swine farm |
| 21/12/2020 | Finish construction and start commission date for AWMS in Tuchong swine farm |
| 01/01/2021 | The project was put into operation |
| 06/09/2021 | The first submission date to GS |
| 18/10/2021 | Stakeholder consultation physical meeting |

B.5.2 Ongoing Financial Need

>>

According to the results of investment analysis described in section B.4, IRR without carbon revenue of the project is 5.51%, which is lower than 9.5% benchmark of construction project for animal industry. The project can apply for carbon credits under GS project, the IRR can reach to 10.68% with carbon revenue. Therefore, the revenue from Gold Standard certification is material to the ongoing sustainability of the project.

Conclusion of the assessment and demonstration of additionality

To summarize, “the Project is undertaken without being registered as a GS project activity” is not financially attractive to investors, thus it is not feasible. Being registered as a GS project, the VERs revenues can alleviate the identified barriers, therefore the proposed project is additional.

B.6. Sustainable Development Goals (SDG) outcomes

Relevant Target/Indicator for each of the three SDGs

| Sustainable Development Goals Targeted | Most relevant SDG Target | SDG Impact |
| --- | --- | --- |
|  |  | Indicator (Proposed or SDG Indicator) |
| 13 Climate Action | Target 13.2 integrate climate change measures into national policies, strategies and planning The project activity is designed to introduce new animal waste management systems to a group of 9 swine farms to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. Besides, the project will provide an opportunity for local residents to learn and raise awareness on climate change and mitigation measures on the stakeholder consultation fiscal meeting. | 13.2.1 Number of countries with nationally determined contributions, long-term strategies, national adaptation plans, strategies as reported in adaptation communications and national communications Project Monitoring Indicator: Amount of GHGs emission avoided or sequestered (tCO2e) |
| 8 Decent Work and Economic Growth | Target 8.5 By 2030, achieve full and productive employment and decent work for all women and men, including for young people and persons with disabilities, and equal pay for work of equal value The project activity will provide full-time job opportunities for locals during project implementation and monitoring activities irrespective of gender or any other status. | 8.5.1 Average hourly earnings of employees, by sex, age, occupation and persons with disabilities Project Monitoring Indicator: Total number of jobs. |
| 7 Affordable and Clean Energy | Target 7.2 By 2030, increase substantially the share of renewable energy in the global energy mix The project activity is designed to introduce new AWMSs to treat the manure and wastewater from the 9 swine farms. The biogas generated during the anaerobic digestion treatment process will be captured for power generation, the electricity generated are all used by the operation of AWMSs and the swine farms, which is supplied by the grid company in baseline scenario. The grid company is dominated by thermal power generation. | 7.2.1 Renewable energy share in the total final energy consumption Project Monitoring Indicator: Total electricity produced (MWh) |

B.6.1 Explanation of methodological choices/approaches for estimating the SDG Impact

>>

| SDG contribution and project monitoring indicator | Project monitoring indicator | Baseline outcome | Project outcome |
| --- | --- | --- | --- |
| SDG 8 Decent Work and Economic Growth | Total number of jobs | In baseline situation, no new job created. Therefore, the baseline outcome is 0. | In project situation, the number of jobs created for males and females will be recorded. Source of data is record keeping book and it will be cross checked by the labor contracts. Net benefit of SDG8=Project outcome of SDG8 – Baseline outcome of SDG8 |
| SDG 7 Affordable and Clean Energy | Total electricity produced | In baseline situation, no biogas can be generated and utilized. No clean energy can be generated in baseline situation. The electricity consumption of the swine farm is supplied by the grid company, which is dominated by thermal power generation. Therefore, the baseline outcome is 0. | For ex ante calculation, in project situation, the amount of electricity generated by capturing biogas annually can be estimated through the amount of manure of swine farm and estimated biogas. During the monitoring period, the amount of electricity generated by capturing biogas can be monitored by the electricity meters. Net benefit of SDG7=Project outcome of SDG7 – Baseline outcome of SDG7 |
| SDG 13 Climate Action | Amount of GHGs emissions avoided or sequestered | In baseline situation, Amount of GHGs emissions avoided or sequestered is 0 tCO2e. Therefore, the baseline outcome is 0. | In project situation, the amount of GHGs emissions avoided or sequestered can be obtained by baseline emission – project emissions – leakage emissions, and the baseline emission, project emissions, leakage emissions can be determined by ACM0010” GHG emission reductions from manure management systems (Version 08.0)”, the specific calculation method are described as follows. Net benefit of SDG13=Project outcome of SDG13 – Baseline outcome of SDG13 |

Baseline emissions

The baseline is the AWMSs identified through the baseline selection procedure in Section B.4 of this PDD. As per paragraph 26 of the applied methodology,

Baseline emissions are：

BEy=BECH4,y+BEN2O,y+BEelecheat,y         (Equation 1 in ACM0010)

where:

| BEy | Baseline emissions in year y (t CO2/yr) |
| --- | --- |
| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| BEN2O, y | Baseline N2O emissions in year y (t CO2/yr) |
| BEelec/heat,y | Baseline CO2 emissions from electricity and/or heat used in the baseline (t CO2/yr) |

i) Baseline CH4 emissions (BECH4, y)

BECH4,y=GWPCH4*DCH4*j,LT(MCFj*B0,LT*NLT*VSLT,y*MS%Bl,j)    (Equation 2 in ACM0010)

where:

| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| --- | --- |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/t CH4) |
| DCH4 | Density of CH4 (t/m3). 0.00067t/m3 at room temperature(20℃)and 1am pressure. |
| MCFj | Annual methane conversion factor (MCF) for the baseline AWMSj. IPCC 2006 Guildings, table 10.17, chapter 10,volume 4. |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg -dm) |
| NLT | Annual average number of animals of type LT for the year y (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%Bl,j | Fraction of manure handled in system j in the baseline. In this project, the baseline manure management system is uncovered anaerobic lagoon only. The amount of manure handled by the anaerobic lagoon is 100%. MS%Bl,j =100% |
| LT | Type of livestock |
| j | Type of treatment system |

Estimation of various variables and parameters for above equations:

(A) VSLT,y shall be determined in one of the following ways, presented in the order of preference.

Option 1:

Using published country specific data. If the data is expressed in kilogram volatile solid excretion per day on a dry-matter basis (kg -dm per day), multiply the value with ndy (number of days treatment plant was operational in year y).

Option 2:

Estimation of VSLT,y based on dietary intake of livestock:

VSLT,y=GELT*1-DELT100+(UE*GELT)*1-ASHEDLT*ndy                (Equation 3 in ACM0010)

where:

| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| --- | --- |
| GELT | Daily average gross energy intake (MJ/animal/day) |
| DELT | Digestible energy of the feed (per cent) |
| UE | Urinary energy (fraction of GELT) |
| ASH | Ash content of manure (fraction of the dry matter feed intake) |
| EDLT | Energy density of the feed fed to livestock type LT (MJ/kg -dm) |
| ndy | Number of days treatment plant was operational in year y |

Option 3:

Scaling default IPCC values VSdefault to adjust for a site-specific average animal weight as shown in equation below:

VSLT,y=WsiteWdefault*VSdefault*ndy                      (Equation 4 in ACM0010)

where:

| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| VSdefault | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population (kg-dm/animal/day) |
| ndy | Number of days treatment plant was operational in year y |

Option 4:

Utilizing published IPCC defaults for VSLT,y (IPCC 2006 guidelines, volume 4, chapter 10), multiply the value by ndy (number of days in year y).

Developed countries VSLT,y values may be used provided the following conditions are satisfied:

(a)The genetic source of the production operations livestock originates from an Annex I Party;

(b)The farm use formulated feed rations (FFR) which are optimized for the various animal(s), stage of growth, category, weight gain/productivity and/or genetics;

(c) The use of FFR can be validated (through on-farm record keeping, feed supplier, etc.); and

(d) The project specific animal weights are more similar to developed country IPCC default values.

There is no published country specific data available, so we could not use Option 1. The energy intake of the swine is not available, so we could not use Option 2. Option 3 utilizes the average weight of the swine, this data is available and therefore Option 3 is adopted to calculate VSLT,y.

Since option 3 is preferred for determining VSLT,y,  therefore Option 4 is not needed to be taken into account.

(B)Annual average number of animals of type LT (NLT) shall be determined in one of the following ways, presented in order of preference

Option 1:

NLT=Nda,LT*Np,LT365          (Equation 5 in ACM0010)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| Nda,LT | Number of days animal of type LT is alive in the farm in the year y (number) |
| Np,LT | Number of animals of type LT produced annually for the year y (number) |

Option 2:

If the project developer can monitor in a reliable and traceable way the daily stock of animals in the farm, discounting dead animals and animals discarded from the productive process from the daily stock, then the annual average number of animals (NLT) may be calculated as follows:

NLT=1365NAA,LT365     (Equation 6 in ACM0010)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| NAA,LT | Daily stock of animals of type LT in the farm, discounting dead and discarded animals (number) |

There are two types of swine in this project, i.e., Market swine and Breeding swine. For Market swine, since there is no way to trace the daily stock, so the Option 1 is adopted to calculate NLT for Market swine. For Breeding swine, the PP can monitor the daily stock of breeding swine in a reliable way, discounting dead breeding swine and discarded them from the productive process from the daily stock. So, the Option 2 is adopted to calculate NLT for Breeding swine.

(C) Maximum methane producing potential(B0,LT)

According to applied methodology ACM0010(Version 08.0), this value varies by species and diet. Default values are used, and they are taken from tables 10A-4 through 10A-9 (IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10).

The proposed project is located in Hubei Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

(D) Annual methane conversion factor (MCFj) for the baseline AWMSj

(a)The MCFj values given in table 10.17, chapter 10, volume 4, IPCC 2006 Guidelines should be used. MCFj values depend on the annual average temperature where the anaerobic manure treatment facility in the baseline existed. For this project, the annual average temperature is 16℃ and the value of 75% is applied.

(b)A conservativeness factor should be applied by multiplying MCFj values (estimated as per above bullet) with a value of 0.94, to account for the 20% uncertainty in the MCFj values as reported by IPCC 2006.

ii)Baseline N2O emissions (BEN2O,y)

BEN2O,y=GWPN2O*CFN2O-N,N*11000*（EN2O,D,y+EN2O,ID,y）        (Equation 7 in ACM0010)

where：

| BEN2O,y | Annual baseline N2O emissions in (t CO2e/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%Bl,j              (Equation 8 in ACM0010)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N).(Estimated with site-specific, regional or national data if such data is available, otherwise use default EF3 from table 10.21, chapter 10, volume 4, in the IPCC 2006 Guidelines for National Greenhouse Gas Inventories). The site-specific, regional or national data are not available, so this project activity adopts default EF3. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (5) or (6) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%Bl,j                   (Equation 9 in ACM0010)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N). (Estimated with site-specific, regional or national data if such data is available. Otherwise, default values for EF4 from table 11.3, chapter 11, volume 4 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories can be used). The site-specific, regional or national data are not available, so this project activity adopts default EF4. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (5(a)) or (5(b)) (number) |

Estimation of various variables and parameters for above equations:

(A)Procedure for estimating NEXLT,y

Option 1:

NEXLT,y=Nintake*1-Nretention*ndy           (Equation 1 in Appendix of ACM0010)

where:

| Nintake | Daily N intake per animal (kg N/animal/yr) |
| --- | --- |
| Nretention | Portion of that N intake that is retained in the animal (kg N retained/animal/yr) |
| ndy | Number of days treatment plant was operational in year y |

Nintake may be calculated using:

Nintake=GE18.45*(CP1006.25)                  (Equation 1a in Appendix of ACM0010)

where:

| CP | Crude per cent of protein (per cent) |
| --- | --- |
| GE | Gross energy intake of the animal (MJ/animal/day-) |
| 18.45 | Conversion factor for dietary GE per kg of dry matter (MJ/kg). This value is relatively constant across a wide range of forage and grain-based feeds commonly consumed by livestock |
| 6.25 | Conversion from kg of dietary protein to kg of dietary N, kg feed protein (kg N)-1 |

Option 2:

In the absence of availability of project specific information on protein intake, which should be justified in the “GS-PDD”, national or regional data should be used for the nitrogen excretion NEXLT,y, if available. In the absence of such data, default values from table 10.19 of the IPCC 2006, volume 4, chapter 10) may be used and should be corrected for the animal weight at the project site in the following way:

NEXLT,y=WsiteWdefault*NEXIPCC default                     (Equation 2 in Appendix of ACM0010)

where:

| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| NEX IPCC default | Default value for the nitrogen excretion per head of a defined livestock population (kg N/animal/year) |

NEXIPCC default=NrateT*TAM1000*365                (Equation 10.30 in 2006 IPCC)

where:

| Nrate(T) | the default N excretion rate, kg N/ (1000 kg animal mass)/ day, table 10.19, chapter 10, volume 4 of IPCC 2006 Guidelines |
| --- | --- |
| TAM | Typical animal mass for livestock in kg/animal |

For this project, neither specific information on Portion of that N intake nor site-specific national or regional data is available. So, the Option 2 is adopted to calculate NEXLT,y.

iii) Baseline CO2 emission from electricity and/or heat used in the baseline

BEelecheat,y=BEEC,y+BEHG,y              (Equation 9 in ACM0010)

where:

| BEelec/heat,y | Baseline CO2 emissions from electricity and/or heat used in the baseline (t CO2/yr) |
| --- | --- |
| BEEC,y | Baseline emissions associated with electricity generation in year y (t CO2/yr) |
| BEHG,y | Baseline emissions associated with heat generation in year y (t CO2/yr) |

The baseline scenario of this project is uncovered anaerobic lagoon, and no heat used in the baseline, only minor electricity will be used, so the emission can be excluded for simplification. In addition, the biogas generated during the treatment process in this project will be captured for power generation and all the electricity generated from this project will be used by the operation of AWMSs and the 9 swine farms. The electricity generated will not be connected to another user or to the regional power grid. So, the baseline CO2 emission from electricity and/or heat used in the baseline is 0, which is conservative.

Project Emissions

Two stages are involved in the manure treatment for the project activity: (1) anaerobic digester; (2) aerobic treatment of biogas liquid in lagoon.

Project emissions are estimated as follows:

PEy=PEAD,y+PEAer,y+PEN2O,y+PEECFC,y            (Equation 11 in ACM0010)

where：

| PEy | Project emissions in year y |
| --- | --- |
| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e/yr) |
| PEAer,y | Project CH4 emissions from aerobic AWMS treatment (t CO2e/yr) |
| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| PEEC/FC,y | Project emissions from electricity consumption and fossil fuel combustion (t CO2e/yr) |

i) Project emissions associated with the anaerobic digester in year y (PEAD,y)

Based on the methodology ACM0010” GHG emission reductions from manure management systems (Version 08.0)”. PEAD,y is determined using the methodological tool “Project and leakage emissions from anaerobic digesters”.

According to the TOOL14” Project and leakage emissions from anaerobic digesters (Version 02.0)”, the project emissions associated with the anaerobic digester (PEAD,y) are determined as follows:

PEAD,y=PEEC,y+PEFC,y+PECH4,y+PEflare,y              (Equation 1 in Tool 14)

where:

| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e) |
| --- | --- |
| PEEC,y | Project emissions from electricity consumption associated with the anaerobic digester in year y (t CO2e) |
| PEFC,y | Project emissions from fossil fuel consumption associated with the anaerobic digester in year y (t CO2e) |
| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| PEflare,y | Project emissions from flaring of biogas in year y (t CO2e) |

Since the electricity consumption of the anaerobic digestion system cannot be measured separately from the entire AWMS, so the Project emissions from electricity consumption associated with the anaerobic digester and that is not related to the anaerobic digester will be calculated together.

The project emissions from electricity consumption calculated according to the Tool 05 “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”,

PEEC,y=j,LTECPJ,J,y*EFEF,j,y*(1+TDLj,y)              (Equation 1 in Tool 05)

Where

| PEEC,y | Project emissions from electricity consumption in year y (t CO2e) |
| --- | --- |
| EGPJ,J,y | Quantity of electricity consumed by the project electricity consumption source j in year y (MWh/yr) |
| EFEF,j, y | Emission factor for electricity generation for source j in year y (t CO2/MWh) |
| TDLj,y | Average technical transmission and distribution losses for providing electricity to source j in year y |

As biogas will be produced during the anaerobic treatment of the project, and the biogas will be collected for power generation and all the power will be used for the operation of AWMSs firstly and then surplus electricity will be supplied to the swine farm. So, there is no outsourcing power consumption, i.e., the PEEC,y= 0 in pre-calculation. During the monitoring period, the actual electricity consumption from the region grid of the AWMSs will be monitored by electric meters.

The anaerobic digestion process of this project does not involve the use of fossil fuels, so the project emissions from fossil fuel consumption associated with the anaerobic digester is 0, i.e., PEFC,y=0

While the flaring will be not taken into account as the equipment will only be used when there will be surplus biogas, normally there will be no surplus biogas during the operational period of the project, so PEflare,y=0.

In case, there is residual gas stream which will be flared by flaring, and the project emissions from flaring of biogas (PEflare,y) shall be estimated using the tool 06 “ Project emissions from flaring”(version 04.0). As per Feasibility Study Report, all the biogas produced will be used to produce electricity, so, PEflare,y=0 for pre-calculation. While, during the monitoring period, the amount of biogas flared will be monitored by flow meters. The specific calculate method are as follows:

The calculation procedure in this tool determines the project emissions from flaring the residual gas (PEflare,y) based on the flare efficiency (ηflare,m) and the mass flow of methane to the flare (FCH4,RG,m). The flare efficiency is determined for each minute m of year y based either on monitored data or default values.

The calculation procedure of project emissions from flaring is given in the following steps:

STEP 1: Determination of the methane mass flow of the residual gas;

STEP 2: Determination of the flare efficiency;

STEP 3: Calculation of project emissions from flaring.

Step 1: Determination of the methane mass flow in the residual gas

The “Tool to determine the mass flow of a greenhouse gas in a gaseous stream” shall be used to determine the following parameter:

The following requirements apply:

(a) The gaseous stream tool shall be applied to the residual gas;

(b) The flow of the gaseous stream shall be measured continuously;

(c) CH4 is the greenhouse gas i for which the mass flow should be determined;

(d) The simplification offered for calculating the molecular mass of the gaseous stream is valid (equations 3 and 17 in the tool); and

(e) The time interval t for which mass flow should be calculated is every minute m.

FCH4,m, which is measured as the mass flow during minute m, shall then be used to determine the mass of methane in kilograms fed to the flare in minute m (FCH4,RG,m). FCH4,m shall be determined on a dry basis.

Therefore, option A is adopted to calculate the mass flow of the residual biogas for flaring as per Too 08 “Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)”.

As per paragraph 23 of Tool 8:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)”, the way to prove that the gaseous stream is dry needs to demonstrate that the temperature of the gaseous stream (Tt) is less than 60ºC (333.15 K) at the flow measurement point. For this project, the flowmeters installed in the outlet of the anaerobic tanks and the temperature of the anaerobic treatment unit of this project is designed as medium temperature, and the optimal temperature range is 35 ~ 38ºC. Therefore, the gas temperature measured by the flowmeter does not exceed 60ºC, it can be demonstrated that the gaseous stream is dry.

The mass flow of greenhouse gas i (Fi,t) is determined as follows:

Fi,t=Vt,db*υi,t,db*ρi,t               (Equation 5 in Tool 08)

ρi,t=Pt*MMiRu*Tt                     (Equation 6 in Tool 08)

where:

| Fi,t | Mass flow of greenhouse gas i in the gaseous stream in time interval t (kg gas/h) |
| --- | --- |
| Vt,db | Volumetric flow of the gaseous stream in time interval t on a dry basis (m³ dry gas/h) |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |
| Pt | Absolute pressure of the gaseous stream in time interval t (Pa) |
| MMi | Molecular mass of greenhouse gas i (kg/kmol) |
| Ru | Universal ideal gases constant (Pa.m3/kmol.K) |
| Tt | Temperature of the gaseous stream in time interval t (K) |

Step 2: Determination of flare efficiency

The flare efficiency depends on the efficiency of combustion in the flare and the time that the flare is operating. For determining the efficiency of combustion of enclosed flares there is the option to apply a default value or determine the efficiency based on monitored data. For open flares a default value must be applied. The time the flare is operating is determined by monitoring the flame using a flame detector and, for the case of enclosed flares, in addition the monitoring requirements provided by the manufacturer’s specifications for operating conditions shall be met.

The flare in this project belongs to open flares.  According to Tool 06 paragraph 18: in the case of open flares, the flare efficiency in the minute m (ηflare,m) is 50% when the flame is detected in the minute m (Flamem), otherwise ηflare,m is 0%.

Since the flame is not detected in the minute, therefore, fixed value of 0% for the flare efficiency will be applied for this project, and this is for conservative.

Step 3: Calculation of project emissions from flaring

Project emissions from flaring are calculated as the sum of emissions for each minute m in year y, based on the methane mass flow in the residual gas (FCH4,RG,m) and the flare efficiency (ηflare,m), as follows:

PEflare,y=GWPCH4,y*m=1525600FCH4,GR,m*1-ηflare,m*10-3

(Equation 15 in Tool 06)

Where:

| PEflare,y | Project emissions from flaring of the residual gas in year y (tCO2e) |
| --- | --- |
| GWPCH4 | Global warming potential of methane valid for the commitment period (tCO2e/tCH4) |
| FCH4,RG,m | Mass flow of methane in the residual gas in the minute m (kg) |
| ηflare,m | Flare efficiency in minute m |

As described above, FCH4,RG,m will be determined according to equation 5 and 6 in Tool 08,

PEflare,y=GWPCH4,y*Vt,db*υi,t,db*ρi,t*1-ηflare,m*10-3       (Combine Equation 15 in Tool 06 and Equation 5 in Tool 08)

| Vt,db | Volumetric flow of the residual gas for flaring in time interval t on a dry basis (m³ dry gas/h) |
| --- | --- |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream for flaring in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |

Since the gaseous stream will be distributed though the same pipeline, and the temperature and pressure of gaseous stream can be stable. Therefore the υi,t,db and ρi,t  of residual gas will be same with the biogas produced in the anaerobic digester.

Determination of project emissions of methane from the anaerobic digester (PECH4,y)

PECH4,y was determined following the step 4 of the applied tool “Project and leakage emissions from anaerobic digesters(Version 02.0)”.Project emissions of methane from the anaerobic digester include emissions during maintenance of the digester, physical leaks through the roof and side walls, and release through safety valves due to excess pressure in the digester. These emissions are calculated using a default emission factor (𝐸𝐹𝐶𝐻4, 𝑑𝑒𝑓𝑎𝑢𝑙𝑡), as follows:

PECH4,y=QCH4,y*EFCH4,default*GWPCH4             (Equation 4 in Tool 14)

where:

| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| --- | --- |
| QCH4,y | Quantity of methane produced in the anaerobic digester in year y (t CH4) |
| EFCH4,default | Default emission factor for the fraction of CH4 produced that leaks from the anaerobic digester (fraction) |
| GWPCH4 | Global warming potential of CH4 (t CO2 / t CH4) |

Estimation of various variables and parameters for above equations:

(A) Quantity of methane produced in the anaerobic digester QCH4,y

According to the TOOL14” Project and leakage emissions from anaerobic digesters (Version 02.0)”. There are two different procedures to determine the quantity of methane produced in the digester in year y (𝑄𝐶𝐻4,𝑦). For large scale projects only Option 1 shall be used. For small scale projects, project participants may choose between Option 1 or Option 2. The proposed project belongs to large scale projects, so QCH4,y was determined following step 1 and Option 1 of the applied tool. Below is the formula used for the calculation of QCH4,y.

Option1: Procedure using monitored data

𝑄𝐶𝐻4,𝑦 shall be measured using the “Tool to determine the mass flow of a greenhouse gas in a gaseous stream”. When applying the tool, the following applies:

(a) The gaseous stream to which the tool is applied is the biogas collected from the digester;

(b) CH4 is the greenhouse gas i for which the mass flow should be determined; and

(c) The flow of the gaseous stream should be measured on an hourly basis or a smaller time interval; and then accumulated for the year y. Please note that units need to be converted to tons, when applying the results in this tool.

The mass flow of greenhouse gas i (Fi,t) is determined as follows:

Fi,t=Vt,db*υi,t,db*ρi,t               (Equation 5 in Tool 08)

ρi,t=Pt*MMiRu*Tt                  (Equation 6 in Tool 08)

where:

| Fi,t | Mass flow of greenhouse gas i in the gaseous stream in time interval t (kg gas/h) |
| --- | --- |
| Vt,db | Volumetric flow of the gaseous stream in time interval t on a dry basis (m³ dry gas/h) |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |
| Pt | Absolute pressure of the gaseous stream in time interval t (Pa) |
| MMi | Molecular mass of greenhouse gas i (kg/kmol) |
| Ru | Universal ideal gases constant (Pa.m3/kmol.K) |
| Tt | Temperature of the gaseous stream in time interval t (K) |

In summary, the Project emissions associated with the anaerobic digester in year y (t CO2e) is the sum of the Project emissions of methane from the anaerobic digester in year y (t CO2e), the project emissions from electricity consumption associated with the anaerobic digester and that is not related to the anaerobic digester and the project emission from flaring the biogas. i.e., PEAD,y= PECH4, y+ PEEC,y+ PEflare,y.

ii) Project CH4 emissions from aerobic AWMS treatment (PEAer, y)

IPCC guidelines specify emissions from aerobic lagoons as 0.1 per cent of total methane generating potential of the waste processed, which can be used as a default for all types of aerobic AWMS treatment.

PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j+PEsl,y               (Equation 12 in ACM0010)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (5(a)) or (5(b)) |
| PEsl,y | Project CH4 emissions from sludge disposed of in storage pit prior to disposal during the year y (t CO2e/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

All sludge produced from the aerobic composting will be used for land application, which is calculated as leakage emission, no sludge will be produced in the process of the manure treatment system in this project. So the PEsl,y=0.

So, PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j       (Equation 12 in ACM0010)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (5(a)) or (5(b)) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

iii) Project N2O emissions in year y (PEN2O,y)

PEN2O,y=GWPN2O*CFN2O-N,N*11000*(EN2O,D,y+EN2O,ID,y)          (Equation 14 in ACM0010)

where:

| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

Option1:

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%j             (Equation 15 in ACM0010)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (5(a)) or (5(b)) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%j     (Equation 16 in ACM0010)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (5(a)) or (5(b)) (number) |

Option2:

EN2O,D,y=jEFN2O,D,j*m=112(QEM,m*NEM,m)           (Equation 17 in ACM0010)

EN2O,ID,y=EFN2O,ID*j,LTFgasMS,j,LT*m=112(QEM,m*NEM,m)                (Equation 18 in ACM0010)

where:

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N) |
| QEM,m | Monthly volume of the effluent mix entering the manure management system (m3/month) |
| [N]EM,m | Monthly total nitrogen concentration in the effluent mix entering the manure management system (kg N/m3) |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kg N2O-N/kg NH3-N and NOX-N) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |

The same method used to estimate the emissions in the baseline should be used to estimate the project emissions of nitrous oxide, so the Option 1 is used to calculate the Project N2O emissions PEN2O, y.

iV) Project emissions from use of heat and/or electricity (PEelec/heat)

PEECFC,y=PEEC,y+jPEFC,j,y        (Equation 19 in ACM0010)

where:

| PEEC,y | Project emissions from electricity consumption in year y. The project emissions from electricity consumption will be calculated following the latest version of the “Tool to calculate baseline, project and/or leakage emissions from electricity consumption”. In case, the electricity consumption is not measured then the electricity consumption shall be estimated as follows ECPJ,y=iCPi,y*8760, where CPi,y is the rated capacity (in MW) of electrical equipment i used for the project activity. |
| --- | --- |
| PEFC,y | Project emissions from fossil fuel combustion in process j during the year y. The project emissions from fossil fuel combustion will be calculated following the latest version of the “Tool to calculate project or leakage CO2 emissions from fossil fuel combustion”. For this purpose, the processes j in the tool corresponds to all fossil fuel combustion in the AWMS (not including fossil fuels consumed for transportation of feed material and sludge or any other on-site transportation). |

These emissions should only be considered for consumption of electricity or heat that is not related to the anaerobic digester. As described in above, since the electricity consumption that is not related to the anaerobic digester cannot be separated from the total electricity consumption, therefore the emission for consumption of electricity is calculated in PEFC,y.

the same for the PEFC,y, please refer to PEFC,y calculation.

Therefore ,PEelec/heat=0

Leakage

Leakage covers the emissions from land application of treated manure as well as the emissions related to anaerobic digestion in a digester, occurring outside the project boundary. These emissions are estimated as net of those released under project activity and those released in the baseline scenario. Net leakage is only considered if they are positive.

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y+LEAD,y            (Equation 20 in ACM0010)

Where:

| LEPJ, N2O, y | Leakage N2O emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEBL, N2O, y | Leakage N2O emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEPJ, CH4, y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| LEBL, CH4, y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEAD, y | Leakage emissions associated with the anaerobic digester in year y (t CO2e) |

i) Estimation of leakage N2O emissions released during baseline scenario from land application of the treated manure in year y, LEBL, N2O, y

LEBL,N2O,y=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)       (Equation 21 in ACM0010)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT         (Equation 22 in ACM0010)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT        (Equation 23 in ACM0010)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT      (Equation 24 in ACM0010)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (5(a)) or (5(b)) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

ii) Estimation of leakage N2O emissions released during project activity from land application of the treated manure in year y, LEPJ, N2O

LEPJ,N2O=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)  (Equation 25 in ACM0010)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 26 in ACM0010)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 27 in ACM0010)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT    (Equation 28 in ACM0010)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (5(a)) or (5(b)) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

It is possible to measure the quantity of manure applied to land in kg manure/yr (QDM) and the nitrogen concentration in kg N/kg manure (NDM) in the manure to estimate the total quantity of nitrogen applied to land. In this case, n=1N1-RN,n*LTNEXLT,y*NLT should be substituted byQDM*NDM. 

iii) Estimation of leakage CH4 emissions from land application of the treated manure

The calculation of methane emissions from land application of manure in the baseline and project cases are estimated as below:

LEBL,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LT(B0,LT*NLT*VSLT,y*MS%j)   (Equation 29 in ACM0010)

LEPJ,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j    (Equation 30 in ACM0010)

where:

| LEBL,CH4,y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEPJ,CH4,y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to sludge being treated |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| DCH4 | Density of CH4 (t/m3) |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| NLT | Annual average number of animals of type LT estimated as per equation (5(a)) or (5(b)), expressed (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |
| MCFd | Methane conversion factor (MCFj) assumed to be equal to 1 |

iV) Estimation of leakage emissions associated with the anaerobic digester

LEAD,y is determined using the methodological tool “Project and leakage emissions from anaerobic digesters(Version 02.0).

The leakage emissions associated with the anaerobic digester (𝐿𝐸𝐴𝐷,𝑦) depend on how the digestate is managed. They include emissions associated with storage and composting of the digestate and are determined as follows:

LEAD,y=LEstorage,y+LEcomp,y     (Equation 5 in Tool 14)

where:

| LEAD,y | Leakage emissions associated with the anaerobic digester in year y (t CO2e) |
| --- | --- |
| LEstorage,y | Leakage emissions associated with storage of digestate in year y (t CO2e) |
| LEcomp,y | Leakage emissions associated with composting digestate in year y (t CO2e) |

For subsequent treatment stages, the reduction of the nitrogen during a treatment stage is estimated based on referenced data for different treatment types. Emissions from the next treatment stage are then calculated following the approach outlined above, but with nitrogen adjusted for the reduction from the previous treatment stages by multiplying by (1-RN), where RN is the relative reduction of nitrogen from the previous stage. The relative reduction (RN) of nitrogen depends on the treatment technology and should be estimated in a conservative manner. Default values for different treatment technologies can be found in appendix 1 (values for TN).

The anaerobic digestion process of this project is carried out in a fully enclosed system, the flaring will be not taken into account as the equipment will only be used when there will be surplus biogas, normally there will be no surplus biogas during the operational period of the project. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. Wastewater from the new animal waste management systems will be treated aerobically and then used for agriculture irrigation.so, the Estimation of leakage emissions associated with the anaerobic digester is 0. i.e., LEAD,y =0.

Emission reduction

The emission reduction ERy by the project activity during a given year y is the difference between the baseline emissions (BEy) and the sum of project emissions (PEy) and leakage, as follows:

ERy=BEy-PEy-LEy              (Equation 31 in ACM0010)

Further, in estimating emissions reduction for claiming certified emissions reductions, if the calculated CH4 baseline emissions from anaerobic lagoons are higher than the measured CH4 generated in the anaerobic digester in the project situation (QCH4,y in the tool “Project and leakage emissions from anaerobic digesters”), then the latter shall be used to calculate the emissions reduction for claiming certified emissions reductions. Therefore, the actual methane captured from an anaerobic digester shall be compared to the (BECH4,y - PEAD,y in the tool “Project and leakage emissions from anaerobic digesters”) and if found lower, then (BECH4,y - PEAD,y) (which is a component of BEy -PEy ) in Equation (Equation 31 in ACM0010) is replaced by QCH4,y.

B.6.2 Data and parameters fixed ex ante

SDG13

| Data/parameter table 9. |  |
| --- | --- |
| Data/parameter | GWPCH4 |
| Unit | tCO2e/tCH4 |
| Description | Global Warming Potential of CH4 |
| Source of data | IPCC AR5 |
| Value(s) applied | 28 |
| Choice of data or Measurement methods and procedures | All requests for Preliminary Review and /or Design Certification of GS4GG projects and PoAs submitted after 01 January 2021 shall use the AR5 GWP values for demonstrating additionality, determining contributions to the Sustainable Development Goals (SDGs) and calculating ex-ante emission reductions and removals according to the applicability of GWP for GS for the global goals projects. As per para. 2.1.2 of GS document “APPLICABILITY OF GLOBAL WARMING POTENTIAL FOR GOLD STANDARD FOR THE GLOBAL GOALS PROJECTS”: All monitoring, verifications and requests for performance certification of Certified products and/or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values for calculating emission reductions and removals accrued by GS4GG projects and PoAs. For this project, the start date of crediting period is 01/01/2021, the calculation emission reductions after 01/01/2021 should be applies AR5 GWP value. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter 10. |  |
| --- | --- |
| Data/parameter | GWPN2O |
| Unit | tCO2e/tN2O |
| Description | Global Warming Potential of N2O |
| Source of data | IPCC AR5 |
| Value(s) applied | 265 |
| Choice of data or Measurement methods and procedures | All requests for Preliminary Review and /or Design Certification of GS4GG projects and PoAs submitted after 01 January 2021 shall use the AR5 GWP values for demonstrating additionality, determining contributions to the Sustainable Development Goals (SDGs) and calculating ex-ante emission reductions and removals according to the applicability of GWP for GS for the global goals projects. As per para. 2.1.2 of GS document “APPLICABILITY OF GLOBAL WARMING POTENTIAL FOR GOLD STANDARD FOR THE GLOBAL GOALS PROJECTS”: All monitoring, verifications and requests for performance certification of Certified products and/or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values for calculating emission reductions and removals accrued by GS4GG projects and PoAs. For this project, the start date of crediting period is 01/01/2021, the calculation emission reductions after 01/01/2021 should be applies AR5 GWP value. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter table 11. |  |
| --- | --- |
| Data/parameter | DCH4 |
| Unit | t/m3 |
| Description | Density of CH4 |
| Source of data | ACM0010 Version 08.0 |
| Value(s) applied | 0.00067 |
| Choice of data or Measurement methods and procedures | 0.00067 t/m3 at room temperature 20℃ and 1 atm pressure. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter table 13. |  |
| --- | --- |
| Data/parameter | MCFj |
| Unit | - |
| Description | Methane conversion factor for the baseline AWMSj |
| Source of data | IPCC 2006 table 10.17, chapter 10, volume 4 |
| Value(s) applied | 70.05% |
| Choice of data or Measurement methods and procedures | MCFj value for uncovered anaerobic lagoon (baseline AWMS) is chosen. A conservativeness factor has been applied by multiplying MCFj value (i.e. 75%) with a value of 0.94, to account for the 20 per cent uncertainty in the MCFj values as reported by IPCC 2006. For this project, the annual average temperature is 16℃ and the value of 75 % is applied. Therefore, MCFj value of 70.5% will be applied. |
| Purpose of data | project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter table 14. |  |
| --- | --- |
| Data/parameter | Wdefault |
| Unit | kg |
| Description | Default average animal weight of a defined population |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | Wdefault(Market swine)=28kg Wdefault(Breeding swine)=28kg |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | N/A |

| Data/parameter table 15. |  |
| --- | --- |
| Data/parameter | VSdefault |
| Unit | kg-dm/animal/day |
| Description | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | VSdefault(Market swine)=0.3 VSdefault(Breeding swine)=0.3 |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter table 17. |  |
| --- | --- |
| Data/parameter | NEXIPCC default |
| Unit | kg N/ animal/year |
| Description | Default value for the nitrogen excretion per head of a defined livestock population |
| Source of data | Calculated by the equation: NEXIPCC default =Nrate(T)* TAM/1000*365 |
| Value(s) applied | NEXIPCC default (Market swine) =4.29 NEXIPCC default (Breeding swine) =2.45 |
| Choice of data or Measurement methods and procedures | NEXIPCC default is calculated as equation 10.30 in IPCC2006, Nrate(T) and TAM are default value from IPCC 2006 |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

| Data/parameter | Nrate,(T) |
| --- | --- |
| Unit | kg N (1000 kg animal mass)-1 day-1 |
| Description | default N excretion rate |
| Source of data | IPCC 2006 table 10.19, chapter 10, volume 4 |
| Value(s) applied | Nrate,(T) (Market swine) =0.42 Nrate,(T)(Breeding swine) =0.24 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter | TAM |
| --- | --- |
| Unit | kg animal-1 |
| Description | typical animal mass for livestock category |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | TAM (Market swine) =28 TAM (Breeding swine) =28 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter table 4. |  |
| --- | --- |
| Data/parameter | Fgas MS,j,LT |
| Unit | Fraction |
| Description | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management |
| Source of data | IPCC 2006 table 10.22, chapter 10, volume 4 |
| Value(s) applied | FgasMS,j,LT, (anaerobic lagoon) : 40% FgasMS,j,LT , (solid storage) : 45% |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter table 2. |  |
| --- | --- |
| Data/parameter | EFN2O,D,j |
| Unit | Kg N2O-N/kg N |
| Description | Direct N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 table 10.21, chapter 10, volume 4 |
| Value(s) applied | EFN2O, D=0 for anaerobic lagoon and digester, EFN2O, D=0.01 for arobic lagoon |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter table 3. |  |
| --- | --- |
| Data/parameter | EFN2O,ID,j |
| Unit | kgN2O-N/kg NH3-N and NOX-N |
| Description | Indirect N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 table 11.3, chapter 11, volume 4 |
| Value(s) applied | 0.01 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFCH4,default |
| --- | --- |
| Unit | t CH4 leaked / t CH4 produced |
| Description | Default emission factor for the fraction of CH4 produced that leaks from the anaerobic digester (fraction) |
| Source of data | IPCC (2006) |
| Value(s) applied | 0.05 |
| Choice of data or Measurement methods and procedures | UASB (Upflow Anaerobic Sludge Blanket) type digesters, floating gas holders with no external water seal |
| Purpose of data | Calculation of project emissions |
| Additional comment | PECH4,y for ex ante estimation adopted equation(4) of Methodological tool “ Project and leakage emissions from anaerobic digesters”, the amount of biogas collected at the digester will be monitored in section B.7 of PDD. |

| Data/parameter table 1. |  |
| --- | --- |
| Data/parameter | RVS,n |
| Unit | Fraction |
| Description | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated |
| Source of data | Refer to FSR |
| Value(s) applied | RVS,n, aerobic treatment anaerobic digester: 2%,45% and 35% for leakage N2O emission released during project activity RVS,n, one cell lagoon: 85% for leakage N2O emission released during baseline scenario RVS,n; ACM0010; Feasibility Study Report. Project Activity (aerobic treatment- anaerobic digester); 80% (cover first cell of two lagoon); 2%, 45% and 35%. baseline; 85%(one cell lagoon); /. |
| Choice of data or Measurement methods and procedures | For this project, values from Table provided in Appendix 1 of ACM0010 and Feasibility Study Report are compared in order to adopt the most conservative value for the given technology. |
| Purpose of data | project emission/ leakage calculation |
| Additional comment | N/A |

| Data/parameter table 19. |  |
| --- | --- |
| Data/parameter | RN,n |
| Unit | Fraction |
| Description | Nitrogen reduction factor |
| Source of data | Refer to Annex 1 of methodology ACM0010 |
| Value(s) applied | RN,n, anaerobic digester : 0.25 RN,n, uncovered anaerobic lagoon : 0.80 |
| Choice of data or Measurement methods and procedures | Anaerobic digester has been assumed as a covered first cell of two cell lagoon. Uncovered anaerobic lagoon has been assumed as one cell lagoon. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter table 6. |  |
| --- | --- |
| Data/parameter | EF1, EF4, EF5 |
| Unit | kg N2O-N/kg N for EF1, EF5 and [kg N2O-N/(kg NH3-N and NOX-N) for EF4 |
| Description | Emission factor for N2O emissions from N inputs; from N leaching and runoff; from atmospheric deposition of N on soils and water surfaces |
| Source of data | IPCC 2006 Guidelines default values are be used, since country specific or region specific data are not available. EF1 from table 11.1, chapter 11, volume 4. EF4 and EF5 from table 11.3, chapter 11, volume 4 |
| Value(s) applied | EF1 = 0.010 EF4 =0.010 EF5 = 0.0075 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter table 5. |  |
| --- | --- |
| Data/parameter | Fgasm |
| Unit | Fraction |
| Description | Fraction of N lost due to volatilization |
| Source of data | Default values from table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.2 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter table 7. |  |
| --- | --- |
| Data/parameter | Fleach |
| Unit | Fraction |
| Description | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff |
| Source of data | Default values from table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.3 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter table 12. |  |
| --- | --- |
| Data/parameter | MCFd |
| Unit | - |
| Description | Methane conversion factor for leakage calculation |
| Source of data | Methodology ACM0010(version 08.0) |
| Value(s) applied | 1 |
| Choice of data or Measurement methods and procedures | According to paragraph 51 of the methodology ACM0010” GHG emission reductions from manure management systems (version08.0)”, Methane conversion factor for leakage calculation assumed to be equal 1 |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | EFEF,j,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Emission factor for electricity generation |
| Source of data | Published by Ministry of Ecology and Environment of China which is the DNA of China. |
| Value(s) applied | 0.57205 |
| Choice of data or Measurement methods and procedures | According to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | Ru |
| --- | --- |
| Unit | Pa.m3/kmol.K |
| Description | Universal ideal gases constant |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 8,314 |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | MMi |
| --- | --- |
| Unit | kg/kmol |
| Description | Molecular mass of greenhouse gas i |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 16.04 kg/mol for methane |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | ηflare,m |
| --- | --- |
| Unit | % |
| Description | Flare efficiency in minute m |
| Source of data | Tool 06” Project emissions from flaring (Version 04.0)” |
| Value(s) applied | 0% |
| Choice of data or Measurement methods and procedures | 0% is the conservative value. |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

B.6.3 Ex ante estimation of SDG Impact

>>

SDG7

The biogas generated during the treatment process will be captured for power generation, the electricity generated are all used by the operation of AWMSs and the 9 swine farms, which was supplied by the grid company without the project. For ex ante estimation, electricity generation of 40,134 MWh can be generated by the project annually.

SDG 8

Total number of full-time jobs created by the project will be reported annually based on record keeping book and be cross checked by the labor contracts. For ex ante estimation, 18 full-time jobs are created including 9 males and 9 females.

SDG 13

As described in section B.6.1, the Net benefit of SDG13=Project outcome of SDG13 – Baseline outcome of SDG13. In baseline situation, the amount of GHGs emissions avoided or sequestered is 0 tCO2e. Therefore, the net benefit of SDG13 is equal to Project outcome of SDG13.  In project situation, the amount of GHGs emissions avoided or sequestered can be obtained by baseline emission – project emissions – leakage emissions, and the baseline emission, project emissions, leakage emissions can be determined by ACM0010” GHG emission reductions from manure management systems (Version 08.0)”, the specific calculation process has been described in section B.6.1 of this PDD and the result is described as follows:

Calculation of the baseline emission:

BEy=BECH4,y+BEN2O,y+BEelecheat,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | / |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFj | 75% | 75% | % |
| Conservative Factor | 0.94 | 0.94 | / |
| MCFj with cons. Factor | 0.705 | 0.705 | / |
| Bo,LT | 0.29 | 0.29 | m3 CH4 /kg_dm |
| NLT | 130,976 | 226,952 | No of heads |
| Wsite | 68.4 | 90.0 | kg |
| Wdefault | 28 | 28 | kg |
| VSdefault | 0.3 | 0.3 | kg-dm/day |
| VSLT,y | 267.49 | 351.96 | kg-dm/animal/year |
| MS%Bl,j | 100% | 100% | % |
| Ndy | 365 | 365 | days |
| Sub total | 134,377 | 306,374 | tCO2e |
| BECH4 | 440,751 |  | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| EFN2O,ID,j | 0.01 | 0.01 | kg N2O-N/kg N |
| Nrate(t) | 0.42 | 0.24 | kg N/1000kg animal mass/day |
| NEXIPCCdefault | 4.29 | 2.45 | kg N/animal/year) |
| Wsite | 68.4 | 90.0 | kg |
| TAM | 28.00 | 28.00 | kg |
| Wdefault | 28 | 28 | kg |
| NEXlt,y | 10.49 | 7.88 | kg N/animal/year |
| NLT | 130,976 | 226,952 | No of heads |
| MS%Bl,j | 100% | 100% | / |
| Fgasm | 40% | 40% | / |
| GWP | 265 | 265 | / |
| CFN20,N-N | 1.57 | 1.57 | Conversion Factor N2O-N to N20 |
| BEN2O,y | 5,268 |  | tCO2e |

Baseline Emissions: BECH4,y+ BEN2O,y =440,751 tCO2e +5,268 tCO2e =446,019 tCO2e

Calculation of the project emission

PEy=PEAD,y+PEAer,y+PEN2O,y+PEECFC,y

| Parameter | Value | Unit |
| --- | --- | --- |
| ECPJ,j,y | 0 | MWh/yr |
| EFEF,j,y | 0.57205 | tCO2/MWh |
| TDLj,y | 20% | / |
| PEEC,y | 0 | tCO2e |

| Parameter | Value | Unit |
| --- | --- | --- |
| GWPCH4 | 28 | / |
| EFCH4,default | 0.05 | / |
| QCH4,y | 12604.51 | t CH4 |
| PECH4,y | 17,647 | tCO2e |

PEAD,y= PEEC,y+ PECH4,y=0 tCO2e + 17,647 tCO2e= 17,647 tCO2e

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | kg N2O-N/kg N |
| DCH4 | 0.00067 | 0.00067 | kg N2O-N/kg N |
|  | 0.001 | 0.001 | kg N/1000kg animal mass/day |
| FAer | 54% | 54% | kg N/animal/year |
| 1-Rvs,1 | 54% | 54% | kg |
| B0,LT | 0.29 | 0.29 | m3CH4/kg-dm |
| NLT | 130,976 | 226,952 | No of heads |
| VSLT,y | 267.49 | 351.96 | kg-dm/animal/year |
| MS%j | 100% | 100% | / |
| Sub total | 55 | 127 | tCO2e |
| PEAer,y | 183 |  | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| NEXLT,y | 10.49 | 7.88 | kg N/animal/year |
| NLT | 130,976 | 226,952 | No of heads |
| MS%j | 46% | 46% | / |
| Sub total | 0.00 | 0.00 | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0.006 | 0.006 | kg N2O-N/kg N |
| NEXLT,y | 10.49 | 7.88 | kg N/animal/year |
| NLT | 130,976 | 226,952 | No of heads |
| MS%j | 54% | 54% | / |
| Sub total | 4,437.07 | 5,780.78 | kg N2O-N/year |
| EN2O,D,y | 10,217.85 |  | kg N2O-N/year |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
|  | Market Swine | Breeding Swine |  |
| EFN2O,iD,j | 0.01 | 0.01 | kg N2O/kg N |
| Fgasm | 0.4 | 0.4 | / |
| NEXLT,y | 10.49 | 7.88 | kg N/animal/year |
| NLT | 130,976 | 226,952 | No of heads |
| MS%j | 46% | 46% | % |
| GWP | 265 | 265 | / |
| Sub total | 2,535.47 | 3,303.30 | kg N2O-N/year |
| EN2O,ID,j | 5,839 |  | kg N2O-N/year |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,iD,j | 0.01 | 0.01 | kg N2O/kg N |
| Fgasm | 0.4 | 0.4 | / |
| NEXLT,y | 10.49 | 7.88 | kg N/animal/year |
| NLT | 130,976 | 226,952 | No of heads |
| MS%j | 46% | 46% | % |
| GWP | 265 | 265 | / |
| Sub total | 3,327.80 | 4,335.59 | kg N2O-N/year |
| EN2O,ID,y | 5,839 |  | kg N2O-N/year |
| PEN2O,Y | 9,878 |  | tCO2e |

Project emission:

PEy=PEAD,y+PEAer,y+PEN2O,y= 17,647 tCO2e +183 tCO2e +9,878 tCO2e =

27,708 tCO2e

Calculation of leakage

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| NLT | 130,976 | 226,952 | No of heads |
| Nrate | 0.42 | 0.24 | kg N/1000kg animal mass/day |
| NEXLT,y | 10.49 | 7.88 | kg/animal-yr |
| RN,n | 80% | 80% | / |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | kg N2O-N/kg N |
| Fleach | 0.3 | 0.3 | kg N |
| Fgasm | 0.2 | 0.2 | kg NH3-N,Nox-N/kg N |
| GWP | 265 | 265 | / |
| LEN2O,land | 2,746.76 | 3,578.58 | tCO2e |
|  | 6,325.33 |  | tCO2e |
| LEN2O,runoff | 618.02 | 805.18 | tCO2e |
|  | 1,423.2 |  | tCO2e |
| LEN2O,vol | 549.35 | 715.72 | tCO2e |
|  | 1,265.07 |  | tCO2e |
| LEB,N2O | 3,754 |  | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| NLT | 130,976 | 226,952 | No of heads |
| NEXLT | 10.49 | 7.88 | kg/yr |
| RN | 25% | 25% | - |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | kg N2O-N/kg N |
| Fleach | 0.3 | 0.3 | kg N |
| Fgasm | 0.2 | 0.2 | kg NH3-N,Nox-N/kg N |
| LEN2O,land | 10,300.33 | 13,419.67 | tCO2e |
|  | 23,720 |  | tCO2e |
| LEN2O,runoff | 2,317.57 | 3,019.43 | tCO2e |
|  | 5,337 |  | tCO2e |
| LEN2O,vol | 2,060.07 | 2,683.93 | tCO2e |
|  | 4,744 |  | tCO2e |
| LEP,N2O | 14,076 |  | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | / |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | 267.49 | 351.96 | kg/animal/day |
| NLT | 130,976 | 226,952 | No of heads |
| B0 | 0.29 | 0.29 | m3 CH4/kg-VS |
| RVS | 85% | 85% | / |
| MS%j | 100% | 100% | / |
| LEB,CH4 | 28,590.78 | 65,186.06 | tCO2e |
| Total | 93,777 |  | tCO2e |

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | / |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | 267.49 | 351.96 | kg/animal/day |
| NLT | 130,976 | 226,952 | numbers |
| B0 | 0.29 | 0.29 | m3 CH4/kg-VS |
| RVS | 2% | 2% | / |
| RVS | 45% | 45% | / |
| RVS | 35% | 35% | / |
| MS%j | 100% | 100% | / |
| LE p,CH4 | 66,778.53 | 152,252.90 | tCO2e |
| Total | 219,031 |  | tCO2e |

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y=

(14,076-3,754) tCO2e +(219,031-93,777) tCO2e =135,577 tCO2e

Emission Reduction: ERy=BEy-PEy-LEy=

(446,019-18,103-135,577) tCO2e =282,734tCO2e

So, the amount of GHGs emission avoided or sequestered in project situation in the first crediting period is listed below:

| Year | Baseline emissions (tCO2e) | Project emissions (tCO2e) | Leakage emissions (tCO2e) | Emission reductions (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/01/2021-31/12/2021 | 446,019 | 27,708 | 135,577 | 282,734 |
| 01/01/2022-31/12/2022 | 446,019 | 27,708 | 135,577 | 282,734 |
| 01/01/2023-31/12/2023 | 446,019 | 27,708 | 135,577 | 282,734 |
| 01/01/2024-31/12/2024 | 446,019 | 27,708 | 135,577 | 282,734 |
| 01/01/2025-31/12/2025 | 446,019 | 27,708 | 135,577 | 282,734 |
| Total | 2,230,095 | 138,540 | 677,885 | 1,413,670 |
| Average | 446,019 | 27,708 | 135,577 | 282,734 |

Therefore, the amount of GHGs emission avoided or sequestered of the SDG13 impact for ex-ante emission are 282,734 tCO2e.

B.6.4 Summary of ex ante estimates of each SDG Impact

Ex-ante estimation of SDG 7 Outcome

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| 01/01/2021-31/12/2021 | Total electricity produced:0 MWh | Total electricity produced: 40,134 MWh | electricity generation of 40,134 MWh can be generated by the project annually |
| 01/01/2022-31/12/2022 | Total electricity produced:0 MWh | Total electricity produced : 40,134 MWh | electricity generation of 40,134 MWh can be generated by the project annually |
| 01/01/2023-31/12/2023 | Total electricity produced: 0 MWh | Total electricity produced :40,134 MWh | electricity generation of 40,134 MWh can be generated by the project annually |
| 01/01/2024-31/12/2024 | Total electricity produced: 0 MWh | Total electricity produced: 40,134 MWh | electricity generation of 40,134 MWh can be generated by the project annually |
| 01/01/2025-31/12/2025 | Total electricity produced: 0 MWh | Total electricity produced : 40,134 MWh | electricity generation of 40,134 MWh can be generated by the project annually |
| Total | Total electricity produced: 0 MWh | Total electricity produced: 401,340 MWh | Total electricity produced: 401,340 MWh |
| Total number of crediting years | 5 |  |  |
| Annual average over the crediting period | Total electricity produced: 0 MWh | Total electricity produced: 40,134 MWh | Total electricity produced: 40,134 MWh |

Ex-ante estimation of SDG 8 Outcome

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| 01/01/2021-31/12/2021 | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| 01/01/2022-31/12/2022 | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| 01/01/2023-31/12/2023 | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| 01/01/2024-31/12/2024 | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| 01/01/2025-31/12/2025 | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| Total | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |
| Total number of crediting years | 5 |  |  |
| Annual average over the crediting period | 0 full-time jobs created. | 18 full-time jobs created including 9 male and 9 female | 18 full-time jobs created including 9 male and 9 female |

Ex-ante estimation of SDG 13 Outcome

| Year | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| 01/01/2021-31/12/2021 | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |
| 01/01/2022-31/12/2022 | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |
| 01/01/2023-31/12/2023 | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |
| 01/01/2024-31/12/2024 | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |
| 01/01/2025-31/12/2025 | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |
| Total | 0 tCO2e | 1,413,670 tCO2e | 1,413,670 tCO2e |
| Total number of crediting years | 5 |  |  |
| Annual average over the crediting period | 0 tCO2e | 282,734 tCO2e | 282,734 tCO2e |

B.7. Monitoring plan

B.7.1 Data and parameters to be monitored

SDG 13

| Data / Parameter | Np,LT |
| --- | --- |
| Unit | Number |
| Description | Number of animals of type LT produced annually for the year y |
| Source of data | Project proponents |
| Value(s) applied | 130,976 heads of commercial swine in stock (NLT) are used to ex-estimated, which is sourced from FSR. NLT is the design scale of swine farms and it is also the maximum population of swine in the swine farms. Based on 180 days of swine alive in the farm, about 265,590 head of commercial pigs can be produced annually (Np,LT) as per equation 5 in ACM0010 above. |
| Measurement methods and procedures | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals, can track automatically. This electronic ear tag will be connected to the Data Collection System (DCS), which can store and read information. Therefore, the number of swine produced in the farm can be monitored through the auto tracking devices of electronic ear tag once pig slaughter monthly and obtained by the DCS. At the same time, the technicians in farms will record manually the number of swine produced in the farms monthly. Also, the number of monthly exported from the stock will also be recorded. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | Indirect information (food purchase records and sale records) will be crosschecked. |

| Data / Parameter | Nda,LT |
| --- | --- |
| Unit | Number |
| Description | Number of days animal of type LT is alive in the farm in the year y |
| Source of data | Project proponents |
| Value(s) applied | 180days which sourced from FSR, and this value is in line with the number of days for pigs to be slaughtered by existing large-scale breeding groups in China. |
| Measurement methods and procedures | The days of swine alive in the farm will be recorded manually by the responsible staff. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals. This electronic ear tag will be connected to the Data Collection System (DCS), which can store and read information. Therefore, the days of swine alive in the farm can be traced through the electronic ear tag by the technical staff in each farm and obtained by the DCS. |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | Number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Project proponents |
| Value(s) applied | 226,952 heads of breeding swine are used to ex-estimated. And this value is sourced from FSR, which is the design scale of swine farms and it is also the maximum population of swine in the swine farms. |
| Measurement methods and procedures | Monitored daily. Archive electronically during project plus 5 years |
| Monitoring frequency | daily |
| QA/QC procedures | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals. This electronic ear tag will be connected to the Data Collection System (DCS), which can store and read information. The technicians in farms monitor and record the number of breeding swine in “Breeding Swine stock record” through the auto tracking devices of electronic ear tag daily, of which new imported animals are included and dead and discharge animals are excluded. The annual average number of animals (NAA,LT ) is calculated as an average of the daily stock of market and breeding swine in the farms without considering dead animals and discarded animals. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | The project proponents monitor the population of breeding swine through the auto device, which is connected to the Data Collection System (DCS). Therefore, the data of NAA, LT can be obtained through DCS. |

| Data / Parameter | Wsite |
| --- | --- |
| Unit | kg |
| Description | Average animal weight of a defined livestock population at the project site |
| Source of data | Project proponents |
| Value(s) applied | 68.4kg for commercial pigs and 90.0kg for breeding pigs |
| Measurement methods and procedures | Measured by the weight measurer |
| Monitoring frequency | monthly |
| QA/QC procedures | This parameter is used in equation 4 for estimating VSLT,y using option 3, and in equation 2 (appendix 2) for estimating NEXLT,y when using IPCC 2006 default values. Sampling procedures can be used to estimate this variable, taking into account the following guidance: (a) To ensure representativeness, each defined livestock population should be classified into a minimum of three age categories; (b) For each defined livestock population, a minimum of one monthly sample per age category should be taken; (c) When estimating baseline emissions and emissions released during baseline scenario from land application of the treated manure in the leakage section, the lower bound of the 95% confidence interval obtained from the sampling measurements should be used; (d) When estimating project emissions and emissions released during project activity from land application of the treated manure in the leakage section, the upper bound of the 95% confidence interval obtained from the sampling measurements should be used. |
| Purpose of data | Used for estimating VSLT,y |
| Additional comment | Sampling procedures and method is described in Section B7.2 of PDD. |

| Data/parameter | ndy |
| --- | --- |
| Unit | number |
| Description | Number of days treatment plant was operational in year y. |
| Source of data | Project proponents |
| Value(s) applied | 365 days for ex ante estimation. The actual number of days treatment plant was operational used in the monitoring periods will be monitored by Project proponents |
| Measurement methods and procedures | The days of treatment plant in each farm was operational will be recorded by the responsible staff. |
| Monitoring frequency | Daily |
| QA/QC procedures | The daily readout of flowmeter installed at the outlet of the anaerobic digestion in each farm can be crosscheck that the treatment plant is operational. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | N/A |

| Data / Parameter | FAer |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of volatile solids directed to aerobic treatment |
| Source of data | Feasibility Study Report |
| Value(s) applied | 54% are used in the pre-calculation, which sourced from FSR. As this parameter is not monitored in the actual operation. So, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%. |
| Measurement methods and procedures | the maximum value is applied due to not monitored in operation period. |
| Monitoring frequency | Annually |
| QA/QC procedures | The value of this parameter in the emission reduction calculation is 100%, which is conservative. |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | vf |
| --- | --- |
| Unit | m3 |
| Description | Biogas flow |
| Source of data | Project proponents |
| Value(s) applied | 31,354,493 m3 of biogas can be generated by the project annually, which sourced from FSR. |
| Measurement methods and procedures | Measured by flow meters |
| Monitoring frequency | Continuously by flow meter and reported cumulatively on weekly basis |
| QA/QC procedures | The calibration of flow meters, including the frequency of calibration, should be done in accordance with national standards or requirements. |
| Purpose of data | To calculated CH4 generated in the anaerobic digester in the project situation QCH4,y |
| Additional comment | The biogas flow will be measured at four points, as shown in the figure. But if the project participants can demonstrate that leakage in distribution pipeline is zero, it need be measured at any three points. The biogas flow to electricity or heat equipment in a moment can be considered destroyed, by monitoring that the equipment was working at this time. For the proposed project, all the biogas produced will be used for electricity generation, therefore the biogas produced from the anaerobic digestion, the amount of biogas used for electricity generation and the amount of biogas flared will be monitored through the flow meters. |

| Data/parameter | ECPJ,j,y |
| --- | --- |
| Unit | MWh |
| Description | Quantity of electricity consumed by the proposed project in year y |
| Source of data | Direct measurement or calculated based on measurements from more than one electricity meters |
| Value(s) applied | 0 MWh for ex ante estimation. During the monitoring period, if the electricity consumption sourced from the gird company, then the value will be determined as per the electricity meters monitoring and cross-check with the grid statement. |
| Measurement methods and procedures | Use electricity meters installed at the electricity consumption sources. |
| Monitoring frequency | Continuous measurement and at least monthly recording |
| QA/QC procedures | The calibration of meters, including the frequency of calibration, should be done in accordance with national standards or requirements. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | TDLj,y |
| --- | --- |
| Unit | / |
| Description | Average technical transmission and distribution losses for providing electricity to source j in year y |
| Source of data | tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Value(s) applied | 20% |
| Measurement methods and procedures | Default value of 20% will be applied, according to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Monitoring frequency | This value will be renewed once the tool is updated |
| QA/QC procedures | Default value of 20% is applied, which is the maximum value as per Methodology tool 05 Version 03.0 and conservative |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | Vt,db |
| --- | --- |
| Unit | m³ dry gas/h |
| Description | Volumetric flow of the gaseous stream in time interval t on a dry basis |
| Source of data | Flowmeter |
| Value(s) applied | Biogas were estimated according to the amount of manure for Ex ante estimation. Total 31,354,493 m3 of biogas is expected to be generated by the project annually, which sourced from Feasibility Study Report. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in monthly operation report. The daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Periodic calibration against a primary device provided by an independent accredited laboratory is mandatory for all projects applying large scale methodology(ies). Calibration and frequency of calibration is according to manufacturer’s specifications |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Vi,t,db |
| --- | --- |
| Unit | m³ gas i/m³ dry gas |
| Description | Volumetric fraction of greenhouse gas i in a time interval t on a dry basis |
| Source of data | gas analyser |
| Value(s) applied | CH4 fraction of biogas is 60% according to FSR |
| Measurement methods and procedures | Measured continuously and record hourly by online gas analysers installed at the outlet of the anaerobic digesters and saved automatically in the DCS (Data Collection System). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in monthly operation report. Daily and monthly CH4 concentration was calculated based on the 24-hour average and daily average CH4 concentration. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Calibration should include zero verification with an inert gas (e.g. N2) and at least one reading verification with a standard gas (single calibration gas or mixture calibration gas). All calibration gases must have a certificate provided by the manufacturer and must be under their validity period. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Tt |
| --- | --- |
| Unit | K |
| Description | Temperature of the gaseous stream in time interval t |
| Source of data | Measured by instrument |
| Value(s) applied | 293.15K was used in the pre-calculation for the density of CH4 as the applied methodology ACM0010 (Version 08.0), in actual monitoring period, the temperature of the gaseous stream will be monitored by the recordable electronic signal. The temperature Tt(K) is calculated as the equation T(K)=t(°C) +273.15 |
| Measurement methods and procedures | Measured continuously by the biogas flow meters, since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The readout of flowmeter was indicated degrees Celsius (°C). Therefore, the temperature Tt(K) is calculated as the equation T(K)=t(°C) +273.15 The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in monthly operation report. Daily and monthly temperature was calculated based on the 24-hour average and daily average temperature. |
| Monitoring frequency | Continuous unless differently specified in the underlying methodology |
| QA/QC procedures | Periodic calibration against a primary device provided by an independent accredited laboratory is mandatory. Calibration and frequency of calibration is according to manufacturer’s specifications |
| Purpose of data | Calculation of project emissions |
| Additional comment | Provided all parameters are converted to normal conditions during the monitoring process, this parameter may not be needed except for moisture content determination and therefore it should be metered only when performing such measurements (with same frequency) |

| Data/parameter | Pt |
| --- | --- |
| Unit | Pa |
| Description | Pressure of the gaseous stream in time interval t |
| Source of data | Measured by instrument |
| Value(s) applied | 101,325 Pa for the pre-calculation for the density of CH4 as the applied methodology ACM0010 (Version 08.0), in actual monitoring period, the pressure of the gaseous stream will be monitored by the recordable electronic signal. |
| Measurement methods and procedures | Measured continuously by the biogas flow meters since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in monthly operation report. Daily and monthly pressure was calculated based on the 24-hour average and daily average pressure. |
| Monitoring frequency | Continuous unless differently specified in the underlying methodology |
| QA/QC procedures | Periodic calibration against a primary device provided by an independent accredited laboratory is mandatory. Calibration and frequency of calibration is according to manufacturer’s specifications |
| Purpose of data | Calculation of project emissions |
| Additional comment | Provided all parameters are converted to normal conditions during the monitoring process, this parameter may not be needed except for moisture content determination and therefore it should be metered only when performing such measurements (with same frequency) |

| Data/parameter | ρi,t |
| --- | --- |
| Unit | kg gas i/m³ gas i |
| Description | Density of greenhouse gas i in the gaseous stream in time interval t |
| Source of data | Calculated |
| Value(s) applied | 0.67 kg/m3 for ex-estimated. In monitoring period, this parameter will be calculated based on temperature of the gaseous stream in time interval t and pressure of the gaseous stream in time interval t. |
| Measurement methods and procedures | Calculated based on temperature of the gaseous stream in time interval t and pressure of the gaseous stream in time interval t. |
| Monitoring frequency | Pressure and temperature are measured continuously by the biogas flow meters |
| QA/QC procedures | Calculated value according to the pressure and temperature |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | MS%j |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of manure handled in system j in project activity |
| Source of data | Project proponents |
| Value(s) applied | 100% and 54% for ex-estimated since the treatment process of this project is an anaerobic-aerobic combined treatment technology, main including anaerobic fermentation and aerobic composting. The value is sourced from Feasibility Study Report. As this parameter is not monitored in the actual operation. so, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%. |
| Measurement methods and procedures | All the manure flew into AWMSs to be treated; no any manure will be discharged outside. Therefore, the value of this parameter is 100%. |
| Monitoring frequency | Annually |
| QA/QC procedures | 100% is the maximum value and conservative. |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | B0, LT |
| --- | --- |
| Unit | m3 CH4/kg -dm |
| Description | Maximum methane producing potential of the volatile solid generated by animal type LT |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | B0, LT (Market swine) =0.29 B0, LT (Breeding swine) =0.29 |
| Measurement methods and procedures | B0, LT can be measured as per ISO 11734:1995. As this parameter is not monitored in the actual operation. so, in the monitoring period 0.29 m3CH4/kg -dm is still applied. While the actual methane producing potential of the volatile solid generated by swine manure is 479.4ml/g VS according to the public literature,which is higher. Therefore 0.29 m3CH4/kg -dm applied in monitoring period is conservative. |
| Monitoring frequency | Annually |
| QA/QC procedures | The value is taken from published sources. The parameter value should be updated on latest available public data source. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | NEXLT,y |
| --- | --- |
| Unit | kg N/animal/year |
| Description | Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2 |
| Source of data | Calculated as the equation of option 2 in Appendix 2: While, NEXIPCC default calculated as equation 30 of the IPCC 2006, volume 4, chapter 10 NEXIPCC default =Nrate(T)* TAM/1,000*365 |
| Value(s) applied | NEXLT,y(Market swine) =10.49 NEXLT,y (Breeding swine) =7.88 |
| Measurement methods and procedures | Nrate(T) and TAM are default value from IPCC 2006 The measurement methods and procedures for Wsite, please refer to the monitoring parameter Wsite |
| Monitoring frequency | Annually |
| QA/QC procedures | The parameter is calculated as the equation of option 2 in Appendix 2 of methodology ACM0010 Version 08 and the value of other parameter applied in the equation is taken from published sources (2006 IPCC). The value should be updated on latest available public data source. QA/QC procedure for Wsite, please refer to the monitoring parameter Wsite |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

| Data/parameter | Type |
| --- | --- |
| Unit | / |
| Description | Type of barn and AWMS |
| Source of data | Project proponent |
| Value(s) applied | NA |
| Measurement methods and procedures | The swine barn and AWMS layout and configuration are collected. Archive electronically during project plus 5 years. |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | After the first verification, only changes in the type of barn and AWMS will be reported. |
| Purpose of data | Confirm whether the implementation of project as design |
| Additional comment | Swine barn and AWMS layout and configuration. |

| Data/parameter | T |
| --- | --- |
| Unit | ℃ |
| Description | Annual average ambient temperature at project site |
| Source of data | Official publicly available information |
| Value(s) applied | 16 °C for ex-ante estimated. |
| Measurement methods and procedures | The data published by Official public information can prove that the annual average ambient temperature at project site. |
| Monitoring frequency | Monthly |
| QA/QC procedures | The actual temperature during the monitoring period will be compared with the value of ex-ante estimated, i.e., 16 °C In the monitoring period, to be conservative, corresponding MCFj with lower annual average ambient temperature as per Appendix 3 of ACM0010(Version08.0) will be used for calculation after comparing the actual value and the value of ex-ante estimated. |
| Purpose of data | Used to select the annual MCFj from IPCC 2006 Guidelines |
| Additional comment | NA |

SDG 8

| Data / Parameter | Total number of jobs |
| --- | --- |
| Unit | Number |
| Description | Jobs created for both men and women. |
| Source of data | Record keeping book |
| Value(s) applied | Will be reported for each monitoring period. For ex ante estimation, 18 full-time jobs are created including 9 males and 9 females during the operation period of the project. |
| Measurement methods and procedures | The number of full-time jobs created for males and females will be recorded. Source of data is record keeping book and it will be cross checked by the labor contracts. |
| Monitoring frequency | Once for each monitoring period. |
| QA/QC procedures | After the first verification, only changes in employees will be reported. The results will also be cross checked with labor contract. |
| Purpose of data | To demonstrate contribution to SDG 8. |
| Additional comment | N/A |

SDG 7

| Data / Parameter | EGd,y |
| --- | --- |
| Unit | MWh |
| Description | Total electricity produced |
| Source of data | Direct measurement by the electricity meters |
| Value(s) applied | For ex estimated 40,134 MWh of electricity can be generated by the project annually. |
| Measurement methods and procedures | Monitored by electricity meters installed at the generator outlet in each farm. Archive electronically during project plus five years. |
| Monitoring frequency | Continuous measurement and at least monthly recording |
| QA/QC procedures | The calibration of electricity meters, including the frequency of calibration, should be done in accordance with national standards or requirements. |
| Purpose of data | To demonstrate contribution to SDG 7. |
| Additional comment | N/A |

Mitigation Measure for Safeguarding Principles

| Data / Parameter | Employee Training of biogas safety operation |
| --- | --- |
| Unit | / |
| Description | Employees will be trained on the safety operation of the biogas |
| Source of data | Training Notice |
| Value(s) applied | / |
| Measurement methods and procedures | The Employee Training of biogas safety operation will be determined by the training notice. |
| Monitoring frequency | / |
| QA/QC procedures | Meeting attendance record will be cross-checked. |
| Purpose of data | To demonstrate Mitigation Measure for Safeguarding Principles |
| Additional comment | N/A |

B.7.2 Sampling plan

>>

The sampling objective: determining the Average animal weight of a defined livestock population at the project site during the crediting period with a 95/10 confidence/precision.

According to “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, the sampling plan is as follows:

| Parameter | Wsite |
| --- | --- |
| Objectives and reliability requirements | Determining the Average animal weight of a defined livestock population at the project site during the crediting period. According to the “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, PP shall use 95/10 confidence/precision as the criteria for the reliability of sampling efforts for large-scale project. According to the methodology” ACM0010” GHG emission reductions from manure management systems (Version 08.0)”, each defined livestock population should be classified into a minimum of three age categories; The three age categories of marketing swine are classified according to the age in days, i.e. Nursery phase with 30-60days, Growing phase with 60-130days and Mature phase with 130-180days. The three age categories of breeding swine are classified according to the age in days, i.e. Nursery phase with 30-70days, Growing phase with 70-220days and Mature phase with 220-310days. For each defined livestock population, a minimum of one monthly sample per age category should be taken. In this project, the monitoring activities of Wsite will be conducted in the three age groups of Nursery phase, growing phase and mature phase in each swine farm at least one monthly. |
| Target population and sampling frame | For the ex-calculation, A total of 130,976 market swine and 226,952 breeding swine included in this project and the data of the average animal weight of a defined livestock population at the project site is from Feasibility Study Report. As the applied value of 130,976 market swine and 226,952 breeding swine is the design scale, which is the maximum population of swine in the swine farms. And the actual scale of the farm will not exceed the design value during the operation period of swine farms, therefore the sample size is calculated according to the design scale and fixed during the monitoring period, it is also conservative. |
| Sampling method | There are 9 subsidiary farms involved in the project, all the swine farms including market swine and breeding swine. As this project involved 9 swine farms and two types of swine i.e., Market swine and Breeding swine, and as per applied methodology, each defined livestock population should be classified into a minimum of three age categories, so the sampling method is Stratified random sampling. The specific sampling methods are as follows: For the sampling, PP should calculate the overall sample size based on the design scale of each swine population of pigs in stock firstly. As the project involved 9 swine farms, so the sample size in each swine farm should be determined based on the proportion of the number of each farm in the total number of 9 farms. Similarly, the sample size of each age group of Market swine and Breeding swine in a farm were also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm. After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample can be conducted in every swine farm. Since swine in different age are kept in the different pig houses, samples can be randomly selected from pig houses of this age group. After the samples are selected, the weight is measured by weight measurers and recorded in the weight record table. |

The sampling size:

According to the standard of the “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, PP shall use 95/10 confidence/precision as the criteria for the reliability of sampling efforts for large-scale project. The sampling size will be calculated as the Appendix 6 of the guideline of the “Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”.

According to statistical principles and sampling survey method, the sample size should be determined as follows:

n1=Z2σ2d2=Z2σ2/X2d2/X2=Z2V2/e2

n2= n1N/(N+n1)

n3=Bn2

n4= n3/r

n= n4 (110%)

Where:

Z     the Z-statistic and the value taken is 1.96 (corresponding to 95% confidence level);

σ      the standard deviation;

d      the maximum error of estimate;

X      the mean value of samples;

V      the coefficient of variation, V= σ/X≤1 and its maximum value 1 is taken;

e      the allowed relative sampling error (“precision”) and the value taken is 10%;

N     the total number of involved swine 357,928;

B     the survey design effect and the value taken is 1 because for stratification sampling the B value is less than or equal to 1;

r       the survey replies rate and the value taken is 100%;

110%   10% contingency was added to produce the final sample size;

n1, n2, n3, n4, n      the adjusted values after each step with n being the final number of samples to be taken. Actually, n4 is the required number of samples based on calculation, 10% contingency was added to n4 to produce n in this sampling.

For pre-calculation, since the total number of involved swine is 357,928, N in the calculation of sample size is 357,928, and the calculation result is as follows:

| Z | V | e | N | B | r |
| --- | --- | --- | --- | --- | --- |
| 1.96 | 1 | 10% | 357,928 | 1 | 100% |
| n1 | Z2V2/e2 | 384 |  |  |  |
| n2 | n1N/(N+n1) | 384 |  |  |  |
| n3 | Bn2 | 384 |  |  |  |
| n4 | n3/r | 384 |  |  |  |
| n | n4 (110%) | 423 |  |  |  |

The calculation result of n is 423. Therefore, 423 samples should be sufficient to satisfy the desired confidence and precision. 423 samples will be selected from the sampling frame according to the stratified sampling method every month.

As the applied value of 130,976 market swine and 226,952 Breeding swine is the design scale, during the monitoring periods, the actual breeding scale of the farm will not exceed the design value, so the sample size calculated from the design values can be used in monitoring period and it is conservative.

Stratified sampling (with samples of 423) was used for the monitoring of the Wsite. The specific sampling methods are as follows: For the sampling, PP should calculate the overall sample size based on the population of pigs in stock firstly. As the project involved 9 swine farms, so the sample size in each swine farm should be determined based on the proportion of the number of each farm in the total number of 9 swine farms. Similarly, the sample size of each age group of Market swine and Breeding swine in a farm were also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm.

The distribution of the samples in each age category in each farm shows as follow:

| NO. | Swine farm | Sample number | Marketing swine |  |  | Breeding swine |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Nursery phase | Growing phase | Mature phase | Nursery phase | Growing phase | Mature phase |
| 1 | Yongjiahe Swine farm | 109 | 13 | 13 | 14 | 16 | 24 | 29 |
| 2 | Zhoujiachong Swine farm | 42 | 5 | 6 | 5 | 6 | 10 | 10 |
| 3 | Jiucaiyuan Swine farm | 36 | 4 | 5 | 5 | 5 | 8 | 9 |
| 4 | Gaoshanmiao Swine farm | 27 | 3 | 3 | 4 | 4 | 6 | 7 |
| 5 | Shayang 1st phase Swine farm | 17 | 2 | 2 | 2 | 3 | 4 | 4 |
| 6 | Shayang 2nd phase Swine farm | 24 | 3 | 3 | 3 | 4 | 5 | 6 |
| 7 | Ezhou Swine farm | 16 | 2 | 2 | 2 | 3 | 3 | 4 |
| 8 | Tuchong Swine farm | 92 | 10 | 13 | 11 | 14 | 22 | 22 |
| 9 | Sangzihu Swine farm | 90 | 10 | 12 | 12 | 14 | 19 | 23 |
| Total |  | 453 | 52 | 59 | 58 | 69 | 101 | 114 |

Implementation

The Sampling process will start as soon as the target population is determined and will be finished before the beginning of next monitoring periods. The Sampling process will be determined by the GS monitoring team.

The one monthly monitoring activity of the samples will be completed in the 9 swine farms during each monitoring periods.

The monitoring data will be collected and recorded throughout the entire crediting period. All archived data and documentation will be kept for at least 2 years after the end of the last crediting period..

Procedures for Administering Data Collection and Minimizing Non-sampling Errors

The monitoring forms will be filled out daily by the Breeders in the 9 swine farms to record the animal weight of the samples. Then average animal weight of a defined livestock population at the project site was calculated based on these data by the monitoring team member. The data will be reviewed by the project developer and VVB.

If the recorded raw data on the monitoring form are reasonable and basically consistent with the actual growth state, the raw data will be archived.

If the data record is missing or damaged or the target animal was dead during the monitoring periods, the following makeup process will be conducted:

1) The general principle is that zero value will be used for the missing or damaged data. This is most conservative approach. The monitoring personnel will be trained before the starting of the project operation to ensure that each team member is fully aware of and able to strictly follow this conservative principle. During the monitoring process, the monitoring personnel will be required to strictly abide by the above conservative principle in data recording, i.e., use zero values for all the missing or damaged data.

2) If this is due to the working error of the monitoring personnel, further train the person until he or she can perform the job properly. And in the meantime, use zero value for the missing or damaged data;

3) If this is due to the inability or attitude of a particular worker in monitoring team, dismiss such worker and re-hire those with proper ability and attitude. And in the meantime, use zero value for the missing or damaged data;

4) If some data recorded are significantly higher than the normal range and inconsistent with normal growth, the monitoring personnel should ask for the reason. If the measurement is high due to the damage of weighing scales or other measurement equipment, zero value will be used for that day's data. And need to calibrate and maintain the weighing scale or replace the measuring equipment immediately and avoid this situation in the future.

If the monitoring results are satisfactory in terms of correct reporting, data completeness and correct analysis, the data will be accepted for the monitoring report.

QA/QC Procedures

Before implementing the project, the project owner will train the personnel of monitoring teams on how to properly conduct the monitoring process.

If the data reported by the team member significantly deviates from the normal range, the monitoring personnel should write down the reasons and report to the team leader, any action is forbidden before the permission. The monitoring team will arrange a research according to the attached form. At the same time, when the verification group has any doubt with the right result, they can arrange a related research.

The project owner should enters all the measured data into the data sheet each day, using Excel to calculate the weighted mean value of Average animal weight of a defined livestock population at the project site from all the sample, compared with the everyday normal growth state.

B.7.3 Other elements of monitoring plan

>>

1.Monitoring framework

The project owner will be responsible for the whole monitoring work. The GS Monitoring Team will be established to collect and record monitoring data within the project boundary. The GS monitoring team will be responsible for the normal operation of the manure treatment system and the collection and record of all the monitoring data. All the data will be reviewed by the project developer and VVB. Each member of the GS monitoring team will be trained by the project owner at least once a year. The overall monitoring system structure of the project shows as below:

Figure 5 The Organization Structure of the Monitoring Team

2.Principle of Monitoring

Every swine farm should monitor all the data mentioned description in section B7.1 in this PDD.

The installation of relevant monitoring instruments and meters shall be carried out in accordance with industry requirements and manufacturer specifications and shall be calibrated regularly as required.

If point of power measurement is changed or added during the crediting period, this should be documented transparently in the monitoring reports, and the procedure for post registration changes shall be followed.

3. Monitoring equipment and installation

Installation and configuration of meters are shown as Figure 7. In order to ensure measurements with a low degree of uncertainty, the data metering equipment will be calibrated and checked by an appropriately qualified third party according to an appropriate national standard. The calibration records will be appropriately maintained and made available for review by VVB.

1. Weight measurer; 2. Flow Meter 1; 3. Gas analyzer; 4. Flow Meter 2; 5. Flow Meter 3; 6. Electricity Meter 2; 7. Electricity Meter 1

Figure 6. Installation and Configuration of Monitoring equipment

4.Parameters to be monitored

The monitoring requirements for this methodology include the monitoring of parameters for both baseline and project emissions calculations. All provisions in the methodology and relevant tools shall apply, as described for each parameter in section B.7.1 of this PDD.

For Mitigation Measure for Safeguarding Principles, Employee Training of biogas safety operation will be monitored through the training record.

For the parameters of SDG 8, total number of full-time jobs created by the project will be determined based on record keeping book and cross-checked by labor contract..

For the parameters of SDG 7, the amount of electricity generation by capturing biogas (EGd,y) annually will be monitored by electricity meters installed at the outlet of biogas generator in each farm.

For the parameter of SDG13, the parameters that need to be monitored as the description of section B7.1 are as follows:

a) Number of animals of type LT produced annually for the year y, Np,LT;

b) Number of days animal of type LT is alive in the farm in the year y, Nda,LT;

c) Daily stock of animals in the farm, discounting dead and discarded animals, NAA,LT;

d) Average animal weight of a defined livestock population at the project site, Wsite;

e) Number of days treatment plant was operational in year y, ndy;

f) Quantity of electricity consumed by the proposed project in year y, ECPJ,j,y;

g) Average technical transmission and distribution losses for providing electricity to source j in year y, TDLj,y;

h)  Biogas flow produced by the anaerobic digestion, Vf;

i) Volumetric flow of the gaseous stream in time interval t on a dry basis, Vt,db;

j) Volumetric fraction of greenhouse gas I in a time interval t on a dry basis, m³ gas i/m³ dry gas, Vi,t,db.

k) Temperature of the gaseous stream in time interval t, Tt;

l) Pressure of the gaseous stream in time interval t, Pt;

m) Density of greenhouse gas i in the gaseous stream in time interval t, ρi,t；

n) Fraction of volatile solids directed to aerobic treatment, FAer；

o) Fraction of manure handled in system j in project activity, MS%j.

p) Maximum methane producing potential of the volatile solid generated by animal type LT, B0,LT;

q) Type of barn and AWMS, Type;

r) Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2, NEXLT,y

s) Annual average ambient temperature at project site, T.

For the number of animals of type LT produced annually for the year y and the daily stock of animals in the farm, discounting dead and discarded animals can be sourced from the Export record form of Market swine and the Breeding Swine stock record.

For number of days animal of type LT is alive in the farm in the year y is sourced from the Export record form of Market swine and cross-checked by sales record, the data is usually 180 days.

The monitoring activities of the average animal weight of a defined livestock population at the project site will be conducted by sampling in the three age groups of market swine and breeding swine respectively at least one monthly. The monthly data is recorded by the PP and then average animal weight of a defined livestock population at the project site was calculated based on these data..

The number of days treatment plant was operational is determined according to actual operating days of the manure treatment system during the monitoring period. The data is sourced from the operational recorded by the responsible staff and cross-checked by the production record from the DCS system.

The amount of biogas flow produced by the anaerobic digestion; the amount of biogas utilized and the amount of biogas flared will be monitored by flows and the data can be sourced from the production record from DCS system.

The quantity of electricity consumed from the grid company by the proposed project is sourced from the electricity meters monitoring and Cross-check with the grid statement issued by the power grid company.

Average technical transmission and distribution losses for providing electricity to source j is sourced form the tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” and will be updated once the tool is updated.

Volumetric flow of the gaseous stream in time interval t on a dry basis and the volumetric fraction of greenhouse gas i in a time interval t on a dry basis can be monitored by the flowmeter and gas analyser, the data are sourced from the the production record from DCS system.

The temperature and pressure of the gaseous stream in time interval t is measured by the instruments with recordable electronic signal, the monitoring is continuous, and the change is very small. so, the data can be obtained from the production record from DCS system. And the Density of greenhouse gas i in the gaseous stream in time interval t should be calculated based on the temperature and pressure of the gaseous stream in time interval t..

For Fraction of volatile solids directed to aerobic treatment, FAer and Fraction of manure handled in system j in project activity, MS%j, since there is no monitoring equipment to monitor in the actual operation. So, in the monitoring period, to be conservative, the value of the two parameters in the emission reduction calculation is 100%.

For B0,LT, a default value from IPCC 2006 is applied, therefore it is not monitored during the operation period.

For Type of barn and AWMSs, the swine barn and AWMSs layout and configuration are collected, which can be prove the Type of barn and AWMS, and only changes in the type of barn and AWMSs will be reported after the first verification.

For NEXLT,y, the parameter is calculated as the equation of option 2 in Appendix 2 of methodology ACM0010 Version 08.0 and the value of other parameter applied in the equation is taken from IPCC 2006, which is a published source. The parameter value will be updated on latest available public data source. While for parameter Wsite, it is monitored during this MR, please refer to monitoring parameter Wsite

For annual average ambient temperature at project site, the monthly data can be sourced from Publicly available data.

5. Data collection and management, QA/QC and Emergency procedure

Data collection and management

The monitoring activities will be conducted by GS monitoring team. Then the survey result should be summarized by monitoring team member. The data will be reviewed by the project developer and VVB.

If the recorded raw data on the summary table of each parameter are reasonable and basically consistent with the actual state, the raw data will be archived. In addition, the original record file also should be preservation.

If the data record is missing or damaged during the monitoring periods, the following makeup process will be conducted:

1) The general principle is that conservative values will be used for the missing or damaged data. This is most conservative approach. The monitoring personnel will be trained before the starting of the project operation to ensure that each team member is fully aware of and able to strictly follow this conservative principle. During the monitoring process, the monitoring personnel will be required to strictly abide by the above conservative principle in data recording, i.e., use conservative values for all the missing or damaged data.

2) If this is due to the working error of the monitoring personnel, further train the person until he or she can perform the job properly. And in the meantime, use conservative value for the missing or damaged data.

3) If this is due to the inability or attitude of a particular worker in monitoring team, dismiss such worker and re-hire those with proper ability and attitude. And in the meantime, use conservative value for the missing or damaged data.

4) If some data recorded are significantly higher or lower than the normal range and inconsistent with actual situation, the monitoring personnel should ask for the reason. If the measurement is high or low due to the damage of measurement equipment, conservative value will be used. And need to calibrate or replace the measuring equipment immediately and avoid this situation in the future.

The PP shall perform statistical analysis on the data collated using Microsoft excel in an automated tool that will use standard deviation and mean values to determine outliers. If the monitoring results are satisfactory in terms of correct reporting, data completeness and correct analysis, the data will be accepted for the monitoring report.

QA/QC Procedures

Before implementing the project, the project owner will train the personnel of monitoring teams on how to properly conduct the monitoring process. The project proponent commits itself to accurate and transparent record keeping, monitoring and evaluation. All supporting documentation and records for the project must be easily accessible for spot checking and cross referencing by a third party; these records will be kept by PP. An auditor must also be able to cross reference pertinent project documentation, including but not limited to on-site operation diary, weight record sheet, sales record, etc.

Emergency procedure

Employees need to be familiar with the process flow and emergency situations that may occur during project operation and operate in strict accordance with the technical specifications of each process treatment equipment and monitoring equipment. During the operation of the project, all equipment and related instruments shall be checked regularly according to the manufacturer’s recommendations. If necessary, calibration and/or proper repair have to be carried out immediately.

In case of malfunction and/or damage of any system or piece of the process, it may cause damage to the operation of the system, especially serious damage to the biogas generation and combustion system it must be dealt with as soon as possible and reported to the responsible technician immediately.

SECTION C. DURATION AND CREDITING PERIOD

C.1. Duration of project

C.1.1 Start date of project

>>09/09/2020, the date on singing the biogas power generator purchase contract.

C.1.2 Expected operational lifetime of project

>>15 years

C.2. Crediting period of project

C.2.1 Start date of crediting period

>>01/01/2021 or two years prior to the date of Project Design Certification, whichever is later.

C.2.2 Total length of crediting period

>>15 years. As per section 4.1.5 of GS4GG Community Services Activity Requirements (Version 1.2), the crediting period is 5 years for each Design Certification Renewal Cycle and in total two Design Certification Renewal Cycles.

SECTION D. SUMMARY OF SAFEGUARDING PRINCIPLES AND GENDER SENSITIVE ASSESSMENT

D.1 Safeguarding Principles that will be monitored

A completed Safeguarding Principles Assessment is in Appendix 1, ongoing monitoring is summarised below.

| Principles | Mitigation Measures added to the Monitoring Plan |
| --- | --- |
| Principle 1 Human Rights | Not required |
| Principle 2 Gender Equality and Women’s Rights | Not required |
| Principle 3 Community Health, Safety and Working Conditions | Not required |
| Principle 4 Cultural Heritage, Indigenous Peoples, Displacement and Resettlement | Not required |
| Principle 5 Corruption | Not required |
| Principle 6 Economic Impacts | Not required |
| Principle 7.1 Emissions | Not required |
| Principle 7.2 Energy supply | Not required |
| Principle 8 Water | Not required |
| Principle 9 Environment, Ecology and Land Use | Required |

D.2. Assessment that project complies with GS4GG Gender Sensitive requirements

| Question 1 - Explain how the project reflects the key issues and requirements of Gender Sensitive design and implementation as outlined in the Gender Policy? | The project reflects the key gender issues and requirements of Gender Sensitive design and implementation. To ensure gender sensitive design and implementation, the project shall address any gender inequalities and gender-related risks that may be identified in project gender analyses. Gender-sensitive approaches in stakeholder consultation will also be ensured by promoting information sharing equitably with women and men stakeholders, which will be made available and presented in an accessible format across all stakeholder groups including those more marginalized. The approach will also include opportunities for stakeholders to share information in a two -way exchange, give regular feedback during implementation and ensure their views and priorities are incorporated in project design and implementation. |
| --- | --- |
| Question 2 - Explain how the project aligns with existing country policies, strategies and best practices | The project does not involve and is not complicit in any form of discrimination based on gender, race, religion, sexual orientation or any other basis. The project respects all the rights to the women conferred to them by the Constitution of the People's Republic of China, the Labour Law of the People's Republic of China, Special provisions on labour protection of female employees and other related regulation and policies. All these articles embrace the gender equality and social inclusion principles in a way or other. The project will create full-time jobs for local women and ensure equal pay for work of equal value regardless of gender, which align with the spirit of ensure gender equality and protect women's rights. According to the labor contract signed with employees, 9 men and 9 women are employed. |
| Question 3 - Is an Expert required for the Gender Safeguarding Principles & Requirements? | No gender experts will be required for the Gender Safeguarding Principles & Requirements. The project shall neither contributes to discrimination against women nor shall it reinforce gender-based discrimination and/or inequalities. Gender equality will be attained by ensuring that both men and women will have equal opportunities to full-time jobs created and trainings. The project will further ensure equal pay for equal value of work. |
| Question 4 - Is an Expert required to assist with Gender issues at the Stakeholder Consultation? | No gender experts will be required to engage in the Stakeholder Consultation. The physical meeting will follow the GS4GG Stakeholder Consultation and Engagement Requirements (Version 1.2), which includes gender guidelines, and minimum group of stakeholders to be consulted. All assessment questions related to safeguarding principles, including principle 2 “Gender Equality and Women’s Rights”, have been discussed during the stakeholder consultation meeting. Refer to Appendix 1 of the PDD for more details of the results for Safeguarding Principal Assessment, which was held on 18/10/2021. Refer to Appendix 1 of the PDD for more details of the results for Safeguarding Principal Assessment. |

SECTION E. SUMMARY OF LOCAL STAKEHOLDER CONSULTATION

The below is a summary of the 2 step GS4GG Consultation for monitoring purposes. Please refer to the separate Stakeholder Consultation Report for a complete report on the initial consultation and stakeholder feedback round.

E.1 Summary of stakeholder mitigation measures

>> The proposed project is a retroactive project, and the stakeholder consultation physical meeting was carried out on 18/10/2021.

Local stakeholder consultation of the project was organized in line with the GS4GG stakeholder Procedure Requirements and Guidelines.

The project owner invited the stakeholders to participate in the stakeholder consultation meeting on the project through email and public notice. In order to facilitate the invitees to understand the purpose of the meeting and relevant information of the project, the project owner briefly introduced the key information of the project during the invitation. The project owner invited 20 people to participate the physical meeting and tried to invite the same number of males and females.

20 people, including local residents and experts from local government attended in the 1st round of Local stakeholder consultation physical meeting held on 18/10/2021 in the 4th floor of Zhengbang Group, Ziwen Road, Zengdian Town, Yunmeng County, Xiaogan City, Hubei Province. In the meeting, the project owner introduced the project activity and its social and environmental impacts in detail, published input & grievance mechanism, the self-assessment of sustainable Development Goals (SDGs), discussed and confirmed the ownership of GS emission reduction with stakeholders.

The comments received during the stakeholder consultation meeting can summarized below:

Comment: Will the project have any adverse impact on the surrounding environment after it is put into operation?

This aspect was considered at the beginning of the construction of the project, the project has no adverse impact on the environment, and has an EIA approval.

Comment: What are the current international mechanisms for certifying emission reductions?

Currently there are three main mechanisms: CDM, VCS and GS.

Comment: How long is the first crediting period for GS projects generally?

Five years.

The 2nd round of Stakeholder Consultation, the Stakeholder feedback round is held for two months between 10/09/2021 and 09/11/2021. All stakeholders that participated in the Stakeholder Consultation physical meeting were invited to the Stakeholder Feedback Round, including those who were invited but unable to attend. The electronic version and paper copy Project Information Note and Draft PDD are sent to the stakeholders for review and feedback. People can give feedback by replying the invitation email, etc., and all the feedback were recorded.

During the 2nd round of Stakeholder Consultation, the project owner invited and consulted relevant experts, including the staff of relevant institutions and organizations with professional knowledge related to project information and safeguarding requirements, to ensure that the implementation of the project meets the requirements of GS4GG Requirements, and will not affect the rights of local residents or local ecology and hydrology while bringing benefits to local residents. On the aspects of human rights, gender equality and women's rights, community health, safety and working conditions, cultural heritage, indigenous people, displacement and reset, corruption, economic impacts, climate and energy, water, and environment, ecology and land use, the project owner consulted the experts of the Department of Ecology and Environment  of Hubei Province, Department of Agriculture and Rural Affairs of Hubei Province, Hubei Provincial Development and Reform Commission, Ministry of Ecology and Environment of the People's Republic of China, and Women's Federation of Hubei Province mainly through email. These experts affirmed the practice of creating full-time jobs for the local area and encouraging the recruitment of women in the project, and also affirmed the implementation of the project will be no negative impact in these aspects, and there are no comments on project design and monitoring methods. In addition to experts’ responses to safeguarding questions, no other stakeholder feedback was received during the Stakeholder Feedback Round.

E.2 Final continuous input / grievance mechanism

| Method | Include all details of Chosen Method (s) so that they may be understood and, where relevant, used by readers. |
| --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | A grievance book was put in at the entrance of each swine farm. All stakeholders are allowed to record their grievances or comments in the book. The project owner will assign specific person to deal with related issues. When assessing the relevance and appropriateness of the stakeholders’ comments, the project owner will apply a gender lens. |
| GS Contact (mandatory) | help@goldstandard.org |
| Other | Feedback can be made directly through visiting Jiangxi Zhengbang Breeding Co. Ltd. (north of Hu Dong 4th Road and east of Chanye Road) or through e-mail 57566640@qq.com or call 135 6445 9487. |
| Nominated Independent Mediator (optional) | All stakeholders can also express their feedback or comment through sending email to the nominated independent mediator (hbnyncj_1996@sina.com) i.e. Department of Agriculture and Rural Affairs of Hubei Province. |

Appendix 1 - Safeguarding Principles Assessment

Complete the Assessment below and copy all Mitigation Measures for each Principle into SECTION D above. Please refer to the instructions in the Guide to Completing this Form.

| Assessment Questions/ Requirements | Justification of Relevance (Yes/potentially/no) | How Project will achieve Requirements through design, management or risk mitigation. | Mitigation Measures added to the Monitoring Plan (if required) |
| --- | --- | --- | --- |
| Principle 1. Human Rights |  |  |  |
| - The Project Developer and the Project shall respect internationally proclaimed human rights and shall not be complicit in violence or human rights abuses of any kind as defined in the Universal Declaration of Human Rights - The Project shall not discriminate with regards to participation and inclusion | No | 1.The project activity is designed to introduce new animal waste management systems to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. The project is in Hubei Province, china. The project is implemented under the laws of China and will not lead to violations or human rights abuses in any kind. 2. All farmers near the project region that respect the principles and values of sustainable development can equally participate and benefit from the project. The project does not discriminate on gender, race, religion, sexual orientation or any other aspect. | Not required |
| Principle 2. Gender Equality |  |  |  |
| - The Project shall not directly or indirectly lead to/contribute to adverse impacts on gender equality and/or the situation of women - Projects shall apply the principles of nondiscrimination, equal treatment, and equal pay for equal work - The Project shall refer to the country’s national gender strategy or equivalent national commitment to aid in assessing gender risks - (where required) Summary of opinions and recommendations of an Expert Stakeholder(s) | No | 1.The project will not directly or indirectly reinforce gender-based discrimination and shall not lead to/contribute to adverse impacts on gender equality and/or the situation of women. In fact, the project increases women’s access to or control of resources, entitlements and benefits by providing easy access to equal job opportunities. 2.The project will not set up any barriers to the employment of women. All employees have benefits based on pregnancy, maternity/paternity leave, or marital status according to the the Labour Law of the People's Republic of China, Special provisions on labour protection of female employees and other related regulation and policies. 3. The project complies with the Labor Law and China's gender related policies. 4. There is no opinions and recommendations of an Expert Stakeholder(s). That’s because the project will not directly or indirectly reinforce gender-based discrimination and shall not lead to/contribute to adverse impacts. Therefore, the safeguarding principle related to Gender Equality and Women’s Rights is not triggered during the project design and implementation. | Not required |
| Principle 3. Community Health, Safety and Working Conditions |  |  |  |
| - The Project shall avoid community exposure to increased health risks and shall not adversely affect the health of the workers and the community | No | The project activity is designed to introduce new animal waste management systems to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. The biogas generated during the treatment process will be captured for power generation. After anaerobic digestion, the fermented sludge will be treated in aerobic composting system, which will be used as fertilizer. The fertilizer can be distributed to the local people for free regularly. So, the Project doesn’t increase the exposure to health risks and adversely affect the health of the workers and the community. In fact, this project can bring benefits to the workers and the community. | Not required |
| Principle 4.1 Sites of Cultural and Historical Heritage |  |  |  |
| Does the Project Area include sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture? | No | The new animal waste management system is not used in sites, structures, or objects with historical, cultural, artistic, traditional or religious values or intangible forms of culture. The project does not utilise Cultural Heritage, including the knowledge, innovations, or practices of local communities, affected communities. | Not required |
| >> |  |  |  |
| Principle 4.2 Forced Eviction and Displacement |  |  |  |
| Does the Project require or cause the physical or economic relocation of peoples (temporary or permanent, full or partial)? | No | The free distribution of the fertilizer produced by this project can help to reduce the costs of purchasing fertilizer for the local people. All the investment for the animal waste management system is provided by project owner. It obviously does not cause physical or economic relocation of peoples. | Not required |
| >> |  |  |  |
| Principle 4.3 Land Tenure and Other Rights |  |  |  |
| - Does the Project require any change, or have any uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership? - For Projects involving land use tenure, are there any uncertainties with regards to land tenure, access rights, usage rights or land ownership? | No | This project is operated in the swine farm owned by the project owner.so, there is no uncertainties related to land tenure arrangements and/or access rights, usage rights or land ownership. The project does not require any change to land tenure arrangements and/or other rights such as resource access rights, community-based property rights and customary rights, which is confirmed by an expert invited by the project owner. | Not required |
| >> |  |  |  |
| Principle 4.4 - Indigenous people |  |  |  |
| Are indigenous peoples present in or within the area of influence of the Project and/or is the Project located on land/territory claimed by indigenous peoples? | No | Indigenous people have the same and equal opportunity to obtain the fertilizer and they will not be affected directly or indirectly in a negative way by the project. In addition, there are no indigenous people present within the area of influence nor the project is located on territory claimed by indigenous people. This is confirmed by an expert invited by the project owner. | Not required |
| >> |  |  |  |
| Principle 5. Corruption |  |  |  |
| - The Project shall not involve, be complicit in or inadvertently contribute to or reinforce corruption or corrupt Projects | No | The project is implemented on the ground by the social enterprise. The ethical codes of the project partners are against corruption. In addition, the companies comply with all related economic laws and regulations of China. This Project has a zero tolerance to corruption policy, anything generated as project revenue shall be spent towards the project monitoring, repair and maintenance, project operation and costs against project verification and issuance of the emission reduction credits. Hence, the project doesn’t involve any transaction of cash and/or kind between the project participant and the beneficiary. | Not required |
| Principle 6.1 Labour Rights |  |  |  |
| - The Project Developer shall ensure that all employment is in compliance with national labour occupational health and safety laws and with the principles and standards embodied in the ILO fundamental conventions - Workers shall be able to establish and join labour organisations - Working agreements with all individual workers shall be documented and implemented and include: - Working hours (must not exceed 48 hours per week on a regular basis), AND - Duties and tasks, AND - Remuneration (must include provision for payment of overtime), AND - Modalities on health insurance, AND - Modalities on termination of the contract with provision for voluntary resignation by employee, AND - Provision for annual leave of not less than 10 days per year, not including sick and casual leave. - No child labour is allowed (Exceptions for children working on their families’ property requires an Expert Stakeholder opinion) - The Project Developer shall ensure the use of appropriate equipment, training of workers, documentation and reporting of accidents and incidents, and emergency preparedness and response measures | No | The project complies with the Labour Law of the People's Republic of China, Special provisions on labour protection of female employees and other related regulation and policies. The project owner will sign contracts with employees. The labor contracts specify working hours, tasks and payments. All employees have benefits based on social security, pregnancy, maternity/paternity leave, or marital status according to the labor laws of China. Besides, the employees also have the right to establish labor unions and to carry on labor union activities. All employees would provide their age information document, e.g. ID, when signing the labour contract, and the project owner did not and will not employ any child labour. | Not required |
| Principle 6.2 Negative Economic Consequences |  |  |  |
| - Does the project cause negative economic consequences during and after project implementation? | No | The equipment procurement cost of the project will be borne by the project owner. The project has positive economic benefits due to the reduced cost of buying fertilizer for local farmer. The project will create fair full-time job opportunities for local people, which is good for the local economy. No risks are foreseen for the local economy through the realization of the project. | Not required |
| >> |  |  |  |
| Principle 7.1 Emissions |  |  |  |
| Will the Project increase greenhouse gas emissions over the Baseline Scenario? | No | The project activity is designed to new animal waste management systems to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. The project activity will reduce of GHG in the atmosphere through avoiding methane emissions from anaerobic treatment of swine manure and wastewater. | Not required |
| >> |  |  |  |
| Principle 7.2 Energy Supply |  |  |  |
| Will the Project use energy from a local grid or power supply (i.e., not connected to a national or regional grid) or fuel resource (such as wood, biomass) that provides for other local users? | No | The project activity will replace the current open anaerobic lagoons with 9 new closed anaerobic digesters. The biogas generated during the treatment process will be captured for power generation. All the power is used by the operation of AWMSs and the 9 swine farms and will not be connected to another user or to the regional power grid. So, the project activity will use electricity from the proposed project and the Central China Power Gird (only happened when the electricity consumption generated by the project is not sufficient), which is a reginal grid of China. It will not affect the energy consumptions or power supply or fuel resource supply that provides for other local users. | Not required |
| >> |  |  |  |
| Principle 8.1 Impact on Natural Water Patterns/Flows |  |  |  |
| Will the Project affect the natural or pre-existing pattern of watercourses, ground-water and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity? | No | The project will not affect the natural or pre-existing pattern of watercourses, groundwater and/or the watershed(s) such as high seasonal flow variability, flooding potential, lack of aquatic connectivity or water scarcity. This is confirmed by an expert invited by the project owner. | Not required |
| >> |  |  |  |
| Principle 8.2 Erosion and/or Water Body Instability |  |  |  |
| - Could the Project directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion? - Is the Project’s area of influence susceptible to excessive erosion and/or water body instability? | No | All animal manure will be put into the new animal waste management systems to treat and is prohibited to discharge into the Groundwater and surface water. Therefore, AWMSs used in the project area could not directly or indirectly cause additional erosion and/or water body instability or disrupt the natural pattern of erosion. The fertilizer produced in this project meets the relevant Chinese fertilizer implementation standards, so the project also could not directly or indirectly impact on surface and ground waters or soil erosion on slopes. | Not required |
| >> |  |  |  |
| Principle 9.1 Landscape Modification and Soil |  |  |  |
| Does the Project involve the use of land and soil for production of crops or other products? | No | Comprehensive utilization and resourceful treatment of the manure waste are encouraged by the local government. All the swine farms of the project have obtained necessary approval from the local government, including Planning Permit on Land for Construction Use, Environmental Impact Assessment etc. Hence, the project does not involve the use of land and soil for production of crops or other products. | Not required |
| >> |  |  |  |
| Principle 9.2 Vulnerability to Natural Disaster |  |  |  |
| Will the Project be susceptible to or lead to increased vulnerability to wind, earthquakes, subsidence, landslides, erosion, flooding, drought or other extreme climatic conditions? | No | The new animal waste management systems used in the project area does not involve any land use changes. It would not lead to the exacerbation of impacts caused by natural or man-made hazards, such as landslides or floods. | Not required |
| >> |  |  |  |
| Principle 9.3 Genetic Resources |  |  |  |
| Could the Project be negatively impacted by or involve genetically modified organisms or GMOs (e.g., contamination, collection and/or harvesting, commercial development, or take place in facilities or farms that include GMOs in their processes and production)? | No | The project does not involve the use of GMOs, so it will not have negatively impacted by the use of genetically modified organisms or GMOs. | Not required |
| >> |  |  |  |
| Principle 9.4 Release of pollutants |  |  |  |
| Could the Project potentially result in the release of pollutants to the environment? | No | The project is designed to introduces new animal waste management systems to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons. during the treatment process, the biogas was captured for power generation, the fermented sludge was treated in aerobic composting system, the wastewater was treated aerobically and then used for agriculture irrigation, so no pollutants release to the environment. | Not required |
| >> |  |  |  |
| Principle 9.5 Hazardous and Non-hazardous Waste |  |  |  |
| Will the Project involve the manufacture, trade, release, and/ or use of hazardous and non-hazardous chemicals and/or materials? | Potential | The project activity will replace the current open anaerobic lagoons with 9 new closed anaerobic digesters. The biogas generated during the treatment process will be captured for power generation. If biogas is not handled properly during the operation period of the project, methane explosion may be caused. | Required. For this project, the biogas will be generated during the treatment process, if biogas is not handled properly during the operation period of the project, methane leakage/explosion may be caused. So, the Mitigation Measures were needed. In this project, the mitigation measure for safeguarding principles was that project owner will conduct regular training on biogas safety and leakage to project employees to improve their proficiency in the treatment system and ensure that biogas leakage and safety hazards will not occur, and the employee training of biogas safety operation will be monitored through the training record. Also, the Mitigation Measure for Safeguarding Principles, i.e., the parameter of “Employee Training of biogas safety operation” was included in the monitoring plan in section B.7.1 and B.7.3. |
| >> |  |  |  |
| Principle 9.6 Pesticides & Fertilisers |  |  |  |
| Will the Project involve the application of pesticides and/or fertilisers? | No | The project activity does not involve any application of pesticides and/or fertilizers. | Not required |
| >> |  |  |  |
| Principle 9.7 Harvesting of Forests |  |  |  |
| Will the Project involve the harvesting of forests? | No | The project activity does not involve the harvesting of forests. | Not required |
| >> |  |  |  |
| Principle 9.8 Food |  |  |  |
| Does the Project modify the quantity or nutritional quality of food available such as through crop regime alteration or export or economic incentives? | No | The project activity does not involve the modification of the quantity or nutritional quality of food available. | Not required |
| >> |  |  |  |
| Principle 9.9 Animal husbandry |  |  |  |
| Will the Project involve animal husbandry? | No | The project is designed to introduces new animal waste management systems to treat the manure and wastewater from the 9 swine farms to avoid methane emissions generated in the baseline uncovered anaerobic lagoons, belongs to waste disposal and resource utilization. | Not required |
| >> |  |  |  |
| Principle 9.10 High Conservation Value Areas and Critical Habitats |  |  |  |
| Does the Project physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified? | No | The project is not located in an area within a high conservation value area or within critical natural habitats. The project activity does not physically affect or alter largely intact or High Conservation Value (HCV) ecosystems, critical habitats, landscapes, key biodiversity areas or sites identified. In addition, the “Measures for the Management of Livestock and Poultry Breeding in Hubei Province” clearly stipulate the prohibited construction area. | Not required |
| >> |  |  |  |
| Principle 9.11 Endangered Species |  |  |  |
| a. Are there any endangered species identified as potentially being present within the Project boundary (including those that may route through the area)? - Does the Project potentially impact other areas where endangered species may be present through transboundary affects? | No | There are no endangered species identified as potentially being present within the Project boundary. In addition, the new animal waste management systems used in the project will not pose a threat to any species. | Not required |
| >> |  |  |  |

Appendix 2- Contact information of project participants

| Organization name | Jiangxi Zhengbang Breeding Co. Ltd |
| --- | --- |
| Registration number with relevant authority | 913601007542124562 |
| Street/P.O. Box | north of Hu Dong 4th Road and east of Chanye Road |
| Building | / |
| City | Nanchang City |
| State/Region | Jiangxi province |
| Postcode | 330096 |
| Country | China |
| Telephone | 135 6445 9487 |
| E-mail | 57566640@qq.com |
| Website | http://www.zhengbang.com/index.html |
| Contact person | Shen Min |
| Title | Deputy General Manager |
| Salutation | Mr. |
| Last name | Shen |
| Middle name | / |
| First name | Min |
| Department | / |
| Mobile | / |
| Direct tel. | +86 0791--8639 7834 |
| Personal e-mail | 57566640@qq.com |

| Organization name | Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. |
| --- | --- |
| Registration number with relevant authority | 91310115301334442X |
| Street/P.O. Box | No.2815 Longteng Avenue |
| Building | Sales Center of Baihuiyuan |
| City | Shanghai |
| State/Region | / |
| Postcode | 200232 |
| Country | China |
| Telephone | +86 21 6127 2386 |
| E-mail | shulan.tu@profitcarbon.com |
| Website | www.profitcarbon.com |
| Contact person | Tu shulan |
| Title | Senior Project Manager |
| Salutation | Ms. |
| Last name | Tu |
| Middle name | Shu |
| First name | Lan |
| Department | Technology Department |
| Mobile | 186 9490 7124 |
| Direct tel. | +86 21 6127 2386 |
| Personal e-mail | shulan.tu@profitcarbon.com |

Appendix 3- LUF Additional Information

Not applicable.

Appendix 4-Summary of Approved Design Changes

Not applicable.

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.2 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Inclusion criteria table added Gender sensitive requirements added Prior consideration (1 yr rule) and Ongoing Financial Need added Safeguard Principles Assessment as annex and a new section to include applicable safeguards for clarity Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on Stakeholder Consultation information required Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.1 | 24 August 2017 | Updated to include section A.8 on ‘gender sensitive’ requirements |
| 1.0 | 10 July 2017 | Initial adoption |

http://www.mee.gov.cn/xxgk2018/xxgk/xxgk02/202101/t20210105_816131.html

http://mee.gov.cn/xxgk2018/xxgk/xxgk03/202012/W020201230736907682380.pdf

http://www.moa.gov.cn/nybgb/2017/dbq/201801/t20180103_6134011.htm

http://www.gov.cn/zhengce/zhengceku/2021-12/27/content_5664765.htm

http://gsxt.amr.jiangxi.gov.cn/stateinfopages/yxgs/companyinfo.jsp?pripid=MzYwMTAwNDAwMDAzMDY4OQu002Cu002C&zchregno=MzYwMTAwNTIwMDAyNDY4&regno=MzYwMTAwNTIwMDAyNDY4

https://www.creditchina.gov.cn/xinyongxinxixiangqing/xyDetail.html?searchState=1&entityType=1&keyword=%E4%B8%8A%E6%B5%B7%E7%9B%88%E7%A2%B3%E7%8E%AF%E5%A2%83%E8%83%BD%E6%BA%90%E7%A7%91%E6%8A%80%E6%9C%89%E9%99%90%E5%85%AC%E5%8F%B8&uuid=f314d8d01428b2cab58f6afd8d5f3d69&tyshxydm=91310115301334442X

http://www.gsxt.gov.cn/index.html

http://www.gov.cn/zhengce/zhengceku/2021-12/27/content_5664765.htm

https://cdm.unfccc.int/filestorage/O/J/W/OJW9BM0HFSPXNGVD1KER3C2QU86I5T/EB%2075_repan14_ACM0010_ver%2008.0.pdf?t=dFJ8cjE3cjdhfDCyF2GhsQBFiRMqyhMti42B

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-7.0.pdf/history_view

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-05-3.0.pdf/history_view

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-06-3.0.pdf/history_view

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-08-3.0.pdf/history_view

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-14-v2.pdf/history_view

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-24-v1.pdf

https://baike.baidu.com/item/%E6%B1%9F%E8%A5%BF%E6%AD%A3%E9%82%A6%E5%85%BB%E6%AE%96%E6%9C%89%E9%99%90%E5%85%AC%E5%8F%B8/2002449?fr=aladdin

湖北_百度百科 (baidu.com)

http://politics.people.com.cn/n/2013/1126/c1001-23662445.html

https://oss.baigongbao.com/2020/12/14/MRyhTKQcWC.pdf

http://www.moa.gov.cn/gk/tzgg_1/tfw/201801/t20180111_6134801.htm

https://xueqiu.com/1422246446/180509860

https://www.cadforex.com/tongzl/90515.html

https://wiki.mbalib.com/wiki/%E6%8A%98%E7%8E%B0%E7%8E%87

Only a small portion of the solid organic fertilizers generated by the project will be supplied to the farmers living around for free due to few villagers live in the surrounding area and the effluent through anaerobic process will also be used for irrigation the surrounding crops. The extra organic fertilizer will be sold into the market, which is the most direct revenue for PP for this project.

https://unfccc.int/

https://cdm.unfccc.int/

https://cdm.ccchina.org.cn/

https://www.goldstandard.org/

https://verra.org/

http://nyt.hubei.gov.cn/

https://cdm.unfccc.int/

https://www.goldstandard.org/

https://verra.org/

湖北_百度百科 (baidu.com)

This formula refers to the formula 10.30 in chapter 10 volume IPCC2006.

The parameter table numbers are added for all parameter involved in methodology. However, table numbers for other parameters sourced from2006 IPCC such as Nrate (T), TAM and parameters such as EFCH4,default , EFEF,j,y, Ru, MMi and ηflare,m that is sourced from the applied Tool 14,Tool 05, Tool 08 and Tool 06 respectively is not included.

As per tool 14:” project and leakage emission from anaerobic digesters (version 02.0)”, the value this parameter should be determined according to the type of digester used in the project activity and the digester type shall be identified by manufacturer information. For this project, as per the purchasing contract of Anaerobic digester, the type of digester used in the project activity belongs to UASB type digester, so the Default emission factor for the fraction of CH4 produced that leak from the anaerobic digester (fraction) is 0.05 t CH4 leaked / t CH4 produced.

For proposed project, Before the treated manure is applied to the land, it undergoes three stages of homogenization pretreatment, anaerobic fermentation, and aerobic composting. The corresponding RVS of each stage is 2%, 45%, and 35%, respectively. In addition, the leakage calculated based on this value is more conservative than the 80% of the cover first cell of two lagoon in Appendix 1 of the methodology.

For proposed project, the baseline is uncovered anaerobic lagoon which belongs to the anaerobic treatment of One-cell lagoon in the Appendix 1 of applied methodology ACM0010 (version 08.0), so, the RVS,n is 85% which is the most conservative value.

The treatment process of this project is an anaerobic-aerobic combined treatment technology. The anaerobic process is carried out in a closed anaerobic fermentation tank, so the treatment process of this project is similar to the covered first cell of two cell lagoon in the applied methodology appendix 1. so, the RN,n is 25% which is the most conservative value.

For proposed project, the baseline is uncovered anaerobic lagoon which similar to the anaerobic treatment of One-cell lagoon in the Appendix 1 of applied methodology ACM0010 (version 08.0), so, the RN,n is 80% which is the most conservative value.

https://cdm.unfccc.int/DNA/bak/index.html

https://www.mee.gov.cn/ywgz/ydqhbh/wsqtkz/202012/t20201229_815386.shtml. Partial electricity used by the project are from Central China Power Grid (CCPG). And as per 2019 China regional power grid carbon dioxide baseline emission factor published by Ministry of Ecology and Environment of the People’s Republic of China, the EFgrid,OM,y of CCPG is 0.8587 tCO2/MWh and the EFgrid,BM,y of CCPG is 0.2854 tCO2/MWh. Therefore, based on equation (16) of Tool 07 (Version 7.0), EFgrid,CM,y of CCPG can be calculated as 0.8587 tCO2/MWh *0.5+0.2854 tCO2/MWh *0.5=0.57205 tCO2/MWh

this parameter is not used in the calculation of emission reductions, but according to applied methodology ACM0010 (version08.0), this parameter needs to be monitored.

According to the material balance diagram in the Feasibility Study Report, the number of materials entering the anaerobic system is 100% and the number of materials entering the aerobic system is 54%.

The proposed project is located in Jiangxi Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

http://www.doc88.com/p-9902182440715.html

https://www.doc88.com/p-73947158230.html

According to “Principles & Requirements (version 1.2)”, the project start date is defined as: Examples of the start date may include the date on which contracts have been signed for equipment or construction/operation services required for the Project. Minor pre-project expenses, such as the contracting of services/payment of fees for feasibility studies or preliminary surveys, shall not be considered in the determination of the start date as they do not necessarily indicate the commencement of implementation of the Project. For distributed technology projects, the start date is the date of implementation of the first unit under the project.

This project is not a distributed technology project, so the start date of this project is the date on which contracts have been signed for equipment or construction/operation services required for the Project. The earliest purchase date of the biogas power generator of the project is 09/09/2020, so the Start date of project is 09/09/2020.

As per manufacturer specifications of the power generator, the technical lifetime is more than 15 years.

The date of all the Animal Manure Management System put into operation.

For this project, the biogas will be generated during the treatment process, if biogas is not handled properly during the operation period of the project, methane leakage/explosion may be caused. So, the Mitigation Measures were needed. In this project, the mitigation measure for safeguarding principles was that project owner will conduct regular training on biogas safety and leakage to project employees to improve their proficiency in the treatment system and ensure that biogas leakage and safety hazards will not occur, and the employee training of biogas safety operation will be monitored through the training record. Also, the Mitigation Measure for Safeguarding Principles, i.e., the parameter of “Employee Training of biogas safety operation” was included in the monitoring plan in section B.7.1 and B.7.3.

http://www.npc.gov.cn/zgrdw/npc/xinwen/2019-01/07/content_2070261.htm

http://jxf.jiangxi.gov.cn/art/2020/7/24/art_39212_2648549.html

http://sft.hubei.gov.cn/bsfw/bmcxfw/gfxwjcxxt/202102/t20210203_3333799.shtml